// FILE:    KG_RAM_Link1.cmd

//

// TITLE:   Linker Command File For 28335 examples that run out of RAM

//

//          This ONLY includes all SRAM blocks on the 28335 device.

//          This does not include flash or OTP.

//

//          Keep in mind that L0 and L1 are protected by the code

//          security module.

//

//          What this means is in most cases you will want to move to

//          another memory map file which has more memory defined.

/* Define the memory block start/length for the F28335

PAGE 0 will be used to organize program sections

PAGE 1 will be used to organize data sections

Notes:

Memory blocks on F28335 are uniform (ie same

physical memory) in both PAGE 0 and PAGE 1.

That is the same memory region should not be

defined for both PAGE 0 and PAGE 1.

Doing so will result in corruption of program

and/or data.

L0/L1/L2 and L3 memory blocks are mirrored - that is

they can be accessed in high memory or low memory.

For simplicity only one instance is used in this

linker file.

Contiguous SARAM memory blocks can be combined

if required to create a larger memory block.

*/

MEMORY

{

PAGE 0 :

/* BEGIN is used for the "boot to SRAM" bootloader mode      */

/* BOOT_RSVD is used by the boot ROM for stack.               */

/* This section is only reserved to keep the BOOT ROM from    */

/* corrupting this area during the debug process              */

BEGIN      : origin = 0x000000, length = 0x000002     /* Boot to M0 will go here                      */

BOOT_RSVD  : origin = 0x000002, length = 0x00004E     /* Part of M0, BOOT rom will use this for stack */

RAMM0      : origin = 0x000050, length = 0x0003B0

RAML0      : origin = 0x008000, length = 0x001000

RAML1      : origin = 0x009000, length = 0x001000

RAML2      : origin = 0x00A000, length = 0x001000

RAML3      : origin = 0x00B000, length = 0x001000

ZONE7A     : origin = 0x200000, length = 0x00FC00    /* XINTF zone 7 - program space */

CSM_RSVD   : origin = 0x33FF80, length = 0x000076     /* Part of FLASHA.  Program with all 0x0000 when CSM is in use. */

CSM_PWL    : origin = 0x33FFF8, length = 0x000008     /* Part of FLASHA.  CSM password locations in FLASHA            */

ADC_CAL    : origin = 0x380080, length = 0x000009

RESET      : origin = 0x3FFFC0, length = 0x000002

IQTABLES   : origin = 0x3FE000, length = 0x000b50

IQTABLES2  : origin = 0x3FEB50, length = 0x00008c

FPUTABLES  : origin = 0x3FEBDC, length = 0x0006A0

BOOTROM    : origin = 0x3FF27C, length = 0x000D44

PAGE 1 :

RAMM1      : origin = 0x000400, length = 0x000400     /* on-chip RAM block M1 */

RAML4      : origin = 0x00C000, length = 0x001000

RAML5      : origin = 0x00D000, length = 0x001000

RAML6      : origin = 0x00E000, length = 0x001000

RAML7      : origin = 0x00F000, length = 0x001000

ZONE7B     : origin = 0x20FC00, length = 0x000400     /* XINTF zone 7 - data space */

}

SECTIONS

{

/* Setup for "boot to SARAM" mode:

The codestart section (found in DSP28_CodeStartBranch.asm)

re-directs execution to the start of user code.  */

codestart        : > BEGIN,     PAGE = 0

ramfuncs         : > RAML0,     PAGE = 0

.text            : > RAML1,     PAGE = 0

/*   .cinit           : > RAML0,     PAGE = 0		*/

/*   .pinit           : > RAML0,     PAGE = 0		*/

.switch          : > RAML0,     PAGE = 0

.data			: > RAML2,	   PAGE = 0		/* KG added line */

.bss				: > RAML3,	   PAGE = 0		/* KG added line */

.stack           : > RAMM1,     PAGE = 1

.ebss            : > RAML4,     PAGE = 1

.econst          : > RAML5,     PAGE = 1

.esysmem         : > RAMM1,     PAGE = 1

IQmath           : > RAML1,     PAGE = 0

IQmathTables     : > IQTABLES,  PAGE = 0, TYPE = NOLOAD

IQmathTables2    : > IQTABLES2, PAGE = 0, TYPE = NOLOAD

FPUmathTables    : > FPUTABLES, PAGE = 0, TYPE = NOLOAD

/*   DMARAML4         : > RAML4,     PAGE = 1  */

/*   DMARAML5         : > RAML5,     PAGE = 1  */

/*   DMARAML6         : > RAML6,     PAGE = 1  */

/*   DMARAML7         : > RAML7,     PAGE = 1  */

ZONE7DATA        : > ZONE7B,    PAGE = 1

.reset           : > RESET,     PAGE = 0, TYPE = DSECT /* not used                    */

csm_rsvd         : > CSM_RSVD   PAGE = 0, TYPE = DSECT /* not used for SARAM examples */

csmpasswds       : > CSM_PWL    PAGE = 0, TYPE = DSECT /* not used for SARAM examples */

/* Allocate ADC_cal function (pre-programmed by factory into TI reserved memory) */

.adc_cal     : load = ADC_CAL,   PAGE = 0, TYPE = NOLOAD

}