;****************************************************************************************

; file = ex0.asm

; Quick examples of Assembler directives & 28F335 DSP code

; Dr. Eric Schwartz, Jan 23, 2012

; Dr. Eric Schwartz, Jan 24, 2011

; Dr. Karl Gugel, May/2009

;

; To be assembled using Code Composer Studio which requires a linker command

; file to tell CCR where to place code & data into DSP SRAM.

; The command file used = 28335_RAM_lnk.com

; Important Code locations:

;	.text	RAML1   (internal DSP memory) starting address = 09000 Hex, 4K Words

;	.data   RAML2	(internal DSP memory) starting address = 0A000 Hex, 4K Words

;

; Special Note:

;		Assembler directives are used to place data and variables into memory.

;		They are not F335 instructions and thus are not executed at "run-time".

;		When this program is loaded into memory, the data (created above by the

;		assembler) is also copied down into memory. This is called "load-time".

;

.global		_c_int00	;This assembler directive allows _c_int00 to be a

;  global variable. This tells the linker where your

;  program (.text code) begins and where to boot from.

;

; Additional References:

; 	Assembler Directives: spru513c.pdf

;	Memory Map/Hardware Related: sprs439i.pdf

;	CPU Registers & Assembly Code: spru430.pdf (version E)

;****************************************************************************************

;***************************** Program Constants ****************************************

; Creating constants using the .set assembler directive. This should be at the top of your

; program.  This is like a define statement in C.

num1		.set	0h		;assembly-time constant (hex number)

num2		.set	11110000b	;assembly-time constant (binary no.)

num3		.set	65535		;assembly-time constant (decimal no.)

count		.set	3		;number of characters to add in EEL4744 ('E'+'E'+'L')

data_sect	.set	0xa000   	;constant that is actually the starting addr of .data section

bss_sect	.set	0xb000   	;constant that is actually the starting addr of .bss section

;****************************************************************************************

;******************* DATA ALLOCATION SECTION - Variables/Data ***************************

; Data can go before or after your program code but should not be placed in the middle

; nof a program for clarity reasons.

.data	;data section, see the command linker file, this puts the

;  following data defined below in a block of internal SRAM

;  starting at 0xA000.

counter	.word   0h  	;define word, two bytes placed in memory stating in the data section

num4   	.word   01234h	;define word, two bytes placed in memory

char  	.byte   'E' 	;define string, ASCII characters placed in memory as WORDS!

.byte   'E', 'L', '4', '7', '4', '4'

.byte	"0123"	;here is another way to specify a string of WORDS!

.word	0xfab4	;another way to specify a hex number (WORD)

val1  	.word	32		;places 32 decimal (20 hex) in memory (WORD) at label val1 at load-time

val2  	.word	512		;places 512 decimal(200 hex) in memory (WORD) at label val2 at load-time

val3  	.long	0x12345678	;places two words in memory (lower word/lower addr, LITTLE endian)

;.BSS SECTION is used to reserve space in SRAM for run-time results.

; See the command linker file, the starting address is 0xB000

.bss	results,3	;reserves three words at label 'results' in the .bss section

.bss	sum,1		;reserves one word at label 'sum' in the .bss section

;.global directive lets you to see the assigned addresses in map file.

.global		num1,num2,num3,num3,num4,counter,char,val1,val2,val3,results,sum

;****************************************************************************************

;******************** Brief Introduction to CPU Model ***********************************

; CPU Registers:

;	ACC		Accumulator (32 bits) comprised of AH (upper 16 bits) and AL (lower 16 bits)

;	XAR0	Auxiliary Register0 (32 bits) comprised of AR0H (upper 16 bits) and AR0 (lower 16 bits)

;	XAR1	Auxiliary Register1 (32 bits) comprised of AR1H (upper 16 bits) and AR1 (lower 16 bits)

;	XAR2	Auxiliary Register2 (32 bits) comprised of AR2H (upper 16 bits) and AR2 (lower 16 bits)

;	XAR3	Auxiliary Register3 (32 bits) comprised of AR3H (upper 16 bits) and AR3 (lower 16 bits)

;	XAR4	Auxiliary Register4 (32 bits) comprised of AR4H (upper 16 bits) and AR4 (lower 16 bits)

;	XAR5	Auxiliary Register5 (32 bits) comprised of AR5H (upper 16 bits) and AR5 (lower 16 bits)

;	XAR6	Auxiliary Register6 (32 bits) comprised of AR6H (upper 16 bits) and AR6 (lower 16 bits)

;	XAR7	Auxiliary Register7 (32 bits) comprised of AR7H (upper 16 bits) and AR6 (lower 16 bits)

;	XT	Multiplicand Register (32 bits) comprised of T (upper 16 bits) and TL (lower 16 bits)

;	P	Product Register (32 bits) comprised PH (upper 16 bits) and PL (lower 16 bits)

;	PC	Program Counter (22 bits)

;	SP	Stack Pointer (16 bits)

;	DP	Data Page Register (16 bits)

;	ST1,ST0	Status Registers (flags)

;****************************************************************************************

;****************** F335 Program Examples ***********************

.text	;Program section, see the command linker file, program code

;  should be placed in the text section which starts at 0x9000

_c_int00:	;This label tells the linker where the entry (starting) point for

;  the first instruction in your program.

; Below demonstrates immediate and register to register addressing

; Short example1 to sum 1st three chars in string EEL4744.

; This illustrates immediate, indirect and register addr modes

MOV	AR0,#char   	;address of 1st char in EEL4744

MOV	AR1,#sum   		;address of where final sum (result)will be stored

MOV	AR2,#counter	;address of counter saved in memory

MOV	AH,#count   	;load the count value & save in memory counter

MOV	*AR2,AH

MOV	AL,#0   		;clear initial sum

TOP1		ADD	AL,*AR0   	;add char value to sum

INC	AR0     		;increment pointer used to get a char value

DEC	*AR2    		;decrement counter in memory

B	TOP1,NEQ		;if the Z flag is 0, branch up and repeat adding

MOV	*AR1,AL

NOP				;No operation to view sum in memory at addr results

NOP				;  put some instructions (space) inbetween the next example

;  for stepping through with the debugger so no regs change.

; Short example2 to illustrate direct addressing and mathematical & shift operations.

MOV	DP,#data_sect>>6		;set datapage pointer to point to .data section

MOV	T,@val1			;load T using direct addressing mode

MPYU	ACC,T,@val2		;val1[] * val2[] => ACC, 16 bit unsigned multiply

MOV	DP,#bss_sect>>6		;set datapage pointer to point to .bss section

MOV	@results,AL		;save lower word only, which assumes answers is 16 bits or less

MOV	DP,#data_sect>>6		;set datapage pointer back to .data section

MOV	AL,@val1			;add val1[] + val2[]

ADD	AL,@val2

MOV	DP,#bss_sect>>6	;set datapage pointer to point to .bss section

MOV	@results+1,AL	;save val1[] + val2[]

;Shift left example

MOV	AH,#num3		;set AH to a predetermined value to see if the

;  next instruction affects AH

LSL	AL,#5			;logical shift left AL by 5 places

MOV	@results+2,AL

NOP

NOP

END1		B	END1,UNC		;infinite loop to prevent program from trying to execute

;  un-initialized (no program) memory.