“Pin-occhio”

Final Report

University of Florida

Department of Electrical and Computer Engineering

EEL 5666

Intelligent Machines Design Laboratory

Jeremy Mayer

12/06/00

### Table of Contents

PAGE

ABSTRACT								3

INTRODUCTION							3

INTEGRATED SYSTEM						4

ACTUATION								6

SENSORS								7

SENSOR TESTING							8

TESTING RESULTS							9

BEHAVIORS								10

CONCLUSION							12

APPENDIX								14

### Abstract

This paper describes the design of an intelligent machine that will display 3D objects and digitize 3D objects for display as a computer image.  The apparatus will consist of many axially aligned steel pins.  When the pins are displaced, together they form 3D surfaces.

### Introduction

The objective for this apparatus is the ability to display 3D objects and the ability to digitize 3D objects.  After brainstorming possible implementations for the 3D display device, I decided that the device should actually create physical three-dimensional surfaces.  This requirement excluded any implementation that involves a projection of light to somehow create the illusion of a 3D object (such as a hologram).

I also thought that the device should not require the use of consumables and the device should be able to quickly reconfigure itself in order to display another object.  This would set the device apart from current rapid prototyping technologies.  These machines require a type of consumable material which is used to construct a 3D object and the time to construct an object can be several hours.

The approach that I ended up selecting, is what I refer to as “pin-occhio”.  It helps to visualize the child’s toy that has hundreds of steel pins axially aligned (see photo in Figure 1).  By placing an object into the pin matrix (a hand for example), the pins get axially displaced, and a projection of the object is displayed on the other side.  My intent is to design and build a pin matrix apparatus that can be controlled by the computer.

Figure 1. Picture of pin matrix

Integrated System

When I began thinking about designing a machine that would be able to control the displacement of many pins, it was clear that one of the difficulties to overcome would be the proximity of the individual pins.  Because I wanted the pins to be tightly packed together (~2mm on center), it would be difficult to fit a mechanism that could control the individual pin displacements into the same space.

The solution that I came up with to this problem was to use push-pull cables to control the individual pins.  With these cables, I can spread the pin proximity from 1mm on center to 1inch on center, which will allow sufficient space for the displacement control mechanism.

A schematic diagram of the pin displacement control mechanism is shown in Figure 2.  The diagram in Figure 2 is a side view and only contains 5 cables.  An actual apparatus may fit up to 2000 cables into the upper plate.  For this amount of pins, the upper plate could be separated and placed at different levels to conserve space.

The sleeves of the push-pull cables are secured into the upper plate. The position of the upper plate is controlled by a stepper-motor & lead screw to translate up and down (always parallel to the lower plate).  The lower plate is part of the lower chassis.  Also attached to the lower chassis are parts called swivels, which can be toggled out of the path of the cable by activating the appropriate solenoid.

Upper Plate Translates.

Solenoids

Swivels

Push-Pull Cables

Figure 2. Schematic diagram of the mechanism

If the upper plate is translating downward and none of the solenoids are energized, all 5 cables will be pushed by the top surface of the swivel and will be displacing the respective pins at the other end.  When a solenoid is energized (see 2nd solenoid above), the top surface of the swivel is moved out of the path of the cable, so the cable and attached pin will stop translating up, while all of the neighboring cables will still be translating.

With this system, the displacement of every pin can be controlled by energizing the correct solenoid at the correct time during the translation of the upper plate.

A schematic of the entire systems is shown in Figure 3.

Stepper Motor and Lead Screw to move upper plate

In(8) Out(8)

MC68HC11

Stepper Motor Driver

5V   Power Supply

12V (Dual Votage)

8 Relays

Pins attatched at ends of these cables

Display Panel

*bump switch

*toggle switch

*led

Figure 3. System block diagram

Actuation

Pin-occhio has 11 actuators which are controlled by the HC11 microprocessor.

- Stepper Motor (1)

- Solenoids (8)

- Bump Switch (1)

- Toggle Switch (1)

The Stepper Motor has a motor controller box that requires a square wave input from the microprocessor for speed and position control.  The Solenoids will be connected to relays because they require high current.  A bump switch is used as a continue button and a toggle switch is used as a mode switch.

### Sensors

This apparatus has a newly designed novel sensor for pin displacement sensing.  When the apparatus operates as an input device, the part in Figure 2, labeled swivel acts as the pin displacement sensor.  Figure 4 illustrates how the sensor operates. When the core of the push-pull cable passes the top of the sensor, the sensor snaps to the closed position.  This closes a circuit.  At that instant, the microprocessor will look at a timer and determine the pin displacement.

+5V

+5V

Input Port

(reads 5V)

Input Port

(reads 0V)

Push/Pull Cables

Figure 4. Schematic of sensor operation

Sensor Testing

A simple test was developed to examine the performance of the pin offset sensors.  A flow chart for this test is shown in Figure 5.

Start

Send Staircase Pattern to Pins

Finish

Gather Pin Offset Sensor Data

Perform Data Manipulation

Send Data to Pins

Figure 5. Pin offset sensor test flow chart

A staircase pattern is sent to the pins, then the sensors are used to record back those pin displacements.  Some data manipulation is required before the recorded data can be sent back to the pins.  If the data is sent directly, during playback many of the solenoids will be energized much longer than they need to be.  The data manipulation routine looks at the 16 bytes of recorded data for transitions from logic one to logic zero.  Only the individual bits at the transition point will remain at logic zero.  That bit will be changed to logic one for the remaining bytes of data.  This ensures that the solenoid will only fire at the proper time slice.

Figure 6. Pin offset sensor test results

Testing Results

The results shown in Figure 6 are nearly perfect.  In each trial there is an error only in the Pin 6 displacement data.  Although I was not able to correct this problem during the development of this test, I am confident that I will be able to fix the sensor to perform as well as its neighbors.  I will also change the code to provide 32 index positions instead of 16 index positions over the 24mm total offset.  The goal will be the ability to read the pin displacements to a tolerance of  0.75mm with 100% accuracy.

### Behaviors

Pin-occhio has 3 behaviors:

- Display a 3D image from the computer screen.

- Digitize a 3D surface for conversion to a computer screen image.

- Stand-Alone Mode: Input a 3D object and then display a mirror image of the same object.

The first two behaviors are used together with a Java applet program running on a PC.  The Java applet has a VRML graphics image of the pins.  On the computer screen the pins are displaced and then the send button is pressed.  Serial data that represents the pin offsets are sent to the HC11.  The HC11 then controls the mechanism to display the proper pin offsets.  The reverse is also possible.  A 3D surface is pressed into the actual pins and this data is sent to the PC for graphics display.

The Internet

Server

Client Applet

Figure 7. The signed applet talks to server using a socket connection

Figure 7 illustrates how the simulation is connected to the internet and Figure 8 is a flow chart for the program running on the HC11. The actual code can be found in the appendix.

Poll for SCI 34 Bytes

Extend Pins to Max Height

Display the 3D Data

Wait for Enter Press

Wait for Enter Press

Read back Pin Offsets

Send SCI 34 Bytes

Start

Mode

(1) Display Mode                               (2) Digitize Mode

Figure 8. Flow chart for the HC11 program (behaviors 1 and 2)

The third behavior is controlled completely by an HC11 assembly language program.  A shape is pressed into the pins and the machine will then display the inverse of the inputted shape.  A flow chart for this program is shown in Figure 9.  Much of the code for this behavior was cut from the code that is used for behaviors 1 and 2 (see appendix).

Extend Pins to the Max Height

Wait for Enter Press

Read back Pin Offsets

Perform Data Manipulation

Send Data to Pins

Wait for Enter Press

Clear Pins

Start

Figure 9. Flow chart for the HC11 program (behavior 3)

### Conclusion

This 8 pin prototype was a warm-up project for something bigger and better to come in the future.  My thesis project at CIMAR (center for intelligent machines and robotics) will involve the construction of a similar mechanism but one that will have many more pins.  With more pins, actual 3D surfaces will be able to be generated, as opposed to the planar shapes that this mechanism can generate.

Overall the prototype has proved that the idea will work.  In addition, I have learned valuable lessons that should make the next mechanism better.  One of the lessons learned involves the pin offset sensor.  Closing the circuit did not always work as I had hoped, which lead to bad data.  In the follow up design I would like to try to incorporate an optical sensor at each pin in order to read pin offsets.  This should provide much more reliable data.  Another lesson, had to do with power consumption.  The current design does not lend itself well to expansion because the solenoids draw too much power.  For the next design I will need to find an actuator that has a smaller travel and one that requires much less power.

As the number of pins increases, the question that comes to mind is, “where will I find more output ports on the controller to control them all?”  The solution I hope will be ‘raster scanning’ the solenoids.  This is the first bit of future work that I will need to do before even beginning the design of the bigger mechanism.  The circuit will be more complex than controller to relay to solenoid, but with a raster scan the number of output ports needed is now only two times the square root of the amount of pins.

### Appendix

Assembly code for program for behaviors 1 and 2:

*********************************************************************

* Pinocchio Demo Day Code, Jeremy Mayer, 11/30/00

*********************************************************************

***************************************************

** SYMBOL DEFINITIONS

***************************************************

* 68HCII REGISTERS

***************************************************

STACK	EQU	$8FFF

BASE	EQU	$1000	;BEGINNING OF REGISTERS

PORTA	EQU	$00

TOC2	EQU	$18	;OUTPUT COMPARE 2 REGISTER

TCTL1	EQU	$20	;TIMER CONTROL REGISTER

TMSK1	EQU	$22	;TIMER MASK1 REGISTER

TFLG1	EQU	$23	;TIMER FLAG1 REGISTER

TMSK2	EQU	$24	;TIMER MASK2 REGISTER

SCSR	EQU	$2E

SCDR	EQU	$2F

BAUD	EQU	$2B

SCCR1	EQU	$2C

SCCR2	EQU	$2D

EOS	EQU 	$04    ; User-defined End Of String (EOS) character

OUTP2  	EQU $4000

* MASKS

BIT7	EQU	%10000000

BIT6	EQU	%01000000

BIT10	EQU	%00000011

BIT1	EQU	%00000010

BIT0	EQU	%00000001

INV6	EQU	%10111111

***************************************************

** DATA SECTION

***************************************************

ORG	$00DC

JMP	OC2_ISR	;OC2 INTERRUPT VECTOR (IN BUFFALO)

ORG	$20

PWMLO	FDB	$300	;LOW OUTPUT TIME

PWMHI	FDB	$300	;HIGH OUTPUT TIME

CTR	RMB	2

CTR2  	RMB	2

STOP	RMB	1

CTR3	FCB	$00

CTR4 	RMB	1

BITCOM	FCB	$00

LASTBYT	RMB	1

TEMP	RMB	1

ORG	$8000	;Data Blocks of 34 solenoid patterns

TABLE	RMB	34

FCB	EOS

FCB	EOS

***************************************************

** MAIN PROGRAM

***************************************************

ORG	$9000

*--------------------------------------------------

* INITIALIZATION

*--------------------------------------------------

* INITIALIZE STACK & Registers

LDS	#STACK

LDX	#BASE

* Set the Pre-Scaler Frequency for TCNT (during first 64 cycles)

BCLR	TMSK2,X BIT10	;PR1:PRO=00 FOR 32.768 msec (default)

LDAA #$FF		;close all solenoids

STAA OUTP2

JSR InitSCI

* INITIALIZE OUTPUT COMPARE OC2

* (Bit7:6=OM2:OL2  Hi=11, Lo=10, Toggle=01, Disconnect=00)

BSET	TCTL1,X BIT7	;OM2:OL2=10

BCLR	TCTL1,X BIT6	;  FOR SET TO LOW

begin	ldaa #$00

staa CTR3

staa BITCOM

staa LASTBYT

JSR MODE		;puts mode into accum. A

ANDA #%00000001

BEQ digit

bra display

digit	jmp digitiz

display

************1. loop till recieve 34 bytes

************2. pins 4 display algorithm

************3. wait

************4. clear

ldab #$00

ldy #TABLE

byteme

jsr InChar

staa 0,Y

incb

cmpb #34		;get the 34 bytes into table

beq GOTIT

iny

bra byteme

GOTIT

LDX #BASE

LDAA PORTA,X		; set plate to go down

ANDA #%11101111

STAA PORTA,X

*--------------------------------------------------

*2. Step through 34 increments

*--------------------------------------------------

ldx #$8000

LDAB 0,X		;index 0

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 1

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 2

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 3

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 4

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 5

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 6

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 7

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 8

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 9

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 10

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 11

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 12

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 13

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 14

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 15

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 16

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 17

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 18

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 19

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 20

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 21

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 22

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 23

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 24

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 25

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 26

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 27

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 28

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 29

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 30

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 31

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 32

stab OUTP2

JSR ADV

INX

LDAB 0,X		;index 33

stab OUTP2

JSR ADV

*********** stepper motor now at down position

LDAA #$FF

STAA OUTP2

JSR WAIT

JSR CLEAR

jmp begin

digitiz

************1. extend pins				;put digitize3D code here

************2. wait

************3. record

************4. send 34 bytes

ldaa #$00		;initialize data manipulation variables

staa CTR3

staa BITCOM

staa LASTBYT

LDX #BASE

LDAA PORTA,X		; set plate to go down

ANDA #%11101111

STAA PORTA,X

*--------------------------------------------------

* Get all Pins in upmost postion and open all solenoids

*--------------------------------------------------

HERE

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV			;#28

JSR ADV

JSR ADV

JSR ADV

JSR ADV

LDAB #00		;index 32 (Open all Solenoids)

stab OUTP2

JSR ADV

JSR ADV		;34th ADV

*********** stepper motor now at down position

LDAA #$FF		;close all Solenoids

STAA OUTP2

JSR WAIT

JSR RECORD

JSR MANIP

LDX #$8000		;send out 35 bytes on Serial Port

JSR OutStr

jmp begin

***************************************************

** SUBROUTINE MANIP - data manipulation: recorded bytes are

** cleaned up to be used to display back pin offsets.  Only bit transitions

** from 1 to 0 remain 0 in the cleaned up data.  This ensures that the

** solenoids will only come on at one time slice on play back.

***************************************************

MANIP	ldx #$8000

B0	ldaa 0,x

staa TEMP

ldab BITCOM

andb #%00000001			;? is BITCOM bit0 == 0?

bne SETNX0

bra G0

SETNX0	ldaa 0,x			;this pin is complete keep storing 1's

ora #%00000001

staa 0,x

bra B1

G0

ldaa LASTBYT

anda #%00000001			;? is LASTBYT bit0 == 1? brnch if 0

beq SETNX0

ldaa 0,x

anda #%00000001			;? is bit0 == 0? brnch if 1

bne SETNX0

*	;this is a valid solenoid throw spot

*	;bit0 is already clear

ldab BITCOM

orb #%00000001

stab BITCOM			;set BITCOM bit0

****************************************************************

B1

ldab BITCOM

andb #%00000010			;? is BITCOM bit1 == 0?

bne SETNX1

bra G1

SETNX1	ldaa 0,x			;this pin is complete keep storing 1's

ora #%00000010

staa 0,x

bra B2

G1

ldaa LASTBYT

anda #%00000010			;? is LASTBYT bit1 == 1?

beq SETNX1

ldaa 0,x

anda #%00000010			;? is bit1 == 0?

bne SETNX1

*	;this is a valid solenoid throw spot

*	;bit0 is already clear

ldab BITCOM

orb #%00000010

stab BITCOM			;set BITCOM bit1

****************************************************************

B2

ldab BITCOM

andb #%00000100			;? is BITCOM bit2 == 0?

bne SETNX2

bra G2

SETNX2	ldaa 0,x			;this pin is complete keep storing 1's

ora #%00000100

staa 0,x

bra B3

G2

ldaa LASTBYT

anda #%00000100			;? is LASTBYT bit2 == 1?

beq SETNX2

ldaa 0,x

anda #%00000100			;? is bit2 == 0?

bne SETNX2

*	;this is a valid solenoid throw spot

*	;bit2 is already clear

ldab BITCOM

orb #%00000100

stab BITCOM			;set BITCOM bit2

****************************************************************

B3

ldab BITCOM

andb #%00001000			;? is BITCOM bit3 == 0?

bne SETNX3

bra G3

SETNX3	ldaa 0,x			;this pin is complete keep storing 1's

ora #%00001000

staa 0,x

bra B4

G3

ldaa LASTBYT

anda #%00001000			;? is LASTBYT bit3 == 1?

beq SETNX3

ldaa 0,x

anda #%00001000			;? is bit3 == 0?

bne SETNX3

*	;this is a valid solenoid throw spot

*	;bit2 is already clear

ldab BITCOM

orb #%00001000

stab BITCOM			;set BITCOM bit3

****************************************************************

B4

ldab BITCOM

andb #%00010000			;? is BITCOM bit4 == 0?

bne SETNX4

bra G4

SETNX4	ldaa 0,x			;this pin is complete keep storing 1's

ora #%00010000

staa 0,x

bra B5

G4

ldaa LASTBYT

anda #%00010000			;? is LASTBYT bit4 == 1?

beq SETNX4

ldaa 0,x

anda #%00010000			;? is bit4 == 0?

bne SETNX4

*	;this is a valid solenoid throw spot

*	;bit4 is already clear

ldab BITCOM

orb #%00010000

stab BITCOM			;set BITCOM bit4

****************************************************************

B5

ldab BITCOM

andb #%00100000			;? is BITCOM bit5 == 0?

bne SETNX5

bra G5

SETNX5	ldaa 0,x			;this pin is complete keep storing 1's

ora #%00100000

staa 0,x

bra B6

G5

ldaa LASTBYT

anda #%00100000			;? is LASTBYT bit5 == 1?

beq SETNX5

ldaa 0,x

anda #%00100000			;? is bit5 == 0?

bne SETNX5

*	;this is a valid solenoid throw spot

*	;bit4 is already clear

ldab BITCOM

orb #%00100000

stab BITCOM			;set BITCOM bit5

*****************************************************************

B6

ldab BITCOM

andb #%01000000			;? is BITCOM bit6 == 0?

bne SETNX6

bra G6

SETNX6	ldaa 0,x			;this pin is complete keep storing 1's

ora #%01000000

staa 0,x

bra B7

G6

ldaa LASTBYT

anda #%01000000			;? is LASTBYT bit6 == 1?

beq SETNX6

ldaa 0,x

anda #%01000000			;? is bit6 == 0?

bne SETNX6

*	;this is a valid solenoid throw spot

*	;bit6 is already clear

ldab BITCOM

orb #%01000000

stab BITCOM			;set BITCOM bit6

*****************************************************************

B7

ldab BITCOM

andb #%10000000			;? is BITCOM bit7 == 0?

bne SETNX7

bra G7

SETNX7	ldaa 0,x			;this pin is complete keep storing 1's

ora #%10000000

staa 0,x

bra B8

G7

ldaa LASTBYT

anda #%10000000			;? is LASTBYT bit7 == 1?

beq SETNX7

ldaa 0,x

anda #%10000000			;? is bit7 == 0?

bne SETNX7

*	;this is a valid solenoid throw spot

*	;bit6 is already clear

ldab BITCOM

orb #%10000000

stab BITCOM			;set BITCOM bit7

*******************************************************************

B8

LDAA TEMP

STAA LASTBYT		;before inx store current byte as lastbyte

INX

INC CTR3

LDAA CTR3

CMPA #34

BNE AGAIN		;increment x and loop through all 34 time steps

BRA DONE

AGAIN	JMP B0

DONE

RTS

***************************************************

** SUBROUTINE ADVance – send 256 rising edge square wave

***************************************************

ADV	PSHX

LDX #BASE

LDAA #$ff

STAA CTR

ldaa #0

STAA STOP

* ENABLE OC2 INTERRUPT

BSET TMSK1,X BIT6

* TURN ON INTERRUPT SYSTEM

CLI

WAIT41	LDAB STOP

CMPB #1

BNE WAIT41

* TURN OFF INTERRUPT SYSTEM

SEI

PULX

RTS

***********************************************************************

** SUBROUTINE Record - records the 8 pin offsets

***********************************************************************

RECORD

LDX #BASE

LDAA PORTA,X		; set plate to go up

ORA #%00010000

STAA PORTA,X

LDX #$8000

jsr KILLT

LDAB $2000		;index 32

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 31

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 30

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 29

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 28

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 27

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 26

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 25

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 24

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 23

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 22

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 21

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 20

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 19

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 18

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 17

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 16

stab 0,X

JSR ADV

jsr KILLT

LDAB $2000		;index 15

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 14

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 13

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 12

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 11

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 10

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 9

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 8

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 7

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 6

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 5

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 4

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 3

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 2

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 1

stab 0,X

JSR ADV

INX

jsr KILLT

LDAB $2000		;index 0

stab 0,X

JSR ADV

JSR ADV			;home at 34 again

JSR ADV			; advance 512 counts

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV			;40th ADV

JSR ADV			;confused here? extra 2 ADV's seems to help (still slightly off)

JSR ADV

LDX #BASE

LDAA PORTA,X		; set plate to go down

ANDA #%11101111

STAA PORTA,X

DELAY	DEC CTR			;kill 256 E-clocks before direction change

BNE DELAY

JSR ADV			; advance 512 counts

JSR ADV

JSR ADV

JSR ADV

JSR ADV

JSR ADV			;total=34

LDAA PORTA,X		; set plate to go up

ORA #%00010000

STAA PORTA,X

DELY	DEC CTR			;kill 256 E-clocks before direction change

BNE DELY

RTS

***************************************************

** SUBROUTINE WAIT - wait for bump switch press

***************************************************

WAIT	ldx #BASE

LDAA PORTA,X		;turn on waiting light

ANDA #%11011111

STAA PORTA,X

contin	LDAA PORTA,X		;poll untill continue switch is pushed

ANDA #%00000010

BNE contin

LDAA PORTA,X		;turn waiting light off

ORA #%00100000

STAA PORTA,X

RTS

***********************************************************************

** SUBROUTINE CLEAR - moves stepper up 36 x 256 counts, down 512 counts

***********************************************************************

CLEAR

LDX #BASE

LDAA PORTA,X		; set plate to go up

ORA #%00010000

STAA PORTA,X

ldaa #40		; call ADV 40 times, later move back down 6 ADV's

staa CTR2

MAS	JSR ADV

DEC CTR2

BNE MAS

LDAA PORTA,X		; set plate to go down

ANDA #%11101111

STAA PORTA,X

DELY2	DEC CTR			;kill 256 E-clocks before direction change

BNE DELY2

JSR ADV			; advance 512 counts

JSR ADV

JSR ADV			;at zero here

JSR ADV

JSR ADV			;down one more

JSR ADV

LDAA PORTA,X		; set plate to go up

ORA #%00010000

STAA PORTA,X

DELY3	DEC CTR			;kill 256 E-clocks before direction change

BNE DELY3

RTS

***********************************************************************

** SUBROUTINE MODE - decide mode (display 3d or digitize 3d)

*   Send back decision in accum. A

* toggle switch (PA0), continue switch (PA1), LED (PA5)

***********************************************************************

MODE	ldaa #$15

staa CTR4

modlp	dec CTR4		;blink 15 cycles

beq mlpend

LDX #BASE

LDAA PORTA,X		;turn waiting light off

ORA #%00100000

STAA PORTA,X

LDAA PORTA,X		;different flash speeds for 2 modes

ANDA #%00000001

BNE notdis

ldaa #$00		;kill time with enter button to escape

ldab #$20

eat	psha

LDAA PORTA,X		;if enter is pushed get out

ANDA #%00000010

BEQ mlp

pula

deca

bne eat

decb

bne eat

notdis	ldaa #$00		;kill time with enter button to escape

ldab #$20

eat2	psha

LDAA PORTA,X		;if enter is pushed get out

ANDA #%00000010

BEQ mlp

pula

deca

bne eat2

decb

bne eat2

LDAA PORTA,X		;turn on waiting light

ANDA #%11011111

STAA PORTA,X

ldaa #$00		;kill time with enter button to escape

ldab #$20

eat3	psha

LDAA PORTA,X		;if enter is pushed get out

ANDA #%00000010

BEQ mlp

pula

deca

bne eat3

decb

bne eat3

bra modlp		; loop back for $5 times

mlp	pula

mlpend	LDAA PORTA,X		;turn waiting light off

ORA #%00100000

STAA PORTA,X

ldaa PORTA,X

RTS

***************************************************

** INTERRUPT SERVICE ROUTINE

***************************************************

ORG	$A000

* INTERRUPT FROM OC2?

OC2_ISR	LDX   	#BASE

BRCLR	TFLG1,X BIT6 RT_OC2	;IGNORE ILLEGAL

*					;   INTERRUPT

DEC CTR

BNE CONT

INC STOP

* Disable OC2 interrupt

BCLR TMSK1,X BIT6

CONT

*--------------------------------------------------

* SERVICE OUTPUT COMPARE 2

*--------------------------------------------------

* CLEAR OC2 FLAG

BCLR	TFLG1,X INV6

* WAS LAST OUTPUT HIGH? (Bit7:6=OM2:OL2  Hi=11,Lo=10)

BRSET	TCTL1,X BIT6 LASTHI ;BRANCH ON YES

* PROGRAM NEXT OUTPUT TO BE HIGH

*   SET OC2 OUTPUT ACTION

BSET	TCTL1,X BIT6	;OM2:OL2=11 (bit 7 already 1)

*   SET HIGH OUTPUT COMPARE TIME

LDD	TOC2,X

ADDD	PWMLO

STD	TOC2,X

BRA	RT_OC2

* PROGRAM NEXT OUTPUT TO BE LOW

*   SET OC2 OUTPUT ACTION

LASTHI	BCLR	TCTL1,X BIT6	;OM2:OL2=10

*   SET LOW OUTPUT COMPARE TIME

LDD	TOC2,X

ADDD	PWMHI

STD	TOC2,X

* RETURN FROM INTERRUPT

RT_OC2	RTI

***********************************************************************

** SUBROUTINE KILLT - wait

***********************************************************************

KILLT	ldaa #$00

ldab #$ff

burn2	deca

bne burn2

decb

bne burn2

RTS

************************************************************************

*                       SUBROUTINE - InitSCI

* Description: This subroutine initializes the BAUD rate to 9600 and

*              sets up the SCI port for 1 start bit, 8 data bits and

*              1 stop bit.  It also enables the transmitter and receiver.

*              Effected registers are BAUD, SCCR1, and SCCR2.

* Input         : None.

* Output        : Initializes SCI.

* Destroys      : None.

* Calls         : None.

************************************************************************

*

InitSCI PSHA				; Save contents of A register

PSHX

LDX #BASE

LDAA #$30

STAA BAUD,X			; Set BAUD rate to 9600

LDAA #$00			; Set SCI Mode to 1 start bit,

STAA SCCR1,X			;     8 data bits, and 1 stop bit.

LDAA #%00001100			; Enable SCI Transmitter

STAA SCCR2,X			;     and Receiver

PULX

PULA			; Restore A register

RTS			; Return from subtoutine

************************************************************************

*                SUBROUTINE  -  InChar

* Description: Receives the typed character into register A.

* Input         : None

* Output        : Register A = input from SCI

* Destroys      : Contents of Register A

* Calls         : None.

************************************************************************

*

InChar

LDX #BASE

LDAA SCSR,X			 ; Check status reg.

ANDA #%00100000                  ;     (load it into A reg)

*                      			 ; Check if receive buffer full

BEQ InChar                       ; Wait until data present

LDAA SCDR,X	                 ; SCI data ==> A register

RTS                              ; Return from subroutine

************************************************************************

*                       SUBROUTINE -  OutChar

* Description: Outputs the character in register A to the screen after

*              checking if the Transmitter Data Register is Empty

* Input         : Data to be transmitted in register A.

* Output        : Transmit the data.

* Destroys      : None.

* Calls         : None.

************************************************************************

*

OutChar  PSHB				; Save contents of B register

ldy #BASE

Loop1   LDAB SCSR,Y			; Check status reg (load it into B reg)

ANDB #%1000000			; Check if transmit buffer is empty

BEQ Loop1			; Wait until empty

STAA SCDR,Y			; A register ==> SCI data

PULB				; Restore B register

RTS				; Return from subtoutine

*

************************************************************************

*                       SUBROUTINE -  OutStr

* Description: Outputs the string terminated by two EOS's. The starting location

*              of the string is pointed by X register. Calls the OutChar

*              subroutine to display a character on the screen and

*              exit once EOS has been reached.

* Input         : Starting location of the string to be transmitted

*               : (passed in X register)

* Output        : Prints the string.

* Destroys      : Contents of X register.

* Calls         : OutChar.

************************************************************************

*

OutStr:	PSHA

OutStr1:

LDAA	0,X			; Get a character (put in A register)

CMPA	#EOS			; Check if it's EOS

BEQ	onemore			; check for two EOS's in a row

BRA	notend

onemore	INX

LDAA 	0,X

CMPA	#EOS

BEQ	Done

DEX

LDAA 0,X

notend  BSR     OutChar			; Print the character by calling OutChar

INX

BRA     OutStr1

Done:

PULA

RTS				; Return from subtoutine

************************************************************************

*                       END OF PROGRAM

************************************************************************