figure 3

Sensor readings:

Below is a table of the analog readings obtained from sensors attached to the TJPRO 11 board.  I used a program that I wrote using ICC11 to determine the low value and the high value for each sensor.

|  | LowValue | High Value |
| --- | --- | --- |
| Left IR Detector | 80 | 127 |
| Right IR Detector | 85 | 127 |
| Back Bumper | 120 | 130 |
| Designed speed sensor | 19 | 30 |
| Designed can sensor | 56 | 65 |

Arm and actuation:

The arms only purpose is to pick up empty cans that are found.  It will consist of two servos and some plastic material.  It is a Big Grip Robot Kit, upon finding an empty can using the special sensor, the arm will automatically pick it up using the two servos.  The first servo is a grip servo used to grip the empty can, the second servo is a lift servo used to lift the empty can.  The arm was purchased at Lynxmotion, INC for $75.  For future references, I may make a trash can connected to the platform whereas the arm can drop the empty can.

Actuation:

# Actuation for Stoker was accomplished by using two hacked Hi-Tec HS-422 servos. The process of hacking the servos is straightforward. The information was provided by Randy in the Intelligent Machine Design Laboratory.  Full details of the servo hack can be found on Mekatronix.com the hacked servos are powered by the motor driver on the TJPRO11. Software to control these servos has already been written. To provide adequate power a six-pack of batteries supplied the required voltage. Note: The battery pack must be attached to the TJPRO 11 board.

# Behaviors:

# 1- Object avoidance

# 2- Searching for can

# 3- Determine if empty or full can

# 4- Picking can up

The behaviors of Stoker are simplistic because its only purpose is to wander a room looking for empty cans and pick them up. 1) I have programmed basic obstacle avoidance (See Appendix A) behavior into Stoker.  If it comes within a foot of an object or person (At this time Stoker cannot determine the difference), it will make a random turn and go to another location.  2) Stoker will arbitrary search for an empty or full can.  3) When it finds a can it will determine if it is empty or full using a designed sensor.  4) If it is empty it will pick the can up using an autonomous arm controlled by two servos.  If it is full Stoker will back up two feet and then go forward while making a turn.

Conclusion:

This written report can be changed due to the complexity of the task and better performance of the Stoker 2000.   The project taught me how to improvise and draw on the information that has been taught to me in the Electrical and Computer Engineering program. Up to this point, I have used the skills I have gained in microprocessors and digital design to perform my project.  However, many other engineering classes have been used as sources too.   I built a robot that was simple, but still accomplished the task of finding a can, determining if it is empty or full, and pick it up using an arm if it is empty.  This was my first robot and I am happy with what I accomplished.  One limiting factor in Stoker is that it cannot operate in a small area. This factor cannot be changed without tossing out the entire platform and using a more nimble platform. Also, in the future I would like to incorporate a trash can that the empty cans will be dropped in after been picked up.  Overall, this class has been very good; I have gained a lot of knowledge that will help me in the future.

Appendix A:

Stoker.c

#include <tjpbase.h>

#include <stdio.h>

#include <hc11.h>

#include <math.h>

#define FRONT	(BUMPER > 19) && (BUMPER < 30)

#define BOTH	(BUMPER > 56) && (BUMPER < 65)

#define BACK	(BUMPER > 120) && (BUMPER < 130)

#define RFIR          analog(2)

#define LFIR          analog(3)

#define AVOID_THRESHOLD 107

void smooth(void);

void forward_avoid(void);

void picupcantest_avoid(void);

void picupcan_avoid(void);

void turn(void);

int speedr, speedl, servo_index, irdr, irdl, k, j, hold, mikael3, hold1, hold2, hold3, hold5,

v, i, s, w, l, m, p, x, mikael, mikael1, mikael2;

unsigned int PW;

int LASTL = 0 , LASTR = 0 ;

void main(void)

{

init_analog();

init_motortjp();

init_clocktjp();

init_servotjp();

IRE_ON;

while (1)

{

forward_avoid();

}

}

//object avoidance forward

void forward_avoid(void)

{

irdr = RFIR;

irdl = LFIR;

if (irdl > AVOID_THRESHOLD)

speedr = MAX_SPEED;

else

speedr = -MAX_SPEED;

if (irdr > AVOID_THRESHOLD)

speedl = MAX_SPEED;

else

speedl = -MAX_SPEED;

smooth();

if (FRONT)

{

speedl = -MAX_SPEED;

speedr = -MAX_SPEED;

smooth();

//   printf("Front %d", BUMPER);

picupcantest_avoid();

}

}

//turn if going backwards and wants to go forward to avoid can

void turn(void)

{

unsigned rand;

rand = TCNT;

if (rand & 0x0001)

/*turn left*/

while(1)

{

hold5 = 75 ;

for ( v = 0 ; v < hold5; v++)

{

speedr = -MAX_SPEED;

speedl = MAX_SPEED;

smooth();

}

goto help5;

}

else

/*turn right*/

while(1)

{

hold5 = 75 ;

for ( w = 0 ; w < hold5; w++)

{

speedr = MAX_SPEED;

speedl = -MAX_SPEED;

smooth();

}

goto help5;

}

help5:;

}

//so there is no jumps in speeds incrementally speeding up and down

void smooth(void)

{

int L = 0, R = 0, LX = 0, RX = 0;

LX = fabs((speedl - LASTL) / 10);

RX = fabs((speedr - LASTR) / 10);

if (speedl > LASTL)

{

while (L < LX)

{

speedl = (LASTL + 10);

motorp(LEFT_MOTOR, speedl);

++L;

}

}

else if (speedl < LASTL)

{

while (L < LX)

{

speedl = (LASTL - 10);

motorp(LEFT_MOTOR, speedl);

++L;

}

}

if (speedr > LASTR)

{

while (R < RX)

{

speedr = (LASTR + 10);

motorp(RIGHT_MOTOR, speedr);

++R;

}

}

else if (speedr < LASTR)

{

while (R < RX)

{

speedr = (LASTR - 10);

motorp(RIGHT_MOTOR, speedr);

++R;

}

}

LASTL = speedl;

LASTR = speedr;

}

void picupcantest_avoid(void)

{

speedl = -MAX_SPEED;

speedr = -MAX_SPEED;

smooth();

hold = 400 ;

for ( m = 0 ; m < hold; m++)

{

speedl = -MAX_SPEED;

speedr = -MAX_SPEED;

smooth();

if (BOTH)

{

//   						printf("Both %d", BUMPER);

hold1 = 700 ;

for ( j = 0 ; j < hold1; j++)

{

speedr = MAX_SPEED;

speedl = MAX_SPEED;

smooth();

if(BACK)

{

speedl = -MAX_SPEED;

speedr = -MAX_SPEED;

smooth();

turn();

wait(35);

goto help;

}

}

turn();

wait(35);

goto help;

}

}

picupcan_avoid();

help:;

}

//controlls the arm to pick can up and let it down

void picupcan_avoid(void)

{

while(1)

{

mikael=300 ;

for( x = 0 ; x < mikael; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

speedr = 0 ;

speedl = 0 ;

smooth();

servo_index = 0 ;

PW = - 4000 ;

servo(servo_index, PW);

mikael1= 300 ;

for( x = 0 ; x < mikael1; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

servo_index = 0 ;

PW = 0 ;

servo(servo_index, PW);

mikael2=300 ;

for ( x = 0 ; x < mikael2; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

servo_index = 1 ;

PW = 1000 ;

servo(servo_index, PW);

mikael2=300 ;

for ( x = 0 ; x < mikael2; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

mikael3=300 ;

for ( x = 0 ; x < mikael3; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

servo_index = 0 ;

PW = 4000 ;

servo(servo_index, PW);

mikael3=300 ;

for ( x = 0 ; x < mikael3; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

goto help23;

}

help23:;

while(1)

{

hold2=4000 ;

for ( k = 0 ; k < hold2; k++)

{

//   		printf("runka2 %d", BUMPER);

speedr = -MAX_SPEED;

speedl = MAX_SPEED;

smooth();

}

goto help24;

}

help24:;

while(1)

{

speedr = 0 ;

speedl = 0 ;

smooth();

mikael=300 ;

for( x = 0 ; x < mikael; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

speedr = 0 ;

speedl = 0 ;

smooth();

servo_index = 0 ;

PW = - 4000 ;

servo(servo_index, PW);

mikael1= 300 ;

for( x = 0 ; x < mikael1; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

servo_index = 0 ;

PW = 0 ;

servo(servo_index, PW);

mikael2=300 ;

for ( x = 0 ; x < mikael2; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

servo_index = 1 ;

PW = 5000 ;

servo(servo_index, PW);

mikael2=300 ;

for ( x = 0 ; x < mikael2; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

servo_index = 1 ;

PW = 0 ;

servo(servo_index, PW);

mikael2=300 ;

for ( x = 0 ; x < mikael2; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

servo_index = 0 ;

PW =4000 ;

servo(servo_index, PW);

mikael1= 300 ;

for( x = 0 ; x < mikael1; x++)

{

speedr = 0 ;

speedl = 0 ;

smooth();

printf("Front %d", BUMPER);

}

servo_index = 0 ;

PW = 0 ;

servo(servo_index, PW);

goto help55;

}

//back up and go forward

help55:;

hold3 = 700 ;

for ( j = 0 ; j < hold3; j++)

{

speedr = MAX_SPEED;

speedl = MAX_SPEED;

smooth();

if(BACK)

{

speedl = -MAX_SPEED;

speedr = -MAX_SPEED;

smooth();

turn();

wait(35);

goto help30;

}

}

turn();

wait(35);

help30:;

}