University of Florida

Department of Electrical and Computer Engineering

EEL 5666

Intelligent Machines Design Laboratory

Fall 2000

Pinky: Final Report

Shawn Gu

Email: gux@ecel.ufl.edu

December 06, 2000

TAs: Rand Chandler, Scott Nortman

Instructor: Dr. A. A. Arroyo

Table of Contents

Abstract………………………………………………………………………Page 3

Executive Summary………………………………………………………….Page 4-5

Introduction………………………………………………………………….Page 6

Integrated System……………………………………………………………Page 7-8

Mobile Platform……………………………………………………………..Page 9

Actuation………………………………………………………………….…Page 10

Sensors: Experimental Layout and Results…………………………………Page 11-17

Behaviors……………………………………………………………………Page 18

Conclusion…………………………………………………………………..Page 19

Appendix A: Pinky's ICC codes………………………………………….…Page 20-68

Appendix B: Pinky simulator's Java codes…………………………………Page 69-111

Appendix C: Micro Mouse Contest Rules and Maze drawing……………..Page 112-114

Abstract

Pinky is a "Micro Mouse" robot that travels through a maze, maps its course, and finds the shortest path possible to the destination.  Autonomous behaviors such as wall collision avoidance, wall gap detection, high precision steering control, and maze-solving algorithm must be implemented, to achieve the objectives of a Micro Mouse robot.

Executive Summary

Pinky's chassis consists of the base portion of the TJ chassis and Knex toy components.  Its only processor is the MTJPRO11 microcontroller board, designed by Mekatonix, Inc.  The MTJPRO11 board contains the Motorola 68HC11 microprocessor and 32K of SRAM, as well as eight analog input ports, four digital output ports, three digital input ports, and one digital input/output port.  It also provides access to the SPI communication system of the HC11 processor through its Port D pins.  Two Hitec HS-422 servos were used for movement.  Pinky is powered with six NiCad rechargeable batteries connected in series.

Wall collision and robot steering control are accomplished by four photo-microsensors mounted on the two wings extended from Pinky's chassis.  Wall gap detection and turning adjustments are done using three IR sensors mounted on the front, left, and right sections of the robot.  The cell counting behavior, which the maze-solving algorithm requires, is done by the two CdS cells mounted at the bottom of the chassis.  One microswitch is used as a general purpose push button to start off each of the three phases of the robot's maze-solving behavior.

Pinky's program implements four major behaviors:

1). Keep the robot moving forward straight down the center of the maze 'corridor' without running into the walls (this is not as easy as it sounds!).

2). Perform near 90 degree turns, left and right, when a wall gap or an intersection is detected, and a decision to turn is made.  The turn must be executed so that the robot is lined up to move off in the desired new direction without running into walls.

3). Perform near 180 degree turns, when a dead-end is detected, and when the center of the maze is reached.  An alternative is to reverse the motors, but I found the former performing better.

4). Solve the maze.  The algorithm I used is an off-shoot of the Bellman algorithm.  Simply put, the program marks each cell of the maze as an element in an array, and update the values of the array element with the distance of that cell to the starting cell of the maze, as the robot is traveling through the maze.  Any path from the starting cell to the center of the maze can be remembered by tracing through the array elements' values in either ascending or descending order.  If enough time is spent on the maze mapping, a best possible solution can be found eventually.

Introduction

The idea of Pinky comes from the so-called "Micro Mouse" contests, where small robots race to find the center of a maze.  These contests are sponsored across the world and attract large number of contestants, due to the high level of challenge involved in building a successful and competitive Micro Mouse.  A Micro Mouse is a clever little gadget with a silicon brain that finds its way through an arbitrary maze.  It competes against other members of its species, racing as fast as it can.  In a Micro Mouse contest the contestants design and build a small, self-contained, autonomous robot to negotiate and solve an unknown maze in the shortest time.  Pinky was designed and built with the purpose of one day competing in a Micro Mouse contest.

Integrated System

Pinky is designed as a Micro Mouse robot, therefore the main task is in program and sensor development, rather than hardware esthetics.  The control center of Pinky is the MTJPRO11 micro-controller, which contains a Motorola 68HC11 microprocessor and 32K of SRAM, plus various input/output ports.  Four types and a total of ten sensors were used on Pinky.  Three IRs mounted on the front, left, and right sections of the chassis are connected to analog ports PE1, PE2, and PE3.  They are used to collect wall information inside the maze, and aid the robot's turning behaviors.  Two CdS cells are connected to analog ports PE4 and PE7, and they are used mainly for cell counting.  One bumper switch is connected to PE0, and used only as a general-purpose push button.  Four photo-micro-sensors are used for wall tracking and therefore keeping the mouse in the center of the maze corridors.  Three of them are connected to digital input ports PD0, PD1, and PD2.  The last one is connected to analog port PE6.  Due to CdS cell's strong sensitivity to light interference, they are mounted in a well-shielded position under the chassis, and a green visible light LED is used to provide the light source.  The LED's power and ground are connected to the power and ground pins of PA4.  Another green visible light LED is mounted on top of the platform and used as an indicator of certain accomplished actions.  Its power and ground are driven by PA6.

These sensors are used to feed information into the maze-solving algorithm.  The IR sensors provide the wall information such as wall gap and dead-end, and help Pinky making 90 and 180 degree turns.  The CdS cells keep track of the robot's position in the maze.  And the reflective switches (photo-micro-sensors) keep the mouse running straight.

The algorithm I used to very similar to the Bellman algorithm, which is basically a flooding method that updates maze information in the robot's brain as its moving along in the maze.  A generalized flowchart of the algorithm is shown below in Figure 1.  A detailed program flowchart would difficult to present in the paper since it more than two pages long.

Figure 1- Block Diagram for the Bellman Flooding Algorithm

Mobile Platform

Pinky's platform only has a few basic requirements.  The most important one is the platform's size constraints.  The maze corridors are only 180mm wide, and there must be at least 30mm left on each side for the robot to have enough time to react and adjust itself when it strays from the center of the corridor.  The base portion of the TJ platform is adequate, after a few modifications.  First, the servos are mounted from the inside of the enclosing bracket of the TJ platform, and saving approximately 1cm on each side.  Second, the standard TJ wheels are discarded and replaced by two model airplane wheels I found at a local hobby shop.  The new wheels have longer diameters and are much thinner comparing to the TJ wheels.  This replacement saves another half centimeter on each side.  The wheels are then trimmed and cut to save an extra half a centimeter each.  A good portion of the front and back sections of the TJ base was also cut and trimmed to give a total base length of around 13cm.  Since the walls are 9cm high, there is no restriction on the size of the platform above that height.  Therefore all circuitry are built on the top platform.  Two wings are extended from the main body to cover over the adjacent maze walls.  Each wing has two reflective switches mounted facing downward to the top of the walls.  The length of the wing span has no restrictions, as long as the sensors can be directly on top of the walls.  More than a dozen pieces of Knex toys are used to build the top portion of the platform, including the wings, and mounting cases for control switches and the micro-controller.  Overall, the platform is built to limit the body size below the wall top as much as possible, and make the top portion as tall as needed to mount all the necessary circuitry.

Actuation

Pinky uses the basic locomotive scheme from the TJ robots to accomplish simple movements.  Two HiTec HS-422 servos to provide locomotion and an enhanced plastic caster (since Pinky has bigger wheels than the TJ) to provide balance.  The servos have been hacked to act like bi-directional motors by disconnecting the internal potentiometer from the gear mechanism.  One side effect of using the hacked-servo design was each robot had to be individually calibrated.  This was done by first setting the potentiometers (by hand) to a position such that the servo ceased moving.  Near the end of the project it was determined that the left servo was not working properly, as it did not seem to understand anything other than 0 or 100 percent of the PWM.  It was proven not to be the Port A pins' fault, since I observed the PWM generated from those pins as the motor speeds were swept through using a test program.  Still, 0 and 100 percent were all Pinky needed to go forward, stop, and make 90 and 180 degree turns.  The right motor had to be given 150 percent of the PWM in order to keep the robot going relatively straight without any adjustments.

Sensors

All four types of sensors used on this robot are used to detect certain features of the maze, therefore giving the micro mouse enough information to control its motion and direction.  The micro-switch serves only as a push button.  Originally bumper switches were considered to detect wall collision.  When it was determined that wall collision can be eliminated 90 to 95% of the time, I decided not to implement the bumpers.  The hacked analog IR sensors are used to provide information about what is ahead of and next to the robot as its moving around in the maze.  One IR sensor on each section of the chassis is enough to know if there is a wall in front of or next to the robot approximate 5cm away.  The IR emitters are connected in series with 1Kohm resistors to limit the amount of IR transmitted, and therefore shortening the range of the IR detector.  The CdS cells' purpose is to detect the white strips we taped on the maze floor.  Each strip marks edge of a maze cell (180mmx180mm).  When the strip is detected, then the robot knows that it has traveled to another cell, therefore able to keep track of its position in the maze.  I thought about using shaft encoders to do this task, but I was warned about wheel slippage.  Considering the high accuracy required by the maze algorithm, I decided not to use shaft encoders.  Two CdS cells are mounted at the bottom left edge of the chassis in a line perpendicular to the white strip on the floor, this gives a much better chance for the thin strip to be captured by the sensors.  A green visible light LED is used to provide the light source for the CdS cells.  The orientation of the sensors are shown in the following figure:

Figure 2-Sensor Layout

The reflective switches shown in the diagram above are used to accomplish wall following and wall gap detection.  Wall following is done by adjusting the motor speeds as the sensor readings change, this allows the robot to remain relatively in the middle of the passageway in the maze.  When the mouse is positioned in the middle, the left and right most reflective switches give readings of logic TRUE.  As the robot slides to one way, the left for instance, the left most sensor would go FALSE as soon as the light bean emitted by the sensor "falls off" the top of the wall.  If the motors aren't adjusted quick enough, the second left most sensor reading would soon become TRUE as it "gets on" the wall top.  As long as the motors do not run too fast, there is enough time for the process to react to these changes in the sensor readings and adjust the motors accordingly.  If there were at least three or four of these sensors on each wing, then I would not worry about motor speeds at all, since the sensors never lose track of the walls.  A simple diagram is shown below to illustrate this concept:

Figure 3-Reflective Switchs

| Sensor Type | Purpose | Number used | Location |
| --- | --- | --- | --- |
| Microswitch | Collision detection | 3 | 2 Front, 1 back |
| IR | Wall avoidance, clear path detection, communication | 3 | 2 Front-forward, 1back |
| Photoresistor | Search target recognition | 4 | Front-down |
| Reflective switch | Wall gap detection, motion control | 4 | 2 Leftwing-down, 2 rightwing-down |

Table 1-Sensor Information

Experiments and Results

Experiment 1: IR reception vs. distance between robot and maze wall.

| Distance(cm) | LEFT | RIGHT |
| --- | --- | --- |
| 1 | 118 | 120 |
| 2 | 118 | 119 |
| 3 | 117 | 119 |
| 4 | 116 | 118 |
| 5 | 116 | 117 |
| 6 | 114 | 112 |
| 7 | 112 | 110 |
| 8 | 109 | 106 |
| 9 | 106 | 103 |
| 10 | 103 | 101 |
| 11 | 101 | 100 |
| 12 | 101 | 98 |
| 13 | 100 | 97 |
| 14 | 98 | 97 |
| 15 | 98 | 96 |
| 16 | 95 | 96 |
| 18 | 92 | 93 |
| 20 | 92 | 92 |
| 22 | 91 | 92 |
| 24 | 90 | 91 |
| 26 | 90 | 90 |

Table 2-IR Data

These sensor readings show that when the distance increases to 10cm there is a significant drop in the digital values comparing to the maximum digital value.  Since each cell in the maze is 18cm x 18cm, 10cm is a good distance to detect if there is a wall in front of the mouse in every cell.  Therefore, 103 and 101 are used as threshold values for the left and right IRs respectively.

Figure 4-IR graph

Experiment 2: CdS cells readings vs. Color and Voltage level

These CdS cells are needed to capture the white strips taped on the maze floor.  The first major concern was that the CdS cells can not react fast enough as the mouse is running across the white strips.  After a simple test in which I manually moved the mouse across a white tape at a speed much faster than the maximum servo speed, it was determined this will not be a problem.  The exact speed was not recorded, I just know it was much faster than the speed in which the mouse will be running.

The other concern was the CdS readings would vary too much as the battery voltage drops.  Light intensity was not a concern because the sensors are well shielded and the floor was painted with non-gloss black paint.

Figure 5-CdS Graph

It can be seen that the voltage drop from 7.2V to 6V did not affect the readings much.  In fact I doubt the readings will be affected if the voltage does not drop below 5V.  There is a significant difference between the readings of black and white, and the value 190 was chosen as the threshold for all of the CdS cells used for Pinky.

Experiment 3: Reflective switches readings vs. Distance

|  | Pinky |  |  |  | Brain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance | RS0 | RS1 | RS2 | RS3 | RS0 | RS1 | RS2 | RS3 |
| 2-5 mm | 246 | 246 | 247 | 246 | 246 | 247 | 245 | 248 |
| 10 mm | 245 | 244 | 246 | 245 | 245 | 246 | 244 | 246 |
| 15 mm | 243 | 243 | 244 | 243 | 243 | 244 | 244 | 244 |
| 20 mm | 200 | 205 | 198 | 202 | 198 | 205 | 202 | 201 |
| 100 mm | 14 | 16 | 12 | 14 | 15 | 16 | 12 | 14 |

Table 3--Reflective Switch Data

These sensors only give digital outputs, and it is digital output that I really want.  Since the sensors are mounted on the wing and the distance between them and the walls are almost always constant, the only tests I needed to run was to find out if the sensors return TRUE readings at that distance.  The distance is approximate 1cm, and the wall tops are painted with semi-gloss white paint.  These sensors are extremely resistant to light interference.  They are connected to Port A on the TJPRO board, and I confirmed that variations in surrounding light intensity do not cause any incorrect readings.

Behaviors

Pinky's program implements four major behaviors:

1). Keep the robot moving forward straight down the center of the maze 'corridor' without running into the walls.  This behavior's implementation is described in the last section when the reflective switches were explained.

2). Perform near 90 degree turns, left and right, when a wall gap or an intersection is detected, and a decision to turn is made.  The turn must be executed so that the robot is lined up to move off in the desired new direction without running into walls.  This is implemented by using the front IR.  The motors are made to turn either left or right, and they are stopped only when the front IR passes a certain threshold value located through testing.

3). Perform near 180 degree turns, when a dead-end is detected, and when the center of the maze is reached.  An alternative is to reverse the motors, but I found the former performing better.  This is accomplished in the same manner as behavior number 2.

4). Solve the maze.  The algorithm I used is an off-shoot of the Bellman algorithm.  Simply put, the program marks each cell of the maze as an element in an array, and update the values of the array element with the distance of that cell to the starting cell of the maze, as the robot is traveling through the maze.  Any path from the starting cell to the center of the maze can be remembered by tracing through the array elements' values in either ascending or descending order.  If enough time is spent on the maze mapping, a best possible solution can be found eventually.  See Appendix A and B for the maze-solving codes.

Conclusion

The cell counting process had to be implemented with white tape on the floor and CdS cells, due to the inaccuracy introduced by the shaft encoder.  The cell counting required a relatively high degree of precision (in centimeters), and we had no other economical solutions to the problems other than white tapes and CdS cells.  Other than that, Pinky and the Brain are quite capable of competing in a real Micro Mouse contest.

Appendix A (ICC code for Pinky)

/*****************************************************************

*			Pinky 2000								*

*File Name: Pinky2000.c									*

*Programmer: Xuan Gu, with contribution from Edison Rodriguez	*

*Date: November, 23 2000									*

*Description: Bellman style with some Gux flavors added maze	*

*		    solving program for Micromouse robots Pinky and the*

*		    Brain.									*

*****************************************************************/

#include "pink2000.h"

/****************************** Main Program **********************************/

void main(void)

{

int i;

init_analog();		//three functions must be called first

init_motortjp();

init_clocktjp();

//	while(BUMPER < 120);

//	report_path();

//	wait(2000);

*(unsigned char *) 0x7000=0x0B; // Turn on 3 IR emitters, 0000 1011

// Initialize the array

maze[0] = 0;

for (i=1; i<=143; i++)

maze[i] = UNEXPLORED;

maze[65] = CENTER;

maze[66] = CENTER;

maze[77] = CENTER;

maze[78] = CENTER;

while(BUMPER < 120);	// wait for bumper to be pressed to start

wait(2000);			// 2 seconds to run away from the robot!!!

while (FINISHED == FALSE)

{

read_walls();		//get values for WALL_F, WALL_L, WALL_R.  WALL_B isn't really necessary...

if (direction == NORTH)

{

LOOK_F = LOOK_N;

LOOK_B = LOOK_S;

LOOK_L = LOOK_W;

LOOK_R = LOOK_E;

}

else if (direction == SOUTH)

{

LOOK_F = LOOK_S;

LOOK_B = LOOK_N;

LOOK_L = LOOK_E;

LOOK_R = LOOK_W;

}

else if (direction == WEST)

{

LOOK_F = LOOK_W;

LOOK_B = LOOK_E;

LOOK_L = LOOK_S;

LOOK_R = LOOK_N;

}

else if (direction == EAST)

{

LOOK_F = LOOK_E;

LOOK_B = LOOK_W;

LOOK_L = LOOK_N;

LOOK_R = LOOK_S;

}

if (FOUNDIT == FALSE)

{

//if doesn't see a clear entrance into the center of the maze

if (((cell_ptr+LOOK_F > 143 || cell_ptr+LOOK_F < 0) || (maze[cell_ptr+LOOK_F] != CENTER) || (maze[cell_ptr+LOOK_F] == CENTER && WALL_F != 0))

&&

((cell_ptr+LOOK_B > 143 || cell_ptr+LOOK_B < 0) || (maze[cell_ptr+LOOK_B] != CENTER) || (maze[cell_ptr+LOOK_B] == CENTER && WALL_B != 0))

&&

((cell_ptr+LOOK_L > 143 || cell_ptr+LOOK_L < 0) || (maze[cell_ptr+LOOK_L] != CENTER) || (maze[cell_ptr+LOOK_L] == CENTER && WALL_L != 0))

&&

((cell_ptr+LOOK_R > 143 || cell_ptr+LOOK_R < 0) || (maze[cell_ptr+LOOK_R] != CENTER) || (maze[cell_ptr+LOOK_R] == CENTER && WALL_R != 0)))

{

if (WALL_F == 0) // if front wall is open

{

if (maze[cell_ptr+LOOK_F] == UNEXPLORED)  //and if forward cell is unexplored,

{

move_forward();

cell_ptr = cell_ptr + LOOK_F;		// update cell pointer (position in maze)

maze[cell_ptr] = maze[cell_ptr+LOOK_B]+1; // assign Bellman number

}

else //if forward cell is explored, and not the center

{

if (WALL_L == 0 && maze[cell_ptr+LOOK_L] == UNEXPLORED) //and if left is open and unexplored

{									//explore left cell

turn_left();

move_forward();

cell_ptr = cell_ptr + LOOK_L;

change_dir_left();

maze[cell_ptr] = maze[cell_ptr+LOOK_R]+1;

}

else if (WALL_R == 0 && maze[cell_ptr+LOOK_R] == UNEXPLORED)	//else if right is open and unexplored

{									//explore right cell

turn_right();

move_forward();

cell_ptr = cell_ptr + LOOK_R;

change_dir_right();

maze[cell_ptr] = maze[cell_ptr+LOOK_L]+1;

}

else

{//F is open but explored,  L and R are either closed or explored, simply go forward

move_forward();

cell_ptr = cell_ptr + LOOK_F;

}

}

}

else if (WALL_L == 0) // if front is closed, and left is open

{

if (maze[cell_ptr+LOOK_L] == UNEXPLORED) //and if left cell is unexplored

{

turn_left();

move_forward();

cell_ptr = cell_ptr + LOOK_L;

change_dir_left();

maze[cell_ptr] = maze[cell_ptr+LOOK_R]+1;

}

else //if left cell is already explored

{

if (WALL_R == 0 && maze[cell_ptr+LOOK_R] == UNEXPLORED) // and if right is open and unexplored

{									//explore right cell

turn_right();

move_forward();

cell_ptr = cell_ptr + LOOK_R;

change_dir_right();

maze[cell_ptr] = maze[cell_ptr+LOOK_L]+1;

}

else if (WALL_R != 0)  //F is closed, L is open but explored, R is closed, simply go left

{

turn_left();

move_forward();

cell_ptr = cell_ptr + LOOK_L;

change_dir_left();

}

}

}

else if (WALL_R == 0)	// if F and L are closed, and right is open

{

turn_right();

move_forward();

cell_ptr = cell_ptr + LOOK_R;

change_dir_right();

if(maze[cell_ptr+LOOK_R] == UNEXPLORED) // if right is unexplored

maze[cell_ptr] = maze[cell_ptr+LOOK_L]+1;

}

else

{	// reached a deadend, reverse course

turn_around();

move_forward();

cell_ptr = cell_ptr + LOOK_B;

change_dir_reverse();

}

}

else //if at the only entrance to the center, just move in

{  //if center is in the front adjacent cell with a clear path, go inside

if(maze[cell_ptr+LOOK_F] == CENTER && WALL_F == 0)

{

FOUNDIT = TRUE;

cell_ptr = cell_ptr + LOOK_F;

move_forward();

}

//if center is in the left adjacent cell with a clear path, go inside

else if(maze[cell_ptr+LOOK_L] == CENTER && WALL_L == 0)

{

FOUNDIT = TRUE;

cell_ptr = cell_ptr + LOOK_L;

turn_left();

move_forward();

change_dir_left();

}

//if center is in the right adjacent cell with a clear path, go inside

else if(maze[cell_ptr+LOOK_R] == CENTER && WALL_R == 0)

{

FOUNDIT = TRUE;

cell_ptr = cell_ptr + LOOK_R;

turn_right();

move_forward();

change_dir_right();

}

}

}

/******************start of the traceback process, from center to the maze entrance******************/

else if (FOUNDIT == TRUE && TRACEDONE == FALSE)

{

if(maze[cell_ptr] == CENTER)	//if at center,

{

temp = maze[cell_ptr+LOOK_B];  //transfer the bellnumber of last cell travelled to temp

cell_ptr = cell_ptr+LOOK_B;	 //point to the last cell travelled

turn_around_center();		 //turn around

maze[cell_ptr] = maze[cell_ptr+LOOK_F]+1;	//add 1 to the center's bellnumber, assign to that cell

change_dir_reverse();

flash_led();

while(BUMPER < 120);	// wait for bumper to be pressed to start traceback

wait(2000);			// 2 seconds to run away from the robot!!!

move_forward();			 //move forward

}

else

{

if(((cell_ptr+LOOK_F <= 143 && cell_ptr+LOOK_F >= 0) && //if the forward adjacent cell ptr is within range

(maze[cell_ptr+LOOK_F] == temp-1 && WALL_F == 0))) //and its bellnumber is ONE less the current cell's

{										//bellnumber, and the front wall of the current cell is open:

cell_ptr = cell_ptr+LOOK_F;				//Point to the forward adjacent cell

temp = maze[cell_ptr];					// temp holds the current cell's bellnumber

if (maze[cell_ptr] == 0)					// if the forward adjacent cell is the start of the maze

TRACEDONE = TRUE;					// then set TRACEDONE flag

maze[cell_ptr] = maze[cell_ptr+LOOK_B]+1;	// add 1 to current cell's bellnumber and assign to the front cell

move_forward();						// move to the forward cell

}

else if(((cell_ptr+LOOK_L <= 143 && cell_ptr+LOOK_L >= 0) && //if the left adjacent cell......

(maze[cell_ptr+LOOK_L] == temp-1 && WALL_L == 0)))

{

cell_ptr = cell_ptr+LOOK_L;

temp = maze[cell_ptr];

if (maze[cell_ptr] == 0)

TRACEDONE = TRUE;

maze[cell_ptr] = maze[cell_ptr+LOOK_R]+1;

turn_left();

move_forward();

change_dir_left();

}

else if(((cell_ptr+LOOK_R <= 143 && cell_ptr+LOOK_R >= 0) && // if the right adjacent cell......

(maze[cell_ptr+LOOK_R] == temp-1 && WALL_R == 0)))

{

cell_ptr = cell_ptr+LOOK_R;

temp = maze[cell_ptr];

if (maze[cell_ptr] == 0)

TRACEDONE = TRUE;

maze[cell_ptr] = maze[cell_ptr+LOOK_L]+1;

turn_right();

move_forward();

change_dir_right();

}

}

}

/***********************get from the start to finish as quick as possible***********************/

//Follows the path mapped by the TRACEBACK process and go from the start to center of maze.

//Similar process as the TRACEBACK routine, except start from the entrance and end at the center,

//also, search for the adjacent cell that has a bellnumber ONE less than the current cell's, and

//move to that cell.  Also, this process does NOT update any bellnumbers.

else if (TRACEDONE == TRUE && FINISHED == FALSE)

{

if(((cell_ptr+LOOK_B <= 143 && cell_ptr+LOOK_B >= 0) &&

(maze[cell_ptr+LOOK_B] == maze[cell_ptr]-1 && WALL_B == 0)))

{

cell_ptr = cell_ptr+LOOK_B;

if (maze[cell_ptr] == CENTER)

{

FINISHED = TRUE;

PATH_END = path_ptr - 1 ;

}

turn_around();

change_dir_reverse();

flash_led();

while(BUMPER < 120);	// wait for bumper to be pressed to start the race

wait(2000);			// 2 seconds to run away from the robot!!!

move_forward();

}

else if(((cell_ptr+LOOK_F <= 143 && cell_ptr+LOOK_F >= 0) &&

(maze[cell_ptr+LOOK_F] == maze[cell_ptr]-1 && WALL_F == 0)))

{

cell_ptr = cell_ptr+LOOK_F;

if (maze[cell_ptr] == CENTER)

{

FINISHED = TRUE;

PATH_END = path_ptr - 1 ;

}

move_forward();

}

else if(((cell_ptr+LOOK_L <= 143 && cell_ptr+LOOK_L >= 0) &&

(maze[cell_ptr+LOOK_L] == maze[cell_ptr]-1 && WALL_L == 0)))

{

cell_ptr = cell_ptr+LOOK_L;

if (maze[cell_ptr] == CENTER)

{

FINISHED = TRUE;

PATH_END = path_ptr - 1 ;

}

turn_left();

move_forward();

change_dir_left();

}

else if(((cell_ptr+LOOK_R <= 143 && cell_ptr+LOOK_R >= 0) &&

(maze[cell_ptr+LOOK_R] == maze[cell_ptr]-1 && WALL_R == 0)))

{

cell_ptr = cell_ptr+LOOK_R;

if (maze[cell_ptr] == CENTER)

{

FINISHED = TRUE;

PATH_END = path_ptr - 1 ;

}

turn_right();

move_forward();

change_dir_right();

}

}

}

flash_led();

record_path();			// record the path

}

/******************************* End of Main Program ****************************/

/****************************** read_walls **************************************/

void read_walls(void)

{

if (LEFT_IR < LEFT_IR_THRESHOLD)

WALL_L = 0;

else

WALL_L = 1;

if (RIGHT_IR < RIGHT_IR_THRESHOLD)

WALL_R = 0;

else

WALL_R = 1;

if (FRONT_IR < FRONT_IR_THRESHOLD)

WALL_F = 0;

else

WALL_F = 1;

}

/********************************************************************************/

/******************************* command_motor **********************************/

void command_motor(void)

{

motorp(LEFT_MOTOR, SPEED_L);

motorp(RIGHT_MOTOR, SPEED_R);

wait(35);

}

/********************************************************************************/

/************************************ go ****************************************/

void go(void)

{

SPEED_L = L_TEST_SPEED;

SPEED_R = R_TEST_SPEED;

}

/********************************************************************************/

/*********************************** stop ***************************************/

void stop(void)

{

SPEED_L = MIN_SPEED;

SPEED_R = MIN_SPEED;

}

/********************************************************************************/

/****************************** move_forward ************************************/

void move_forward(void)

{

int i = 4;

go();

command_motor();	// go straight ahead into the maze

while(CDS_FRONT >= CDS_FRONT_THRESHOLD && CDS_BACK >= CDS_BACK_THRESHOLD)

{

keep_straight();

}

wait(270);

stop();

command_motor();

PORTA = PORTA | PA6_MASK;	//turn on PA6

wait(500);

PORTA = PORTA & PA6_INV;		//turn off PA6

wait(50);

while (i > 0)

{

i--;

PORTA_TEMP = PORTA;

PORTA_TEMP = PORTA_TEMP & RRS0_MASK;

if (LRS0 < LRS0_THRESH || PORTA_TEMP != 0x0001)

{

PORTA_TEMP = PORTA;

PORTA_TEMP = PORTA_TEMP & SENSOR_MASK;

if (LRS0 >= LRS0_THRESH)

PORTA_TEMP = PORTA_TEMP + 0x0010;

// steering routine, keeps Pinky centered

switch (PORTA_TEMP)		     // LRS0 LRS1 RRS1 RRS0

{

case 0x0000:			// 0    0    0   0

if (LEFT_IR >= LEFT_IR_THRESHOLD && RIGHT_IR >= RIGHT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD)

{

if (LEAN_DIR == LEFT)

lean_right();

else

lean_left();

}

wait(50);

break;

case 0x0001:			// 0    0    0   1

if (LEFT_IR >= LEFT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD)

lean_left();

wait(50);

break;

case 0x0002:			// 0    0    1	  0

if (FRONT_IR < FRONT_IR_THRESHOLD)

lean_left();

wait(50);

break;

case 0x0003:			// 0	0    1    1

if (LEFT_IR >= LEFT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD)

lean_left();

wait(50);

break;

case 0x0004:			// 0    1    0    0

if (FRONT_IR < FRONT_IR_THRESHOLD)

lean_right();

wait(50);

break;

case 0x0010:			// 1    0    0    0

if (RIGHT_IR >= RIGHT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD)

lean_right();

wait(50);

break;

case 0x0012:			// 1    0    1    0

if (FRONT_IR < FRONT_IR_THRESHOLD)

lean_left();

wait(50);

break;

case 0x0013:			// 1    0    1    1

if (FRONT_IR < FRONT_IR_THRESHOLD)

lean_left();

wait(50);

break;

case 0x0014:			// 1    1    0    0

if (RIGHT_IR >= RIGHT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD)

lean_right();

wait(50);

break;

case 0x0015:			// 1    1    0    1

if (FRONT_IR < FRONT_IR_THRESHOLD)

lean_right();

wait(50);

break;

default:

wait(50);

break;

}

}

}

}

/********************************************************************************/

/****************************** lean_left **************************************/

void lean_left(void)

{

stop();

command_motor();

LEAN_DIR = LEFT;

SPEED_L = MIN_SPEED;

SPEED_R = R_TEST_SPEED;

command_motor();

wait(20);

//	PORTA_TEMP = PORTA;

//	PORTA_TEMP = PORTA_TEMP & LRS1_MASK;

//	if (LRS0  < LRS0_THRESH && PORTA_TEMP != 0x0004 && LEFT_IR >= LEFT_IR_THRESHOLD)

//		wait(40);

stop();

command_motor();

}

/********************************************************************************/

/****************************** lean_right **************************************/

void lean_right(void)

{

LEAN_DIR = RIGHT;

SPEED_L = L_TEST_SPEED;

SPEED_R = MIN_SPEED;

command_motor();

wait(20);

//	PORTA_TEMP = PORTA;

//	PORTA_TEMP = PORTA_TEMP & RRS10_MASK;

//	if (PORTA_TEMP == 0x0000 && RIGHT_IR >= RIGHT_IR_THRESHOLD)

//		wait(40);

stop();

command_motor();

}

/********************************************************************************/

/******************************* turn_left **************************************/

void turn_left(void)

{

SPEED_L = -L_MAGIC_SPEED;

SPEED_R = R_MAGIC_SPEED;

command_motor();

if (WALL_F == 0)

{

wait(520);

stop();

command_motor();

}

else

{

wait(300);

while(FRONT_IR >= FRONT_TURN_THRESHOLD);

//		PORTA_TEMP = PORTA;

//		PORTA_TEMP = PORTA_TEMP & RRS10_MASK;

//		while(PORTA_TEMP != 0x0003)

//		{

//			PORTA_TEMP = PORTA;

//			PORTA_TEMP = PORTA_TEMP & RRS0_MASK;

//		}

stop();

command_motor();

PORTA_TEMP = PORTA;

PORTA_TEMP = PORTA_TEMP & RRS0_MASK;

if(PORTA_TEMP == 0x0000)

lean_right();

stop();

command_motor();

}

wait(2000);

}

/********************************************************************************/

/****************************** turn_right **************************************/

void turn_right(void)

{

SPEED_L = L_MAGIC_SPEED;

SPEED_R = -R_MAGIC_SPEED;

command_motor();

if (WALL_F == 0)

{

wait(320);

stop();

command_motor();

}

else

{

wait(200);

while(FRONT_IR >= FRONT_TURN_THRESHOLD);

//		PORTA_TEMP = PORTA;

//		PORTA_TEMP = PORTA_TEMP & LRS1_MASK;

//		while(LRS0 <= LRS0_THRESH || PORTA_TEMP == 0x0000)

//		{

//			PORTA_TEMP = PORTA;

//			PORTA_TEMP = PORTA_TEMP & LRS1_MASK;

//		}

wait(50);

stop();

command_motor();

if(LRS0 < LRS0_THRESH)

lean_left();

stop();

command_motor();

}

wait(2000);

}

/********************************************************************************/

/****************************** turn_around *************************************/

void turn_around(void)

{

wait(2000);

SPEED_L = -L_AROUND_SPEED;

SPEED_R = R_AROUND_SPEED;

command_motor();

if (WALL_F == 0 || WALL_L == 0 || WALL_R == 0)

{

wait(850);

while(FRONT_IR >= FRONT_TURN_THRESHOLD);

stop();

command_motor();

}

else

{

while(FRONT_IR >= FRONT_TURN_THRESHOLD);

PORTA_TEMP = PORTA;

PORTA_TEMP = PORTA_TEMP & RRS0_MASK;

while(LRS0 <= LRS0_THRESH || PORTA_TEMP == 0x0000)

{

PORTA_TEMP = PORTA;

PORTA_TEMP = PORTA_TEMP & RRS0_MASK;

}

stop();

command_motor();

}

wait(2000);

}

/********************************************************************************/

/***************************** change_dir_reverse *******************************/

void change_dir_reverse(void)

{

if (direction == NORTH)

direction = SOUTH;

else if (direction == SOUTH)

direction = NORTH;

else if (direction == WEST)

direction = EAST;

else if (direction == EAST)

direction = WEST;

}

/********************************************************************************/

/****************************** change_dir_left *********************************/

void change_dir_left()

{

if (direction == NORTH)

direction = WEST;

else if (direction == SOUTH)

direction = EAST;

else if (direction == WEST)

direction = SOUTH;

else if (direction == EAST)

direction = NORTH;

}

/********************************************************************************/

/****************************** change_dir_right ********************************/

void change_dir_right()

{

if (direction == NORTH)

direction = EAST;

else if (direction == SOUTH)

direction = WEST;

else if (direction == WEST)

direction = NORTH;

else if (direction == EAST)

direction = SOUTH;

}

/********************************************************************************/

/******************************** victory_dance *********************************/

void record_path(void)

{

int i;

for (i = 0; i <= 143; i++)

{

*(unsigned char *) (0xB600 + i) = 0x00 + maze[i];

}

}

void report_path(void)

{

int i = 0;

int j = 0;

/* VT100 clear screen */

char clear[]= "\x1b\x5B\x32\x4A\x04";

/* VT100 position cursor at (x,y) = (3,12) command is "\x1b[3;12H"*/

char place[]= "\x1b[1;1H";	/*Home*/

//	printf("%s", clear);

//	printf("%s", place);

//	printf("\t\t\tPINKY'S MAZE MAP!!\n\n");

for (i = 143; i >= 0; i--)

{

if (j%12 == 0)

{

//			printf("\n\n");

j = 0;

}

j++;

if (i == 65 || i == 66 || i == 77 || i == 78)

{

//			printf("X    ");

}

else

{

//			printf("%3d  ", *(unsigned char *) (0xB600 + i));

}

}

}

void flash_led(void)

{

int i;

for (i = 0; i < 5; i++)

{

PORTA = PORTA & PA6_INV;		//turn off PA6

wait(500);

PORTA = PORTA | PA6_MASK;	//turn on PA6

wait(500);

}

}

void turn_around_center(void)

{

wait(2000);

SPEED_L = -L_AROUND_SPEED;

SPEED_R = R_AROUND_SPEED;

command_motor();

wait(1100);

stop();

command_motor();

}

void keep_straight(void)

{

PORTA_TEMP = PORTA;

PORTA_TEMP = PORTA_TEMP & SENSOR_MASK;

if (LRS0 >= LRS0_THRESH)

PORTA_TEMP = PORTA_TEMP + 0x0010;

// steering routine, keeps Pinky centered

switch (PORTA_TEMP)		     // LRS0 LRS1 RRS1 RRS0

{

case 0x0000:			// 0    0    0   0

if (LEFT_IR >= LEFT_IR_THRESHOLD && RIGHT_IR >= RIGHT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD)

{

if (LEAN_DIR == LEFT)

lean_right();

else

lean_left();

}

wait(50);

break;

case 0x0001:			// 0    0    0    1

if (LEFT_IR >= LEFT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD && PORTA_LAST > 0x0003)

lean_left();

//				else if (LEFT_IR >= LEFT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD && PORTA_LAST <= 0x0003 && HERE == 0)

//				{

//					lean_right();

//					HERE = 1;

//				}

wait(50);

break;

case 0x0002:			// 0    0    1	  0

if (FRONT_IR < FRONT_IR_THRESHOLD && PORTA_LAST > 0x0003)

lean_left();

//				else if (LEFT_IR >= LEFT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD && PORTA_LAST <= 0x0003 && HERE == 0)

//				{

//					lean_right();

//					HERE = 1;

//				}

wait(50);

break;

case 0x0003:			// 0	0    1    1

if (LEFT_IR >= LEFT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD && PORTA_LAST > 0x0003)

lean_left();

//				else if (LEFT_IR >= LEFT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD && PORTA_LAST <= 0x0003 && HERE == 0)

//				{

//					lean_right();

//					HERE = 1;

//				}

wait(50);

break;

case 0x0004:			// 0    1    0    0

if (FRONT_IR < FRONT_IR_THRESHOLD && (PORTA_LAST != 0x0004 && PORTA_LAST != 0x0010 && PORTA_LAST != 0x0014 && PORTA_LAST != 0x0000))

lean_right();

wait(50);

break;

case 0x0010:			// 1    0    0    0

if (RIGHT_IR >= RIGHT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD && (PORTA_LAST != 0x0004 && PORTA_LAST != 0x0010 && PORTA_LAST != 0x0014 && PORTA_LAST != 0x0000))

lean_right();

wait(50);

break;

case 0x0012:			// 1    0    1    0

if (FRONT_IR < FRONT_IR_THRESHOLD)

lean_left();

wait(50);

break;

case 0x0013:			// 1    0    1    1

if (FRONT_IR < FRONT_IR_THRESHOLD)

lean_left();

wait(50);

break;

case 0x0014:			// 1    1    0    0

if (RIGHT_IR >= RIGHT_IR_THRESHOLD && FRONT_IR < FRONT_IR_THRESHOLD && (PORTA_LAST != 0x0004 && PORTA_LAST != 0x0010 && PORTA_LAST != 0x0014 && PORTA_LAST != 0x0000))

lean_right();

wait(50);

break;

case 0x0015:			// 1    1    0    1

if (FRONT_IR < FRONT_IR_THRESHOLD)

lean_right();

wait(50);

break;

default:

wait(50);

break;

}

PORTA_LAST = PORTA;

PORTA_LAST = PORTA_LAST & SENSOR_MASK;

if (LRS0 >= LRS0_THRESH)

PORTA_TEMP = PORTA_TEMP + 0x0010;

go();

command_motor();

}

/*****************************************************************

*			Pinky 2000								*

*File Name: Pinky2000.h									*

*Programmer: Xuan Gu, with contribution from Edison Rodriguez	*

*Date: November, 23 2000									*

*Description: Bellman style with some Gux flavors added maze	*

*		    solving program for Micromouse robots Pinky and the*

*		    Brain.									*

*****************************************************************/

/***************************************************************/

#include <analog.h>

#include <motortjp.h>

#include <clocktjp.h>

#include (vectors.h>

#include <tjpbase.h>

#include <stdio.h>

#include <hc11.h>

/****************************************************************/

/***************** constants for motor control ******************/

#define LEFT_MOTOR		0

#define RIGHT_MOTOR		1

#define L_MAGIC_SPEED	100

#define R_MAGIC_SPEED	150

#define MIN_SPEED		0

#define L_AROUND_SPEED	100

#define R_AROUND_SPEED	100

#define L_TEST_SPEED	100

#define R_TEST_SPEED	150

/****************************************************************/

/************* sensor definitions and sensor masks *************/

#define BUMPER		analog(0)

#define FRONT_IR	analog(1)

#define RIGHT_IR	analog(2)

#define LEFT_IR	analog(3)

#define LRS0		analog(6)

#define CDS_FRONT	analog(7)

#define CDS_BACK	analog(4)

#define SENSOR_MASK 0x0007 // 0000 0111 for LRS1, RRS1, and RRS0

#define LRS1_MASK	0x0004 // 0000 0100 for LRS1

#define RRS1_MASK	0x0002 // 0000 0010 for RRS1

#define RRS0_MASK	0x0001 // 0000 0001 for RRS0

#define RRS10_MASK	0x0003 // 0000 0011 for RRS1 and RRS0

/***************************************************************/

/********************* sensor thresholds ***********************/

#define FRONT_IR_THRESHOLD		100

#define LEFT_IR_THRESHOLD		95

#define RIGHT_IR_THRESHOLD		95

#define CDS_FRONT_THRESHOLD		190

#define CDS_BACK_THRESHOLD		190

#define FRONT_TURN_THRESHOLD		97

#define LEFT_TURN_THRESHOLD		93

#define RIGHT_TURN_THRESHOLD		93

/***************************************************************/

/**************** constants for Bellman algorithm **************/

#define FALSE	0

#define TRUE	1

#define UNEXPLORED	255	//Bellman number for unexplored cells

#define CENTER	-1		//...................center cells

#define NORTH	0		//direction the mouse is traveling in

#define SOUTH	1

#define WEST	2

#define EAST	3

#define LOOK_N	12		//array index offsets

#define LOOK_S	-12

#define LOOK_W	1

#define LOOK_E	-1

/***************************************************************/

/******************** miscellenous constants *******************/

#define LEFT	0

#define RIGHT	1

#define LRS0_THRESH 210

#define PA6_MASK	0x40		//0100 0000 for PORT A

#define PA6_INV	0xB7		//1011 1111 for PORT A

/****************************************************************/

/***** global variables used for motor control and Bellman *****/

int SPEED_L, SPEED_R;		// left and right motor speeds

int PORTA_TEMP;	// Unsiged is 16 bits, maybe not the best choice.  Decided to use integer.

int PORTA_LAST = 0x0011;  // Last PORT A reading (1 001 to begin with)

int HERE = 0;

int LEAN_DIR;			//indicates which way the robot leaned toward last time

int direction = NORTH;			// Holds the current direction of the mouse's motion

int cell_ptr = 0;			// array index pointer

int LOOK_F;			// array index offsets used when peeking to adjacent cells

int LOOK_B;

int LOOK_L;

int LOOK_R;

int WALL_F;			// holds the wall information of the current cell

int WALL_B;

int WALL_L;

int WALL_R;

int FOUNDIT = FALSE;	//found center flag, set when center is found

int TRACEDONE = FALSE;	//trace back complete flag, set when maze entrance is reached

int FINISHED = FALSE;	//race finished, set when center is reached again

int maze[144];			//array that holds the Bellman numbers of the 144 cells in the maze

int temp;				//a temporary variable

int path[144];			//array that holds the recorded path

int path_ptr = 0;	//path array pointer

int PATH_END;			//points to the last step in the recorded path

/******************************************************************************/

/************************** Function Prototypes *******************************/

void read_walls();

void move_forward();

void turn_left();

void turn_right();

void turn_around();

void turn_around_center();

void lean_left();

void lean_right();

void command_motor();

void go();

void stop();

void change_dir_reverse();

void change_dir_left();

void change_dir_right();

void record_path();

void report_path();

void flash_led();

void keep_straight();

/******************************************************************************/

Appendix B (Java Simulator Code for Pinky, simulator can be found at http://grove.ufl.edu/~gux/MyMaze.html)

import java.applet.*;

import java.awt.*;

public class MyMaze extends Applet

{

static boolean FOUNDIT = false;	//found center flag

static boolean TRACEDONE = false; //trace back complete flag

static boolean FINISHED = false; //race finished

static Cell [] maze = new Cell[144];

static String message = "Bellman # is: 0";

private Font font;

static int temp = 0;

Image mouse;

Image background;

public final int NORTH = 0;

public final int SOUTH = 1;

public final int WEST = 2;

public final int EAST = 3;

public int direction;

public int cell_ptr;

public final int LOOK_N = 12;

public final int LOOK_S = -12;

public final int LOOK_W = 1;

public final int LOOK_E = -1;

public final int MOVE_N = -32;

public final int MOVE_S = 32;

public final int MOVE_W = -32;

public final int MOVE_E = 32;

public int b_xpos = 0;

public int b_ypos = 0;

public int m_xpos = 0;

public int m_ypos = 0;

public int[] front_n = new int[2];

public int[] front_s = new int[2];

public int[] front_w = new int[2];

public int[] front_e = new int[2];

public boolean mouseDown(Event e, int x, int y)

{

if (FOUNDIT == false)

{	//if doesn't see a clear entrance into the center of the maze

if (((cell_ptr+LOOK_N > 143 || cell_ptr+LOOK_N < 0) || (maze[cell_ptr+LOOK_N].bellnum != -1) || (maze[cell_ptr+LOOK_N].bellnum == -1 && maze[cell_ptr].north != 0))

&&

((cell_ptr+LOOK_S > 143 || cell_ptr+LOOK_S < 0) || (maze[cell_ptr+LOOK_S].bellnum != -1) || (maze[cell_ptr+LOOK_S].bellnum == -1 && maze[cell_ptr].south != 0))

&&

((cell_ptr+LOOK_W > 143 || cell_ptr+LOOK_W < 0) || (maze[cell_ptr+LOOK_W].bellnum != -1) || (maze[cell_ptr+LOOK_W].bellnum == -1 && maze[cell_ptr].west != 0))

&&

((cell_ptr+LOOK_E > 143 || cell_ptr+LOOK_E < 0) || (maze[cell_ptr+LOOK_E].bellnum != -1) || (maze[cell_ptr+LOOK_E].bellnum == -1 && maze[cell_ptr].east != 0)))

//	if (cell_ptr != 67)

{

if (direction == NORTH)  // when facing north

{

if (maze[cell_ptr].north == 0) // if north wall is open

{

if (maze[cell_ptr+LOOK_N].bellnum == 255)  //and if north cell is unexplored,

{	m_ypos = m_ypos + MOVE_N;		// move north,

cell_ptr = cell_ptr + LOOK_N;		// update cell pointer (position in maze)

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_S].bellnum+1; // assign Bellman number

}

else if (maze[cell_ptr+LOOK_N].bellnum != 0) //if north cell is explored, and not 0...???????

{

if (maze[cell_ptr].west == 0 && maze[cell_ptr+LOOK_W].bellnum == 255) //and if west is

{									//open and unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");	//explore west

m_xpos = m_xpos + MOVE_W;

cell_ptr = cell_ptr + LOOK_W;

direction = WEST;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_E].bellnum+1;

}

else if (maze[cell_ptr].east == 0 && maze[cell_ptr+LOOK_E].bellnum == 255)//else if east is

{									//open and unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");	//explore east

m_xpos = m_xpos + MOVE_E;

cell_ptr = cell_ptr + LOOK_E;

direction = EAST;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_W].bellnum+1;

}

else

{

m_ypos = m_ypos + MOVE_N;	//north, west, and east are all either closed or explored, simply go north

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");

cell_ptr = cell_ptr + LOOK_N;

direction = NORTH;

}

}

}

else if (maze[cell_ptr].west == 0) // if north is closed, and west is open

{

if (maze[cell_ptr+LOOK_W].bellnum == 255) //and if west is unexplored,

{

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");

m_xpos = m_xpos + MOVE_W;		//explore west

cell_ptr = cell_ptr + LOOK_W;

direction = WEST;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_E].bellnum+1;

}

else if (maze[cell_ptr+LOOK_W].bellnum != 0) //if west is already explored, and not 0....??

{

if (maze[cell_ptr].east == 0 && maze[cell_ptr+LOOK_E].bellnum == 255) // and if east is open and

{									// unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");	// explore east

m_xpos = m_xpos + MOVE_E;

cell_ptr = cell_ptr + LOOK_E;

direction = EAST;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_W].bellnum+1;

}

else if (maze[cell_ptr].east != 0)  //north is closed, east is open but explored, west is closed, simply go east

{

m_xpos = m_xpos + MOVE_W;

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");

cell_ptr = cell_ptr + LOOK_W;

direction = WEST;

}

}

}

else if (maze[cell_ptr].east == 0)	// if north and west are closed, and east is open

{

if(maze[cell_ptr+LOOK_E].bellnum == 255) // if east is unexplored

{

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif"); //explored east

m_xpos = m_xpos + MOVE_E;

cell_ptr = cell_ptr + LOOK_E;

direction = EAST;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_W].bellnum+1;

}

else if (maze[cell_ptr+LOOK_E].bellnum != 0) //if east is already explored, and not 0....??

{

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");

m_xpos = m_xpos + MOVE_E;  //north and west are closed, east is open but explored, just go east

cell_ptr = cell_ptr + LOOK_E;

direction = EAST;

}

}

else

{	// reached a deadend, reverse course

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif");

direction = SOUTH;

}

}

else if (direction == SOUTH)  // when facing south

{

if (maze[cell_ptr].south == 0) // if south wall is open

{

if (maze[cell_ptr+LOOK_S].bellnum == 255)  //and if south cell is unexplored,

{	m_ypos = m_ypos + MOVE_S;		// move south,

cell_ptr = cell_ptr + LOOK_S;		// update cell pointer (position in maze)

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_N].bellnum+1; // assign Bellman number

}

else if (maze[cell_ptr+LOOK_S].bellnum != 0) //if south cell is explored, and not 0...???????

{

if (maze[cell_ptr].east == 0 && maze[cell_ptr+LOOK_E].bellnum == 255) //and if east is

{									//open and unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");	//explore east

m_xpos = m_xpos + MOVE_E;

cell_ptr = cell_ptr + LOOK_E;

direction = EAST;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_W].bellnum+1;

}

else if (maze[cell_ptr].west == 0 && maze[cell_ptr+LOOK_W].bellnum == 255)//else if west is

{									//open and unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");	//explore west

m_xpos = m_xpos + MOVE_W;

cell_ptr = cell_ptr + LOOK_W;

direction = WEST;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_E].bellnum+1;

}

else

{

m_ypos = m_ypos + MOVE_S; //south, west, and east are all either closed or explored, simply go south

cell_ptr = cell_ptr + LOOK_S;

direction = SOUTH;

}

}

}

else if (maze[cell_ptr].east == 0) // if south is closed, and east is open

{

if (maze[cell_ptr+LOOK_E].bellnum == 255) //and if east is unexplored,

{

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");

m_xpos = m_xpos + MOVE_E;		//explore east

cell_ptr = cell_ptr + LOOK_E;

direction = EAST;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_W].bellnum+1;

}

else if (maze[cell_ptr+LOOK_E].bellnum != 0) //if east is already explored, and not 0....??

{

if (maze[cell_ptr].west == 0 && maze[cell_ptr+LOOK_W].bellnum == 255) // and if west is open and

{									// unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");	// explore west

m_xpos = m_xpos + MOVE_W;

cell_ptr = cell_ptr + LOOK_W;

direction = WEST;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_E].bellnum+1;

}

else if (maze[cell_ptr].west != 0)  //south is closed, west is open but explored, east is closed, simply go west

{

m_xpos = m_xpos + MOVE_E;

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");

cell_ptr = cell_ptr + LOOK_E;

direction = EAST;

}

}

}

else if (maze[cell_ptr].west == 0)	// if south and east are closed, and west is open

{

if(maze[cell_ptr+LOOK_W].bellnum == 255) // if west is unexplored

{

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif"); //explored west

m_xpos = m_xpos + MOVE_W;

cell_ptr = cell_ptr + LOOK_W;

direction = WEST;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_E].bellnum+1;

}

else if (maze[cell_ptr+LOOK_W].bellnum != 0) //if west is already explored, and not 0....??

{

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");

cell_ptr = cell_ptr + LOOK_W;

direction = WEST;

m_xpos = m_xpos + MOVE_W;  //south and east are closed, west is open but explored, just go west

}

}

else

{	// reached a deadend, reverse course

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");

direction = NORTH;

}

}

else if (direction == WEST)  // when facing west

{

if (maze[cell_ptr].west == 0) // if west wall is open

{

if (maze[cell_ptr+LOOK_W].bellnum == 255)  //and if west cell is unexplored,

{	m_xpos = m_xpos + MOVE_W;		// move west,

cell_ptr = cell_ptr + LOOK_W;		// update cell pointer (position in maze)

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_E].bellnum+1; // assign Bellman number

}

else if (maze[cell_ptr+LOOK_W].bellnum != 0) //if west cell is explored, and not 0...???????

{

if (maze[cell_ptr].south == 0 && maze[cell_ptr+LOOK_S].bellnum == 255) //and if south is

{									//open and unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif");	//explore south

m_ypos = m_ypos + MOVE_S;

cell_ptr = cell_ptr + LOOK_S;

direction = SOUTH;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_N].bellnum+1;

}

else if (maze[cell_ptr].north == 0 && maze[cell_ptr+LOOK_N].bellnum == 255)//else if north is

{									//open and unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");	//explore north

m_ypos = m_ypos + MOVE_N;

cell_ptr = cell_ptr + LOOK_N;

direction = NORTH;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_S].bellnum+1;

}

else

{

m_xpos = m_xpos + MOVE_W; //west, south, and north are all either closed or explored, simply go west

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");

cell_ptr = cell_ptr + LOOK_W;

direction = WEST;

}

}

}

else if (maze[cell_ptr].south == 0) // if west is closed, and south is open

{

if (maze[cell_ptr+LOOK_S].bellnum == 255) //and if south is unexplored,

{

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif");

m_ypos = m_ypos + MOVE_S;		//explore south

cell_ptr = cell_ptr + LOOK_S;

direction = SOUTH;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_N].bellnum+1;

}

else if (maze[cell_ptr+LOOK_S].bellnum != 0) //if south is already explored, and not 0....??

{

if (maze[cell_ptr].north == 0 && maze[cell_ptr+LOOK_N].bellnum == 255) // and if north is open and

{									// unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");	// explore north

m_ypos = m_ypos + MOVE_N;

cell_ptr = cell_ptr + LOOK_N;

direction = NORTH;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_S].bellnum+1;

}

/******!!!!!****/	else if (maze[cell_ptr].north != 0)  //west is closed, south is open but explored, and, north is closed, simply go south

{

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif");

m_ypos = m_ypos + MOVE_S;

cell_ptr = cell_ptr + LOOK_S;

direction = SOUTH;

}

}

}

else if (maze[cell_ptr].north == 0)	// if west and south are closed, and north is open

{

if(maze[cell_ptr+LOOK_N].bellnum == 255) // if north is unexplored

{

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif"); //explored north

m_ypos = m_ypos + MOVE_N;

cell_ptr = cell_ptr + LOOK_N;

direction = NORTH;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_S].bellnum+1;

}

else if (maze[cell_ptr+LOOK_N].bellnum != 0) //if north is already explored, and not 0....??

{

m_ypos = m_ypos + MOVE_N;  //west and south are closed, north is open but explored, just go north

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");

direction = NORTH;

cell_ptr = cell_ptr + LOOK_N;

}

}

else

{	// reached a deadend, reverse course

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");

direction = EAST;

}

}

else if (direction == EAST)  // when facing east

{

if (maze[cell_ptr].east == 0) // if east wall is open

{

if (maze[cell_ptr+LOOK_E].bellnum == 255)  //and if east cell is unexplored,

{	m_xpos = m_xpos + MOVE_E;		// move east,

cell_ptr = cell_ptr + LOOK_E;		// update cell pointer (position in maze)

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_W].bellnum+1; // assign Bellman number

}

else if (maze[cell_ptr+LOOK_E].bellnum != 0) //if east cell is explored, and not 0...???????

{

if (maze[cell_ptr].north == 0 && maze[cell_ptr+LOOK_N].bellnum == 255) //and if north is

{									//open and unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");	//explore north

m_ypos = m_ypos + MOVE_N;

cell_ptr = cell_ptr + LOOK_N;

direction = NORTH;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_S].bellnum+1;

}

else if (maze[cell_ptr].south == 0 && maze[cell_ptr+LOOK_S].bellnum == 255)//else if south is

{									//open and unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif");	//explore south

m_ypos = m_ypos + MOVE_S;

cell_ptr = cell_ptr + LOOK_S;

direction = SOUTH;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_N].bellnum+1;

}

else

{

m_xpos = m_xpos + MOVE_E; //east is open, north and south are all either closed or explored, simply go east

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");

cell_ptr = cell_ptr + LOOK_E;

direction = EAST;

}

}

}

else if (maze[cell_ptr].north == 0) // if east is closed, and north is open

{

if (maze[cell_ptr+LOOK_N].bellnum == 255) //and if north is unexplored,

{

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");

m_ypos = m_ypos + MOVE_N;		//explore north

cell_ptr = cell_ptr + LOOK_N;

direction = NORTH;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_S].bellnum+1;

}

else if (maze[cell_ptr+LOOK_N].bellnum != 0) //if NORTH is already explored, and not 0....??

{

if (maze[cell_ptr].south == 0 && maze[cell_ptr+LOOK_S].bellnum == 255) // and if south is open and

{									// unexplored

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif");	// explore south

m_ypos = m_ypos + MOVE_S;

cell_ptr = cell_ptr + LOOK_S;

direction = SOUTH;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_N].bellnum+1;

}

else if (maze[cell_ptr].south != 0)  //east is closed, north is open but explored, south is closed, simply go north

{

m_ypos = m_ypos + MOVE_N;

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");

cell_ptr = cell_ptr + LOOK_N;

direction = NORTH;

}

}

}

else if (maze[cell_ptr].south == 0)	// if east and north are closed, and south is open

{

if(maze[cell_ptr+LOOK_S].bellnum == 255) // if south is unexplored

{

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif"); //explored south

m_ypos = m_ypos + MOVE_S;

cell_ptr = cell_ptr + LOOK_S;

direction = SOUTH;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_N].bellnum+1;

}

else if (maze[cell_ptr+LOOK_S].bellnum != 0) //if south is already explored, and not 0....??

{

m_ypos = m_ypos + MOVE_S;  //east and north are closed, south is open but explored, just go south

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif");

cell_ptr = cell_ptr + LOOK_S;

direction = SOUTH;

}

}

else

{	// reached a deadend, reverse course

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");

direction = WEST;

}

}

}

else //if at the only entrance to the center, just move in

{  //if center is in the north adjacent cell with a clear path, go inside

if(maze[cell_ptr+LOOK_N].bellnum == -1 && maze[cell_ptr].north == 0)

{

direction = NORTH;

cell_ptr = cell_ptr + LOOK_N;

//		maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_S].bellnum+1;

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");

m_ypos = m_ypos + MOVE_N;	//go inside the center

FOUNDIT = true;

}

//if center is in the south adjacent cell with a clear path, go inside

else if(maze[cell_ptr+LOOK_S].bellnum == -1 && maze[cell_ptr].south == 0)

{

direction = SOUTH;

cell_ptr = cell_ptr + LOOK_S;

//		maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_N].bellnum+1;

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif");

m_ypos = m_ypos + MOVE_S;	//go inside the center

FOUNDIT = true;

}

//if center is in the west adjacent cell with a clear path, go inside

else if(maze[cell_ptr+LOOK_W].bellnum == -1 && maze[cell_ptr].west == 0)

{

direction = WEST;

cell_ptr = cell_ptr + LOOK_W;

//		maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_E].bellnum+1;

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");

m_xpos = m_xpos + MOVE_W;	//go inside the center

FOUNDIT = true;

}

//if center is in the east adjacent cell with a clear path, go inside

else if(maze[cell_ptr+LOOK_E].bellnum == -1 && maze[cell_ptr].east == 0)

{

direction = EAST;

cell_ptr = cell_ptr + LOOK_E;

//		maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_W].bellnum+1;

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");

m_xpos = m_xpos + MOVE_E;	//go inside the center

FOUNDIT = true;

}

}

}

else if (FOUNDIT == true && TRACEDONE == false) //start of the traceback process, from center to the maze entrance

{

if(maze[cell_ptr].bellnum == -1)

{

temp = maze[cell_ptr+LOOK_W].bellnum;

direction = WEST;

cell_ptr = cell_ptr+LOOK_W;

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");

m_xpos = m_xpos + MOVE_W;

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_E].bellnum+1;

}

else

{

/*	if(cell_ptr <= 143 && cell_ptr > 132)

{

direction = EAST;

cell_ptr = cell_ptr+LOOK_E;

temp = maze[cell_ptr].bellnum;

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");

m_xpos = m_xpos + MOVE_E;

if (maze[cell_ptr].bellnum == 0)

{

TRACEDONE = true;

}

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_W].bellnum+1;

}

*/

if(((cell_ptr+LOOK_N <= 143 && cell_ptr+LOOK_N >= 0) &&

(maze[cell_ptr+LOOK_N].bellnum == temp-1 && maze[cell_ptr].north == 0)))

{

direction = NORTH;

cell_ptr = cell_ptr+LOOK_N;

temp = maze[cell_ptr].bellnum;

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");

m_ypos = m_ypos + MOVE_N;

if (maze[cell_ptr].bellnum == 0)

{

TRACEDONE = true;

}

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_S].bellnum+1;

}

else if(((cell_ptr+LOOK_S <= 143 && cell_ptr+LOOK_S >= 0) &&

(maze[cell_ptr+LOOK_S].bellnum == temp-1 && maze[cell_ptr].south == 0)))

{

direction = SOUTH;

cell_ptr = cell_ptr+LOOK_S;

temp = maze[cell_ptr].bellnum;

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif");

m_ypos = m_ypos + MOVE_S;

if (maze[cell_ptr].bellnum == 0)

{

TRACEDONE = true;

}

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_N].bellnum+1;

}

else if(((cell_ptr+LOOK_W <= 143 && cell_ptr+LOOK_W >= 0) &&

(maze[cell_ptr+LOOK_W].bellnum == temp-1 && maze[cell_ptr].west == 0)))

{

direction = WEST;

cell_ptr = cell_ptr+LOOK_W;

temp = maze[cell_ptr].bellnum;

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");

m_xpos = m_xpos + MOVE_W;

if (maze[cell_ptr].bellnum == 0)

{

TRACEDONE = true;

}

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_E].bellnum+1;

}

else if(((cell_ptr+LOOK_E <= 143 && cell_ptr+LOOK_E >= 0) &&

(maze[cell_ptr+LOOK_E].bellnum == temp-1 && maze[cell_ptr].east == 0)))

{

direction = EAST;

cell_ptr = cell_ptr+LOOK_E;

temp = maze[cell_ptr].bellnum;

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");

m_xpos = m_xpos + MOVE_E;

if (maze[cell_ptr].bellnum == 0)

{

TRACEDONE = true;

}

maze[cell_ptr].bellnum = maze[cell_ptr+LOOK_W].bellnum+1;

}

}

}

else if (TRACEDONE == true && FINISHED == false) //get from the start to finish as quick as possible

{

if(((cell_ptr+LOOK_N <= 143 && cell_ptr+LOOK_N >= 0) &&

(maze[cell_ptr+LOOK_N].bellnum == maze[cell_ptr].bellnum-1 && maze[cell_ptr].north == 0)))

{

direction = NORTH;

cell_ptr = cell_ptr+LOOK_N;

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");

m_ypos = m_ypos + MOVE_N;

if (maze[cell_ptr].bellnum == -1)

{

FINISHED = true;

}

}

else if(((cell_ptr+LOOK_S <= 143 && cell_ptr+LOOK_S >= 0) &&

(maze[cell_ptr+LOOK_S].bellnum == maze[cell_ptr].bellnum-1 && maze[cell_ptr].south == 0)))

{

direction = SOUTH;

cell_ptr = cell_ptr+LOOK_S;

mouse = this.getImage(this.getDocumentBase(), "mouse_s.gif");

m_ypos = m_ypos + MOVE_S;

if (maze[cell_ptr].bellnum == -1)

{

FINISHED = true;

}

}

else if(((cell_ptr+LOOK_W <= 143 && cell_ptr+LOOK_W >= 0) &&

(maze[cell_ptr+LOOK_W].bellnum == maze[cell_ptr].bellnum-1 && maze[cell_ptr].west == 0)))

{

direction = WEST;

cell_ptr = cell_ptr+LOOK_W;

mouse = this.getImage(this.getDocumentBase(), "mouse_w.gif");

m_xpos = m_xpos + MOVE_W;

if (maze[cell_ptr].bellnum == -1)

{

FINISHED = true;

}

}

else if(((cell_ptr+LOOK_E <= 143 && cell_ptr+LOOK_E >= 0) &&

(maze[cell_ptr+LOOK_E].bellnum == maze[cell_ptr].bellnum-1 && maze[cell_ptr].east == 0)))

{

direction = EAST;

cell_ptr = cell_ptr+LOOK_E;

mouse = this.getImage(this.getDocumentBase(), "mouse_e.gif");

m_xpos = m_xpos + MOVE_E;

if (maze[cell_ptr].bellnum == -1)

{

FINISHED = true;

}

}

}

if(FINISHED == false)

message = "Bellman# is:" + maze[cell_ptr].bellnum;

else

message = "WOOHOO!!!";

repaint();

return true;

}

// Draw the applet whenever necessary

public void paint(Graphics g)

{

int bwidth = background.getWidth(this);

int bheight = background.getHeight(this);

int mwidth = mouse.getWidth(this) - 5;

int mheight = mouse.getHeight(this) - 5;

g.drawImage(background, b_xpos, b_ypos, bwidth, bheight, this);

g.drawImage(mouse, m_xpos+11*32+4, m_ypos+11*32+4, mwidth, mheight, this);

g.setColor(Color.red);

g.setFont(font);

g.drawString(message, 388, 200);

}

public void update(Graphics g)

{

paint(g);

}

public static void main(String[] args)

{

}

// Initialize the applet

public void init()

{

int i;

this.setBackground(Color.white);

mouse = this.getImage(this.getDocumentBase(), "mouse_n.gif");

background = this.getImage(this.getDocumentBase(), "MyMaze4.gif");

font = new Font("sansserif", Font.BOLD, 16);

maze[0] = new Cell(0,1,1,1);

maze[1] = new Cell(1,1,0,1);

maze[2] = new Cell(1,1,0,0);

maze[3] = new Cell(1,1,0,0);

maze[4] = new Cell(1,1,0,0);

maze[5] = new Cell(1,1,0,0);

maze[6] = new Cell(1,1,0,0);

maze[7] = new Cell(1,1,0,0);

maze[8] = new Cell(1,1,0,0);

maze[9] = new Cell(1,1,0,0);

maze[10] = new Cell(1,1,0,0);

maze[11] = new Cell(0,1,1,0);

maze[12] = new Cell(0,0,1,1);

maze[13] = new Cell(0,1,0,1);

maze[14] = new Cell(1,1,0,0);

maze[15] = new Cell(1,1,0,0);

maze[16] = new Cell(1,1,0,0);

maze[17] = new Cell(0,1,0,0);

maze[18] = new Cell(1,1,0,0);

maze[19] = new Cell(1,1,0,0);

maze[20] = new Cell(1,1,0,0);

maze[21] = new Cell(1,1,0,0);

maze[22] = new Cell(0,1,1,0);

maze[23] = new Cell(0,0,1,1);

maze[24] = new Cell(0,0,1,1);

maze[25] = new Cell(0,0,1,1);

maze[26] = new Cell(0,1,0,1);

maze[27] = new Cell(1,1,0,0);

maze[28] = new Cell(1,1,0,0);

maze[29] = new Cell(1,0,0,0);

maze[30] = new Cell(1,1,0,0);

maze[31] = new Cell(1,1,0,0);

maze[32] = new Cell(1,1,0,0);

maze[33] = new Cell(0,1,1,0);

maze[34] = new Cell(0,0,1,1);

maze[35] = new Cell(0,0,1,1);

maze[36] = new Cell(0,0,1,1);

maze[37] = new Cell(0,0,1,1);

maze[38] = new Cell(0,0,1,1);

maze[39] = new Cell(0,1,0,1);

maze[40] = new Cell(1,1,0,0);

maze[41] = new Cell(0,1,0,0);

maze[42] = new Cell(1,1,0,0);

maze[43] = new Cell(1,1,1,0);

maze[44] = new Cell(0,1,1,1);

maze[45] = new Cell(0,0,1,1);

maze[46] = new Cell(0,0,1,1);

maze[47] = new Cell(0,0,1,1);

maze[48] = new Cell(0,0,1,1);

maze[49] = new Cell(0,0,1,1);

maze[50] = new Cell(0,0,1,1);

maze[51] = new Cell(0,0,1,1);

maze[52] = new Cell(0,1,0,1);

maze[53] = new Cell(1,0,0,0);

maze[54] = new Cell(1,1,0,0);

maze[55] = new Cell(0,1,1,0);

maze[56] = new Cell(0,0,1,1);

maze[57] = new Cell(0,0,1,1);

maze[58] = new Cell(0,0,1,1);

maze[59] = new Cell(0,0,1,1);

maze[60] = new Cell(0,0,1,1);

maze[61] = new Cell(0,0,1,1);

maze[62] = new Cell(0,0,0,1);

maze[63] = new Cell(0,0,1,0);

maze[64] = new Cell(0,0,1,1);

maze[65] = new Cell(0,1,0,1);

maze[66] = new Cell(0,1,0,0);

maze[67] = new Cell(0,0,1,0);

maze[68] = new Cell(0,0,1,1);

maze[69] = new Cell(0,0,0,1);

maze[70] = new Cell(0,0,0,0);

maze[71] = new Cell(0,0,1,0);

maze[72] = new Cell(0,0,1,1);

maze[73] = new Cell(0,0,1,1);

maze[74] = new Cell(0,0,1,1);

maze[75] = new Cell(0,0,1,1);

maze[76] = new Cell(0,0,1,1);

maze[77] = new Cell(1,0,0,1);

maze[78] = new Cell(1,0,1,0);

maze[79] = new Cell(0,0,1,1);

maze[80] = new Cell(0,0,1,1);

maze[81] = new Cell(0,0,1,1);

maze[82] = new Cell(0,0,1,1);

maze[83] = new Cell(0,0,1,1);

maze[84] = new Cell(0,0,1,1);

maze[85] = new Cell(0,0,0,1);

maze[86] = new Cell(0,0,1,0);

maze[87] = new Cell(0,0,1,1);

maze[88] = new Cell(1,0,0,1);

maze[89] = new Cell(1,1,0,0);

maze[90] = new Cell(1,1,0,0);

maze[91] = new Cell(1,0,1,0);

maze[92] = new Cell(0,0,0,1);

maze[93] = new Cell(0,0,1,0);

maze[94] = new Cell(0,0,1,1);

maze[95] = new Cell(0,0,1,1);

maze[96] = new Cell(0,0,1,1);

maze[97] = new Cell(0,0,1,1);

maze[98] = new Cell(0,0,1,1);

maze[99] = new Cell(1,0,0,1);

maze[100] = new Cell(1,1,0,0);

maze[101] = new Cell(1,1,0,0);

maze[102] = new Cell(1,1,0,0);

maze[103] = new Cell(1,1,0,0);

maze[104] = new Cell(1,0,1,0);

maze[105] = new Cell(0,0,1,1);

maze[106] = new Cell(0,0,1,1);

maze[107] = new Cell(0,0,1,1);

maze[108] = new Cell(0,0,1,1);

maze[109] = new Cell(1,0,1,1);

maze[110] = new Cell(1,0,0,1);

maze[111] = new Cell(1,1,0,0);

maze[112] = new Cell(1,1,1,0);

maze[113] = new Cell(1,1,0,1);

maze[114] = new Cell(1,1,0,0);

maze[115] = new Cell(1,1,0,0);

maze[116] = new Cell(1,1,0,0);

maze[117] = new Cell(1,0,1,0);

maze[118] = new Cell(0,0,1,1);

maze[119] = new Cell(0,0,1,1);

maze[120] = new Cell(0,0,1,1);

maze[121] = new Cell(1,1,0,1);

maze[122] = new Cell(1,1,0,0);

maze[123] = new Cell(0,1,0,0);

maze[124] = new Cell(1,1,0,0);

maze[125] = new Cell(1,1,0,0);

maze[126] = new Cell(1,1,1,0);

maze[127] = new Cell(1,1,0,1);

maze[128] = new Cell(1,1,0,0);

maze[129] = new Cell(1,1,0,0);

maze[130] = new Cell(1,0,1,0);

maze[131] = new Cell(0,0,1,1);

maze[132] = new Cell(1,0,0,1);

maze[133] = new Cell(1,1,0,0);

maze[134] = new Cell(1,1,0,0);

maze[135] = new Cell(1,0,0,0);

maze[136] = new Cell(1,1,0,0);

maze[137] = new Cell(1,1,0,0);

maze[138] = new Cell(1,1,0,0);

maze[139] = new Cell(1,1,0,0);

maze[140] = new Cell(1,1,0,0);

maze[141] = new Cell(1,1,0,0);

maze[142] = new Cell(1,1,0,0);

maze[143] = new Cell(1,0,1,0);

maze[0].bellnum = 0;

for (i=1; i<=143; i++)

maze[i].bellnum = 255;

maze[65].bellnum = -1;

maze[66].bellnum = -1;

maze[77].bellnum = -1;

maze[78].bellnum = -1;

direction = NORTH;

cell_ptr = 0;

//		message = "Bellnum# is:" + maze[cell_ptr].bellnum;

}

}

import java.applet.*;

import java.awt.*;

public class Cell

{

public int north, south, west, east, bellnum;

public Cell(int north, int south, int west, int east, int bellnum)

{

this.north = north;

this.south = south;

this.west = west;

this.east = east;

this.bellnum = bellnum;

}

public Cell(int north, int south, int west, int east)

{

this(north, south, west, east, 0);

}

}

Appendix C

Figure 6-Maze Built for this Project

Micro Mouse Contest Rules (Some rules vary)

I. Specifications for the maze

- The maze shall comprise 16 x 16 multiples of an 18 cm x 18 cm unit square. The walls constituting the maze shall be 5 cm high and 1.2 cm thick. Passageways between the walls shall be 16.8 cm wide. The outside wall shall enclose the entire maze.

- The sides of the maze shall be white, and the top of the walls shall be red. The floor of the maze shall be made of wood and finished with a non-gloss black paint. The coating on the top and sides of the walls shall be selected to reflect infred-red light and the coating on the floor shall absorb it.

- The start of the maze shall be located at one of the four corners. The starting square shall have walls on three sides. The starting square orientation shall be such that when the open wall is to the "north", outside maze walls are on the "west" and "south". At the center of the maze shall be a large opening which is composed of 4 unit squares. This central square shall be the destination. A red post, 20 cm high, and 2.5 cm on each side, may be placed at the center of the large destination square if requested by the handler.

- Small square posts, each 1.2 cm x 1.2 cm x 5 cm high, at the four corners of each unit are called latice points. The maze shall be constituted such that there is at least one wall touching each lattice point, except for the destination square.

- The dimensions of the maze shall be accurate to within 5% or 2 cm, whichever is less. Assembly joints on the maze floor shall not incolve steps greater than 0.5 mm. The change of slope at an assembly joint shall not be greater than 4. Gaps between the walls of adjacent squares shal not be greater than 1 mm.

II. Specifications for the Micro Mouse

- A Micro Mouse shall be self contained. It shall not use an energy source employing a combustion process.

- The length and width of a Micro Mouse shall be restricted to a square region of 25 cm x 25 cm. The dimensions of a Micro Mouse which changes its geometry during a run shall never be greater than 25 cm x 25 cm. The height of a Micro Mouse is unrestricted.

- A Micro Mouse shall not leave anything behind while negotiating the maze.

- A Micro Mouse shall not jump over, climb, scratch, damage, or destroy the walls of the maze.

III. Rules for the Contest

The basic function of a Micro Mouse is to travel from the start square to the destination square. This is called a run. The time it takes is called the run time. Traveling from the destination square back to the start square is not considered a run. The total time from the first activation of the Micro Mouse until the start of each run is also mesured. This is called the maze time. If a mouse requires manual assistance at any time during the contest it is considered touched. By using these three parameters the scoring of the contest is designed to reward speed, efficiency of maze solving, and self-reliance of the Micro Mouse.

- The scoring of a Micro Mouse shall be done by computing a handicapped time for each run. This shall be calculated by adding the time for each run to 30 of the maze time associated with that run and subtracting a 10 second bonus if the Micro Mouse has not been touched yet1. 1 For example assume a Micro Mouse, after being on the maze for 4 minutes without being touched, starts a run which takes 20 seconds; the run will have a handicapped time of: 20+4300-10=18seconds The run with the fastest handicapped time for each Micro Mouse shall be the offical time of that Micro Mouse.

- Each contesting Micro Mouse shall be subject to a time limit of 15 minutes on the maze. Within this time limit, the Micro Mouse may make as many runs as possible.

- When the Micro Mouse reaches the maze center it may be manually lifted out and restarted or it may make its own way back to the start square. Manually lifting it out shall be considered touching the Micro Mouse and will cause it to loose the 10 second bonus on all further runs.

- The time for each run shall be measured from the moment the Micro Mouse leaves the start square until it enters the finish square. The total time on the maze shall be measured from the time the Micro Mouse is first activated. The mouse does not have to move when it is first activated but it must be positioned in the start square ready to run.

- The time taken to negotiate the maze shall be measured either manually by the contest officials or by infra-red sensors set at the start and destination. If infra-red sensors are used, the start sensor shall be positioned at the boundary between the start square and the next unit square. The destination sensor shall be placed at the entrance to the destination square. The infra-red beam of each sensor shall be horizontal and positioned approximately 1 cm above the floor.

- The starting procedure of the Micro Mouse shall not offer a choice of strategies to the handler.

- Once the maze configuration for the contest is disclosed, the operator shall not feed the Micro Mouse with any maze information.

- The illumination, temperature, and humidity of the room in which the maze is located shall be those of an ambient enviornment. Requests to adjust the illumination may be accepted at the discretion of the contest officials.

- If a Micro Mouse appears to be malfunctioning, the handlers may ask the judges for permission to abandon the run and restart the Micro Mouse at the beginning. A Micro Mouse shall not be re-started merely because it has taken a wrong turn.

- If a Micro Mouse team elects to stop because of technical problems, the judges may, at their discretion, permit the team to run again later in the contest with a 3 minute maze time penalty1. 1 For example, assume a Micro Mouse is stopped after 4 minutes; it must be restarted as if it had already run for 7 minutes, and will have only 8 more minutes to run.

- If any part of a Micro Mouse is replaced during its performance, such as batteries or EPROMS, or if any significant adjustment is made, the memory of the maze within the Micro Mouse shall be erased before restarting. Slight adjustments, such as to the sensors may be allowed at the discretion of the judges, but operation of speed or strategy controls is expressly forbidden without a memory erasure.

- No part of the Micro Mouse (with the possible excpetion of batteries) shall be transferred to another Micro Mouse. For example if one chassis is used with two alternative controllers, then they are the same Micro Mouse and must perform within a single 15 minute allocation. The memory must be cleared with the change of a controller.

- The contest officials shall reserve the right to stop a run, or disqualify a Micro Mouse, if they believe its continued operation is endangering the condition of the maze.