University of Florida

department of electrical engineering

eel 5934

intelligent machines design laboratory

Wayne devoe

TABLE OF CONTENTS

ABSTRACT................................................................................................................3

INTRODUCTION.......................................................................................................4

ZAPHOD’S OVERRIDE FEATURE...........................................................................5

- The Rear Motor..........................................................................................5

- The Front Motor.........................................................................................7

ZAPHOD’S END-OF-THE-WORLD DETECTION....................................................8

ZAPHOD’S COLLISION DETECTION......................................................................8

CONCLUSION...........................................................................................................10

PROGRAM CODE......................................................................................................11

ABSTRACT

This is the last of three papers that tracked the design and fabrication of Zaphod, an autonomous agent.  Zaphod was develop as part of the University of Florida’s Robotics class, headed by Dr. Doty.  This paper covers the work that Wayne DeVoe, one of two students working on Zaphod, focused on during the development of Zaphod.

INTRODUCTION

The Robotics class at the University of Florida teaches design principles for an autonomous agent.  As part of this process, the students work to build an agent of their own.  References and ideas for the design and fabrication of Zaphod, the autonomous agent in question, were obtained from Dr. Doty, the teacher assistants, and the following books:  The 6.270 Robot Builder’s Guide, by Fred Martin, Mobile Robots:  Inspiration to Implementation, by Joseph Jones and Anita Flynn, and Designing Autonomous Agents:  Theory and Practice from Biology to Engineering, edited by Pattie Maes.  The Motorola 68HC11 evbu board with expansion circuitry, designed by the Machine Intelligence Laboratory group at the University of Florida, was used as the processor for Zaphod.

The behaviors that Zaphod presently possesses are collision avoidance, collision detection, override control, end-of-the-world detection, and person detection.  The collision avoidance and the end-of-the-world detection are realized with IR-LEDs and IR detectors.  The override control was done by exploiting the fact that Zaphods body was once an RC car.  Collision detection was accomplished with the Siemen IR emitter/detector.  Finally, a pyro-sensor was used to detect people.

This report covers the override control, end-of-the-world detection, and collision detection.  The remaining behaviors that are featured on Zaphod are discussed by my lab partner, Anita Nariani.

Zaphod’s Structure

Myself and Anita decided on modifying a RC car that we purchased at Toys R Us for the main structure of Zaphod.  Originally this sounded like a real good idea that would save us the time it would take to construct a body from scratch.  Boy! were we wrong.  Many difficulties arose while attempting to implement Zaphod on the RC car.  Most students in the class used the circular robot with two modified servos as the means of locomotion.  With a circular robot it was much easier to control a robots movements.  This is due to the ability of a two wheeled circular robot to stop and turn in place.  A four wheeled robot  can not do this.  This made turning complex.  One design goal that we needed to keep in mind was that we wanted to still be able to use the remote control on Zaphod, giving override control.

ZAPHOD’S OVERRIDE FEATURE

The Rear Motor:  Forward and Backward Motion

In order to be able to still control the RC car by remote the RC car’s circuitry had to remain in place and the 68HC11 needed a way to control the motors.  For the rear motor, the motor for forward and backward motion, we found that we could control it by using a motor driver chip with a higher amp rating:  the TI SN754410NE.  The motor control leads were placed over the DC motor in the rear and the original wires that were connected to the RC cars circuit board were cut.

How did we control the rear motor in override mode if it was disconnected from the RC car’s circuit board?  To achieve this goal we had to find a way to detect if the remote interface was being used.  By probing the ASIC chip that is on the RC car while we were running it by remote it was discovered that either the node at pin 10 or pin 9 would drop in voltage if the RC car was being driven in forward or reverse.  To detect this voltage drop a voltage divider was placed at those nodes.  One thing that we discovered about the voltage divider was that the total resistance across the series resistance had to be above 1 MegOhm, otherwise the voltage divider would drop both nodes to ground, giving erroneous readings.  Subsequently the voltage divide was routed into the AD port on the HC11, see figure 1.

With this implementation remote override could be detected by looking at the AD port into which the voltage divider of figure 1 was routed into.  When the remote interface is activated, by trying to move the car in forward or reverse, Zaphod’s program will detect the voltage drop.  Zaphod then begins to continually scan the voltage divider and to decide if the motor should be driven in forward or reveres.

Figure 1

The Front Motor:  Right and Left Steering

A similar approach to that used to control the rear motor was applied to trying to control the front motor.  Anita and myself probed the ASIC chip and found that pins 4 and 5 changed when to RC car was to turn right or left.  Unlike the rear motor we could not cut the motor leads because the front motor used some kind of feedback system that we did not understand.  Thus, the drain node of transistors were placed at the nodes for pin 4 and 5.  The gate of these nodes were connected to the motor drive chip, and the source of the transistors were grounded.  The NTE 67 n-channel MOSFET is the transistor that was used, see figure 2. By doing this the nodes at pin 5 and 4 became OR gates.  Either the remote interface could drop one of the nodes to ground or the 68HC11 could.
	With this type of implementation the remote control could still be used on the RC car.  The only stipulation is that the motor control routing in Zaphods program had to look at the nodes and decide if a person is trying to control him by remote access.  If so the program would have to continually probe the nodes at pin 9 and 10 and decide how to drive the rear motor.  The motor control routine structure contained two main parts.  Activation of the two parts were controlled by using flags.  If the override flag was off, 68HC11 control, the program would exhibit obstacle avoidance.  When the flag would become set, the 68HC11 relinquish control of the front motor and continuously scans pins 9 and 10 to decide how to drive the rear motor.

Figure 2

END-OF-THE-WORLD DETECTION

End-of-the-world detection is done to prevent Zaphod from falling of a ledge.  An IR detector and IR-LED was used to scan in front of Zaphod to see if there is a sudden drop off in front of him.  The Sharp IR detectors were used to detect the IR that is reflected back from the IR-LED.  The IR-LED was pointing in the same direction as the Sharp can (the IR detector) and has a narrow been so that there is not much scattering.

Before doing this, one of the AD ports on the 68HC11 had to be multiplexed because 6 of the 8 were all ready being used by other detectors.  The multiplexer inputs were selected by with the 3 lower bits at address 0x6000 via the MMIO.

Testing was done to see what kind of reading would be received by the Sharp can in order to tell if there is a sudden drop off in front of Zaphod.  When Zaphod is on level ground the reading into the AD port comes back as 114.  If an opening occurs then the reading will drop.  The routine to control this feature takes a look at the reading coming back on the AD port and sees if the reading is below 114.  If so, it alerts the motor control routine by setting a flag.  The program used had Zaphod doing a three-point-turn, avoiding going off the ledge.

COLLISION DETECTION

So you may be thinking that the collision avoidance does not work correctly if we are trying to detect a collision.  Like all things in life, nothing is perfect.  Every once and a while Zaphod will get caught on something or hit something that the front sharp cans will not detect.  In order to let Zaphod still go on without someone coming to his rescue, a pseudo bumper was installed.  It is called a pseudo bumper because the collision detection exploits the fact that the RC car was designed with differential drive.  Differential drive cause the motor with the most frictional resistance to stop spinning if the other wheel is receiving very little resistance.  Consequently, whenever Zaphod gets stuck or hits an object it is guaranteed that one of the rear wheels will stop spinning.

Siemmen’s emitter/detectors are used to see if one of the rear wheels stops spinning.  The inner rim on the wheels is a shinny silver plastic.  It was lined with black electrical tape so that the Siemmen’s emitter/detector could see transition when it is pointed toward the wheel’s inner rim.  The Driving circuitry for the Siemmen’s is given in figure 3.

With all the circuitry in place the Siemmen’s emitter/detector creates a shaft encoder.  The shaft encoder produces a 5 volt pulse due to transition form silver to black on the wheels inner rim.  This signal can been directed into PA7, the pulse accumulator.

Figure 3

PA7 was multiplexed because both wheels had to have shaft encoders.  The most significant bit of the MMIO at 0x6000 was used as the select line for the PA7 multiplexer.

The pulse accumulator counts the number of pulses that it is receiving.  Thus by looking at its reading one can determine if the wheels are spinning or are stagnant.  I accomplished this by loading the pulse accumulator register with 0 and then waiting a set amount of time.  After the time has elapsed, the register is read.  If it is still zero then it can be inferred that the wheel is not spinning and the bump flag will be set to let other processes know that Zaphod is not moving.

Like the other routines that control Zaphod’s movement, the collision detection has him do a three point turn to get out of the collision.  Later, the shaft encoders can be used for automatic transmission.   Zaphod moves at different speeds on different surfaces.  With automatic transmission the shaft encoder can be used to have Zaphod move at a constant velocity on all surfaces.

CONCLUSION

During the course of the 1995 fall semester, myself and by lab partner, Anita, were able to get three working sensor and five different behaviors to work on Zaphod.  Program codes are includes in at the end of the report.  We also gained a betting sense of the procedure involved with design, building, testing, and debugging an autonomous agent.  Overall I feel that I have gained a tremendous amount of practical skills that will help me with my future endeavors as an Electrical Engineer.