# PUFF: The Autonomous Agent

For EEL 5666

Intelligent Machine Design Lab

Steve McKinnon

Instructor: Keith L. Doty

TA:  Scott Jantz

FALL 97

Table of Contents

Abstract………………………………………………………………………….	3

Executive Summary……………………………………………………………..	4

Introduction………………………………………………………………………	5

Integrated System……………………………………………………………….	6

Mobile Platform…………………………………………………………………	7

Actuation…………………………………………………………………………	8

Sensors…………………………………………………………………………..	8

IR………………………………………………………………………	8

Bump…………………………………………………………………..	9

Ultrasonic………………………………………………………………	9

CDS Photo Resistor……………………………………………………	13

Behaviors………………………………………………………………………..	13

Collision Avoidance…………………………………………………..	13

Bump Detection……………………………………………………….	14

Hiding/Searching………………………………………………………	15

Predator/Prey Detection……………………………………………….	15

Conclusion………………………………………………………………………	16

Appendix………………………………………………………………………..	17

Source Code…………………………………………………………..	17

Abstract

This document is a basic summary of a project completed for EEL 5666: Intelligent Machine Design Lab (IMDL).  The project required for this course was the design, testing, and implementation of an autonomous agent.  This agent must utilize four basic sensors to perform a variety of behaviors.  These requirements are left simple and vague, to promote creativity and allow undergraduates like myself to participate in this unique class.

Executive Summary

For the completion of this project an autonomous agent must be designed and implemented.  The agent must have four sensors, and one of these sensors must be of the student’s design.  Also, the agent must perform at least four different functions.

The project discussed in this document contains one basic robot platform with basic sensor suite.  Using this sensor suite the robot will be able to “sense” its environment.  However, two algorithms will be used to simulate two different agents.  One agent will be a predator who will seek out a prey.  On the other hand, there will be a prey who will avoid the predator agent.  After both algorithms have been designed and implemented, they will hopefully interact with each other in a similar relationship found in nature.

Introduction

This document contains the information obtained through the design and implementation of the autonomous agent to be developed for IMDL.  The scope of this paper is confined to the basic robot platform and the algorithms to be used.  The paper discusses the integrated system and the individual sensors used to perceive the agent’s environment.  This document also discusses the variety of behaviors that were performed with the basic sensor suite.  Finally, the paper closes with a summary of work accomplished and discusses the limitations of the work performed.  Future work on this project is also mentioned in the conclusion.

Integrated System

The goal of this project was to design an autonomous agent with a variety of functions and behaviors.  Specifically, this agent could simulate predator and prey behaviors depending on the arbitrator process imbed in the machine’s code.  Mekatronix’s Talrik Junior was the platform for this robot’s design.  A M68HC11 microprocessor is the heart of the TJ.  The TJ is also has two types of sensors, and the modified design in this project adds two additional sensors.  Using all of these sensors, the microprocessor is able to sense some characteristics about its environment and behavior accordingly.  The figure below shows the sensors as inputs to the microprocessor which uses this to perform the associated behaviors.

### Mobile Platform

The platform used is Mekatronix Talrik Jr shown below in Figure 2.  This robot has a very compact design making it agile and very maneuverable.  In fact, the wheels are placed in a manner allowing the agent to spin on its center.  The TJ has a single chip computer board (Novasoft MSCC11), and a battery supply located beneath its topside.   For this project the TJ platform was modified with two additional sensors.  The modified TJ platform is shown in Figure 3 below.

Figure 2b

Figure 3:  Modified TJ Platform

### Actuation

The robot is driven by two independent servo motors.   These servos act essentially as DC motors with feedback circuitry.  A 50Hz signal is used to drive the servos in a given direction.  Depending on the duty cycle of this signal the servos will drive the wheels at a given speed.  The servos are the only type of actuation in the system.  The software for the motor drivers can be found in the TJ application package from Novasoft.

## Sensors

IR Emitters and Detectors:

The TJ also detect infared light, or IR.  Two infared IR emitters are used to project 40kHz IR light.  Moreover, there are two IR detectors which can detect this projected light.  There are one of these detectors mounted on the right side of the robot and another sensor mounted on the left side.  In this arrangement, the TJ can detect which direction the light is coming from.  Furthermore, the IR detectors (Sharp GP1U58) are modified to give an analog voltage corresponding to the amount of IR received.  The nominal value for these IR detectors is 88 (decimal) on the A/D port and the saturation value is about 130.  The left sensor and right sensors are connected to the A/D converter on pins PE6 and PE7 respectively.  Software to modulated the IR emitters correctly can be found in the TJ applications package from Novasoft.

Using the analog output from the IR detectors the direction and intensity of incoming light can be measured.  If the TJ approaches an object at close range the IR from the emitters is reflected off the object and back to the IR detectors.  As a result, an object can be detected and avoided through arbitration of the motors.  One of the major drawback to this sensor is its occasional problems sensing its environment.  For example, if the object is of the dark in color less light will be reflected.  It is almost impossible to determine range without complex algorithms.  Moreover, the TJ has problems detecting some shapes such as skinny or round due to lack of reflected light.  It is important to leave the TJ in a non-complex environment for reliable interaction.

Bump Detectors:

The bump detectors are the simplest sensor located on the TJ.  There are three forward facing bump switches and one on the rear.  Each sensor is simple a switch that is either closed or open.  The forward looking bump switches are all interconnected with a pull down resistor and then connect to the PC5 of the microprocessor.  Therefore, if the switch becomes closed at any time the signal becomes high on the input pin.

A bumper is placed around the outside of the TJ over these bumper switches, so that collisions can be detected from other areas besides the switches themselves.  It is important to have a properly placed bumper, so that if a collision does occur the bumper returns to its original position after the TJ has separated itself from the object.  This sensor has the most priority over the other sensors in this project.  If I collision is detected by the switches the TJ immediately backs up before returning to its original function.

Ultrasonic Motion Detector:

This sensor is a modified version of a electronics kit provided from Jameco Electronics.  The part number for this motion detector kit is 125090 and can be purchased by calling the vendor at 1-800-831-4242.  The kit itself contains a matched pair of ceramic transducers which convert movement to electrical energy and vice versa.  Through some modification, the basic functionality inherent to these transducers will be used in a way for one robot to detect the direction and range of another robot.  Furthermore, this will allow them the robots to interact in a specific algorithm chosen.

JAMECO'S ULTRASONIC MOVEMENT DETECTOR:

The kit provided by Jameco provides two ultrasonic transducers advertised to detect motion from 4-7m away.  A crystal provides an operation frequency of 40kHz to the two transducers.  The kit also provides a variety of additional characteristics.  First, an LED provides visual evidence that movement has been sensed.  Second, there is a PCB-mounted switch which can be used to switch between an automatic reset about 0.3 seconds after the detector has been triggered or to stay latched on.  Finally, the unit provides pads which allow for additional circuits such as buzzers or lights.

The transmitter circuit consists of a crystal-locked cmos gate oscillator feeding a 40kHz square wave to a cmos driver.  This in turn drives the transmitter in anti-phase to get the maximum output.  The transmitter circuit is shown below in Figure 4.

The receiver circuit (Figure 5) takes the electrical output of the transducer and amplifies the signal over a transistor and operational amplifier.  Then, the output is converted to DC in a peak detector and taken to the non-inverting input of another operation amplifier.  The feedback circuit of this operation amplifier can be adjusted by a potentiometer.  This second op amp quickly adjusts to a steady high output if there is no change in the incoming signal.

Figure 5: Receiver Circuit

Sound waves reflected by objects arrive at the receiver in different phases.  Moreover, if the sound waves are in phase they create a larger signal whereas if the sound waves return out of phase they cancel each other out.  Due to this characteristic if an object moves towards or away from the receiver it cause the receiver signal to alternate cycles from high to low.  This is the change that triggers the device.  They steady high output of the second op amp is pulled down causing the cmos gates to switch.

KIT MODIFICATION AND INDENTED ALGORITHM:

The transmitter and receiver circuits will be separated for use with the autonomous agents.  Since, circuitry is common to both the transmitter and receiver some hardware modifications will be made.  The predator robot will carry two receiver circuits and will listen for the prey.  The receiver pair will be located on different sides of the robot, so that the direction of the signal can be quickly perceived by the agent.  The algorithm to sense direction is similar to the IR obstacle avoidance, except that the robot's arbitrator will turn the motors in the direction of the strongest signal, rather than avoiding this.  The predator will "listen" for the signal that the prey will transmit.

PERFORMANCE AND OBSERVATION OF SENSOR CHARACTERISTICS:

An experiment was used to determine some of the characteristics of this ultrasonic motion detector.  The Jameco kit provides a LED which lights up every time the device is triggered by movement, and provided an easy method of testing the limitations of the sensor.

IR sensors have a limitation when searching for a reflected signal.  For example, a stronger IR signal can be measured off a light surface than a dark surface.  However, these ultrasonic transducers do not have this same limitation.  Sound waves are not effected by the color of a reflecting surface.  However, they are effected by the shape.  A concave object will provide a stronger reflected signal than a flat object.  Moreover, a flat object will reflect a stronger signal than a convex object.

The receiver circuit's sensitivity can be adjusted by the potentiometer.  Varying the sensitivity of the device, a flat and concave object were used to determine the greatest distance that could be detected.  The following data was obtained,

RESISTANCE(OHMS)	DISTANCE OF	DISTANCE OF

FLAT OBJECT(FT) OF CONCAVE OBJECT(FT)

----------------------------------------------------------

100				4			6

2.5K				4.5			6.5

5K				5			7.25

7.5K				6			8

10K				7			9.5

This data shows one of the limitations of the sensor.  For if used for collision avoidance, a robot would not easily detect some objects due to shape.  Moreover, if this sensor was used to range object error should be expected.  The receiver circuit is very sensitive, and would result in greater range errors with greater distances.

This motion detector sensor is the most elaborate sensor found in this project.  Modifications must be made from the current design however.  Through implementation of this sensor with the robot a problem has been observed.  The A/D converter samples at a given rate which is much slower than oscillating signal given from the receiver circuitry.  The output waveform would probably be readily usable if a low pass filter was used on the current output waveform.  Despite this problem, the TJ can detect motion consistently.  Range and directions problems will hopefully be rendered at a later date.

CDS Photo Resistor:

The CDS photo resistor sensor is very simple.  The sensor itself is a simple voltage divider as shown in Figure 6.  The value of the resistor can be adjusted to give a variety of sensitivities to different types of light.  For example, a 1k and 100k resistor force the circuit to be sensitive to bright and dark light respectively.

The voltage over the CDS cell is taken to the A/D converter on the microprocessor.  There are three of these sensors placed on the robot, and they all vary in their direction.

## Behaviors

Collision Avoidance:

Collision Avoidance is accomplished with the IR system.  The IR detectors are connected to the analog port of the microprocessor.  When a tolerance is exceeded on one of the detectors the system’s arbitrator “senses” an object and sends the proper speed and direction outputs to the motors.  For example, if the left IR detector give a value above a certain tolerance the arbitrator tells the robot to turn right, or away from the object.

One of the biggest problems with this behavior is called the “Braitenburg Trap.”  This situation occurs when both the right and left detectors sense a value above threshold.  The system’s arbitrator wants to go both direction, so when implemented the robot just seems to shake back and forth.  To solve this problem the arbitrator looks to see when this situation happens and then tells the motors to back up for a set amount of time (see source code in ARBIT_SR).

Bump Detection:

This behavior depends on the bump switches located around the outside of the TJ.  If they are triggered the arbitrator simply backs up for a set amount of time.  In fact, this behavior has the highest priority in the system.

Hiding/Searching:

This behavior can either be seen as hiding or searching depending on if the algorithm being used is that of the prey or predator respectively.  In either case, the system uses the CDS cells to search for the darkest part of the room.  Once all three CDS sensors register a dark reading, then the robot knows it has reached its destination.  In the prey’s case, it now hides until a predator robot comes.  In contrast, when the predator robot reaches its destination it will poke around searching for the prey who should have left by then.

Predator/Prey Detection:

The ultrasonic motion receivers are used for this behavior.  The prey uses this sensor when it is hiding.  Once it detects a prey in its vicinity it begins to run away.  Similarly, the predator uses this sensor to detect the prey.  However, with a better output signal from the motion detector the predator should be able to chase the prey with this same signal.

Similar to the IR system, there are two receivers placed on opposite sides of the robot.  This should give the arbitrator some sense of direction.  However, due to the fact the receiver circuit is too sensitive or the A/D converter does not sample fast enough this sensor can only be used to detect motion.

Conclusion

The goals of this project have been achieved for the most part.  The final robot platform has a great sensor suite which has many possible functions.  However, the M68HC11E2 only has 2K EEPROM and 256 bytes of RAM, so the memory capacity is limited.  With a bigger memory, the entire sensor suite could be utilized to perform elaborate and more “intelligent” behaviors.  Moreover, modification still needs to be done on the ultrasonic sensor.  This sensor has some advantages over using IR for collision avoidance or object chasing.  If the output of this sensor was passed through a low pass filter the signal may actually be used for a more useful function.  Eventually, both the prey and predator algorithms will be completed, and both designs will be implemented together to hopefully simulate some behaviors found in nature.

Appendix

Source code in assembly:

*Steve McKinnon

*IMDL

*12/1/97

*shell v1.4 collison avoidance WITH BUMP DETECTION (FINAL)

********************************************************************

EEPROM		EQU	$B600

RAM		EQU	$0000

RAMEND		EQU	$01FF

REGS		EQU	$1000

OPTION		EQU	$1039

ADCTL		EQU	$1030

ADR1		EQU	$1031

ADR2		EQU	$1032

ADR3		EQU	$1033

ADR4		EQU	$1034

TMSK1		EQU	$1022

TMSK2		EQU	$1024

TFLG2		EQU	$1025

PACTL		EQU	$1026

PORTC		EQU	$1003

*	Constants

NSAMP		EQU	100

BIT0		EQU	%00000001

BIT1		EQU	%00000010

BIT6		EQU	%01000000	;LEFT SENSOR

BIT7		EQU	%10000000	;RIGHT SENSOR

INV6		EQU	%10111111

STSIZE		EQU	20

****** DATA ********

ORG	$00

PID	RMB	1	;PROCESS ID

SP1	RMB	2	;STACK POINTER FOR IR READ

SP2	RMB	2	;STACK POINTER FOR SONIC READ

SP3	RMB	2	;STACK POINTER FOR PHOTO READ

SP4	RMB	2	;STACK POINTER FOR ARBITRATOR

SP5	RMB	2	;STACK POINTER FOR MOTOR ROUTINE

STACK1	RMB	STSIZE

STACK2	RMB	STSIZE

STACK3	RMB	STSIZE

STACK4	RMB	STSIZE

STACK5	RMB	STSIZE

BUMP	RMB	1

IR_LEFT	RMB	1

IR_RIGHT RMB	1

SONAR_LEFT RMB	1

SONAR_RIGHT RMB	1

CDS_LEFT RMB	1

CDS_CENTER RMB	1

CDS_RIGHT RMB	1

MOTOR_LEFT RMB	2

MOTOR_RIGHT RMB	2

ACTUAL_LEFT RMB 2

ACTUAL_RIGHT RMB 2

_SERVOS_BUSY:

RMB       2

_SIGNAL_STATE:

RMB       2

_CURRENT_WIDTH:

RMB       2

_WIDTH:

RMB       4

_current_mode

rmb     1

****** INTERRUPT VECTORS *******

ORG	$FFFE

FDB	MAIN

ORG     $FFE4

fdb     _IRREAD

ORG	$FFE6

FDB	_SERVO_HAND

ORG	$FFF0

FDB	RTI_ISR

**********************************************************************8

ORG	$F800

MAIN	LDX	#REGS

*Turn on A/D System

LDAA	#%10000000		; Bit 7 (ADPU) of OPTION

STAA	OPTION-REGS,X

* Wait 100 ms for Pump Charge to Stabilize

LDAA	#40			; 2 E cycles

WAIT1	DECA				; 2 E cycles

BNE	WAIT1			; 3 E cycles

*

BCLR	PACTL,X BIT1	;SET INTERRUPT RATE FOR 8.19MSEC...

BSET	PACTL,X BIT0

BSET	TMSK2,X BIT6	;ENABLE RTI INTERRUPT

*

LDX	#STACK1+STSIZE-1	;INITIALIZE STACK POINTERS FOR PROCESSES

STX	SP1

LDX	#STACK2+STSIZE-1

STX	SP2

LDX	#STACK3+STSIZE-1

STX	SP3

LDX	#STACK4+STSIZE-1

STX	SP4

LDX	#STACK5+STSIZE-1

STX	SP5

*					;INITIALIZE ALL STACKS

LDS	SP5		;ASSUME PREVIOUS PID WAS 5

LDX	#MOTOR_SR

PSHX

LDY	#0

PSHY			;IY

PSHY			;IX

PSHY			;A AND B

LDAA	#BIT6		;CCR WITH X=1 AND I=0

PSHA			;CCR

STS	SP5

*

LDS	SP4		;ASSUME PREVIOUS PID WAS 4

LDX	#ARBIT_SR

PSHX

LDY	#0

PSHY			;IY

PSHY			;IX

PSHY			;A AND B

LDAA	#BIT6		;CCR WITH X=1 AND I=0

PSHA			;CCR

STS	SP4

*

LDS	SP3		;ASSUME PREVIOUS PID WAS 3

LDX	#PHOTO_SR

PSHX

LDY	#0

PSHY			;IY

PSHY			;IX

PSHY			;A AND B

LDAA	#BIT6		;CCR WITH X=1 AND I=0

PSHA			;CCR

STS	SP3

*

LDS	SP2		;ASSUME PREVIOUS PID WAS 2

LDX	#SONIC_SR

PSHX

LDY	#0

PSHY			;IY

PSHY			;IX

PSHY			;A AND B

LDAA	#BIT6		;CCR WITH X=1 AND I=0

PSHA			;CCR

STS	SP2

*

LDAA	#1

STAA	PID

LDS	SP1		;INITIALIZE CURRENT PROCESS AS 1

*

LDY	#3000

STY	ACTUAL_LEFT	;INITALIZE CURRENT MOTOR WIDTH

STY	ACTUAL_RIGHT

JSR	_INIT_SERVOS	;INTIALIZE SERVOS

JSR	_INIT_IR	;INTIALIZE IR EMITTERS

JSR	_IR_ON		;TURN ON IR

CLI			;TURN ON INTERRUPTS

JMP	IR_SR		;GO TO PROCESS 1

****************************************

************************************

RTI_ISR LDX	#REGS

BRCLR	TFLG2,X BIT6 ENDRTI	;IGNORE ILLEGAL INTERRUPT

BCLR	TFLG2,X INV6		;CLEAR FLAG

LDAA	PID			;FIND CURRENT PID

CMPA	#1

BEQ	SAVEP1

CMPA	#2

BEQ	SAVEP2

CMPA	#3

BEQ	SAVEP3

CMPA	#4

BEQ	SAVEP4

CMPA	#5

BEQ	SAVEP5

BRA	ENDRTI			;IGNORE ILLEGAL PID

SAVEP1	STS	SP1			;SET NEXT PROCESS

LDAA	#2

STAA	PID

LDS	SP2

BRA	ENDRTI

SAVEP2	STS	SP2

LDAA	#3

STAA	PID

LDS	SP3

BRA	ENDRTI

SAVEP3	STS	SP3

LDAA	#4

STAA	PID

LDS	SP4

BRA	ENDRTI

SAVEP4	STS	SP4

LDAA	#5

STAA	PID

LDS	SP5

BRA	ENDRTI

SAVEP5	STS	SP5

LDAA	#1

STAA	PID

LDS	SP1

BRA	ENDRTI

ENDRTI	RTI

************************************

****************************************

IR_SR	BRA	IR_SR

LDX	#REGS

LOOP1	LDAA	#%00010110	;SAMPL

STAA	ADCTL-REGS,X

LDAA	#6		 ; 2 E cycles

WAITR	DECA			 ; 2 E cycles

BNE	WAITR		 ; 3 E cycles

LDAA	ADR3

STAA	IR_LEFT

LDAA	#%00010111	;SAMPR

STAA	ADCTL-REGS,X

LDAA	#6		 ; 2 E cycles

WAITL	DECA			 ; 2 E cycles

BNE	WAITL		 ; 3 E cycles

LDAA	ADR4

STAA	IR_RIGHT

BRA	LOOP1

****************************************

SONIC_SR LDX	#REGS

LOOP2	SEI

LDAA	#%00010001	;SAMPL

STAA	ADCTL-REGS,X

LDAA	#6		 ; 2 E cycles

WAITR	DECA			 ; 2 E cycles

BNE	WAITR		 ; 3 E cycles

LDAA	ADR1

STAA	SONAR_LEFT

LDAA	#%00010101	;SAMPR

STAA	ADCTL-REGS,X

LDAA	#6		 ; 2 E cycles

WAITL	DECA			 ; 2 E cycles

BNE	WAITL		 ; 3 E cycles

LDAA	ADR2

STAA	SONAR_RIGHT

CLI

BRA	LOOP2

***************************************

PHOTO_SR LDX	#REGS

LOOP3	BRA	LOOP3

***************************************

ARBIT_SR LDX	#REGS

JSR	LOOP4

BRA	ARBIT_SR

LOOP4	LDAA	PORTC

ANDA	#%00100000	;CHECK PC5

BNE	GOBACK		;BUMP DETECTION IF NOT ZERO

LDAA	CDS_RIGHT

CMPA	#253

BLT	STOPQU		;IF CDSRIGHT DARK CHECK LEFT

BRA	SKIP		;ELSE SKIP

STOPQU	LDAA	CDS_LEFT

CMPA	#253

BLT	MSTSTOP		;IF CDSLEFT DARK TOO THEN STOP

BRA	SKIP		;ELSE SKIP

MSTSTOP	JSR	STOP

RTS

SKIP	LDAA	IR_LEFT		;COLLISION AVOIDANCE

LDAB	IR_RIGHT

CMPB	#98

BLT	RLOW

CMPA	#98

BLT	GOLEFT

LDAA	IR_LEFT

LDAB	IR_RIGHT

CMPB	#250

BGT	RHIGH

CMPA	#250

BGT	GOLEFT

*

LDAA	IR_LEFT

CMPA	IR_RIGHT

BLT	GOLEFT

BRA	GORIGHT

*

RHIGH	CMPA	#250

BGT	GOBACK

BRA	GORIGHT

*

RLOW	CMPA	#98

BLT	FORWARD

BRA	GORIGHT

GOBACK	LDY	#3500

STY	MOTOR_LEFT

LDY	#2400

STY	MOTOR_RIGHT

JSR	DELAY

RTS

GOLEFT	LDY	#3500		;WORKS

STY	MOTOR_LEFT

LDY	#3500

STY	MOTOR_RIGHT

RTS

LILLEFT	LDAA	CDS_LEFT

SUBA	CDS_RIGHT

CMPA	#150

*	BPL	STR

LDY	#3000		;WORKS

STY	MOTOR_LEFT

LDY	#3500

STY	MOTOR_RIGHT

RTS

GORIGHT	LDY	#2300

STY	MOTOR_LEFT

LDY	#2400

STY	MOTOR_RIGHT

RTS

LILRIGHT LDAA	CDS_RIGHT

SUBA	CDS_LEFT

CMPA	#100

*	BLT	STR

LDY	#2300

STY	MOTOR_LEFT

LDY	#3000

STY	MOTOR_RIGHT

RTS

FORWARD	LDAA	CDS_CENTER	;NO COLLSION, SO FIND DARKNESS

CMPA	#250

BLT	STR		;GOSTRAIGHT IF DARK AHEAD

LDAA	CDS_LEFT

SUBA	CDS_RIGHT

BLT	LILRIGHT	;LEAN RIGHT

BRA	LILLEFT		;LEAN LEFT

STR	LDY	#2300		;STRAIGHT MOTOR VALUES

STY	MOTOR_LEFT

LDY	#3500

STY	MOTOR_RIGHT

RTS

STOP	LDY	#3000		;STOP ROUTINE

STY	MOTOR_LEFT

STY	MOTOR_RIGHT

WAIT4	LDAA	SONAR_LEFT	;WAIT FOR PREDATOR

CMPA	#180

BHI	DONE4		;DETECT WITH LEFT...RUN!

LDAA	SONAR_RIGHT

CMPA	#180

BHI	DONE4		;DETECT WITH RIGHT...RUN!

BRA	WAIT4

DONE4	LDY	#3500		;BACK UP OUT OF HIDING AND GO...

STY	MOTOR_LEFT

LDY	#2800

STY	MOTOR_RIGHT

JSR	DELAY		;WAIT TO FINISH BACKING UP...

RTS

***************************************

MOTOR_SR

LDD	ACTUAL_LEFT

CPD	MOTOR_LEFT

BLT	INCL

BEQ	FIXLEFT

LDD	ACTUAL_LEFT

SUBD	#50

STD	ACTUAL_LEFT

BRA	FIXLEFT

INCL	LDD	ACTUAL_LEFT

ADDD	#50

STD	ACTUAL_LEFT

FIXLEFT	LDD	#0

LDX	ACTUAL_LEFT

PSHX

JSR	_SERVO		;LEFT

PULX

*

LDD	ACTUAL_RIGHT

CPD	MOTOR_RIGHT

BLT	INCR

BEQ	FIXR		;DO NOTHING

LDD	ACTUAL_RIGHT

SUBD	#50

STD	ACTUAL_RIGHT

BRA	FIXR

INCR	LDD	ACTUAL_RIGHT

ADDD	#50

STD	ACTUAL_RIGHT

FIXR	LDD	#1

LDX	ACTUAL_RIGHT

PSHX

JSR	_SERVO		;RIGHT

PULX

BRA	MOTOR_SR

***************************************

DELAY	LDX	#5		;APPROX = 2 SEC

OUTER	LDY	#10000		;4 CYCLES

INNER	CMPA	$00		;3

DEY			;4

BNE	INNER		;3

DEX			;3

BNE	OUTER		;3

RTS

***************************************Source Code for IR modulation and to drive the servos can be found in the TJ applications software from Novasoft.