University of Florida

Department of Electrical and Computer Engineering

EEL 5666

Intelligent Machines Design Laboratory

## ‘Thing’

The Robotic Hand in a Box

# Date:	4/25/01

Student Name:		Chung Chov

TA:	Scott Nortman

Rand Chandler

Instructor:	A. A. Arroyo

Table of Contents

Abstract……………………………………………………………………..pg.03.

Introduction…………………………………………………………………pg.03.

Integrated Systems………………………………………………………….pg.04.

Mobile Platform…………………………………………………………….pg.05.

Actuation……………………………………………………………………pg.05.

Sensors…………………………..………………………………………….pg.06.

Behaviors…………………………………………………………………...pg.07.

Experimental Layout and Results…………………………………………..pg.07.

Conclusion………………………………………………………………….pg.08.

Documentation……………………………………………………………..pg.08.

Appendices…………………………………………………………………pg.08.

# Abstract

‘Thing’ is an autonomous robot that roams around until it finds white blocks to pick up. It will appear to be nothing more than just a box on wheels. Once it finds a block, it will determine if it is black or white. If it is white, a door will open and a hand will come out to pick the object and go back in. If the block is black, it will move them out of the way or it will try and place a white block next to it if it has one. ‘Thing’ is also able to determine if the object is small enough to pick up. ‘Thing’ also has collision avoidance for large objects that it cannot pick up. The wheels will be controlled by hacked servos. The arm and hand will be controlled by servos. The IR sensors are used for the collision avoidance. The block detector will be a combination of IRs and photoresistors. The sensors, motors, and behaviors are all controlled by a PIC Chip microcontroller.

# Introduction

The main purpose of any robot is to make life easier for humans. In the home, robots can clean, take care of the lawn, or provide security for the home. Since I am a messy person and I have a tendency to leave things lying around, I wanted a robot to pick things up for me and put them in place.

# Integrated System

The microcontroller for ‘Thing’ is the Microchip PIC16F877. The PIC chip will have input coming in from the IR sensors, bump switches, and photoresistors. The PIC chip will determine which sensors are used. For instance, the photoresistor will not need to be on until ‘Thing’ has found an object and needs to determine what color it is. The PIC chip controls when the servos (that moves the hand and arm around) are activated. ‘Thing’ will roam in its environment avoid objects that are bigger than the objects that it can pick up. The data from the IRs are used to determine the whether the object is small enough. Once it has found the object it can pick up, photoresistors determine wither or not the object is black or white. If it has found a white object a door will open up and hand come out to pick it up and retract with the door closing behind it. Bump switches surround the robot since the IRs are only on the front of the robot. Because of the ease of the PIC Chip, the sensors where directed connected to the PIC Chip with the exception of the three diodes need for the clock of the IRs.

Microchip PIC16F877

# Mobile Platform

‘Thing’ will be made out of balsa wood cut out from the ‘T-tech’ machine in lab. Since ‘Thing’ is a hand in a box, the hand had to be designed first. To do this, prototype was created out of Popsicle sticks. This allowed for any mechanical problems to be worked out. Later the hand was drawn in AutoCAD and cut out using the T-Tech machine. Then the ‘box’ was designed around the hand and arm. This was cut out by hand since it was large enough to do so. Two problems occurred when the hand and arm were mounted on to the body. One problem was that the arm would not extend or retract with out hitting the front of the robot. This was solved by using a rubber band to hold the arm above the robot while it was retract or extended by the servo. However the rubber band only last a couple of stretches. A permanent fix to this would be a spring. Another problem was that when fully extended the robot tipped over. To solve this, the battery pack was placed at the back of the robot to counter balance the arm. Another problem that arose was when ‘Thing’ actually tried to pick something up. It did not have enough gripping force to hold onto anything. This was because the hand had fishing line to contract the joints of the finger. A partial fix to this was replacing the fishing line (which had some elasticity) with wires.

# Actuation

‘Thing’ has two servos and two hacked servos. The hacked servos provide ‘Thing’ with mobility. One servo is connected to control wires attached to the fingers in the hand. When the servo turns it will shorten the control wires causing the fingers of the hand to curl, thus grab. The other servo is attached to the arm at the where it is connected to the base of the robot. This servo extends and retracts the arm and hand. The PIC Chip uses one timer to control all the servos.

# Sensors

The bump sensors are momentary switches connected to ground on pins on the PIC Chip that have internal pull up resistors. The IRs used on ‘Thing’ are the SHARP GP2D02. This is an all-in-one unit. It contains both the emitter and detector. Its range is 80 cm to 6 cm and about 100mm wide (figure 1). The pins for it are power, ground, clock, and data. The clock is pulled low for 70ms and then pulsed to read the 8 bit serial data. Two of these IRs will be used for collision avoidance. A third IR is used in the ‘block detector’ to find objects that are small enough to pick. By placing the collision avoidance IRs higher that IR in the block detector, small objects lower than the collision avoidance IRs will be missed but still detected by the block detector. The block detector is main sensor for ‘Thing.’ It is a combination of SHARP GP2D02, photoresistor, and two incandescent lamps used to detect white or black blocks. The photoresistor is in series with a 1k resistor. The output from that is attached the analog input of the PIC Chip. At approximately 8cm away, the photoresistors can read the light reflecting off of the object. When the object is black, the light is about half the light read in the room. Because of this, it may be possible to read the light in any given room and still be able to detect if the block is white or not (e.g. self calibrating sensor).

# Behavior

The robots main behavior is to avoid obstacles. While it is avoid obstacles, it searches for objects small enough to pick up. Once it finds an object it will go and pick it up off the ground if it is white. If the object is black it will try and move the object away. If however ‘Thing’ already has picked up an object it will place it next to the black object.

# Experimental Layout and Results

To determine what kind of data the block detector would send, black and white objects were placed at varies distances and in different lighting environments.

| Lighting | Normal |  | Dark |  | Bright |  |
| --- | --- | --- | --- | --- | --- | --- |
| Distance | Black | White | Black | White | Black | White |
| No object in front of sensor | .27 | .27 | .19 | .19 | .72 | .72 |
| 30cm | .22 | .25 | .19 | .18 | .63 | .75 |
| 25cm | .21 | .24 | .18 | .16 | .57 | .75 |
| 20cm | .19 | .22 | .17 | .15 | .58 | .76 |
| 15cm | .20 | .21 | .13 | .10 | .50 | .75 |
| 10cm | .13 | .22 | .10 | .14 | .43 | .70 |
| 8cm | .14 | .25 | .08 | .14 | .37 | .67 |
| 5cm | .10 | .31 | .07 | .23 | .31 | .66 |
| 1cm | .10 | 1.18 | .09 | 1.04 | .15 | 1.77 |

* Measurements in volts

80 cm -------------------------            					Fig 1

30 cm--------------------------

6 cm-----------------------------

|-----100mm---|

# Conclusion

I found that it is one thing to say that your robot will do this and that and another for it to do so. Much of what my robot does was dictated to me by my robot. Instead on it doing this, it actually does that. Since I was using a different microprocessor then the most the class, I wrote most the code by myself with the help of Joshua Philips since had experience with this processor. I glad I used this chip since it provided something new for both the class and myself. One of the main problems I had with software was trying to have it actively search for objects instead of randomly running into them. I believe can fix this with time.

# Documentation

Fred Martin, The 6.270 Robot Builder’s Guide 2nd edition, MIT Media Lab, Cambridge, MA, 1992.

IMDL class: Instruction from Dr. Arroyo, Dr. Schwartz, Rand Chandler, and Scott Nortman

# Appendices

***************************************8chung.asm**********************************

LIST	P=PIC16F877	;

include "p16f877.inc"	;

include	"def_equ.inc"	;	personal defines and equates

;*******************************************************************************

org	0x00		;	Set RESET vector

goto	INIT		;	to beginning of program.

org	0x04		;	set INTERRUPT vector to

goto	INTRUPT		;	beginning of INTERRUPT service routine

org	0x05		;	start of program

INIT	bcf	STATUS,RP0	;	select bank 0

bcf	STATUS,RP1	;	select bank 0

include	"init01.inc"	;	all reg and port settings in this file

;*******************************************************************************

;	MAIN PROGRAM

;*******************************************************************************

bsf	IRFCLK		;

bsf	IRLCLK		;

bsf	IRRCLK		;

clrf	IRTEST		;

movlw	0x06		;

clrf	WHITE		;

bsf	IRR		;

bcf	IRF		;

movlw	0x00		;

movwf	IRLDATA		;

movwf	IRRDATA		;

movwf	IRFDATA		;

movlw	HANDOP		;

movwf	HANDPOS		;

movlw	RETRACT		;

movwf	ARMPOS		;

movlw	PWM2		; initialize to move forward

movwf	SPEEDL		;

movlw	PWM1		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	FORWARD		;

call 	WAIT		; wait 5 secs

call	CALIB		;

bsf	INTCON,GIE	; enable unmasked interrupts

;-----------------------------------------------------------------------------------

;

;	LEFT MOTOR (FORWARD = SPEEDL  = 14 / SPEEDL = 0A)

;	RIGHT MOTOR (FORWARD = SPEEDR = 0A / SPEEDR = 14)

;

;-----------------------------------------------------------------------------------

HERE	btfss	STOP		;

goto	FOR_C		;

goto	STOP_R		;

FOR_C	btfss	FORWARD		;

goto	REV_C		;

goto	FORW_R		;

REV_C	btfss	REVERSE		;

goto	LT_C		;

goto	REV_R		;

LT_C	btfss	LTURN		;

goto	RT_C		;

goto	LT_R		;

RT_C	btfss	RTURN		;

goto	ERR		;

goto	RT_R		;

ERR	movlw	0x00		;

movwf	SPEEDR		;

movwf	SPEEDL		;

clrf	DIRECT		;

bsf	STOP		;

goto	HERE		;

;--------------------------------------------------------------------------------------

STOP_R	call	WAIT		;

call	CHCK		;

btfsc	WORB,0x00	; 1 = white / 0 = black

goto	BIB		;

goto	BIW		;

BIB	movlw	HANDCL		;

movwf	HANDPOS		;

call	WAIT		;

movlw	EXTEND		;

movwf	ARMPOS		;

call	WAIT		;

movlw	HANDOP		;

movwf	HANDPOS		;

call	WAIT		;

movlw	RETRACT		;

movwf	ARMPOS		;

movlw	PWM1		; if ball is black go in reverse

movlw	SPEEDL		;

movlw	PWM2		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	REVERSE		;

goto	HERE		;

BIW	movlw	EXTEND		;

movwf	ARMPOS		;

call	WAIT

movlw	HANDCL		; if ball is white make right turn

movwf	HANDPOS		;

call	WAIT		;

movlw	RETRACT		;

movwf	ARMPOS		;

movlw	PWM2		;

movwf	SPEEDL		;

movlw	PWM1		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	RTURN		;

goto	HERE		;

;--------------------------------------------------------------------------------------

FORW_R	btfss	FBUMP		;

goto	BUMPEDF		;

btfss	LBUMP		;

goto	BUMPEDL		;

btfss	RBUMP		;

goto	BUMPEDR		;

btfss	BBUMP		;

goto	BUMPEDB		;

movlw	0x6e		; 9inches

subwf	IRLDATA, 0	;

btfsc	STATUS, 0x00	;

goto	FLNEAR		; IRLDATA - 8C > 0

goto	FLFAR		; IRLDATA - 8C < 0

FLNEAR	movlw	0x8c		; to get here the Left IR sensed an object

subwf	IRRDATA, 0	;

btfsc	STATUS, 0x00	;

goto	BOTH_IR		; IRRDATA - 8C > 0

goto	OBJECTL		; IRRDATA - 8C < 0

FLFAR	movlw	0x8c		; to get here the Left IR did not sense an object

subwf	IRRDATA, 0	;

btfsc	STATUS, 0x00	;

goto	OBJECTR		; IRRDATA - 8C > 0

goto	NOOBJ		; IRRDATA - 8C < 0

BOTH_IR	movlw	PWM1		;

movwf	SPEEDL		;

movlw	PWM2		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	REVERSE		;

goto	HERE		;

OBJECTL	movlw	PWM2		;

movwf	SPEEDL		;

movlw	PWM2		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	RTURN		;

goto	HERE		;

OBJECTR	movlw	PWM1		;

movwf	SPEEDL		;

movlw	PWM1		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	LTURN		;

goto	HERE		;

NOOBJ	movlw	d8cm		;

subwf	IRFDATA, 0	;

btfsc	STATUS, 0x00	;

goto	BALL		; IRFDATA - d8cm > 0

goto	NOBALL		; IRFDATA - d8cm < 0

BALL	clrf	SPEEDR		;

clrf	SPEEDL		;

clrf	DIRECT		;

bsf	STOP		;

goto	HERE		;

NOBALL	movlw	PWM2		;

movwf	SPEEDL		;

movlw	PWM1		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	FORWARD		;

goto	HERE		;

;--------------------------------------------------------------------------------------

REV_R	movf	TMR0,0x00	;

movwf	DELAYT		;

bcf	STATUS,  0x00	;

rrf	DELAYT, 1	;

bcf	STATUS,  0x00	;

rrf	DELAYT, 0	;

movwf	TEMP1		;

movwf	TEMP2		;

movwf	TEMP3		;

DELAYB	btfss	BBUMP		;

goto	BUMPEDB		;

btfss	LBUMP		;

goto	BUMPEDL		;

btfss	RBUMP		;

goto	BUMPEDR		;

btfss	FBUMP		;

goto	BUMPEDF		;

decfsz	TEMP3,0x01	;

goto	DELAYB 		;

movwf	TEMP3		;

decfsz	TEMP2,0x01	;

goto	DELAYB		;

movwf	TEMP2		;

decfsz	TEMP1,0x01	;

goto	DELAYB		;

movf	TMR0,0x00	; get a random # from TIMER 0

sublw	0x7F		;

btfss	STATUS,0x00	;

goto	RP_RL		;

goto	RP_RR		;

RP_RL	movlw	PWM1		;

movwf	SPEEDL		;

movlw	PWM1		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	LTURN		;

goto	HERE		;

RP_RR	movlw	PWM2		;

movwf	SPEEDL		;

movlw	PWM2		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	RTURN		;

goto	HERE		;

;--------------------------------------------------------------------------------------

LT_R	movf	TMR0,0x00	;

movwf	DELAYT		;

bcf	STATUS,  0x00	;

rrf	DELAYT, 1	;

bcf	STATUS,  0x00	;

rrf	DELAYT, 0	;

movwf	TEMP1		;

movwf	TEMP2		;

movwf	TEMP3		;

DELAYL	btfss	RBUMP		;

goto	BUMPEDR		;

btfss	FBUMP		;

goto	BUMPEDF		;

btfss	BBUMP		;

goto	BUMPEDB		;

btfss	LBUMP		;

goto	BUMPEDL		;

decfsz	TEMP3,0x01	;

goto	DELAYL 		;

movwf	TEMP3		;

decfsz	TEMP2,0x01	;

goto	DELAYL		;

movwf	TEMP2		;

decfsz	TEMP1,0x01	;

goto	DELAYL		;

movlw	PWM2		;

movwf	SPEEDL		;

movlw	PWM1		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	FORWARD		;

goto	HERE		;

;--------------------------------------------------------------------------------------

RT_R	movf	TMR0,0x00	;

movwf	DELAYT		;

bcf	STATUS,  0x00	;

rrf	DELAYT, 1	;

bcf	STATUS,  0x00	;

rrf	DELAYT, 0	;

movwf	TEMP1		;

movwf	TEMP2		;

movwf	TEMP3		;

DELAYR	btfss	LBUMP		;

goto	BUMPEDL		;

btfss	FBUMP		;

goto	BUMPEDF		;

btfss	BBUMP		;

goto	BUMPEDB		;

btfss	RBUMP		;

goto	BUMPEDR		;

decfsz	TEMP3,0x01	;

goto	DELAYR 		;

movwf	TEMP3		;

decfsz	TEMP2,0x01	;

goto	DELAYR		;

movwf	TEMP2		;

decfsz	TEMP1,0x01	;

goto	DELAYR		;

movlw	PWM2		;

movwf	SPEEDL		;

movlw	PWM1		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	FORWARD		;

goto	HERE		;

;--------------------------------------------------------------------------------------

;	BUMP ROUNTINES

;--------------------------------------------------------------------------------------

BUMPEDR	movlw	PWM2		; turn right slightly

movwf	SPEEDL		;

movlw	PWM2		;

movwf	SPEEDR		;

movlw	0x41		;

movwf	DELAYT		;

call	DELAY2		;

movlw	PWM1		; go reverse

movwf	SPEEDL		;

movlw	PWM2		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	REVERSE		;

goto	HERE		;

BUMPEDL	movlw	PWM1		; turn left slightly

movwf	SPEEDL		;

movlw	PWM1		;

movwf	SPEEDR		;

movlw	0x41		;

movwf	DELAYT		;

call	DELAY2		;

movlw	PWM1		; go reverse

movwf	SPEEDL		;

movlw	PWM2		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	REVERSE		;

goto	HERE		;

BUMPEDF	movlw	PWM1		; go reverse

movwf	SPEEDL		;

movlw	PWM2		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	REVERSE		;

goto	HERE		;

BUMPEDB	movlw	PWM2		; go forward

movwf	SPEEDL		;

movlw	PWM1		;

movwf	SPEEDR		;

clrf	DIRECT		;

bsf	FORWARD		;

goto	HERE		;

;-------------------------------------------------------------------------------

;	INTERRUPT ROUNTINES

;-------------------------------------------------------------------------------

include	"intrupt.inc"	; interrupt rountine

;-------------------------------------------------------------------------------

;	SUBROUTINES

;-------------------------------------------------------------------------------

include	"subrtn.inc"	; subroutines

;-------------------------------------------------------------------------------

end;

***************************************def_equ.inc*********************************

;*******************************************************************************

;

;	LABELS

;

;*******************************************************************************

#define IRLDAT	PORTB,0x07	;1

#define IRFDAT	PORTB,0x06	;1

#define IRRDAT	PORTB,0x05	;1

#define FBUMP	PORTB,0x03	;1

#define LBUMP	PORTB,0x02	;1

#define BBUMP	PORTB,0x01	;1

#define	RBUMP	PORTB,0x00	;1

#define	IRLCLK	PORTD,0x07	;0

#define	IRFCLK	PORTD,0x06	;0

#define	IRRCLK	PORTD,0x05	;0

#define	EYEL	PORTA,0x00	;

#define	EYEM	PORTA,0x01	;

#define	EYER	PORTA,0x02	;

#define	FOUND	LOOK,0x00	;

#define	SERVOL	PORTC,0x04	;

#define	SERVOR	PORTC,0x05	;

#define	HAND	PORTC,0x06	;

#define	ARM	PORTC,0x07

#define STOP	DIRECT, 0x00	; 0000 0001

#define FORWARD	DIRECT, 0x01	; 0000 0010

#define REVERSE	DIRECT, 0x02	; 0000 0100

#define LTURN	DIRECT, 0x03	; 0000 1000

#define RTURN 	DIRECT, 0x04	; 0001 0000

#define	SYNC5	DIRECT, 0x05	; 0010 0000

#define	SYNC6	DIRECT, 0x06	; 0100 0000

#define NEWSPD	DIRECT, 0x07	; 1000 0000

#define LWHEEL	SPDCTRL, 0x00	; 0000 0001

#define	RWHEEL	SPDCTRL, 0x01	; 0000 0010

#define HANDCTL	SPDCTRL, 0x02	; 0000 0100

#define	ARMCTL	SPDCTRL, 0x03	; 0000 1000

#define	IRREAD	IRTEST, 0x00	; 0000 0001

#define IRR	IRTEST, 0x01	; 0000 0010

#define	IRF	IRTEST, 0x02	; 0000 0100

#define IRL	IRTEST, 0x03	; 0000 1000

;------------------------------------timer 1

#define PWM2	0x0a		; 1.022ms at 1:2 prescalar -100%

#define PWM125	0x0d		; -50%

#define PWM15	0x0f		; 1.533ms at 1:2 prescalar 0%

#define	PWM175	0x11		; 50%

#define PWM1	0x14		; 2.044ms at 1:2 prescalar 100%

#define PWM18	0xb0		; 17.9872ms at 1:2 prescalar

#define PWM185	0xb5		; 18.4982ms at 1:2 prescalar

#define PWM19	0xba		; 19.0092ms at 1:2 prescalar

#define	PWM20	0xc4		; 20.0312ms

;------------------------------------DISTANCE DEFINATION

#define	d8cm	0xe6		; 8cm

#define	VFAR	0x8c		; 30cm

#define	FAR	0x92

#define	NEAR	0xa5

#define	VNEAR	0xb9

#define	VVNEAR	0xc5

#define	HANDOP	0x0a		;

#define	HANDCL	0x14		;

#define	EXTEND	0x05		;

#define	RETRACT	0x19		;

COUNT	equ	0x24		;

ARMPOS	equ	0x28		;

ARMCNT	equ	0x29		;

HANDCNT	equ	0x2a		;

HANDPOS	equ	0x2b		;

TEMP1	equ	0x2c		;

TEMP2	equ	0x2d		;

TEMP3	equ	0x2e		;

TEMP4	equ	0x2f		;

RWT	equ	0x30		;

LWTH	equ	0x31		;

LWTL	equ	0x32		;

TM2CNT	equ	0x33		;

IRRDATA equ	0x34		;

IRLDATA equ	0x35		;

IRFDATA	equ	0x36		;

IRTEST	equ	0x37		;

DIRECT	equ	0x38		;

SPDCTRL	equ	0x39		;

SPEEDL	equ	0x3a		;

SPEEDR	equ	0x3b		;

PERIOD	equ	0x3c		;

SPDTL	equ	0x3d		;

SPDTR	equ	0x3e		;

LOOKR	equ	0x3f		;

LOOKLT	equ	0x40		;

LOOKMT	equ	0x41		;

LOOKRT	equ	0x42		;

WHITE	equ	0x43		;

WORB	equ	0x44		;

LOOK	equ	0x45		;

DELAYT	equ	0x46

TABLE	equ	0x50		;

ENDTBL	equ	0x68		;

W_TEMP	equ	0x70		; temporary register for w available in all banks

S_TEMP	equ	0x71		; temp reg for the status reg (all banks)

***************************************init01.inc**************************************

;*******************************************************************************

;

;	INTIALIZE REGISTERS / PORTS / INTERUPTS

;

;*******************************************************************************

;*******************************************************************************

;

;	INTERRUPT CONTROL REGISTER

;

;	initialize to:

;

;	GLOBAL INTERRUPT ENABLE		: DISABLED	0

;	PERIPHERAL INTERRUPT ENABLE	: ENABLED	1

;	TIMER0 OVERFLOW INTERRUPT	: ENABLED	1

;	EXTERNAL INTERRUPT PIN		: DISABLED	0

;	PORTB INTERRUPT ON CHANGE	: DISABLED	0

;	LOWER 3 BITS ARE FLAGS 		: DONT CARE	xxx

;

;*******************************************************************************

movlw	0x60		;	initialize interrupt control reister

movwf	INTCON		;

;*******************************************************************************

;*******************************************************************************

;

;	OPTION REGISTER

;

;	initialize to:

;

;	PORTB INTERNAL PULLUP RESISTORS	: ENABLED	0

;	EXTERNAL INTERRUPT EDGE SELECT	: RISING EDGE	1

;	TIMER0 CLOCK SOURCE SELECT	: Fosc/4	0

;	TIMER0 SOURCE EDGE SELECT	: low to high	0

;	PRESCALER ASSIGNMENT		: TIMER0	0

;	LOWER 3 BITS ARE PRESCALER	: 1/2		000

;

;*******************************************************************************

bsf	STATUS,RP0	;	select bank 1

bcf	STATUS,RP1	;	select bank 1

movlw	0x40		;

movwf	OPTION_REG	;	initialize option register

;*******************************************************************************

;

;	PERIPHERAL INTERRUPT REGISTER 1

;

;	initialize to:

;

;	PARALLEL SLAVE PORT R/W INTERRUPT ENABLE	: DISABLED	0

;	A/D CONVERTER INTERRUPT ENABLE			: ENABLED	1

;	USART RECIEVE INTERRUPT ENABLE			: DISABLED	0

;	USART TRANSMIT INTERRUPT ENABLE			: DISABLED	0

;	SYNCHRONOUS SERIAL PORT INTERRUPT ENABLE	: DISABLED	0

;	CAPTURE/COMPARE 1 INTERRUPT ENABLE		: DISABLED	0

;	TIMER2 TO PR2 MATCH INTERRUPT			: ENABLED	1

;	TIMER1 OVERFLOW INTERRUPT ENABLE		: ENABLED	1

;

;*******************************************************************************

movlw	0x43	;

movwf	PIE1	; initialize PIE1 register

;*******************************************************************************

;

;	PERIPHERAL INTERRUPT REGISTER 2

;

;	initialize to:

;

;	BIT 7-5					: ALWAYS 000

;	EEPROM WRITE INTERRUPT ENABLE		: DISABLED 0

;	BUS COLLISION INTERRUPT ENABLE		: DISABLED 0

;	BIT 2-1					: ALWAYS 00

;	CAPTURE COMPARE 2 INTERRUPT ENABLE	: DISABLED 0

;

;*******************************************************************************

movlw	0x00;

movwf	PIE2; initialize PIE2 register

;*******************************************************************************

;

;	PORT A (6 bits wide)

;

;	For all ports data direction registers (TRISx) a 1 makes the pin an input

;

;	Analog input 0 thru 4 are on this port along with

;	TIMER0's external clock input pin

;	Assign analog pins here or in port e

;	All other pins are inputs

;

;*******************************************************************************

movlw	0xff		;

movwf	TRISA		; data direction register for PORTA

;*******************************************************************************

;

;	PORT B (8 bits wide)

;

;	General purpose in-out.

;	All pins will be used as inputs.

;

;*******************************************************************************

movlw	0xff		;1111 1111

movwf	TRISB		; data direction register for PORTB

;*******************************************************************************

;

;	PORT C (8 bits wide)

;

;	General purpose in-out, PWM output, SPI, USART, CAPTURE 1&2

;

;

;*******************************************************************************

movlw	0x0f		; 1110 0000

movwf	TRISC		; data direction register for PORTC

;*******************************************************************************

;

;	PORT D (8 bits wide)

;

;	Port D can be used as a parallel slave port or general in-out

;

;*******************************************************************************

movlw	0x00		; 0000 0000

movwf	TRISD		; data direction register for PORTD

;*******************************************************************************

;

;	PORT E (3 bits wide)

;

;	Port E can be used as general purpose in-out, Analog input 7 thru 5,

;	or as the control bits for parallel slave port mode.

;

;*******************************************************************************

movlw	0x0F		;

movwf	TRISE		; data direction register for PORTE

;*******************************************************************************

;

;	ANALOG-TO-DIGITAL REGISTER 1

;

;	This register selects the port configurations for analog or digital

;	input and selects the values for Vref+

;	This register also right/left justifies the A/D result register

;	The result is 10 bits wide in a 16 bit wide register.

;

;	The pins will be used as follows:

;

;	PORT E A/D pins = Digital I/O

;	PORT A	PIN0	= Digital I/O

;		PIN1	= Digital I/O

;		PIN2	= Digital I/O

;		PIN3	= Digital I/O

;		PIN4	= Digital I/O

;		PIN5	= Digital I/O

;*******************************************************************************

movlw	0x00		;

movwf	ADCON1		; config A/D

;*******************************************************************************

;

;	ANALOG-TO-DIGITAL REGISTER 0

;

;	This register is mainly used to start and stop the conversions

;	and select which analog input is to be used for the next conversion.

;

;	note: 	The required pause before the next acquisition can begin is

;		2*the value selected in bits 7 and 6 - in this case Fosc/32

;		would yield about .8us @ 20Mhz

;

;*******************************************************************************

bcf	STATUS,RP0	;

bcf	STATUS,RP1	; SELECT BANK 0

bsf	ADCON0,0x07	;

bcf	ADCON0,0x06	; conversion clock set to Fosc/32

bcf	ADCON0,0x05	;

bcf	ADCON0,0x04	;

bcf	ADCON0,0x03	; select analog input 0

bcf	ADCON0,0x00	; turn off the A/D converter (set this bit to enable)

;*******************************************************************************

;

;	TIMER2	Setup

;

;*******************************************************************************

movlw	0x7F		; prescale of 16

movwf	T2CON		; post scale of 16

;*******************************************************************************

;

;	TIMER1 SETUP

;

;	Timer1 will be used to generate the pulses for the motors

;

;	prescale = 1:1

;

;*******************************************************************************

movlw	0x01		;	0011 0101

movwf	T1CON		;

*********************************************subrtn.inc********************************

;*******************************************************************************

;*******************************************************************************

;

;	SUBROUTINES

;

;*******************************************************************************

;*******************************************************************************

DRIVE	movf	SPEEDL, 0	;

movwf	SPDTL		;

movf	SPEEDR,	0	;

movwf	SPDTR		;

movf	HANDPOS, 0	;

movwf	HANDCNT		;

movlw	PWM20		;

movwf	PERIOD		;

movf	ARMPOS, 0	;

movwf	ARMCNT		;

return			;

;-------------------------------------------------------------------------------

DELAY	movlw	0xFA		;

movwf	COUNT		; 250 cycles

decfsz	COUNT,0x01	;

goto	$ - 1		;

return			;

;-------------------------------------------------------------------------------

CALIB	bcf	ADCON0, 0x03	;

bcf	ADCON0, 0x04	;

bcf	ADCON0, 0x05	;

bsf	ADCON0, 0x00	;

movlw	0x64		;

movwf	TEMP1		;

decfsz	TEMP1, 1	;

goto	$ - 1		;

bsf	ADCON0, 0x02	;

btfsc	ADCON0, 0x02	;

goto	$ - 1		;

movf	ADRESH, 0	;

movwf	WHITE		;

return			;

;-----------------------------------------------------------

CHCK	bcf	ADCON0, 0x03	;

bcf	ADCON0, 0x04	;

bcf	ADCON0, 0x05	; select a/d 0

bsf	ADCON0, 0x02	; start a conversion

btfsc	ADCON0, 0x02	; poll for conversion finished

goto	$ - 1		;

movf	ADRESH,0x00	; grab conversion results

subwf	WHITE, 0	;

btfss	STATUS,0x00	; if a borrow occurs then ball is white (WHITE - W_reg < 0)

goto	NOTIT		; this opcode is skipped if the ball is white

bsf	WORB,0x00	;this is executed is ball is white

return			;

NOTIT	bcf	WORB,0x00	; ball is not white

return			;

;-----------------------------------------------------------

WAIT	movlw	0x92		; WAIT 5 secs

movwf	TEMP1		;

movwf	TEMP2		;

movwf	TEMP3		;

FOR1	decfsz	TEMP3,0x01	;

goto	$ -1		;

movwf	TEMP3		;

decfsz	TEMP2,0x01	;

goto	FOR1		;

movwf	TEMP2		;

decfsz	TEMP1,0x01	;

goto	FOR1		;

return			;

;-----------------------------------------------------------

DELAY2	movf	DELAYT,0x00	; WAIT VARIABLE TIME

movwf	TEMP1		;

movwf	TEMP2		;

movwf	TEMP3		;

DELAYF	decfsz	TEMP3,0x01	;

goto	$ -1		;

movwf	TEMP3		;

decfsz	TEMP2,0x01	;

goto	DELAYF		;

movwf	TEMP2		;

decfsz	TEMP1,0x01	;

goto	DELAYF		;

return			;

*************************************intrupt.inc****************************************

;*******************************************************************************

;

;	INTERRUPT ROUTINE

;

;*******************************************************************************

INTRUPT	movwf	W_TEMP		; save w reg contents

;-------------------------------------------------------------------------------------------------

btfss	PIR1,TMR1IF	; Check if Timer 1 interrupt

goto	NEXT1		;

bcf	PIR1,TMR1IF	; clear flag

; code for timer 1 int goes here

goto	INT_END		;

;-------------------------------------------------------------------------------------------------

NEXT1	btfss	INTCON,0x02	; Check if Timer 0 interrupt

goto	NEXT2		;

bcf	INTCON,0x02	; clear flag

LWHL	btfss	LWHEEL		;

decfsz	SPDTL, 1	;

goto	RWHL		;

bcf	SERVOL		;

bsf	LWHEEL		;

goto	RWHL		;

RWHL	btfss	RWHEEL		;

decfsz	SPDTR, 1	;

goto	GRAB		;

bcf	SERVOR		;

bsf	RWHEEL		;

goto	SERCYC		;

GRAB	btfss	HANDCTL		;

decfsz	HANDCNT, 1	;

goto	REACH		;

bcf	HAND		;

bsf	HANDCTL		;

goto	SERCYC		;

REACH	btfss	ARMCTL		;

decfsz	ARMCNT, 1	;

goto	SERCYC		;

bcf	ARM		;

bsf	ARMCTL		;

goto	SERCYC		;

SERCYC	decfsz	PERIOD, 1	;

goto	INT_END		;

call	DRIVE

bsf	SERVOL		;

bsf	SERVOR		;

bsf	HAND		;

bsf	ARM		;

bcf	LWHEEL		;

bcf	RWHEEL		;

bcf	HANDCTL		;

bcf	ARMCTL		;

goto	INT_END		;

;-------------------------------------------------------------------------------------------------

NEXT2	btfss	PIR1,TMR2IF	; Check if Timer 2 interrupt

goto	NEXT3		;

bcf	PIR1,TMR2IF	; clear flag

btfsc	IRREAD		; check if measurement has been initiated

goto	TESTOK		; if yes go check if measurement is finished else proceed with init

decfsz	TM2CNT,0x01	; wait for 70 ms before reading I.R.

goto	INT_END		;

movlw	0x06		; reset 70 ms counter

movwf	TM2CNT		;

btfsc	IRR		; Check which IR to start read

goto	RIGHTIR		;

btfsc	IRF		;

goto	FRONTIR		;

goto	LEFTIR		;

TESTOK	btfsc	IRR		; Check which IR to begin read

goto	RIGHTOK		;

btfsc	IRF		;

goto	FRONTOK		;

goto	LEFTOK		;

;________________________________________________________________________________________RIGHT IR

RIGHTIR	bcf	IRRCLK		; make clock signal low to initiate a measurement

btfsc	IRRDAT		; check for data signal to go low (ack that a measurement is in progress)

goto	$ - 1		;

bsf	IRREAD		; set flag to indicate a measurement has been started

goto	INT_END		;

RIGHTOK	btfss	IRRDAT		; check for measurement complete

goto	INT_END		; else end interrupt so other routines may continue processing

bsf	IRRCLK		; start bit

call	DELAY		;

bcf	IRRCLK		;

call	DELAY		;

RIR7	bsf	IRRCLK		; bit 7

btfss	IRRDAT		;

goto	RIRLO7		;

goto	RIRHI7		;

RIRLO7	bcf	IRRDATA,0x07	;

goto	RIR6		;

RIRHI7	bsf	IRRDATA,0x07	;

goto	RIR6		;

RIR6	call	DELAY		; bit 6

bcf	IRRCLK		;

call	DELAY		;

bsf	IRRCLK		;

btfss	IRRDAT		;

goto	RIRLO6		;

goto	RIRHI6		;

RIRLO6	bcf	IRRDATA,0x06	;

goto	RIR5		;

RIRHI6	bsf	IRRDATA,0x06	;

goto	RIR5		;

RIR5	call	DELAY		; bit 5

bcf	IRRCLK		;

call	DELAY		;

bsf	IRRCLK		;

btfss	IRRDAT		;

goto	RIRLO5		;

goto	RIRHI5		;

RIRLO5	bcf	IRRDATA,0x05	;

goto	RIR4		;

RIRHI5	bsf	IRRDATA,0x05	;

goto	RIR4		;

RIR4	call	DELAY		; bit 4

bcf	IRRCLK		;

call	DELAY		;

bsf	IRRCLK		;

btfss	IRRDAT		;

goto	RIRLO4		;

goto	RIRHI4		;

RIRLO4	bcf	IRRDATA,0x04	;

goto	RIR3		;

RIRHI4	bsf	IRRDATA,0x04	;

goto	RIR3		;

RIR3	call	DELAY		; bit 3

bcf	IRRCLK		;

call	DELAY		;

bsf	IRRCLK		;

btfss	IRRDAT		;

goto	RIRLO3		;

goto	RIRHI3		;

RIRLO3	bcf	IRRDATA,0x03	;

goto	RIR2		;

RIRHI3	bsf	IRRDATA,0x03	;

goto	RIR2		;

RIR2	call	DELAY		; bit 2

bcf	IRRCLK		;

call	DELAY		;

bsf	IRRCLK		;

btfss	IRRDAT		;

goto	RIRLO2		;

goto	RIRHI2		;

RIRLO2	bcf	IRRDATA,0x02	;

goto	RIR1		;

RIRHI2	bsf	IRRDATA,0x02	;

goto	RIR1		;

RIR1	call	DELAY		; bit 1

bcf	IRRCLK		;

call	DELAY		;

bsf	IRRCLK		;

btfss	IRRDAT		;

goto	RIRLO1		;

goto	RIRHI1		;

RIRLO1	bcf	IRRDATA,0x01	;

goto	RIR0		;

RIRHI1	bsf	IRRDATA,0x01	;

goto	RIR0		;

RIR0	call	DELAY		; bit 0

bcf	IRRCLK		;

call	DELAY		;

bsf	IRRCLK		;

btfss	IRRDAT		;

goto	RIRLO0		;

goto	RIRHI0		;

RIRLO0	bcf	IRRDATA,0x00	;

bcf	IRR		;

bsf	IRF		;

goto	IR_END		;

RIRHI0	bsf	IRRDATA,0x00	;

bcf	IRR		;

bsf	IRF		;

goto	IR_END		;

;________________________________________________________________________________________FRONT IR

FRONTIR	bcf	IRFCLK		; make clock signal low to initiate a measurement

btfsc	IRFDAT		; check for data signal to go low (ack that a measurement is in progress)

goto	$ - 1		;

bsf	IRREAD		; set flag to indicate a measurement has been started

goto	INT_END		;

FRONTOK	btfss	IRFDAT		; check for measurement complete

goto	INT_END		; else end interrupt so other routines may continue processing

bsf	IRFCLK		; start bit

call	DELAY		;

bcf	IRFCLK		;

call	DELAY		;

FIR7	bsf	IRFCLK		; bit 7

btfss	IRFDAT		;

goto	FIRLO7		;

goto	FIRHI7		;

FIRLO7	bcf	IRFDATA,0x07	;

goto	FIR6		;

FIRHI7	bsf	IRFDATA,0x07	;

goto	FIR6		;

FIR6	call	DELAY		; bit 6

bcf	IRFCLK		;

call	DELAY		;

bsf	IRFCLK		;

btfss	IRFDAT		;

goto	FIRLO6		;

goto	FIRHI6		;

FIRLO6	bcf	IRFDATA,0x06	;

goto	FIR5		;

FIRHI6	bsf	IRFDATA,0x06	;

goto	FIR5		;

FIR5	call	DELAY		; bit 5

bcf	IRFCLK		;

call	DELAY		;

bsf	IRFCLK		;

btfss	IRFDAT		;

goto	FIRLO5		;

goto	FIRHI5		;

FIRLO5	bcf	IRFDATA,0x05	;

goto	FIR4		;

FIRHI5	bsf	IRFDATA,0x05	;

goto	FIR4		;

FIR4	call	DELAY		; bit 4

bcf	IRFCLK		;

call	DELAY		;

bsf	IRFCLK		;

btfss	IRFDAT		;

goto	FIRLO4		;

goto	FIRHI4		;

FIRLO4	bcf	IRFDATA,0x04	;

goto	FIR3		;

FIRHI4	bsf	IRFDATA,0x04	;

goto	FIR3		;

FIR3	call	DELAY		; bit 3

bcf	IRFCLK		;

call	DELAY		;

bsf	IRFCLK		;

btfss	IRFDAT		;

goto	FIRLO3		;

goto	FIRHI3		;

FIRLO3	bcf	IRFDATA,0x03	;

goto	FIR2		;

FIRHI3	bsf	IRFDATA,0x03	;

goto	FIR2		;

FIR2	call	DELAY		; bit 2

bcf	IRFCLK		;

call	DELAY		;

bsf	IRFCLK		;

btfss	IRFDAT		;

goto	FIRLO2		;

goto	FIRHI2		;

FIRLO2	bcf	IRFDATA,0x02	;

goto	FIR1		;

FIRHI2	bsf	IRFDATA,0x02	;

goto	FIR1		;

FIR1	call	DELAY		; bit 1

bcf	IRFCLK		;

call	DELAY		;

bsf	IRFCLK		;

btfss	IRFDAT		;

goto	FIRLO1		;

goto	FIRHI1		;

FIRLO1	bcf	IRFDATA,0x01	;

goto	FIR0		;

FIRHI1	bsf	IRFDATA,0x01	;

goto	FIR0		;

FIR0	call	DELAY		; bit 0

bcf	IRFCLK		;

call	DELAY		;

bsf	IRFCLK		;

btfss	IRFDAT		;

goto	FIRLO0		;

goto	FIRHI0		;

FIRLO0	bcf	IRFDATA,0x00	;

bcf	IRR		;

bcf	IRF		;

goto	IR_END		;

FIRHI0	bsf	IRFDATA,0x00	;

bcf	IRR		;

bcf	IRF		;

goto	IR_END		;

;___________________________________________________________________________left IR

LEFTIR	bcf	IRLCLK		; make clock signal low to initiate a measurement

btfsc	IRLDAT		; check for data signal to go low (ack that a measurement is in progress)

goto	$ - 1		;

bsf	IRREAD		; set flag to indicate a measurement has been started

goto	INT_END		;

LEFTOK	btfss	IRLDAT		; check for measurement complete

goto	INT_END		; else end interrupt so other routines may continue processing

bsf	IRLCLK		; start bit

call	DELAY		;

bcf	IRLCLK		;

call	DELAY		;

LIR7	bsf	IRLCLK		; bit 7

btfss	IRLDAT		;

goto	LIRLO7		;

goto	LIRHI7		;

LIRLO7	bcf	IRLDATA,0x07	;

goto	LIR6		;

LIRHI7	bsf	IRLDATA,0x07	;

goto	LIR6		;

LIR6	call	DELAY		; bit 6

bcf	IRLCLK		;

call	DELAY		;

bsf	IRLCLK		;

btfss	IRLDAT		;

goto	LIRLO6		;

goto	LIRHI6		;

LIRLO6	bcf	IRLDATA,0x06	;

goto	LIR5		;

LIRHI6	bsf	IRLDATA,0x06	;

goto	LIR5		;

LIR5	call	DELAY		; bit 5

bcf	IRLCLK		;

call	DELAY		;

bsf	IRLCLK		;

btfss	IRLDAT		;

goto	LIRLO5		;

goto	LIRHI5		;

LIRLO5	bcf	IRLDATA,0x05	;

goto	LIR4		;

LIRHI5	bsf	IRLDATA,0x05	;

goto	LIR4		;

LIR4	call	DELAY		; bit 4

bcf	IRLCLK		;

call	DELAY		;

bsf	IRLCLK		;

btfss	IRLDAT		;

goto	LIRLO4		;

goto	LIRHI4		;

LIRLO4	bcf	IRLDATA,0x04	;

goto	LIR3		;

LIRHI4	bsf	IRLDATA,0x04	;

goto	LIR3		;

LIR3	call	DELAY		; bit 3

bcf	IRLCLK		;

call	DELAY		;

bsf	IRLCLK		;

btfss	IRLDAT		;

goto	LIRLO3		;

goto	LIRHI3		;

LIRLO3	bcf	IRLDATA,0x03	;

goto	LIR2		;

LIRHI3	bsf	IRLDATA,0x03	;

goto	LIR2		;

LIR2	call	DELAY		; bit 2

bcf	IRLCLK		;

call	DELAY		;

bsf	IRLCLK		;

btfss	IRLDAT		;

goto	LIRLO2		;

goto	LIRHI2		;

LIRLO2	bcf	IRLDATA,0x02	;

goto	LIR1		;

LIRHI2	bsf	IRLDATA,0x02	;

goto	LIR1		;

LIR1	call	DELAY		; bit 1

bcf	IRLCLK		;

call	DELAY		;

bsf	IRLCLK		;

btfss	IRLDAT		;

goto	LIRLO1		;

goto	LIRHI1		;

LIRLO1	bcf	IRLDATA,0x01	;

goto	LIR0		;

LIRHI1	bsf	IRLDATA,0x01	;

goto	LIR0		;

LIR0	call	DELAY		; bit 0

bcf	IRLCLK		;

call	DELAY		;

bsf	IRLCLK		;

btfss	IRLDAT		;

goto	LIRLO0		;

goto	LIRHI0		;

LIRLO0	bcf	IRLDATA,0x00	;

bsf	IRR		;

bcf	IRF		;

goto	IR_END		;

LIRHI0	bsf	IRLDATA,0x00	;

bsf	IRR		;

bcf	IRF		;

goto	IR_END		;

IR_END	bcf	IRREAD		; reset flag to indicate a measurement has not been started

goto	INT_END		;

NEXT3	btfss	PIR1,ADIF	; Check if A 2 D interrupt

goto	NEXT3		;

bcf	PIR1,ADIF	; clear flag

goto	INT_END		;

;-------------------------------------------------------------------------------------------------

INT_END	movf	W_TEMP,0x00	; restore w reg contents

retfie			;