# University of Florida

Department of Electrical and Computer Engineering

# EEL 5666

Intelligent Machines Design Laboratory

#### Joshua Phillips

#### Graffiti-Bot

4-25-2001

Table of Contents

Abstract.....................................................................................................................pg. 03.

Introduction..............................................................................................................pg. 03.

Integrated System.....................................................................................................pg. 03.

Mobile Platform........................................................................................................pg. 03.

Actuation...................................................................................................................pg. 04.

Sensors.......................................................................................................................pg. 04.

Behaviors...................................................................................................................pg. 09.

Conclusion.................................................................................................................pg. 14.

Documentation (Assembly code).............................................................................pg. 15.

Appendices (Touch pad data sheet)….......................................................................pg. 46.

Abstract

This project will focus on moving a robot in a straight line and then extracting some behaviors from this ability. A closed loop system for accurate movement will be implemented in Graffiti-bot. This system must allow for movement in a straight line and precise turns. These behaviors will then allow graffiti-bot to write messages in block letters. This report will discuss all aspects of the robot but will concentrate on the closed loop system for accurate movement.

## Introduction

Graffiti-bot will be a simple symmetrical rectangular box platform. Since one of the overall goals of the robot will be to move in a straight line, the location of the wheels, motors, and sensors will be important. Good symmetry about the robot will help reduce but not eliminate the tendency of most platforms to stray from their original path. For example, wheels of unequal radius (even if not noticeable by the human eye) will eventually cause the robot to veer off course and the driving motors will seldom react exactly the same given the same input. When all of these factors are taken into account, the need for a feedback system is quite apparent. The standard behavior of obstacle avoidance will also be implemented. It is anticipated that the feedback system introduced above will eliminate some of the problems with typical avoidance systems.

Integrated System

### The brain for Graffiti-bot will be implemented with a Microchip PIC16F877 Microcontroller. Several sensors will be used to gather information about the robots environment. Some of these sensors will be infrared sensors, “bump” switches, and a PS/2 optical mouse for accurate movement.  The microcontroller will be used to gather and interpret the information from the sensors about the environment and update the robots behaviors accordingly. A Liquid Crystal Display (LCD) will be mounted on Graffiti-bot and used for troubleshooting and general information display. All of these components will be

## Mobile Platform

The physical platform for the robot is made of balsa wood and cutout on the “T-tech” machine in lab. The basic structure of the robot will be a simple rectangular box. Graffiti-bot is a two-wheeled robot. The wheels will be rear mounted and the front of the robot will rest on the PS/2 mouse. Accurate measurements must be made, as the mounting of the PS/2 mouse is critical. The motors should also be mounted as symmetrically as possible to aid in moving straight. The wheels are three-inch diameter and were obtained from Mekatronix.

Actuation

Graffiti-bot will use two Nema size 17 stepper motors to drive the wheels. The motors were obtained from Jameco part number 155432. The stepper motors will make navigation and steering corrections easier as a fairly accurate measurement of the number of revolutions can be obtained by counting the number of pulses sent to each motor. This can then be combined with the circumference of each wheel to yield the distance each wheel traveled. To interface the motors to the microprocessor, Allegro Microsystems part number UCN5804B BiMos II Unipolar Stepper-Motor Driver is used. These I.C.’s make an easy 3 wire interface for controlling the motors. One general-purpose output pin for each control signal: motor direction, output enable, and one step input.

Sensors

The main sensors used for collision avoidance are two Sharp part number GP2D02 High Sensitive Distance Measuring Sensors. These each contain an I.R. transmitter, receiver, and a lens that can be changed to alter the range of the device. The Sharp GP2D02 can sample about once every 72 milliseconds and sends it data via a synchronous serial stream. The interface with the I.R. sensor is a fairly simple four-wire interface but it should be noted that the user must supply the clock and that the clock pin on the sensor is an open drain input. Also, the clock line is really a control line and must do more than simply supply a clock signal. For example, to instruct the sensor to take a measurement the clock pin must be held low for 70 milliseconds. One circuit that works well and is simple is shown below.

Vdd

Microprocessor

8 Bit Serial Output

3	4

1	2

Clock Signal

The diode only allows the microprocessor to pull the open drain clock pin low. The data sheet for the Sharp GP2D02 is included in the appendix.

The main sensor for gathering data about the relative movement of the robot is a Logitech PS/2 optical mouse part number 830386-0000. Using an optical mouse is advantageous because there are no moving parts and it never needs cleaning. Also the resolution of an optical mouse is usually better than a standard ball mouse. As the PS/2 protocol is rather lengthy I will discuss only the information necessary to implement the PS/2 mouse into a feedback system on a mobile robot. For the remainder of this discussion of the PS/2 protocol and interface the term “host” shall refer to the unit querying the mouse and the term “device” shall refer to the mouse itself.

PS/2 Protocol

The PS/2 protocol is a four-wire interface consisting of power, ground, clock, and data. The clock and data lines are bi-directional open collector signals. Clock and data normally float high and are pulled low by either device or host. The data and clock line are NEVER asserted high by the host or device. The information is transmitted via an 11 bit serial stream consisting of one start bit (low), eight data bits (LSB first), one odd parity bit, and one stop bit (high). Three to four of these 11 bit streams are usually combined into one packet depending on the mode of operation and information requested from the device. Communication can occur from host to device or from device to host. In either case the mouse always provides the clock signal. The data packet format, two types of communication, the electrical interface, and the device initialization are described in detail below.

Data Packet Format

The following packet is the format used in default mode. Other mice may include options to add extra packets for more buttons or features but default mode is fine for this project.

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte 1 | Y ovfl | X ovfl | Y sign | X sign | 1 | 0 | R Button | L Button |
| Byte 2 | X7 | X6 | X5 | X4 | X3 | X2 | X1 | X0 |
| Byte 3 | Y7 | Y6 | Y5 | Y4 | Y3 | Y2 | Y1 | Y0 |

- Y ovfl  :  overflow flag for the Y movement accumulator (Byte 3).

  - 1 = an overflow occurred.

  - 0 = an overflow did not occur.

- X ovfl  :  overflow flag for the X movement accumulator (Byte 2).

  - 1 = an overflow occurred.

  - 0 = an overflow did not occur.

- Y sign  :  sign bit for Byte 3

  - 1 = Byte 3 is negative (mouse is moving backwards).

  - 0 = Byte 3 is zero or positive (mouse is moving forwards).

- X sign  :  sign bit for Byte 2

  - 1 = Byte 2 is negative (mouse is moving left).

  - 0 = Byte 2 is zero or positive (mouse is moving right).

- R Button  :  Right mouse button status bit

  - 1 = right mouse button is pressed.

  - 0 = right mouse button is not pressed.

- L Button  :  Left mouse button status bit

  - 1 = left mouse button is pressed.

  - 0 = left mouse button is not pressed.

Byte 2 is the amount of movement in the x direction (left or right) the mouse has detected since the last transmission. Byte 3 is the amount of movement in the y direction (forwards or backwards) the mouse has detected since the last transmission.

Host to Device Communication

- To change the mode of the device to “host request to send” mode the host must follow the following procedure:

  - Hold the clock line low for at least 100 microseconds (longer is ok).

  - Pull the data line low.

  - Release the clock line.

- Once the device is in “host request to send” mode it will pulse the clock low 11 times.

- The host changes data when the clock is low.

- The device latches data when the clock is high.

- After sensing a valid stop bit the device will pull data low and pulse the clock line low one more time to indicate that a framing error did not occur.

- If a framing error did occur (invalid stop or parity bit) the device will send the “resend” command (0hFE) to the host.

- If the data transfer was successful the device will respond by sending an acknowledge byte (0hFA) followed by any data the host command requires.

Device to Host Communication

- When the device has information to send to the host it checks the state of the clock and data line.

  - If Data and Clock are both high the bus state is idle and the device will transmit the data.

  - If the Clock line is low the Host is inhibiting transmission and the device will continue to accumulate data and wait for the host to release the clock line.

  - If Clock is high and Data is low the host has requested “host request to send” mode is ready to transmit data to the device.

- If the bus is idle the device will transmit the data by pulsing the clock line low once and then high 11 more times while changing the data line appropriately.

- The device changes the data when the clock is high.

- The host should latch the data when the clock is low.

- If invalid data is detected (bad parity bit) the host can issue the Resend command (0hFE).

Electrical Interface

As mentioned above the clock and data lines are bi-directional open collector signals. One interface (the one I used) that works well is shown below. This setup requires two general-purpose input pins with internal weak pull-up resistors, two general-purpose output pins, and two NPN switching transistors. The output pins turn on the switching transistors to pull the appropriate line (clock or data) low. The internal weak pull-up resistors on the microprocessor allow the line to float high when the transistors are off.

MICROPROCESSOR										DATA

CLOCK

DATA CONTROL

CLOCK CONTROL

##### PS/2 MOUSE

DATA

CLOCK

If internal weak pull-up resistors are not available then external pull-up resistors (5 to 10 kOhms) can be added as follows:

Vdd	Vdd

MICROPROCESSOR										DATA

CLOCK

DATA CONTROL

CLOCK CONTROL

##### PS/2 MOUSE

DATA

CLOCK

Device Initialization

The PS/2 protocol allows several modes of operation for a mouse such as Polling Mode or Continuous Stream Mode. I decided to use a controlled continuous stream mode. A few initializations must be made upon powering up or hot-plugging the mouse, as it will remain in disabled mode until given an enable code by the user. Before sending the enable code and setting the mode the host must wait about for the device to complete its power on self-test and calibration. This typically takes between 300 and 1000 milliseconds. Once the self-test and calibration is complete the device will transmit the code 0hAA followed by 0h00. Once this “ready” code is received an enable command (0hF4) must be sent. This will enable the mouse to transmit data in its default mode. The default mode is continuous stream mode (which is the mode I will be using) so no other initialization is needed. A full list of commands and modes is contained in a sample of pages from a Synaptics PS/2 touch pad data sheet in the appendix of this document. The full document will be included on the floppy disk accompanying this report This document is a good resource for adjusting the resolution of the mouse and troubleshooting.

Useful Hints for Using the PS/2 Mouse

- When enabled, the PS/2 mouse sends a packet of data upon an event. For example, if the mouse is not being moved and no buttons have been pressed or released, the mouse will not transmit any data. Once the mouse detects movement or a button changes state the mouse will transmit the data if the bus is idle (clock and data lines both high). This is important if you are going to poll the device clock during transmission. If the mouse does not change state the clock will remain high permanently and you will be polling for a very long time. A timeout routine should be included if you plan to poll the device clock line.

- If the bus is not idle and the mouse has data to send, the mouse will continue accumulating x and y movement data until the bus is idle. This could result in an overflow of the x or y movement byte if transmission is inhibited by the host for too long. For this reason the overflow bit should always be checked when using the mouse x and y movement data.

- In order not to lose packets, the host should inhibit transmission by pulling the clock line low anytime the host is not monitoring the bus. For example, if the microprocessor is reading the I.R. data and the mouse is moving, the mouse will transmit its data packet if the bus is idle. The processor would miss this unless the clock line was held low in which case the movement would just be added into the next packet of data. (An ideal way to solve this problem is to have a dedicated system reading the mouse continuously, but I went for a one-chip design.)

- Remember the mouse always supplies the clock, once enabled the mouse will always send data packets (event driven) if the bus is idle.

- Any command sent by the host will cause the motion accumulators to be cleared.

Bump Switches

The last sensor used in Graffiti-bot is a normally open single pull single throw switch. The switch is attached to a bumper that runs along portions of the exterior that would be prone to bumping into objects or walls should the other sensors fail. These switches connect a general-purpose input pin to either ground or Vdd depending on the state of the switch. Polling the different input pins allows graffiti-bot to determine which side of the robot has collided with an object.

Behaviors

All of the sensors generate stimuli for the robot to respond to. These responses to the sensor input can then be abstracted to behaviors. The first behavior of Graffiti-bot will be obstacle avoidance. The robot must be able to protect itself from injury and must also ensure it does not injure other objects. This behavior is accomplished via the Sharp I.R. sensors. Taking samples from the I.R. once every 70 milliseconds, the I.R. data is tested against a fixed value (or distance). When the object is closer than the preset threshold, the robot will choose a random course of action (except, of course, continuing toward the object).

Another behavior that Graffiti-bot is capable of is moving in a straight line. This is accomplished using the PS/2 mouse. The X accumulator is sampled about 100 times a second along with the “X sign” bit to tell the direction and magnitude of deviation from a straight line. Depending on the direction and magnitude, the appropriate wheel speed is reduced or increased as necessary until the robot moves straight. It is important to align the mouse accurately as a misaligned mouse will yield false readings and cause the robot to move in circles or other undesired motion. The mouse should be mounted with the bisecting line along the vertical of the mouse aligned with the bisecting line along the vertical of the robot as seen from a birds eye view in the following diagram:

Note: The mouse can be mounted anywhere along the blue line.			          The preferable location has the center of the mouse			          “eye” located half the distance of the red line above the		           intersection of the red and blue lines.

To fine-tune the control system, an error much larger than would be experienced during normal operation was forced into the system. The feedback system variables were changed incrementally and the deviation (measured in degrees) from the desired path after 10 ft was recorded. The test would be similar to the following diagram:

Actual Path	Desired Path

	  10 Feet

After several tests the following graph was formed:

Note: The point at zero is invalid as it causes oscillation and other unwanted behavior.

The final result of the experiment was that the optimal threshold for comparison with the X magnitude was between 0h06 and 0h07. This value will increase or decrease depending on the alignment of the mouse, symmetry in the mounting of the motors, wheel alignment and wheel similarity. Using a final result of seven in the threshold register, the robot was able to travel 10 feet with a deviation of one degree or less from the desired path.

The last behavior Graffiti-bot exhibits is message writing. By attaching a solenoid-controlled pen to the robot platform simple text can be written in block letters. The ability to make an accurate 90-degree turn is essential to make block letters. This can easily accomplished with the stepper motors and checked with the mouse. Using the diagram below the correct number of steps needed to turn the robot 90-degrees about the wheel center can be calculated.

Forward

R

90 turn

After the 90 degree turn each wheel has traveled a distance of R*/2 along the dotted green line. The number of steps is then calculated as:

Number of steps = ( R * /2 ) / (number of degrees per step *  wheel radius)

The robot will have a difficult time making an exact 90-degree turn due to the discreetness of the stepper motors. The 1.8 degree per step motors on Graffiti-bot were able to make an 89.5 degree turn. This is accurate enough for the purposes of this project, however, after several turns the accumulated error begins to show. This can be rectified by turning 90.5 degrees half of the time and turning 89.5 degrees the other half of the time. With these details implemented, Graffiti-bot successfully writes messages.

Conclusion

Graffiti-bot is a success. The ability to drive straight was successful and provided the means to develop the other behaviors. However, the method of integration used amongst the sensors and peripheral devices could be improved. Using a single processor to control every device on the robot is space saving but complicates the software development. Using a dedicated system for each sensor or peripheral device removes some of the burden from the main processor and makes software development much easier. If done again, I wouldn’t hesitate to adopt this philosophy.

;	Graffiti-bot Code for IMDL spring 2001 Prof Arroyo by Joshua Phillips

LIST	P=PIC16F877	;

include "p16f877.inc"	;

;*******************************************************************************

;	LABELS

;*******************************************************************************

#define IRFDAT	PORTB,0x07	;

#define IRFCLK	PORTB,0x06	;

#define MSDATA	PORTB,0x05	;

#define	MSCLOCK	PORTB,0x04	;

#define MSCCTRL	PORTB,0x03	;

#define MSDCTRL	PORTB,0x02	;

#define IRRDAT	PORTB,0x01	;

#define IRRCLK	PORTB,0x00	;

#define MOTORL	PORTA,0x00	;

#define MOTORR	PORTA,0x01	;

#define	MOTL_OE	PORTA,0x02	;

#define	MOTR_OE	PORTA,0x03	;

#define	MOTR_DR	PORTC,0x07	;

#define	MOTL_DR	PORTA,0x05	;

#define	RS_LQD	PORTC,0x00	;

#define	RW_LQD	PORTC,0x01	;

#define	E_LQD	PORTC,0x02	;

#define	BUMP_F	PORTC,0x06	;

#define BUMP_L	PORTC,0x05	;

#define BUMP_R	PORTC,0x04	;

#define	MARKER	PORTC,0x03	;

#define	LQD	PORTD		;

#define	BUSYFL	PORTD,0x07	;

#define Y_OVER	BUTTONS,0x07	;

#define	X_OVER	BUTTONS,0x06	;

#define	Y_SIGN	BUTTONS,0x05	;

#define	X_SIGN	BUTTONS,0x04	;

#define	R_BUT	BUTTONS,0x01	;

#define	L_BUT	BUTTONS,0x00	;

COUNT	equ	0x24		;

COUNT1	equ	0x25		;

COUNT2	equ	0x26		;

MSINFO	equ	0x27		; no init needed

PARITY	equ	0x28		; no init needed

LQDDATA	equ	0x29		; no init needed

REVTEMP	equ	0x2A		; no init needed

REVTMP2	equ	0x2B		; no init needed

REVTMP3	equ	0x2C		; no init needed

REVTMP4	equ	0x2D		; no init needed

TIMEOUT	equ	0x2F		; no init needed

INCNT	equ	0x30		; no init needed

OUTCNT	equ	0x31		; no init needed

RWT	equ	0x32		; Period for Right wheel pulse (8 Bit)

LWTH	equ	0x33		; Period for Left wheel upper byte

LWTL	equ	0x34		; Period for left wheel lower byte

TM2CNT	equ	0x35		; 70 ms timer in TIMER2 interrupt routine (IR)

IRDATA	equ	0x36		; no init needed

IRTEST	equ	0x37		; flag for IR routines (indicates a measurement has been

started)

IR	equ	0x38		; flag for main routine to validate data / flag for IR

routines to choose front or rear IR

IRRDATA	equ	0x39		; no init needed

LW_CNT	equ	0x3A		; counters for mouse to be read once every LW_CNT times the

wheel is turned

RW_CNT	equ	0x3B		; counters for LCD to be updated once every RW_CNT times

the wheel is turned

STCOUNT	equ	0x3C		; timeout counter for mouse routine

IDLE_MS	equ	0x3D		; flag for main routine to recognize a timeout, no init

needed

BUTTONS	equ	0x3E		; mouse data

X_MAG	equ	0x3F		; mouse data

Y_MAG	equ	0x40		; mouse data

RAND	equ	0x41		; a random number

TOUT_2	equ	0x42		; another timeout counter

TOUT_3	equ	0x43		; another timeout counter

PULSES	equ	0x44		; for navigation

DIST	equ	0x45		; for navigation

NAV_OK	equ	0x46		; for nav init to 0x00

SECONDS	equ	0x47		;

ODELAY	equ	0x48		;

IDELAY	equ	0x49		;

NDELAY	equ	0x4A		;

MS_CNTR	equ	0x4B		; init to 3

MS_CNTL	equ	0x4C		; init to 3

RADIUS	equ	0x4D		;

RPULSES	equ	0x4E		;

LPULSES	equ	0x4F		;

RWT_TMP	equ	0x50		;

LWTHTMP	equ	0x51		;

LWTLTMP	equ	0x52		;

ACCRCNT	equ	0x53		;

ACCLCNT	equ	0x54		;

TOPSPD	equ	0x55		;

MAX_ACC	equ	0x56		;

MS_USE	equ	0x57		;

MALIGNB	equ	0x58		;

MALIGNF	equ	0x59		;

FIRST_F	equ	0x5A		;

W_TEMP	equ	0x70		; temporary register for w available in all banks

S_TEMP	equ	0x71		; temp reg for the status reg (all banks)

org	0x00		;	Set RESET vector

goto	INIT		;	to beginning of program.

org	0x04		;	set INTERRUPT vector to

goto	INTRUPT		;	beginning of INTERRUPT service routine

org	0x05		;	start of program

INIT	bcf	STATUS,RP0	;	select bank 0

bcf	STATUS,RP1	;	select bank 0

;*******************************************************************************

;	INTIALIZE REGISTERS / PORTS / INTERUPTS

;*******************************************************************************

;*******************************************************************************

;	INTERRUPT CONTROL REGISTER

;	initialize to:

;	GLOBAL INTERRUPT ENABLE		: DISABLED	0

;	PERIPHERAL INTERRUPT ENABLE	: ENABLED	1

;	TIMER0 OVERFLOW INTERRUPT	: ENABLED	1

;	EXTERNAL INTERRUPT PIN		: DISABLED	0

;	PORTB INTERRUPT ON CHANGE	: DISABLED	0

;	LOWER 3 BITS ARE FLAGS 		: DONT CARE	xxx

;*******************************************************************************

movlw	0x60		;	initialize interrupt control reister

movwf	INTCON		;

;*******************************************************************************

;*******************************************************************************

;	OPTION REGISTER

;	initialize to:

;	PORTB INTERNAL PULLUP RESISTORS	: ENABLED	0

;	EXTERNAL INTERRUPT EDGE SELECT	: RISING EDGE	1

;	TIMER0 CLOCK SOURCE SELECT	: Fosc/4	0

;	TIMER0 SOURCE EDGE SELECT	: low to high	0

;	PRESCALER ASSIGNMENT		: TIMER0	0

;	LOWER 3 BITS ARE PRESCALER	: 1/256		111

;*******************************************************************************

bsf	STATUS,RP0	;	select bank 1

bcf	STATUS,RP1	;	select bank 1

movlw	0x47		;

movwf	OPTION_REG	;	initialize option register

;*******************************************************************************

;	PERIPHERAL INTERRUPT REGISTER 1

;	initialize to:

;	PARALLEL SLAVE PORT R/W INTERRUPT ENABLE	: DISABLED	0

;	A/D CONVERTER INTERRUPT ENABLE			: DISABLED	0

;	USART RECIEVE INTERRUPT ENABLE			: DISABLED	0

;	USART TRANSMIT INTERRUPT ENABLE			: DISABLED	0

;	SYNCHRONOUS SERIAL PORT INTERRUPT ENABLE	: DISABLED	0

;	CAPTURE/COMPARE 1 INTERRUPT ENABLE		: DISABLED	0

;	TIMER2 TO PR2 MATCH INTERRUPT			: ENABLED	1

;	TIMER1 OVERFLOW INTERRUPT ENABLE		: ENABLED	1

;*******************************************************************************

movlw	0x03	;

movwf	PIE1	; initialize PIE1 register

;*******************************************************************************

;	PERIPHERAL INTERRUPT REGISTER 2

;	initialize to:

;	BIT 7-5					: ALWAYS 000

;	EEPROM WRITE INTERRUPT ENABLE		: DISABLED 0

;	BUS COLLISION INTERRUPT ENABLE		: DISABLED 0

;	BIT 2-1					: ALWAYS 00

;	CAPTURE COMPARE 2 INTERRUPT ENABLE	: DISABLED 0

;*******************************************************************************

movlw	0x00;

movwf	PIE2; initialize PIE2 register

;*******************************************************************************

;	PORT A (6 bits wide)

;	Analog input 0 thru 4 are on this port along with

;	TIMER0's external clock input pin

;	A0 will be the left motor signal

;	A1 will be the right motor signal

;	A2 will be the motor driver ouput enable signal (active low)

;	A4 and A5 will be motor direction pins

;	All other pins are inputs

;*******************************************************************************

movlw	0x00		;

movwf	TRISA		; data direction register for PORTA

;*******************************************************************************

;	PORT B (8 bits wide)

;	General purpose in-out.

;	Bit 0 and 1 will control the rear IR.

;	Bit 2 will control the mouse data line.

;	Bit 3 will control the mouse clock line.

;	Bit 4 will be used as mouse clock.

;	Bit 5 will be used as mouse data.

;	Bit 6 will be the IR front clock.

;	Bit 7 will be the IR front data.

;	All pins will be used as inputs.

;*******************************************************************************

movlw	0xB2		;

movwf	TRISB		; data direction register for PORTB

;*******************************************************************************

;	PORT C (8 bits wide)

;	General purpose in-out, PWM output, SPI, USART, CAPTURE 1&2

;	PORT C PINS 0,1,2 will be control lines for the LQD

;	PORT C PIN 7 will be MOTOR LEFT DIRECTION

;*******************************************************************************

movlw	0x70		; 0111 0000

movwf	TRISC		; data direction register for PORTC

;*******************************************************************************

;	PORT D (8 bits wide)

;	Port D can be used as a parallel slave port or general in-out

;	This port will send data to the LQD

;	all pins are outputs

;*******************************************************************************

movlw	0x00		; 0000 0000

movwf	TRISD		; data direction register for PORTD

;*******************************************************************************

;	PORT E (3 bits wide)

;	Port E can be used as general purpose in-out, Analog input 7 thru 5,

;	or as the control bits for parallel slave port mode.

;*******************************************************************************

movlw	0x07		;

movwf	TRISE		; data direction register for PORTE

;*******************************************************************************

;	ANALOG-TO-DIGITAL REGISTER 1

;	This register selects the port configurations for analog or digital

;	input and selects the values for Vref+

;	This register also right/left justifies the A/D result register

;	The result is 10 bits wide in a 16 bit wide register.

;	The pins will be used as follows:

;	PORT E A/D pins = Digital I/O

;	PORT A	PIN0	= Digital I/O

;		PIN1	= Digital I/O

;		PIN2	= Digital I/O

;		PIN3	= Digital I/O

;		PIN4	= Digital I/O

;		PIN5	= Digital I/O

;*******************************************************************************

movlw	0x06		;

movwf	ADCON1		; config A/D

;*******************************************************************************

;	ANALOG-TO-DIGITAL REGISTER 0

;	This register is mainly used to start and stop the conversions

;	and select which analog input is to be used for the next conversion.

;	note: 	The required pause before the next acquisition can begin is

;		2*the value selected in bits 7 and 6 - in this case Fosc/32

;		would yield about .8us @ 20Mhz

;*******************************************************************************

bcf	STATUS,RP0	;

bcf	STATUS,RP1	; SELECT BANK 0

bsf	ADCON0,0x07	;

bcf	ADCON0,0x06	; conversion clock set to Fosc/32

bcf	ADCON0,0x05	;

bcf	ADCON0,0x04	;

bcf	ADCON0,0x03	; select analog input 0

bcf	ADCON0,0x00	; turn off the A/D converter

;*******************************************************************************

;	TIMER2	Setup

;*******************************************************************************

movlw	0x7F		; prescale of 16

movwf	T2CON		; post scale of 16

;*******************************************************************************

;	TIMER1 SETUP

;	Timer1 will be used to generate the pulses for the motors

;	prescale = 1:1

;*******************************************************************************

movlw	0x01		;	0011 0101

movwf	T1CON		;

;*******************************************************************************

;	MAIN PROGRAM

;*******************************************************************************

call	MS_TX		; Initialize PS/2 Mouse and write data to LQD

call	LQDINIT		; initialize Liquid Crystal Display

bsf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

clrf	IRTEST		;

clrf	IR		;

clrf	X_MAG		;

clrf	Y_MAG		;

clrf	BUTTONS		;

bsf	MOTL_OE		;

bsf	MOTR_OE		;

bcf	MOTL_DR		; init to move forward

bsf	MOTR_DR		;

movlw	0x00		;

movwf	LWTH		;

movwf	LWTL		;

movwf	RWT		;

movlw	0x06		;

movwf	TM2CNT		; 70 ms timer for IR routine

movlw	0x03		;

movwf	MS_CNTL		;

movwf	MS_CNTR		;

clrf	RWT_TMP		;

clrf	LWTHTMP		;

clrf	LWTLTMP		;

movlw	0x07		;

movwf	ACCRCNT		;

movwf	ACCLCNT		;

movlw	0xFF		;

movwf	TOPSPD		;

movlw	0x03		;

movwf	MAX_ACC		;

bcf	MS_USE,0x00	; (mouse is disabled)

movlw	0x10		;

movwf	SECONDS		;

movlw	0x38		;

movwf	MALIGNB		;

movlw	0x3B		;

movwf	MALIGNF		;

bsf	MARKER		; lift up pen

;--------------------------------------------------------------------------------------------------

; X_MAG AND Y_MAG are 9 bit signed 2's compliment numbers

; the 9th bit is the Y_sign or X_sign bit below in the button register

;--------------------------------------------------------------------------------------------------

;BUTTONS REGISTER

;

;BIT	7		6		5	4	3	2	1		0

;

;NAME	Y_OVER		X_OVER		Y_SIGN	X_SIGN	1	0	R_BUT		L_BUT

;

;IF 1 	overflow	overflow	DOWN	LEFT	1	0	PRESSED		PRESSED

;

;IF 0	no overflow	no overflow	UP	RIGHT	1	0	NOT PRESSED 	NOT PRESSED

;

; (LEFT MOTOR 0 = forward, 1 = reverse) (RIGHT MOTOR 0 = reverse, 1 = reverse)

;--------------------------------------------------------------------------------------------------

WAIT4ME	goto	HERE		;

clrf	BUTTONS		;

call	MS_INFO		; wait until the left mouse button is pushed before

btfsc	BUTTONS,0x00	; doing anything.

goto	LEFTMB		;

btfsc	BUTTONS,0x01	;

goto	RIGHTMB		;

goto	WAIT4ME		;

LEFTMB	bsf	INTCON,GIE	; enable unmasked interrupts

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

bcf	MOTL_OE		;

bcf	MOTR_OE		;

HERE	bsf	INTCON,GIE	; enable unmasked interrupts

bcf	MOTL_OE		;

bcf	MOTR_OE		;

bcf	MS_USE,0x00	; disable mouse

movlw	0x00		;

movwf	LWTH		;

movwf	LWTL		;

movwf	RWT		;

btfss	MOTL_DR		; check direction of LEFT MOTOR

goto	L_FOR		;

goto	L_REV		;

L_FOR	btfsc	MOTR_DR		; LEFT MOTOR is going forwards check RIGHT MOTOR

goto	FORWARD		; BOTH MOTORS FORWARD

goto	R_TURN		; RIGHT TURN

L_REV	btfsc	MOTR_DR		; LEFT MOTOR is going backwards check RIGHT MOTOR

goto	L_TURN		; LEFT TURN

goto	REVERSE		; REVERSE

;-------------------------------------------------------------------------------

FORWARD	bcf	IR,0x00		; choose front IR

btfss	BUMP_F		;

goto	OPT_3		; if front collision go backward

btfss	BUMP_L		; if left collision go backward

goto	OPT_3		;

btfss	BUMP_R		; if right collision go backward

goto	OPT_3		;

btfsc	X_OVER		; if an overflow occured do nothing

goto	STRT		;

btfsc	X_SIGN		;

goto	MOVINGL		; robot is moving left / increase left motor speed?

goto	MOVINGR		; robot is moving right or straight / increase right motor

speed?

MOVINGR	movlw	0x33		; check X_MAG from mouse if less than w-reg dont do

anything

subwf	X_MAG,0x00	; if greater than w-reg adjust motor times to correct

steering

btfss	STATUS,0x00	;

goto	STRT		;

movlw	0x01		;

addwf	RWT,0x01	;

clrf	X_MAG		;

goto	STRT		;

MOVINGL	movlw	0xFF		; take 2's compliment

xorwf	X_MAG,0x01	; flip the bits

incf	X_MAG,0x01	; add 1

movlw	0x33		; check X_MAG from mouse if less than w-reg dont do

anything

subwf	X_MAG,0x00	; if greater than w-reg adjust motor times to correct

steering

btfss	STATUS,0x00	;

goto	STRT		;

movlw	0x01		;

addwf	LWTL,0x01	;

clrf	X_MAG		;

goto	STRT		;

STRT	btfsc	IR,0x07		; check if new data being aquired

goto	$ - 1		;

btfss	IR,0x07		; check if data is valid

goto	$ - 1		;

movlw	0x83		;

subwf	IRDATA,0x00	; check if object is near

btfss	STATUS,0x00	;

goto	HERE		;

movf	TMR0,0x00	; get a random number

movwf	RAND		; store random number

sublw	0x55		; check for a range of 0 to 85

btfsc	STATUS,0x00	; check if a borrow occured

goto	OPT_1		; if between 0 and 85 choose option 1

movf	RAND,0x00	; get same number again

sublw	0xAA		; check for a range of 85 to 170

btfsc	STATUS,0x00	; check if a borrow occured

goto	OPT_2		; if between 85 and 170 choose option 2

goto	OPT_3		; if between 170 and 255 choose option 3

OPT_1	bcf	MOTL_DR		; turn right

bcf	MOTR_DR		;

call	DELAY		;

call	DELAY		;

goto	HERE		;

OPT_2	bsf	MOTL_DR		; turn left

bsf	MOTR_DR		;

call	DELAY		;

call	DELAY		;

goto	HERE		;

OPT_3	bsf	MOTL_DR		; move in reverse mode

bcf	MOTR_DR		;

call	DELAY		;

call	DELAY		;

goto	HERE		;

REVERSE	bsf	IR,0x00		; choose rear IR

btfsc	IR,0x07		; check if new data being aquired

goto	$ - 1		;

btfss	IR,0x07		; check if data is valid

goto	$ - 1		;

movlw	0x83		;

subwf	IRRDATA,0x00	; check if object is near

btfss	STATUS,0x00	;

goto	HERE		;

movf	TMR0,0x00	; get a random number

movwf	RAND		; store random number

sublw	0x55		; check for a range of 0 to 85

btfsc	STATUS,0x00	; check if a borrow occured

goto	OPT_R1		; if between 0 and 85 choose option 1

movf	RAND,0x00	; get same number again

sublw	0xAA		; check for a range of 85 to 170

btfsc	STATUS,0x00	; check if a borrow occured

goto	OPT_R2		; if between 85 and 170 choose option 2

goto	OPT_R3		; if between 170 and 255 choose option 3

OPT_R1	bcf	MOTL_DR		; turn right

bcf	MOTR_DR		;

call	DELAY		;

call	DELAY		;

goto	HERE		;

OPT_R2	bsf	MOTL_DR		; turn left

bsf	MOTR_DR		;

call	DELAY		;

call	DELAY		;

goto	HERE		;

OPT_R3	bcf	MOTL_DR		; move forward

bsf	MOTR_DR		;

call	DELAY		;

call	DELAY		;

goto	HERE		;

R_TURN	bcf	IR,0x00		; choose front IR

btfss	BUMP_L		; if left collision go backward

goto	OPT_3		;

btfss	BUMP_R		; if right collision go backward

goto	OPT_3		;

btfsc	IR,0x07		; check if new data being aquired

goto	$ - 1		;

btfss	IR,0x07		; check if data is valid

goto	$ - 1		;

movlw	0x83		;

subwf	IRDATA,0x00	; check if object is near

btfsc	STATUS,0x00	;

goto	HERE		;

bcf	MOTL_DR		; move forward

bsf	MOTR_DR		;

call	DELAY		;

call	DELAY		;

goto	HERE		;

L_TURN	bcf	IR,0x00		; choose front IR

btfss	BUMP_L		; if left collision go backward

goto	OPT_3		;

btfss	BUMP_R		; if right collision go backward

goto	OPT_3		;

btfsc	IR,0x07		; check if new data being aquired

goto	$ - 1		;

btfss	IR,0x07		; check if data is valid

goto	$ - 1		;

movlw	0x83		;

subwf	IRDATA,0x00	; check if object is near

btfsc	STATUS,0x00	;

goto	HERE		;

bcf	MOTL_DR		; move forward

bsf	MOTR_DR		;

call	DELAY		;

call	DELAY		;

goto	HERE		;

;-------------------------------------------------------------------------------

;	THIS IS THE GRAFFITTI BEHAVIOR (robot will write GO GATORS)

;-------------------------------------------------------------------------------

RIGHTMB	call	LETTERG		;

call	LETTERO		;

call	BLANK_S		;

call	LETTERG		;

call	LETTERA		;

call	LETTERT		;

call	LETTERO		;

call	LETTERR		;

call	LETTERS		;

goto	WAIT4ME		;

;*******************************************************************************

;	INTERRUPT ROUTINE

;*******************************************************************************

INTRUPT	movwf	W_TEMP		; save w reg contents

btfss	PIR1,TMR1IF	; Check if Timer 1 interrupt

goto	NEXT1		;

bcf	PIR1,TMR1IF	; clear flag

LEFT_M	btfss	MOTORL		; check if left motor pulse is high or low

goto	MAKELH		; if low make high

goto	MAKELL		; if high make low

MAKELH	bsf	MOTORL		;

btfsc	NAV_OK,0x00	;

decf	LPULSES,0x01	;

goto	LSETT		; set pulse width

MAKELL	bcf	MOTORL		;

goto	LSETT		; set pulse width

LSETT	bsf	NAV_OK,0x01	; tell ACC routine which motor is being updated

call	ACC		;

movf	LWTHTMP,0x00	;

movwf	TMR1H		;

movf	LWTLTMP,0x00	;

movwf	TMR1L		;

goto	INT_END		;

;-------------------------------------------------------------------------------------------------

NEXT1	btfss	INTCON,0x02	; Check if Timer 0 interrupt

goto	NEXT2		;

bcf	INTCON,0x02	; clear flag

RIGHT_M btfss	MOTORR		; check if right motor pulse is high or low

goto	MAKERH		; if low make high

goto	MAKERL		; if high make low

MAKERH	bsf	MOTORR		;

btfsc	NAV_OK,0x00	;

decf	RPULSES,0x01	;

goto	RSETT		; set pulse width

MAKERL	bcf	MOTORR		;

goto	RSETT		; set pulse width

RSETT	bcf	NAV_OK,0x01	; tell ACC routine which motor is being updated

call	ACC		;

movf	RWT_TMP,0x00	;

movwf	TMR0		;

goto	INT_END		;

;-------------------------------------------------------------------------------------------------

NEXT2	btfss	PIR1,TMR2IF	; Check if Timer 2 interrupt

goto	INT_END		;

bcf	PIR1,TMR2IF	; clear flag

btfss	MS_USE,0x00	;

goto	IR_RD		;

decfsz	MS_CNTR,0x01	;

goto	IR_RD		;

movlw	0x04		;

movwf	MS_CNTR		;

call	MS_INFO		;

call	X_CURS		;

movf	X_MAG,0x00	;

movwf	REVTEMP		;

call	REV_ASC		;

movf	REVTMP3,0x00	;

movwf	LQDDATA		;

call	LQD_SND		;

movf	REVTMP4,0x00	;

movwf	LQDDATA		;

call	LQD_SND		;

call	Y_CURS		;

movf	Y_MAG,0x00	;

movwf	REVTEMP		;

call	REV_ASC		;

movf	REVTMP3,0x00	;

movwf	LQDDATA		;

call	LQD_SND		;

movf	REVTMP4,0x00	;

movwf	LQDDATA		;

call	LQD_SND		;

goto	INT_END		;

IR_RD	btfsc	IR,0x00		; check is front IR (0) or rear IR (1) should be checked

goto	REARIR		;

goto	FRONTIR		;

FRONTIR	bcf	IR,0x07		; for main loop to wait for valid data

btfsc	IRTEST,0x00	; check if measurement has been initiated

goto	TESTOK		; if yes go check if measurement is finished else proceed

with init

decfsz	TM2CNT,0x01	; wait for 70 ms before reading I.R.

goto	INT_END		;

movlw	0x06		; reset 70 ms counter

movwf	TM2CNT		;

bcf	IRFCLK		; make clock signal low to initiate a measurement

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfsc	IRFDAT		; check for data signal to go low (ack that a measurement

is in progress)

goto	$ - 1		;

bsf	IRTEST,0x00	; set flag to indicate a measurement has been started

goto	INT_END		;

TESTOK	btfss	IRFDAT		; check for measurement complete

goto	INT_END		; else end interrupt so other routines may continue

processing

bsf	IRFCLK		; start bit

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bcf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

IR7	bsf	IRFCLK		; bit 7

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRFDAT		;

goto	IRLOW7		;

goto	IRHIGH7		;

IRLOW7	bcf	IRDATA,0x07	;

goto	IR6		;

IRHIGH7	bsf	IRDATA,0x07	;

goto	IR6		;

IR6	call	DELAY		; bit 6

bcf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRFDAT		;

goto	IRLOW6		;

goto	IRHIGH6		;

IRLOW6	bcf	IRDATA,0x06	;

goto	IR5		;

IRHIGH6	bsf	IRDATA,0x06	;

goto	IR5		;

IR5	call	DELAY		; bit 5

bcf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRFDAT		;

goto	IRLOW5		;

goto	IRHIGH5		;

IRLOW5	bcf	IRDATA,0x05	;

goto	IR4		;

IRHIGH5	bsf	IRDATA,0x05	;

goto	IR4		;

IR4	call	DELAY		; bit 4

bcf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRFDAT		;

goto	IRLOW4		;

goto	IRHIGH4		;

IRLOW4	bcf	IRDATA,0x04	;

goto	IR3		;

IRHIGH4	bsf	IRDATA,0x04	;

goto	IR3		;

IR3	call	DELAY		; bit 3

bcf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRFDAT		;

goto	IRLOW3		;

goto	IRHIGH3		;

IRLOW3	bcf	IRDATA,0x03	;

goto	IR2		;

IRHIGH3	bsf	IRDATA,0x03	;

goto	IR2		;

IR2	call	DELAY		; bit 2

bcf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRFDAT		;

goto	IRLOW2		;

goto	IRHIGH2		;

IRLOW2	bcf	IRDATA,0x02	;

goto	IR1		;

IRHIGH2	bsf	IRDATA,0x02	;

goto	IR1		;

IR1	call	DELAY		; bit 1

bcf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRFDAT		;

goto	IRLOW1		;

goto	IRHIGH1		;

IRLOW1	bcf	IRDATA,0x01	;

goto	IR0		;

IRHIGH1	bsf	IRDATA,0x01	;

goto	IR0		;

IR0	call	DELAY		; bit 0

bcf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRFCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRFDAT		;

goto	IRLOW0		;

goto	IRHIGH0		;

IRLOW0	bcf	IRDATA,0x00	;

goto	IR_END		;

IRHIGH0	bsf	IRDATA,0x00	;

goto	IR_END		;

IR_END	bcf	IRTEST,0x00	; reset flag to indicate a measurement has not been started

bsf	IR,0x07		; valid data

goto	INT_END		;

;-------------------------------------------------------------------------------------------------

REARIR	bcf	IR,0x07		; for main loop to wait for valid data

btfsc	IRTEST,0x01	; check if measurement has been initiated

goto	TESTOKR		; if yes go check if measurement is finished else proceed

with init

decfsz	TM2CNT,0x01	; wait for 70 ms before reading I.R.

goto	INT_END		;

movlw	0x06		; reset 70 ms counter

movwf	TM2CNT		;

bcf	IRRCLK		; make clock signal low to initiate a measurement

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfsc	IRRDAT		; check for data signal to go low (ack that a measurement

is in progress)

goto	$ - 1		;

bsf	IRTEST,0x01	; set flag to indicate a measurement has been started

goto	INT_END		;

TESTOKR	btfss	IRRDAT		; check for measurement complete

goto	INT_END		; else end interrupt so other routines may continue

processing

bsf	IRRCLK		; start bit

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bcf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

IRR7	bsf	IRRCLK		; bit 7

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRRDAT		;

goto	LOW7		;

goto	HIGH7		;

LOW7	bcf	IRRDATA,0x07	;

goto	IRR6		;

HIGH7	bsf	IRRDATA,0x07	;

goto	IRR6		;

IRR6	call	DELAY		; bit 6

bcf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRRDAT		;

goto	LOW6		;

goto	HIGH6		;

LOW6	bcf	IRRDATA,0x06	;

goto	IRR5		;

HIGH6	bsf	IRRDATA,0x06	;

goto	IRR5		;

IRR5	call	DELAY		; bit 5

bcf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRRDAT		;

goto	LOW5		;

goto	HIGH5		;

LOW5	bcf	IRRDATA,0x05	;

goto	IRR4		;

HIGH5	bsf	IRRDATA,0x05	;

goto	IRR4		;

IRR4	call	DELAY		; bit 4

bcf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRRDAT		;

goto	LOW4		;

goto	HIGH4		;

LOW4	bcf	IRRDATA,0x04	;

goto	IRR3		;

HIGH4	bsf	IRRDATA,0x04	;

goto	IRR3		;

IRR3	call	DELAY		; bit 3

bcf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRRDAT		;

goto	LOW3		;

goto	HIGH3		;

LOW3	bcf	IRRDATA,0x03	;

goto	IRR2		;

HIGH3	bsf	IRRDATA,0x03	;

goto	IRR2		;

IRR2	call	DELAY		; bit 2

bcf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRRDAT		;

goto	LOW2		;

goto	HIGH2		;

LOW2	bcf	IRRDATA,0x02	;

goto	IRR1		;

HIGH2	bsf	IRRDATA,0x02	;

goto	IRR1		;

IRR1	call	DELAY		; bit 1

bcf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRRDAT		;

goto	LOW1		;

goto	HIGH1		;

LOW1	bcf	IRRDATA,0x01	;

goto	IRR0		;

HIGH1	bsf	IRRDATA,0x01	;

goto	IRR0		;

IRR0	call	DELAY		; bit 0

bcf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

call	DELAY		;

bsf	IRRCLK		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

btfss	IRRDAT		;

goto	LOW0		;

goto	HIGH0		;

LOW0	bcf	IRRDATA,0x00	;

goto	IRR_END		;

HIGH0	bsf	IRRDATA,0x00	;

goto	IRR_END		;

IRR_END	bcf	IRTEST,0x01	; reset flag to indicate a measurement has not been started

bsf	IR,0x07		; for main loop to wait for valid data

goto	INT_END		;

;-------------------------------------------------------------------------------------------------

INT_END	movf	W_TEMP,0x00	; restore w reg contents

retfie			;

;*******************************************************************************

;

;	SUBROUTINES

;

;*******************************************************************************

;-------------------------------------------------------------------------------

;	READ 1 bit of data from mouse

;	Data is sent LSB first.

;-------------------------------------------------------------------------------

MS_BIT	btfss	MSCLOCK		; make sure clock is high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for falling edge

goto	$ - 1		;

btfss	MSDATA		; check if data is '1' or '0'

goto	BIT_0		;

bsf	STATUS,0x00	;

goto	BIT_Rx		;

BIT_0	bcf	STATUS,0x00	;

BIT_Rx	rrf	MSINFO,0x01	; rotate new bit into temp reg

return			;

;-------------------------------------------------------------------------------

;	INFORM MOUSE OF INCOMING Tx and Then Initialize the mouse

;	Holds mouse clock line low for at least 100 uSec

;	At an .0000002 sec per instruction that takes 500 instruction cycles.

;-------------------------------------------------------------------------------

MS_TX	movlw	0xA6		; 166

movwf	COUNT1		;

bsf	MSCCTRL		; pull mouse clock line low for >= 100 uSec

decfsz	COUNT1,0x01	; worth one instruction cycle until count1 = 0

goto	$ - 1		; worth two instruction cycles

; thus 166 * 3 = 498 Instruction cycles

; plus 1 for the decfsz on the last count

; plus the bsf below = 500 I.C.'s

MS_TXR	bsf	MSDCTRL		; pull mouse data line low

bcf	MSCCTRL		; release the mouse clock line

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bsf	MSDCTRL		; Start Bit = 0

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bsf	MSDCTRL		; Bit 0 = 0

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bsf	MSDCTRL		; Bit 1 = 0

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bcf	MSDCTRL		; Bit 2 = 1

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bsf	MSDCTRL		; Bit 3 = 0

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bcf	MSDCTRL		; Bit 4 = 1

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bcf	MSDCTRL		; Bit 5 = 1

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bcf	MSDCTRL		; Bit 6 = 1

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bcf	MSDCTRL		; Bit 7 = 1

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bsf	MSDCTRL		; Parity Bit = 0

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSCLOCK		; wait for mouse to pull clock low

goto	$ - 1		;

bcf	MSDCTRL		; Stop Bit = 1

btfss	MSCLOCK		; wait for mouse to pull clock high

goto	$ - 1		;

btfsc	MSDATA		; wait for mouse to pull data low

goto	$ - 1		;

btfss	MSDATA		; wait for mouse to release data

goto	$ - 1		;

bcf	MSCCTRL		; enable mouse tx

bcf	MSDCTRL		;

movlw	0x08		; Initialize counter for 8 bits

movwf	COUNT2		;

btfsc	MSCLOCK		; wait for mouse to bring clock low

goto	$ - 1		;

btfsc	MSDATA		; wait for mouse to bring data low

goto	$ - 1		;

READB2	call	MS_BIT		; get one bit

decfsz	COUNT2,0x01	; decrement counter

goto	READB2		; get next bit

btfss	MSCLOCK		; make sure clock is high

goto	$ - 1		;				PARITY

btfsc	MSCLOCK		; wait for falling edge

goto	$ - 1		;

btfss	MSCLOCK		; make sure clock is high

goto	$ - 1		;				STOP

btfsc	MSCLOCK		; wait for falling edge

goto	$ - 1		;

btfss	MSCLOCK		; wait for clock to float high again

goto	$ - 1		;

bsf	MSCCTRL		; pull mouse clock line low (disable transmission)

return			;

;-------------------------------------------------------------------------------

;	READ ONE PACKET OF DATA FROM MOUSE

;-------------------------------------------------------------------------------

MS_INFO	bcf	MSCCTRL		; enable mouse tx

bcf	MSDCTRL		;

movlw	0xFF		; This rountine may try to find the beginning of a mouse

data packet 255 times

movwf	INCNT		; If 255 tries all fail the routine times out and exits

leaving the previous values intact

TIMEIN	decfsz	INCNT,0x01	;

goto	TOPL		;

goto	T_OUT		; here is the timeout exit point

TOPL	movlw	0x7F		; 127 * .2usec per instruction * about 6 instructions = 152

usec period of clock inactivity

movwf	OUTCNT		; if the mouse clock is not inactive for this period of

time the rountine has tried to read

OUTERL	decfsz	OUTCNT,0x01	; mouse data in the middle of a packet (who knows what bit

is on) so try again until a timeout occurs

goto	KLP		;

goto	LOOK		;

KLP	btfss	MSCLOCK		; here is where the clock is checked as long as the clock

remains in its inactive state (high)

goto	TIMEIN		; for the duration of the counter (OUTCNT) then the mouse

is between packets and the next clock

goto	OUTERL		; edge (falling) will be the start bit of the first byte of

data in a new packet.

LOOK	movlw	0x08		; Initialize counter for 8 bits

movwf	COUNT2		;

movlw	0xFF		;

movwf	TOUT_3		;

CIL3	decfsz	TOUT_3,0x01	;

goto	CIL4		;

goto	T_OUT		;

CIL4	movlw	0xFF		;

movwf	TOUT_2		;

CHECCK0	btfsc	MSCLOCK		; wait for mouse to bring clock low

goto	CIL1		;

goto	CIL2		;

CIL1	decfsz	TOUT_2,0x01	;

goto	CHECCK0		;

goto	CIL3		;

CIL2	btfsc	MSDATA		; wait for mouse to bring data low

goto	$ - 1		;

READB	call	MS_BIT		; get one bit

decfsz	COUNT2,0x01	; decrement counter

goto	READB		; get next bit

btfss	MSCLOCK		; make sure clock is high

goto	$ - 1		;				PARITY

btfsc	MSCLOCK		; wait for falling edge

goto	$ - 1		;

btfss	MSCLOCK		; make sure clock is high

goto	$ - 1		;				STOP

btfsc	MSCLOCK		; wait for falling edge

goto	$ - 1		;

btfss	MSCLOCK		; wait for clock to float high again

goto	$ - 1		;

btfss	MSINFO,0x02	;

goto	OK_01		;

goto	RD_END		;

OK_01	btfsc	MSINFO,0x03	;

goto	OK_02		;

goto	RD_END		;

OK_02	movf	MSINFO,0x00	;

INFOOK	movwf	BUTTONS		;

movlw	0x08		; Initialize counter for 8 bits

movwf	COUNT2		;

CHECCK1	btfsc	MSCLOCK		; wait for mouse to bring data low

goto	$ - 1		; (get start bit)

btfsc	MSDATA		; wait for mouse to bring clock low

goto	$ - 1		;

READX	call	MS_BIT		; get one bit

decfsz	COUNT2,0x01	; decrement counter

goto	READX		; get next bit

btfss	MSCLOCK		; make sure clock is high

goto	$ - 1		;				PARITY

btfsc	MSCLOCK		; wait for falling edge

goto	$ - 1		;

btfss	MSCLOCK		; make sure clock is high

goto	$ - 1		;				STOP

btfsc	MSCLOCK		; wait for falling edge

goto	$ - 1		;

btfss	MSCLOCK		; wait for clock to float high again

goto	$ - 1		;

movf	MSINFO,0x00	;

movwf	X_MAG		;

movlw	0x08		; Initialize counter for 8 bits

movwf	COUNT2		;

CHECCK2	btfsc	MSCLOCK		; wait for mouse to bring data low

goto	$ - 1		; (get start bit)

btfsc	MSDATA		; wait for mouse to bring clock low

goto	$ - 1		;

READM	call	MS_BIT		; get one bit

decfsz	COUNT2,0x01	; decrement counter

goto	READM		; get next bit

btfss	MSCLOCK		; make sure clock is high

goto	$ - 1		;				PARITY

btfsc	MSCLOCK		; wait for falling edge

goto	$ - 1		;

btfss	MSCLOCK		; make sure clock is high

goto	$ - 1		;				STOP

btfsc	MSCLOCK		; wait for falling edge

goto	$ - 1		;

movf	MSINFO,0x00	;

movwf	Y_MAG		;

goto	RD_END		;

T_OUT	clrf	BUTTONS		;

RD_END	bsf	MSCCTRL		;

return			;

;-------------------------------------------------------------------------------

X_CURS	bcf	E_LQD		;  set cursor address

bcf	RW_LQD		;

bcf	RS_LQD		;

movlw	0x85		;

movwf	LQD		;

bsf	E_LQD		;

nop			;

nop			;

bcf	E_LQD		;

call	CHK_BSY		;

nop			;

nop			;

return			;

;-------------------------------------------------------------------------------

Y_CURS	bcf	E_LQD		;  set cursor address

bcf	RW_LQD		;

bcf	RS_LQD		;

movlw	0xC5		;

movwf	LQD		;

bsf	E_LQD		;

nop			;

nop			;

bcf	E_LQD		;

call	CHK_BSY		;

nop			;

nop			;

return			;

;-------------------------------------------------------------------------------

;	Send a character to the LQD

;	Character to be sent should be in register "LQDDATA"

;-------------------------------------------------------------------------------

LQD_SND	bcf	E_LQD		;

bcf	RW_LQD		;

bsf	RS_LQD		;

movf	LQDDATA,0x00	;

movwf	LQD		; send char info to data bus

bsf	E_LQD		; enable instruction

nop			;

nop			;

bcf	E_LQD		;

call	CHK_BSY		;

return			;

;-------------------------------------------------------------------------------

;	LQDINIT .........DUH!

;-------------------------------------------------------------------------------

LQDINIT	call	CHK_BSY		;

bcf	E_LQD		;

bcf	RW_LQD		;

bcf	RS_LQD		;

movlw	0x38		;

movwf	LQD		;

bsf	E_LQD		;

nop			;

bcf	E_LQD		;

call	CHK_BSY		;

nop			;

nop			;

bcf	E_LQD		;

bcf	RW_LQD		;

bcf	RS_LQD		;

movlw	0x0E		;

movwf	LQD		;

bsf	E_LQD		;

nop			;

bcf	E_LQD		;

call	CHK_BSY		;

nop			;

nop			;

bcf	E_LQD		;

bcf	RW_LQD		;

bcf	RS_LQD		;

movlw	0x06		;

movwf	LQD		;

bsf	E_LQD		;

nop			;

bcf	E_LQD		;

call	CHK_BSY		;

nop			;

nop			;

bcf	E_LQD		;

bcf	RW_LQD		;

bcf	RS_LQD		;

movlw	0x01		;

movwf	LQD		;

bsf	E_LQD		;

nop			;

bcf	E_LQD		;

call	CHK_BSY		;

nop			;

nop			;

movlw	0x58		;

movwf	LQDDATA		;

call	LQD_SND		;

movlw	0x20		;

movwf	LQDDATA		;

call	LQD_SND		;

movlw	0x3D		;

movwf	LQDDATA		;

call	LQD_SND		;

movlw	0x20		;

movwf	LQDDATA		;

call	LQD_SND		;

bcf	E_LQD		;  set cursor address

bcf	RW_LQD		;

bcf	RS_LQD		;

movlw	0xC0		;

movwf	LQD		;

bsf	E_LQD		;

nop			;

nop			;

bcf	E_LQD		;

call	CHK_BSY		;

nop			;

nop			;

movlw	0x59		;

movwf	LQDDATA		;

call	LQD_SND		;

movlw	0x20		;

movwf	LQDDATA		;

call	LQD_SND		;

movlw	0x3D		;

movwf	LQDDATA		;

call	LQD_SND		;

movlw	0x20		;

movwf	LQDDATA		;

call	LQD_SND		;

return			;

;-------------------------------------------------------------------------------

CHK_BSY	bsf	RW_LQD		; check for busy flag = 0

bcf	RS_LQD		;

bsf	STATUS,RP0	;	select bank 1

bcf	STATUS,RP1	;	select bank 1

movlw	0xFF		;

movwf	TRISD		;

bcf	STATUS,RP0	;	select bank 0

bcf	STATUS,RP1	;	select bank 0

KEEPCHK	bcf	E_LQD		;

nop			;

bsf	E_LQD		;

nop			;

btfsc	BUSYFL		;

goto	KEEPCHK		;

bcf	E_LQD		;

bsf	STATUS,RP0	;	select bank 1

bcf	STATUS,RP1	;	select bank 1

movlw	0x00		;

movwf	TRISD		;

bcf	STATUS,RP0	;	select bank 0

bcf	STATUS,RP1	;	select bank 0

return			;

;-------------------------------------------------------------------------------

REV_ASC	movf	STATUS,0x00	;

movwf	S_TEMP		;

bcf	STATUS,RP0	;

bcf	STATUS,RP1	;

movf	REVTEMP,0x00	;

movwf	REVTMP2		;

bcf	REVTMP2,0x00	;

bcf	REVTMP2,0x01	;

bcf	REVTMP2,0x02	;

bcf	REVTMP2,0x03	;

swapf	REVTMP2,0x01	;

movf	REVTMP2,0x00	;

sublw	0x09		;

btfsc	STATUS,0x00	;

goto	NMBR		;

goto	LTTR		;

NMBR	movlw	0x30		;

addwf	REVTMP2,0x00	;

movwf	REVTMP3		;

goto	REVNEXT		;

LTTR	movlw	0x37		;

addwf	REVTMP2,0x00	;

movwf	REVTMP3		;

goto	REVNEXT		;

REVNEXT	movf	REVTEMP,0x00	;

movwf	REVTMP2		;

bcf	REVTMP2,0x04	;

bcf	REVTMP2,0x05	;

bcf	REVTMP2,0x06	;

bcf	REVTMP2,0x07	;

movf	REVTMP2,0x00	;

sublw	0x09		;

btfsc	STATUS,0x00	;

goto	NMBR2		;

goto	LTTR2		;

NMBR2	movlw	0x30		;

addwf	REVTMP2,0x00	;

movwf	REVTMP4		;

goto	REVDONE		;

LTTR2	movlw	0x37		;

addwf	REVTMP2,0x00	;

movwf	REVTMP4		;

goto	REVDONE		;

REVDONE	movf	S_TEMP,0x00	;

movwf	STATUS		;

return			;

;-------------------------------------------------------------------------------

DELAY	movlw	0xFA		;

movwf	COUNT		; 250 cycles

decfsz	COUNT,0x01	;

goto	$ - 1		;

return			;

;-------------------------------------------------------------------------------

DELAY2	movf	SECONDS,0x00	;

movwf	NDELAY		;

bsf	MOTL_OE		;

bsf	MOTR_OE		;

D2NL	decfsz	NDELAY,0x01	; Delay causes a SECONDS second long delay

goto	D2OL		;

goto	D2_EXIT		;

D2OL	movlw	0xFF		;

movwf	ODELAY		;

D2OL2	decfsz	ODELAY,0x01	;

goto	D2IL		;

goto	D2NL		;

D2IL	movlw	0x9D		;

movwf	IDELAY		;

D2IL2	decfsz	IDELAY,0x01	;

goto	D2NOP		;

goto	D2OL2		;

D2NOP	goto	D2IL2		;

D2_EXIT	return			;

;-------------------------------------------------------------------------------

; 90 degree turn routine (to the right)

;-------------------------------------------------------------------------------

NDT_R	movlw	0x92		;

movwf	RPULSES		;

bsf	MOTL_DR		;

bsf	MOTR_DR		;

bcf	MOTL_OE		;

bcf	MOTR_OE		;

bsf	NAV_OK,0x00	;

bsf	INTCON,GIE	; enable unmasked interrupts

CK_P	movf	RPULSES,0x00	;

btfss	STATUS,0x02	;

goto	CK_P		;

goto	DWRT		;

DWRT	bcf	INTCON,GIE	; enable unmasked interrupts

bsf	MOTL_OE		;

bsf	MOTR_OE		;

bcf	NAV_OK,0x00	;

return			;

;-------------------------------------------------------------------------------

; 45 degree turn routine (to the right)

;-------------------------------------------------------------------------------

FFDT_R	movlw	0x4A		;

movwf	RPULSES		;

bsf	MOTL_DR		;

bsf	MOTR_DR		;

bcf	MOTL_OE		;

bcf	MOTR_OE		;

bsf	NAV_OK,0x00	;

bsf	INTCON,GIE	; enable unmasked interrupts

CK_PZ	movf	RPULSES,0x00	;

btfss	STATUS,0x02	;

goto	CK_PZ		;

goto	DWFFRT		;

DWFFRT	bcf	INTCON,GIE	; enable unmasked interrupts

bsf	MOTL_OE		;

bsf	MOTR_OE		;

bcf	NAV_OK,0x00	;

return			;

;-------------------------------------------------------------------------------

; 90 degree turn routine (to the left)

;-------------------------------------------------------------------------------

NDT_L	movlw	0x92		;

movwf	RPULSES		;

bcf	MOTL_DR		;

bcf	MOTR_DR		;

bcf	MOTL_OE		;

bcf	MOTR_OE		;

bsf	NAV_OK,0x00	;

bsf	INTCON,GIE	; enable unmasked interrupts

CK_P2	movf	RPULSES,0x00	;

btfss	STATUS,0x02	;

goto	CK_P2		;

goto	DWLT		;

DWLT	bcf	INTCON,GIE	; enable unmasked interrupts

bsf	MOTL_OE		;

bsf	MOTR_OE		;

bcf	NAV_OK,0x00	;

return			;

;-------------------------------------------------------------------------------

; 45 degree turn routine (to the left)

;-------------------------------------------------------------------------------

FFDT_L	movlw	0x4A		;

movwf	RPULSES		;

bcf	MOTL_DR		;

bcf	MOTR_DR		;

bcf	MOTL_OE		;

bcf	MOTR_OE		;

bsf	NAV_OK,0x00	;

bsf	INTCON,GIE	; enable unmasked interrupts

CK_P2Z	movf	RPULSES,0x00	;

btfss	STATUS,0x02	;

goto	CK_P2Z		;

goto	DWFFLT		;

DWFFLT	bcf	INTCON,GIE	; enable unmasked interrupts

bsf	MOTL_OE		;

bsf	MOTR_OE		;

bcf	NAV_OK,0x00	;

return			;

;-------------------------------------------------------------------------------

; this rountine takes in the value of 1/10ths inches (actually .047 inches per pulse)

; to move in the register DIST

;-------------------------------------------------------------------------------

M_F	movf	DIST,0x00	;

movwf	RPULSES		;

bcf	MOTL_DR		;

bsf	MOTR_DR		;

bcf	MOTL_OE		;

bcf	MOTR_OE		;

bsf	NAV_OK,0x00	;

bsf	INTCON,GIE	; enable unmasked interrupts

CK_P3	movf	RPULSES,0x00	;

btfss	STATUS,0x02	;

goto	CK_P3		;

goto	DM_F		;

DM_F	bcf	INTCON,GIE	; enable unmasked interrupts

bsf	MOTL_OE		;

bsf	MOTR_OE		;

bcf	NAV_OK,0x00	;

return			;

;-------------------------------------------------------------------------------

; this rountine takes in the value of 1/10ths inches (actually .047 inches per pulse)

; to move in the register DIST

;-------------------------------------------------------------------------------

M_B	movf	DIST,0x00	;

movwf	RPULSES		;

bsf	MOTL_DR		;

bcf	MOTR_DR		;

bcf	MOTL_OE		;

bcf	MOTR_OE		;

bsf	NAV_OK,0x00	;

bsf	INTCON,GIE	; enable unmasked interrupts

BACKW	movf	RPULSES,0x00	;

btfss	STATUS,0x02	;

goto	BACKW		;

goto	DM_F12		;

DM_F12	bcf	INTCON,GIE	; enable unmasked interrupts

bsf	MOTL_OE		;

bsf	MOTR_OE		;

bcf	NAV_OK,0x00	;

return			;

;-------------------------------------------------------------------------------

;ACC rountine accelerates the robot to the desired wheel speed to avoid stalls

;-------------------------------------------------------------------------------

ACC	btfss	NAV_OK,0x01	;

goto	R_WH		;

goto	L_W		;

R_WH	movf	RWT,0x01	; check if desired time is zero

btfss	STATUS,0x02	;

goto	RWNOT_Z		;

goto	SLOW_RW		;

SLOW_RW	clrf	RWT_TMP		;

return			;

RWNOT_Z	decfsz	ACCRCNT,0x01	;

return			;

movf	MAX_ACC,0x00	;

movwf	ACCRCNT		;

movf	RWT,0x00	; get user desired wheel time

subwf	RWT_TMP,0x00	; compare current value with dersired value

btfss	STATUS,0x00	;

goto	ACC_RP		; negative acceleration needed

goto	ACC_RN		; positive acceleration needed

ACC_RN	movf	RWT,0x00	; get user desired wheel time

subwf	RWT_TMP,0x00	; compare current value with dersired value

btfss	STATUS,0x02	;

goto	KEEPRAN		;

goto	DONERAN		;

KEEPRAN	decf	RWT_TMP,0x01	; decrement wheel time by one

DONERAN	return			;

ACC_RP	movf	RWT,0x00	; compare current value with dersired value

subwf	RWT_TMP,0x00	;

btfss	STATUS,0x02	;

goto	KEEPRAP		;

goto	DONERAP		;

KEEPRAP	incf	RWT_TMP,0x01	; increment wheel time by one

DONERAP	return			;

L_W	movf	LWTH,0x01	; check if desired time is zero

btfss	STATUS,0x02	;

goto	LWNOT_Z		;

goto	SLOW_LW		;

SLOW_LW	clrf	LWTHTMP		;

return			;

LWNOT_Z	decfsz	ACCLCNT,0x01	;

return			;

movf	MAX_ACC,0x00	;

movwf	ACCLCNT		;

movf	LWTH,0x00	; get user desired wheel time

subwf	LWTHTMP,0x00	; compare current value with dersired value

btfss	STATUS,0x00	;

goto	ACC_LP		; negative acceleration needed

goto	ACC_LN		; positive acceleration needed

ACC_LN	movf	LWTH,0x00	; get user desired wheel time

subwf	LWTHTMP,0x00	; compare current value with dersired value

btfss	STATUS,0x02	;

goto	KEEPLAN		;

goto	DONELAN		;

KEEPLAN	decf	LWTHTMP,0x01	; decrement wheel time by one

DONELAN	return			;

ACC_LP	movf	LWTH,0x00	; compare current value with dersired value

subwf	LWTHTMP,0x00	;

btfss	STATUS,0x02	;

goto	KEEPLAP		;

goto	DONELAP		;

KEEPLAP	incf	LWTHTMP,0x01	; increment wheel time by one

DONELAP	return			;

;-------------------------------------------------------------------------------

;SQUARE rountine will drive the robot in a square shape with dimensions DIST x DIST

;-------------------------------------------------------------------------------

SQUARE	movlw	0x64		; 100 decimal

movwf	DIST		;

call	M_F		;

call	DELAY2		;

call	NDT_R		;

call	DELAY2		;

movlw	0x64		; 100 decimal

movwf	DIST		;

call	M_F		;

call	DELAY2		;

call	NDT_R		;

call	DELAY2		;

movlw	0x64		; 100 decimal

movwf	DIST		;

call	M_F		;

call	DELAY2		;

call	NDT_R		;

call	DELAY2		;

movlw	0x64		; 100 decimal

movwf	DIST		;

call	M_F		;

call	DELAY2		;

call	NDT_R		;

call	DELAY2		;

return			;

;-------------------------------------------------------------------------------

;CIRCLE will drive robot in a circle

;-------------------------------------------------------------------------------

CIRCLE	movlw	0x80		;

movwf	LWTH		;

movlw	0x00		;

movwf	RWT		;

bcf	MOTL_DR		; move forward

bsf	MOTR_DR		;

bcf	MOTL_OE		;

bcf	MOTR_OE		;

call	DELAY2		;

return			;

;-------------------------------------------------------------------------------

LETTERG	bcf	MARKER		;

call	DELAY2		;

movlw	0x7F		;	* draw left portion of G

movwf	DIST		;	*

call	M_F		;	*

call	DELAY2		;	*

bsf	MARKER		; lift up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		; * * * * Top part of G

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; lift up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x20		;

movwf	DIST		;

call	M_F		;	*** PART of G

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movlw	0x20		;

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_L		; turn 90 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;	*

call	M_F		;	* PART of G

call	DELAY2		;	*

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_F		;

call	DELAY2		; *******  PART of G

bsf	MARKER		; pick up pen

call	DELAY2		;

movlw	0x40		; get ready for next letter

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movlw	0x20		; space between letters

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

return			;

;-------------------------------------------------------------------------------

LETTERA	bcf	MARKER		;

call	DELAY2		;

movlw	0x7F		;	* draw left portion of A

movwf	DIST		;	*

call	M_F		;	*

call	DELAY2		;	*

bsf	MARKER		; lift up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		; * * * * Top part of A

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; lift up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		;

call	DELAY2		;

movlw	0x7F		;	* draw right portion of A

movwf	DIST		;	*

call	M_F		;	*

call	DELAY2		;	*

bsf	MARKER		; lift up pen

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		; Middle part of A ****

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; lift up pen

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_L		; turn 90 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movlw	0x40		;  get ready for next letter

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_L		; turn 90 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movlw	0x20		; space between letters

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_L		; turn 90 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

return			;

;-------------------------------------------------------------------------------

LETTERT	bsf	MARKER		;

call	DELAY2		;

movlw	0x7F		;

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		;	DRAW TOP of T *********

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movlw	0x20		;

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x7F		; DRAW VERTICAL PART of T 	*

movwf	DIST		;				*

call	M_F		;				*

call	DELAY2		;				*

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_L		; turn 90 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movlw	0x20		;

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_L		; turn 90 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

return			;

;-------------------------------------------------------------------------------

LETTERO	bcf	MARKER		;

call	DELAY2		;

movlw	0x7F		;	DRAW LEFT VERT OF O

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		;	DRAW TOP OF O

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x7F		;	DRAW RIGHT VERT OF O

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		;	DRAW BOTTOM OF O

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movlw	0x20		; space between letters

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

return			;

;-------------------------------------------------------------------------------

LETTERR	bcf	MARKER		;

call	DELAY2		;

movlw	0x7F		;	DRAW LEFT VERT OF R

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		;	DRAW TOP OF R

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		;	DRAW RIGHT VERT OF R

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		;	DRAW MIDDLE HORZ OF R

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_L		; turn 90 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	FFDT_L		; turn 45 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x5A		;	DRAW DIAGONAL OF R

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	FFDT_L		; turn 45 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movlw	0x20		; space between letters

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_L		; turn 90 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

return			;

;-------------------------------------------------------------------------------

LETTERS	bsf	MARKER		; pick up pen

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		; draw left vert of S

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		; draw TOP of S

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		; draw MIDDLE of S

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_L		; turn 90 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		; draw RIGHT VERT of S

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bcf	MARKER		; put down pen

call	DELAY2		;

movlw	0x40		; draw MIDDLE of S

movwf	DIST		;

call	M_F		;

call	DELAY2		;

bsf	MARKER		; pick up pen

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movlw	0x20		;

movwf	DIST		;

call	M_B		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

return			;

;-------------------------------------------------------------------------------

BLANK_S	bsf	MARKER		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_R		; turn 90 degrees to the right

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movlw	0x40		;

movwf	DIST		;

call	M_F		;

call	DELAY2		;

movf	MALIGNB,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_B		;

call	DELAY2		;

call	NDT_L		; turn 90 degrees to the left

call	DELAY2		;

movf	MALIGNF,0x00	; Approx 2.75 inches to re-align the pen

movwf	DIST		;

call	M_F		;

call	DELAY2		;

return			;

;-------------------------------------------------------------------------------

end;