Sensor Report

Hamelin

IMDL Spring 2005

Matt King

April 19, 2005

Introduction

The main objective was to make my robot move according to different musical pitches.  I built a tone detection circuit and I used an IR sensor to detect the edge-of-the-world or a black line for line following.  I also used bump switches to prevent my robot from running into things.  I will restrict this discussion to my special sensor, the tone detection circuit.  The bump switches and IR configuration that I used were very straightforward and can easily be obtained off of any datasheet.

Special Sensor

Overview

My tone detection circuit consisted of a bank of 567 chips tuned to different frequencies.  I used a small microphone to pick up the notes and then the signal went through an opamp before going to the 567s.  I connected the outputs of the 567 chips to a grid of LEDs and then to the microprocessor.

567 Chips

The 567 chips are 8 pin ICs.  They utilize a phase locked loop design.  They have an analog input and an active low digital output.  Pins 5 & 6 set the characteristic frequency for each chip.  A resistor is connected across pins 5 & 6 and a capacitor is connected to pin 6 and ground.  To obtain certain frequencies I used the formula:

f=1/(RC*1.07)

At first I did extensive calculations and had to find common resistor/capacitor values to give me my desired frequency.  I eventually realized that I should use 10K 15 turn potentiometers to tune the frequency.  The chips are not that accurate and neither is any sound source.  I realized that I got the best results by using a tone generator program and turning the pots till the LED activated.  I then would play the note on my horn and have someone again tune the pots.

Pin 3 is the signal input and the datasheet recommends having a minimum of 200mV.  Pin 8 is the output.  It is normally 5V but when a frequency match occurs it goes to 0V and stays there until the frequency is no longer detected.  I simply ran all of the outputs of the different chips into the external interrupt pins on the Atmega 128 and I set the interrupts to be low level triggered.

Pin 1 is the band limiting capacitor.  It basically makes sure there are no spurious outputs.  I used a 10uF capacitor.  Pin 2 is the lowpass filter.  I used a 1uF capacitor.  The data sheet overstated the values and I obtained better results by using these lower values.

Opamp

I used a LM1458 dual opamp.  I only used one side however.  Pin 1 is the output.  Pin 2 is the inverting input terminal.  This is where I connected the signal.  I connected a coupling capacitor of .47uF in between them.  I found that if I used a 680K feedback resistor that it gave a good average gain value to allow my sax to be heard and ignore most ambient noise.  Pin 3 is the noninverting terminal and I connected two equal resistors in a voltage divider configuration between 5V and ground.  This biased my signal to be around the 2.5V range.  I used another capacitor between the output of the opamp and input of the 567 chips of .01uF.

Microphone

I used a 9.7x6.6mm omnidirectional microphone manufactured by Horn in Japan.  It had a sensitivity of -44dB and a signal to noise ratio of 60 dB.  It worked better than the one that I started off with from Radio Shack but it was not a huge improvement.

Parts

Part                                  Distributor               Price

Microphone                                   www.digikey.com             $1.00

JRC567                                          www.digikey.com	 $1.00 each

LM1458	                               www.digikey.com	 $1.00

Potentiometers                               Radio Shack                      $2.50 each

Contact Switches SW132-ND       www.digikey.com             $2.50 each

IR Sensors OPB745                       www.digikey.com             $3.00 each