AUTOMATED GUIDED VEHICLE:

WRITTEN REPORT 1

Prepared for:	A. A Arroyo, Instructor

Eric M. Schwartz, Instructor

William Dubel, TA

Steven Pickles, TA

Prepared by:	Trevor Skipp, Student

January 25, 2005

University of Florida

Department of Electrical and Computer Engineering

EEL 5666:  Intelligent Machines Design Laboratory

CONTENTS

1. Abstract………………………………………………………………………...……… 3

2. Executive Summary…………………………………………………………………… 4

3. Introduction………………………………………………………………………….… 5

4. Integrated System……………………………………………………………………… 6

5. Mobile Platform…………………………………………………………………..…… 7

6. Actuation………………………………………………………………………………. 8

7. Sensors………………………………………………………………………………… 9

8. Behaviors…………………………………………………………………………….. 10

9. Experimental Layout and Results……………………………………………………. 11

10. Conclusion……………………………………………………………………….…. 12

11. Documentation……………………………………………………………………… 13

12. Appendices………………………………………………………………………….. 14

ABSTRACT

“Automated Guided Vehicle”

By Trevor Skipp

High demands on manufacturers have left their shipping warehouses in havoc.  Human error has a negative effect on safety, efficiency, and quality.  These expenses are reduced with the introduction of an Automated Guided Vehicle, AGV.  A driverless, intelligent forklift uses an optical path to quickly and safely traverse a warehouse.  Its capabilities are enhanced by the ability to send and receive tasks through RF data communication.

EXECUTIVE SUMMARY

INTRODUCTION

Advancements in manufacturing technology allow companies to rapidly produce products.  This has provoked a trend to reduce bulk inventory in favor of short term supplies.  Although this allows corporations more financial freedom, it requires warehouses to accommodate temporary, selective storage.  Improved product handling and speed can be achieved with the implementation of an Automated Guided Vehicle, AGV.

In a traditional warehouse, human safety governs the productivity.  With the help of intelligent computers, the AGV can safely achieve higher speeds.  Precision turning allows it to accurately navigate in tight spaces.

The AGV is highly flexible as a result of remote communication.  Its ability to communicate with other autonomous vehicles provides a seamless operation.  Continuous coordination between vehicles delivers money saving efficiency.

The introduction of unmanned vehicles onto a warehouse floor has favorable effects on safety.  With the aid of environmental sensors, the AGV can detect objects in its collision path.  Automation eliminates vehicle traffic jams and their potential for accidents.

For companies building new warehouses, there are many monetary benefits to investing in intelligent machinery.  The workforce required to run the warehouse and the additional overhead (e.g., insurance) required to support that overhead will be drastically reduced.  Increased product turn-around and faster shipping will result in more satisfied customers.  Also, automation reduces the risks of personal injury.

INTEGRATED SYSTEM

The AGV will operate in a model warehouse, built to scale.  Its primary task will be to relocate pallets in the warehouse.  A user input, via an IR remote control, will notify the AGV that a package has arrived, along with its corresponding part number.  The pallet will be picked up and dropped off at one of several docks at the other end of the warehouse.  Through RF communication, the AGV will tell another autonomous vehicle, the ASRS, which dock the pallet is located at.  The AGV will also relay the pallet’s part number.  The AGV will be able to receive pallet retrieval requests from the remote control.  It will relay the request to the ASRS, pick up the pallet, and deliver it to an outgoing pallet area.

The AGV will be controlled by Microchip’s PIC18F8720 microcontroller.  The processor was chosen for its performance and cost.  Key features include 128KB of addressing space, two integrated RS-232 serial ports, five timers, five Capture/ Compare/ PWM modules, four external interrupts, sixty eight input/ output pins, and low power consumption.  The AGV’s functionality will be achieved by interfacing the processor with external circuitry.

A dual motor controller will accept pulse width modulation signals from the processor.  This will allow the motors to vary in speed and direction.  A five volt regulator will drive select logic chips and sensors.  Additional hardware will support analog sensors.  A LCD screen will be interfaced with the microprocessor.  It will provide the designer with a valuable debugging tool.

MOBILE PLATFORM

I plan for the AGV to be small, light, and agile (Figure 1).  This will be achieved by constructing it from model aircraft balsa wood.  The back of the vehicle will be supported by two large tires, and stability will be maintained with two front casters. 				     Figure 1.

The body will measure 4.5 inches wide, 5 inches long, and 6 inches high.  The LCD screen will be mounted on the back of the robot.

ACTUATION

The AGV will be propelled by two D.C. gear head motors, which will attach to the rear tires.  The AGV will have an excellent turning radius as a result of each wheel’s independent drive.

The forklift mechanism will be powered by a servo motor.  When the servo arm is parallel to the chassis, the fork will be in the down position.  The fork will rise when the arm is perpendicular to the chassis.  This system will allow the AGV to pick model pallets up from the ground.

SENSORS

The AGV will use a total of four infrared sensors to detect a line on the warehouse floor.  Two sensors will be placed side by side in the middle of the chassis.  Software will allow the AGV to follow a black line.  The other sensors will be placed three inches apart on the bottom of the chassis, and the processor will be able to detect intersecting black lines.

The AGV will utilize two infrared sensors to detect obstacles in its forward collision path, and another sensor will detect obstacles in its rear collision path.  If an obstacle is detected, the AGV will stop and wait a predetermined length of time before testing to see if the obstacle is still present.

As a backup and for additional safety, two bump sensors will be integrated into the rear bumper.  These will only be activated if the AGV strikes something while reversing.  If this scenario occurs, the vehicle will immediately shut off.

An RF transceiver will give the AGV the capability to communicate with another autonomous vehicle on the warehouse floor.  The two machines will coordinate pallet location and whether pallets are waiting to be shipped or received.  The AGV will need user input to know when a package enters the warehouse and when to ship one out.  This will be realized using an infrared television remote control.

BEHAVIORS

The AGV will require several behaviors to meet its objective: user input, line following, obstacle detection, and RF communication.

The operator will be able to tell the AGV that a package is waiting in the input terminal.  This will be achieved by using an infrared television remote control.  The operator will simply enter a “1” followed by the part number of the pallet.  The AGV will coordinate its actions with the Automated Storage and Retrieval System, ASRS.							Figure 2.

Through RF communication, the AGV and the ASRS will decide which temporary holding dock to store the pallet in.  The AGV will also tell the ASRS the part number of the pallet and when it will complete the drop off (Figure 2).

The operator will also have the ability to request a part to be delivered to the output terminal.  He or she will enter a “2” followed by the part number of the pallet.  The AGV will request the part from the ASRS, and the ASRS will confirm its delivery to a temporary holding dock.  The AGV will then transport the package to the output dock.

To navigate the warehouse, the AGV will follow black lines on a white floor.  Additional sensors will add the capability to detect intersecting lines.  A counting algorithm will be implemented to keep track of the vehicle’s location.  While traversing the warehouse, safety will be maintained with obstacle and collision detection sensors.

EXPERIMENTAL LAYOUT AND RESULTS

CONCLUSION

DOCUMENTATION

APPENDICES