Recon-bot

University of Florida

Department of Electrical and Computer Engineering

EEL 5666

Intelligent Machines Design Laboratory

Jose Noriega

1/30/07

TAs:

Adam Barnett

Julio Suarez

Instructors:

Dr. Arroyo

Dr. Schwartz

Table of Contents:

Title Page…………………………………………...	1

Table of Contents…………………………………...	2

Abstract……………………………………………..	3

Executive Summary………………………………...	4

Introduction…………………………………	……...	5

Integrated System…………………………………...	5

Mobile Platform…………………………………….	5

Actuation…………………………………………….5

Sensors……………………………………………...	5

Behaviors…………………………………………...	6

Experimental Layout and Results…………………..	6

Conclusion………………………………………….	6

Documentation……………………………………...	6

Appendix……………………………………………	6

Abstract

The recon-bot is a reconnaissance robot that is capable of taking real-time images of a target zone.  The vehicle is rugged for all terrains and is equipped with a human detection sensor that identifies possible intruders in its general vicinity.  The on board electronics are controlled by a Mavric IIB Atmel microcontroller.  The recon-bot utilizes sonar, IR, and motion detection sensors as its primary obstacle avoidance systems.

Executive Summary

The recon-bot is designed to be an outdoor GPS-guided reconnaissance platform.  The machine is given an origin coordinate and a target coordinate.  The microcontroller then identifies the most direct path between those two locations and proceeds towards the objective.  On route to the destination, the sonar, IR, and bump sensors help the recon-bot perform obstacle avoidance.  The rugged design of the chassis allows for a relatively fast robot that can traverse a large area to accomplish the mission.

Once the recon-bot has arrived at its destination, it then begins to survey the target area.  The machine takes a series of images and stores them locally on a SD memory card for later analysis.  Furthermore, during its reconnaissance of the area, the recon-bot will detect human presence using a motion detection sensor.  If a threat is perceived, the camera takes a picture of the intruder.

Introduction

The recon-bot is a rugged reconnaissance machine capable of traveling to a target zone and taking real-time images of the area.  The robot stores the data locally and then returns to its point of origin for data analysis.  In the process of traveling to and from its target zone, the recon-bot also utilizes a human detection sensor to identify possible threats in the area.  If a threat is detected, the robot surveys the area and takes images of possible intruders.

Integrated System

The recon-bot electronics are being powered using the Mavric IIB development board from bdmicro.com.  This board has an Atmega-128 microcontroller and a full set of peripherals and interfaces to connect to the variety of sensors on the recon-bot.

Mobile Platform

The robot chassis consists of a steel-tec frame built using a combination of erector construction sets.  The motors, wheels, and steering are from Lynxmotion and allows the rugged vehicle to traverse tough terrain.

Actuation

The steering is controlled by a servo and the thrust using two Jameco motors.  The board is using the Dubel motor controller driver board discussed in class.

Sensors

The four main sensors on the recon-bot are sonar, IR, GPS, and bump switches.  Using data from all of these systems, the machine avoids obstacles as it travels to its target destination.

Behaviors

The machine has a basic set of modes of operation.  Mode one is mission input mode which is where the user identifies the start location of the robot and its target destination.  After entering the target coordinates and pressing the “Go” button, the machine enters the avoidance mode.  In this mode, the recon-bot will traverse to the target location while utilizing its special sensors to avoid obstacles as well as detect possible human presence en route.  Once the machine has reached its destination, the robot enters acquisition mode, which is where it conducts surveillance of the area by taking pictures of its surroundings.  On the return trip home, the robot is once again in avoidance mode.

Experimental Layout and Results

Conclusion

Documentation

Appendix