Final Report for Thomas

Designed by: Kelly W. Snow

For the EEL 5934

Department Of Engineering

University Of Florida

Under the supervision of Dr. Keith L. Doty

February 14, 1995

# Table of Contents

Page #			Title

3			Abstract

4			Executive Summary

5			Introduction

5			System Overview

6			Main Body Layout

8			Robot Movement

8				Drive Motors

9				Pan/Tilt Servo Head

10 			Sensors

10				Motor Encoders

11				IR Sensors

13				Sonar

14			Behaviors

15			Sensor Characterization and Results

15				IR Sensors

16				Sonar

20			Conclusion

21			Class Comments

22 			References

23			Appendix A - Demo1.lis

# Abstract

The four main systems that make the robot, Thomas, capable of accomplishing the ultimate goal of Landmark Navigation are --

1) Expanded 68HC11 EVBU Board.

2) DC Motor Actuation with Motor Encoders.

3) Infra-Red Emitter/Detector Array.

4) Pan/Tilt Servo Head with Sonar Detector.

Thomas' design makes it rugged, reliable, and esthetically pleasing.  Although the Landmark Navigation software is not yet written, all of the necessary hardware is now in place.  The software included in Appendix A is an object avoidance program that utilizes all of Thomas' systems and shows that Thomas is ready to begin the next step of landmark navigation software development.

# Executive Summary

Two specific goals have driven the design of the robot Thomas.  The first goal is to build a robot with all the capabilities of another Machine Intelligence Laboratory robot, Riker.  The second is to build this new robot much more robust electrically, mechanically and esthetically than its predecessor.

To accomplish the goal of Landmark Navigation, Riker's designer Steven Seed equipped Riker  with IR sensors for short distance object detection, motor encoders for measuring distance traveled, a sonar sensor for long distance object measurements, and an expanded 68HC11 EVBU micro controller to control the systems.

To accomplish the first goal, Thomas has been equipped with each of Riker's features, each of which is a significant upgrade both in technology and accuracy over its older cousin.  Thomas also has a feature never incorporated into Riker, Thomas has a 2-axis Pan/Tilt Servo head that enables it to point it's sonar array in any direction in the upper hemisphere. To accomplish the second goal, Thomas' design is electrically and mechanically robust.  Each piece of Thomas is modular in design to facilitate the installation of new parts and circuit upgrades.  The wire routing and part placement have also been designed  to keep Thomas as neat as possible.

The balance of this paper details how Thomas has met his design goals by showing how each system has been constructed and by giving an example of the software that has been written to drive the systems.  Although the Landmark Navigation software  is not yet written, Thomas has the necessary hardware to accomplish this goal also.  This will be the next step in Thomas' development.

# Introduction

The robot, Thomas' design goal is to duplicate the functions of the robot, Riker.  Riker is a robot, developed for the Machine Intelligence Laboratory. Built by Steven Seed as a Masters project in the spring of 1994, Riker was developed to perform Landmark Navigation.  Thomas' design incorporates all of Riker's capabilities in a more rugged and esthetically pleasing package.

# System OverView

To achieve Thomas' overall goal, it's design uses a variety of sensors and system upgrades.  Figure #1 shows a system level block diagram of Thomas.

Figure #1

As shown in Figure #1, Thomas has the following systems, and/or subsystems:

1) Power Regulation System

2) Memory Expansion

3) DC Motor Actuation

4) Motor Encoder System

5) Pan/Tilt Servo Head

6) Infrared Sensor System

7) Sonar Sensor System

When combined, these systems equip Thomas with every capability possessed by the robot Riker, and, in several instances, Thomas surpasses Riker's capabilities.

# Main Body Layout

Thomas' main body is a box constructed of sheet aluminum, slightly rounded at the back.  The main axle for the two front wheels is pop-rivitted to the bottom as is the back castor.  Figure #2 shows the interior of the main body, as well as Thomas' dimensions.

The main body box contains the following elements:

1) Motors

2) Motor Driver Circuitry

3) Motor Encoder Circuitry

4)Batteries

5) Power Regulation Circuitry

6) Five of the 7 IR-LEDs, these are mounted through the sides of the main body box.

The metal body acts as a Faraday Cage to electromagneticly isolate Thomas' electronics from any noise transmitted from the motors. All signals passed into or out of the main body box pass through one of two 9-pin feed-through connectors.  The back of the main body box can be removed to provide access to the battery pack. Access to the interior of the main body box is provided through the top which is secured with screws.   The mount for the Pan/Tilt Servo head is screwed onto the top plate of the main body box.  Figure #3 shows the Pan/Tilt mounting bracket and head configuration

## Figure #2

Figure #3

# Robot Movement

## Drive Motors

Separate DC motors drive the front wheels.  These motors contain their own gear reduction system and apply their power to the front wheels through a single 14/82 gear pair.  The motors are very efficient, which is useful in reducing power consumption, but can also cause problems.  The largest problem connected with the drive motors is Thomas' momentum.  When Thomas is moving it will not stop immediately, but coasts to a stop over a distance of approximately 2 inches. This causes problems in wall following and distance measuring behaviors.  Thomas uses Pulse Width Modulation [PWM] and a direction bit to control it's two drive motors.  OC2 and OC3 on 68HC11 generate the PWM signals. The direction bits come from the DDO (Digital Data Out) memory mapped I/O as shown in Figure #4.  Both the direction and PWM signals are sent through a TI9310 Optocoupler to maintain electrical isolation of the motors from the electronics.  Next, the signals are fed into a SN754410NE Motor driver chip that drives the motors.

## Figure #4

The motor direction bits of DDO are Bits 0 and 1.

1 = Forward, 0 = Backward.

The percentage of the PWM signal driving the motors controls the motor speed, the higher the percentage, the faster the motor runs.

## Pan/Tilt Servo Head

The Pan/Tilt Servo Head mounted on top of Thomas uses standard Futaba servos to derive its actuation.  As shown in Figure #3, the 190o of servo movement allows accurate positioning of the servo head in any direction in the upper hemisphere.  The PWM signals developed in OC4 and OC5 control servo direction (See Appendix A Memory Location 14B4).  Servos derive their position data from the width of the positive part of the 55Hz PWM Signal. The position of the servo is controlled by controlling the width of the pulse (See Appendix A, Memory Location 1555).

# Sensors

## Motor Encoder

The motor encoder system utilizes both older and newer technologies to perform its function.  Figure #5 shows the circuitry involved with the motor encoder system.  During a modification of the Power Regulation system, the circuit board containing the IR emmiter/detector for the right wheel motor was replaced with the new SFH905 integrated IR emmiter/detector.  The IR. emmiter/detector for the left wheel motor remains the older style.  The motor encoder signals from both encoders are amplified to TTL levels by 2N2222 common emitter amplifier circuits. The rising and falling edges of the outputs of these amplifiers are squared up by 7414 Schmitt Trigger inverters.

Figure #5

The outputs of these inverters are sent to IC1 and IC2 for counting (See Appendix A, memory Location 171E).

## IR Sensors

Thomas uses 7 infrared emmiter/detector pairs to sense the outside world.  Figure #6 is a diagram of the IR sensing system.  The IR sensing system employed in Thomas has two important features.  The first of these is the generation of the 40kHz signal that drives the IR emitters.  This is accomplished by the use of a 74HC390 Dual 4 Stage Ripple Counter.  The 40kHz is derived from the E-Clock by dividing the E-Clock in the following manner --

This method produces a stable 40kHz signal with a duty cycle of 20%.  By dividing the E-Clock in a different order it is possible to produce a 50% 40kHz, but a 20% signal is more efficient because of the lower power consumption.

The second important feature of the IR Sensing System is its ability to illuminate and read a single emmiter/detector pair.  Using the 74HC259 8-Bit Addressable Latch, the 40kHz signal is routed to any one of the 7 IR emitters. The corresponding IR Detector can then be measured.  It is also possible to take readings without turning any IR emitter on.  This allows "Dark" readings to be taken to help eliminate background IR radiation.

## Figure #6

The IR system is controlled through the memory mapped I/O DDO (Digital Data Out).  This Digital Output word is mapped to memory location $0200 - $021F.  When data is written to this location in memory a latching pulse is generated on the DDO line of the PALCE22V10.  This latching pulse latches the data in to a 74HC374 8-Bit Latch.  From the latch the data is used to control the IR System and to control Motor Direction.  The DDO data word is defined as follows:

| DDO $0200 - $021F |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| IR# Bit 2 | IR# Bit 1 | IR# Bit 0 | 40kHz 0=ON 1=OFF | Demux 0=ON 1=OFF | Not Used | Right Wheel 0=Reverse 1=Forward | Left Wheel 0=Reverse 1=Forward |

The IR. Detectors on Thomas have received the "Arial" modification.  This allows the detector to produce an analog output that is fed directly into the 68HC11 A/D system.  A characterization of the IR Sensing System is provided in the Experimental Layout and Result section of this report.

## Sonar

The Sonar system consists of a 40kHz speaker and it's corresponding microphone receiver. Figure #7 shows the circuit used in the sonar system.  The circuit consists of a common emitter amplifier circuit for amplifying the 40kHz signal that is used to drive the speaker.

## Figure #7

This increases the output signal power, and hence increases the range.  The returned signal is amplified through 3 op-amp stages, each stage having a gain of 20.  The resulting signal is shaped to TTL levels using a Darlington pair of 2222 transistors.  The return signal is then sent to IC3 where it is used to determine the time of flight (See Appendix A, Memory Location 137A).

# Behaviors

Four major behaviors have been realized to date.  These four behaviors are contained in the code of the program listed in Appendix A.  The behaviors are:

1) Straight Line -- Adjust motor PWM until both motor encoder counts are equal (See Appendix A, Memory Location 1566).

2) Object Avoidance -- Use IR sensors to detect objects and avoid them (See Appendix A, Memory Location 125A).

3) Sonar Scanning -- Use Sonar to scan 180o in front of Thomas and return Pan angle to closest object (See Appendix A, Memory Location 11A9).

4) Sonar Turn -- Use sonar scanning pan angle to turn Thomas toward scanned object (See Appendix A, Memory Location 111B).

# Sensor Characterization and Results

## IR Sensors

The characterization of the IR sensors was accomplished by taking readings from the seven IR sensors at 1 inch intervals from a light colored piece of card board and then a dark colored piece of cardboard.  Figure #8 shows a plot of this collected data. The graph shows the IR sensor's sensitivity and characteristic response.  Although the response curve is not linear,  the center section from 2 inches to 12 inches is very close to linear.  The graph also shows how close the seven IR sensors are in their response.  The graph indicates a good repeatability from sensor to sensor.

## Figure #8

Figure #9 contains the data used to create Figure #8.  The data has been compressed to fit it onto a single page.  The first column, headed by the variable "n", contains the distance from target data in inches.  The following columns contain the data for the respective IR's.  IR0Ln means IR #0, Light, at n inches,  IR0Dn means IR #0, Dark, at n inches, and so on.

## Figure #9

## Sonar

The sonar system was characterized and the data plotted.  The graph of the data shows how accurate, linear, and reliable the sonar system is.  The characterization of the sonar system was conducted in the following manner.   The system was tested against a hard smooth wall (white board) to obtain a maximum returned signal.  Data points were taken from the maximum return distance (100 inches) to the minimum return distance (8 inches) in 2 inch increments.  Figure #10 is a graph of the data shown in Figure #11.  It should be noted that the software that drives the sonar system sends only a single pulse, counts the number of E-Clocks until the return pulse and displays that number. The software then waits for a key-press the take the next reading.  Therefore, no averaging is being taken of multiple readings.  However, of the 16 bits returned, Bit 15 is disregarded because it is always 0, Bit 14 - Bit 7 are stable, and Bit 6 - Bit 0 are unstable.  This gives the readings 7 bits of significance.

## Figure #10

Figure #11 contains the data used to create Figure #10.  The "Time of Flight" column is only a very rough estimate calculated by multiplying the # of E-Cycles by the time for 1 E-Cycle of 1/2,000,000.  The actual E-Clock cycle was never measured.  The speed of sound was calculated in the following manner -- Two data points, 30 inches apart, were used for each calculation.  Using the following calculation   -- These various calculated speed of sounds were averaged to produce a final result.  The speed of sound given in a standard college physics book is . The experimentally derived speed of sound is  this difference yields an error of 3.1%, assuming that the speed of sound in the lab at the time of the experiment was actually . A returned value of 0000 indicates the reading was too close to be accurate.  A returned value of ffff indicates that no return pulse was detected.

## Figure #11

| Distance | Returned Value |  | Time of Flight | Speed of Sound | Avreage Speed |
| --- | --- | --- | --- | --- | --- |
| [inch] | [Hex] | [Decimal] | [mS] | [Mi/Hr] | of Sound [Mi/Hr] |
| 8 | 0 | 0 | 0 |  |  |
| 10 | 0bcb | 3019 | 1.5095 | 760.957792207792 | 754.855384089956 |
| 12 | 0e2c | 3628 | 1.814 | 757.828367031435 |  |
| 14 | 1092 | 4242 | 2.121 | 757.744145163572 |  |
| 16 | 12e1 | 4833 | 2.4165 | 771.113076021468 |  |
| 18 | 1536 | 5430 | 2.715 | 761.63782598099 |  |
| 20 | 17a1 | 6049 | 3.0245 | 760.194204279387 |  |
| 22 | 19e8 | 6632 | 3.316 | 759.516744812501 |  |
| 24 | 1c05 | 7173 | 3.5865 | 746.870612135154 |  |
| 26 | 1e23 | 7715 | 3.8575 | 734.876246840032 |  |
| 28 | 2099 | 8345 | 4.1725 | 737.978332956145 |  |
| 30 | 2374 | 9076 | 4.538 | 740.865132911205 |  |
| 32 | 24fa | 9466 | 4.733 | 740.462838638338 |  |
| 34 | 27c3 | 10179 | 5.0895 | 745.808555915754 |  |
| 36 | 29ff | 10751 | 5.3755 | 753.057413097175 |  |
| 38 | 2cb1 | 11441 | 5.7205 | 760.194204279387 |  |
| 40 | 2ecb | 11979 | 5.9895 | 753.890072775522 |  |
| 42 | 3151 | 12625 | 6.3125 | 754.641042410827 |  |
| 44 | 33b8 | 13240 | 6.62 | 752.641772621903 |  |
| 46 | 356b | 13675 | 6.8375 | 742.155417239776 |  |
| 48 | 382e | 14382 | 7.191 | 742.155417239776 |  |
| 50 | 3aaa | 15018 | 7.509 | 740.221671716624 |  |
| 52 | 3cf9 | 15609 | 7.8045 | 741.509713777251 |  |
| 54 | 3fae | 16302 | 8.151 | 755.644665652424 |  |
| 56 | 4261 | 16993 | 8.4965 | 779.933861608536 |  |
| 58 | 44b0 | 17584 | 8.792 | 768.33241133444 |  |
| 60 | 4767 | 18279 | 9.1395 | 782.350179940541 |  |
| 62 | 48f2 | 18674 | 9.337 | 759.432147269082 |  |
| 64 | 4b79 | 19321 | 9.6605 | 768.852257350228 |  |
| 66 | 4d5d | 19805 | 9.9025 | 759.940015401451 |  |
| 68 | 4fba | 20410 | 10.205 |  |  |
| 70 | 521f | 21023 | 10.5115 |  |  |
| 72 | 549c | 21660 | 10.83 |  |  |
| 74 | 571b | 22299 | 11.1495 |  |  |
| 76 | 594e | 22862 | 11.431 |  |  |
| 78 | 5c11 | 23569 | 11.7845 |  |  |
| 80 | 5ea5 | 24229 | 12.1145 |  |  |
| 82 | 60e4 | 24804 | 12.402 |  |  |
| 84 | 62ed | 25325 | 12.6625 |  |  |
| 86 | 6487 | 25735 | 12.8675 |  |  |
| 88 | 675a | 26458 | 13.229 |  |  |
| 90 | 6972 | 26994 | 13.497 |  |  |
| 92 | 6c04 | 27652 | 13.826 |  |  |
| 94 | 6e1d | 28189 | 14.0945 |  |  |
| 96 | 7069 | 28777 | 14.3885 |  |  |
| 98 | ffff | 65535 | 32.7675 |  |  |
| 100 | ffff | 65535 | 32.7675 |  |  |

# Conclusion

All of Thomas' systems are functioning and operating well.  While there remains room for improvement in some areas, Thomas' design goals have been met.  Thomas' main body construction is rugged and clean.  The 68HC11 EVBU board memory upgrade and port reconstruction are compact and fully functional.  The DC motor actuation and motor encoder systems are fully functional and efficient.  The short range IR sensing system is working well and senses objects up to 2 feet away.  The Pan/Tilt servo head is operational, but remains somewhat jerky and prone to oscillation.  This is most probably due to the fact that the servos are receiving unregulated  power.  Installing a separate 5Vdc regulated power supply for the servos should eliminate this problem.  The Sonar sensing system is fully operational and very accurate up to 8 feet, however, the sonar system becomes unusable if the servos are moving when the sonar reading is taken.  Solving the servo problem will help improve sonar accuracy. All of the software drivers for all of Thomas' systems have been written and are compatible with each other.  The next step in the development of Thomas is the writing the Landmark Navigation software.  This will most probably prove to be a significant software challenge.  Hopefully some higher level language than Assembly can be found to simplify the task of writing the landmark navigation algorithms.

# Class Comments

The class was entirely enjoyable, and I look forward to continuing my work with Thomas.  However, there were times when I became completely frustrated with the lack of equipment in the IMDL lab.  I eventually decided to do all of my work at home.  Several other students expressed similar frustrations to me over the semester.  I realize that this class is new, and it is my hope and belief that this problem will be rectified by the fall semester.

# REFERENCES

The 6.270 Robot Builders Guide, by Fred G. Martin, MIT, 1992

Technical Document for the Riker Robot, by Steven Seed, Machine Intelligence Laboratory, Department of Electrical Engineering, University of Florida, 1994

Microcomputer Engineering, by Gene H. Miller, GMI Engineering & Management Institute, Flint, Michigan, 1993

HC11 M68HC11 Reference Manual, Motorola Inc., 1991

HC11 M68HC11 E Series Programming Reference Guide, Motorola Inc., 1991

Effective Professional and Technical Writing, by Michael L. Keene, Second Edition, D.C. Heath and Company, Massachusetts.

Mobile Robots: Inspiration to Implementation, by Joseph L Jones, and Anita M. Flynn

# Appendix A -- Demo1.Lis

DEMO1.ASM              Assembled with IASM   04/29/1995  20:51  PAGE 1

1  * Programmer : Kelly Snow

2

3  * Description: This is Simple behavior program .

In this version Thomas will

4  *  go forward until it sees an object ahead, it

will then check it's side

5  *  sensors and turn toward which ever is free. This

version is designed to

6  *  utilize the modified IR. sensor.

7  *

8  * This version uses the motor encoders to adjust

the pulse width when the

9  *  robot is going strait. The PWM is adjusted to

make the motor counts equal.

10  *

11  * This version also uses the new Servo routine that

utilizes an adjustable

12  *  step size to position the servos

13  *

14  * The program periodically scans with the sonar

head and turns Thomas toward

15  *   the closes object it detects

16  *

17  * The IR. routine utilizes a differential (Dark vs.

Light) algorithm

18  *

19  *       NOTE *** IR0 is Connected to PE7 do to

Buffalo constraints on PE0. ****

20  ***************************************

21  * Register Address Definition

22  ***************************************

0000                  23  REGBAS  EQU $1000       ; Base address for register

block

0000                  24  CFORC   EQU $0B ; used to force the OC registers

0000                  25  OC1M    EQU $C  ; Register used by OC1 to determine

which OC pins to

26  *               ;  control

0000                  27  OC1D    EQU $D  ; Register used by OC1 to determine

what to set the

28  *               ;  affected OC pins to.

0000                  29  TCNT    EQU $0E ; T-Count Register

0000                  30  TIC3    EQU $14 ; TCNT value when IC3 triggers

0000                  31  TOC1    EQU $16 ; Timer Output Compare Register for

OC1

0000                  32  TOC2    EQU $18 ; Timer Output Compare Register for

OC2

0000                  33  TOC3    EQU $1A ; Timer Output Compare Register for

OC3

0000                  34  TOC4    EQU $1C ; Timer Output Compare Register for

OC4

0000                  35  TOC5    EQU $1E ; Timer Output Compare Register for

OC5

0000                  36  TCTL1   EQU $20 ; OC2 - OC5 Control Register

0000                  37  TCTL2   EQU $21 ; Select edge for capture for IC1 -

3

0000                  38  TMSK1   EQU $22 ; OC1 - OC5 Interrupt Mask Register

0000                  39  TFLG1   EQU $23 ; OC1 - OC5 Flag Register

0000                  40  TMSK2   EQU $24 ; Used to enable RTI interrupts

0000                  41  TFLG2   EQU $25 ; BIT 7 -> TCNT FLAG

0000                  42  PACTL   EQU $26 ; bit #2 determines if IC4 or OC5

is active

0000                  43  ADCTL   EQU $30 ; A/D Control Register

0000                  44  ADR1    EQU $31 ; A/D Result Register #1

0000                  45  ADR2    EQU $32 ; A/D Result Register #2

0000                  46  ADR3    EQU $33 ; A/D Result Register #3

0000                  47  ADR4    EQU $34 ; A/D Result Register #4

0000                  48  OPTION  EQU $39 ; OPTION Register used to turn on

the A/D

0000                  49  DDO     EQU $0200       ; Digital Data Out

50

51  ***************************************

52  * Subroutines in this program

53  *

54  * Servoini --- Initializes OC4 and OC5 to produce

the PWM for the servos

55  * OC4ISR ----- ISR for OC4

56  * OC5ISR ----- ISR for OC5

57  * MoveHead --- Moves the Pan/Tilt head to the

desired position

58  * ServoCalc -- Used by MoveHead to calculate PWM

High time

59  * Sonar -- Takes sonar reading and stores it in

SonVal

60  * ADSetup -- Setup A/D system

61  * IRRead --- Scan IR Sensors and update IR.

Variables

62  * OC1Setup -- Setup OC1, OC2, OC3 for PWM

63  * CALOFF ---- Calculates the necessary offset for

the waveform generation

64  * OC1ISR ---- OC1 Interrupt Service Routine

65  * AdjPWM ---- Use PWMDC1 and PWMDC2 to establish

offsets for PWM

66  * ICSetup --- Setup IC

67  * SVIC1  ---- IC1 Interrupt Service Routine

68  * SVIC2  ---- IC2 Interrupt Service Routine

69  * SonarTurn -- Turns Thomas towards the detected

object

70  * Scan -- Scans the sonar head through the 180

degrees in front of Thomas and remembers the

71  *               shortest distance

72  * Av10Sonar -- Takes 10 sonar readings and averages

them

73  * RTIISR -- RTI Interrupt Service Routine

74  * RTIINI -- RTI Initialization Routine

75  * Avoid -- Use IRs to avoid all objects encountered

76  * Rotate -- Rotates the Servo Head through 180

degrees in 10 degree increments

77  * Sonar -- Take a single sonar reading

78  * Strait -- Uses Motor Encoders to adjust the PWM

to the motors until the motor encoder

79  *                  counts are equal.

80  * PWMSetup -- Sets up OC1 to control OC2 and 3 for

motor PWM generation.

81  ***************************************

00DF                  82          ORG $00DF       ; Pseudo Vector for OC1

Interrupt

00DF [03] 7E1702      83          JMP OC1ISR      ; Jump to OC1ISR

84

00E5                  85          ORG $00E5       ; Pseudo Vector for IC2

(IC1 is at $E8)

00E5 [03] 7E172C      86          JMP SVIC2       ; Jump to SVIC2

00E8 [03] 7E171E      87          JMP SVIC1       ; Jump to SVIC1

88

00D3                  89          ORG $00D3       ; Pseudo Vector for OC5

00D3 [03] 7E14B4      90          JMP OC5ISR

91

00D6                  92          ORG $00D6       ; Pseudo Vector for OC4

00D6 [03] 7E147A      93          JMP OC4ISR

94

00EB                  95          org $00EB       ; Pseudo Vector for RTI

00EB [03] 7E1240      96          JMP RTIISR

97

1040                  98          ORG $1040       ; Variables are at $1040

1040 [03] 7E1092      99          JMP MAIN        ; Branch to MAIN

100  ****************************************

101  * Define Variables stored in RAM

102

1043      14         103  PAN     FCB !20         ; Pan Servo position

(<=SvSteps)

1044      5F         104  TILT    FCB !95         ; Tilt Servo position

(<=SvSteps)

1045      14         105  NewPan  FCB !20

1046      5F         106  NewTilt FCB !95

1047      14         107  HiPan   FCB !20

1048      5F         108  HiTilt  FCB !95

109

1049      00BE       110  SvSteps FDB !190        ; # of Steps for full Servo

motion

111                          ; SvSteps Must be => 16

112

113  * Used by Servoini, OC4ISR, OC5ISR

114

104B      00         115  int4    FCB !0          ; Used to determine if OC4

will be Low/High

104C      00         116  int5    FCB !0          ; Used to determine is OC5

will be Low/High

104D      0240       117  high4   FDB !576        ; Used by OC4ISR to

determine PWM High time

104F      06B8       118  high5   FDB !1720       ; Used by OC5ISR to

determine PWM High time

1051      020E       119  NewHigh4 FDB !526       ; # of e-clocks for up-time

at servo destination

1053      06B8       120  NewHigh5 FDB !1720      ; # of e-clocks for up-time

at servo destination

121

122  * Variables used by sonar

123

1055      1B         124  DDOV    db %00011011    ; Variable used to store

bit pattern that is written to

125  *                       ;  DDO Register

126

1056      0000       127  ICnt    FDB $0          ; Holds Initial value of

T-Cnt

1058      00         128  oflow   FCB $0          ; Used to indicate if the

TCNT overflows while waiting

129  *                       ;  for return signal

1059      0000       130  SonVal  FDB $0          ; Final Sonar Value

105B      0000       131  HiSonVal FDB $0

105D      0000       132  ASonVal FDB $0

105F      00         133  resultf FCB $0          ; Sonar result soft register

134

135  * Variables used by Rotate

136

1060      08121C26   137  fr db   !8,!18,!28,!38,!48,!58,!68,!78,!88,!98,!108,

!118,!128,!138

303A444E

58626C76

808A

106E      949EA8B2   138  fr1 db  !148,!158,!168,!178,!188,$ff

BCFF

1074      00         139  Times   FCB !0          ; Used to count how Rotate

calls

1075      0000       140  PosPtr  FDB !0          ; Position Pointer

1077      00         141  EndR    FCB !0          ; Used to indicate the end

of rotation cycle

142

143

1078      00         144  IR0     db 0    ; Left Back IR.

1079      00         145  IR1     db 0    ; Left Front IR.

107A      00         146  IR2     db 0    ; Front Left IR.

107B      00         147  IR3     db 0    ; Front Center IR.

107C      00         148  IR4     db 0    ; Front Right IR.

107D      00         149  IR5     db 0    ; Right Front IR.

107E      00         150  IR6     db 0    ; Right Back IR.

151

107F      63         152  PWMDC1  FCB !99         ; OC2 % duty cycle RIGHT

MOTOR

1080      63         153  PWMDC2  FCB !99         ; OC3 % duty cycle LEFT

MOTOR

1081      B5         154  PWMP1P  FCB !181        ; Contains # E-cycles for

0.5% of period (8 bit)

1082      0E34       155  PWMPER  FDB !3636       ; Contains # E-cycles for

period (16 bit)

156

1084      63         157  STRR    FCB !99         ; Value for strait movement

Right

1085      63         158  STRL    FCB !99         ; Value for strait movement

Left

159

1086      05         160  IRL     db $5           ; IR. Set point

161

1087      0000       162  RMCnt   FDB $0000       ; Right Motor Count

1089      0000       163  LMCnt   FDB $0000       ; Left Motor Count

164

108B      00         165  Dark    db !0           ; Used to store dark IR.

reading

166

108C      0000       167  RTIs    FDB !0          ; Used by RTIISR to store

the # OF RTI's

108E      00         168  AV10LC  FCB !0          ; Used by AV10Sonar to

store loops

169

108F      02         170  DegMult FCB !2          ; Used by SonarTurn

1090      0000       171  TargetCnt FDB !0        ; Used by SonarTurn for

Target motor count

172

173  ***************************************

1092 [03] 8E0FFF     174  MAIN    LDS #$0fff      ; Initialize Stack

175

176  * Setup Variables

177

1095 [06] 7F1078     178          CLR IR0

1098 [06] 7F1079     179          CLR IR1

109B [06] 7F107A     180          CLR IR2

109E [06] 7F107B     181          CLR IR3

10A1 [06] 7F107C     182          CLR IR4

10A4 [06] 7F107D     183          CLR IR5

10A7 [06] 7F107E     184          CLR IR6

10AA [02] 861B       185          LDAA #%00011011

10AC [04] B71055     186          Staa DDOV

10AF [04] B61084     187          LDAA STRR

10B2 [04] B7107F     188          STAA PWMDC1

10B5 [04] B61085     189          LDAA STRL

10B8 [04] B71080     190          STAA PWMDC2

10BB [03] CC0E34     191          LDD #!3636

10BE [05] FD1082     192          STD PWMPER

10C1 [02] 8605       193          LDAA #$5

10C3 [04] B71086     194          STAA IRL

10C6 [03] CC0000     195          LDD #!0

10C9 [05] FD108C     196          STD RTIs

197

198  *       JSR ONSCI

199

200  * Setup RTI

201

10CC [06] BD124E     202          jsr RTIINI

203

204  * Setup A/D

205

10CF [06] BD15F9     206          jSR ADSetup

207

208  * Setup PWM

209

10D2 [06] BD16B6     210          JSR PWMSetup

211

212  * Institute PWM

213

10D5 [06] BD167B     214          JSR AdjPWM

215

216  * Initialize Input capture

217

10D8 [06] BD16F1     218          JSR ICSetup

219

220  * Initialize Servos

221

10DB [06] BD1456     222          JSR ServoINI

223

10DE [02] 0E         224          CLI             ; Enable Global Interrupts

225

10DF [06] BD14EE     226  MainLoop JSR MoveHead

10E2 [06] BD125A     227          JSR Avoid

228

229  * Check how long we have been doing avoid

10E5 [05] FC108C     230          LDD RTIs

231  *        CPD #!14500

10E8 [05] 1A831C52   232          CPD #!7250

10EC [03] 2C03       233          BGE rot

10EE [03] 7E10DF     234          JMP MainLoop

10F1 [06] 7F107F     235  rot     CLR PWMDC1

10F4 [06] 7F1080     236          CLR PWMDC2

10F7 [06] BD167B     237          JSR AdjPwm

10FA [06] BD11A9     238          JSR Scan

10FD [06] BD111B     239          JSR SonarTurn

1100 [03] CC0000     240          LDD #!0

1103 [05] FD108C     241          STD RTIs

1106 [03] 7E10DF     242          JMP MainLoop

243

244  * GOTO Buffalo

245

1109 [06] 7F107F     246  Buf     CLR PWMDC1

110C [06] 7F1080     247          CLR PWMDC2

110F [06] BD167B     248          JSR AdjPwm

1112 [03] CC0000     249          LDD #!0

1115 [05] FD108C     250          STD RTIs

1118 [03] 7EE0BF     251          JMP $E0BF        ; goto buffalo

252  ***************************************

253  * SonarTurn Subroutine

254  *

255  * This subroutine turns the robot to point toward

the object it has

256  *  detected. This uses PAN to determine the angle

257  *

258  * Input    : None

259  * Output   : None

260  * Calls    :

261  * Destroys : None

262  ****************************************

111B [03] 36         263  SonarTurn PSHA

111C [03] 37         264          PSHB

111D [04] 3C         265          PSHX

266

267  * Determine if Thomas has to turn Right or Left

268

111E [04] B61047     269          LDAA HiPAN

1121 [02] 8155       270          CMPA #$55       ; Value For Strait ahead

1123 [03] 253B       271          BLO TurnRight   ; if Pan < $55 Turn Right

1125 [03] 2775       272          BEQ STDone      ; if Pan = $55 go straight

ahead

1127 [02] 8055       273          SUBA #$55       ; if Pan > $55 Turn Left, A

= # of degrees to turn

274

275  * Calculate final motor count reading

276

1129 [04] F6108F     277          LDAB DegMult    ; B = Degree Multiplier

112C [10] 3D         278          MUL             ; D = # of Degrees X Degree

multiplier

112D [05] FD1090     279          STD TargetCnt   ; TargetCnt = D

280

281  * if turning Left, Clr Right Wheel Count and turn

right wheel the

282  *   appropriate # of counts

283

1130 [03] CC0000     284          LDD #!0         ;

1133 [05] FD1087     285          STD RMCnt       ; reset right motor count

286

287  * Setup to turn Left

288

1136 [04] B61055     289          LDAA DDOV       ; A = DDOV

1139 [02] 84FC       290          ANDA #%11111100 ; Mask Motor Direction Bits

113B [02] 8B01       291          ADDA #%00000001 ; Turn toward Left

113D [04] B71055     292          STAA DDOV

1140 [04] B70200     293          STAA DDO

294

295  * Give power to the motors

296

1143 [04] B61084     297          LDAA STRR

1146 [02] 44         298          LSRA

1147 [04] B7107F     299          STAA PWMDC1

114A [04] B61085     300          LDAA STRL

114D [02] 44         301          LSRA

114E [04] B71080     302          STAA PWMDC2     ; Half Speed

1151 [06] BD167B     303          JSR AdjPwm

304

305  * Read motor count and compare until done

306

1154 [05] FC1087     307  CHKLLoop LDD RMCnt

1157 [07] 1AB31090   308          CPD TargetCnt   ; RMCnt - TargetCnt = ?

115B [03] 2DF7       309          BLT ChkLLoop     ; If RMCnt < TargetCnt Loop

115D [03] 7E119C     310          JMP STDone      ; if RMCnt => TargetCnt

goto STDone

311

312  * Turn Right

313

1160 [02] 8655       314  TurnRight LDAA #$55       ; if Pan < $55 Turn Right,

1162 [04] B01043     315          SUBA Pan        ; A = # of degrees to turn

316

317  * Calculate final motor count reading

318

1165 [04] F6108F     319          LDAB DegMult    ; B = Degree Multiplier

1168 [10] 3D         320          MUL             ; D = # of Degrees X Degree

multiplier

1169 [05] FD1090     321          STD TargetCnt   ; TargetCnt = D

322

323  * if turning Right, Clr Right Wheel Count and turn

right wheel the

324  *   appropriate # of counts

325

116C [03] CC0000     326          LDD #!0         ;

116F [05] FD1087     327          STD RMCnt       ; reset right motor count

328

329  * Setup to turn Right

330

1172 [04] B61055     331          LDAA DDOV       ; A = DDOV

1175 [02] 84FC       332          ANDA #%11111100 ; Mask Motor Direction Bits

1177 [02] 8A02       333          ORAA #%00000010 ; Turn toward Right

1179 [04] B71055     334          STAA DDOV

117C [04] B70200     335          STAA DDO

336

337  * Give power to the motors

338

117F [04] B61084     339          LDAA STRR

1182 [02] 44         340          LSRA

1183 [04] B7107F     341          STAA PWMDC1

1186 [04] B61085     342          LDAA STRL

1189 [02] 44         343          LSRA

118A [04] B71080     344          STAA PWMDC2     ; Half Speed

118D [06] BD167B     345          JSR AdjPWM

346

347  * Read motor count and compare until done

348

1190 [05] FC1087     349  CHKRLoop LDD RMCnt

1193 [07] 1AB31090   350          CPD TargetCnt   ; RMCnt - TargetCnt = ?

1197 [03] 2DF7       351          BLT ChkRLoop     ; If RMCnt < TargetCnt Loop

1199 [03] 7E119C     352          JMP STDone      ; if RMCnt => TargetCnt

goto STDone

353

354

119C [06] 7F107F     355  STDone  CLR PWMDC1

119F [06] 7F1080     356          CLR PWMDC2

11A2 [06] BD167B     357          JSR AdjPwm      ; Stop motors

11A5 [05] 38         358          PULX

11A6 [04] 33         359          PULB

11A7 [04] 32         360          PULA

11A8 [05] 39         361          RTS

362

363  ***************************************

364  * Scan Subroutine

365  *

366  * This subroutine uses the servo head and the sonar

to scan the area around

367  *  the robot and find the most open space.

368  *

369  * Input    : None

370  * Output   : update HiPan and HiTilt

371  * Destroys : None

372  * Calls    :

373  ***************************************

11A9 [03] 36         374  Scan    PSHA

11AA [03] 37         375          PSHB

11AB [04] 3C         376          PSHX

377

11AC [04] DE00       378          LDX !0

11AE [05] FF105B     379          STX HiSonVal    ; HiSonVal=0

380

11B1 [06] BD132A     381  SMLoop   JSR Rotate     ; Rotate to next position

382

11B4 [02] C603       383          LDAB #$3

11B6 [03] CEFFFF     384  rw1     LDX #$ffff

11B9 [03] 09         385  RW      DEX

11BA [03] 26FD       386          BNE RW

11BC [02] 5A         387          DECB

11BD [03] 26F7       388          BNE RW1         ; Wait for servos to settle

389

11BF [04] B61077     390          LDAA EndR

11C2 [02] 81FF       391          CMPA #$ff

11C4 [03] 2733       392          BEQ SDone

393

394  * Take 10 Sonar Readings and average them and

determine if it is high

395

11C6 [06] BD11FD     396          JSR AV10Sonar

397

11C9 [05] FE105D     398  er      LDX ASonVal

11CC [05] FF1059     399          STX SonVal      ; SonVal = ASonVal

400

11CF [05] FC1059     401          LDD SonVal

11D2 [05] 1A83FFFF   402          CPD #$ffff

11D6 [03] 27D9       403          BEQ SMLoop       ; If Sonval = FFFF

Continue loop

11D8 [05] 1A830000   404          cpd #$0000

11DC [03] 27D3       405          BEQ SMLoop      ; If SonVal = 0000 Continue

Loop

11DE [07] 1AB3105B   406          CPD HiSonVal

11E2 [03] 2503       407          BLO NewHi       ; if SonVal > HiSonVal goto

New Hi

11E4 [03] 7E11B1     408          JMP SMloop       ; If SonVal <= HiSonVal

loop

409

410  * Create new hi value

411

11E7 [05] FD105B     412  NewHi   STD HiSonVal

11EA [04] B61045     413          LDAA NewPAN

11ED [04] B71047     414          STAA HiPan

11F0 [04] B61046     415          LDAA newTilt

11F3 [04] B71048     416          STAA HiTilt

11F6 [03] 7E11B1     417          JMP SMLoop

418

419

11F9 [05] 38         420  SDone   PULX

11FA [04] 33         421          PULB

11FB [04] 32         422          PULA

11FC [05] 39         423          RTS

424  ***************************************

425  * Av10Sonar Subroutine

426  *

427  * This subroutine takes ten sonar readings and

averages them together

428  *

429  * Input    : None

430  * Output   : Update ASonVal

431  * Calls    : Sonar

432  * Destroys : None

433  ***************************************

11FD [03] 36         434  Av10Sonar PSHA

11FE [03] 37         435          PSHB

11FF [04] 3C         436          PSHX

437

1200 [03] CE0000     438          LDX #!0

1203 [05] FF105D     439          STX ASonVal     ; ASonVal = 0

1206 [02] C60B       440          LDAB #!11       ; Loop Count = 11

1208 [04] F7108E     441          STAB AV10LC     ; AV10LC = 11

120B [04] F6108E     442  aloop   LDAB AV10LC     ; B = AV10LC

120E [02] 5A         443          DECB            ; Loop Count = Loop Count -1

120F [03] 272B       444          BEQ Av10Done    ; if Loop Count = 0 go to

AV10Done

1211 [04] F7108E     445          STAB AV10LC     ; AV10LC = AV10LC - 1

1214 [06] BD137A     446          JSR Sonar       ; if Loop Count <> 0 take

sonar reading

447

1217 [05] FE1059     448          LDX Sonval      ; X = Sonar Reading

121A [04] 8C0000     449          CPX #$0000

121D [03] 27EC       450          BEQ aloop       ; If Sonar Reading = 0000

Loop

121F [04] 8CFFFF     451          CPX #$ffff

1222 [03] 27E7       452          BEQ aloop       ; If Sonar reading = ffff

Loop

1224 [05] FC105D     453          LDD ASonval     ; If Valid Sonar reading

1227 [06] F31059     454          ADDD Sonval     ;   D = Sonval + ASonVal

122A [03] 2409       455          BCC noc         ; If No Carry go to noc

122C [03] 04         456          LSRD            ; If carry, D = (Sonval +

ASonVal)/2

122D [02] 8A40       457          ORAA #%1000000  ; Account for carry

122F [05] FD105D     458          STD ASonVal     ; Save new average

1232 [03] 7E120B     459          JMP aloop       ; Loop

1235 [03] 04         460  noc      LSRD           ; If no carry, D = (SonVla

+ ASonVal)/2

1236 [05] FD105D     461          STD ASonVal     ; Save new average

1239 [03] 7E120B     462          JMP aloop       ; Loop

463

123C [05] 38         464  AV10Done PULX

123D [04] 33         465          PULB

123E [04] 32         466          PULA

123F [05] 39         467          RTS

468

469  ***************************************

470  * RTIISR RTI Interrupt Service Routine

471  *

472  * This routine increments a software flag (RTIs)

which is used to time

473  *  how long a particular routine has been running.

474  *

475  * Input    : None

476  * Output   : None

477  * Calls    : None

478  * Destroys : None

479  ***************************************

1240 [05] FE108C     480  RTIISR  LDX RTIs

1243 [03] 08         481          INX

1244 [05] FF108C     482          STX RTIs        ; Increment RTIs

483

1247 [03] CE1000     484          LDX #RegBas

124A [07] 1D25BF     485          BCLR TFLG2,X,%10111111 ; Clear RTI Flag

124D [12] 3B         486          RTI

487

488  ***************************************

489  *  RTIINI  RTI Initialization Subroutine

490  *

491  * This Routine initializes the RTI for use.

492  *

493  * Input    : None

494  * Output   : None

495  * Calls    : None

496  * Destroys : None

497  ***************************************

124E [04] 3C         498  RTIINI  PSHX

499

124F [03] CE1000     500          LDX #RegBas

1252 [07] 1C2440     501          BSET TMSK2,X,%01000000

1255 [07] 1D25BF     502          BCLR TFLG2,X,%10111111

503

1258 [05] 38         504          PULX

1259 [05] 39         505          RTS

506

507  ***************************************

508  * Avoid Behavior Subroutine

509  *

510  * This is the Avoid Obstacles Subroutine

511  *  This routine uses the IR. sensors to avoid the

obstacles it encounters

512  *

513  * Input    : None

514  * Output   : None

515  * Destroys : None

516  * Calls    : AdjPwm, Strait, MoveHead, IRread

517  ***************************************

125A [03] 36         518  Avoid   PSHA

125B [03] 37         519          PSHB

125C [04] 3C         520          PSHX

125D [05] 183C       521          PSHY

522

523  * Straight

524

125F [06] BD1566     525          JSR Strait

526

527  * Read IR. and test forward Sensors (IR2, IR3, IR4)

528  *   Test IR3 ( Front Center )

529

1262 [02] C603       530          LDAB #!3

1264 [06] BD1612     531          JSR IRRead      ; Read IR3

1267 [04] B6107B     532          LDAA IR3        ; A = IR3

126A [04] B11086     533          CMPA IRL        ; Compare A to IRL

126D [03] 2226       534          BHI Turn        ; If IR3 > IRL goto Turn

126F [06] BD1566     535          JSR Strait

536

537  *   Test IR2 (Front Left)

538

1272 [02] C602       539          LDAB #!2

1274 [06] BD1612     540          JSR IRRead      ; Read IR2

1277 [04] B6107A     541          LDAA IR2        ; A = IR2

127A [04] B11086     542          CMPA IRL        ; Compare A to IRL

127D [03] 2244       543          BHI TL          ; If IR2 > IRL Turn Left

127F [06] BD1566     544          JSR Strait

545

546  *   Test IR4 (Front Right)

547

1282 [02] C604       548          LDAB #!4

1284 [06] BD1612     549          JSR IRRead      ; Read IR4

1287 [04] B6107C     550          LDAA IR4        ; A = IR4

128A [04] B11086     551          CMPA IRL        ; Compare A to IRL

128D [03] 2503       552          BLO TLDone      ; If IR4 < IRL Loop

128F [03] 7E12ED     553          JMP TR          ; else turn right

1292 [03] 7E1324     554  TLDone  JMP AvDone

555

556  * If one of the test fails Turn Thomas

557

1295 [02] 8650       558  Turn    LDAA #$50

1297 [04] B7107F     559          STAA PWMDC1

129A [04] B71080     560          STAA PWMDC2

129D [06] BD167B     561          JSR AdjPWM      ; Slow Down For Turns

562

12A0 [02] C601       563          LDAB #!1

12A2 [06] BD1612     564          JSR IRRead

12A5 [04] B61079     565          LDAA IR1        ; A = IR1 (Left Front)

12A8 [04] B11086     566          CMPA IRL        ; Compare IR1 to IRL

12AB [03] 2240       567          BHI TR          ; if Left Blocked, Turn

Right

568

12AD [02] C605       569          LDAB #!5

12AF [06] BD1612     570          JSR IRRead

12B2 [04] B6107D     571          ldaa IR5        ; if Left Clear, Check

right, A = IR5 (Right Front)

12B5 [04] B11086     572          CMPA IRL

12B8 [03] 2209       573          BHI TL          ; if Left Clear & Right

Blocked, Turn Left

12BA [03] CE1000     574          LDX #REGBAS     ; If Left and right Clear,

flip coin

12BD [05] EC0E       575          LDD TCNT,X

12BF [02] C188       576          cmpb #$88

12C1 [03] 2C2A       577          Bge TR          ; if MSB of TCNT in Lower

half turn right

578  *                       ;  Else Turn left.

579

580  *    If IR1 is clear turn until front is clear

581

12C3 [02] 860F       582  TL      LDAA #!15

12C5 [04] B71045     583          STAA NewPAN

12C8 [02] 8609       584          LDAA #!9

12CA [04] B71046     585          STAA NewTilt

12CD [06] BD14EE     586          JSR MoveHead

587

12D0 [04] B61055     588          LDAA DDOV       ; A = DDOV

12D3 [02] 84FC       589          ANDA #%11111100 ; Mask Motor Direction Bits

12D5 [02] 8B01       590          ADDA #%00000001 ; Turn toward Left

12D7 [04] B71055     591          STAA DDOV

12DA [04] B70200     592          STAA DDO

593

12DD [02] C602       594  LLoop   LDAB #!2

12DF [06] BD1612     595          JSR IRRead

12E2 [04] B6107A     596          LDAA IR2        ; A = IR2 (Last IR. to

Clear)

12E5 [04] B11086     597          CMPA IRL

12E8 [03] 2AF3       598          BPL  LLoop      ; If IR4 still blocked

continue turn

599

12EA [03] 7E1324     600          JMP AvDone      ; Ir IR4 clear go strait

601

602  * Turn Right

603

12ED [02] 86A5       604  TR      LDAA #!165

12EF [04] B71045     605          STAA NewPAN

12F2 [02] 8609       606          LDAA #!9

12F4 [04] B71046     607          STAA NewTilt

12F7 [06] BD14EE     608          JSR MoveHead

609

12FA [04] B61055     610          LDAA DDOV       ; A = DDOV

12FD [02] 84FC       611          ANDA #%11111100 ; Mask Motor Direction Bits

12FF [02] 8B02       612          ADDA #%00000010 ; Turn toward Right

1301 [04] B71055     613          STAA DDOV

1304 [04] B70200     614          STAA DDO

615

1307 [02] C604       616  RLoop   LDAB #!4

1309 [06] BD1612     617          JSR IRRead

130C [04] B6107C     618          LDAA IR4        ; A = IR4 (Last IR. to

Clear)

130F [04] B11086     619          CMPA IRL

1312 [03] 22F3       620          BHI  RLoop      ; If IR2 still blocked

continue turn

621

1314 [02] C601       622          LDAB #!1

1316 [06] BD1612     623          JSR IRRead

1319 [04] B61079     624          LDAA IR1        ; A = IR1

131C [04] B1108B     625          CMPA IRL+5

131F [03] 22E6       626          BHI RLoop       ; Move away from side wall

1321 [03] 7E1324     627          JMP AvDone      ; IF IR2 clear go strait

628

1324 [06] 1838       629  AvDone  PULY

1326 [05] 38         630          PULX

1327 [04] 33         631          PULB

1328 [04] 32         632          PULA

1329 [05] 39         633          RTS

634

635  ***************************************

636  * Rotate Subroutine

637  *

638  * This subroutine rotates the servo head through

360 degrees in 10 degree

639  *   increments. Rotates to next position each time

it is called

640  *

641  * Input     : None

642  * Output    : Variable EndR = ff when done rotation

643  * Distroyes : None

644  * Calls     : MoveHead

645  ****************************************

132A [03] 36         646  Rotate  PSHA

132B [03] 37         647          PSHB

132C [04] 3C         648          PSHX

649

650  * increment rotate counter

651

132D [04] B61074     652          ldaa Times

1330 [02] 4C         653          inca

1331 [04] B71074     654          STAA Times

1334 [02] 8101       655          CMPA #!1

1336 [03] 260E       656          BNE MRLoop      ; If not 1st time skip

initialization

657

658  * Move tilt into initial position

659

1338 [02] 8609       660          ldaa #$09

133A [04] B71046     661          STAA newtilt    ; Tilt position for

right-side-up level

133D [03] CE1060     662          LDX #fr

1340 [05] FF1075     663          STX PosPtr      ; Store Initial Position

Pointer

1343 [06] 7F1077     664          CLR Endr

665

666  * Determine next pan position

667

1346 [05] FE1075     668  MRLoop  ldx PosPtr      ; X=address for fr (Pan

Front coordinate)

1349 [04] A600       669          ldaa 0,X        ; A = pan position indicated

by X

134B [02] 81FF       670          CMPA #$ff       ;

134D [03] 2716       671          BEQ TRDone         ; if A=ff goto test for

end

134F [04] B71045     672          staa NewPan     ; if A<>ff store new pan

1352 [03] 08         673          INX             ; increment table pointer

1353 [05] FF1075     674          STX PosPtr      ; Store Next Position

1356 [03] 7E1373     675          JMP rdone       ; goto inner loop

676

1359 [04] B61046     677  TFE     LDAA NewTilt

135C [02] 81BE       678          CMPA #$be       ;

135E [03] 2705       679          BEQ TRDone      ; GOTO totally Done, End of

rotation

1360 [02] 86BE       680          LDAA #$be

1362 [04] B71046     681          STAA NewTilt    ; Tilt position for upside

down level

682

1365 [03] CE1060     683  TRDone  LDX #fr

1368 [05] FF1075     684          STX PosPtr      ; Store Initial Position

Pointer

136B [02] 86FF       685          LDAA #$ff

136D [04] B71077     686          STAA ENDR       ; Indicate end of rotation

1370 [06] 7F1074     687          CLR Times       ; Clear Rotate times counter

688

1373 [06] BD14EE     689  RDone   JSR MoveHead

1376 [05] 38         690          PULX

1377 [04] 33         691          PULB

1378 [04] 32         692          pula

1379 [05] 39         693          RTS

694  ***************************************

695  *       Sonar Subroutine

696  *

697  * This subroutine reads the sonar and updates the

sonar variable

698  *

699  * Input    : None

700  * Output   : SonVal

701  * Destroys : None

702  * Calls    : None

703  ***************************************

704

137A [03] 36         705  Sonar   PSHA

137B [03] 37         706          PSHB

137C [04] 3C         707          PSHX

137D [05] 183C       708          PSHY

709

710  * Initialize

711

137F [06] 7F1058     712          clr oflow

1382 [06] 7F105F     713          CLR resultf

1385 [03] CE0000     714          LDX #$0

1388 [05] FF1056     715          STX ICnt

138B [05] FF1059     716          STX SonVal

138E [03] CE1000     717          LDX #REGBAS

718

719  * Enable IC3

720

1391 [07] 1D2201     721          BCLR tmsk1,x,%00000001

722

723  * Enable IC3 to trigger on any edge

724

1394 [07] 1C2103     725          BSET tctl2,x,%00000011

726

727  * Place current value of TCNT into ICnt

728

1397 [03] CE1000     729          LDX #REGBAS

139A [05] EC0E       730          LDD TCNT,X      ; D=TCNT

139C [05] FD1056     731          STD ICnt        ; TCNT->ICnt

732

733  * turn on 40kHz

734

139F [04] B61055     735          LDAA DDOV

13A2 [02] 8AE0       736          ORAA #%11100000 ; Set bits 7,6,5 to address

of sonar

13A4 [02] 84E7       737          ANDA #%11100111 ; Clear Bits 4,3 to enable

40 kHz

13A6 [04] B71055     738          STAA DDOV

13A9 [04] B70200     739          STAA DDO

740

741  * Wait while pulse is sent

742

13AC [02] C64F       743          LDAB #$004f

13AE [02] 5A         744  SL1     DECB

13AF [03] 2EFD       745          BGT SL1

746

747  *  Turn off 40kHz

748

13B1 [04] B61055     749          LDAA DDOV

13B4 [02] 841F       750          ANDA #%00011111 ; Clear bits 7,6,5

13B6 [04] B71055     751          STAA DDOV       ; Turn off all IR.

13B9 [04] B70200     752          STAA DDO

753

754  * Wait for the amplifier to settle

755

13BC [02] C6FF       756          ldab #$00ff

13BE [02] 5A         757  SL2     DECB

13BF [03] 2EFD       758          BGT SL2         ; Wait for circuit to settle

759

760  * Clear IC3 and TCNT Flags

761

13C1 [07] 1D23FE     762          BCLR TFLG1,X,%11111110

13C4 [07] 1D257F     763          BCLR TFLG2,X,%01111111

764

765  * Look for return flag (IC3) or wait for 2 TCNT

Flags

766

13C7 [02] C602       767          LDAB #$2        ; TCNT loop Count = 2

13C9 [04] A623       768  here    LDAA TFLG1,X    ; A = TFLG1 (Look for

Capture)

13CB [02] 46         769          RORA            ; Move Bit 0 into the Carry

13CC [03] 251A       770          BCS Capture     ; if TFLG1 bit 0 = 1 goto

Capture

771  *                       ; if no capture, test for

time-out

13CE [04] A625       772          LDAA TFLG2,X    ; A = TFLG2

13D0 [03] 2AF7       773          BPL here        ; if Bit 7 = 0 continue

looking

13D2 [07] 1D257F     774          BCLR TFLG2,X,%01111111 ;if Bit 7 = 1, clear

TCNT Flag

775

13D5 [02] 5A         776          DECB            ; Decrement Loop count

13D6 [03] 2705       777          BEQ Timeout     ; If loop count = 0 assume

no return, goto Loop

13D8 [06] 7C1058     778          INC oflow       ; if loop count <> 0

Increment oflow

13DB [03] 20EC       779          BRA here        ; Continue waiting

780

13DD [04] B6105F     781  Timeout ldaa resultf    ; A=Result Flag variable

13E0 [02] 8A01       782          ORAA #%00000001 ; Set time out flag

13E2 [04] B7105F     783          STAA resultf    ; Store resultf

13E5 [03] 7E142C     784          jmp End

785

786  * If return signal is captured, test for TCNT

o-flow.

787

13E8 [06] 7D1058     788  Capture tst oflow

13EB [03] 270D       789          BEQ noflow      ; if oflow=0 calculate time

with no over flow

13ED [03] CCFFFF     790          LDD #$ffff      ; if oflwo<>0 calculate

time with over flow

791

13F0 [06] B31056     792          SUBD ICnt       ; FFFF - ICnt -> D

13F3 [06] E314       793          ADDD TIC3,X     ; D = (FFFF-ICnt)+TIC3

13F5 [05] FD1056     794          STD ICnt

13F8 [03] 200A       795          BRA DBL

796

13FA [05] EC14       797  noflow  LDD TIC3,X      ; Load value of TCNT When

IC3 Triggered

13FC [06] B31056     798          SUBD ICnt       ; TIC3 - ICnt -> D

13FF [05] FD1056     799          STD ICnt        ; ICnt = Timer Difference

i.e. Time of Flight

1402 [03] 2000       800          BRA DBL

801

802  * Check if reading is to close to be accurate

803

1404 [05] FC1056     804  DBL     LDD ICnt        ; D=ICnt

1407 [05] 1A830A00   805          CPD #$0A00      ; Closest allowable reading

is 0A00

140B [03] 2C0B       806          BGE AVGR        ; if ICnt => 0A00 goto AVGR

140D [04] B6105F     807          LDAA resultf    ; if ICnt < 0A00 set result

flag

1410 [02] 8A02       808          ORAA #%00000010 ; set To-Close Flag

1412 [04] B7105F     809          STAA resultf    ; Store result flag

1415 [03] 7E142C     810          JMP End        ; continue BigL

811

812  * Average the old and new readings

813

1418 [05] FC1059     814  AVGR    LDD SonVal      ; D=Sonval

141B [06] F31056     815          ADDD ICnt       ; D=sonVal+ICnt

141E [03] 2506       816          BCS addsome     ; if Carry = 1 the addition

had a carry that must

817  *                       ;  be accounted for

1420 [05] FD1059     818          STD SonVal

1423 [03] 7E142C     819          JMP End

820

1426 [02] 01         821  addsome NOP             ;LSRD

1427 [02] 8A80       822          ORAA #%10000000 ; Set D bit 15 to account

for carry

1429 [05] FD1059     823          STD SonVal

824

825  * determine final output

826

142C [05] FC1059     827  End     LDD SonVal

142F [03] 2E1F       828          BGT OUT         ; If final average > 0 you

have a valid reading

1431 [04] B6105F     829          LDAA resultf    ; if final average = 0 look

at result Flags

1434 [02] 44         830          LSRA            ; Move To-Far bit into Carry

1435 [03] 2408       831          BCC TTC         ; If To-Far bit = 0 goto

Test To Close

1437 [03] CCFFFF     832          LDD #$ffff      ; if To-Far bit = 1 D=ffff

143A [05] FD1059     833          STD SonVal      ; SonVal = ffff to indicate

To-Far

143D [03] 2011       834          BRA OUT

835

143F [02] 44         836  TTC     LSRA            ; Move To-Close bit into

Carry

1440 [03] 2408       837          BCC UNK         ; If To-Close bit = 0 goto

UNKnown

1442 [03] CC0000     838          LDD #$0000      ; If To-Close bit = 1 D=0000

1445 [05] FD1059     839          STD SonVal      ; SonVal = 0000 to indicate

To-Close

1448 [03] 2006       840          BRA OUT

841

144A [03] CCABCD     842  UNK     LDD #$ABCD      ; This is the unknown case

144D [05] FD1059     843          STD SonVal      ; ABCD indicates unknown

problem

844

845

1450 [06] 1838       846  OUT     PULY

1452 [05] 38         847          PULX

1453 [04] 33         848          PULB

1454 [04] 32         849          PULA

1455 [05] 39         850          rts

851  ***************************************

852  * Servoini Subroutine

853  *

854  *   Setup OC4 and OC5 for servo control

855  *

856  * Input    : None

857  * Output   : None

858  * Destroys : None

859  * Calls    : None

860  ***************************************

1456 [04] 3C         861  Servoini PSHX

862

1457 [03] CE1000     863          LDX #REGBAS

864

145A [07] 1D2604     865          BCLR pactl,X,%00000100  ; Enable OC5

866

867  * Set OC4 & OC5 to go low

868

145D [07] 1C200A     869          BSET TCTL1,X,%00001010

1460 [07] 1D2005     870          BCLR TCTL1,X,%00000101

871

872  * Force OC4 & OC5

873

1463 [07] 1C0B18     874          BSET CFORC,X,%00011000

875

876  * Setup OC4 and OC5 to toggle

877

1466 [07] 1C2005     878          BSET TCTL1,X,%00000101

1469 [07] 1D200A     879          BCLR TCTL1,X,%00001010  ; Set OC4 and OC5

to toggle

880

881  * Turn on OC4 and OC5 Interrupts

882

146C [07] 1C2218     883          BSET TMSK1,X,%00011000

884

885  * Clear any old OC4 or OC5 Interrupt flag

886

146F [07] 1D23E7     887          BCLR TFLG1,X,%11100111

888

889  * Clear int4 & int5

890

1472 [06] 7F104B     891          CLR int4

1475 [06] 7F104C     892          CLR int5

893

1478 [05] 38         894          PULX

1479 [05] 39         895          RTS

896

897  ***************************************

898  *  OC4ISR

899  *

900  *   This ISR Controls OC4 and produces the PWM

signal needed to drive a

901  *      Standard Futaba Servo

902  *

903  * Input    : None

904  * Output   : None

905  * Destroys : None

906  * Calls    : None

907  ***************************************

147A [03] CE1000     908  OC4ISR  LDX     #REGBAS

147D [04] B6104B     909          LDAA    int4    ; int4 indicates if OC4 is

to go low or high

1480 [03] 2623       910          BNE     off4    ; if int4 <> 0 calculate

off time

1482 [05] FC104D     911          LDD     high4   ; if int4 = 0 calculate on

time, D=#-f cycles high

1485 [05] 1A8311F8   912          cpd     #!4600  ; Highest acceptable value

1489 [03] 2F06       913          BLE     TLO4    ; if High4<=4300 goto TLO4

148B [03] CC11F8     914          LDD     #!4600  ; if High4>4300, use 4300

148E [03] 7E149A     915          JMP     cont4

1491 [05] 1A830208   916  TLO4    CPD     #!520   ; Lowest acceptable value

1495 [03] 2C03       917          bge     cont4   ; if 400<=High4=>4300 the

number is valid

1497 [03] CC0208     918          LDD     #!520   ; if High4<400 use 400

149A [06] E31C       919  cont4   addd    toc4,X  ; D=high4+toc4

149C [05] ED1C       920          std     toc4,X  ; toc4=high4+toc4

149E [02] 8601       921          ldaa    #!1     ;

14A0 [04] B7104B     922          staa    int4    ; int4 <> 0

14A3 [03] 200B       923          bra     done4

14A5 [03] CC9538     924  off4    LDD     #$9538  ; D=Standard off time

14A8 [06] E31C       925          ADDD    toc4,X  ; D= off time + toc4

14AA [05] ED1C       926          STD     toc4,X  ; toc4=toc4+off time

14AC [02] 4F         927          CLRA

14AD [04] B7104B     928          STAA    int4    ; int4 = 0

14B0 [07] 1D23EF     929  done4   bclr    tflg1,X,%11101111       ; reset OC4

interrupt

14B3 [12] 3B         930          RTI

931  ***************************************

932  *  OC5ISR

933  *

934  *   This ISR Controls OC5 and produces the PWM

signal needed to drive a

935  *      Standard Futaba Servo

936  *

937  * Input    : None

938  * Output   : None

939  * Destroys : None

940  * Calls    : None

941  ***************************************

14B4 [03] CE1000     942  OC5ISR  LDX     #REGBAS

14B7 [04] B6104C     943          LDAA    int5    ; int5 indicates if OC5 is

to go low or high

14BA [03] 2623       944          BNE     off5    ; if int5 <> 0 calculate

off time

14BC [05] FC104F     945          LDD     high5   ; if int5 = 0 calculate on

time, D=#-f cycles high

14BF [05] 1A8311F8   946          cpd     #!4600  ; Highest acceptable value

14C3 [03] 2F06       947          BLE     TLO5    ; if High5<=4300 goto TLO5

14C5 [03] CC11F8     948          LDD     #!4600  ; if High5>4300, use 4300

14C8 [03] 7E14D4     949          JMP     cont5

14CB [05] 1A830208   950  TLO5    CPD     #!520   ; Lowest acceptable value

14CF [03] 2C03       951          bge     cont5   ; if 400<=High5=>4300 the

number is valid

14D1 [03] CC0208     952          LDD     #!520   ; if High5<400 use 400

14D4 [06] E31E       953  cont5   addd    toc5,X  ; D=high5+toc5

14D6 [05] ED1E       954          std     toc5,X  ; toc5=high5+toc5

14D8 [02] 8601       955          ldaa    #!1     ;

14DA [04] B7104C     956          staa    int5    ; int5 <> 0

14DD [03] 200B       957          bra     done5

14DF [03] CC9538     958  off5    LDD     #$9538  ; D=Standard off time

14E2 [06] E31E       959          ADDD    toc5,X  ; D= off time + toc5

14E4 [05] ED1E       960          STD     toc5,X  ; toc5=toc5+off time

14E6 [02] 4F         961          CLRA

14E7 [04] B7104C     962          STAA    int5    ; int5 = 0

14EA [07] 1D23F7     963  done5   bclr    tflg1,X,%11110111       ; reset OC5

interrupt

14ED [12] 3B         964          RTI

965

966  ***************************************

967  * MoveHead Subroutine

968  *

969  *  This subroutine moves the Pan/Tilt servo head to

its

970  *   new position SMOOTHLY

971  *

972  * Input: New head position is stored in NewPan and

NewTilt

973  * Output: The servo head is moved and the final

position is stored in

974  *          PAN and TILT

975  * Destroys: The old values in PAN and TILT are

replaced with the values

976  *            in NewPan and NewTilt

977  * Calls: ServoCalc

978  ***************************************

14EE [03] 36         979  MoveHead PSHA

14EF [03] 37         980          PSHB

14F0 [04] 3C         981          PSHX

982

983  * Disable IC Interrupts

984

14F1 [03] CE1000     985          LDX #REGBAS

986

14F4 [07] 1D2207     987          BCLR tmsk1,x,%00000111

988

989  * Calculate New Servo position High times

990

14F7 [04] B61045     991          LDAA NewPAN

14FA [06] BD1555     992          JSR ServoCalc

14FD [05] FD1051     993          STD NewHigh4

1500 [04] B61046     994          ldaa NewTILT

1503 [06] BD1555     995          JSR ServoCalc

1506 [05] FD1053     996          STD NewHigh5

997

998  * Wait

999

1509 [03] CE0100    1000  MHLoop  LDX #$0100

150C [03] 09        1001  MHL1    DEX

150D [03] 26FD      1002          BNE MHL1

1003

1004  * Adjust Pan High Time

1005

150F [05] FC104D    1006          LDD High4

1512 [07] 1AB31051  1007          CPD NewHigh4    ; High4-NewHigh4=?

1516 [03] 270F      1008          BEQ TTilt       ; if High4=NewHigh4 goto

TTilt

1518 [03] 2D08      1009          BLT Pandn       ; if High4>NewHigh4 goto

pandn

151A [03] 8F        1010          XGDX            ; if High4<NewHigh4 X=High4

151B [03] 09        1011          DEX             ; X=High4-1

151C [05] FF104D    1012          STX High4

151F [03] 7E1527    1013          JMP TTilt

1522 [03] 8F        1014  PanDn   XGDX            ; X=High4

1523 [03] 08        1015          INX             ; X=High4+1

1524 [05] FF104D    1016          STX High4

1017

1527 [05] FC104F    1018  TTilt   LDD High5

152A [07] 1AB31053  1019          CPD NewHigh5    ; High5-NewHigh5=?

152E [03] 2712      1020          BEQ TDone       ; if High5=NewHigh5 goto

TDone

1530 [03] 2D08      1021          BLT Tiltdn      ; if High5>NewHigh5 goto

pandn

1532 [03] 8F        1022          XGDX            ; if High5<NewHigh5 X=High5

1533 [03] 09        1023          DEX             ; X=High5-1

1534 [05] FF104F    1024          STX High5

1537 [03] 7E1509    1025          JMP MHLoop

153A [03] 8F        1026  TiltDn  XGDX            ; X=High5

153B [03] 08        1027          INX             ; X=High5+1

153C [05] FF104F    1028          STX High5

153F [03] 7E1509    1029          JMP MHLoop

1030

1542 [05] FC104D    1031  TDone   LDD High4

1545 [07] 1AB31051  1032          CPD NewHigh4

1549 [03] 26BE      1033          BNE MHLoop

1034

1035  * enable IC Interrupts

1036

154B [03] CE1000    1037  TCDone  LDX #REGBAS

1038

154E [07] 1C2206    1039          BSET tmsk1,x,%00000110

1040

1551 [05] 38        1041          PULX

1552 [04] 33        1042          PULB

1553 [04] 32        1043          PULA

1554 [05] 39        1044          RTS

1045

1046

1047  ***************************************

1048  * ServoCalc

1049  *

1050  * This subroutine calculates the offset necessary

for correct servo

1051  *  placement

1052  *

1053  * NOTE *** This routine is only valid for SvSteps

=> 16

1054  *

1055  * Input:    A = Desired servo position

1056  * Output:   D = Total Hi-Time offset for servo

position

1057  * Destroys: B

1058  * Calls:    None

1059  ***************************************

1060

1555 [04] 3C        1061  ServoCalc PSHX

1556 [03] 36        1062          PSHA            ; A = Desired Servo position

1063

1557 [03] CC1130    1064          LDD #!4400      ; D=# of E-cycles for

complete Servo Movement

155A [05] FE1049    1065          LDX SvSteps     ; X=# of Steps for complete

Servo Movement

155D [41] 02        1066          IDIV            ; X=# of E-Cycles per Step

1067

155E [03] 8F        1068          XGDX            ; B=# of E-Cycles per Step

155F [04] 32        1069          PULA            ; A=Desired Servo Position

1560 [10] 3D        1070          MUL             ; D=Calculated offset for

servo position

1071

1561 [04] C30208    1072          ADDD #!520      ; D=Total Hitime offset for

servo position

1073

1564 [05] 38        1074          PULX

1565 [05] 39        1075          RTS

1076  ***************************************

1077  * Strait Subroutine

1078  *

1079  * This subroutine is called when Thomas is going

straight.  It uses the

1080  *  motor encoders to determine if Thomas is going

straight and makes

1081  *  adjustments accordingly.

1082  ***************************************

1083

1566 [03] 36        1084  Strait  PSHA

1567 [03] 37        1085          PSHB

1568 [04] 3C        1086          PSHX

1087

1569 [04] B61084    1088          LDAA STRR

156C [04] B7107F    1089          STAA PWMDC1

156F [04] B61085    1090          LDAA STRL

1572 [04] B71080    1091          STAA PWMDC2     ; Full Speed Ahead!

1575 [02] 8609      1092          LDAA #$09

1577 [04] B71046    1093          STAA NewTilt

157A [02] 8655      1094          ldaa #$55

157C [04] B71045    1095          STAA NewPAN

157F [06] BD14EE    1096          JSR MoveHead    ; look strait ahead

1097

1582 [04] B61055    1098          LDAA DDOV       ; A = DDOV

1585 [02] 84FC      1099          ANDA #%11111100 ; Mask Motor Direction

1587 [02] 8A03      1100          ORAA #%00000011 ; Go Strait

1589 [04] B71055    1101          STAA DDOV

158C [04] B70200    1102          STAA DDO        ; Go Strait

1103

1104  * Compare RMCnt with LMCnt

1105

158F [05] FC1087    1106          LDD RMCnt

1592 [07] 1AB31089  1107          CPD LMCnt

1596 [03] 2734      1108          BEQ RL100

1598 [03] 2D19      1109          BLT DecRt

1110

1111

1112  * If RMCnt > LMCnt decrement STRR & increment STRL

1113

1114

159A [06] 7C1085    1115          INC STRL

159D [02] 8664      1116          ldaa #!100

159F [04] B11085    1117          cmpa STRL

15A2 [03] 2E28      1118          BGT RL100       ; If 100 > STRL continue

15A4 [02] 8663      1119          LDAA #!99       ; else set STRL = 99

15A6 [04] B71085    1120          STAA STRL

1121

15A9 [06] 7A1084    1122          DEC STRR

15AC [03] 2E1E      1123          BGT RL100       ; If STRR >0 continue

15AE [02] 8601      1124          LDAA #!1        ; else set STRR = 1

15B0 [04] B71084    1125          STAA STRR

1126

1127  * If RMCnt < LMCnt increment STRR & decrement STRL

1128

1129

15B3 [06] 7C1084    1130  DecRt   INC STRR

15B6 [02] 8664      1131          ldaa #!100

15B8 [04] B11084    1132          cmpa STRR

15BB [03] 2E0F      1133          BGT RL100       ; If 100 > STRR continue

15BD [02] 8663      1134          LDAA #!99       ; else set STRR = 99

15BF [04] B71084    1135          STAA STRR

1136

15C2 [06] 7A1085    1137          DEC STRL

15C5 [03] 2E05      1138          BGT RL100       ; If STRL >0 continue

15C7 [02] 8601      1139          LDAA #!1        ; else set STRL = 1

15C9 [04] B71085    1140          STAA STRL

1141

1142  * Check if both motors are below 99%, if so

increment both.

1143

15CC [02] 8699      1144  RL100   ldaa #%99

15CE [04] B11084    1145          CMPA STRR

15D1 [03] 2F0B      1146          BLE CStr        ; If STRR => 99 continue

strait

15D3 [04] B11085    1147          CMPA STRL

15D6 [03] 2F06      1148          BLE CStr        ; If STRL => 99 continue

strait

15D8 [06] 7C1084    1149          INC STRR

15DB [06] 7C1085    1150          INC STRL

1151

1152  * Clear motor counts and re-enable local interrupts

1153

15DE [02] 4F        1154  CStr        CLRA

15DF [02] 5F        1155          CLRB

15E0 [05] FD1087    1156          STD RMCnt

15E3 [05] FD1089    1157          STD LMCnt

1158

1159  * Implement changed parameters

1160

15E6 [04] B61084    1161          LDAA STRR

15E9 [04] B7107F    1162          STAA PWMDC1

15EC [04] B61085    1163          LDAA STRL

15EF [04] B71080    1164          STAA PWMDC2

15F2 [06] BD167B    1165          JSR AdjPWM      ; Full Speed Ahead!

1166

15F5 [05] 38        1167  StrEnd  PULX

15F6 [04] 33        1168          PULB

15F7 [04] 32        1169          PULA

15F8 [05] 39        1170          RTS

1171  ***************************************

1172  * ADSetup Subroutine

1173  *

1174  * This subroutine sets up the A/D system

1175  ***************************************

15F9 [03] 36        1176  ADSetup PSHA

15FA [04] 3C        1177          PSHX

1178

15FB [03] CE1000    1179          LDX #REGBAS     ; X = Register Offset

15FE [07] 1C3980    1180          BSET OPTION,X,%10000000 ; Power up A/D

1181

1182  * Delay for 32.77mS

1183

1601 [07] 1D257F    1184          BCLR TFLG2,X,%01111111 ; Clear Timer

Overflow Flag

1604 [04] E625      1185  AFset1  LDab TFLG2,X    ; Wait for TFLG2 to set

1606 [03] 2AFC      1186          BPL AFset1      ; IF TFLG2 IS POSITIVE, BIT

7=0, GOTO Fset

1187  *                       ;  IF TFLG2 IS NEGATIVE,

BIT 7=1, CONTINUE

1608 [07] 1D257F    1188          BCLR TFLG2,X,%01111111 ; Clear Timer

Overflow Flag

160B [04] E625      1189  AFset2  LDAB TFLG2,X    ; Wait for TFLG2 to set

160D [03] 2AFC      1190          BPL AFset2      ; IF TFLG2 IS POSITIVE, BIT

7=0, GOTO Fset

1191

160F [05] 38        1192          PULX

1610 [04] 32        1193          PULA

1611 [05] 39        1194          RTS

1195

1196

1197  ***************************************

1198  * IRRead Subroutine

1199  *

1200  * This subroutine takes readings from the IR# that

was passed to

1201  *  it in B.  It reads the corresponding sensor and

updates the

1202  *  associated IR. Variable.

1203  *

1204  * This routine takes a differential reading, i.e.

It takes a dark

1205  *  reading, then it takes a light reading, then it

subtracts the

1206  *  two, and returns the difference.

1207  *

1208  * Input    : B=IR#

1209  * Output   : IR# variable is updated

1210  * Destroys : None

1211  * Calls    : None

1212  ***************************************

1612 [03] 36        1213  IRRead  PSHA

1613 [04] 3C        1214                  PSHX

1614 [05] 183C      1215                  PSHY

1616 [03] 37        1216                  PSHB

1217

1617 [03] CE1000    1218          LDX #REGBAS     ; X = Register Offset

1219

1220  * Take Dark A/D Reading

1221  *   IF Reading IR0 Setup to read AD7.

1222

161A [03] 2602      1223          BNE ReadAD      ; if IR# <> 0 Read A/D

161C [02] C607      1224          LDAB #!7        ; if IR# = 0 Setup to read

A/D 7

1225

161E [04] E730      1226  ReadAD  STAB ADCTL,X    ; Start A/D on IR.

1227

1228

1620 [04] A630      1229  ADLoop1 LDAA ADCTL,X    ; Look for CCF

1230

1622 [03] 2AFC      1231          BPL  ADLoop1    ; if CCF = 0 Loop

1624 [04] A631      1232          LDAA ADR1,X     ; Load Reading into A

1626 [04] B7108B    1233          STAA Dark       ; Store dark reading in Dark

1234

1235  * turn on Appropriate IR. Emitter

1236

1629 [02] 58        1237          lslb

162A [02] 58        1238          lslb

162B [02] 58        1239          lslb

162C [02] 58        1240          lslb

162D [02] 58        1241          lslb            ; Move Bit # into position

162E [04] B61055    1242          LDAA DDOV       ; A = DDO Variable

1631 [02] 8403      1243          ANDA #%00000011 ; Mask IR. Control Bits,

BIT 4=0 (TURNS ON

1244  *                                       40kHz)

1633 [02] 1B        1245          ABA             ; A+B=A = New DDO Variable

1634 [04] B71055    1246          STAA DDOV       ; Store New DDO Variable to

DDOV

1247

1248  * Delay for 32.77mS

1249

1637 [07] 1D257F    1250          BCLR TFLG2,X,%01111111

163A [04] E625      1251  Fset1   LDab TFLG2,X    ; Wait for TFLG2 to set

163C [03] 2AFC      1252          BPL Fset1       ; IF TFLG2 IS POSITIVE, BIT

7=0, GOTO Fset

1253  *                       ;  IF TFLG2 IS NEGATIVE,

BIT 7=1, CONTINUE

163E [04] B70200    1254          STAA DDO        ; turn on 8-line demux

1641 [07] 1D257F    1255          BCLR TFLG2,X,%01111111

1644 [04] E625      1256  Fset2   LDAB TFLG2,X    ; Wait for TFLG2 to set

1646 [03] 2AFC      1257          BPL Fset2       ; IF TFLG2 IS POSITIVE, BIT

7=0, GOTO Fset

1258

1259  * Take Light A/D Reading

1260

1648 [04] 33        1261          PULB    ; B=IR#

1649 [03] 37        1262          PSHB

1263

1264  *   IF Reading IR0 Setup to read AD7.

1265

164A [03] 2602      1266          BNE ReadADD      ; if IR# <> 0 Read A/D

164C [02] C607      1267          LDAB #!7        ; if IR# = 0 Setup to read

A/D 7

1268

164E [04] E730      1269  ReadADD  STAB ADCTL,X    ; Start A/D on IR.

1270

1650 [04] A630      1271  ADLoop2 LDAA ADCTL,X    ; Look for CCF

1272

1652 [03] 2AFC      1273          BPL  ADLoop2    ; if CCF = 0 Loop

1654 [04] A631      1274          LDAA ADR1,X     ; Load Reading into A

1275

1276  * Turn off IR.

1277

1656 [04] F61055    1278          LDAB DDOV       ; B = DDO Variable

1659 [02] C403      1279          ANDB #%00000011 ; Mask IR. address and

motor Control Bits

165B [02] CA18      1280          ORAB #%00011000 ; SET BIT3=1 & Bit 4=1

(Demux->Clear,

1281  *                                       40kHz->off)

165D [04] F71055    1282          STAB DDOV       ; B -> DDOV

1660 [04] F70200    1283          STAB DDO        ; Turn off all IR.

1284

1285  * Store Reading into Corresponding Variable

1286

1663 [03] CE1078    1287          LDX #IR0        ; X = Address of IR0

1666 [04] 33        1288          PULB            ; B = IR#

1667 [02] 5D        1289  ADLoop3 TSTB            ; B = IR. #

1668 [03] 2704      1290          BEQ ADone       ; If B = 0 GOTO ADone

166A [03] 08        1291          INX             ; if B <> 0 IncX

166B [02] 5A        1292          DECB            ; DEC B

166C [03] 20F9      1293          BRA ADLoop3     ; Loop

1294

166E [04] B0108B    1295  ADone   SUBA Dark       ; A = Light - Dark

1671 [03] 2A01      1296          BPL ok          ; if Light - Dark => 0 goto

ok

1673 [02] 4F        1297          CLRA            ; if Light - Dark < 0 it = 0

1674 [04] A700      1298  ok      STAA 0,X        ; A -> Appropriate IR.

Variable

1299

1676 [06] 1838      1300          PULY

1678 [05] 38        1301          PULX

1679 [04] 32        1302          PULA

167A [05] 39        1303          RTS

1304

1305  ***************************************

1306  * AdjPWM Subroutine

1307  *

1308  * This subroutine takes the %duty cycle values

stored in PWMDC1, PWMDC2,

1309  *  PAN, and TILT and determines the offset in

E-Cycles for each one. It then

1310  *  institutes the change in the PWM system.

1311  *

1312  * Input    : None

1313  * Output   : None

1314  * Destroys : None

1315  * Calls    : None

1316  ***************************************

167B [03] 36        1317  AdjPWM  PSHA

167C [03] 37        1318          PSHB

167D [04] 3C        1319          PSHX

1320

167E [03] CE1000    1321          LDX #REGBAS

1322

1323  * Adjust for duty cycles > 50%

1324

1681 [02] 5F        1325          CLRB            ; B=0

1682 [04] B6107F    1326          LDAA PWMDC1     ; A=OC2 % of duty cycle

1685 [02] 8132      1327          CMPA #!50       ; See if % of duty cycle >

50%

1687 [03] 2302      1328          BLS ARNZ61      ; If % of duty cycle <= 50%

branch to OC3 duty cycle

1689 [02] CB40      1329          ADDB #%01000000 ; If % of duty cycle > 50%

B=0+01000000

168B [04] B61080    1330  ARNZ61  LDAA PWMDC2     ; A=OC3 % of duty cycle

168E [02] 8132      1331          CMPA #!50       ; See if % of duty cycle >

50%

1690 [03] 2302      1332          BLS ARNZ62      ; If % of duty cycle <= 50%

continue

1692 [02] CB20      1333          ADDB #%00100000 ; If % of duty cycle > 50%

B=B+00100000

1334

1694 [04] E70D      1335  ARNZ62  STAB OC1D,X     ; B->OC1D set the bits that

are to go high at next

1336  *                       ;  interrupt.

1337

1338  * Determine the length of 1 period in E-Cycles and

store in PWMPER and TOC1

1339

1696 [04] B61081    1340          LDAA PWMP1P     ; A=the number of cycles

for 1/2 of 1% of the period

1699 [02] C664      1341          LDAB #!100      ; B=100

169B [10] 3D        1342          MUL             ; D=AXB D=1/2 the number of

cycles for 1 period

169C [03] 05        1343          LSLD            ; Multiply by 2, D=number

of cycles for 1 period

169D [05] FD1082    1344          STD PWMPER      ; D->PWMPER

16A0 [05] ED16      1345          STD TOC1,X      ; D->TOC1

1346

1347  * Determine the Offset count for OC2, OC3, and

store in TOC2, TOC3

1348  *  respectively.

1349

16A2 [04] B6107F    1350          LDAA PWMDC1     ; A=% duty cycle for OC2

16A5 [06] BD16DD    1351          JSR CALOFF      ; Change % duty cycle (A)

to Offset Cnt (D)

16A8 [05] ED18      1352          STD TOC2,X      ; D->TOC2 TOC2=Offset Cnt

for OC2

16AA [04] B61080    1353          LDAA PWMDC2     ; A=% duty cycle for OC3

16AD [06] BD16DD    1354          JSR CALOFF      ; Change % duty cycle (A)

to Offset Cnt (D)

16B0 [05] ED1A      1355          STD TOC3,X      ; D->TOC3 TOC3=Offset Cnt

for OC3

1356

16B2 [05] 38        1357          PULX

16B3 [04] 33        1358          PULB

16B4 [04] 32        1359          PULA

16B5 [05] 39        1360          RTS

1361

1362  ***************************************

1363  * PWMSetup Subroutine

1364  *

1365  * This subroutine sets up OC1, OC2, OC3 for use as

Pulse Width

1366  *  modulation.  OC1 controls OC2 - OC3.

1367  * Input    : None

1368  * Output   : None

1369  * Destroys : None

1370  * Calls    : None

1371  ***************************************

16B6 [03] 36        1372  PWMSetup PSHA

16B7 [04] 3C        1373          PSHX

1374

16B8 [03] CE1000    1375          LDX #REGBAS             ; X=Register offset

1376

1377  * Setup OC1, OC2, OC3,

1378

16BB [07] 1C2050    1379          BSET TCTL1,X,%01010000

16BE [07] 1D20A0    1380          BCLR TCTL1,X,%10100000  ; Set OC2 & OC3 to

toggle

1381

16C1 [07] 1C0CE0    1382          BSET OC1M,X,%11100000

16C4 [07] 1D0C1F    1383          BCLR OC1M,X,%00011111

1384

1385  * Enable interrupts for OC1

1386

16C7 [07] 1D237F    1387          BCLR TFLG1,X,%01111111  ;Clear local OC1

Interrupt

1388

16CA [07] 1C2280    1389          BSET TMSK1,X,%10000000  ; Enable local OC1

interrupts

1390

16CD [04] B61055    1391          ldaa DDOV               ; A = DDOV Bit

pattern

16D0 [02] 84FC      1392          ANDA #%11111100         ; Mask out the

Motor directions bits

16D2 [02] 8A03      1393          ORAA #%00000011         ; Set bits to make

both wheels go forward

16D4 [04] B71055    1394          STAA DDOV               ; A -> DDOV

(Digital Data Out Variable)

16D7 [04] B70200    1395          STAA DDO                ; A -> DDO (Digital

Data Out)

1396

16DA [05] 38        1397          PULX

16DB [04] 32        1398          PULA

16DC [05] 39        1399          RTS

1400

1401  ***************************************

1402  * CALOFF - Subroutine

1403  * Description: This subroutine changes the duty

cycle to offset count.

1404  * If duty < 50% ($32) change to 100-duty

1405  * If duty > 100% ($64) force to 100% ($64)

1406  * multiply by 1% of period

1407  *

1408  * Input    : A=%Duty Cycle

1409  * Output   : D=Offset Count

1410  * Destroys : B

1411  * Calls    : None

1412  ***************************************

1413  * Determine if A is between 50 and 100, if A<50,

A=100-A, if A>100, A=100

1414

16DD [02] 8132      1415  CALOFF  CMPA #!50       ; Compare A to 50

16DF [03] 2404      1416          BHS ARN6A       ; If A=>50 branch

16E1 [02] 16        1417          TAB             ; If A<50 A->B, (B is lost)

16E2 [02] 8664      1418          LDAA #!100      ; A=100

16E4 [02] 10        1419          SBA             ; A=A-B

16E5 [02] 8164      1420  ARN6A   CMPA #!100      ; Compare A to 100

16E7 [03] 2302      1421          BLS ARN6B       ; If A<=100 branch

16E9 [02] 8664      1422          LDAA #!100      ; If A>100, A=100

1423

1424  * Calculate the number of cycles until a state

change is needed.

1425

16EB [04] F61081    1426  ARN6B   LDAB PWMP1P     ; B=# cycles for 1/2 of 1%

of period

16EE [10] 3D        1427          MUL             ; D=1/2 # cycles until

state change

16EF [03] 05        1428          LSLD            ; D=D*2 D=# cycles until

state change

16F0 [05] 39        1429          RTS

1430

1431  **************************************************

1432  * ICSetup Subroutine

1433  *

1434  * This subroutine sets up the input capture to

catch the pulses being sent

1435  *  to it from the motor encoders.

1436  **************************************************

16F1 [03] 36        1437  ICSetup PSHA

16F2 [04] 3C        1438          PSHX

1439

16F3 [03] CE1000    1440          LDX #REGBAS     ; X=Register offset

1441

1442  * Setup IC1, IC2 for keeping track of Left and

Right Motor Data

1443

16F6 [07] 1C213C    1444          BSET TCTL2,X,%00111100  ; Select input

capture of both edges

1445  *                               ;   for IC1 & IC2

1446

1447  * Enable Local interrupts for  IC1, IC2

1448

16F9 [07] 1D23F9    1449          BCLR TFLG1,X,%11111001  ; Clear local IC1,

IC2 Interrupt

1450

16FC [07] 1C2206    1451          bset TMSK1,X,%00000110  ; Enable local IC1,

IC2 interrupts

1452

16FF [05] 38        1453          PULX

1700 [04] 32        1454          PULA

1701 [05] 39        1455          RTS

1456

1457  ***************************************

1458  * OC1ISR - OUTPUT COMPARE 1 SERVICE ROUTINE

1459  * Description: The OC1 Interrupt service Routine

does the following

1460  *  a) Calculate next compare value for OC1 and

store in TOC1

1461  *  b) Calculate next compare value for OC2 and

store in TOC2

1462  *  c) Calculate next compare value for OC3 and

store in TOC3

1463  *  e) Clear Flag

1464  * Input    : None

1465  * Output   : None

1466  * Destroys : None

1467  * Calls    : None

1468  ***************************************

1469

1702 [03] CE1000    1470  OC1ISR  LDX #REGBAS      ; X=Register offset

1471

1472  * Calculate Next value for TOC1

1473

1705 [05] EC16      1474          LDD TOC1,X      ; D=TOC1 (last OC1 Compare

Value)

1707 [06] F31082    1475          ADDD PWMPER     ; D=(last OC1 compare

value) + (# cycles for 1 period)

170A [05] ED16      1476          STD TOC1,X      ; D->TOC1 (Next OC1 compare

value)

1477

1478  * Calculate Next value for TOC2

1479

170C [05] EC18      1480          LDD TOC2,X      ; D=TOC2 (last OC2 Compare

Value)

170E [06] F31082    1481          ADDD PWMPER     ; D=(last OC2 Compare

Value) + (# cycles for 1 period)

1711 [05] ED18      1482          STD TOC2,X      ; D->TOC2 (Next OC2 Compare

Value)

1483

1484  * Calculate Next value for TOC3

1485

1713 [05] EC1A      1486          LDD TOC3,X      ; D=TOC3 (last OC3 Compare

Value)

1715 [06] F31082    1487          ADDD PWMPER     ; D=(last OC3 compare

value) + (# cycles for 1 period)

1718 [05] ED1A      1488          STD TOC3,X      ; D->TOC3 (Next OC3 Compare

Value)

1489

171A [07] 1D237F    1490          BCLR TFLG1,X,%01111111  ; Clear OC1

Interrupt

171D [12] 3B        1491          RTI

1492

1493  ****************************************************

***************************

1494  * SVIC1 - IC1 Interrupt Service Routine

1495  *

1496  * Description: This Service Routine keeps track of

Right Motor Data

1497  *

1498  * Input    : None

1499  * Output   : None

1500  * Destroys : None

1501  * Calls    : None

1502  ****************************************************

***************************

171E [05] FE1087    1503  SVIC1   LDX RMCnt

1721 [03] 08        1504          INX

1722 [05] FF1087    1505          STX RMCnt

1506

1507  * Clear interrupts for IC1

1508

1725 [03] CE1000    1509          LDX #REGBAS

1728 [07] 1D23FB    1510          BCLR TFLG1,X,%11111011  ; Clear local IC1

Interrupt

1511

172B [12] 3B        1512          RTI

1513

1514  ****************************************************

***************************

1515  * SVIC2 - IC2 Interrupt Service Routine

1516  *

1517  * Description: This Service Routine keeps track of

Left Motor Data

1518  *

1519  * Input    : None

1520  * Output   : None

1521  * Destroys : None

1522  * Calls    : None

1523  ****************************************************

***************************

172C [05] FE1089    1524  SVIC2   LDX LMCnt

172F [03] 08        1525          INX

1730 [05] FF1089    1526          STX LMCnt

1527

1528  * Clear interrupts for IC2

1529

1733 [03] CE1000    1530          LDX #REGBAS

1736 [07] 1D23FD    1531          BCLR TFLG1,X,%11111101  ; Clear local IC2

Interrupt

1532

1739 [12] 3B        1533          RTI

1534

1535

1536

1537

Symbol Table

ADCTL            0030

ADDSOME          1426

ADJPWM           167B

ADLOOP1          1620

ADLOOP2          1650

ADLOOP3          1667

ADONE            166E

ADR1             0031

ADR2             0032

ADR3             0033

ADR4             0034

ADSETUP          15F9

AFSET1           1604

AFSET2           160B

ALOOP            120B

ARN6A            16E5

ARN6B            16EB

ARNZ61           168B

ARNZ62           1694

ASONVAL          105D

AV10DONE         123C

AV10LC           108E

AV10SONAR        11FD

AVDONE           1324

AVGR             1418

AVOID            125A

BUF              1109

CALOFF           16DD

CAPTURE          13E8

CFORC            000B

CHKLLOOP         1154

CHKRLOOP         1190

CONT4            149A

CONT5            14D4

CSTR             15DE

DARK             108B

DBL              1404

DDO              0200

DDOV             1055

DECRT            15B3

DEGMULT          108F

DONE4            14B0

DONE5            14EA

END              142C

ENDR             1077

ER               11C9

FR               1060

FR1              106E

FSET1            163A

FSET2            1644

HERE             13C9

HIGH4            104D

HIGH5            104F

HIPAN            1047

HISONVAL         105B

HITILT           1048

ICNT             1056

ICSETUP          16F1

INT4             104B

INT5             104C

IR0              1078

IR1              1079

IR2              107A

IR3              107B

IR4              107C

IR5              107D

IR6              107E

IRL              1086

IRREAD           1612

LLOOP            12DD

LMCNT            1089

MAIN             1092

MAINLOOP         10DF

MHL1             150C

MHLOOP           1509

MOVEHEAD         14EE

MRLOOP           1346

NEWHI            11E7

NEWHIGH4         1051

NEWHIGH5         1053

NEWPAN           1045

NEWTILT          1046

NOC              1235

NOFLOW           13FA

OC1D             000D

OC1ISR           1702

OC1M             000C

OC4ISR           147A

OC5ISR           14B4

OFF4             14A5

OFF5             14DF

OFLOW            1058

OK               1674

OPTION           0039

OUT              1450

PACTL            0026

PAN              1043

PANDN            1522

POSPTR           1075

PWMDC1           107F

PWMDC2           1080

PWMP1P           1081

PWMPER           1082

PWMSETUP         16B6

RDONE            1373

READAD           161E

READADD          164E

REGBAS           1000

RESULTF          105F

RL100            15CC

RLOOP            1307

RMCNT            1087

ROT              10F1

ROTATE           132A

RTIINI           124E

RTIISR           1240

RTIS             108C

RW               11B9

RW1              11B6

SCAN             11A9

SDONE            11F9

SERVOCALC        1555

SERVOINI         1456

SL1              13AE

SL2              13BE

SMLOOP           11B1

SONAR            137A

SONARTURN        111B

SONVAL           1059

STDONE           119C

STRAIT           1566

STREND           15F5

STRL             1085

STRR             1084

SVIC1            171E

SVIC2            172C

SVSTEPS          1049

TARGETCNT        1090

TCDONE           154B

TCNT             000E

TCTL1            0020

TCTL2            0021

TDONE            1542

TFE              1359

TFLG1            0023

TFLG2            0025

TIC3             0014

TILT             1044

TILTDN           153A

TIMEOUT          13DD

TIMES            1074

TL               12C3

TLDONE           1292

TLO4             1491

TLO5             14CB

TMSK1            0022

TMSK2            0024

TOC1             0016

TOC2             0018

TOC3             001A

TOC4             001C

TOC5             001E

TR               12ED

TRDONE           1365

TTC              143F

TTILT            1527

TURN             1295

TURNRIGHT        1160

UNK              144A