EEl 5934

Machine Intelligence Laboratory

Professor Keith L. Doty

TA Scott Jantz

Spring 1996

project by Josh Lane

Robot:  Yard Dog

TABLE OF CONTENTS

Abstract

Executive Summary								3

Introduction									3

Integrated System								4

Mobile Platform								4

Actuation									5

Sensors									6

Behaviors									9

Experimental Layout and Results						10

Conclusion									12

References									13

Appendix 									14

ABSTRACT

As the name implies, Yard Dog simulates some of the simple characteristics often associated with household pets.  In particular, the main function of Yard Dog is to sit around the yard and keep stray animals from trespassing.  Essentially the robot will also warn humans to respect property as well.  To accomplish this goal, Yard Dog, has an aggressive nature when needed.  Otherwise it simply lounges around the yard until its services are needed.  Unlike an animal however, Yard Dog does have certain altered behaviors which dogs may not worry with.  For instance it can not jump therefore it must determine how to negotiate obstacles.  Electronic circuitry must also be protected from water which is accomplished through water sensory.

EXECUTIVE SUMMARY

Yard Dog is a derivative of the initial project robot named Treads.  Having a flexible platform allowed for the survival of this project when time lines weren't met for implementing certain sensors.  The initial project required the robot to seek out a sonar beacon in hostile territory to include land mines and enemies.  When the sonar receiving circuitry was not realized in an over allotment of time, the goals of the project were forced to be changed.  By dropping the beacon seeking idea and the land mine avoidance concept Treads regressed to Yard Dog, a territorial predator.  The current robot still incorporates forward looking IR, an edge of the world detector, pyro sensors, a sonar emitter, and a water sensor.

By having a multitude of working sensors, new behaviors could be realized along with some behaviors which did not change from the initial goals.  An IR emitter and two detectors mounted to the front of the robot perform object determination behaviors.  Based on received input the robot decides if it has to maneuver around or go over objects which block forward progress.  A front water sensor helps the robot avoid potentially harmful water obstacles.  An IR pair located on the under chassis of the robot forward of the center of gravity allows the robot to decide if large edges which could destroy the robot are present.  Two pyro sensors provide the distant eyes for Yard Dog.  Mounted forward an aft, the sensors allow for the detection of large heated obstacles.

The elements of this paper detail how these sensors and behaviors are molded together to give Yard Dog its main characteristic of territorial defense and predation.

1. INTRODUCTION

Yard Dog demonstrates the ruggedness of a tracked vehicle.  Designed to work outside, this robot shows that the applications of robotics can be broadened.  Four sensors provide the foundation for the behaviors associated with Yard Dog.  This robot essentially sees, feels, and defends.  The name indicates that its associated behaviors are related to a dog.  In reality, this robot is far less complicated than a dog, but does emulate certain distinct characteristics associated with a dog such as territorial defense.  This project is original and has the potential for considerable advancement.  All programming required to control the behavior algorithms of Yard Dog is simple and adaptable.

2. INTEGRATED SYSTEM

The mobile robot platform consists of the body and motors of a radio controlled tank. The radio controls are removed from the platform so that the unit will function autonomously.  The tank has two DC motors in the rear of the platform that have their own power supply.  A separate motor controller which drives two alternate motors provides drive control for a moving turret and a rotary barrel rocket launcher located on the turret.

3. MOBILE PLATFORM

As mentioned previously, the mobile platform is a radio controlled tank body  approximately one foot long and eight inches high with full working turret.  The platform is very durable and can function outdoors.  Instead of using servos for motors, pre-existing DC motors are the power to the drive train of the robot.  The motors connect to treads that run the length of the robot body and give the robot incredible traction and flexibility.  Since the DC motors require a high power demand, they have four C cell batteries dedicated as their source.

The tank also has a working turret that can turn either direction a full 360 degrees.  Driven by a worm gear and small motor, the turret holds two pyro sensors mounted back to back.  Also attached to the turret at the front is a rotary barrel rocket launcher.  This launcher is a deterrent factor.

ACTUATION

The implementation of this project requires more than two motor drivers.  Two motor drivers work the DC motors that drive the unit.  Another small motor powers the moving turret of the tank. A set of gears connected to a worm gear on a small motor moves the turret through a full field of rotation.

A servo  serves as a rocket launcher mounted to the turret.  This servo rotates rockets to an ignition source that will fire the selected rockets.  This servo along with the motor that drives the turret is controlled by a second motor driver which is controlled by memory mapped I/O. Figure 1 reveals the circuitry.[3]

Figure 1

As mentioned the motors, servo, and turret are the only moving parts on the robot.

SENSORS

IR  This sensor is common to most mobile autonomous units (MAU).  Two sharp GP1U52X IR cans mounted forward and inward so as to give a focal point five inches in front of the robot work together with a forward IR emitter to perform object determination.  Another IR emitter and detector mounted on the underside of the chassis performs edge of the world detection.  The IR emitter is a stock Radio Shack  emitter with forward current handling of 100 mA.  The IR emitter uses a 40 KHz modulation to work with the Sharp cans which are designed to receive a 40 KHZ signal.  The Sharp IR can was designed to produce a digital output.  In order to take advantage of our analog ports, the Sharp can was reconfigured to have a digital output.  To accomplish, a wire was soldered to the input of the capacitor located on the can.  This prevented the signal from being integrated.  Figure 2 shows the wiring diagram.[5][1]

Figure 2

.

Pyro-electric sensors  determine the presence of nearby heat sources, again being incorporated to determine nearby threats.  Mounted to the rear of the turret, two units  give a 110 degree field of view to the front and rear of the robot respectively. These sensors were purchased from a common outdoor motion light kit.  The company that produces this particular sensor is Regent Lighting.  The model for this kit is MS35.  Shown below in figure 3  is the electrical hack for this product.  First this unit was removed from the lighting assembly.  The actual sensor had a five wire strip that was connected to the main wiring circuit.  In order to use this sensor, the five wire strip was severed and the resulting five lines were appropriately connected to the robot.  See figure 3 for details.

Figure 3

A Sonar Emitter/Receiver serves as an animal deterrent in my project. The sensor, Purchased from Electronic Goldmine, is designed to operate a 40 KHZ. The part number is G5721.  In this project this component doesn't really serve as a sensor but more as a tool and is listed here for its hardware value.

The only sensor not purchased for this project is the water sensor.  Simply put this sensor is a voltage divider with open leads at the voltage division point.  Whenever the sensor comes in touch with water, the water forms a short between the two leads closing the electrical loop.  A line is connected at this point to relate an analog voltage value to the microprocessor.  Figure 4 indicates the circuit.

Figure 4

BEHAVIORS

Object determination.  An algorithm that, based on placement of sensors,  determines if an obstacle can be driven over instead of around.

Four behaviors incorporated together in a simple deterministic manner make Yard Dog a fairly "intelligent" machine.  One of the main goals of this project was to use the flexibility and ruggedness of Yard Dog.  Instead of using the normal bump sensor technique, Yard Dog actually makes a decision on whether obstacles can be overcome or have to be driven around.  This comprises the first behavior known as object determination.  This behavior is accomplished by the placement and collumating of sensors.    IR cans are mounted at a height that serves as a height limit for obstacles that the robot can drive over.  A piece of reflective foil prevents the IR emitter, also mounted a fixed distance above the ground, from shining downward.  Therefore all the reflected IR energy will come from objects which are too tall for Yard Dog to climb.

A second behavior that the robot displays is hydrophobia.  Accomplished with a water sensor, this machine actually has to roll into the water before it decides to avoid it.  The water sensor mounted on the front of Yard Dog tells it to back up and change course before exposing all the circuitry to the potentially destructive element.  Very similar to this behavior is the fear of high edges, which will be grouped with hydrophobia.  Both of these behaviors display essentially the same robot characteristics hence the reason for grouping them together.  If the  robot decides a cliff is too high to overcome, it implements the behavior associated with hydrophobia.

Sitting and Watching is the third behavior displayed.  Occasionally the robot will wander to a new location, but for the most part sits in place watching for trespassers.  The pyro sensors help accomplish this goal.  For fixed amounts of time, Yard Dog will look with the pyro sensors in a certain direction before changing its field of view.  If a threat is identified then the robot exhibits the fourth behavior.

Threat and Aggression composes the fourth behavior of exhibit.  First modulated sonar is emitted to warn animals away.  If this doesn't work, the rotary barrel rocket launcher is fired toward the offender.  This behavior is essentially the "bite" of Yard Dog.

7. EXPERIMENTAL LAYOUT AND RESULTS

Combining the above mentioned behaviors produces a robot that acts like a simple yard dog.  As with most projects the final results are different from the initial expectations.  Initially, the goal for the robot was to navigate into hostile territory and search out a homing beacon that emitted sonar.  As it turns out, I was unable to properly implement the sonar receiving circuitry so the scope of the project changed.  Instead most of the components were still implemented with a new goal in mind, territorial protection.

First an order was chosen to realize the completion of the robot.  IR systems would be realized, followed by pyro sensors, the water sensor, and finally the sonar systems.  This is probably where the mistake was made.  As it turns out, the most difficult sensor which was last in the order was not completed.

This robot lacks elaborate schemes which may been seen in other designs.  For instance, with the implementation of IR, there was no need to have a great deal of number crunching.  To be exact, the front IR sensors work on a reading that is set for approximately five inches in front of the robot.  Room for deviation is allowed, but rapid changes in color may prove the short coming of this scheme.  More thought is given to the edge of the world sensor.  It works by calculating differences.  If the difference between two readings has too large of a deviation, the robot reacts.  Again, a certain change of color may confuse the sensor, but the deviation is large enough to compensate for most occurrences while at the same time providing the necessary height detection.  For the actual saturation numbers, I relied on graphs presented by other students from past work and I also performed simple experimentation.[5][6]

For the water sensor calculations were made easy by setting a low threshold.  First the water sensor is simply a voltage divider.  Under ideal conditions a voltage of half the source voltage would be read at the output pin.  However since water acts as a huge resistance, that voltage reading is lowered significantly.  This reading poses no problem since the robot reacts if the analog reading is above a certain level which clearly indicates the presence of water.

The Pyro sensors provide interesting results if understood properly.  For instance, the initial DC offset voltage which is read into an analog port is above zero.  Depending on the direction of the approaching body, the voltage level jumps above or below this level.  When interpreted properly, this sensor actually can give direction.  See figure 5 for experimental results.

Figure 5

CONCLUSION

About 75 percent of the initial scope of the project was realized.  Again, the main loss of the project was the implementation of a sonar receiver array which was initially planned.  Since the sonar receiver array wasn't used, the single chip board became useless.  All actuation and sensor control could now be accomplished with one processor.

It turned out that pyro sensors became very useful.  The only limitation for pyro sensors is that they must be used with objects with high heat sources.  One could not use these sensors to detect other robots working under normal conditions.

Initially, I tried to use a purchased humidity sensor for a water sensor.  I found out quickly that it is best to build this sensor.  Maybe there exists a good water sensor, but I did not find one.  As most people would note, the use of IR sensors is easy and has been well documented for some time.  This part of the project was simple.

By using memory mapped I/O another motor driver was added.  Flexibility was greatly enhanced for Yard Dog with this addition.

Future work.  Using a fuel engine to provide power to the circuitry would be ideal for this platform.  It needs to work for extended periods of time, and the motors that are currently being used drain power quickly.   From testing of the pyro-electric sensor, it is noted that the sensor gives voltage level fluctuating relative to direction.  If one could use these results to direct the robot's intelligence level would increase significantly.  In this project only a simple routine was incorporated with the pyro sensor to determine direction  There may be a lot of potential with this sensor.   Further development of this sensor algorithm  would be highly applicable.

The main problem concerning the sonar receiver was designing an amplifier that could sort the noise from the signal successfully.  If this part of the project was started at the onset, it probably would have been realized.  By adding this component, the robot could be dedicated to searching out for beacons, the original intent.

REFERENCES

The 6.270 Robot Builders Guide, by Fred G. Martin, MIT, 1992.

Robot Programming, by Reid Harrison, 1994.

Novasoft Mekatronix Division Printed Circuit Board Product Line, by Dr. Keith L. Doty and Mekatronix, 1996.

Exponential and Linear Motor Damping using IC, by Erik de la Iglesia, 1996.

Device Specification For IR Detecting Unit for Remote Control, by Z.  Tani, 1989.

Collision Avoidance Dynamics of an Autonomous Agent, by Dr. Keith L. Doty, 1996.

APPENDIX

[1][2][4][6]

The following code represents test segments and the final convolution of the code.  After the tested segments a rough draft of the final code appears in its rawest form it isn't complete.

/*****  Segment for forward IR******/

void main()

poke(0x7000,0xff)

motor(1,50.0);

motor(0,50.0);

while(1){

/*next segment tests for five inch saturation range*/

if (analog(0)>115||analog(1)>115)

{

motor(1,0.0);

motor(0,0.0);

motor(1,-30.0);motor(0,-30.0);

wait(1000);

motor(1,0.0);

wait(300);

motor(0,50.0);

motor(1,50.0);

}

}

/*Pyro testing Algorithm*/

void main()

/* no need to poke since just testing pyro*/

while(1)

{
if (analog(2)>130 || analog(2)<112)

{

/*memory mapped motor control*/

poke(0x6000, 0b11100000);

wait(500);

poke(0x6000, 0b00000000);

/* return head to original center*/

poke(0x6000,0b11001000);

wait(505);

poke(0x6000,0b00000000);

}

}

*/test code for edge of the world detector*/

void main()

motor(1,75.0);

motor(0,75.0);

while(1)

{

x=analog(3);

if(analog(3)-x>5)

{/*this difference formulate is basically a self adjusting routine*/

motor(0,0.0);

motor(1,0.0);

wait(300);

/* just restart motors since this is a test routine*/

motor(1,75.0);

motor(0,75.0);

}

}

/* Water sensor test*/

void main()

/* no need to poke since just reading an analog input*/

while(analog(4)<50)

{

/**** This while statement indicates for the robot to keep running until water is sensed*****/

motor(1,50.0);

motor(0,50.0);

}

motor(0,0.0);

motor(1,0.0);

/**********************Code for Yard Dog****************

***     Josh Lane  *********

***Version 1.0***********/

/********Global Variables******************/

Pyro_F=analog(0);

Pyro_B=analog(1);

FL_IR=analog(2);

FR_IR=analog(3);

Ed_IR=analog(4);

water=analog(5);

void main()

{/*  all integers, floats, and others must be declared.*/

lm=rm=50.0

poke(0x7000,0xff);

motor(0,lm);

motor(1,rm);

for(i=0;i<5;i++)

{wait(100);

xm=lm+10.0

motor(0,xm);

motor(1,xm);

}

/*drive for ten feet*/

/*this set of code places the yard dog a fixed distance from the starting loc*/

while(analog(5)<50)

{

if(analog(0)||analog(1)>130||analog(0)||analog(1)<90)

{motor(0,0.0);

motor(1,0.0);

}

/*find the enemy*/{

/*redirect for obstacle that is too large to overcome*/

if (analog(2)||analog(3)>110)

{

motor(0,0.0);

motor(1,0.0);

/*reverse direction*/

motor(0,-30.0);

motor(1,-30.0);

wait(2000);

motor(1,0.0);

motor(0,0.0);

motor(1,50.0);

wait(300);

motor(0,50.0);

wait(50);

motor(1,75.0);

motor(0,75.0);

wait(50);

motor(1,100.0);

motor(0,100.0);

}

x=analog(4);

wait(50);

if(analog(4)-x>5)

{motor(1,0.0);

motor(0,0.0);

}

}

motor(1,0.0);

motor(0,0.0);

poke(0x7000,0b00000011)

/*move head left 90 degrees first*/

poke(0x6000,0bxxxxxxxx)

wait(500);

for(i=0;i<32200;i++)

{

/* this bit of code just looks for any large heat oriented moving target*/

if(analog(0)>x||analog(0)<y)

{

/*move the tank to the left and move the turret back 90 degrees right*/

}

if(analog(1)>x||analog(1)<y)

{

/*move the turret to the right and move the tank to the right*/

}

}