Not So Evil Bug

Six Legged Walker

Intelligent Machines Design Lab

##### Final Report

##### Spring 1998

Andor Almasi

April 25, 1998

Prof. Antonio Arroyo

Table of Contents

ABSTRACT	3

EXECUTIVE SUMMARY	3

INTRODUCTION	4

HARDWARE DESIGN	5

Platform design	5

Leg design	5

Body design	7

Electronic hardware	8

SOFTWARE DESIGN	9

Overview	9

Sensor Reading	10

Control	12

Walking	12

PWM generation	13

CONCLUSION	14

Accomplishments	14

Failures	14

Future work	14

Thanks	15

APPENDIX A – Mobile Platform Plans	16

APPENDIX B – Software source code	21

Header	21

Sensor Acquisition	22

Controller	26

Movement generator	33

SCI system	36

#### ABSTRACT

The Not So Evil Bug is a six-legged mobile platform designed to walk and climb/descend high obstacles. The bug is controlled by two HC11s, one for servo control, another for everything else. The current behaviors consist of object-avoidance.

#### EXECUTIVE SUMMARY

This robot is based on a six-legged platform designed to allow the ascent/descent of tall objects. I designed the platform in AutoCAD and cut it out of plywood on the T-Tech machine. I modeled the legs after the grasshopper’s hind legs. They use two servos each, and have two degrees of freedom.

The robot is controlled by two Motorola 68HC11 microprocessors. One of them is housed in a MB2325 board. This processor runs in single-chip mode, and generates the PWM signals controlling the servos.  The other HC11 is on the EVBU board, running in expanded mode. This processor runs all the other code, including sensor reading, object avoidance, movement coordination.

I successfully designed and built the platform, and wrote the software that would enable my robot to do everything advertised. Unfortunately, the platform turned out to be way to heavy, and the robot could not support itself while walking. This is a major failure, since the planned behaviors of the robot rely on the specific platform I designed.

#### INTRODUCTION

Most mobile platforms used for robots employ wheels as a mode of propulsion. Although wheels are a very efficient mode of propulsion, they do have their drawbacks. One of these is their inability to traverse extremely rough terrain. Legged platforms on the other hand do not rely on a constant contact point with the surface, and are thus the better platform to use over rough terrain. I chose to push this idea a bit further and attempted to design a robot that could climb up/down an obstacle about one half the robot’s standing height. In order to do this, the robot needed a way to detect an object’s height/depth and enter a crawl mode in which it would execute a specialized two-legged walk, instead of the normal three-legged walk. The planned behaviors for the robot are object avoidance and climbing.

#### HARDWARE DESIGN

The robot hardware includes the body, legs, servos, battery, sensors and microprocessor boards.

### Platform design

The robot is based on a six-legged mobile platform. My inspiration for this platform was Robobug, designed/built/programmed by David Novick and Jennifer Laine. I first saw Robobug at a demo in 4744, and thought that it was the coolest thing since (apple pie?) The ideas for my own robot started to develop in the coming weeks, during some oh-so-dull 4712 classes. By the start of this semester, I had a good idea of what I wanted my platform to look like. I did the actual design in AutoCAD version 14. The entire platform is built from wood, cut on the T-tech machine in the Intelligent Machines Design Laboratory lab. The design sheets for the cutouts can be found in appendix A.

### Leg design

Figure 1 2DOF of the legs

The most important consideration in designing the legs was to make them long enough to allow the robot to climb over fairly high objects. The robot has six identical legs, two facing forward and four facing toward the rear. The legs have two degrees of freedom, one rotational and one lateral (figure 1). The legs move in a plane parallel to the length of the body. The idea for the general shape and function is most closely related to the rear leg of a

Figure 2 Leg design

grasshopper (figure 2). The upper leg (thigh) holds the servo actuating the lower leg. It is attached to the body via a servo horn on the inner surface of the thigh. The lower leg is actuated by a linkage system. This linkage is designed to magnify the servo’s range of motion. The servo connects to the linkage by a length of 1/16th inch diameter piano wire.  The leg pieces are designed to easily fit into each other during assembly, and to provide good structural support once built. A tubular joint made from 1/8th inch thick aluminum connects the upper and lower legs. The large diameter of the tubing ensures that the joint will be strong and smooth rolling. I originally considered using ball bearings in the joint, but after testing one of the above joints, I decided the added weight and complexity were not worth the minute gain in smoothness. I had to redesign the legs after the first cutout, because I made the notches that connect the pieces incorrectly. I also made the attachment points for the linkages at the wrong place. The first version of the legs served as a prototype for the redesign, and helped point out some of the pitfalls that I would likely encounter in latter designs. One of these was that the notches that connected the pieces together only fit if I used a new drill bit in the T-tech. Most of the time the bit wear can be neglected, but in this case the effects of bit wear are actually doubled. This is because if the bit is thinner than expected, the notch comes out thicker and the hole it fits into comes out thinner. This leads to a warm relationship with the Dremel tool, which I often used to correct the above problem. A late addition to the legs was the feet (figure 3). They prevent the legs from slipping and serve as touch sensors. Some very tacky rubber (from a lint-removing roller) is glued on the bottom of the feet to ensure a good contact with the ground. The foot surfaces are currently too small and make the feet unstable.

Figure 3 Foot design

Body design

The purpose of the body is to provide an attachment point for the legs and to hold all the other components (microprocessor boards, batteries). The body design is centered around the servos and the battery pack. I built a cardboard prototype of the body to avoid any clearance problems in the final design.  Extraneous pieces of wood in the body were cut out to save weight. I added notch attachment points throughout the body to allow for further expansion. The EVBU board rests in the middle of the body, attached by four metal posts. The servo control board rests on the rear of the body, attached by four screws and rubber feet.

Both the legs and the body are painted with flat black spray-paint. This serves as a waterproof barrier and a cosmetic enhancement.

### Electronic hardware

Torque49 oz/in @ 6VSpeed60deg in .17 sec @ 6VWeight1.6 ozSize1.6 x 0.8 x 1.4”

Table 1 Servo specifications

The robot uses two microprocessor boards. One is the EVBU board, the other is an MB2325 board. The EVBU board has the 32k memory expansion, two output ports, one input port and a 40kHz signal generator. The EVBU is powered from the MB2325, which has a 5V voltage regulator on it. The MB2325 is directly attached to a 7.2 volt battery pack (6 subC NiCad cells). Leg movement is provided by two servos per leg (one inside the body, one inside thigh). I am using Hitec SuperSport servos purchased from Mayor Hobby (www.majorhobby.com). The servo specifications are listed in table 1. The servos are powered by a 6 volt voltage regulator attached to a heat sink on the bottom of the robot. Two IR emitter-receiver pairs are attached to the front of the robot, as well as sensor switches actuated by whiskers. There are a total of three whiskers, two in a cross configuration pointing forward and one pointing down. The whiskers are made of thin piano wire that bends some on contact. This ensures that nothing breaks when the robot walks into a wall. As an added benefit, the whisker switches activate over a wide range of approach angles.  The switch sensors (whiskers and foot) are broken up into groups of three, each driving a voltage-divider network. This network reduces nine digital inputs to three analog inputs.

#### SOFTWARE DESIGN

### Overview

IR sensor

Whiskers

Foot sensors

Control1

Control2

Walk

Generator

PWM

Generator

Figure 4 Software Structure

All of the software running on the robot is written in assembly. I chose assembly over IC or ICC because it gives me precise timing control crucial in coordinating leg movements. The software can be broken up into the hierarchical structure shown in figure 4.  The software is broken into three parts. The sensor reading process includes reading all of the sensor inputs and converting them to a more usable form. The control process coordinates movements and behaviors based on sensor outputs. The walk generator process generates the appropriate leg movement sequences for the action requested by the control process. The PWM generator uses those sequences to directly control the servos.  All but the PWM generator code is loaded into the SRAM on the EVBU board. The PWM code is running on the MB2325 board, in EPROM. See Appendix B for a complete software listing.

### Sensor Reading

0000Bad reading0001Low reading, nothing ahead0010High reading, possible obstruction0100Saturated1000

Table 2 IR levels and their meanings

Figure 5 IR readings on different surfaces

This process has the job of reading any and all sensor data and preparing it for use by the control process. For the IR sensors it uses a lookup table to distinguish among five different IR level readings. The final output is in the form of a single 8-bit variable. Four bits are dedicated to each IR sensor. The possible IR readings and their meanings are listed in table 2. This method allows calibration of the IR sensors completely transparently to processes using these readings. The only things that need to be changed in case the sensors need recalibrating are the lookup table values within this procedure. In fact, the IR sensors could be replaced with a completely different type of sensor, without any effect on other methods. T he IR readings are often unreliable, since surface color greatly affects the reflectance of IR. Figure 5 shows the difference in reaction between a white and a dark surface.

The whisker sensors compensate for any errors the IR sensors make. The output format for all the switch sensors is binary. In theory, the analog readings coming from the switch voltage divider network should wiggle one specific bit of the digitized reading for a specific switch. Unfortunately, due to mismatch in resistances this does not work out quite well. Therefore, a lookup table is used to simulate the desired effect. The added benefit of this method is transparency. Once again, any of the foot or whisker sensors could be changed to a different type, without having to change the output.

The foot sensors give feedback about leg position for a dynamically generated walking algorithm. They work the same way as the whiskers above.

My special sensor was going to be an IR range finder. These are used in autofocus cameras to measure the distance between the camera and the subject. They work by emitting a highly localized pulsed IR spot (about six inches diameter at five feet) and measuring the offset of the spot on a specialized IR receiver. This receiver’s current output varies according to where on its surface the IR reading is the highest. Lenses focus both the outgoing and incoming IR beams. A decoder chip takes as input the IR emitter frequency and the IR receiver output and converts it to a more usable form. Unfortunately, I have little idea what that form is. I obtained three cameras that had this sensor in them, but could not get any of the manufacturers (Kodak, Canon, and Pentax) to release any useful information. I also tried contacting the decoder manufacturers (Sharp and Hamamatsu Corp) for a datasheet without success. I managed to figure out how the Pentax sensor works. I found that pin 13 on the Sharp IR3S43A decoder emitted a PWM signal whose duty cycle was relative to the distance to the object. The signal varied linearly with distance, duty cycle decreasing until object distance of about 1 foot, increasing for closer and further distances.  I was going to use this sensor as an edge-of-the-world detector that would prevent my robot from walking off a table, and allow it to judge if it can walk off a platform or not. Unfortunately while trying to separate the sensor from the rest of the robot I managed to fry some mystery component and could never get the sensor to work again. I did not have any success finding an output on the other two sensors.

### Control

The control method evaluates the sensor readings and makes a decision on the next action to send to the legs. The decision process is currently in two phases. In the first phase, IR sensor outputs are evaluated and a recommendation is made to the second phase. The second phase checks the more accurate sensors (whiskers, and depth sensor, if it worked) and makes sure that they do not conflict with the phase 1 recommendation. If so, the recommendation is promoted to an action and is passed on to the walk generator. In case of a conflict, phase 2 reevaluates the possible actions based on its sensor inputs and generates an action. The actions generated by the two control phases emerge into a basic object-avoidance behavior.

### Walking

The walk generator takes the action and turns it into a sequence of servo movements. There are two possible walking methods. The first is to use a hardcoded sequence of movements for each action. The second is to dynamically generate movement sequences based on the action and foot switch inputs. The latter allows would allow to self-compensate for uneven terrain, but is much more difficult to implement. I chose to implement the first method.

Actions are commands like walk fwd, turn, climb, etc. An action is made up of sequences. The time delay between two sequences determines the speed of the action. A sequence is 16 servo position values that are fed into the servo controller. Each sequence is sent to the PWM generator five times. A complete action is treated much like a character string. The sequences from the start of the action are sent to the PWM generator until and ENDACT (end-of-action, $FF) character is encountered. Actions are kept as short as possible. This means that a single execution of the walk action, for example, will move the robot forward one step.

Walking is achieved by lifting up three opposing feet, moving them forward and doing the same for the other three feet. Climbing is achieved by moving opposing pairs of feet onto the object to be climbed. I have not found a good turning algorithm for the robot yet. The most likely possible turning method will move one side of the robot less than the other, much like on a tank.

### PWM generation

The MB2325 board is running the servo control code. The input format is $BB$Cx$yy, where x is the servo number, yy is the servo position in the range of 00 to A4. The input is passed through the SCI port. The output is a constant PWM signal on ports  B and C. The servo control code was developed and written by Jenny Laine.

#### CONCLUSION

### Accomplishments

I have built and designed the platform, installed the basic object-avoidance sensors, written the object-avoidance code and interfaced to the servo controller.

### Failures

The mobile platform turned out to be too heavy. As of now, the robot can stand on all six feet, but falls over as soon as three feet are lifted. This is a major design flaw, and has prevented me from developing all the planned movement actions into the robot. As much as I wish otherwise, I discovered that the laws of physics apply to my robot just as much as everyone else’s.

I was not able to get my special sensor working. This is a minor setback, considering that the robot does not walk.

### Future work

The next step in the robot’s development will be to try a five-legged walk. This involves moving one leg at a time, until a full step is taken. Another alternative is to get some 80 oz/in servos. These would be strong enough to allow the robot to stand stable on three legs.

If neither one of these will work, I will be forced to redesign my platform completely.

I am also planning to integrate the sensor I developed for my senior project onto the robot platform. This will eliminate the need for my current special sensor.

### Thanks

I wish to thank Jenny Laine for developing and letting me use the servo controller code I am running on the MB2325 board.

#### APPENDIX A – Mobile Platform Plans

#### APPENDIX B – Software source code

### Header

*< Assembly file header

*  Includes all registers as 8 bit offsets

*  Includes important addresses as 16 bit values

*  Includes single bit masks

*  Place at end of new assembly programs

*>

SRAM    EQU     $2000           ; start of external RAM on EVBU

CRAM    EQU     $0000           ; start of internal RAM

EPROM   EQU     $D000           ; start of eprom

EEPROM  EQU     $B600           ; start of eeprom

BIT7    EQU     %10000000       ; single bit masks

BIT6    EQU     %01000000

BIT5    EQU     %00100000

BIT4    EQU     %00010000

BIT3    EQU     %00001000

BIT2    EQU     %00000100

BIT1    EQU     %00000010

BIT0    EQU     %00000001

INV6    EQU     %10111111       ; inverses

INV5    EQU     %11011111

INV4    EQU     %11101111

INV3    EQU     %11110111

INV2    EQU     %11111011

INV1    EQU     %11111101

INV0    EQU     %11111110

BASE    EQU     $1000           ; register base

ADCTL   EQU     $30             ; A/D Control/Status

ADR1    EQU     $31             ; A/D Result

ADR2    EQU     $32             ; A/D Result

ADR3    EQU     $33             ; A/D Result

ADR4    EQU     $34             ; A/D Result

BAUD    EQU     $2B             ; Baud Rate Control Register

BPROT   EQU     $35             ; Block Protect

CFORC   EQU     $0B             ; Timer Compare Force

CONFIG  EQU     $3F             ; Security disable, COP, ROM Mapping, EEPROM Enables

COPRST  EQU     $3A             ; Arm/Reset COP Timer Circuitry

DDRC    EQU     $07             ; Data Direction Control for Port C

DDRD    EQU     $09             ; Data Direction Control for Port C

EPROG   EQU     $36             ; EPROM Programming Control

HPRIO   EQU     $3C             ; Highest Priority I-Bit Interrupt amd Misc

INIT    EQU     $3D             ; RAM and Register Mapping

OC1D    EQU     $0D             ; Output Compare 1 Data

OC1M    EQU     $0C             ; Output Compare 1 Mask

OPTION  EQU     $39             ; System Configuration Options

PACNT   EQU     $27             ; Pulse Accumulator Control

PACTL   EQU     $26             ; Pulse Accumulator Control

PIOC    EQU     $02             ; Parallel I/O Control

PORTA   EQU     $00             ; Port A Data

PORTB   EQU     $04             ; Port B Data

PORTC   EQU     $03             ; Port C Data

PORTCL  EQU     $05             ; Port C Latched Data

PORTD   EQU     $08             ; Port D Data

PORTE   EQU     $0A             ; Port E Data

PPROG   EQU     $3B             ; EEPROM Programming Control

SCCR1   EQU     $2C             ; SCI Control 1

SCCR2   EQU     $2D             ; SCI Control 2

SCDR    EQU     $2F             ; SCI Data Register

SCSR    EQU     $2E             ; SCI Status Register

SPCR    EQU     $28             ; Serial Peripheral Control

SPDR    EQU     $2A             ; SPI Data

SPSR    EQU     $29             ; SPI Status Register

TCNT    EQU     $0E             ; Timer Count

TCTL1   EQU     $20             ; Timer Control 1

TCTL2   EQU     $21             ; Timer Control 2

TEST1   EQU     $3E             ; Factory Test

TFLG1   EQU     $23             ; Timer Interrupt Flag 1

TFLG2   EQU     $25             ; Timer Interrupt Flag 2

TIC1    EQU     $10             ; Timer Input Capture 1

TIC2    EQU     $12             ; Timer Input Capture 2

TIC3    EQU     $14             ; Timer Input Capture 3

TIC4    EQU     $1E             ; Timer Input Capture 4

TMSK1   EQU     $22             ; Timer Interrupt Mask 1

TMSK2   EQU     $24             ; Timer Interrupt Mask 2

TOC1    EQU     $16             ; Timer Output Compare 1

TOC2    EQU     $18             ; Timer Output Compare 2

TOC3    EQU     $1A             ; Timer Output Compare 3

TOC4    EQU     $1C             ; Timer Output Compare 4

TOC5    EQU     $1E             ; Timer Output Compare 5

### Sensor Acquisition

*< Title        : Robot sensor data ack prog

* Filename      : sensor.asm

* Programmer    : Andor Almasi

* Date          : Mar 18, 1997

* Version       : 1.0

* Description   : Read in foot, feelers, ir

*>

*<

************************************************************************

*               Data Section

************************************************************************

*>

STACK   EQU     $41

ORG     CRAM

*< The following are the treshold values for the IR sensors

* 0000 < BAD < 0001 < LOW <0010 < HI < 0100 <SAT < 1000

*>

BIT76	EQU		%11000000

BIT65	EQU		%01100000

BIT75	EQU		%10100000

BIT765	EQU		%11100000

BIT32	EQU		%00001100

BIT21	EQU		%00000110

BIT31	EQU		%00001010

BIT321	EQU		%00001110

LIR_BAD	EQU		$59		; left reading too low

LIR_LOW EQU		$65		; left reading far

LIR_HI	EQU		$74		; left reading close

LIR_SAT	EQU		$7E		; left reading saturated

RIR_BAD	EQU		$5C		; right reading too low

RIR_LOW EQU		$68		; right reading far

RIR_HI	EQU		$74		; right reading close

RIR_SAT	EQU		$7E		; right reading saturated

IRSENS	RMB		1		; IR sensor data

*< Whisker sensor data structure

*>

L_C_R	EQU	$90		; L_C_R > 90		1110xxxx

L_X_R	EQU	$85		; L_X_R > 85		1010xxxx

X_C_R	EQU	$79		; X_C_R > 79		1100xxxx

X_X_R	EQU	$65		; X_X_R > 65		1000xxxx

L_C_X	EQU	$55		; L_C_X > 55		0110xxxx

L_X_X	EQU	$35		; L_X_X > 35		0010xxxx

X_C_X	EQU	$20		; X_C_X > 20		0100xxxx

WHISKER	RMB		1		; whisker sensor data

*< Foot sensor data structure, same for left & right

*>

R_M_F	EQU	$9C		; R_M_F > 9C		1110

R_M_X	EQU	$95		; R_M_X > 95		1100

R_X_F	EQU	$85		; R_X_F > 85		1010

R_X_X	EQU	$75		; R_X_X > 75		1000

X_M_F	EQU	$5A		; X_M_F > 5A		0110

X_M_X	EQU	$40		; X_M_X > 40		0010

X_X_F	EQU	$20		; X_X_F > 20		0100

FOOT	RMB		1		; FOOT sensor data

AD     	RMB     8       ; A/D register readings (for debugging)

*<

************************************************************************

*               Define Strings for displaying messages

************************************************************************

*>

Mess1   FCB             LF, CR

FCC     ' AD: '

FCB     EOS

Mess0   FCC     ' Here we go '

FCB     EOS

*<

************************************************************************

*                       MAIN PROGRAM

* Calls         : InitSCI, OutStr, InChar, OutChar

************************************************************************

*>

ORG     SRAM

Main    SEI                     ; turn off interrupt system

LDS     #STACK          ; Define a stack

LDX     #BASE

BSET    OPTION,X BIT7   ; turn on A/D system

LDAA    #40             ; wait 100 us (200 E)

LGF4    DECA                    ; for A/D to charge

BNE     LGF4            ;

*< printout code

*>

JSR     InitSCI         ; init serial Communication

LDX     #Mess0

JSR     OutStr

LDX     #CLS            ;

JSR     OutStr          ; clear the screen

LDX     #Mess0

JSR     OutStr

Again   JSR     ReadSens

LDX     #Mess1

JSR     OutStr

LDX     #FOOT

JSR     OutByt

LDX		#WHISKER

JSR     OutByt

LDX		#IRSENS

JSR		OutByt

LDY     #0

Loop    DEY

LDAB    #1

Loop2           DECB

BNE     Loop2

CPY    	#0

BNE     Loop

BRA     Again

SWI                     ; return to buffalo

*<

***************************************************

** Subroutine to read sensors

** Raw A/D data is stored in AD

***************************************************

*>

ReadSens PSHA

PSHX

PSHY

LDX     #BASE

LDY		#AD				; debug

*< These are pins 44-46, IR inputs *

* Here sensor readings are converted into two 4bit

* values stored in IRSENS

*>

LDAA    #%00010100

STAA    ADCTL,X

*<      Total delay is 2+((2+2+3)*9) = 65 E's *>

LDAA    #9             	; 2 E cycles

LOOP1a  NOP                     ; 2 E cycles

DECA                    ; 2 E cycles

BNE     LOOP1a          ; 3 E cycles

CLR		IRSENS			; void old sensor reading

LDAA    ADR1,X			; the left sensor

CMPA	#LIR_SAT		; convert sensor read

BLO 	NEXT1a

BSET	IRSENS BIT7

BRA		ENDLa

NEXT1a	CMPA	#LIR_HI

BLO 	NEXT2a

BSET	IRSENS BIT6

BRA		ENDLa

NEXT2a	CMPA	#LIR_LOW

BLO 	NEXT3a

BSET	IRSENS BIT5

BRA		ENDLa

NEXT3a	CMPA	#LIR_BAD

BLO 	ENDLa

BSET	IRSENS BIT4

ENDLa	STAA	4,Y				; debug

LDAA    ADR2,X			; the right sensor

CMPA	#RIR_SAT		; convert sensor read

BLO 	NEXT4a

BSET	IRSENS BIT3

BRA		ENDRa

NEXT4a	CMPA	#RIR_HI

BLO 	NEXT5a

BSET	IRSENS BIT2

BRA		ENDRa

NEXT5a	CMPA	#RIR_LOW

BLO 	NEXT6a

BSET	IRSENS BIT1

BRA		ENDRa

NEXT6a	CMPA	#RIR_BAD

BLO 	ENDRa

BSET	IRSENS BIT0

ENDRa	STAA	5,Y				; debug

*< These are pins 45-47-49

*>

LDAA    #%00010000

STAA    ADCTL,X

*<      Total delay is 2+((2+2+3)*18) = 128 E's *>

LDAA    #18            	; 2 E cycles

LOOP1a  NOP                     ; 2 E cycles

DECA                    ; 2 E cycles

BNE     LOOP1a          ; 3 E cycles

CLR		FOOT		; void old sensor reading

LDAA	ADR2,X			; pin 45, right feet

CMPA	#R_M_F		; convert sensor read

BLO 	NEXT1c

BSET	FOOT BIT321

BRA		ENDRc

NEXT1c	CMPA	#R_M_X

BLO 	NEXT2c

BSET	FOOT BIT32

BRA		ENDRc

NEXT2c	CMPA	#R_X_F

BLO 	NEXT3c

BSET	FOOT BIT31

BRA		ENDRc

NEXT3c	CMPA	#R_X_X

BLO 	NEXT4c

BSET	FOOT BIT3

BRA		ENDRc

NEXT4c	CMPA	#X_M_F

BLO 	NEXT5c

BSET	FOOT BIT21

BRA		ENDRc

NEXT5c	CMPA	#X_M_X

BLO 	NEXT6c

BSET	FOOT BIT2

BRA		ENDRc

NEXT6c	CMPA	#X_X_F

BLO 	ENDRc

BSET	FOOT BIT1

ENDRc	STAA	1,Y				; debug

LDAA	ADR3,X

CMPA	#R_M_F		; convert sensor read

BLO 	NEXT1d

BSET	FOOT BIT765

BRA		ENDRd

NEXT1d	CMPA	#R_M_X

BLO 	NEXT2d

BSET	FOOT BIT76

BRA		ENDRd

NEXT2d	CMPA	#R_X_F

BLO 	NEXT3d

BSET	FOOT BIT75

BRA		ENDRd

NEXT3d	CMPA	#R_X_X

BLO 	NEXT4d

BSET	FOOT BIT7

BRA		ENDRd

NEXT4d	CMPA	#X_M_F

BLO 	NEXT5d

BSET	FOOT BIT65

BRA		ENDRd

NEXT5d	CMPA	#X_M_X

BLO 	NEXT6d

BSET	FOOT BIT6

BRA		ENDRd

NEXT6d	CMPA	#X_X_F

BLO 	ENDRd

BSET	FOOT BIT5

ENDRd	STAA	2,Y				; debug

LDAA	ADR4,X			; pin 43, whiskers

CLR		WHISKER			; void old sensor reading

CMPA	#L_C_R			; convert sensor read

BLO 	NEXT1b

BSET	WHISKER BIT765

BRA		ENDWb

NEXT1b	CMPA	#L_X_R

BLO 	NEXT2b

BSET	WHISKER BIT75

BRA		ENDWb

NEXT2b	CMPA	#X_C_R

BLO 	NEXT3b

BSET	WHISKER BIT76

BRA		ENDWb

NEXT3b	CMPA	#X_X_R

BLO 	NEXT4b

BSET	WHISKER BIT7

BRA		ENDWb

NEXT4b	CMPA	#L_C_X

BLO 	NEXT5b

BSET	WHISKER BIT65

BRA		ENDWb

NEXT5b	CMPA	#L_X_X

BLO 	NEXT6b

BSET	WHISKER BIT5

BRA		ENDWb

NEXT6b	CMPA	#X_C_X

BLO 	ENDWb

BSET	WHISKER BIT6

ENDWb	STAA	3,Y				; debug

PULY

PULX

PULA

RTS

### Controller

*< Title        : Robot controller prog

* Filename      : control.asm

* Programmer    : Andor Almasi

* Date          : Mar 18, 1997

* Version       : 1.0

* Description   : guide robot based on sensor data

*>

*<

************************************************************************

*               Data Section

************************************************************************

*>

STACK   EQU     $1ff

ORG     SRAM

JMP		Main

*< The following are the treshold values for the IR sensors

* 0000 < BAD < 0001 < LOW <0010 < HI < 0100 <SAT < 1000

*>

BIT76	EQU		%11000000

BIT65	EQU		%01100000

BIT75	EQU		%10100000

BIT765	EQU		%11100000

BIT32	EQU		%00001100

BIT21	EQU		%00000110

BIT31	EQU		%00001010

BIT321	EQU		%00001110

LIR_BAD	EQU		$59		; left reading too low

LIR_LOW EQU		$65		; left reading far

LIR_HI	EQU		$74		; left reading close

LIR_SAT	EQU		$7E		; left reading saturated

RIR_BAD	EQU		$5C		; right reading too low

RIR_LOW EQU		$68		; right reading far

RIR_HI	EQU		$77		; right reading close

RIR_SAT	EQU		$81		; right reading saturated

IRSENS	RMB		1		; IR sensor data

*< Whisker sensor data structure

*>

L_C_R	EQU	$90		; L_C_R > 90		1110xxxx

L_X_R	EQU	$85		; L_X_R > 85		1010xxxx

X_C_R	EQU	$79		; X_C_R > 79		1100xxxx

X_X_R	EQU	$65		; X_X_R > 65		1000xxxx

L_C_X	EQU	$55		; L_C_X > 55		0110xxxx

L_X_X	EQU	$35		; L_X_X > 35		0010xxxx

X_C_X	EQU	$20		; X_C_X > 20		0100xxxx

WHISKER	RMB		1		; whisker sensor data

*< Foot sensor data structure, same for left & right

*>

R_M_F	EQU	$9C		; R_M_F > 9C		1110

R_M_X	EQU	$95		; R_M_X > 95		1100

R_X_F	EQU	$85		; R_X_F > 85		1010

R_X_X	EQU	$75		; R_X_X > 75		1000

X_M_F	EQU	$5A		; X_M_F > 5A		0110

X_M_X	EQU	$40		; X_M_X > 40		0010

X_X_F	EQU	$20		; X_X_F > 20		0100

FOOT	RMB		1		; FOOT sensor data

AD     	RMB     1       ; A/D register readings (for debugging)

AD1		RMB		1

AD2		RMB		1

AD3		RMB		1

AD4		RMB		1

AD5		RMB		1

AD6		RMB		1

AD7		RMB		1

ACTION	RMB		2		; address of action stored here

SPEED	RMB		2		; address of speed of action

FAST	EQU		$8000

SLOW	EQU		$0000

Hello	FCC		'hello.... '

FCB		EOS

STAND	FCC		'< STAND IN PLACE >'

FCB		EOS

FWD		FCC		'< WALK FORWARD >'

FCB		EOS

BWD		FCC		'< WALK BACKWARD >'

FCB		EOS

TURNL	FCC		'< TURNING LEFT >'

FCB		EOS

TURNR	FCC		'< TURNING RIGHT >'

FCB		EOS

SHARPL	FCC		'< SHARP LEFT >'

FCB		EOS

SHARPR	FCC		'< SHARP RIGHT >'

FCB		EOS

CLIMB	FCC		'< CLIMB >'

FCB		EOS

*<

************************************************************************

*                       MAIN PROGRAM

************************************************************************

*>

Main    SEI                     ; turn off interrupt system

LDS     #STACK          ; Define a stack

LDX     #BASE

BSET    OPTION,X BIT7   ; turn on A/D system

LDAA    #40             ; wait 100 us (200 E)

LGF4    DECA                    ; for A/D to charge

BNE     LGF4            ;

*< printout code

*>

JSR     InitSCI         ; init serial Communication

LDX     #CLS            ;

JSR     OutStr          ; clear the screen

LDX     #Hello

JSR     OutStr

Again   JSR     ReadSens

JSR		WhatToDo

BRA     Again

*<

***************************************************

** Subroutine to decide action based upon sensor

** readings

***************************************************

*>

WhatToDo PSHA

PSHB

PSHX

PSHY

LDX		SLOW

STX		SPEED					; default speed is slow

BRCLR	0,X %00001111 STAND_R	; bad reading, stop

BRCLR	0,X %11110000 STAND_R	; bad reading, stop

BRSET	0,X %10001000 BACK_R	; back up

BRSET	0,X %10000000 SHR_R		; sharp right

BRSET	0,X %00001000 SHL_R		; sharp left

BRSET	0,X %01000000 TR_R		; turn right

BRSET	0,X %00000100 TL_R		; turn left

BRSET	0,X %00100000 SFWD_R	; forward slow

BRSET	0,X %00000100 SFWD_R	; forward slow

BRSET	0,X %00010000 FFWD_R	; forward fast

BRSET	0,X %00000001 FFWD_R	; forward fast

STAND_R	LDX		#STAND				; store recommendation

STX		ACTION

BRA		CHK_WH

BACK_R	LDX		#BWD				; store recommendation

STX		ACTION

BRA		CHK_WH

SHR_R	LDX		#SHARPR				; store recommendation

STX		ACTION

BRA		CHK_WH

SHL_R	LDX		#SHARPL				; store recommendation

STX		ACTION

BRA		CHK_WH

TR_R	LDX		#TURNR				; store recommendation

STX		ACTION

BRA		CHK_WH

TL_R	LDX		#TURNL				; store recommendation

STX		ACTION

BRA		CHK_WH

FFWD_R	LDX		#FWD				; store recommendation

STX		ACTION

LDX		FAST				; store speed

STX		SPEED

BRA		CHK_WH

SFWD_R	LDX		#FWD				; store recommendation

STX		ACTION

CHK_WH	LDX		#WHISKER

*< these are the final word in the action that will be taken *>

BRCLR	0,X %11100000 EXEC	; execute IR recommendation

BRSET	0,X %10100000 BACK	; go back

BRSET	0,X	%10000000 SHR	; sharp right

BRSET	0,X %00100000 SHL	; sharp left

BRSET	0,X	%01000000 CLMB	; climb

BACK	LDX		BWD					; store action

STX		ACTION

BRA		EXEC

SHL		LDX		SHARPL				; store action

STX		ACTION

BRA		EXEC

SHR		LDX		SHARPR				; store action

STX		ACTION

BRA		EXEC

CLMB	LDX		CLIMB				; store action

STX		ACTION

EXEC	JSR		Move

PULY

PULX

PULB

PULA

RTS

*<

**********************************************************************

*			leg control subroutine (temporary one)

* input: 	ACTION - start of sequence to execute

*			SPEED - delay between sequence steps

**********************************************************************

*>

Move	PSHA

PSHB

PSHX

PSHY

LDX		ACTION

JSR		OutStr

LDY		SPEED	; delay before going to next seq

Loop    DEY				;

LDAB    #$15	;

Loop2	DECB			;

BNE     Loop2	;

CPY    	#0		;

BNE     Loop	;

PULY			; outta here

PULX

PULB

PULA

RTS

*<

***************************************************

** Subroutine to read sensors

** Raw A/D data is stored in AD

***************************************************

*>

ReadSens PSHA

PSHX

PSHY

LDX     #BASE

LDY		#AD				; debug

*< These are pins 44-46, IR inputs *

* Here sensor readings are converted into two 4bit

* values stored in IRSENS

*>

LDAA    #%00010100

STAA    ADCTL,X

*<      Total delay is 2+((2+2+3)*9) = 65 E's *>

LDAA    #9             	; 2 E cycles

LOOP1a  NOP                     ; 2 E cycles

DECA                    ; 2 E cycles

BNE     LOOP1a          ; 3 E cycles

CLR		IRSENS			; void old sensor reading

LDAA    ADR1,X			; the left sensor

CMPA	#LIR_SAT		; convert sensor read

BLO 	NEXT1a

BSET	IRSENS BIT7

BRA		ENDLa

NEXT1a	CMPA	#LIR_HI

BLO 	NEXT2a

BSET	IRSENS BIT6

BRA		ENDLa

NEXT2a	CMPA	#LIR_LOW

BLO 	NEXT3a

BSET	IRSENS BIT5

BRA		ENDLa

NEXT3a	CMPA	#LIR_BAD

BLO 	ENDLa

BSET	IRSENS BIT4

ENDLa	STAA	4,Y				; debug

LDAA    ADR2,X			; the right sensor

CMPA	#RIR_SAT		; convert sensor read

BLO 	NEXT4a

BSET	IRSENS BIT3

BRA		ENDRa

NEXT4a	CMPA	#RIR_HI

BLO 	NEXT5a

BSET	IRSENS BIT2

BRA		ENDRa

NEXT5a	CMPA	#RIR_LOW

BLO 	NEXT6a

BSET	IRSENS BIT1

BRA		ENDRa

NEXT6a	CMPA	#RIR_BAD

BLO 	ENDRa

BSET	IRSENS BIT0

ENDRa	STAA	5,Y				; debug

*< These are pins 45-47-49

*>

LDAA    #%00010000

STAA    ADCTL,X

*<      Total delay is 2+((2+2+3)*18) = 128 E's *>

LDAA    #18            	; 2 E cycles

LOOP1a  NOP                     ; 2 E cycles

DECA                    ; 2 E cycles

BNE     LOOP1a          ; 3 E cycles

CLR		FOOT		; void old sensor reading

LDAA	ADR2,X			; pin 45, right feet

CMPA	#R_M_F		; convert sensor read

BLO 	NEXT1c

BSET	FOOT BIT321

BRA		ENDRc

NEXT1c	CMPA	#R_M_X

BLO 	NEXT2c

BSET	FOOT BIT32

BRA		ENDRc

NEXT2c	CMPA	#R_X_F

BLO 	NEXT3c

BSET	FOOT BIT31

BRA		ENDRc

NEXT3c	CMPA	#R_X_X

BLO 	NEXT4c

BSET	FOOT BIT3

BRA		ENDRc

NEXT4c	CMPA	#X_M_F

BLO 	NEXT5c

BSET	FOOT BIT21

BRA		ENDRc

NEXT5c	CMPA	#X_M_X

BLO 	NEXT6c

BSET	FOOT BIT2

BRA		ENDRc

NEXT6c	CMPA	#X_X_F

BLO 	ENDRc

BSET	FOOT BIT1

ENDRc	STAA	1,Y				; debug

LDAA	ADR3,X

CMPA	#R_M_F		; convert sensor read

BLO 	NEXT1d

BSET	FOOT BIT765

BRA		ENDRd

NEXT1d	CMPA	#R_M_X

BLO 	NEXT2d

BSET	FOOT BIT76

BRA		ENDRd

NEXT2d	CMPA	#R_X_F

BLO 	NEXT3d

BSET	FOOT BIT75

BRA		ENDRd

NEXT3d	CMPA	#R_X_X

BLO 	NEXT4d

BSET	FOOT BIT7

BRA		ENDRd

NEXT4d	CMPA	#X_M_F

BLO 	NEXT5d

BSET	FOOT BIT65

BRA		ENDRd

NEXT5d	CMPA	#X_M_X

BLO 	NEXT6d

BSET	FOOT BIT6

BRA		ENDRd

NEXT6d	CMPA	#X_X_F

BLO 	ENDRd

BSET	FOOT BIT5

ENDRd	STAA	2,Y				; debug

LDAA	ADR4,X			; pin 43, whiskers

CLR		WHISKER			; void old sensor reading

CMPA	#L_C_R			; convert sensor read

BLO 	NEXT1b

BSET	WHISKER BIT765

BRA		ENDWb

NEXT1b	CMPA	#L_X_R

BLO 	NEXT2b

BSET	WHISKER BIT75

BRA		ENDWb

NEXT2b	CMPA	#X_C_R

BLO 	NEXT3b

BSET	WHISKER BIT76

BRA		ENDWb

NEXT3b	CMPA	#X_X_R

BLO 	NEXT4b

BSET	WHISKER BIT7

BRA		ENDWb

NEXT4b	CMPA	#L_C_X

BLO 	NEXT5b

BSET	WHISKER BIT65

BRA		ENDWb

NEXT5b	CMPA	#L_X_X

BLO 	NEXT6b

BSET	WHISKER BIT5

BRA		ENDWb

NEXT6b	CMPA	#X_C_X

BLO 	ENDWb

BSET	WHISKER BIT6

ENDWb	STAA	3,Y				; debug

PULY

PULX

PULA

RTS

### Movement generator

*< Title        : Robot movement coordination

* Filename      : move.asm

* Programmer    : Andor Almasi

* Date          : Apr 22, 1997

* Version       : 1.0

* Description   :

*>

*<

************************************************************************

*               Data Section

************************************************************************

*>

STACK   EQU     $1ff

ORG     SRAM

JMP 	Main

*<

************************************************************************

*               Define Strings for displaying messages

************************************************************************

*>

TEMP1	RMB		2

Spd		FCC		' Enter servo speed: '

FCB		EOS

SERVO	RMB		1		; current servo count

SPEED	RMB		2		; time between movements

ACTION	RMB		2		; which action to perform

ENDACT	EQU		$FF		; end action delimiter

ROT_ADJ	EQU		$5A		; servo rotation adjustment factor

*< posi+tion tables, hold sequences of positions for 16 servos

* a set of sequences form an action

* body part notation [LEFT/RIGHT][FRONT/CENTER/BACK][UPPER/LOWER]

*

* servo number  c0  c1  c2  c3  c4  c5  c6  c7  c8  c9  ca  cb  cc  cd  ce  cf

*<inverse:		NO  NO  NO  NO  NO  NO                  YES YES YES YES YES YES *>

*<body part:	LFL LFU RCL RCU RRL RRU N/C N/C N/C N/C RFL RFU LCL LCU LRL LRU *>

STAND	FCB		$30,$35,$40,$30,$30,$35,$00,$00,$00,$00,$30,$35,$40,$30,$30,$35

FCB		ENDACT

*<body part:	LFL LFU RCL RCU RRL RRU N/C N/C N/C N/C RFL RFU LCL LCU LRL LRU *>

FWD		FCB		$40,$50,$50,$20,$30,$35,$00,$00,$00,$00,$30,$35,$40,$30,$50,$20

FCB		$40,$50,$50,$20,$20,$25,$00,$00,$00,$00,$40,$20,$30,$30,$50,$20

FCB		$50,$40,$40,$40,$20,$25,$00,$00,$00,$00,$40,$20,$30,$30,$20,$40

FCB		ENDACT

*<body part:	LFL LFU RCL RCU RRL RRU N/C N/C N/C N/C RFL RFU LCL LCU LRL LRU *>

BWD		FCC		'< WALK BACKWARD >'

FCB		ENDACT

*<body part:	LFL LFU RCL RCU RRL RRU N/C N/C N/C N/C RFL RFU LCL LCU LRL LRU *>

TURNL	FCC		'< TURNING LEFT >'

FCB		ENDACT

*<body part:	LFL LFU RCL RCU RRL RRU N/C N/C N/C N/C RFL RFU LCL LCU LRL LRU *>

TURNR	FCC		'< TURNING RIGHT >'

FCB		ENDACT

*<body part:	LFL LFU RCL RCU RRL RRU N/C N/C N/C N/C RFL RFU LCL LCU LRL LRU *>

SHARPL	FCC		'< SHARP LEFT >'

FCB		ENDACT

*<body part:	LFL LFU RCL RCU RRL RRU N/C N/C N/C N/C RFL RFU LCL LCU LRL LRU *>

SHARPR	FCC		'< SHARP RIGHT >'

FCB		ENDACT

*<body part:	LFL LFU RCL RCU RRL RRU N/C N/C N/C N/C RFL RFU LCL LCU LRL LRU *>

CLIMB	FCC		'< CLIMB >'

FCB		ENDACT

*<

************************************************************************

*                       MAIN PROGRAM

************************************************************************

*>

Main    SEI                     ; turn off interrupt system

LDS     #STACK          ; Define a stack

LDX     #BASE

LDAA	#$2000

STAA	$b900	; notify user to switch

*< printout code

*>

JSR     InitSCI         ; init serial Communication

*		LDX		#Spd

*		JSR		OutStr

*		JSR		InByt

*		JSR		InByt

*		LDD		TEMP1

*		STD		SPEED

*		LDX		#TEMP1

*		JSR		OutByt

*		JSR		OutByt

LDAA	#0

STAA	$b900	; notify user to switch

LDY		#0		; delay gives time to switch

Loope   DEY				;

LDAB    #$15	;

Loop2e	DECB			;

BNE     Loop2e	;

CPY    	#0		;

BNE     Loope	;

LDX		#$3000

STX		SPEED

LDAA	#5

Again	LDX		#STAND

STX		ACTION

JSR     Move

DECA

BNE		Again

KeepOn	LDX		#FWD

STX		ACTION

JSR     Move

BRA     KeepOn

*<

**********************************************************************

*			leg control subroutine

* input: 	ACTION - start of sequence to execute

*			SPEED - delay between sequence steps

**********************************************************************

*>

Move	PSHA

PSHB

PSHX

PSHY

LDX		ACTION

NEXT_SQ	LDAA	#$c0

STAA	SERVO	; current servo

NEXT_SV	LDAA	SERVO	;

CMPA	#$d0	; is it beyond last servo

BEQ		END_S	;

LDAB	#5		; send same thing five times

SendOvr	LDAA	#$BB	; header

JSR		OutChar ; header sent

LDAA	SERVO	; servo number

JSR		OutChar ; servo num sent

LDAA	0,X		; servo position

CMPA	#ENDACT	; is it end of sequence?

BEQ		END_A	; outta here

LDY		#SERVO

BRSET	0,Y BIT3 NoAdj 	; rotation adjustment check

LDAA	#ROT_ADJ		; load adjustment factor

SUBA	0,X		; adjusted position

NoAdj	JSR		OutChar	; position sent

DECB			;

BNE		SendOvr	;

INC		SERVO	; next servo

INX				; next position

BRA		NEXT_SV	; do same for next servo

END_S	LDY		SPEED	; delay before going to next seq

Loop    DEY				;

LDAB    #$15	;

Loop2	DECB			;

BNE     Loop2	;

CPY    	#0		;

BNE     Loop	;

BRA		NEXT_SQ	; do next sequence

END_A	PULY			; outta here

PULX

PULB

PULA

RTS

### SCI system

*< Title        : SCI system functions

* Filename      : sci.asm

* Programmer    : Andor Almasi

* Date          : Feb 15, 1997

* Version       : 1.0

* Description   : InitSCI, OutChar, OutStr, InChar, OutXY, MakeBCD

*>

*<***********************************************************************

*       Common definitions, assumes that header.asm is already included

************************************************************************>

*<OutChar, OutStr, OutXY*>

CLS     FCB     ESC,$5B,$32,$4A         ; ANSI sequence to clear screen

FCB     ESC,$5B,$3B,$48         ; and move cursor to home

FCB     EOS                     ; EOS character

*<OutChar, OutStr, OutXY, InChar*>

EOS     EQU     $04             ; User-defined End Of String (EOS) character

CR      EQU     $0D             ; Carriage Return Character

LF      EQU     $0A             ; Line Feed Character

ESC     EQU     $1B             ; Escape Character

SP      EQU     $20             ; Space Character

*<***********************************************************************

*                       SUBROUTINE - InitSCI

* Description: This subroutine initializes the BAUD rate to 9600 and

*          sets up the SCI port for 1 start bit, 8 data bits and

*          1 stop bit.  It also enables the transmitter and receiver.

*          Effected registers are BAUD, SCCR1, and SCCR2.

* Input         : None.

* Output        : Initializes SCI.

* Destroys      : None.

* Calls         : None.

************************************************************************

*<Baud rate defs:       $30     9600

*                       $31     4800

*                       $32     2400

*                       $33     1200

*                       $34     0600

*                       $35     0300

*                       $36     0150

*                       $37     0075 *>

RATE    EQU     $30

InitSCI PSHA                ; Save contents of A register

LDY     #BASE

LDAA    #RATE       ; Set BAUD rate

STAA    BAUD,Y

CLR     SCCR1,Y     ; Set SCI Mode to 1/8/1

LDAA    #$0C        ; Enable SCI Transmitter

STAA    SCCR2,Y     ;     and Receiver

PULA                            ;Restore A register

RTS                     ; Return from subtoutine

*<

************************************************************************

*                       SUBROUTINE -  OutChar

* Description: Outputs the character in register A to the screen after

*              checking if the Transmitter Data Register is Empty.

* Input         : Data to be transmitted in register A.

* Output        : Transmit the data.

* Destroys      : None.

* Calls         : None.

************************************************************************

*>

OutChar PSHB                    ; Save contents of B register

LDY     #BASE

Loop1   LDAB    SCSR,Y          ; Check status reg (load it into B reg)

ANDB    #$80            ; Check if transmit buffer is empty

BEQ     Loop1           ; Wait until empty

STAA    SCDR,Y          ; Register A ==> SCI data

PULB                    ; Restore B register

RTS                     ; Return from subtoutine

*<

************************************************************************

*                       SUBROUTINE -  OutStr

* Description: Outputs the string terminated by EOS. The starting

*          location of the string is pointed by X register. Calls

*          the OutChar subroutine to display a character on the screen

*          and exit once EOS has been reached.  In order to print the

*          string properly with RTI, it automatically disables and

*          enables interrupts.

* Input         : Starting location of the string to be transmitted

*               : (passed in X register)

* Output        : Prints the string.

* Destroys      : Contents of X register.

* Calls         : OutChar.

************************************************************************

*>

OutStr  PSHA                    ; Save contents of A register

LDY     #BASE

SEI                     ; Disable interrupts

Loop2   LDAA    0,X             ; Get a character (put in A register)

CMPA    #EOS            ; Check if it's EOS

BEQ     Done            ; Branch to Done if it's EOS

JSR     OutChar         ; Print the character by calling OutChar

INX                             ; Increment index

BRA     Loop2           ; Branch to Loop2 for the next char.

Done    CLI                     ; Enable interrupts

PULA                    ; Restore A register

RTS                     ; Return from subtoutine

*<

************************************************************************

*                SUBROUTINE  -  InChar

* Description: Receives the typed character into register A.

* Input         : None

* Output        : Register A = input from SCI

* Destroys      : Contents of Register A

* Calls         : None.

************************************************************************

*>

InChar  LDX             #BASE

LDAA    SCSR,X    ; Check status reg.

ANDA    #$20      ; Check if receive buffer full

BEQ     InChar    ; Wait until data present

LDAA    SCDR,X    ; SCI data ==> A register

RTS              ; Return from subroutine

*<***************************************************************************

* OutByt - convert the byte at X to two

* ASCII characters and output. Return X pointing

* to next byte.

* This is from the buffalo source code

*****************************************************************************

*>

OutByt  PSHA

LDAA 0,X                        ;get data in a

PSHA                            ;save copy

BSR  OUTLHLF            ;output left half

PULA                            ;retrieve copy

BSR  OUTRHLF            ;output right half

PULA

INX

RTS

OUTLHLF LSRA                            ;shift data to right

LSRA

LSRA

LSRA

OUTRHLF ANDA #$0F                       ;mask top half

ADDA #$30                       ;convert to ascii

CMPA #$39

BLE  OUTA                       ;jump if 0-9

ADDA #$07                       ;convert to hex A-F

OUTA    JSR  OutChar            ;output character

RTS

*<

**********************************************************************

* InByt - reads two ascii numbers and converts them to hex,

* returns them in TEMP + 1, shifting TEMP+1 to TEMP

* Uses buffalo function HEXBIN (modified)

*

**********************************************************************

*>

InByt	JSR		InChar

JSR		HEXBIN

JSR		InChar

JSR 	HEXBIN

RTS

*<

*****************

*   HEXBIN(a) - Convert the ASCII character in a

* to binary and shift into TEMP1.  Assumes correct hex input

*****************

*>

HEXBIN	PSHA

PSHB

PSHX

JSR  	UPCASE		; convert to upper case

CMPA 	#'0'

BLT  	HEXRTS		; jump if a < $30

CMPA 	#'9'

BLE  	HEXNMB		; jump if 0-9

CMPA 	#'A'

BLT  	HEXRTS		; jump if $39> a <$41

CMPA 	#'F'

BGT  	HEXRTS		; jump if a > $46

ADDA 	#$9			; convert $A-$F

HEXNMB	ANDA 	#$0F		; convert to binary

LDX  	#TEMP1

LDAB 	#4

HEXSHFT ASL  	1,X			; 2 byte shift through

ROL  	0,X	       	; carry bit

DECB

BGT 	HEXSHFT		; shift 4 times

ORAA 	1,X

STAA 	1,X

HEXRTS	PULX

PULB

PULA

RTS

*<

*****************

*  UPCASE(a) - If the contents of A is alpha,

* returns a converted to uppercase.

*****************

*>

UPCASE	 CMPA #'a'

BLT  UPCASE1	   jump if < a

CMPA #'z'

BGT  UPCASE1	   jump if > z

SUBA #$20	   convert

UPCASE1  RTS