/* initializing globals */

int left_eye, right_eye, left_edge, right_edge, left_motor, right_motor;

float forward = 100.0;

float reverse = -100.0;

float stop = 0.0;

int delay = 100;

float ir_motor_l, ir_motor_r, edge_motor_l, edge_motor_r, bump_motor_l;

float bump_motor_r, tract_motor_l, tract_motor_r, comp_motor_l, comp_motor_r;

int traction_old, traction_new, motor_flip;

int obstacle_detected, edge_detected, bumped, no_traction, flip, offcourse;

void wait(int milli_seconds)

{ long timer_a;

timer_a = mseconds() + (long) milli_seconds;

while(timer_a >mseconds()) {

defer();

}

}

void sensor_read ()

poke(0xffb9,0xff);

{ while (1)

{

traction_old = traction_new;

traction_new = analog(9);

bump = analog(8);

flip = analog(7);

if (flip < 100) /* tells where ir sensors are looking when righted */

{left_eye = analog(3);

right_eye = analog(4);

left_edge = analog(5);

right_edge = analog(6);

motor_flip = 1;

left_motor = 0;

right_motor = 1;

}

else

{left_eye = analog(5); /* tells where ir is looking when upside down */

right_eye = analog(6);

left_edge = analog(4);

right_edge = analog(3);

motor_flip = -1; /* reverses motor direction when upside down */

left_motor = 1; /* tells which motor is on the left when upside down */

right_motor = 0;

}

}

}

void edge_detection ()

/* if edge is detected, back up then turn right */

{ while (1)

{

if left_edge <115 || right_edge <115)

{ edge_detected = 1;

edge_motor_l = reverse;

edge_motor_r = reverse;

wait(500);

edge_motor_l = stop;

edge_motor_r = forward;

wait(250);

}

else

{ edge_detected = 0;

}

}

}

/* traction monitor */

void traction_monitor()

{

int i = 0;

while (1)

{

if (traction_new == traction_old)

{ i = i + 1;

}

else

{ i = 0;

}

if (i > 2000) /* if traction is lost, back up and turn right */

{ no_traction = 1;

tract_motor_l = reverse;

tract_motor_r = reverse;

wait(500);

tract_motor_l = forward;

tract_motor_r = stop;

wait(250);

}

else

{ no_traction = 0;

}

}

}

/*  object avoidance  behavior */

void object_avoidance()

{ while (1)

{

if (right_eye >99 || left_eye >90 )

{obstacle_detected = 1;

if (right_eye > left_eye)

{ir_motor_r = forward;

ir_motor_l = stop ;

wait(250);

}

else if ( left_eye > right_eye )

{  ir_motor_r = stop ;

ir_motor_l = forward;

wait(250);

}

else

/* if object is perpendicular to path, back up then turn right */

{  ir_motor_l = reverse;

ir_motor_r = reverse;

wait(250);

ir_motor_l = forward;

ir_motor_r = stop;

wait(250);

}

}

else

{ obstacle_detected = 0;

}

}

}

/* heading detection */

void compass_module ()

{

}

/*  bump  sense behavior */

void bump_behavior() /* if bumped, back up and turn right */

{ while(1)

{ if (bump < 190)

{ bumped = 1;

bump_motor_l = reverse;

bump_motor_r = reverse;

wait(150);

bump_motor_l = forward;

bump_motor_r = stop;

wait(150);

}

else

{ bumped = 0;

}

}

}

/*  this arbitrates between object avoidance, bump behaviors */

void behavior_arbitrate()

{

float mpowerold_l = 0.0;

float mpowerold_r = 0.0;

float mpowernew_l, mpowernew_r, mpowerx_l, mpowerx_r;

while(1)

{

if (edge_detected)

{  mpowernew_l = edge_motor_l;

mpowernew_r = edge_motor_r;

}

else if (bumped)

{ mpowernew_l = bump_motor_l;

mpowernew_r = bump_motor_r;

}

else if (no_traction)

{ mpowernew_l = tract_motor_l;

mpowernew_r = tract_motor_r;

}

else if (obstacle_detected)

{ mpowernew_l = ir_motor_l;

mpowernew_r = ir_motor_r;

}

else if (offcourse)

{ mpowernew_l = comp_motor_l;

mpowernew_r = comp_motor_r;

}

else

{ mpowernew_l = forward;

mpowernew_r = forward;

}

mpowerx_l = (mpowernew_l + 19.0 * mpowerold_l)/20.0;

mpowerold_l = mpowerx_l;

mpowerx_r = (mpowernew_r + 19.0 * mpowerold_r)/20.0;

mpowerold_r = mpowerx_r;

motor(left_motor, mpowerx_l * flip_motor);

motor(right_motor, mpowerx_r * flip_motor);

}

}

void main()

{

start_process(sensor_read());

start_process(edge_avoidance());

start_process(traction_monitor());

start_process(obstacle_avoidance());

start_process(compass_module());

start_process(behavior_arbitrate());

}