Date:             8/ 9 / 00

Student Name:         Jeff Davis

Robot Name:  Rusty Wonder

TA:        Scott Jantz

Scott Kanowitz

Instructors:     A. A. Arroyo

E.M. Schwartz

Intelligent Machines Design Laboratory

# University Of Florida

Department of Electrical and Computer Engineering

EEL5666

Final Report

Table of Contents:

3. Abstract

4. Introduction

5. Executive Summary

6. Integrated System

7. Mobile Platform

8. Actuation, Sensors

9. Behaviors

10. Experimental Layout, Conclusion, Documentation

11. Appendix A (Program Code)

18. Voice Record/Playback Circuit

Abstract

My original goal in IMDL was to design an autonomous agent that would act as a mobile thermostat.  The robot would roam a house sensing the temperature, and when a given threshold temperature is sensed the robot would either turn on or turn off the heating or cooling unit.

I changed the scope of my project from actual climate control to a talking temperature probe that demonstrates a robotic system which is able to react to changes in temperature and tell you if he is hot, cold, or comfortable.

Introduction

Although I did not end with a robot that actually manipulates the climate in a house or a room, my robot could sense the ambient temperature, avoid objects, get bored or tired, and react by playing prerecorded messages though a speaker connected to a voice record/playback IC bought form Radio Shack.  In this report I will discuss the integrated system, mobile platform, actuation, sensors, behaviors, and experimental layout.

#### Executive Summary

#### Although I had originally intended to control the climate with my robot, I was unable to achieve this due to financial and time constraints.  However, I was able to illustrate that my goal could have been accomplished by designing a robot that talked and said if he was hot, cold, or comfortable.

#### By changing the scope of my project, I was able to be more creative with the behavior and body of my robot.  In addition to reacting to temperature, Rusty can sing and tell you when he sees an object, bumps a wall, or decides to take a break.

#### Deciding on a personality and vocabulary was the final step in the creation of my robot.  I had several ideas, but was limited by the 20-second capacity of the voice chip.

#### I would eventually like to have a few different personalities developed that could be quickly loaded into the robot’s microprocessor and voice chip.  Two or more of these robots could then communicate using infrared emission and detection making it possible to have a simple conversation among multiple robots using the voice chip.

#### Integrated System

The TJPRO was used to implement the design.  The TJPRO is a trademark of MEKATRONIX.  The TJPRO was a good choice for the basic design since it is small, easy to navigate, and expand upon.

Attached to the body by a short spring is a plastic bowl with a lid that houses the audio system.  Additional circuitry was required for the voice record/playback to interface with the TJPRO.

The voice chip is connected to the data bus of the microcontroller through a latch.  The data sent to the voice chip is a pointer to the beginning of a given message.  By sending predefined constant pointers as parameters to the “say” function the TJPRO can now talk.

Mobile Platform

The TJPRO platform was used and slightly modified in order to house the voice record/playback system.  The design is suitable for roaming and obstacle avoidance, therefore making it a sufficient medium for the application.

Keith L. Doty designed the platform.  Detailed plans are given in the Talrik Junior Assembly Manual at http://www.mekatronix.com/manuals/tj/tjam.pdf.  Figure 1 shows a profile of the TJ platform.

Figure 1

Actuation

Two hacked servos were used for the robot’s own transportation.  Because the load that the robot must support was very little, high torque was not required. The servos were a little different than the ones described in the assembly manual.  The potentiometers inside the servos had to be taken out because they could not be fixed into position.

### Sensors

Infrared and bump sensors were used for navigation and obstacle avoidance.  A 10 K thermistor  (Radio Shack part number 217-110A)

was used to sense the ambient temperature.  Figure 2 shows the thermistor circuit used.

Figure 2

Behaviors

When Rusty is reset, the sensors are calibrated.  Then he introduces himself and begins basic obstacle avoidance while sensing the temperature.  The temperature at reset is what Rusty considers comfortable.  If it gets one or two degrees hotter or colder, he will say he is hot or cold.  When he becomes comfortable again he will tell you.  He also says, “Must avoid objects” when the infrared system detects an object and “Ouch!” when he bumps a wall.  When he has bumped a wall eight times, he gets tells you he’s shutting down and sleeps for 15 seconds.  He then wakes up singing Mr. Roboto by Styx and repeats the entire process.

Experimental Layout

Testing was done in steps as each piece of the project was completed.  Once the TJPRO was built and functioning the thermistor was added and tested.  The voice record/playback circuitry was then assembled and tested.  Finally, the entire system was integrated and tested as a whole.

#### Conclusion

There were a few slight difficulties in the realization of this project.  First, the servos were not of the kind described in the TJPRO assembly manual, so a lot of time was wasted trying to set the potentiometers in the beginning.  Finally, I removed them from the servo housing completely.  Then, I had trouble with one of my IR detectors, so that slowed progress for about a week.  Eventually these problems were worked out and the voice system was added.  Initially, I was having trouble getting the chip to start and stop playing at different points within the message space, but I was able to solve the problem with software.  By connecting the voice IC’s chip enable pin to the least significant bit of the data bus, I could enable and disable the chip by clearing and setting this bit.

Documentation

Talrik Junior Assembly Manual:  Keith L. Doty Copyright 1999 Mekatronix

Program Code

/**********************************************************************

* Title        rus1beta.c

* Programmer   Jeff Davis

* Date         July, 2000

* Version      1

*

* Description

*	Code for Rusty Wonder the talking temperature probe..

**********************************************************************/

/*************************** Includes ********************************/

#include <tjpbase.h>

#include <jeff.h>

#include <stdio.h>

/************************ End of Includes ****************************/

/*************************** Constants *******************************/

#define 	HELLO	18*8

#define 	OUCH	12*8

#define 	OBJECT	13*8

#define 	C3P0	5*8

#define 	BENNY	4

#define 	HOT	8*8

#define	COLD	9*8

#define	COZY	10*8

#define 	ROBOT	15*8

/************************ End of Constatnts **************************/

/********************** Function Prototypes **************************/

void calibrate_infrared(void);

void calibrate_thermistor(void);

void intro(void);

void avoid(void);

void climate(void);

void bored(void);

void tired(void);

void turn(void);

void say(int MSP);

/******************* End of Function Prototypes **********************/

/************************** Global Variables *************************/

int AVOID_THRESHOLD_L, AVOID_THRESHOLD_R;

int comfy, coolFlag=0, warmFlag=0, niceFlag=1;

int avoid_count=0;

char clear[]= "\x1b\x5B\x32\x4A\x04";

char place[]= "\x1b[1;1H";

/*********************** End of Global Variables *********************/

void main(void)

/****************************** Main *********************************/

{

init_analog();

init_motortjp();

init_clocktjp();

VOICE_OFF;

calibrate_infrared();

calibrate_thermistor();

intro();

wait(1000);

while(1)

{

avoid();

climate();

}

}

/**************************** End of Main ******************************/

void calibrate_infrared()

/************************* Calibrate Infrared **************************/

{

int i, avgl, avgr, suml = 0, sumr = 0;

for(i=0; i<100; i++)

{

suml = suml + LEFT_IR;

sumr = sumr + RIGHT_IR;

wait(50);

}

AVOID_THRESHOLD_L = suml/100;

AVOID_THRESHOLD_R = sumr/100;

printf("%s", clear);

printf("%s", place);

printf("\nLeft Threshold: %d", AVOID_THRESHOLD_L);

printf("\nRight Threshold: %d\n", AVOID_THRESHOLD_R);

}

/************************** End of Calibrate IR ***************************/

void calibrate_thermistor()

/************************** Calibrate Thermistor **************************/

{

int i, sum = 0;

for (i=0; i<100; i++){

sum = sum + THERMISTOR;

wait(50);

}

comfy = sum/100;

printf("\nComfortable: %d\n", comfy);

}

/*********************** End of Calibrate Thermistor***********************/

void intro()

/******************************* Intro ***********************************/

{

say(HELLO);

}

/*************************** End of Intro ********************************/

void tired()

/******************************* Tired ***********************************/

{

motorp(RIGHT_MOTOR, 0);

motorp(LEFT_MOTOR, 0);

wait(1000);

say(C3P0);

wait(15000);

bored();

}

/*************************** End of Tired ********************************/

void bored()

/******************************* Bored ***********************************/

{

say(ROBOT);

wait(2000);

avoid();

}

/*************************** End of Bored ********************************/

void avoid()

/******************************* Avoid ***********************************/

{

int irdr, irdl, speedr, speedl;

irdr = RIGHT_IR;

irdl = LEFT_IR;

if (irdl > AVOID_THRESHOLD_L){

say(OBJECT);

speedr = -MAX_SPEED;

}

else

speedr = MAX_SPEED;

if (irdr > AVOID_THRESHOLD_R){

say(OBJECT);

speedl = -MAX_SPEED;

}

else

speedl = MAX_SPEED;

motorp(RIGHT_MOTOR, speedr);

motorp(LEFT_MOTOR, speedl);

if(FRONT_BUMP)

{

say(OUCH);

motorp(LEFT_MOTOR, -MAX_SPEED);

motorp(RIGHT_MOTOR, -MAX_SPEED);

wait(600);

avoid_count++;

turn();

}

wait(35);

if (avoid_count > 7) {

avoid_count = 0;

tired();

}

}

/*************************** End of Avoid ********************************/

void climate()

/****************************** Climate **********************************/

{

int i, avg, sum;

sum = 0;		// Reinitiailze sum

for(i = 0; i<10; i++){

sum = sum + THERMISTOR;

}

avg = sum/10;

if(NICE && (warmFlag || coolFlag)) {

say(COZY);

printf("\n\nBREAKPOINT1\n");

nicceFlag = 1;

warmFlag = 0;

coolFlag = 0;

wait(2000);

return;

}

if(WARM && (niceFlag || coolFlag)) {

say(HOT);

printf("\n\nBREAKPOINT2\n");

niceFlag = 0;

warmFlag = 1;

coolFlag = 0;

wait(2000);

return;

}

if(COOL && (warmFlag || niceFlag)) {

say(COLD);

printf("\n\nBREAKPOINT3\n");

niceFlag = 0;

warmFlag = 0;

coolFlag = 1;

wait(2000);

return;

}

}

/*************************** End of Climate ******************************/

void say(MSP)

/******************************* Say ***********************************/

{

SET_BIT(MSP, 0x01)

VOICE = MSP;

VOICE_ON;

wait(5);

VOICE_OFF;

}

/**************************** End of say ********************************/

void turn(void)

/************************************************************************

* Function:  Will turn in a random direction for a "random" amount of

*            time, dictated by the fast changine lower bits in

*	      mseconds().

* Returns:  None

*

* Inputs

*   Parameters: None

*   Globals:    None

*   Registers:  TCNT

* Outputs

*   Parameters: None

*   Globals:    None

*   Registers:  None

* Functions called: motorp(), wait()

* Notes:

************************************************************************/

{

int i;

unsigned rand;

rand = TCNT;

if (rand & 0x0001)

/*turn left*/

{

motorp(RIGHT_MOTOR, MAX_SPEED);

motorp(LEFT_MOTOR, -MAX_SPEED);

}

else

/*turn right*/

{

motorp(RIGHT_MOTOR, -MAX_SPEED);

motorp(LEFT_MOTOR, MAX_SPEED);

}

i=(rand % 1024);

if(i>250) wait(i); else wait(250);

}

/***********************End Function turn ****************************/

Header File

#define THERMISTOR	   analog(7)

#define COOL	THERMISTOR > comfy + 2

#define WARM	THERMISTOR < comfy

#define NICE	THERMISTOR <= comfy + 2 && THERMISTOR >= comfy

#define ON	1

#define OFF	0

#define VOICE	*(unsigned char volatile *)(0x6000)

/* Enable  */

#define VOICE_ON     CLEAR_BIT(VOICE, 0x01)

/*Disable */

#define VOICE_OFF    SET_BIT(VOICE, 0x01)

Voice Record/Playback Circuit

Voice Chip is ISD1000A available at  Radio Shack.