MindStamp™

USERS MANUAL

MTJRPO11 Version

by

Keith L. Doty

Copyright © 1999 Mekatronix™.

Version 01

AGREEMENT

This is a legal agreement between you, the end user, and Mekatronix™. If you do not agree to the terms of this Agreement, please promptly return the purchased product for a full refund.

- Copyright Notice. Mekatronix™ hereby grants to any individuals or organizations permission to reproduce and distribute copies of this document, in whole or in part, for any personal or non-commercial educational use only. This copyright notice must accompany any copy that is distributed.

- Copy Restrictions. Other than cases mentioned in Copyright Notice, no part of any Mekatronix™ document may be reproduced in any form without written permission of Mekatronix™. For example, Mekatronix™ does not grant the right to make derivative works based on these documents without written consent.

- Software License. Mekatronix™ software is licensed and not sold. Software documentation is licensed to you by Mekatronix™, the licensor and a corporation under the laws of Florida. Mekatronix™ does not assume and shall have no obligation or liability to you under this license agreement. You own the diskettes on which the software is recorded but Mekatronix™ retains title to its own software. The user may not rent, lease, loan, sell, distribute Mekatronix™ software, or create derivative works for rent, lease, loan, sell, or distribution without a contractual agreement with Mekatronix™.

- Limited Warranty. Mekatronix™ strives to make high quality products that function as described.  However, Mekatronix™ does not warrant, explicitly or implied, nor assume liability for, any use or applications of its products. In particular, Mekatronix™ products are not qualified to assume critical roles where human or animal life may be involved. For unassembled kits, the user accepts all responsibility for the proper functioning of the kit. Mekatronix™ is not liable for, or anything resulting from, improper assembly of its products, acts of God, abuse, misuses, improper or abnormal usage, faulty installation, improper maintenance, lightning or other incidence of excess voltage, or exposure to the elements. Mekatronix™ is not responsible, or liable for, indirect, special, or consequential damages arising out of, or in connection with, the use or performances of its product or other damages with respect to loss of property, loss of revenues or profit or costs of removal, installation or re-installations. You agree and certify that you accept all liability and responsibility that the products, both hardware and software and any other technical information you obtain has been obtained legally according to the laws of Florida, the United States and your country.  Your acceptance of the products purchased from Mekatronix™ will be construed as agreeing to these terms.

MANIFESTO

Mekatronix™ espouses the view that personal autonomous physical agents will usher in a whole new industry, much like the personal computer industry before it, if modeled on the same beginning principles:

- Low cost,

- Wide availability,

- Open architecture,

- An open, enthusiastic, dynamic community of users sharing information.

Our corporate goal is to help create this new, exciting industry!

WEB SITE:  http://www.mekatronix.com

Address technical questions to  tech@mekatronix.com

Address purchases and ordering information to an authorized Mekatronix Distributor http://www.mekatronix.com/distributors

Disclaimer

While Mekatronix™ has placed considerable effort into making these instructions accurate, Mekatronix™ does not warrant the results and the user assumes the risks to equipment and person that are involved.

TABLE of CONTENTS

PART ONE

MindStamp ™ Description, Setup and Software

1.	Mindstamp™ microcontroller prototyping	7

1.1	MTJPRO11™ MindStamp™ KIT COMPONENTS and Options	7

1.1.1	Optional MindStamp™ Items	7

1.1.2	MindStamp™ Power Pack	7

1.1.3	Operation from a DC Power Supply	7

1.2	MindStamp™ Software Development Systems	9

1.3	Personal Computer Requirements	9

1.4	Physical Configuration MindStamp™	11

2.	mindstamp™ Assembly	11

3.	Operating Mindstamp™ with icc11 windows	14

4.	Operating Mindstamp™ with icc11 DOS	14

5.	Operating Mindstamp™ with ic WINDOWS	14

6.	Freeware software options	15

7.	MindStamp™ Programming SET-UP	15

7.1	Computer Requirements	15

7.1.1	Primitive System (DOS)	16

7.1.2	Operating from a WINDOWS System (Recommended)	16

7.1.3	Com port problems to avoid	16

7.2	Serial Communication between Host Computer and MindStamp™	17

7.2.1	Plugging MB2325 into the host computer	17

7.2.2	Connecting the MB2325 to the MTJPRO11	17

7.2.3	Verify serial connection	17

7.3	Software Language Support	18

8.	ICC11 and MindStamp™ Software Installation summary	19

9.	Installation of ICC11 on a pc	19

10.	MindStamp™ DISTRIBUTION SOFTWARE	20

10.1	Integrating MindStamp™ Distribution Software with ICC11 for Windows	20

10.2	Description of MindStamp™ installms.bat	20

10.3	Contents of the Distribution Software Package	21

11.	Editing, Compiling, Downloading MindStamp™ Programs:  Summary	23

IMPORTANT NOTE	24

12.	MINdstamp™ with icc11 windows	24

12.1	IDE Compiler and Linker Setup for MindStamp™	24

12.2	IDE Setup for Downloading into MindStamp™	25

12.3	COMPILE and EDIT on IDE	26

12.4	DOWNLOADING using IDE	26

12.5	Execution of  MindStamp™ Code with IDE	27

13.	DOWNLOADINg with the HSSDL11	27

13.1	Installation of HSSDL11	28

13.2	HSSDL11 Setup	28

13.3	HSSDL11 Download Procedure	29

13.4	HSSDL11 Operation Tips	29

14.	MINDSTAMP™ for icC11 DOS	30

14.1	MindStamp™ ICC11 DOS Setup	30

14.2	Compile with ICC11 DOS	30

14.3	PCBUG11 Download Procedure for MindStamp™	31

14.4	Execute a MindStamp™ Program	32

14.5	A WINDOWS Development Environment in DOS	32

14.6	MindStamp™ Library Code	32

14.7	Generation of MindStamp™’s Library with DOS Script	33

14.8	Brief Descriptions of MindStamp™ Programs	34

15.	Programming your MindStamp™ with Interactive c	34

15.1	Installing and Using IC	34

15.2	Serial Communications with the MindStamp™.	35

15.3	Loading Pcoder with  IC WINDOWS	36

15.4	Loading Pcoder with IC DOS and other Methods	37

15.5	Loading and Unloading Programs in IC	38

15.6	Loading and Unloading Programs in  IC WINDOWS	38

15.7	Interactive C	38

15.8	MindStamp™ Commands for IC WINDOWS	39

15.9	Writing your own IC Programs	40

16.	experiments WITH icC11 and IC	42

17.	Analog input experiment	43

18.	Reading a Potentiometer with the analog input port	46

19.	CdS cell light sensor	49

20.	Thermistor	50

21.	LED output	50

22.	Multiple LED output	50

23.	Light meter	50

24.	InfraRed LIGht meter	50

25.	Measure Intensity of an IR LED Light source	50

LIST of FIGURES

Figure 1. This layout of the MindStamp™ shows placement of the MTJPRO11 microcontroller and other components. The basic kit does not include the optional solderless breadboards. A Velcro strap slips around the battery pack and through slots in the platform to hold the battery pack in place.	10

Figure 2. The AC Power adapter jack plugs into the power jack on the MindStamp™ platform. A serial cable connects a PC COM port to the MB2325 circuit board. The MindStamp™ battery pack provides enough current to supply small motor current surges without causing the processor regulator from dropping out and resetting the processor.	11

Figure 3.  Before you can operate MindStamp™, you will need to wire up the switch connections, battery and the Mekatronix 6-wire serial (SCI) cable connection as illustrated here.	13

Figure 4.  The switch attached to analog channel-2 (PortE- pin 2, i.e., PE2) allows you to digitize the two possible input voltages of 5volts and 0volts. The 6 wire serial cable connecting the MB2325 and the MTJPRO11 reminds you that all the power and switch connections (Figure 3) must be in place. In this and succeeding diagrams, leaves out the power and switch connections for clarity.	45

Figure 5. This experiment allows you to measure the analog voltage across a potentiometer. The A/D converter of the MTJPRO11 microcontroller generates the digital equivalent of the analog voltage across the potentiometer. The analog input voltage to channel-2 on PE2 changes as you vary the potentiometer position.	48

Figure 6. The Cadmium Sulfide (CdS) cell resistance changes with the intensity of the incident light and is known as a photoresistor. As the light increases the resistance decreases. By measuring the voltage change across the cell using an analog input the circuit measures the relative light intensity. Try different resistors in the voltage divider network and experimentally determine the effect that has on the response of the system to the same light sources.	50

Figure 7.  A simple voltage divider consisting of a resistor in series with a thermistor allows you to measure temperature.	50

Figure 8. This circuit alows you to drive an Light Emitting Diode (LED). Note jumper (circled in red) to enable digital output on the IRLED output header. You can just as well use the IRLED OUTPUT HEADER to drive digitial outputs other than LEDs.	50

Figure 9. This circuit illustrates how you can use the digital output latch to control eight LEDs. Note the position of the jumper (red circle). Of course, you can attach other two terminal devices to the output latch as long as they do not require more than 5volts and about 10ma each. For more sophisticated use of the digital output latch refer to the manufacturers specification.	50

Figure 10. Construct a light meter by attaching a CdS voltage divider network and eight LEDs to the MTJPRO11 microcontroller as shown. Note the position of the jumper (red circle). The value read on PE2, when sent to the IR emitter output register drives the lights according to the binary pattern of ones in the digital output register.	50

Figure 11. The IR receiver detects 40KHZ modulated 940nm IR light. This circuit measures IR now instead of visible light. The visible LEDs provides a binary display of the relative, modulated IR light intensity.	50

Figure 12. This circuit looks identical to the previous one. However, the visible LED on pin-1 is replaced with an IR LED with 940nm center frequency.	50

LIST of TABLES

Table 1 List of MindStamp Components and Options	8

Table 2 MindStamp™ Power Pack	8

Table 3 Solderless Breadboards	8

Table 4 MindStamp™ Sensor Kit	9

Table 5 MindStamp™ Software Support	9

Table 6 PC Requirements	9

PART ONE

MindStamp ™ Description, Setup and Software

# Mindstamp™ microcontroller prototyping

The MindStamp™ microcontroller prototyping system allows you to conveniently develop microcontroller hardware and software applications for the Motorola MC68HC11 family.  You can use MindStamp™ to design interfaces and control software for motors, sensors, instruments, robots, games, electronic projects, experiments, appliances and other embedded applications.

Figure 1illustrates a typical layout of the MindStamp™ prototyping system.

## MTJPRO11™ MindStamp™ KIT COMPONENTS and Options

Refer to Table 1 to find an itemized list of the MindStamp™ kit components and options available. You can purchase the Power Pack kit, Solderless Breadboards and the MindStamp™ Sensor Kit separately. They are not part of the basic MindStamp™ kit since some individuals may already have these components. The currently available optional MindStamp™ components available from Mekatronix are listed below.

### Optional MindStamp™ Items

(Check with an authorized Mekatronix Distributor www.mekatronix.com/distributors for pricing.)

- Power Pack (Table 2)

- Solderless breadboard: two will mount on the MindStamp™ platform (Table 3),

- LCD Screen,

- Sonar,

- Contact Force Sensor

- MindStamp™ Sensor Package (Table 4),

- Servos,

- D.C. gearhead motors with motor driver circuit (hacked servos).

### MindStamp™ Power Pack

All MindStamp™ systems require a Power Pack or equivalent. The MindStamp™ operates from a 6 AA NiCd rechargeable battery pack with an AC Adapter to provide continuous trickle charge current. The battery pack makes MindStamp™ portable and the AC Adapter trickle charge permits long hours of use without recharging. Should the batteries become discharged, the AC Adapter will take 6 hours to recharge them fully.

### Operation from a DC Power Supply

You can also connect a 6-7volt, one ampere, D.C. power supply to the battery header BATT. Such a supply may not be able to provide motor surge currents like NiCd batteries, even for small motors, so operation with motors may be severely limited.

Warning!!!

Be sure to connect the polarity of an external power supply correctly or you will destroy all of MindStamps™ electronic integrated circuit chips.

Warning!!!

Do not plug in the AC ADAPTER when an external power supply is being used. Use AC ADAPTER only with Mekatronix  6-AA NiCd Power Pack.

Table 1 List of MindStamp Components and Options

| MindStamp Components | Component Description | Qty |
| --- | --- | --- |
| MSABSP | An ABS plastic platform base, predrilled and cut | 1 |
| MTJPRO11A | MTJPRO11 microcontroller board, assembled | 1 |
| MB2325A | MB2325A communications board, assembled | 1 |
| BCSNAPS6 | 6 AA battery holder | 1 |
| Screw 4 – 40 ½ | Machine Screw 4-40 ½ inch | 14 |
| Screw 4 – 40 ¼ | Screw #4 ¼ inch long | 4 |
| Standoff 4 – 40 ¼ | Standoff#4 ¼ inch long | 4 |
| Standoff 4 – 40 ¾ | #4 Standoff ¾ inch long | 12 |
| LED Red T1-3/4 | LED Red size T1-3/4 | 1 |
| SWSPST | Switch SPST | 1 |
| SWPB | Switch Push Button | 1 |
| SWDPDT | Switch DPDT | 1 |
| LED Mount T1-3/4 | LED Mount T1-3/4 | 1 |
| Charge Jack | Charge Jack | 1 |
| Ribbon Cable, 60-wire | Wire for all cabling /30cm | 1 |
| Connector 9V | Connector, 9V with leads | 1 |
| Res 150 Axial | Resistor 150 Ohm Axial ¼ watt | 2 |
| F1x36 | 36 pin female connector |  |

Table 2 MindStamp™ Power Pack

| MindStamp Power Pack |  |  |
| --- | --- | --- |
| AA NiCd | AA NiCd Batteries | 6 |
| BC12A500 | 12V 500mA charger | 1 |

Table 3 Solderless Breadboards

| Breadboard | Solderless Breadboard (Up to two mount on an MSABSP) | 1 or 2 |
| --- | --- | --- |

Table 4 MindStamp™ Sensor Kit

| Component | Description |  |
| --- | --- | --- |
| LED Red T1-3/4 | LED Red size T1-3/4 | 10 |
| Res 470 Axial | Resisitor 470 Ohm Axial 1/4 watt | 10 |
| Res 10K Axial | Resisitor 10K Ohm Axial 1/4 watt | 2 |
| Thermistor | Thermistor | 1 |
| CdS Cell | CdS Cell | 1 |
| 10K Pot | 10K Potentiometer | 1 |
| SWSPDT | Switch SPDT | 1 |
| GP1U58X | IR Receiver | 1 |
| LED IR | IR LED | 1 |

## MindStamp™ Software Development Systems

Mekatronix supports three software development environments and provides two, non-supported freeware environments as a service to our clients. Refer to Table 5. For more details refer to Section 7.3.

Table 5 MindStamp™ Software Support

| MindStamp™ Software Development System | Mekatronix Support |
| --- | --- |
| ICC11 for WINDOWS | Yes |
| ICC11 for DOS | Yes |
| IC for WINDOWS | Yes |
| IC for DOS | No (freeware) |
| Assembly Language | No (freeware) |

## Personal Computer Requirements

To run MindStamp™, MTJPRO11 version, you will need a PC and a serial cable from a PC COM port (default is COM1) to the 25-pin D-connector on the MB2325 board. PC hardware system requirements depend upon the software development system you choose. Refer to Table 6 for specifics. For additional information refer to Section 7.1.

Table 6 PC Requirements

| MindStamp™ Software Development System | PC Requirements |
| --- | --- |
| ICC11 for WINDOWS | PC running WIN95 |
| ICC11 for DOS | 386 PC or Better |
| IC for WINDOWS | PC running WIN95 |
| IC for DOS | 386 PC or Better |
| Assembly Language | Any IBM Compatible PC |

Figure 1. This layout of the MindStamp™ shows placement of the MTJPRO11 microcontroller and other components. The basic kit does not include the optional solderless breadboards. A Velcro strap slips around the battery pack and through slots in the platform to hold the battery pack in place.

## Physical Configuration MindStamp™

Figure 2 illustrates the MindStamp™ prototyping environment. The batteries make MindStamp™ portable. Although the MTJPRO11 has volatile RAM, the batteries save the memory contents as long as they remain connected, even with the on/off switch in the off position. The AC Adapter keeps trickle-charging the batteries as you work with MindStamp™ and will give many hours of uninterrupted operation under light load. If the batteries should discharge, the AC Adapter takes 6 hours to recharge the batteries. Leave the AC Adapter plugged into the “power jack” receptacle at all times, if possible. This keeps the batteries charged under many experimental loads. Hence, discharged batteries will not interrupt your development time.

Serial Cable: 25-Pin RS232C Connection

AC

Power

Adapter

Charger

COM1

PERSONAL COMPUTER

Figure 2. The AC Power adapter jack plugs into the power jack on the MindStamp™ platform. A serial cable connects a PC COM port to the MB2325 circuit board. The MindStamp™ battery pack provides enough current to supply small motor current surges without causing the processor regulator from dropping out and resetting the processor.

# mindstamp™ Assembly

Before you can run MindStamp™ you must first assemble the mechanically components and then electrically wire up the switches and the power ciruit.

- Insert ½ inch 4/40 machine screws through each corner of the platform and on the underneath side screw in ¾ inch threaded offsets to serve as feet.

- Use the ¼ inch 4/40 machine screws, washers and nuts to mount the MB2325 and MTJPRO11 circuit boards as illustrated in Figure 1. Use two screws to mount the MTJPRO11. Place the screws on diagonally opposite corners.

- If you purchased solderless breadboards, or already have your own, there are predrilled holes to mount them with machine screws, washers and nuts. The three holes allow mounting the most popular commercial varieties. On the underneath side screw on threaded offsets to each breadboard mounting screw to give the same height as the corner feet. This will give the strong mechanical support needed as you insert components into the breadboard.

- Mount the switches, power jack and the red Power-On LED onto the MindStamp™ platform. The small black LED plastic mount should be snapped into the LED hole. The LED then snaps into the mount.

- Wire up the red LED, On/Off, Download/Run and Reset switches and the battery cable in Figure 3.

WARNING!!!

Always snap the battery cable onto the battery pack BEFORE plugging the female connector into the MTJPRO11 male header labeled BATT. Otherwise, inadvertent touching the snap clip to the wrong polarity will destroy all the integrated circuits on the MTJPRO11. Once snapped to the battery case leave the cable snapped. Disconnect the battery pack from the MTJPRO11 by disconnecting the female connector from the MTJPRO11 board only. Leave the snapped end connected to the battery pack.

- Connect the J2 header on the MB2325 and the SCI header on the MTJPRO11 with a six-wire female-female connector (Refer to Figure 3).

- Wire up the two 150 ohm ¼ watt resistors in parallel. Solder one end of each resistor directly to the power jack terminal for good heat sinking.

Figure 3.  Before you can operate MindStamp™, you will need to wire up the switch connections, battery and the Mekatronix 6-wire serial (SCI) cable connection as illustrated here.

# Operating Mindstamp™ with icc11 windows

Before you can start a MindStamp™ prototyping system session first

- Assemble the MindStamp™ system (Section 2),

- Install and setup the ICC11 Integrated Development Environment (IDE) (Section 8, 12 and 12.2),

- Install the MindStamp™ software (Section 10.1).

After carrying out the first three steps, start a MindStamp™ session by performing the following:

- Connect the batteries and plug in the AC Adapter to power up MindStamp™. The part labeled “Powerjack” is really a recharge jack.

- Flip the  on/off toggle switch to on,

- Flip the download/run switch to download,

- Establish a serial connection with your PC (Sections 7.2 or 15.2), and

- Edit, Compile and Download to MindStamp™ (Section 12).

# Operating Mindstamp™ with icc11 DOS

Before you can start a MindStamp™ prototyping system session first

- Assemble the MindStamp™ system (Section 2),

- Install and setup ICC11 DOS (Section 8 and 14),

- Install the MindStamp™ software (Section 10.1).

After carrying out the first three steps, start a MindStamp™ session by performing the following:

- Connect the batteries and plug in the AC Adapter to power up MindStamp™. The part labeled “Powerjack” is really a recharge jack.

- Flip the  on/off toggle switch to on,

- Flip the download/run switch to download,

- Establish a serial connection with your PC (Sections 7.2 or 15.2), and

- Setup the DOS environment (Section 14.1),

- Edit, Compile and Download to MindStamp™ (Section 14).

# Operating Mindstamp™ with ic WINDOWS

Before you can start a MindStamp™ prototyping system session first

- Assemble the MindStamp™ system (Section 2),

- Install and setup IC WINDOWS (Section 15.1),

- Install the MindStamp™ software (Section 10.1).

After carrying out the first three steps, start a MindStamp™ session by performing the following:

- Connect the batteries and plug in the AC Adapter to power up MindStamp™. The part labeled “Powerjack” is really a recharge jack.

- Flip the  on/off toggle switch to on,

- Flip the download/run switch to download,

- Establish a serial connection with your PC (Sections 7.2 or 15.2), and

- Edit, Compile and Download to MindStamp™ (Section 15).

# Freeware software options

If you did not purchase ICC11 or the commercial version of IC and you do not have your own C compiler for the MC68HC11 family of computers, you have several options:

- You can program in the non-commercial (freeware) version of IC (IC DOS) or

- You can program in MC68HC11 Assembly language, assemble your programs from the Motorola freeware Assembler, and download assembled code (“s19” files) with PCBUG11(freeware).

The freeware version of PCBUG11, IC and the MC68HC11 Assembler can be downloaded from www.mekatronix/downloads. An IC Users Manual can also be downloaded from the Mekatronix web site.

NOTE: Mekatronix does not support these programs and recommends the commercial versions for higher reliability, expanded function, and technical support.

The Motorola Assembler will generate “s19”files from your assembly language programs. “s19” files are ASCII files that the Motorola boot loader can read and convert to MC68HC11 machine code. The PCBUG11 download process for “s19”files is described in Section 14.3 of this manual.

Assembly language programming is beyond the scope of this manual. A good, readable beginning textbook for learning MC68HC11 assembly language programming is:

Microcomputer Engineering, Gene H. Miller, Prentice Hall, 1993, ISBN 0-13-584475-4

# MindStamp™ Programming SET-UP

The following sections instruct you on how to setup and use your MindStamp™ Distribution Software package and integrate it with the compiler systems ICC11 WIN95 or ICC11 DOS, or the interpreter systems IC WIN95 or IC DOS.

## Computer Requirements

Program development for the MindStamp™ can be performed either a WINDOWS or DOS system. In a windows system you have the option of running DOS, but not vice-versa. In either case you will find the Mekatronix High Speed Serial Downloader (HSSDL11) a useful tool in a windows environment. The HSSDL11 downloads programs into the MindStamp™ microcontroller at a blistering 115.2KBaud.

### Primitive System (DOS)

ALERT!

MICROSOFT™ has announced that DOS will be phased out within a few years. This may affect your choice of WINDOWS or DOS software. In particular, when you upgrade your computer system in the future, you may have software compatibility problems if you run your MindStamp™ from DOS. Mekatronix™ recommends the WINDOWS system for longer term utility!

The following computer system will meet all the basic hardware support for developing MindStamp™ programs and applications.

- 386-PC or later processor running under DOS with several Megs of available hard disk space and 1 Meg of RAM.

- An available serial port. We will assume that COM1 is used.  See the next section about COM port problems to avoid.

- 1.44 Meg floppy

- Optional, but very useful, a terminal program such as Kermit for DOS.

### Operating from a WINDOWS System (Recommended)

The following high-end hardware offers a more sophisticated, easier to use, more productive working environment.

- A high speed (100MHz or better) computer running Windows 95

- NO modem on COM1

- A mouse on COM2

- A printer

- Access to the Internet, especially the World Wide Web,

- Windows terminal (terminal.exe), or, ICC11 IDE Terminal.

### Com port problems to avoid

Interactive C for DOS (IC-DOS) and Motorola’s PCBUG11 , both freeware, use the serial port in a manner that does not permit sharing the serial interrupt.  Therefore, the port that communicates with the MindStamp™ should not share the same interrupt with any other system resource.  For example, COM1 and COM3 share the same interrupt in DOS, as do COM2 and COM4. Therefore, if the MindStamp™ is connected to COM1, nothing else should be connected to COM1 or COM3 or use their interrupts.  We have seen some extreme instances when the internal modem of a computer had to be removed to get IC or PCBUG running. The modem used the same interrupt as the MindStamp™ COM port and disrupted the MindStamp™ serial communication.

## Serial Communication between Host Computer and MindStamp™

This section assumes you have powered a properly assembled MindStamp™ with the Mekatroix™ 12volt 500ma AC adapter and the microprocessor functions. Without either of these prerequisites, you cannot begin.

The MB2325 serial communications board generates the proper voltage conversions that permit the user to download and upload code and data to MTJPRO11™ via a 6-wire serial communications link. Whenever testing programs on MindStamp™, the user will connect the host computer with MindStamp™ via this interface.

### Plugging MB2325 into the host computer

The MB2325 serial communications board has a 25 pin, female, type D-connector. On some computers, COM1 is a 25pin male D-connector.  If your computer has a 9-pin connector you will need a 9 to 25 pin converter (available at most computer and electronics stores).  When the MB2325 is plugged into the COM1 port of the host computer, D1 will light when the host computer is powered. For laptops in sleep mode, D1 will not light until the laptop switches to active mode.  The D1 light lets you know that the serial port on the host computer is working.  If D1 does not light, then, assuming the MB2325 board is functional, something may be wrong with the serial connection to the host computer. In any case, the MindStamp™ will not be able to communicate with the host computer until the serial connection/protocol problem is resolved.

### Connecting the MB2325 to the MTJPRO11

After you connect the MB2325 to your personal computer with a serial cable, take the 6-pin rainbow colored cable and insert one end onto J2 of the MB2325. Connect the other end onto the 6-pin header labeled SCI on the MTJPRO11™ printed circuit board. The J2 and SCI header pins nearest the edge of their respective boards should be connected by the same wire.

### Verify serial connection

Turn the power switch on. The red power-on LED on the MindStamp™’s platform should light (If not, make sure the batteries have a full charge and are connected. Also, verify you have plugged the AC adapter into a wall socket and that you have inserted the jack into the receptacle on the MindStamp™ platform). Flip the down-load / run switch to the down-load position. Flip the power- save / on switch to the on position. Press and momentarily hold reset (the red button) on the MindStamp™.  With reset pressed, D2 on the MB2325 should light. When reset is released, D2 on the MB2325 turns off.

If the lights on the MB2325 board do not function as specified, try reversing the 6-wire serial cable. If that fails, verify that no other programs or devices are using the serial port. If you assembled the MB2325 yourself and the above solutions do not work, then you check to see that the LEDs were soldered in with the correct polarization.

## Software Language Support

MindStamp™ programs can be developed under Image Craft C (ICC11), the compiled version of C for the MC68HC11, or under Interactive C (IC), the C interpreter with multi-tasking developed for MIT’s 6.270 course.

ICC11 comes in a DOS and a WINDOWS version. The WINDOWS version offers an Integrated Development Environment (IDE).  Mekatronix™ sells both versions of ICC11 through its distributors. ICC11 windows version is the preferred programming language for extensive program development on MindStamp™ as it gives you complete control over all the existing hardware resources and even ones that you add yourself.

While Mekatronix recommends ICC11 for most applications, but for some applications IC has definite advantages. For example, in some school laboratory environments where student use MindStamp™ as a project board, a prototyping board or a circuit experiment, the interactive capability of IC offers ease of use and quick results. ICC11, of course, may also be used in such circumstances with good effect.

IC DOS is freeware, is supplied at no charge, and can be downloaded from the Mekatronix web site (www.mekatronix.com). IC for WIN95 can be purchased from a Mekatronix distributor. IC for WIN95 allows you to use IC in a WINDOWS environment and is much more convenient and easier to use than the IC DOS version. Either version of IC lacks some application flexibility on the MindStamp™ as compared to ICC11 and probably should not be used for most embedded applications. In ICC11 you can write you own interrupt drivers for add-on motors, servos and sensors, but not in IC. IC, on the other hand, has built in multitasking, which can be useful for some applications.

IC originally developed around robot applications but is not restricted to them. To control the processor resources with IC you will necessarily use some “robot” commands. If these robot commands do not perform the exact functions you want for your application, you will need to use ICC11. Another limitation of IC is that all processor resources are not available for user applications. For example, IC employs the MC68HC11 “output compare four” (OC4) pin to control its multi-tasking timing and is not available for other usage. The rapid changes of OC4 will actually burn out any servo attached to the PA4 header on MindStamp™’s MTJPRO11 microcontroller board.

WARNING!

IC code thrashes any servo attached to PA4 and will easily destroy the servo in a short amount of time. Disconnected any servo from PA4 before MindStamp™ runs IC code.

# ICC11 and MindStamp™ Software Installation summary

If you purchased the ICC11 software this section provides guidance in its installation and integration with the MindStamp™ software. Be sure to refer to your ICC11 manual for installation particulars for the ICC11 system. The IDE environment makes the windows version an efficient tool for programming your Mekatronix MindStamp™ and makes life much easier than the DOS version. Mekatronix highly recommends the WINDOWS version. Experience has shown it is well worth the additional cost.

Note: In typed commands to your computer, angle brackets indicate parameters for which you must substitute the appropriate information. Do not actually type the angle brackets. For example, <enter> means “Press the Enter key on the keyboard ; <filename> means “Type the filename alphanumeric key sequence on the keyboard and the path to it, if necessary.”

#### Summary of Installation Process

(For details read the sections that follow this one)

- Insert ICC11 diskette and install ICC11, or

Download from an authorized Mekatronix distributor (www.mekatronix.com/distributors) web site and execute the installation program.

- Remove ICC11 diskette, insert the most recent version of the MindStamp™ Distribution Software diskette and install, or

Download from an authorized Mekatronix web site and install. Installation essentially means copying files into appropriate directories of c:\iccms.

# Installation of ICC11 on a pc

Insert ICC11 diskette and install ICC11, or download from an authorized Mekatronix distributor (www.mekatronix.com/distributors) web site. For the WINDOWS version you will see an executable file such as v51win.exe. Execute it and follow directions. A DOS version will not have “win” in the name. Specify c:\iccms as your root directory in either case. Let the installation update your DOS CONFIG file. Reboot. For the WINDOWS version put the ICC11 icon shortcut on your desktop.

After ICC11 installation, you should observe the following directory structure

- c:\iccms

- Bin

- Examples

- Include

- Lib

- Libsrc

In the DOS version you will find the icc11.exe file in Bin. To invoke the C-compiler in DOS simply type icc11 with appropriate parameters at the DOS prompt. A DOS batch file mssetup.bat allows you to set up Linker options which will continue in force until you change them. This saves considerable typing effort when compiling files in DOS.

In the WINDOWS version, the binary folder Bin contains the IDE executable file ICC11ide.exe. For convenience, you may want to create a shortcut to this program on your desktop.  After setting up IDE (see below), installing the MindStamp™ software (Section 10.1) and establishing a serial connection (7.2 or 15.2), you can begin a MindStamp™ session by opening IDE with a double click on its icon. IDE also supplies a terminal program which accepts serial communication from MindStamp™.

# MindStamp™ DISTRIBUTION SOFTWARE

The distribution software includes files with important instructions and information. This information, with some additional comments and elaboration, is repeated here for your convenience. Being familiar with the contents of this package will help you find the information you need to install and use it with your MindStamp™.

## Integrating MindStamp™ Distribution Software with ICC11 for Windows

Integrating MindStamp™ software amounts to copying MindStamp™ files into iccms directory at the appropriate places.

#### Prerequisites

- The C-Compiler has already been installed into the directory iccms and

- The batch file installms.bat will execute from the directory holding the MindStamp™ Distribution Software.

Execute the installation batch file installms.bat with an argument specifying the root directory where the MindStamp™ files will be loaded. If no directory is specified, c:\iccms is assumed. The latter is recommended, since all Mekatronix batch files assume the root is c:\iccms.

## Description of MindStamp™ installms.bat

This batch file integrates the MindStamps™ libraries and code with the compiler installation. See below for a listing of this file.

installms.bat

@echo off

if "%1"=="" goto noargument

copy mssetup.bat %1

copy msReadme.txt %1

copy msProg.txt %1

copy ms_Install_Readme.txt %1

copy IDEsetup.txt %1

copy binms\*.* %1\bin\*.*

copy inclms\*.* %1\include\*.*

copy libms\*.* %1\lib\*.*

if not exist %1\libsrms md %1\libsrms

copy libsrms\*.* %1\libsrms\*.*

if not exist %1\mss19 md %1\mss19

copy mss19\*.* %1\mss19\*.*

if not exist %1\mscode md %1\mscode

copy mscode\*.* %1\mscode\*.*

md %1\pcbug11

copy pcbug11\*.* %1\pcbug11\*.*

md %1\assembler

copy assembler\*.* %1\assembler\*.*

goto end

: noargument

copy mssetup.bat c:\iccms

copy msReadme.txt c:\iccms

copy msProg.txt c:\iccms

copy ms_Install_Readme.txt c:\iccms

copy IDEsetup.txt c:\iccms

copy binms\*.* c:\iccms\bin\*.*

copy inclms\*.* c:\iccms\include\*.*

copy libms\*.* c:\iccms\lib\*.*

if not exist c:\iccms\libsrms md c:\iccms\libsrms

copy libsrms\*.* c:\iccms\libsrms\*.*

if not exist c:\iccms\mss19 md c:\iccms\mss19

copy mss19\*.* c:\iccms\mss19\*.*

if not exist c:\iccms\mscode md c:\iccms\mscode

copy mscode\*.* c:\iccms\mscode\*.*

md c:\iccms\pcbug11

copy pcbug11\*.* c:\iccms\pcbug11\*.*

md c:\iccms\assembler

copy assembler\*.* c:\iccms\assembler\*.*

: end

## Contents of the Distribution Software Package

The file msreadme.txt summarizes the contents of the distribution software package.

*************************************************************************

BE SURE TO REFER TO THE MindStamp™ USERS MANUAL FOR DETAILED INSTRUCTIONS!

*************************************************************************

CONTENTS OF THIS DISK

MindStamp™ SUPPORT SOFTWARE

============================================================================

This disk includes the following directories:

<Assembler> 	Freeware assembler for the M68HC11

<Binms>	Batch files to compile and load MindStamp™ programs

<Inclms>	MindStamp™ include files used specifically by MindStamp™ programs

<Libms> 	MindStamp™ library files

<Libsrms>	Object code for MindStamp™ library files

<PCBUG11>	PCBug11 monitor for the M68HC11, Motorola Freeware

<mscode>    Applications Source code for MindStamp™

----------------------------------------------------------------------------

The distribution also contains the following files:

IDEsetup.txt	How to use IDE to edit, compile, download to MindStamp™.

installms.bat	Batch file to install the MindStamp™ distribution software into your ICCMS directory. This batch file is useful for both DOS and WINDOWS versions.

ms_install_readme.txt

Brief directions on installation of MindStamp™ distribution software.

msProg.txt

How to compile and download code into the MindStamp™ using ICC11 DOS.

msReadme.txt	This file.

mssetup.bat	      Batch file to setup paths and ICC11 linker parameters.

----------------------------------------------------------------------------

Assembler directory

This directory contains as11.exe, a freeware assembler for the M68HC11

----------------------------------------------------------------------------

Binms directory

ICC11 WINDOWS does not require any of the MindStamp™ batch files in this directory, namely, compms.bat, loadms.bat, clms.bat. In ICC11 DOS these batch files allow you, from a DOS window, to

1. Compile a MindStamp™ program,

2. Download a MindStamp™ program in Motorola ".s19" file format into the MindStamp™ using PCBUG11 (PCBUG11 has a number of severe limitations. For trouble free operation, ease of use, convenience and more than 10 times faster downloading we recommend you purchase the inexpensive Mekatronix HSSDL11. Our programmers use nothing else!),

3. Do both a compile and load of a MindStamp™ program into the MindStamp™ microcontroller, respectively.

To use loadms.bat, your program's ".s19" file must reside in the directory mss19 which must be in the root iccms at the same level as the pcbug11 directory.

----------------------------------------------------------------------------

Include directory Inclms

This directory contains all the header files <file.h> required by MindStamp™

applications written in C. As you add new applications you will add new header

files to this directory. For example, you can purchase and add the SERVO software kit that permits calibrating and controlling up to three servos on the MindStamp™.

----------------------------------------------------------------------------

Library directory Libms

This library directory contains the basic MindStamp™ library file "libms.a"

----------------------------------------------------------------------------

Library Source directory Libsrms

MindStamp™ library object files.

----------------------------------------------------------------------------

PCBUG11 directory

This is Motorola's monitor program for the HC11. Mrc11.bat is used by loadms.bat

Also included is the batch file MSCC11.bat, which will automatically load a program

into a M68HC11E2 chip.

HIGHLY RECOMMENDED:

Buy the HSSDL11 (well worth the price!) and do not use PCBUG11. PCBUG11 hangs up the COM port and you have to kill the DOS window using PCBUG11 in order to use any other serial device on your computer and vice- versa.

----------------------------------------------------------------------------

MindStamp™ Code directory mscode

This directory contains simple application programs.

----------------------------------------------------------------------------

mss19

ICC11 WINDOWS does not require this directory, but ICC11 DOS does.

PCBUG11 downloads s19 files from this directory when you invoke the batch file

loadms.bat. Path and name length restrictions make it almost impossible to use

PCBUG11 in complex directory structures. The directory name mscode is too long(!) and cannot be used! Be sure your ".s19" file names are no more than eight characters long (not counting the extension ".s19") and that this file is in mss19 directory at the same level as the pcbug11 directory in the iccms directory. Correct installation will guarantee this. However, creative programmers who prefer a different directory structure are advised of this severe path-length limitation aspect of pcbug11.

----------------------------------------------------------------------------

# Editing, Compiling, Downloading MindStamp™ Programs:  Summary

If you have IDE on your PC, you will probably want to use the IDE editor for developing C language programs. Otherwise, pick your favorite program editor, but be sure to save the program as a text file with the ".c" suffix. Sections 12, 12.2, 12.3, 12.4, 12.5 and 12.5 explain how to setup and use the IDE environment for editing, compiling and downloading.

If you have ICC11 DOS, go to a DOS window, if your computer supports WINDOWS, or a DOS prompt and compile the program using the batch commands compms.bat. Download your program into the MindStamp™ using either loadms.bat in the DOS window. Execute the program. Repeat this process until the program performs satisfactorily.

Section 14 explains how to use ICC11 DOS. The file msProg.txt in the distribution software provides instruction on how to edit, compile, download, and execute your MindStamp™ using ICC11 DOS. Further details for DOS operation can be found in Section 14. If you have purchased ICC11 WINDOWS, you do not need to read msProg.txt. Instead, read IDEsetup.

Section 15 explains how to use MindStamp with Interactive C (IC).

### IMPORTANT NOTE

If you execute a MindStamp™ program that uses functions in serialms.c and you loaded the ".s19" file with PCBUG11, then you must close the DOS window in WIN95, because PCBUG11 locks up the COM port. Hyper Terminal will not be able to communicate through the COM port with the MindStamp™. For some archaic reason PCBUG11 in DOS hogs the COM port and does not release it unless you kill the DOS window.

SOLUTION

Eliminate PCBUG11 use. Buy the (inexpensive and well worth it!) Mekatronix high-speed serial downloader (HSSDL11) for the MC68HC11 and avoid the pain of closing the DOS window and opening and setting up the ICC11 parameters every time you reopen the DOS window!

When using the HSSDL11 with IDE Terminal or the Hyper Terminal on the same COM port, say COM1, you must close IDE Terminal or disconnect Hyper Terminal (type: ALT-C d, or select the Hyper Terminal telephone icon with receiver off the hook) to download. After download, select IDE Terminal or connect Hyper Terminal (type: ALT-C c, or select Hyper Terminal telephone icon with receiver on the hook). You do not have to do anything with HSSDL11 since it does not hog COM1, an extremely useful convenience because it retains its state and lets you quickly download a program you repeatedly compile and change in DOS. Just switch back and forth between the DOS window and the HSSDL11 window. For convenience, keep both open and side by side!

# MINdstamp™ with icc11 windows

## IDE Compiler and Linker Setup for MindStamp™

Once you setup the IDE environment, you will not have to change it from invocation to invocation, since it remembers its most recent state. For the MTJPRO11 version of MindStamp perform the following setup procedure.

- Double click on ICC11ide.exe icon to enter the Integrated Development Environment (IDE).

- Under Options in the menu bar select Compile.

- In Compiler window select the Linker tab.

- In the Linker setup window enter the following:

a. Text section:		0x8000

b. Stack:			0xffbf

c. Data Section:

d. Additional Libraries:	libms

(You may wish to use the Setup Wizard to save this configuration under the name MindStamp™.)

If you specified c:\iccms as your root during installation, then

e.  Library Path: c:\iccms\Lib

will be the default.

- Select Preprocessor under Compiler:

If you specified c:\iccms as your root during installation, then

Include Paths: 		c:\iccms\include

will be the default.

- Close Compiler window.

Continue immediately to the next section.

## IDE Setup for Downloading into MindStamp™

- Select Target on main menu bar.

- Select Bootstrap Download Mode.

- Select Target on main menu again.

Verify that Bootstrap Download Mode has a check mark by it and then

- Select Terminal.

A window opens. Expand it to fill your screen. At the bottom of the window, you will see several selection buttons. These buttons allow us to configure for bootstrapping loading.

- Select Bootstrap Options. The HC11 window opens.

- In Bootloader Programming select "External RAM".

- Set baud rate to default (1200)

- Close HC11 window.

You are now ready to edit, compile and download programs to your MindStamp™ from the IDE.

## COMPILE and EDIT on IDE

- Select File in the menu bar and click on Open.

Browse to select the source code file of interest, for example, c:\iccms\mscode\<file.c>, and open it.

A window opens which allows you to edit the program. Refer to the ICC manual for details on editor commands.

- From Window on the main IDE menu bar, select Tile to tile the Status and Editor windows (ALT-T).

- Select Compile in menu bar and then select Compile to Executable.

The compiler compiles the program in the currently active edit window. If no edit window is open and selected, no compilation is possible.

- Watch the Status window. A successful compile ends with the word SUCCEEDED. Otherwise, note errors, correct your program in the edit window, and repeat Steps 3 and 4.

## DOWNLOADING using IDE

You must carry out the setup procedures in Section 9 before using IDE to download any “s19” files into the MindStamp™. Your setup will be in the state that you left it in at the end of your previous IDE session.

Connect your PC, serial cable, MB2325 communications board and the six-wire Mekatronix serial cable C2325 to the MindStamp™ as described in Section 7.2.

Open IDE by double clicking on your ICC11 icon.

- Select Target in the menu bar.

- If Bootstrap Download Mode is checked, select Terminal and skip Step 3. Otherwise select Bootstrap Download Mode and do Step 3.

- Select Target again and click Terminal.

You now have the terminal window open.

- If necessary, select Browse to find the  “s19” file of interest. Select the desired “s19” file.

- Flip the MindStamp™’s Download/Run switch to Download.

- Press the MindStamp™’s red Reset button.

Make sure diode D2 on the MB2325 board lights when reset is held down and goes off when reset is released.

- Select Bootstrap Download in the IDE Terminal window.

- Select OK on the IDE message.

- The “s19” file loads into the MindStamp™.

You are now ready to execute the code on the MindStamp™.

## Execution of  MindStamp™ Code with IDE

The IDE terminal simulator (Terminal) accepts MindStamp™ serial output directly! This allows you to monitor MindStamp™ IO while the MindStamp™ remains connected to the PC, a nice, convenient feature of IDE.

#### Execute Program Procedure

- Download “s19” file into MindStamp™ from Terminal window (refer to Section 12.4).

- Flip MindStamp™ Download/Run switch to Run.

- Press MindStamp™ red Reset button.

Your program will start to run. If your program transmits serial output, it will appear on the terminal simulator of IDE that you currently have open. If your MindStamp™ produces IO to the Terminal screen, you may have to press reset several times or hold it down for a second to get clean communication.

# DOWNLOADINg with the HSSDL11

The Mekatronix product, the extremely low cost High-Speed-Serial–Downloader-11 (HSSDL11), provides an ultra fast (115.2Kbaud) serial download rate from your PC to the MindStamp™. This greatly reduces your development time and improves your productivity. Contact a Mekatronix distributor for purchasing.

Mekatronix encourages the use of the HSSDL11 software for downloading code onto the MindStamp™. The HSSDL11 operates in WIN95 and can be opened simultaneously with the IDE window. You can conveniently compile and edit in IDE. After downloading using the HSSDL11, you can open up Terminal in IDE to receive data from the MindStamp™ if the program produces serial output.

NOTE: Terminal in IDE and HSSDL11 both use the same COM port (default: COM1). The HSSDL11 releases the COM port after a download, even with the application window still open. Hence, HSSDL11 can remain open when Terminal in IDE is opened. However, you cannot download from HSSDL11 when Terminal or any other device captures the COM port. You will have to close the Terminal window (but not IDE!) or other device holding the COM port before HSSDL11 can download.

## Installation of HSSDL11

Just copy the HSSDL11 directory with all its subdirectories and files into you hard drive and establish a shortcut to it from your desktop.

## HSSDL11 Setup

The HSSDL11 remembers its setup, so you will only have to perform this procedure once or whenever you want to make changes. The currently selected “s19” file name appears above the HSSDL11 menu bar for convenience. Repeated compiles and loads of the same program during development require only pressing the HSSDL11 menu Download button or the d-key on the keyboard. For more HSSDL11 downloading details refer to Sections 13.3. If your program does not produced output, you may want to select the Auto-download feature in the Config menu. This feature will automatically download the active “.s19” file when your press reset on your MindStamp™, a really handy feature. Auto-download locks up the COM port attached to the MindStamp™. You may not wish to use Auto-download if your MindStamp™ program produces output or you wish to use the COM port in some other application at the same time. Alternatively, you can deselect the Auto-download feature to release the COM port if you want to shift back and forth between HSSDL11 and some other program requiring the same COM port.

- Open the HSSDL11.

- Confirm default Settings:

Clock:	8MHz

Port:	COM1

- Click on Config button and verify the default settings:

Default:   NO item is checked.

CONFIG:	DO NOT CHECK ANY ITEMS HERE!

Features:	Check High Speed option.

Check Auto-download if desired (see comments above)

Some older computers will not allow you to communicate at 115.2Kbaud and you may have to "unclick this option". The baud rate then defaults to 9600.

- Click on Config button again to leave configuration mode.

## HSSDL11 Download Procedure

Be sure the MindStamp™ power switch is on, the MindStamp™ power light is on, the physical serial connection is installed, the  HSSDL11 has been setup and its window open.

- If necessary, click on Program button to browse and find the MindStamp™ “s19” file of interest. Select Open to open the desired “s19” file. For example, you might select 						c:\iccms\mscode\mstst.s19.

- Flip the MindStamp™’s Download/Run switch to Download.

- Press the MindStamp™’s red Reset button.

Make sure diode D2 on the MB2325 board lights when reset is held down and goes off when reset is released.

- Select Download in the HSSDL menu.

First, the 256 bytes of the bootloader file loads at 1200 baud. Next, your program loads at 115.2Kbaud into the MindStamp™.

You are now ready to execute the code on the MindStamp™.

## HSSDL11 Operation Tips

Often the download will fail the very first time you try after opening the HSSDL11. In fact, it may take two or three tries initially as the program learns which version of the MC68HC11 microcontroller that you have. Thereafter, downloads usually succeed on the first attempt.

Precaution!

A PC executing background programs may cause the HSSDL11 to loose synchronization with the MindStamp™ serial port at 115.2Kbaud, so you may need to close such programs if you get repeated boot failures. At other times WIN95 does some automatic housekeeping that also interferes with the high-speed download and a retry will be necessary.

We have found that under normal operations with several programs and windows open, the HSSDL11 has no problems.

If the Download prints error message that the COM port is not available, it means that some other application is using COM1 and must be closed before the HSSDL11 can operate.

Sometimes you will forget to press the reset button on the MindStamp™ or to place the MindStamp™ in download mode. The HSSDL11 will signal a boot error! Check your switch settings, press reset and verify the D2 light on the MB2325 board flashes on when the reset button is held down. Try again. Typically, after a failure, the HSSDL11 will fail a second time, and, on occasion, a third time, even with everything correct. Do not worry. Press reset and repeat the Download command each time. For some reason the HSSDL11 sometimes remembers immediate past failures and then recovers on the second or third attempt. Of course, this problem is avoided altogether if you don’t make a mistake with the switches!

# MINDSTAMP™ for icC11 DOS

This section explains how to setup MindStamp™ to work with DOS.

## MindStamp™ ICC11 DOS Setup

The mssetup.bat file defines the paths to the Include and Lib directories as well as establish the necessary linker options for ICC11. Observe that the code begins at 0x8000 and the stack at 0xffbf. Note that the MC68HC11 stack grows toward lower memory addresses, hence, the stack grows toward your data and code sections and could potentially cause havoc with your program if it gets too large. Be aware of this possibility for large programs.

The library option –lms in the linker setup is essential and refers to Libms.a. You can add object files to this library following the template in Libmsmake.bat.

If you have the DOS version of the compiler and the library source code, you can remake the library Libms.a. with Libmsmake.bat.

Contents of mssetup.bat:

doskey

PATH %PATH%;C:\ICCMS\bin

set ICC_INCLUDE=C:\ICCMS\include

set ICC_LIB=C:\ICCMS\lib

set ICC11_LINKER_OPTS=-btext:0x8000 -lms -dinit_sp:0xffbf -dheap_size:0

## Compile with ICC11 DOS

If you purchased the ICC11 C DOS compiler, you can write MindStamp™ programs in C and compile.

1. Open a DOS window in a WINDOWS environment or boot up in DOS.

2. At the DOS prompt, change the directory to iccms.

3. Execute mssetup.

4. Type

compms <filename> <enter>

where <filename> is a C program in the directory mscode (default). Do not put the .c extension on the name. The batch file will do that for you. You can also provide a complete path to <filename> if it is not in the mscode directory.

5. To compile other files during a session, repeat only Step 4. Each time you open a new DOS window, however, you must perform Steps 2 and 3.

If you are in an exclusively DOS environment, Step 1 is automatic, otherwise the procedure is the same.

## PCBUG11 Download Procedure for MindStamp™

Mekatronix provides these instructions for your convenience. Mekatronix, however, does not support, nor recommends, PCBUG11. If you cannot get PCBUG11 to operate properly Mekatronix suggests either obtaining the HSSDL11 and/or the ICC11 IDE system to download files.

You may be able to open a DOS window in WIN95 and download using PCBUG11. However, be aware that PCBUG11 might lockup the COM port, even after you exit the program and even after you exit the DOS window! In such cases, you will have to restart WIN95. For some PC systems PCBUG11 will not function in a DOS window and you must boot up your PC in DOS in order to use PCBUG11.

- Connect C2325, 6-wire serial communications cable, to the Mekatronix MB2325 serial communications board at one end and to the SCI male header on the MTJPRO11 at the other end. The MB2325 is connected via a 25-pin D-connector and cable to your Personal Computer. This setup establishes a link from MindStamp™ to your PC. Check to make sure diode D1 lights with the PC powered up.

- Turn on MindStamp™ power with POWER-ON Switch.

- Switch DOWNLOAD/RUN Switch to DOWNLOAD.

- Press RESET switch. D2 lights when you hold RESET down. If it does not light, reverse the C2325 6-pin connector on the MB2325 circuit board.

- Be sure the "s19" file you wish to download is in the mss19 directory.

- Go to DOS window and prompt.

- Perform the Compiler setup sequence (Section 14.1) if this is first time in this DOS window.

- Type 		    loadms <filename> <COM_PORT_NUMBER><enter>

For example, type

loadms mstst 1 <enter>

and PCBUG11 will download mstst.s19 into MindStamp™'s RAM. The serial communications will be via COM1 ( COM1 is the default if no com-port is specified). The cable from the MB2325 board should be connected to COM1 on your PC in this situation.

- If your program does not produce serial input or read serial input, Steps 3 to 8 can be repeated unchanged after each compile. However, if your program does use serial IO, then you must terminate the DOS window before enabling Hyper Terminal and executing the MindStamp™ program. When finished with your experiment, you must repeat the four-step sequence COMPILE WITH ICC11 described in the previous paragraph in order to be able to compile again. You cannot simply execute step 4 in that sequence.

If you operate in a DOS only environment you may want to use Kermit for serial IO. It is beyond the scope of this manual to go into details on how to use Kermit or other DOS terminal programs. There is no Mekatronix tech support for this type of operation of MindStamp™.

## Execute a MindStamp™ Program

This procedure assumes you have downloaded an “s19” file and that the MindStamp™ power is on.

- Flip DOWNLOAD/RUN switch to RUN.

- Press RESET.

- MindStamp™ will now execute the program you previously downloaded.

NOTE

If your program uses serial IO and you downloaded with PCBUG11 in a DOS window, you must close the DOS window before Hyper Terminal (or whatever terminal simulator you use) can grab COM1 to accept serial input from MindStamp™.

## A WINDOWS Development Environment in DOS

If you have ICC11 DOS and operate in a WINDOWS environment, you can set up an environment similar to IDE, but not quite as convenient. To do this, open your favorite editor in a window and then open your program in the editor. Next, open the HSSDL11, Hyper Terminal and a DOS window simultaneously. After editing programs in your edit window, you can compile programs in the DOS window and then download the resulting “s19” file from the HSSDL11 window. If your MindStamp™ outputs characters, then you can use Hyper Terminal to display those characters. These four windows, collectively, make a reasonably convenient working environment. Remember that Hyper Terminal must be “disconnected” from the COM port before the HSSDL11 can use it and reconnected before it can accept serial input from a MindStamp™ program.

## MindStamp™ Library Code

Before using any of the MindStamp™ library functions in Libsrms the appropriate initialization programs must be executed. The init_<program> for the different library functions are:

init_analog();

init_clockms();

init_motorms();

/* No init routine in muxsenms.c */

init_serial();

init_servos();

/* No init routine in Vectors.c */

The MindStamp™ library functions contain these programs.  A brief description of the MindStamp™ library functions appears below.

analog.c

This library program defines init_analog() and analog((<channel_number>) functions. The function analog(<channel_number>) reads the analog-to-digital converter output of <channel_number> (0  <channel_number>  7 ) .

clockms.c

Generates global variables msec, seconds, minutes, hours, days. A global variable timerms represents a free running counter which counts in milliseconds and is available for user programs. The function wait(<milliseconds>) permits your programs to execute a delay of 0  <milliseconds>  65,535 milliseconds.

motorms.c

MindStamp™ motor drivers. This program generates a periodic pulse whose width produces a duty cycle from 0 to 99 %. The period or repetition rate equals 20ms.

serialms.c

Defines the Mekatronix serial functions. Use <stdio.h> whenever possible, but

the function read_int() here is quite useful. Also, executing init_serial() will

initialize appropriately for <stdio.h> as well.

servoms.c

(Option. Not included in MindStamp™ Distribution Software.)

Driver for servos attached to PA3, PA4, PA5, PA6 and PA7 on the MindStamp™ MTJPRO11 microcontroller circuit board.

Warning!

Do not attach anything to PA4 when running Interactive C.

Vectors.c

Defines all the MC68HC11 interrupt vectors generically. User and future Mekatronix interrupt service routine names must equate to the predefined generic names.

## Generation of MindStamp™’s Library with DOS Script

The distribution code shipped to you will provide libms.a compiled in a compatible version of the compiler you purchased and you do not need to do anything here. This section is for experts who wish to add object files or remake libms.a. You must have source files or object files to remake or add files to a library. Library source files must be purchased separately. If you have source files, just compile them to object files and add. Refer to the ICC11 manual for details.

For the DOS version use the batch file Libmsmake.bat (listed below). For the Windows IDE version, you simply duplicate the process indicated by the batch file.

Contents of Libmsmake.bat:

echo off

echo This script file makes a library file for the MindStamp™ Routines

echo Title		Make MindStamp™ Libraries

echo Programmer	Keith L. Doty

echo Date		May 5, 1999

icc11 -c vectors.c

icc11 -c serialtp.c

icc11 -c motorms.c

icc11 -c analog.c

icc11 -c clockms.c

del ..\lib\libms.a

ilib -a libms.a  serialtp.o

ilib -a libms.a  motorms.o

ilib -a libms.a  analog.o

ilib -a libms.a  vectors.o

ilib -a libms.a  clockms.o

ilib -t libms.a

copy libms.a ..\lib\*.*

To add other libraries you must compile <file.c> to object code <file.o> and then execute

ilib –a libms.a  <file.o>

## Brief Descriptions of MindStamp™ Programs

The “.s19” files for the programs described below can be loaded directly into your MindStamp™

and executed. You can find further descriptions of the files in the “.c” versions. To really understand the details of each program, however, read the “.c” code itself!

# Programming your MindStamp™ with Interactive c

Whether you have the commercial or freeware version of IC, usage is similar, although the commercial version is much more convenient and has more features. The following sections explain how to program MindStamp™ using Interactive C.

## Installing and Using IC

You can download the freeware IC DOS and the IC manual from the Mekatronix web site under Free Stuff without charge. Mekatronix does not support IC DOS and only supplies it as a convenience to MindStamp™ users who wish to minimize costs. A low cost commercial version, IC WINDOWS, may be more cost effective in the long run since it operates in WINDOWS and supplies more features and conveniences.

Before installing IC, first check a few things.

Check List.

- Is your MindStamp™ plugged in and the power on? You can also power MindStamp™ with a NiCd rechargeable battery 6-pack. In such cases, be sure to charge all the batteries and properly connect the battery pack.

- Does your computer run DOS or WINDOWS? Does your computer have a serial port available? There should not be conflicting hardware demands on the serial interrupt. For example, if you are using COM1, COM3 should be left free. Likewise with COM2 and COM4.  We will assume here that you are using COM1.

- Do you have a serial cable coming from your serial port that ends in a 25 pin D connector? You will need this cable to connect to your MB2325 board.

- Install IC

IC WIN95.

Follow installation instructions. Name your root file c:\icms.

IC DOS.

Create a directory on your computer for IC on DOS:

At the DOS prompt		type  		mkdir ic

Change to the IC directory,  	type : 		cd ic

Download IC DOS from Mekatronix web site into c:\ic directory.

- Download the MindStamp™ Distribution Software from the Distributor’s web site as instructed by the Distributor. Copy the distribution files into the icms or ic directory, respectively, for WIN95 or DOS versions.

After completing steps four and five, you are ready to start talking to your MindStamp™.

## Serial Communications with the MindStamp™.

To serially communicate with your MindStamp™ follow the following steps:

- Place your MindStamp™ next to your computer.

- Connect the MB2325 to your serial cable. The light labeled D1 should light.  If this works proceed to step 3.

- Connect the C2325 cable to the MB2325 on the connector labeled J2 with the blue wire toward the J2 label.  Now connect the other end of this cable to the six pin connector labeled SCI on the MTJPRO11 circuit board with the blue wire closer to the center of the board.  Be careful not to offset the cable, all six pins should connect to the six wires of the cable.

## Loading Pcoder with  IC WINDOWS

IC was designed originally to execute on the MIT Handy Board, which has a rather obscure hack on the serial input. IC WINDOWS takes advantage of that hack, but it makes for strange initial loading of the system on a MindStamp™.

- Turn on the MindStamp™ and place the Download/Run toggle switch into Download.

- From Windows execute IC.

IC will give an error message that the board is not responding. Not to worry! The board is not responding because you have not had a chance to load the p-coder and relevant libraries into the MindStamp™.

- Select the Yes button in the IC error Window to configure the board.

The system is configured to enable COM1 for serial communication. If you wish to use another COM port select it on the screen that now appears. To make your selection the default, you will need to change the Port = 1 command in the ic.ini file to Port = n, where n is the COM port number you picked.

- Select the Download Pcode button in the opened IC Window.

IC will now open the Libs directory.

- Select the file TJ-pro.icd file to open.

- Press the red Reset button on the MindStamp™ and select the ok button in the window.

You will observe a flurry of loading activity with the green LEDs on the MB2325 board rapidly flashing, and then another error message! IC  cannot communicate with the p-coder program because the MindStamp™ is in Download mode and not Run mode!

- Place the MindStamp™ Download/Run toggle switch into Run.

- Press the red Reset button.

- Select the Yes button in the IC error Window.

IC will then load the remaining code onto the MindStamp™ and you are ready to go.

As long as you keep power to the MindStamp™ memory, the p-coder will not have to be reloaded. If you disconnect or discharge the batteries or you overwrite memory you will have to repeat the pcoder load procedure described above.

## Loading Pcoder with IC DOS and other Methods

You can load the IC virtual machine pcoder software Pcode_rw.s19 using any of the three downloading methods discussed earlier:

- HSSDL11 (Section 12.5) (Recommended)

- ICC11 IDE (Section 12.4)

- PCBUG11 (Section 14.3) (Not recommended, but free)

- Or you can use the init_tj batch file (uses PCBUG11!) described here.

If you use any of the first three methods, be sure to load the files

twoservo.icb

twoservo.c

motorp.c

using the IC command line before you load any program.

#### Download with init_tj Method

- Place the Download / Run switch to the Download position and turn the MindStamp™ on with the power switch. The red light should come on.  Press the reset switch. When you press the reset switch, the light D2 on the MB2325 should come on. When you release the switch it should go off.  If this happens continue, otherwise check connections, power, and cable orientation.

- To load the IC DOS pcoder into the MindStamp™ with init_tj method you must boot up in DOS, change the directory to c:\ic and then

If you are using COM1			type: 		init_tj 1

If you are using COM2 			type:		init_tj 2

This action will download the brains of IC, the pcode interpeter known as the pcoder. During the transfer you should see the D1 and D2 lights on the MB2325 board blink rapidly. After 30 seconds the screen should prompt you to “press any key to continue” at this point switch the Download / Run switch to Run and press reset.  Then press any key on the keyboard.

- After a few seconds you will see default.lis and other files loading. Next, you will see a C> prompt, indicating that your MindStamp™ is now running IC. On the other hand, if you see synchronizing with board messages, then press Ctrl-C on the keyboard and repeat Step 2, or you may have to start all over.

## Loading and Unloading Programs in IC

Once the pcoder has been loaded you can reboot your system into WINDOWS and work from a DOS window when running IC. This allows you to open an edit window without closing IC as you would have to do in DOS only mode. Of course, if DOS is all you have, you will have to exit IC each time you make a program change. After compiling the new version, you will then exit your editor, execute IC and load the new version for execution. IC WINDOWS lets you avoid this tedious process.

- Now that you have loaded IC, you can load a test program. At the IC prompt, this is the interactive Command Line of IC,

type: 		load mstst.c

- IC loads your program into the MindStamp memory. When you get the C> prompt back, you can run the program by pressing the reset button.

- In a DOS environment you will have to exit IC to edit your program. When finished editing, exit your editor, return to IC by executing ic. You will not have to reload IC after the first time, because it is already in the MindStamp memory. Type the command load <file>.c into the IC command line. If the file is different from the previous file you must first unload the previous file with the command unload <file>.c and then load the new file with load <new_file>.c. In the example, to load msex1.c you first must unload mstst.c:

type: 		unload mstst.c

type: 		load msex1.c

- In a WINDOWS environment, you will not have to exit IC, simply switch back and forth between the DOS window executing IC and your editor window.

## Loading and Unloading Programs in  IC WINDOWS

The procedure for loading and unloading code in IC WINDOWS is essentially the same as for IC DOS, with several major differences. In IC WIN95 you never leave the windows environment. You do not have to boot up in DOS to load the pcoder and then back to WINDOWS to open both a DOS window and an edit window. The browser on IC WIN95 also gives you complete freedom to load <file>.c located in any directory. IC WIN95 implements more C programming functionality than IC DOS and has the added feature that Mekatronix supports it.

## Interactive C

Now that you have learned to run IC on your MindStamp™ and load programs to the MindStamp™, let us develop a greater understanding of the MindStamp™ and how to write software for it.  Interactive C is just that, interactive.  At the C> prompt you can type any IC command in standard ANSI C format. Each line will return a value. For example, type the command:

C> 1+2;

IC returns

<int> 3

This value was calculated on the MindStamp™’s own computer not on your PC.

## MindStamp™ Commands for IC WINDOWS

As the experiments illustrate in subsequent sections, you can attach and control small model aircraft servos and motors to MindStamp™. These motors should not require more than 1 ampere of current under load. If you purchase IC for WINDOWS you will receive the IC distribution software described in this section.

Several commands in IC are not present in ANSI C. These commands have important uses in MindStamp™.  Here we discuss the motor driver commands for modified servos that act as D.C. gearhead motors. IC’s servo command, and not IC’s motor command, controls their actuation. Since this nomenclature and usage may confuse the programmer, Mekatronix provides a routine called motorp.c to control MindStamp™’s motors. The file motorp.c is automatically loaded into the default.lis file when IC is loaded.

Motor Commands

The motorp(<motor>,<speed>)command, where

<motor> equals 0 (PA7) or 1 (PA3) and –100%< <speed> <100%,

allows you to control small, hacked, model aircraft servos attached to PA7 and PA3. The Pulse-Width-Modulated (PWM) signal generated on microcontroller pins PA7 and PA3 is ON from about 1000 (-100%) to 2000 (100%) microseconds and repeats every 20 milliseconds.

Before turning on the motors make sure they are secure and will not move or cause harm to anyone or anything. The motors must be enabled with the command

servo_on(); //I know, confusing!

either in a program or at the IC command line.

The hacked servos will probably spin as soon as you turn them on with the above function, so it is wise to follow with motorp(0,0) and motorp(1,0) to definitely stop the hacked servos attached to microcontroller pins PA7 and PA3, respectively:

servo_on();

motorp(0,0)

motorp(1,0)

After turning the servos on, you can use the commands motorp at will:

motorp(1,100); 	turn on the PA3 motor to full forward, ie 100 percent.

motorp(1,-100); 	turn on the PA3 motor to full backward, ie -100 percent.

motorp(0,50); 	turn on the PA7 motor to 50% forward speed.

Note that a negative number in the speed will cause that motor to go backwards.

IMPORTANT!

ULESS YOU CHANGE THE MOTOR SPEEDS WITH A DIFFERENT motorp() COMMAND, THE MOTORS WILL RUN AT THE LAST SPECIFIED SPEED FOREVER.

WARNING!

SPEED PERCENTAGES ARE TYPICALLY NOT LINEAR! FOR EXAMPLE, 50% WOULD BE HALF SPEED IN A LINEAR SCALE, BUT IT IS NOT. YOU CAN CALIBRATE THE MOTOR SPEED CONTROL OF YOUR MINDSTAMP™ IN SOFTWARE. ALTHOUGH YOU CAN IMPROVE THE SPEED CONTROL CONSIDERABLY THIS WAY, BE WARNED THAT EVEN WITH CALIBRATION THE MOTORS SPEEDS CANNOT BE PRECISELY CONTROLLED.

To attach and precisely control precision motors to MindStamp™, you will need to use the ICC11 WINDOWS support software.

#### Analog Inputs

The 68HC11 supports eight analog input channels. The command analog(<channel_number>), where <channel_number> an integer from 0 to 7, reads these channels (microcontroller pins PE0 to PE7, respectively).

Digital Output Port

Eight digital outputs are connected to a memory-mapped port on the MindStamp™. The poke command may be used to turn them on:

type: 		poke(0x7000,0xff);

The first number in the poke command is the hex address of the port and the second number is the hex byte to be written to that port in this case it is all high.

## Writing your own IC Programs

Programs in IC are close to Standard C, with a few exceptions. For example, there is no need to use the include statement, since all required functions must be loaded with your program. Neither the Standard C input-output functions nor IC’s printf function for an LCD screen display, make sense on your MindStamp™.  Mekatronix resolves this problem with the special code file serial.c to allow the MindStamp™ to send data on the RS-232 bus to your PC. Since IC also uses the serial port, a program that uses the serial port will usually cause communication malfunction. The solution is to exit IC while the MindStamp™ runs a program that requires IO. To enter program mode again you will have to restart IC as outlined previously. This can cause software problems and is a pain. So if you want to do a lot of IO, use ICC11 for WINDOWS instead.

When the power switch is turned off while MindStamp™ is plugged into the charger, power is still supplied to the SRAM to prevent loss of code and data in memory.  This feature has a decided advantage. Programs in storage will be there at power on. If the pcoder was loaded in your last programming session you can conveniently restore it by simply typing IC.

PART TWO

MindStamp ™ Experiments

# experiments WITH icC11 and IC

For clarity of exposition, the electronic interfacing and programming experiments described here use extremely simple circuits and ICC11 and IC programs to drive them. As more applications develop we will include them. If you wish to contribute, please email us a WORD “.doc” file attachment describing your experiment using the standard format used to describe the experiments in this manual. Send the email to tech@mekatronix.com with the subject heading “MindStamp Application”

Interactive C (IC) for WINDOWS provides an easy working environment for carrying out simple experiments and developing programs to test and operate simple electronic projects. You can also use IC DOS or ICC11 for WINDOWS or DOS. If you use ICC11 WINDOWS or ICC11 DOS, you will need to write simple IO programs to realize the Command Line IO of IC. For complex projects that require you to write your own interrupt drivers, you must use ICC11 (WINDOWS or DOS version) because IC does not let you have access to those resources.

The examples in this manual provide ICC11 and IC solutions for the experiments. These examples should help you develop your own projects and solutions.

# Analog input experiment

Circuit Schematic	Figure 4

Objective

Learn about Analog-to-Digital Conversion and how to read an analog channel using MindStamp™.

#### Theory

The MindStamp™ has 8 analog-to-digital (A/D) channels. Each channel converts an analog voltage applied to its input into an 8 bit digital number. 5volts converts to the binary number 111111112  which equals the decimal number 255. 2.5volts converts to 128 or  binary 100000002

and 0volts converts to binary 000000002. Voltages between 5volts and 0volts are converted to the binary number closest to it. For example, 4.99volts also converts to the binary number 111111112.

#### Circuit Description

The switch allows you to connect either 5volts or ground (not at the same time!) to analog channel two (processor pin PE2). With the switch in the 5volt position, the processor will read 255 from channel two of the Analog-to-Digital converter and 0 when the switch grounds the input.

#### ICC11 Program	mscode/analogx1.c

After downloading the “.s19” file for the compiled program, open Terminal in the IDE to receive the input generated by the last line. Each time you want to make a new reading, simply press reset and the program runs again.

//Header files

#include <msbase.h>

#include <stdio.h>

void main(void)

{

//Initialize the Analog-to-Digital-Converter and the serial communications

// interface

init_analog();

init_serial();

//Print the value of the ADC output

printf("\nThe digital value of the analog voltage on channel two equals %d\n", analog(2));

}

#### IC Program

Type on the IC command line the function

analog(2)

IC will return the digital value of the analog voltage on channel two on the command line.

Experiment

Wire up the circuit as shown in the reference figure and execute the programs. Run either the ICC11 or IC program with the switch at 5volts. What is the value displayed? Now switch the analog input to the ground position and run the program again. In IC you can use the up-arrow key to rewrite the previous command in the Command Line. In ICC11 just press the reset button. What is value displayed?

Figure 4.  The switch attached to analog channel-2 (PortE- pin 2, i.e., PE2) allows you to digitize the two possible input voltages of 5volts and 0volts. The 6 wire serial cable connecting the MB2325 and the MTJPRO11 reminds you that all the power and switch connections (Figure 3) must be in place. In this and succeeding diagrams, leaves out the power and switch connections for clarity.

# Reading a Potentiometer with the analog input port

Circuit Schematic

#### Objective

Generate a variety of digital representations of voltages between 0volts and 5volts

Theory

A potentiometer voltage divider circuit splits the input voltage according to the position of the center tap or wiper. This voltage will vary from rail-to-rail, i.e., the voltages across the other two terminals of the potentiometer. The number 255 corresponds to 5 volts and 0 to 0volts.

Circuit Description

A potentiometer wiper connects to channel two (PE2) of the ADC. Five volts and ground (the rail voltages) connect across the full10K ohm resistance of the potentiometer. As the wiper moves the voltage input to PE2 changes continuously from one rail voltage to the other.

#### ICC11 Program	mscode/analogx1.c

After downloading the “.s19” file for the compiled program, open Terminal in the IDE to receive the input generated by the last line. Each time you want to make a new reading, simply press reset and the program runs again.

//Header files

#include <msbase.h>

#include <stdio.h>

void main(void)

{

//Initialize the Analog-to-Digital-Converter and the serial communications

// interface

init_analog();

init_serial();

//Print the value of the ADC output

printf("\nThe digital value of the analog voltage on channel two equals %d\n", analog(2));

}

#### IC Program

Each time you turn the potentiometer shaft 1/8 of a turn type

analog(2)

on the IC command line. The command line will return the value.

Experiment

Wire up the circuit as shown in the reference figure and execute the programs.For each value of the potentiometer setting plot the decimal number output. Use the x-axis for the potentiometer setting and the y-axis for the output number. If you turned the potentiometer exactly 1/8 of a turn each time (this is hard to do), you should get a straight line.

Here is an easier experiment. Mark the potentiometer position for each of the six voltages 0, 1, 2, 3, 4, 5.

Figure 5. This experiment allows you to measure the analog voltage across a potentiometer. The A/D converter of the MTJPRO11 microcontroller generates the digital equivalent of the analog voltage across the potentiometer. The analog input voltage to channel-2 on PE2 changes as you vary the potentiometer position.

# CdS cell light sensor

Circuit Schematic	Figure 6

#### Objective

Explore the characteristics of a photoresistor.

Theory

The resistance of a CdS cell varies with light intensity. This variation in resistance allows you to electronically measure incident light intensity.

Circuit Description

A 10Kohm resistor in series with the variable resistance CdS photoresistor realizes a voltage divider. The divided voltage connects to analog channel two (PE2). The voltage increases as the incident light decreases and increases as the incident light decreases.

#### ICC11 Program	mscode/analogx1.c

After downloading the “.s19” file for the compiled program, open Terminal in the IDE to receive the input generated by the last line. Each time you want to make a new reading, simply press reset and the program runs again.

//Header files

#include <msbase.h>

#include <stdio.h>

void main(void)

{

//Initialize the Analog-to-Digital-Converter and the serial communications

// interface

init_analog();

init_serial();

//Print the value of the ADC output

printf("\nThe digital value of the analog voltage on channel two equals %d\n", analog(2));

}

#### IC Program

Each time you change the intensity of the light incident on the CdS cell type

analog(2)

on the IC command line. The command line will return the value.

Experiment

Wire up the circuit as shown in the reference figure and execute the programs.

- Verify that the resistance of the CdS cell does decrease as more light shines on it.

- Calculate the value of the CdS cell knowing the other resistor and the analog voltage?

- Determine various output voltages for a) a dark room, b) a dark room with a flashlight shining directly on the CdS cell and c) with the lights on in the room.

Figure 6. The Cadmium Sulfide (CdS) cell resistance changes with the intensity of the incident light and is known as a photoresistor. As the light increases the resistance decreases. By measuring the voltage change across the cell using an analog input the circuit measures the relative light intensity. Try different resistors in the voltage divider network and experimentally determine the effect that has on the response of the system to the same light sources.

# Thermistor

Circuit Schematic	Figure 7

Objective

To measure the temperature of various environments electronically.

Theory

The resistance of a thermistor varies with temperature. By placing it in a voltage divider circuit you can measure changes in temperature by measuring the voltage drop across the thermistor.

Circuit Description

A 10Kohm resistor in series with thermistor realizes a voltage divider. The divided voltage connects to analog channel two (PE2) and is proportional to ambient temperature. You may want to put an extension cable on the thermistor to measure temperatures more easily.

#### ICC11 Program	mscode/analogx1.c

After downloading the “.s19” file for the compiled program, open Terminal in the IDE to receive the input generated by the last line. Each time you want to make a new reading, simply press reset and the program runs again.

//Header files

#include <msbase.h>

#include <stdio.h>

void main(void)

{

//Initialize the Analog-to-Digital-Converter and the serial communications

// interface

init_analog();

init_serial();

//Print the value of the ADC output

printf( "\nThe digital value of the analog voltage on channel two equals %d\n", analog(2));

}

#### IC Program

Each time you alter the temperature exposure to the thermistor, type

analog(2)

on the IC command line. The command line will return the value.

Experiment

Wire up the circuit as shown in the reference figure and execute the programs. Hold the thermistor between your thumb and index finger of one hand.

- How does the voltage change as your fingers heat up the thermistor?

- Does the thermistor resistance increase or decrease with temperature?

- Measure the resistance of the thermistor when placed in your freezer, refrigerator, close to a hot light bulb? (Be careful in all these experiments that you do not damage the appliance or subject equipment to damage or harm yourself.)

Thermistor

Figure 7.  A simple voltage divider consisting of a resistor in series with a thermistor allows you to measure temperature.

# LED output

Circuit Schematic	Figure 8

Objective

Generate light under computer control using digital output control.

Theory

Digital outputs permit you to turn external devices off and on. The digital drivers must be able to furnish the voltage and current loads demanded by the device it drives. In this example, the device is a light emitting diode (LED).

Circuit Description

An LED in series with a 470ohm current-limiting resistor connects to one of the eight digital outputs of the MTJPRO11 microcontroller. Note the polarity of the diode. The anode must be connected to the digital output pin (IRLED OUTPUT HEADER pin-1 in the circuit).

#### ICC11 Program	mscode/LEDx.c

After downloading the “.s19” file for the compiled program, open Terminal in the IDE to receive the input generated by the printf() function. Each time you want to toggle the light, simply press any key on the keyboard.

//Header files

#include <msbase.h>

#include <stdio.h>

void main(void)

{

char c;

init_serial();

while(1)

{

*(unsigned char *)(0x7000) = 0x01;

printf("\nType any character to turn LED off\n");

c = get_char();

*(unsigned char *)(0x7000) = 0x00;

printf("\nType any character to turn LED on\n");

c = get_char();

}

}

#### IC Program

To turn on the LED execute type

poke(0x7000,0x01);

on the IC command line.

To turn the LED off Type

poke(0x7000,0x00);

Experiment

Wire up the circuit as shown in the reference figure and execute the programs. On the 40KHz header, circled in red on the diagram, pin-1 must be shorted to the middle pin with a jumper to enable the digital output latch. This experiment can be preformed with both visible and infrared (IR) LEDs. Of course, with IR LEDs you cannot see what is going on. You will need an IR detector to determine if the light is indeed turning on and off.

Figure 8. This circuit alows you to drive an Light Emitting Diode (LED). Note jumper (circled in red) to enable digital output on the IRLED output header. You can just as well use the IRLED OUTPUT HEADER to drive digitial outputs other than LEDs.

# Multiple LED output

Circuit Schematic	Figure 9

Objective

Explore generation of multiple digital outputs.

Theory

Digital outputs permit you to turn external devices off and on. The digital drivers must be able to furnish the voltage and current loads demanded by the device it drives. In this example, the driver simultaneously controls eight light emitting diode (LED).

Circuit Description

The 470 ohm resistors limit the current into the LEDs. You can also use 680 or 330 ohm resistors. An eight-resistor sip network makes wiring this circuit much easier than with individual resistors. On the 40KHz header, circled in red on the diagram, pin-1 must be shorted to the middle pin with a jumper to enable the digital output latch.

#### ICC11 Program	mscode/displayx.c

//Header files

#include <msbase.h>

#include <stdio.h>

void main(void)

{

int i, k;

init_serial();

init_clock();

printf("\nSelect one of the following (Type 1, 2 or 3)\n\n",

"  1. Test LEDs\n"

"  2. Visible Binary Counter\n"

"  3. Moving Dot\n");

i = read_int();

if(i==1) *(unsigned char *)(0x7000) = 0xff;

k = 0;

if(i==2)

while(1)

{

k++;

*(unsigned char *)(0x7000) = k;

wait(100);

}

k = 1;

if(i==3)

while(1)

{

k= k<<1;

*(unsigned char *)(0x7000) = k;

wait(100);

if (k>255) k=1;

wait(100);

}

}

#### IC Program

Test the LEDs by turning them all on by typing

poke(0x7000,0xff);

on the command line.

To flash the lights in binary sequence to create a Visible Binary Counter type

{int i; while(1){i++; poke(0x7000,i); sleep(0.1);}}

on the IC command line.

To create a Moving Dot type

{int i=1; while(1){i=i*2; poke(0x7000,i); sleep(.1); if (i>255) i=1;}}

on the IC command line.

Experiment

Wire up the circuit as shown in the reference figure and execute the programs.

- Modify the Moving Dot  program to move two “dots” at once.

- Invent a moving dot pattern and write a program to realize it

Figure 9. This circuit illustrates how you can use the digital output latch to control eight LEDs. Note the position of the jumper (red circle). Of course, you can attach other two terminal devices to the output latch as long as they do not require more than 5volts and about 10ma each. For more sophisticated use of the digital output latch refer to the manufacturers specification.

# Light meter

Circuit Schematic	Figure 10

Objective

Construct a light meter using a photoresistor as a light sensor and eight LEDs as the light meter reading.

Theory

The processor converts the analog voltage proportional to the ambient light incident on the photoresistor to binary form and uses that binary number to drive the eight LED output lamps to create a binary meter reading. The photoresistor has a slow dynamic response and may take several seconds to reach a stable value after you change the amount of light falling on it by a large amount.

Circuit Description

This circuit combines the two circuits in Figure 6 and Figure 9. On the 40KHz header, circled in red on the diagram, pin-1 must be shorted to the middle pin with a jumper to enable the digital output latch.

#### ICC11 Program	mscode/litemtr.c

// The terminal output will produce the hexadecimal equivalent to the binary

// light meter reading, but the LEDs will display the binary pattern.

//Header files

#include <msbase.h>

#include <stdio.h>

void main(void)

{

int i;

char c;

init_serial();

init_analog();

printf("\n\nType any key to take another reading\n");

while(1)

{

HOME_SCREEN

c = analog(2);

printf("Light meter reading is %x   \b\b\b", c );

*(unsigned char *)(0x7000) = c;

c = get_char();

}

}

#### IC Program

Type

while(1) poke(0x7000,analog(2));

on the command line. IC will return the binary output of the light sensor on the command line and the LEDs will display the binary pattern.

Experiment

Wire up the circuit as shown in the reference figure and execute the programs.

- Expose the photoresistor to different light sources and measure their relative intensity.

- Devise a calibration scheme for measuring different light sources against some unit you devise. For example, use a candle at one foot in a dark room as your basic light unit!

Figure 10. Construct a light meter by attaching a CdS voltage divider network and eight LEDs to the MTJPRO11 microcontroller as shown. Note the position of the jumper (red circle). The value read on PE2, when sent to the IR emitter output register drives the lights according to the binary pattern of ones in the digital output register.

# InfraRed LIGht meter

Circuit Schematic	Figure 11

#### Objective

Develop an Infrared Light Meter.

Theory

IR light, invisible to the human eye, can be detected by this circuit. The detector is sensitive to 940nm infrared light modulated at 40KHz. The modulation allows the circuit to discriminate the modulated IR from other sources of IR such as light bulbs and the sun. Your TV Remote Control transmits 940nm IR at 40KHZ, so this circuit should detect it.

Circuit Description

A Mekatronix analog IR receiver connects to analog channel two (PE2). Make the source cable about 1foot long so you can move it about. This sensor outputs a voltage proportional to the intensity of the 40KHz modulated IR light impinging upon it. The IR receiver has a DC bias, so the minimum reading is around hexidecimal 52 to 57, even when there is no IR light. Connect the LEDs and the 40KHz jumper (circled in red) as shown.

#### ICC11 Program	mscode/irmeter.c

// The terminal output will produce the hexadecimal equivalent to the binary

// light meter reading, but the LEDs will display the binary pattern.

//Header files

#include <msbase.h>

#include <stdio.h>

void main(void)

{

int i;

char c;

init_serial();

init_analog();

CLEAR_SCREEN

printf("\n\nType any key to take another reading\n");

while(1)

{

HOME_SCREEN

c = analog(2);

printf("IR Light meter reading is %x   \b\b\b", c );

*(unsigned char *)(0x7000) = c;

c = get_char();

}

}

#### IC Program

Type

while(1) poke(0x7000,analog(2));

on the command line. IC will return the binary output of the light sensor on the command line and the LEDs will display the binary pattern.

Experiment

Wire up the circuit as shown in the reference figure and execute the programs.

- Point a TV remote at the IR detector at various ranges. Try this experiment at 0, 10, 20, and 30 feet if you can.  Did you notice any significant changes in the measured intensity?

- Point the sensor at bright sunlight and then put it in a closed box or dark room. Did you see any difference?

- Point the IR receiver at a florescent light. Is there a source of 40KHz IR in your florescent light?

Figure 11. The IR receiver detects 40KHZ modulated 940nm IR light. This circuit measures IR now instead of visible light. The visible LEDs provides a binary display of the relative, modulated IR light intensity.

# Measure Intensity of an IR LED Light source

Circuit Schematic		Figure 12

#### Objective

Create an IR light source and measure the intensity at various distances.

Theory

IR light, invisible to the human eye, can be detected by this circuit. The detector is sensitive to 940nm infrared light modulated at 40KHz. The modulation allows the circuit to discriminate the modulated IR from other sources of IR such as light bulbs and the sun.  Since the least significant visible LED in the display is replaced by an IR LED, that bit will always be sent a logic 1 to keep the IR light modulated. Only the other seven display LEDs have meaning (You cannot see the IR one anyway, so it should not be too confusing).

Circuit Description

A Mekatronix analog IR receiver connects to analog channel two (PE2). Make the source cable about 1foot long so you can move it about. This sensor outputs a voltage proportional to the intensity of the 40KHz modulated IR light impinging upon it.  Replace the visible LED on pin-1 (square pad, left-most LED in the diagram) with an IR LED on a 1 ½ foot cable so you can easily move the source around around. The jumper must be moved on the 40KHz header, circled in red on the diagram. The middle pin must be shorted to pin-3 with a jumper to modulate the digital output at 40KHz. This forces the all the LEDs to modulate at 40KHz. The IR LED light will not be visible to your eye. The other LEDs will dim considerably. Connect the other LEDs as shown.

#### ICC11 Program	mscode/irmeter.c

// The terminal output will produce the hexadecimal equivalent to the binary

// light meter reading, but the LEDs will display the binary pattern, except // for the least significant bit.

//Header files

#include <msbase.h>

#include <stdio.h>

void main(void)

{

int i;

char c;

init_serial();

init_analog();

CLEAR_SCREEN

printf("\n\nType any key to take another reading\n");

while(1)

{

HOME_SCREEN

c = analog(2);

printf("IR Light meter reading is %x   \b\b\b", c );

*(unsigned char *)(0x7000) = c | 0x01; // 0x01 keeps the IR LED on

c = get_char();

}

}

#### IC Program

Type

while(1) poke(0x7000,analog(2));

on the command line. IC will return the binary output of the light sensor on the command line and the LEDs will display the binary pattern.

Experiment

Wire up the circuit as shown in the reference figure and execute the programs.

- Point a TV remote at the IR detector at various ranges. Try this experiment at 0, 4, 6, 8, 10, 12, 14, and 16 inches.  Do you notice any significant changes in the measured intensity?

- Make a plot of intensity versus distance

Figure 12. This circuit looks identical to the previous one. However, the visible LED on pin-1 is replaced with an IR LED with 940nm center frequency.

Refer to the Servo Hack application note under FreeStuff on the Mekatronix web site. You can download the hack description free.