/************************************************

NZINGA AUGUSTO IMDL

SUMMER 2000

SETTER

set table

**************************************************/

#include <tjpbase.h>

#include <stdio.h>

#include <hc11.h>

/****************DEFINES*****************************************/

/***********FUNCTION PROTOTYPES**************/

void callibrate(void);			//callibrate the sensors

int aligned(void);				//returns 0 if not aligned

void lft_rtangle_turn(void);	//make a left right angle turn

void left_turn(void);	 		//make a slight 20-70 degree left turn

void right_turn(void);			//make a slight 20-70 degree right turn

void left_juke(void);			//jerk left

void right_juke(void);			//jerk right

void drop_plate(void);			//drop one plate

void drop_fork(void);			//drop fork

void drop_knife(void);			//drop knife

void go_straight(void);			//go straight forward, making any adjustments

void back(void);				//backup

int measure(int);				//pass in edge number and measure length

int edge_detected(void);		//return 1 if an edge is found

int corner_detected(void);		//if a corner is detected return a 1

int calculate(int);				//calculate number of possible places

void set_places(int);			//set places along that edge

void set_rtangle(void);			//set # of pulses required for a 90 turn

void measure_on(void);			//start measuring a side

int measure_off(void);			//stop measuring the side and return the length

void align_wedge(void);			//after the edge is detected, align body to edge follow

void stop(void);					// STOP

/*edge numbers and lengths will be held in an array, where the array index

is the edge number and each field contains the lenght of the indexed edge*/

//motor 2 = left, motor 3 = right

/****************GLOBAL VARIABLES**************************/

int ON_TABLE, FRONT_ON_TABLE, BLACK, value1, value2, value3, value4, drop, xtra;					//value of IR read when on the table

int ir0,ir1, ir2, ir3, ir4, ir5, ir6, ir7, NO_PLATE;

int SIDE1, SIDE2, JUKE=20, TURN, BACK, distance=0, move=21, plate_count=0, go=22, m_on=0, drop=0;

/************************MAIN***********************/

void main(void)

{

init_motorme();

init_serial();

init_clocktjp();

init_analog();

*(unsigned char*) 0x7000 = 0xAA;

*(unsigned char*) 0x6000 = 0x50;

*(unsigned char*) 0x6000 = 0xaa;

ir4 = analog(4);

ir1 = analog(1);

ir2 = analog(2);

ir3 = analog(3);

ir5 = analog(5);

ir6 = analog(6);

ir7 = analog(7);

callibrate();

m_on=0;

go_straight();

lft_rtangle_turn();

measure_on();

go_straight();

SIDE1=measure_off();				//measure side1

printf("\n %d", SIDE1);

lft_rtangle_turn();

measure_on();  					//measure side2

go_straight();

SIDE2=measure_off();

lft_rtangle_turn();

measure_on();

set_places(SIDE1);              //set first side

lft_rtangle_turn();

measure_on();

set_places(SIDE2);              //set second side

lft_rtangle_turn();

measure_on();

set_places(SIDE1);              //set third side

lft_rtangle_turn();

measure_on();

set_places(SIDE2);            //set last side

return;

}

/*****************FUNCTIONS**********************************/

void callibrate()

{

ON_TABLE = (ir1+ir2+ir3)/3 - 6;		//find average of IR values

printf("\n part1");

FRONT_ON_TABLE = (ir4) -6;

NO_PLATE = ir5;

printf("\n part2");

return;

}

/*************************************/

int aligned()

{

if(ir1>=ON_TABLE && ir2<ON_TABLE && ir3>=ON_TABLE)

return 1;

return 0;

}

/**************************************/

void lft_rtangle_turn()

{

int count = 0;

distance = 0;

motorme(RIGHT_MOTOR, 30);		//left turn

motorme(LEFT_MOTOR, -36);

wait(1500);

while(count < 200)

{

ir7 = analog(7);

printf("\n PRINT %d", ir7);

motorme(RIGHT_MOTOR, 30);		//left turn

motorme(LEFT_MOTOR, -36);

if ((ir1>=ON_TABLE && ir3>=ON_TABLE && ir2<ON_TABLE && ir4>=FRONT_ON_TABLE) ||

(ir1>=ON_TABLE && ir3>=ON_TABLE && ir2>=ON_TABLE && ir4>=FRONT_ON_TABLE) ||

(ir4>=FRONT_ON_TABLE && ir1>=ON_TABLE))

{

motorme(RIGHT_MOTOR,0);

motorme(LEFT_MOTOR,0);

break;

}

ir7 = analog(7);

printf("\n PRINT %d", ir7);

ir1 = analog(1);

ir2 = analog(2);

ir3 = analog(3);

ir4 = analog(4);

ir7 = analog(7);

printf("\n PRINT %d", ir7);

count++;

}

return ;

}

/*****************************************/

void left_turn()

{

motorme(RIGHT_MOTOR,move);			//ir1 is off table

motorme(LEFT_MOTOR,-move);

return;

}

/*****************************************/

void right_turn()

{

motorme(RIGHT_MOTOR,-move);			//ir3 is off table

motorme(LEFT_MOTOR,move);

return;

}

/*****************************************/

void left_juke()

{

motorme(RIGHT_MOTOR,move);			//all off table JUKE left

motorme(LEFT_MOTOR,-move);

wait(150);

motorme(RIGHT_MOTOR,0);

motorme(LEFT_MOTOR,0);

motorme(RIGHT_MOTOR,move);

motorme(LEFT_MOTOR,move);

wait(150);

return;

}

/*****************************************/

void right_juke()

{

motorme(RIGHT_MOTOR,-move);			//all on table JUKE right

motorme(LEFT_MOTOR,move);

wait(150);

motorme(RIGHT_MOTOR,0);

motorme(LEFT_MOTOR,0);

motorme(RIGHT_MOTOR,move);

motorme(LEFT_MOTOR,move);

wait(200);

return;

}

/*****************************************/

void drop_plate()

{

motorme(RIGHT_MOTOR,0);

motorme(LEFT_MOTOR,0);

*(unsigned char*) 0x6000 = 0x51;

wait(50);

while(ir6<100)

{

printf("\np IR6 %d", ir6);

*(unsigned char*) 0x6000 = 0x51;

ir6 = analog(6);

}

*(unsigned char*) 0x6000 = 0x50;

drop = 1;

motorme(RIGHT_MOTOR, 25);

motorme(LEFT_MOTOR, 25);

wait(65);

return;

}

/*****************************************/

void drop_fork()

{

motorme(RIGHT_MOTOR,0);

motorme(LEFT_MOTOR,0);

wait(10000);

*(unsigned char*) 0x6000 = 0xd0;       //start motor

wait(200);

*(unsigned char*) 0x6000 = 0x50;

return;

}

/*****************************************/

void drop_knife()

{

motorme(RIGHT_MOTOR,0);

motorme(LEFT_MOTOR,0);

*(unsigned char*) 0x6000 = 0x00;       //drop back

wait(10000);

*(unsigned char*) 0x6000 = 0x80;       //start motor

wait(200);

*(unsigned char*) 0x6000 = 0x50;

return;

}

/*****************************************/

void go_straight()

{ // go straight unitl a corner is detected

if((m_on==1) && (ir7 > 125))

{

distance++;

motorme(RIGHT_MOTOR, 25);

motorme(LEFT_MOTOR, 25);

wait(90);

}

while (corner_detected() == 0)

{

ir1 = analog(1);

ir2 = analog(2);

ir3 = analog(3);

ir5 = analog(5);

ir7 = analog(7);

printf("\nIR1  %d", ir1);

printf("\nIR2  %d", ir2);

printf("\nIR3  %d", ir3);

printf("\nIR4 %d", ir4);

printf("\nIR5  %d", ir5);

printf("\nIR7 %d", ir7);

if((m_on==1) && (ir7 > 125))

{

while(ir7 > 125)

ir7 = analog(7);

distance++;

}

printf("\ndistance %d", distance);

if(ir1>=ON_TABLE && ir3>=ON_TABLE && ir2<ON_TABLE)

{

motorme(RIGHT_MOTOR, go);			//on edge

motorme(LEFT_MOTOR, go);

}

ir4 = analog(4);

if(ir4 <FRONT_ON_TABLE) continue;

if ((ir1>=ON_TABLE && ir2>=ON_TABLE && ir3>=ON_TABLE)||

(ir1>=ON_TABLE && ir2<ON_TABLE && ir3<ON_TABLE))

{

right_juke();

if((m_on==1) && (ir7 > 125))

{

while(ir7 > 125)

ir7 = analog(7);

distance++;

}

}

ir4 = analog(4);

if(ir4 <FRONT_ON_TABLE) continue;

if ((ir1<ON_TABLE && ir2<ON_TABLE && ir3<ON_TABLE)||

(ir1<=ON_TABLE && ir2<=ON_TABLE && ir3>=ON_TABLE))

{

left_juke();

if((m_on==1) && (ir7 > 125))

{

while(ir7 > 125)

ir7 = analog(7);

distance++;

}

}

ir4 = analog(4);

if(ir4 <FRONT_ON_TABLE) continue;

if (ir1<ON_TABLE && ir2>=ON_TABLE && ir3>=ON_TABLE)

{

left_turn();

}

ir4 = analog(4);

if(ir4 <FRONT_ON_TABLE) continue;

if ((ir1>=ON_TABLE && ir2>=ON_TABLE && ir3<ON_TABLE) ||

(ir1>=ON_TABLE && ir2<=ON_TABLE && ir3<=ON_TABLE))

{

right_turn();

}

}  //end while

return;

}

/*****************************************/

void back()

{

int k;

unsigned rand;

rand = TCNT;

if (rand & 0x0001)//change to if (ir[2]<ON_TABLE || IR[3]<ON_TABLE)

{/* back up turning right */

motorme(0,-60);

motorme(1,-30);

wait(BACK);

}

else

{/* back up turning left */

motorme(0,-30);

motorme(1,-60);

wait(BACK);

}

}

/*****************************************/

int edge_detected()

{

/* An edge is detected, 1 is returned if the edge is on the left, or

2 is returned if its on the right.  Zero is returned if no edge */

ir4 = analog(4);

ir5 = analog(5);

if (ir4 < ON_TABLE)

return 1;

if (ir5 < ON_TABLE || ir1 < ON_TABLE)

return 2;

return 0;

}

/*****************************************/

int corner_detected()

{

/* An corner is detected, return 1 */

ir4 = analog(4);

if (ir4 < FRONT_ON_TABLE)

return 1;

return 0;

}

/*****************************************/

int calculate(int edge)

{

if(edge < 7) return 0;

else if(edge >= 7 && edge <= 10) return 2;

else if(edge > 10 && edge <= 14) return 2;

else return 4;

}

/*****************************************/

void set_places(int edge)

{

int d=8;				//plate drop vaiable

int count, upper_limit, calc=1;

//calc = calculate(edge);

//value1 = edge/calc;

value1 = edge/2;

printf("\n this is dist %d", distance);

printf("\n this is value1 %d", value1);

if((m_on==1) && (ir7 > 125))

{

while(ir7 > 125)

ir7 = analog(7);

distance++;

}

while(corner_detected() == 0)

{

ir1 = analog(1);

ir2 = analog(2);

ir3 = analog(3);

ir4 = analog(4);

ir5 = analog(5);

ir6 = analog(6);

ir7 = analog(7);

printf("\nIR1  %d", ir1);

printf("\nIR2  %d", ir2);

printf("\nIR3  %d", ir3);

printf("\nIR4  %d", ir4);

printf("\nIR5  %d", ir5);

if((m_on==1) && (ir7 > 125))

{

while(ir7 > 125)

ir7 = analog(7);

distance++;

}

if(distance == value1)

{

value1 = -1;

drop_plate();

}

if((drop == 1) && ir5>(NO_PLATE+6))

{

drop_fork();

drop = 2;

}

if((drop == 2) && ir5<= (NO_PLATE+3))

{

drop_knife();

drop = 0;

}

if(ir1>=ON_TABLE && ir3>=ON_TABLE && ir2<ON_TABLE)

{

motorme(RIGHT_MOTOR, go);			//on edge

motorme(LEFT_MOTOR, go);

}

if ((ir1>=ON_TABLE && ir2>=ON_TABLE && ir3>=ON_TABLE) ||

(ir1>=ON_TABLE && ir2<ON_TABLE && ir3<ON_TABLE))

{

motorme(RIGHT_MOTOR,-move);			//all on table JUKE right

motorme(LEFT_MOTOR,move);

wait(150);

motorme(RIGHT_MOTOR,0);

motorme(LEFT_MOTOR,0);

motorme(RIGHT_MOTOR,move);

motorme(LEFT_MOTOR,move);

wait(200);

}

if (ir1<ON_TABLE && ir2<ON_TABLE && ir3<ON_TABLE)

{

motorme(RIGHT_MOTOR,move);			//all off table JUKE left

motorme(LEFT_MOTOR,-move);

wait(150);

motorme(RIGHT_MOTOR,0);

motorme(LEFT_MOTOR,0);

motorme(RIGHT_MOTOR,move);

motorme(LEFT_MOTOR,move);

wait(150);

if((m_on==1) && (ir7 > 125))

{

while(ir7 > 125)

ir7 = analog(7);

distance++;

}

}

if (ir1<ON_TABLE && ir2>=ON_TABLE && ir3>=ON_TABLE)

{

motorme(RIGHT_MOTOR,move);			//ir1 is off table

motorme(LEFT_MOTOR,-move);

}

/*   if((distance != 0) && (distance != edge) && (distance != (calc*value1))  //drop plate

&& (distance== value1))

{

printf("\n this is dist %d", distance);

printf("\n this is value1 %d", value1);

drop_plate();

value1 = 100;                  //added for demo day

}

if(distance == value1)

{

drop_plate();

value1 == -1;

}

if((drop == 1) && distance==d)

{

drop_fork();

drop = 0;

}  */

if (ir1>=ON_TABLE && ir2>=ON_TABLE && ir3<ON_TABLE)

{

motorme(RIGHT_MOTOR,-move);			//ir3 is off table

motorme(LEFT_MOTOR,move);

/*  if((m_on==1) && (ir7 > 125))

{

while(ir7 > 125)

ir7 = analog(7);

distance++;

} */

}

}

motorme(RIGHT_MOTOR,0);

motorme(LEFT_MOTOR,0);

return ;

}

/*****************************************/

void measure_on()

{

m_on = 1;

distance = 0;

return ;

}

/*****************************************/

int measure_off()

{

m_on = 0;

return distance;

}

/*****************************************/

void align_wedge(void)

{

motorme(RIGHT_MOTOR,20);			//just GO SLOW

motorme(LEFT_MOTOR, 20);

if(ir4<FRONT_ON_TABLE && ir5<FRONT_ON_TABLE)

{ // ir1 is off the table

go_straight();

lft_rtangle_turn();

return;

}

if(ir4<FRONT_ON_TABLE && ir5<FRONT_ON_TABLE)

{ // both front sensors are off the table, back up and turn left

motorme(RIGHT_MOTOR, -30);

motorme(LEFT_MOTOR, -30);

wait(700);

motorme(RIGHT_MOTOR, 30);			//turn

motorme(LEFT_MOTOR, -30);

wait(1000);

go_straight();

lft_rtangle_turn();

return;

}

if(ir4<FRONT_ON_TABLE) //if front left is hanging off table back up make a sharp turn left

{

motorme(RIGHT_MOTOR, -30);

motorme(LEFT_MOTOR, -30);

wait(700);

motorme(RIGHT_MOTOR, 25);			//turn

motorme(LEFT_MOTOR, -30);

wait(2000);

}

return;

}

/*****************************************/

void stop(void)

{

motorme(RIGHT_MOTOR, 0);

motorme(LEFT_MOTOR, 0);

return;

}