Summer 2004                                               EEL 5666

FUPA

The Garbage Can Kicking-Over Robot

Jeff Cohen

7/30/04

## Table of Contents

Abstract…………………………………………………………………………………....3

Executive Summary…………………………………………………………………….…4

Introduction………………………………………………………………………….…….5

Integrated System…………………………………………………………………….……5

Power……………………………………………………................……………………...8

Actuation…………………………………………………………………………………..8

Sensors…………………………………………………………………………………….9

Behaviors………………………………………………………………………………...12

Components…………………………………………………………...............................13

Conclusion………………………………………………………………………..……...14

Documentation……………………………………………………………………….......14

Appendices……………………………………………………………………………….14

## Abstract

FUPA is an autonomous robot.  FUPA simulates a fifth grader biking home from school on Fridays which is both trash and recycling day.  FUPA will follow the sidewalk and be on the lookout for empty cans along the way.  When a can is found, the color will be analyzed.  If there is a red recycling bin, FUPA will knock over the can with an arm that swings out from the side.  FUPA will then move on to the next house, and this cycle will continue for ever.  When a can is found, the correct colored LED will also illuminate on an LED bar.  FUPA will also have an obstacle destruction feature from an IR sensor, and obstacle detection from two bump switches.

## Executive Summary

FUPA is a 2-WD autonomous robot.  It is based on an STK500 board with an ATMega32 for its processor.  FUPA follows a high contrast line using two Hamamatsu P5587 IR detectors.  When a third Hamamatsu P5587 IR detector detects a line to the right of the track, a CMU cam takes a picture.  If there is a red can there, FUPA will swing its arm out, and knock it over.  Also, a red LED will illuminate.  However, if the can was blue or green, the arm will not swing out, but the corresponding colored LED will light up.

If along the way, FUPA should detect an obstacle ahead (seen by a Sharp GP2D12 IR sensor), then FUPA enters ‘destroy mode’.  In destroy mode, FUPA speeds off of the track, and knocks the obstacle out of the way before returning to the track at the point FUPA left off at.

If for some reason the obstacle detection does not work, FUPA is also equipped with two bump switches which when activated will cause FUPA to cease all operations.  And of course, all of the different operations include LCD feedback so you can see what FUPA is currently doing.

Introduction

Fridays used to be the best day of the week in fifth grade.  This was both garbage day and recycling day.  When school would let out, a fun game was to knock over all the recycling bins as one biked home from school.  This is exactly what FUPA will do.  This paper describes a robot that will follow a high contrast “sidewalk” made of electrical tape.  When the robot arrives at a house with garbage cans, using a CMU-CAM for color detection, it will knock over all red cans with a side-swinging arm.  If any obstacle is detected in FUPA’s path, FUPA will swing around and knock it out of the way.

This report will detail all of FUPA’s electrical and mechanical features and characteristics.  First, the platform will be presented, followed by a detailed description about each of FUPA’s subsystems.

## Integrated System

FUPA will have an STK500 as a platform, powered by an Atmega32 chip.  Figure 1 (below) is a block diagram of FUPA’s components.  Figure 2 (below) is a flow chart of FUPA’s software operations.

LCD

STK500

ATMega32

2 High Torque Servos

Two 7.8V NiMH Batteries

Power Board Consisting of Three Voltage Regulators

1

2

3

Servo for Swinging Arm

CMU Cam

Bump Switches for Obstacle Detection

One Sharp GP2D12 for Obstacle Destruction

Two P5587 for Line Tracking

One P5587 for Tab Detection

Fig. 1:  Block diagram of FUPA

FUPA’s entire chassis is made from 1/8” birch plywood, and is painted pink with PPG paint.  The top is removable to allow access to the wires and I/O pins.  In addition, the front is on hinges so the STK500 can be easily removed.  See Fig. 3.  The LCD screen is the only device attached to the power pins on the STK500 board.

Initialize Servos, Camera, UART, A/D, and Put Arm at home position.

Poll Line Track Sensors, Tab Sensor, and A/D Value

Configure Bump Switches for Interrupt 0

Obstacle

Position to Knock Over Can and Return to Line

Tab

Get Color

Light Up Correct LED

If Red, Swing Out Arm

Fig. 2: Flow chart for FUPA’s software

Fig. 3: Picture of FUPA’s easy access panels

Power

FUPA is powered by two rechargeable 7.2V, 3300mAh, NiMH batteries.  The board is powered directly from these batteries.  The camera gets its power from a 7V regulator, the servos controlling the wheels get there power from a 5V regulator, and the arm servo and sensors get there power from a second 5Vregulator.  See Fig. 4.  All regulators were fitted with heat fins to help dissipate heat.

Fig. 4: Picture of FUPA’s voltage regulators and heat fins

Actuation

FUPA will have two wheels which will each be powered by a hacked high-torque Futaba servo (S03 TXF 2BB).  Two ¼” wood spheres are attached to a dowel rod to act as the front two stabilizers (See Fig. 5).  The swinging arm will also be powered with a servo.  It will use an un-hacked Futaba S03N 2BB.

Fig. 5: Picture of FUPA’s servos and pseudo-wheels

## Sensors

Since FUPA needs to follow a sidewalk, electrical tape will be used to represent a sidewalk.  FUPA will follow this line using Hamamatsu P5587 IR detectors.  Tabs, noting where FUPA should analyze a can’s color, will also be detected using Hamamatsu P5587 IR detectors. Bump switches will be used for collision detection, and a Sharp GP2D12 IR sensor will be used for obstacle avoidance.  A CMU-CAM is what will differentiate the color of different cans.

Hamamatsu P5587 IR Detectors

Two of these sensors are used for line tracking, and one is used for tab detection.  Using Will Dubel’s circuit (Fig. 6), these sensors emit a ‘high’ value if it reads black or nothing at all.  They emit a ‘low’ value when they are on a white surface.  Through

Fig. 6: Line tracking schematic

experimentation, it was determined that they work best for me 1/8” off of the ground and pointed downwards at a 40° from the horizon.  FUPA’s actual line tracking circuit board and tab sensor can be seen in Fig. 5.

Bump Switches

The bump switches are just two standard SPST switches.  One of each switch’s pins is attached to ground, and the other pin is attached to an interrupt pin.  When either of these pins go ‘low’, the robot shuts down immediately.  The bump switches can be seen in Fig. 3.

Sharp GP2D12 IR Sensor

The Sharp GP2D12 IR sensor is used for obstacle destruction, and can detect ranges from 10cm to 80cm.  It is constantly returning a value to the STK500.  If nothing is in FUPA’s path, then the sensor will return a zero.  However, when an object enters FUPA’s path, the sensor returns a value which increases as the objects distance from FUPA decreases.  When the sensor returns a value of $B0 or higher, FUPA will enter ‘destroy mode’.  This sensor can be seen above the front door in Fig. 3.

CMUcam

The CMUcam uses the UART to communicate with my ATmega32 chip at a speed of 38,400 baud.  This is the fastest rate that can be used while still maintaining a 0.2% error rate.  The settings of the camera changed from default mode are enabling Polling Mode and Raw Data Mode.  Polling Mode makes the processor run faster.  It makes the camera only send 1 packet of information when requested vs. a constant 17 f.p.s.  Raw Data Mode makes the camera transfer data bytes instead of ASCII characters.  This saves the user from having to convert ASCII to useable data.

To use the CMUcam successfully, the camera requires a lot of white light.  Pictures taken in my bedroom come out red.  To fix this problem, ten, 16,000 m.c.d., 5 mm, white LEDs were ordered.  After adding the lights, pictures were of very good quality.  A sample picture can be seen in Fig. 7.

Fig. 7: A sample picture taken by the CMUcam with the white LEDs as background light

A second problem with the camera is that the picture is not centered.  It is off by around 10 degrees.  I had to tilt the camera when installed to account for this error.  The tilted camera and the white LEDs can be seen in Fig. 3.

The voltage regulator on the camera gets very hot.  To combat this problem, I installed a heat fin purchased from Radio shack.

Behaviors

Line Tracking

FUPA follows a high contrast line using two IR sensors

Destroy Mode

FUPA enters destroy mode when an obstacle is detected in its path.  When FUPA enters this mode, the following steps are taken:

1) Turn off tab sensor

2) Move ahead for 1.5 seconds

3) Turn 45 degrees left and go straight for 0.5 seconds

4) Swing out arm

5) Reverse for 1.5 seconds

6) Turn right until the line is reacquired

7) Turn on tab sensor and continue to line track

In destroy mode, the obstacle is knocked out of the way, so FUPA can continue on its original path.

Camera Mode

Camera mode is entered when a tab is detected with the IR sensor.  In this mode, FUPA is line tracking while snapping a picture to determine which future action to take.

Components

| Quantity | Part | Price | Source |
| --- | --- | --- | --- |
| 2 | Wheels | 6.00 | Mark III |
| 2 | High Torque Futaba Servos | 21.00 | Mark III |
| 1 | Futaba Servo | 10.50 | Mark III |
| 1 | Sharp GP2D120 IR Sensor | 8.50 | Mark III |
| 1 | Sharp GP2D120 IR Sensor Cable | 1.10 | Mark III |
| 2 | Bump Switches | 2.39 | Radio Shack |
| 8 | White LEDs and Holders | 15.00 | E-Bay |
| 3 | DB9 Connectors | 4.80 | Radio Shack |
| 2 | Breadboards | 4.00 | Radio Shack |
| 4 | Heat Sinks | 6.00 | Radio Shack |
| 2 | Female Battery Terminals | 8.00 | Radio Shack |
| 1 | Variable Voltage Regulator | 2.50 | Radio Shack |
| 1 | Paint and Clear Coat | 11.00 | Lowe's |
| 2 | Hinges | 2.00 | Lowe's |
| 1 | Latch | 3.50 | Lowe's |
| 1 | Velcro | 2.50 | Lowe's |
| 1 | STK500 | 80.00 | Digikey |
| 1 | ATMega32 | 9.00 | Digikey |
| 1 | CMUcam | 70.00 | Used |
| 3 | Hamamatsu P5587 IRs | 12.00 | Digikey |
| 1 | Potentiometer | 3.00 | Radio Shack |
| 1 | Power Jack | 5.00 | Radio Shack |
| 3 | Red, Green, Blue LEDs | 6.50 | Radio Shack |
| 3 | Various Sized Screws | 4.50 | Lowe's |
| 2 | Goop | 8.00 | Lowe's |
| 1 | Solder | 4.00 | Radio Shack |
| 2 | LED Holders | 2.00 | Radio Shack |
| 1 | LCD Screen | 8.00 | Used |
| 1 | Shipping Charges | 7.00 |  |
| Total |  | 327.79 |  |

Wire, wood, and wood glue were provided by the lab.

Conclusion

FUPA was very successful.  It works as expected, and I even added the destroy mode feature and the LED bar.  The only thing that I wish I had done different was using sonar instead of tabs/IR to determine where a can is.  Overall this robot was very time consuming, eating well over 300 hours of time to get everything working 100%.  But, after completion, I have never had FUPA give me a false color reading or veer off of the track

## Documentations

1) Will Dubel’s line tracking paper

2) Bryan Arkin’s ‘letter’ macros

## Appendices

Final Code

;**************************

;*   Set VTarget = 5.1V   *

;*   Set ARef = 4.9V      *

;**************************

;Jeffrey S. Cohen

;v10 with obstacle knockdown

;7/30/2004

;EEL5666C

.INCLUDE "m32def.inc"

.CSEG

;********************

;*   Reset Vector   *

;********************

.ORG $000

RJMP 	Reset

;**************************

;*   Interrupt 0 Vector   *

;**************************

;Bump switch interrupt

.ORG $002

RJMP	ShutDown

;*****************

;*   Constants   *

;*****************

;Servo Register Renames

.EQU	RSERVO_H	=OCR1BH

.EQU	RSERVO_L	=OCR1BL

.EQU	LSERVO_H	=OCR1AH

.EQU	LSERVO_L	=OCR1AL

.EQU	ARM_SERVO	=OCR0

;Registers

.DEF 	Temp 		=r16 			; Temporary Reg 1

.DEF 	Temp2 		=r17 			; Temporary Reg 2

.DEF 	Temp3 		=r18			; Temporary Reg3

.DEF 	PrevIR 		=r19 			; Previous IR reading

.DEF	ADCCounter	=r20			; Counter for # of times ADC reports back a close value

.DEF	LCDReg		=r21

.DEF 	Delay1 		=r29 			; Delay Reg 1

.DEF 	Delay2 		=r30 			; Delay Reg 2

.DEF	Delay3		=r31			; Delay Reg 3

;Ports

.EQU	LCD_PORT		=PORTC

.EQU	LCD_DDR			=DDRC

;Constant Values

.EQU	ADC_Distance	=$B0		; Distance where ADC reads as object too close

.EQU	Space			=$20

.EQU	CR				=$0D

;**************

;*   Macros   *

;**************

;Prints Letters to LCD display

.macro letter

LDI 	LCDReg,@0

OUT 	LCD_PORT,LCDReg

SBI 	LCD_PORT,5

RCALL 	Latch

RCALL 	Delay5ms

LDI 	LCDReg,@1

OUT 	LCD_PORT,LCDReg

SBI 	LCD_PORT,5

RCALL 	Latch

RCALL 	Delay5ms

.endmacro

;************

;*   Main   *

;************

.ORG $60

Reset:

RGB_VALUE:	.DB 0

S1:			.DB 0

S2:			.DB 0

R_VALUE:	.DB 0

G_VALUE:	.DB 0

B_VALUE:	.DB 0

Rdev:		.DB 0

Gdev:		.DB 0

Bdev:		.DB 0

S9:			.DB 0

LDI 	Temp,low(RAMEND) 	; Set low stackptr

OUT 	SPL,Temp

LDI 	Temp,high(RAMEND) 	; Set high stackptr

OUT 	SPH,Temp

RCALL	PortInit			; Initializes port directions

SBI 	PortD,2				; Pull up bump switch pin

RCALL	LCDInit				; Initializes LCD Screen

RCALL	UART_Init			; Initialize UART

RCALL	Camera_Init			; Initialize Camera

RCALL	BufferFlush			; Empty out all junk data in buffer sent from camera during initialization

RCALL	PWMInit_Wheels		; Initializes PWM for Wheels

RCALL	ADC_Init			; Initializes ADC for IR obstacle detection

SEI							; Enable Interrupts

StartAgain:

CBI		PortB,0				; Turn off colored LEDs

CBI		PortB,1

CBI		PortB,4

RCALL	PWMInit_Arm			; Initializes PWM for the arm servo

RCALL	LineTrack

;*****************

;*   Port Init   *

;*****************

PortInit:

CLR	Temp

OUT DDRA,Temp

; INPUT REGISTER

; Pin0 = Sharp IR obstacle avoidance

; Pin1 =

; Pin2 =

; Pin3 =

; Pin4 =

; Pin5 = IR input (Tab Detector)

; Pin6 = IR input (Left)

; Pin7 = IR input (Right)

SER	Temp

OUT DDRB,Temp

; OUTPUT REGISTER

; Pin0 = Red LED control pin

; Pin1 = Green LED control pin

; Pin2 =

; Pin3 = Servo PWM (Arm)

; Pin4 = Blue LED control pin

; Pin5 =

; Pin6 = White LED control pin

; Pin7 =

SER Temp

OUT LCD_DDR,Temp

; OUTPUT REGISTER

; Pin0 = LCD DB4

; Pin1 = LCD DB5

; Pin2 = LCD DB6

; Pin3 = LCD DB7

; Pin4 = LCD enable

; Pin5 = LCD register select

; Pin6 =

; Pin7 =

LDI Temp, 0b11111010

OUT DDRD,Temp

; Pin0 = UART input (Receive)

; Pin1 = UART output (Transmit)

; Pin2 = Bump switches (INT0)

; Pin3 =

; Pin4 = Servo PWM (Right)

; Pin5 = Servo PWM (Left)

; Pin6 =

; Pin7 =

LDI Temp, 0b00000000	;Configure interrupt

OUT MCUCR,Temp

LDI	Temp, 0b01000000	; Enable bump switch interrupt

OUT GICR, Temp

RET

;**************************

;*   LCD Initialization   *

;**************************

;** PC0 = DB4 (LCD pin7)

;** PC1 = DB5 (LCD pin8)

;** PC2 = DB6 (LCD pin9)

;** PC3 = DB7 (LCD pin10)

;** PC4 = E (LCD pin6)

;** PC5 = RS (LCD pin4)

;** GND = VSS (LCD pin1)

;** GND = R/W (LCD pin5)

;** VTG = VDD (LCD pin2)

LCDInit:

;Power-On 15ms Delay

RCALL 	Delay5ms

RCALL 	Delay5ms

RCALL 	Delay5ms

;Begin 4-Bit Enable

LDI 	Temp, 3

OUT 	LCD_PORT,Temp

RCALL 	Latch

RCALL 	Delay5ms

RCALL 	Latch

RCALL 	Delay1p5ms

RCALL 	Latch

RCALL 	Delay5ms

LDI		Temp, 2

OUT 	LCD_PORT,Temp

RCALL 	Latch

RCALL 	Delay1p5ms

;Begin 2-Line Enable

RCALL 	Latch

RCALL 	Delay1p5ms

LDI		Temp, 8

OUT 	LCD_PORT,Temp

RCALL 	Latch

RCALL 	Delay1p5ms

;Display on, Cursor on, Blink on

LDI		Temp, 0

OUT 	LCD_PORT,Temp

RCALL 	Latch

RCALL 	Delay1p5ms

LDI		Temp, 15

OUT 	LCD_PORT,Temp

RCALL 	Latch

RCALL 	Delay1p5ms

;Clear screen, cursor home

LDI		Temp, 0

OUT 	LCD_PORT,Temp

RCALL 	Latch

RCALL 	Delay1p5ms

LDI		Temp, 1

OUT 	LCD_PORT,Temp

RCALL 	Latch

RCALL 	Delay5ms

;Initialization Complete

RET

;*************************

;*   PWM Setup (Wheels)  *

;*************************

;1/8MHz = .125us per cycle

;using 64 prescaler, 1 increment of TCNT = 8us

;

;Set potentiometer so the wheel does not turn

; if the pulse is high for 1.5ms

;

;1.5ms = 187.50

;

;187.50/2=93.75

;===================================

;T=20ms, then .5T=10ms 10ms/8us=1250 ($04E2)

;

;     ^     <-- Top = 1250

;   /   \

; /       \ <-- Bottom = 0

;

;1250-93.75=1156.25 ~1156 rise

;ICR1A <-- Top

;OCR1A  <-- 1156 ($0484)

;===============================================

;Full Clockwise	(Right Side)		: 1172 0x491

;

;Neutral							: 1156 0x484

;

;Full C. Clockwise	(Left Side)		: 1141 0x477

;===============================================

PWMInit_Wheels:

LDI Temp,0b11110000

; bits7,6	=1,1 Set up and clear down (OC1A)

;     5,4	=1,1 Set up and clear down (OC1B)

; bits3,2	=0,0 Force output compare OFF A and B

; bits1,0	=0,0 PWM phase and Freq correct

OUT TCCR1A,Temp

LDI Temp,0b00010011	; bit7		=0 IC noise canceler OFF

; bit6 	   	=0 Not used in our mode

; bit5		=0 Reserved

; bits4,3	=1,0 PWM phase and Freq correct

; bits2,1,0	=Clock Select Prescaler

;			 0,1,1 : (1/8Mhz)*64=8us

OUT TCCR1B, Temp

; Sets Top

LDI Temp,0x04

LDI Temp2,0xE2

OUT ICR1H,Temp

OUT ICR1L,Temp2

; Sets Pulse On Period

LDI Temp,0x04

LDI Temp2, 0x84

OUT RSERVO_H,Temp

OUT LSERVO_H,Temp

OUT RSERVO_L,Temp2

OUT LSERVO_L,Temp2

; Start TCNT's at $00

LDI Temp, 0

OUT TCNT1H, Temp

OUT TCNT1L, Temp

RET

;**********************

;*   PWM Setup (Arm)  *

;**********************

;1/8MHz = .125us per cycle

;using 256 prescaler, 1 increment of TCNT = 32us

;

;1.0ms = 31.25

;2.0ms = 62.50

;

;One period = (255+256)*32us = 16.352ms

;

;T=16.35ms, then .5T=8.175ms

;================================================

;8.175ms-0.5ms = 7.675ms

;

;7.675ms / 32us = 239.84 ~ 240 (1ms pulse = 240)

;================================================

;8.175ms-1.0ms = 7.175ms

;

;7.175ms / 32us = 224.22 ~ 224 (2ms pulse = 224)

;================================================

PWMInit_Arm:

; Sets OCR0

;219=swing out

;245=home position

LDI Temp,245		; Set arm to home position on startup

OUT ARM_SERVO,Temp

; Start TCNT's at $00

LDI Temp, 0

OUT TCNT0, Temp

LDI Temp,0b01110100

; bit7		=0   Force output compare off

; bits6,3	=1,0 CTC

; bits5,4	=1,1 Set OC0 on upcount, clear on downcount

;			=0,0 DISCONNECTED

; bits2,1,0	=1,0,0 Set 256 prescalar

OUT TCCR0,Temp

RET

;*******************

;*   ADC IR Setup  *

;*******************

ADC_Init:

LDI Temp,0b00100000		; bits7,6		=0,0 		Use AREF as reference

; bit5			=1 	 		Left adjust

; bits4,3,2,1,0	=0,0,0,0,0	Select ADC0

OUT ADMUX,Temp

LDI Temp,0b11100110		; bit7		=1 	   Enable ADC

; bit6		=1 	   Start conversions

; bit5		=1 	   Auto trigger enabled

; bit4		=0 	   Interrupt flag (not important)

; bit3		=0 	   ADC interrupt disable

; bits2,1,0	=1,1,0 prescaler=64

OUT ADCSR,Temp

LDI	TEMP,(SFIOR&0b00011111)	; Set ADC to free running mode

OUT	SFIOR,Temp

CLR	ADCCounter				; Clear ADC counter

RET

;******************

;*   UART Setup   *

;******************

UART_Init:

LDI		Temp,12				; Selects baud rate (8 LSBs) 38.4k bps

OUT		UBRRL,Temp			; Clock=8MHz

LDI		Temp,0b00000000		; Bit7 = 0					Must be 0 when writing to UBRRH

; Bits6,5,4 = 0,0,0			Reserved

; Bits3,2,1,0 = 0,0,0,0		Selects baud rate (4 MSBs)

OUT		UBRRH,Temp

LDI		Temp,0b10000110		; Bit7 = 1		Selects proper register

; Bit6 = 0		Asynchronous mode

; Bits5,4 = 0,0	Parity disabled

; Bit3 = 0		Select 1 stop-bit

; Bits2,1 = 1,1	Select 8-bit frame

; Bit0 = 0		Clock polarity....set to 0 if asynchronous

OUT		UCSRC,Temp

LDI		Temp,0b00011000		; Bit7 = 0	Rx complete interrupt disable

; Bit6 = 0	Tx complete interrupt disable

; Bit5 = 0	Data register empty interrupt disable

; Bit4 = 1	Rx enable

; Bit3 = 1	Tx enable

; Bit2 = 0	Select 8-bit frame

; Bit1 = 0	Rx 9th bit disabled

; Bit0 = 0	Tx 9th bit disabled

OUT		UCSRB,Temp

RET

;********************

;*   Camera Setup   *

;********************

Camera_Init:

SBI		PortB,6				; Turn on white LEDs for picture

RCALL	DelayS

LDI		Temp,'R'			; Reset camera

RCALL	Send

LDI		Temp,'S'

RCALL	Send

LDI		Temp,CR

RCALL	Send

RCALL	DelayL

LDI		Temp,'P'			; Enable polling mode

RCALL	Send

LDI		Temp,'M'

RCALL	Send

LDI		Temp,' '

RCALL	Send

LDI		Temp,'1'

RCALL	Send

LDI		Temp,CR

RCALL	Send

RCALL	DelayS

LDI		Temp,'R'			; Enable raw data output

RCALL	Send				; Disable 'ACK'/'NAK' responses

LDI		Temp,'M'

RCALL	Send

LDI		Temp,' '

RCALL	Send

LDI		Temp,'3'

RCALL	Send

LDI		Temp,CR

RCALL	Send

RCALL	DelayS

RET

;*****************************************

;*   Flush out old received characters   *

;*****************************************

BufferFlush:

SBIS	UCSRA,RXC

RET

IN		Temp,UDR

RJMP	BufferFlush

;**************************

;*   Get Mean RGB values  *

;**************************

GetMean:

LDI		Temp,'G'		; Get mean values

RCALL	Send

LDI		Temp,'M'

RCALL	Send

LDI		Temp,CR

RCALL	Send

RCALL	Receive1		; 255 (decimal)

RCALL	Receive2		; 'S'

RCALL	Receive3		; Red

RCALL	Receive4		; Green

RCALL	Receive5		; Blue

RCALL	Receive6		; Rdev

RCALL	Receive7		; Gdev

RCALL	Receive8		; Bdev

RCALL	Receive9		; ':'

LDS		Temp,Rdev

LDS		Temp2,Gdev

LDS		Temp3,Bdev

CP		Temp,Temp2

BRGE	NotGreen

RJMP	NotRed

NotGreen:

CP		Temp,Temp3

BRGE	FoundRed

RJMP	FoundBlue

NotRed:

CP		Temp2,Temp3

BRGE	FoundGreen

RJMP	FoundBlue

FoundRed:

SBI		PortB,0			; Turn on Red LED

RCALL	ClearLCD

letter 5,2				; load 'R'

letter 6,5				; load 'e'

letter 6,4				; load 'd'

RCALL	SwingArm		; Knock over can

RJMP	MoveOffTab

FoundGreen:

SBI		PortB,1			; Turn on Green LED

RCALL	ClearLCD

letter 4,7				; load 'G'

letter 7,2				; load 'r'

letter 6,5				; load 'e'

letter 6,5				; load 'e'

letter 6,14				; load 'n'

RJMP	MoveOffTab

FoundBlue:

SBI		PortB,4			; Turn on Blue LED

RCALL	ClearLCD

letter 4,2				; load 'B'

letter 6,12				; load 'l'

letter 7,5				; load 'u'

letter 6,5				; load 'e'

RJMP	MoveOffTab

MoveOffTab:

LDI	Temp, 0x04		; Moves robot forward for 1sec to get off tab

LDI Temp2,0x89

LDI Temp3, 0x7F

OUT RSERVO_H,Temp

OUT LSERVO_H,Temp

OUT RSERVO_L,Temp2

OUT LSERVO_L,Temp3

LDI Temp,0b11110000

; bits7,6	=1,1 Set up and clear down (OC1A)

;     5,4	=1,1 Set up and clear down (OC1B)

; bits3,2	=0,0 Force output compare OFF A and B

; bits1,0	=0,0 PWM phase and Freq correct

OUT TCCR1A,Temp

RCALL	DelayL

JMP		StartAgain

;**********************

;*   Knock over can   *

;**********************

SwingArm:

; Sets OCR0

;219=swing out

;245=home position

LDI Temp,219

OUT ARM_SERVO,Temp

RET

;*******************************************

;*   Line Track  With Obstacle Avoidance   *

;*******************************************

;===============================================

; Full Clockwise	(Right Side)	: 1172 0x491

; Neutral							: 1156 0x484

; Full C. Clockwise	(Left Side)		: 1141 0x477

;

; High if black or nothing

; Low if White

;===============================================

LineTrack:

CLR		ADCCounter			; Clear the ADC "object is too close" register

ADC_Obstacle_Check:

IN		Temp,ADCH			; Check A/D value

CPI		Temp,ADC_Distance

BRGE	LineTrack2

INC		ADCCounter			; If object is too close, increment the counter

CPI		ADCCounter, 3

BRNE	ADC_Obstacle_Check

RCALL	StopForObstacle

JMP		LineTrack

LineTrack2:

;Check Right Sensor

RCALL	Delay5ms

RCALL	Delay5ms

RCALL	Delay5ms

RCALL	CheckForTabs

SBIC	PINA,7

RJMP	RightSeesBlack

;Right Sensor sees White

;Check Left Sensor

SBIC	PINA,6

RJMP	TurnLeft

;Left sees White

RJMP	OffTrack

RightSeesBlack:

;Check Left Sensor

SBIC	PINA,6

RJMP	Straight

;Left sees White

RJMP	TurnRight

TurnLeft:

; Sets Pulse On Period

LDI	Temp, 0x04

LDI Temp2,0x91

LDI Temp3, 0x7F

OUT RSERVO_H,Temp

OUT LSERVO_H,Temp

OUT RSERVO_L,Temp2

OUT LSERVO_L,Temp3

CLR prevIR

RJMP LineTrack

TurnRight:

; Sets Pulse On Period

LDI	Temp, 0x04

LDI Temp2,0x89

LDI Temp3, 0x7D

OUT RSERVO_H,Temp

OUT LSERVO_H,Temp

OUT RSERVO_L,Temp2

OUT LSERVO_L,Temp3

CLR prevIR

RJMP LineTrack

Straight:

; Sets Pulse On Period

LDI	Temp, 0x04

LDI Temp2,0x89

LDI Temp3, 0x7F

OUT RSERVO_H,Temp

OUT LSERVO_H,Temp

OUT RSERVO_L,Temp2

OUT LSERVO_L,Temp3

CLR prevIR

RJMP LineTrack

Offtrack:

INC		prevIR

CPI		prevIR,210

BRNE	LineTrack

RCALL	ShutDown

;**********************

;*   Check for Tabs   *

;**********************

CheckForTabs:

SBIC	PINA,5

RJMP	TabDetected

RET

TabDetected:

;	RCALL	ClearLCD

;	letter 5,4				; load 'T'

;	letter 6,1				; load 'a'

;	letter 6,2				; load 'b'

RCALL	GetMean

FoundTab:

RJMP FoundTab

;*****************

;*   Shut Down   *

;*****************

;Determines Which message to print on LCD screen

ShutDown:

LDI 	Temp,0b00000000	; Disconnect PWM

OUT 	TCCR1A,Temp

RCALL	ClearLCD

CheckIfOffTrack:

CPI		prevIR,150

BRNE	CheckIfBump

RCALL	PrintOffTrack

RJMP	RobotSleep

CheckIfBump:

RCALL	PrintBump

RobotSleep:

RJMP	RobotSleep

;**********************************

;*   Waits for obstacle to move   *

;**********************************

StopForObstacle:

RCALL	ClearLCD

RCALL	PrintObstacle

RCALL	ClearTCNT

;Reverse 1.75sec

LDI		Temp, 0x04

LDI 	Temp2,0x7F

LDI 	Temp3, 0x89

OUT 	RSERVO_H,Temp

OUT 	LSERVO_H,Temp

OUT 	RSERVO_L,Temp2

OUT 	LSERVO_L,Temp3

RCALL	DelayL

RCALL	DelayS

RCALL	DelayS

RCALL	DelayS

RCALL	ClearTCNT

;Left 1 sec

LDI		Temp, 0x04

LDI	 	Temp2,0x91

LDI 	Temp3, 0x84

OUT 	RSERVO_H,Temp

OUT 	LSERVO_H,Temp

OUT	 	RSERVO_L,Temp2

OUT	 	LSERVO_L,Temp3

RCALL	DelayL

RCALL	ClearTCNT

;Straight 1 sec

LDI	Temp, 0x04

LDI Temp2,0x89

LDI Temp3, 0x7F

OUT RSERVO_H,Temp

OUT LSERVO_H,Temp

OUT RSERVO_L,Temp2

OUT LSERVO_L,Temp3

RCALL	DelayL

LDI 	Temp,0b00000000		; Disconnect PWM

OUT 	TCCR1A,Temp

RCALL	KnockOverObstacle

RCALL	PWMInit_Wheels

;Reverse 1sec

LDI	Temp, 0x04

LDI Temp2,0x7F

LDI Temp3, 0x89

OUT RSERVO_H,Temp

OUT LSERVO_H,Temp

OUT RSERVO_L,Temp2

OUT LSERVO_L,Temp3

RCALL	DelayL

RCALL	ClearTCNT

;Backwards Right 1.25sec

LDI		Temp, 0x04

LDI	 	Temp2,0x77

LDI 	Temp3, 0x84

OUT 	RSERVO_H,Temp

OUT 	LSERVO_H,Temp

OUT	 	RSERVO_L,Temp2

OUT	 	LSERVO_L,Temp3

RCALL	DelayL

;	RCALL	DelayS

;Return to going straight

LDI	Temp, 0x04

LDI Temp2,0x89

LDI Temp3, 0x7F

OUT RSERVO_H,Temp

OUT LSERVO_H,Temp

OUT RSERVO_L,Temp2

OUT LSERVO_L,Temp3

RCALL	ClearLCD

RET

;************************************************

;*   Swings Out Arm To Knock Over An Obstacle   *

;************************************************

KnockOverObstacle:

LDI Temp, 0

OUT TCNT0, Temp

; Sets OCR0

;219=swing out

;245=home position

LDI Temp,219

OUT ARM_SERVO,Temp

RCALL	DelayL

; Start TCNT's at $00

LDI Temp, 0

OUT TCNT0, Temp

; Sets OCR0

;219=swing out

;245=home position

LDI Temp,245

OUT ARM_SERVO,Temp

RET

;***********************

;*   Print Off Track   *

;***********************

PrintOffTrack:

letter 4,15					; load 'O'

letter 6,6					; load 'f'

letter 6,6					; load 'f'

letter 10,0					; load ' '

letter 5,4					; load 'T'

letter 7,2					; load 'r'

letter 6,1					; load 'a'

letter 6,3					; load 'c'

letter 6,11					; load 'k'

RET

;******************

;*   Print Bump   *

;******************

PrintBump:

letter	4,2 				; load "B"

letter 	7,5 				; load "u"

letter 	6,13 				; load "m"

letter 	7,0 				; load "p"

RET

;**********************

;*   Print Obstacle   *

;**********************

PrintObstacle:

letter	4,15				; Load 'O'

letter	6,2					; Load 'b'

letter	7,3 				; Load 's'

letter	7,4 				; Load 't'

letter	6,1 				; Load 'a'

letter	6,3 				; Load 'c'

letter	6,12				; Load 'l'

letter	6,5 				; Load 'e'

RET

;*****************

;*   Clear LCD   *

;*****************

ClearLCD:

LDI		LCDReg,0

OUT 	LCD_PORT,LCDReg

CBI 	LCD_PORT,5

RCALL 	Latch

RCALL 	Delay5ms

LDI 	LCDReg,1

OUT 	LCD_PORT,LCDReg

CBI 	LCD_PORT,5

RCALL 	Latch

RCALL 	Delay5ms

RET

;*************

;*   Latch   *

;*************

;Used to Make A Falling Edge

Latch:

SBI		LCD_PORT,4 			; set E=1

CBI 	LCD_PORT,4			; set E=0

RET

;************

;*   Send   *

;************

Send:

SBIS	UCSRA,UDRE

RJMP	Send

OUT		UDR,Temp

RET

;****************

;*   Receive1   *

;****************

Receive1:

SBIS	UCSRA,RXC

RJMP	Receive1

IN		Temp,UDR

STS		S1,Temp

RET

;****************

;*   Receive2   *

;****************

Receive2:

SBIS	UCSRA,RXC

RJMP	Receive2

IN		Temp,UDR

STS		S2,Temp

RET

;***************

;*   Receive3  *

;***************

Receive3:

SBIS	UCSRA,RXC

RJMP	Receive3

IN		Temp,UDR

STS		R_VALUE,Temp

RET

;****************

;*   Receive4   *

;****************

Receive4:

SBIS	UCSRA,RXC

RJMP	Receive4

IN		Temp,UDR

STS		G_VALUE,Temp

RET

;****************

;*   Receive5   *

;****************

Receive5:

SBIS	UCSRA,RXC

RJMP	Receive5

IN		Temp,UDR

STS		B_VALUE,Temp

RET

;****************

;*   Receive6   *

;****************

Receive6:

SBIS	UCSRA,RXC

RJMP	Receive6

IN		Temp,UDR

STS		Rdev,Temp

RET

;****************

;*   Receive7   *

;****************

Receive7:

SBIS	UCSRA,RXC

RJMP	Receive7

IN		Temp,UDR

STS		Gdev,Temp

RET

;****************

;*   Receive8   *

;****************

Receive8:

SBIS	UCSRA,RXC

RJMP	Receive8

IN		Temp,UDR

STS		Bdev,Temp

RET

;****************

;*   Receive9   *

;****************

Receive9:

SBIS	UCSRA,RXC

RJMP	Receive9

IN		Temp,UDR

STS		S9,Temp

RET

;*************************

;*   Clears Wheel TCNT   *

;*************************

ClearTCNT:

LDI Temp, 0

OUT TCNT1H, Temp

OUT TCNT1L, Temp

RET

;****************

;*   Delay 1s   *

;****************

DelayL:

; =============================

;     8000000 cycles:

; -----------------------------

; delaying 7999992 cycles:

LDI  	Delay1, $48

WGLOOP0:

LDI		Delay2, $BC

WGLOOP1:

LDI		Delay3, $C4

WGLOOP2:

DEC		Delay3

BRNE	WGLOOP2

DEC  	Delay2

BRNE 	WGLOOP1

DEC  	Delay1

BRNE 	WGLOOP0

; -----------------------------

; delaying 6 cycles:

LDI  	Delay1, $02

WGLOOP3:

DEC  	Delay1

BRNE 	WGLOOP3

; -----------------------------

; delaying 2 cycles:

NOP

NOP

; =============================

RET

;*******************

;*   Delay 0.25s   *

;*******************

DelayS:

; =============================

;     1000000 cycles:

; -----------------------------

; delaying 999999 cycles:

LDI 	Delay1,$09

WGLOOP00:

LDI  	Delay2,$BC

WGLOOP11:

LDI  	Delay3,$C4

WGLOOP22:

DEC  	Delay3

BRNE 	WGLOOP22

DEC 	Delay2

BRNE 	WGLOOP11

DEC  	Delay1

BRNE 	WGLOOP00

; -----------------------------

; delaying 1 cycle:

NOP

; =============================

RET

;****************

;*   Delay 5ms  *

;****************

Delay5ms:

; =============================

;    delay loop generator

;     40000 cycles:

; -----------------------------

; delaying 39999 cycles:

LDI  Delay1,$43

LoopD:

LDI  Delay2,$C6

LoopE:

DEC  Delay2

BRNE LoopE

DEC  Delay1

BRNE LoopD

; -----------------------------

; delaying 1 cycle:

NOP

; =============================

RET

;******************

;*   Delay 1.5ms  *

;******************

Delay1p5ms:

; =============================

;    delay loop generator

;     12000 cycles:

; -----------------------------

; delaying 11997 cycles:

LDI  Delay1, $1F

Delay1p5msLOOP0:

LDI  Delay2, $80

Delay1p5msLOOP1:

DEC  Delay2

BRNE Delay1p5msLOOP1

DEC  Delay1

BRNE Delay1p5msLOOP0

; -----------------------------

; delaying 3 cycles:

LDI  Delay1, $01

Delay1p5msLOOP2:

DEC  Delay1

BRNE Delay1p5msLOOP2

; =============================

RET