# MARVIN

# Report on the Sonar Trilateration System

Bryan A. Davis

### Computational NeuroEngineering Laboratory

### University of Florida

### Gainesville, FL  32611

### Introduction

The paper describes the progress on implementing complex navigational behavior on a mobile platform, called MARVIN: a Mobile Autonomous Roaming Vehicle with Integrated Navigation. Specifically, this paper addresses the progress and characterization of the special system on MARVIN: the ultrasonic trilateration system. I am currently experiencing difficultly in getting the ultrasonic system to work. This paper is organized into two substantive sections: the theory behind the ultrasonic triangulation system and the current state of the hardware.

Navigating robots have one principle task: to know where they are at all times. If a robot can accomplish this, then all sorts of interesting behaviors can be exhibited by the robot. Unfortunately, knowing one’s location is not a trivial task. More complex animals typically use vision along with the brain’s unparalleled pattern recognition ability. Ocean mammals use sonar to find prey and may also use it to navigate along the ocean bottom. In the 1990s the U.S. Military created the Global Positioning System (GPS) which uses trilateration of radio signals to determine location with 1 meter accuracy. MARVIN will attempt to accomplish this feat using trilateration of ultrasonic pulses from transmitters placed throughout the room.

Theory: Trilateration

If a robot’s distance to a known location can be accurately determined, then the location of the robot can be narrowed down to some point on a circle of a radius equal to the distance from the known location. If the distance to two known locations is known, then the robot’s location can be narrowed down to the intersection of two circles, which is typically two points that lie on both circles. Thus, just with two known distances, the robots location can be narrowed down to one of two possible locations. If the known locations are at the edge of the room, then one of these two possible positions can be eliminated and the robot’s location can be determined, as shown in figure 1.

Wall

Sonar Beacons

Robot is here

Distance determined by timing

Fig. 1. Calculating position using two sonar beacons and a timing reference. The timing difference between the reference and the sonar pulse places the robot in a circle about the sonar beacon. Using two beacons, the robot’s location is limited to two points on intersecting circles. If one of the points is behind a wall, then the robot’s location is known.

Once we calculate the distance to two beacons, we can use geometry to determine our location. To do this, it is convenient to set up a coordinate system. We will choose the origin to be located at the center of the line joining the two beacons and the x-axis parallel to this line (fig. 2).

Wall

Beacon A

(-D,0)

Robot

(x,y)

Beacon B

(D,0)

a

b

The robot’s location (x,y) can be determined by solving the following equations:

x+D2+y2=a2	(1)

x−D2+y2=b2	(2)

The solution for x is found by simply subtracting (1) from (2), which results in:

x=a2−b24D	(3)

Once x is found, we can substitute x into (1) and (2), add and solve for y to get:

y=a2+b22−x2−D2	(4)

Thus, the only math needed is a few multiplies, adds, right-shifts (for division by 2) and the square-root, which can be implemented using a look-up table. Alternatively, we can send the distance information to a remote computer via the wireless link to calculate the location.

We can also determine the error in our location given some error in the distance measurement. The simplest way to do this is by calculating partial derivatives of (3) and solve for dx, which gives us:

dx=a⋅da−b⋅db2D	(5)

If assume that the error in a and b are identical zero-mean uncorrelated random variables, then it can be shown (left as an exercise for the reader) that the standard deviation of the error in x is a2+b22D times the standard deviation of the error in a and b. If we substitute x and y for a and b in this expression, we get x2+y2+D22D. Thus, the error in the x-direction is the least, when the robot is directly between the two beacons and increases as the robot moves away from this point.

To determine the error in the y direction, we simply take the partial derivative of (1) + (2) and substitute in for dx from (5) and solve for y. Doing this we have:

dy=12ya⋅da+b⋅db+xDa⋅da−b⋅db	(6)

Thus, the error in the y-direction is inversely proportional to the distance to the line joining the two beacons. This means that if we are near the line joining the two beacons, any small error in either distance measurement will be amplified greatly in the location coordinates! If we solve for a and b in (5) and (6) and assume that the error for a and b are independent zero-mean random variables with the same variance, we can create plots for the error in x and y as ratios for the standard deviation of the error in a and b. In simpler terms, we are looking at the sensitivity of the location error for a given measurement error as a function of the location. Fig. 3 shows the sensitivity of the x value (the abscissa) to errors in measurement as a function of location, and fig. 4 shows the same for the y value (the ordinate).

Fig. 3. Sensitivity of the abscissa to measurement error.

Fig. 4. Sensitivity of the ordinate to measurement error.

From fig. 3 we see that the x-axis coordinate has no “dead zones” when it comes to error sensitivity. The error sensitivity is simply increases monotonically with the distance from the mid-point of the two beacons. The y-axis coordinate however, is highly sensitive to error when it is near the extended line connecting the two beacons. This means that we must arrange the beacons so that the robot is never directly between them (this is rather difficult with more than two beacons), or disregard the measurements taken between the two beacons the robot is collinear with.

For MARVIN, the plan is to use three beacons. A third beacon can provides redundancy in case one of the beacons fails, one of the beacons is obscured by an object which is causing it to give a false reading, or the robot is collinear with two of the beacons. With only two beacons, we would have no way of knowing if one of the beacons was obscured without using additional information (such as odometry information to determine a large jump in absolute position is false). The equations for three beacons are more complicated for two reasons: 1) the coordinate system cannot be set up as nicely with three beacons and 2) the system is over-determined. In dealing with an over-determined system, we can simply reject one of the distance measurements, or we can use the least-squares method to incorporate all three readings. The problem with the least-squares method (other than the fact that it is computationally expensive), is that it treats all measurement values as equally valid. If this is not the case (i.e. one reading is erroneous due to obscuration), then least-squares will still give bogus results.

Perhaps the best method would be to calculate the location using each of the three sets of two beacons and determine if they are consistent. If they are, then simply use the average of the three. If they are not, then use odometry information to determine which (if any) of the three results match with what is expected and use that information. If none of them match, e.g. if someone picks up the robot (thus invalidating odometry information) and moves it to a location where one of the beacons is obscured resulting in a false reading, then this method will fail.

### Theory: Sonar Ranging

Also, as compared to radio waves, sound travels very slow. This gives us potentially a much higher resolution for the distance. The theoretical limit for the distance resolution ∆x is simply the timing resolution ∆t times the speed of sound, which, neglecting temperature variation, is ~333 m/s.  The maximum timing resolution of MARVIN’s input capture function is ~1/(8 MHz), =  125ns. Thus the theoretical distance resolution is 0.042mm. Of course, there are many other limiting factors in determining this resolution, the most significant of which is the bandwidth of the ultrasonic sensors.

The practical resolution of distance measurement is mainly due to the bandwidth of the mechanical and electrical filters used in the transmitting and receiving the sound pulse. Because many objects create sound filters are necessary to reduce interference with these other sources of sound. Also, efficient audio drivers typically have a narrow band in which they are efficient. Having a narrow-band transducer or filter necessarily means that the transient response of the filter will peak some time after the initial signal is heard. For example, fig. 6 shows the response of a 2nd-order and a 6th-order Butterworth band-pass filter to a square-wave input (typical of the output of a 555 timer). The 2nd-order system is supposed to model the audio response of a narrow-band ultrasonic transducer given a square-wave input. The 6th-order filter is supposed to model the response at the output of the receiver’s filter. This model is shown in fig. 5.  The filters are assumed to have a 3-dB pass-band of 2 kHz centered about 40 kHz.

555 Timer

Txc

Rx

Band-pass filter

Fig. 5. Model of the sonar transmit and receive circuits. The 555 timer outputs a square-wave at 40 kHz. The ultrasonic transmitter acts as a band-pass filter as does the ultrasonic receiver. To reduce noise, the receiver circuit also contains a filter. The overall result is that the square-wave input is filtered by three cascaded band-pass filters. If we assume that each of these filters is 2nd-order (the simplest possible linear band-pass filter), then the total system response is 6th-order.

Fig. 6. Response of a 2nd-order (green) and a 6th-order (blue) Butterworth band-pass filter to a square-wave input oscillating at the filter’s center frequency. The center frequency here is 40 kHz and the 3-dB bandwidth of the filter is 2 kHz.

You can see from fig. 6 that the 2nd-order filter response reaches 50% of the peak response after about 0.1 ms and the 6th-order filter response reaches 50% of the peak response in 0.35 ms. If we assume that a received signal could reach the threshold required to trigger anywhere between 0 and 0.35 ms, then the resulting distance resolution is now 16 cm—over half a foot! To increase the resolution (reduce the value), we must lower the receive threshold to something much less than 50%, or we must utilize some other information, such as the fact that pulses that originated further from the receiver will take longer to rise above threshold. However, we don’t actually need to figure this out as we can characterize the system and find the optimal curve fitting empirically.

### Current Progress

So far, I am still working on getting one transmit receive pair to work. I am able to transmit and receive the 40 kHz frequency, but right now, the range for which the signal is stronger than most background noise is only a few inches. I believe the problem lies both in the receiver and the transmitter. Firstly, looking at the oscilloscope readings, noise from talking or especially banging or snapping or jingling of keys causes a much larger response in the receiver than does the transmitter tone. Even if I place sharp filters on the receiver, I still believe these highly transient (and thus broadband) pulses will cause false readings on the receiver threshold output. This problem may not be terrible since there is a very short window for which they could interfere with the sonar pulse, but then it is a problem that must be dealt with in software. More devastatingly, the thermal/EM noise generated on the circuit dominates the transmitter noise when the transmitter is about more than 2 feet away. The only solution I can come up with for this is to increase the transmitter power or use better op-amps on the receiver. I can use two 9-volt batteries for the transmitter instead of one, and I have already ordered the MAX266 switched capacitor filter from Dallas semiconductor. A circuit using this chip is on the IMDL website. Another option is to try to find better transducers, but I am running out of time on this one. Hopefully one of these solutions will work.

If these options don’t work, I may have to use IR triangulation (with fixed IR beacons), or just forgo the absolute positioning system altogether. The system works fine without it for a few minutes, as long as there are no funny obstacles or no one moves the robot.