# MARVIN

# Mobile Autonomous Roving Vehicle with Integrated Navigation

Bryan A. Davis

### July 2, 2004

### Computational NeuroEngineering Laboratory

### University of Florida

### Gainesville, FL  32611

### Abstract

Marvin is a navigation robot that uses precise actuation and absolute positioning via beacons to determine its location in a room. The precision positioning is due to using a stepper-motor drive system, and the absolute positioning is achieved using sonar trilateration. Marvin communicates its sensor data via a wireless link to a PC, which then issues simple motion primitives to the robot. This paper describes the goals, implementations and results obtained when building and testing Marvin.

### Executive Summary

This paper describes the goals, implementations and results of the autonomous robot Marvin, built during the summer of 2004 in IMDL at the University of Florida. The purpose of this robot is to demonstrate the ability of a robot robustly navigate in a room. Navigation is a difficult problem for an autonomous robot to solve. For most robots, no attempt is made to know its location in a room; rather they rely on other methods of solving the particular problem at hand.

Marvin uses various sensors and actuators to accomplish his task. The main sensor system that is unique to Marvin is the ultrasonic distance sensing mechanism. This system sends out a radio pulse to one of three remotely located beacons. The beacon then sends back an ultrasonic pulse. The robot can then determine the distance to the beacon based on the time delay. Once the distance to two or three beacons is know, Marvin can then determine his location based on geometry.

The other system that is unique to Marvin is the stepper motor drive system. Stepper motors allow Marvin to travel a specified distance with an error on the order of a millimeter (depending on the floor). They also allow the robot to turn accurately (typically within 2º) and travel in a very straight line.

In addition to these systems, Marvin has IR ranging sensors that allow it to detect obstacles to avoid collisions. A long-range sensor is mounted on a servo-controlled turret to allow Marvin to scan the room in order to map it. Marvin also has bump switches to detect collisions in case the IR sensors failed to detect an object.

Finally, to control his behavior, Marvin is interfaced to PC via a wireless link. The PC reads in data from the robots sensors, processes the data, receives user input and then gives commands that tell the robot to move forward or turn a particular number of steps.

### Introduction

One of the basic problems in mobile robotics is determining accurately where the robot is located. This problem seems simple to animals, but in reality it is terribly complex. Animals are able to do it quite well using a range of highly specialized sensors (olfactory, vision, sense of touch, etc) along with complex signal processing structures in the brain that far surpass the abilities of any artificial sensory processing systems. For robots, the solution usually is to forgo any explicit knowledge of location absolute position and rely on other information to solve the problem at hand. For this project however, we are interested specifically in knowing the robot’s location in a room, even as the robot navigates around the room. While this is not necessary for many problems (say a robot vacuum cleaner that moves around randomly), there are many problem such as the vacuum cleaning or security robot problems that would benefit greatly if it could accurately determine its location.

The robot described here has this as its goal: to navigate about a room following commands, and maintain knowledge of its location even after long periods of operation.  This robot is called MARVIN: a Mobile Autonomous Roving Vehicle with Integrated Navigation. Marvin is a mechanically simple robot that navigates about a small room based on commands given by a central computer. It can scan the room for obstacles using infrared (IR) transmitter/sensor pairs, and transmit their location back to the central computer via a wireless link. Additionally, Marvin is able to determine its location using two or three ultrasonic beacons placed in the room, and relays this information back to the central computer. The computer gives commands the robot to move a certain distance in a straight line, or to turn about a certain angle. The distance and angle traveled by Marvin is accurately determined through the use of stepper motors for the wheel drive system.

Integrated System

Marvin consists of two physical systems that operate together: The mobile platform (MP) which gathers all sensory information and controls the motors, and the remote PC which receives all sensory information from the MP, processes it, receives input from the user (or other process) and sends primitive motion commands back to the robot. The PC system consists of a wireless transmitter connected to a serial port, and software running on Matlab®, that performs the signal processing.

The MP consists of five subsystems: the stepper motor actuators, the servo-controlled IR sensors, the LCD panel, the wireless link to the PC and the ultrasonic subsystem. The most complex of these—the ultrasonic subsystem, consists of a set of RF transmitters and a ultrasonic receiver. As part of the ultrasonic subsystem, but not located on the MP are two or three ultrasonic beacons that a placed in fixed locations somewhere in the room. These beacons consist of a RF receiver and a ultrasonic transmitter. The two platforms with each of their subsystems are shown in Fig. 1.

Matlab

Serial

RF

Rx/Tx

PC

Mobile Platform (MP)

RF

Rx/Tx

StepperMotors

IR / Bump Sensors

Micro-

Ctrller

Ultrasonic Subsystem

933MHz

Beacon

400 MHz

40kHz

Fig. 1. Structural Diagram of Marvin. Marvin consists of two platforms, one mobile and the other on the PC, along with three sonar beacons.

LCD

panel

The stepper motors are able to drive Marvin an exact distance (in a straight line) or turn him an exact angle about a point. This is difficult to accomplish using DC motors even with encoders because it is difficult to get the motor speeds exactly the same (there usually is some wooble). However, even with the precision stepper motors, the errors in position due to slippage will accumulate over distance, so that eventually the robot will lose track of its location. To account for this, the robot has an ultrasonic subsystem, that works similar to the U.S. Military’s Global Positioning System (GPS). The ultrasonic subsystem sends out RF pulses to a set of two or more beacons. The beacons then immediately send back sound pulses. From the delay, between the RF “ping” and ultrasonic “pong”, Marvin can calculate the distance to the ultrasonic beacons. Once this is known, using simple geometry will give us the location of the MP.

Additionally, we need some sensors to monitor the environment in order to avoid obstacles and potentially map out an area. This is accomplished with IR transmitter/detector pairs and a bump skit. Most of the IR sensors are mounted at various locations on the mobile platform facing forward in order to detect and avoid obstacles in the MP’s path. Also, a circular bump skit is placed around the MP which can detect if the MP collides with any obstacle that is missed by the IR sensors. Another IR sensor is mounted on top of a servo motor turret. This sensor can rotate 180 degrees from left to front to right on the MP. This sensor is a long range IR sensor that can determine the distance of objects up to 6 feet away. This sensor is primarily used for mapping the environment.

We decided that Marvin would be best implemented on two platforms: the mobile platform (MP) to collect data and drive the motors, and the PC platform to perform the processing. Two platforms have some significant disadvantages: 1) The robot is no longer self-contained, 2) a communication protocol and communication equipment must be developed, and 3) there are additional points of failure for the system. Despite these problems, we decided to add a PC platform because: 1) Processing the ultrasonic data requires some involved complicated data processing such as computing sines, cosines and square-roots and matrix-inverses (for least-squares), which would tax our timing sensitive microcontroller or require an additional DSP, 2) since Marvin is not entirely autonomous (it must be told where to go), the PC makes a very nice user interface, and 3) development of the algorithms is much easier on the PC.

### Mobile Platform

The mobile platform itself is made of ⅛” plywood. The platform was designed on AutoCAD®, cut-out using the IMDL lab’s T-Tech machine and assembled with wood glue. The platform design is strongly inspired by the Talrik Jr. design (TJ) used extensively in the Machine Intelligence Laboratory (MIL). The bottom of the platform consists of a 6”x6”x3” box enclosed on five sides, which contains the stepper motors, battery, and circuit boards. On the outside is the power switch. The top of the platform is a 9” diameter disk with tabs cut-out so that it mates with the box and the LCD panel. It is hinged to the box for easy access to the components inside of the box. The LCD panel is a triangular prism that sits on the back of the disk and houses the LCD panel. It is angled so it is easy to read. The prism also houses a switch which can be used to reset the microcontroller (however, I usually just turn the power off and on again). Surrounding the platform disk is a set of four bump switches connecting to a bump skirt. The bump skit is a thin ring of plywood that floats around the disk, held in place by tabs that rest on top and beneath the disk. The design is such that the bump skirt can move a few millimeters around the disk in order to activate the bump switches, but will not fall off of the platform. In the center of the platform is a sonic waveguide made of sheet metal that guides the ultrasonic sound waves from any direction in the plane of the MP towards an ultrasonic transducer. This is necessary because of the high directionality of the ultrasonic transducers. Fig 2. is a profile look at the MP, and Fig. 3 is an overhead look.

Bump Skirt

LCD Panel

Sonic Waveguides

Ultrasonic Transducer

IR Servo

Wheels

Foot

Fig 2. Profile view of the mobile platform.

IR Sensors

Power Switch

Sonic Waveguide

IR Sensors

LCD Panel

Bump Skirt

Ultrasonic Transducer

IR Servo

Fig 3. Top view of the mobile platform.

The stepper motors are placed at the bottom of the box slightly forward of center. The shafts of the two stepper motors extend through holes in the box, and wooden wheels 3” in diameter (made with the T-Tech) are attached on the outside. Since the MP has only two wheels, a metal foot obtained from a small spice jar is placed centered on the back of box. The foot is curved in two dimensions so that only one point of it touches the ground at any time. We decided not to use a caster because it cause slippage as it changes direction.

Because it only has two wheels and a foot, the MP must be precisely balanced. If the center of mass is too close to the two wheels, then the MP will rock forward when braking. But if the center of mass is too far back, then too much weight will be placed on the foot resulting in friction that the wheels might slip trying to overcome. One can mitigate this by placing the foot farther back (say on an extension) but then the friction force would become very high when turning in place. So by strategically placing the foot and moving the battery pack, we can achieve a balance between these goals.

### Actuation

For the objectives of this project, we needed to use actuators with precision control. The options are basically DC motors with encoders or stepper motors. DC motors with encoders have a few advantages: they are cheap if you build your own encoders, they are powerful, they are relatively efficient, and they have a flat torque curve. Stepper motors on the other hand, cost more (although mine were only $8 a piece), aren’t very powerful for their size, are very inefficient at slow speeds and lose torque rapidly as speed increases. Additionally, stepper motors can slip if they are moved too fast, causing a loss of known position, whereas DC motors with steppers don’t have this problem (although they can lose traction which is just as bad).

#### Stepper Motors

Stepper motors have a few advantages however: they do not require any encoders because the controller specifies every step that the motor makes. This means that the controlling software knows where the motor is positioned at all times (assuming no slipping). With encoders, a DC motor controller can accomplish the same thing, but it must use some feedback control system that inevitably has some delay. This is not a problem with one motor, but when two motors are used, any slight variation in speed between them will result in a change in direction, which can result in huge errors in position as distance accumulates. Two stepper motors however can be driven in exact synchrony resulting in perfectly straight travel assuming perfect wheels and a perfectly flat surface. This is true even if the stepper motors are have different torque characteristics or are driven by varying voltages! (This is assuming that the torque is always large enough to accelerate the MP at the given rate of speed, but this is an easy assumption to meet by just reducing acceleration). Additionally, since every step is precisely controlled, stepper motors can give varying acceleration profiles with little regard to motor torque (again as long as the load torque never exceeds the motor torque, the acceleration is independent of torque).

The stepper motors in the mobile platform project are EAD Motors ZB17BBK-11 and were obtained from All Electronics Corp. CAT# SMT-72 for $7.75 each. These are two-phase stepper motors (the most common type), with 200 full-steps per rotation (1.8º per step). They have 35 oz-in. of holding torque (the torque at no speed) at 8.5 V run in the bipolar configuration. These motors are size NEMA 17, which is a standard. I am using a battery of 6-1.25V cells for a total of ~7.5 V. In theory this would give me about 30 oz-in of holding torque. The current in this configuration is 0.85 A according to the datasheet. However, as speed (frequency) increases the coil reluctance increases, which reduces the current and hence the torque.

At first I tried to use the stepper motors that came from older 5-¼” disk drives. These motors are great because they are free, but they do not have very much torque, and the torque reduces even more rapidly than do my current motors. In general, the lower the voltage of the motors (the higher the current), the less holding torque you get for a given power, but the less the torque falls off with speed.

#### Stepper Motor Wiring

Two-phase stepper motors can be wired as unipolar or bipolar, as shown in Fig.  4.

Fig. 4. The two ways to wire a two-phase stepper motor. The difference is that the unipolar connects the black and white wires to V+, and either A or A’ , and B or B’ to V–. The bipolar connection connects A to V– and A’ to V+ or vice versa, and A to V– and A’ to V+ or vice versa. These figures were taken from the motor’s datasheet.

The bipolar configuration gives more holding torque, since the current is traveling through two coils instead of 1. But this increases the inductance of the circuit, which limits top-speed due to the low-pass effect of an inductor. Also, the bipolar mode requires an H-bridge motor driver, since each coil terminal must be connected alternatively to V+ and V–, whereas the unipolar mode can be driven by four transistors (either Darlington pairs or FETs). Because efficiency is a concern, we are using the bipolar mode to double our low-speed torque.

In order to drive each motor, we use four half-H-bridge drivers, one half-H-bridge for each coil. The chips that we are using are two SN754410, each of which drives 1 amp of current for each half-H-bridge and has four half-H-bridges in a 16-pin DIP package. The voltage output of each half-H-bridge is controlled by one input pin, and each pair of half-H-bridges are controlled by an enable pin which will tri-state the output when it’s disabled. In our circuit, we connect each of the eight inputs to an output port (port C) on the microcontroller, and we tie all four enable pins together and connect them to a microcontroller pin (port G0).

#### Stepper Motor Control

Each of the coils (A and B) in the two diagrams must be energized in a sequence in order to make the motor step. There are a few different ways to step the motor (wave-step, full-step or half-step), but we will cover the half-step method for the bipolar connection here as it is the best one for our purposes. The stepping sequence is given in Fig. 5. The half-step sequence effectively doubles the resolution of the motor since we are able to move half-way between two steps. For our motors, this gives us 400 steps per revolution.

Fig. 5. Half-step sequence for a bipolar stepper motor. The A and A’ are the two terminals of one coil and B and B’ are the two terminals of the other coil. The value (+ or –) in the table represents the voltage applied to the terminal. The motor direction is reversed by stepping backwards through the table. This table was taken from the motor’s datasheet.

To control the speed of the motor, we simply control the frequency at which we step the motor. The simplest way to do this is to simply wait for some time between the motor steps. This wait time is inversely proportional to the motor speed. Of course, since the microcontroller is doing many other things, this method is not possible. Another alternative would be to use the PWM functionality, but each coil would require its own PWM timer, and we only have two on the ATmega128. The solution is to use the output compare interrupt that gets called for each step transition and iterate through a lookup table to determine the step sequence. We can iterate forward or backward though the table to determine direction.

The above method allows us to control the speed of the stepper motor by controlling how much we increment the output compare register during each call. In order to control the acceleration of the motor (which is necessary to prevent wheel slippage and to ensure that the load torque does not exceed the motor torque), we must gradually increase or decrease this increment. Furthermore, if we wish to have constant acceleration, we must increase or decrease the output compare increment such that the inverse of the increment is increased or decreased linearly. An easy way to accomplish this is to have another output compare interrupt that updates the first output compare register periodically.

However, this method will not work if we decelerate at a constant rate end up stopping in a given number of steps, because the second output compare function does not know how many interrupts of the first output compare function will occur between calls (it could if it used some lengthy calculations, but this is not ideal). The method that we use to decelerate is to increase the value of the output compare increment such that the deceleration is approximately constant.

To determine this formula, consider that for constant acceleration (or deceleration ) a from an initial velocity (to a final velocity) of 0 at the origin,

vx=2ax	(1)

where v(x) is the velocity at distance x from the origin. This formula can be found in any standard physics textbook (perhaps as vi2 – vf2= 2a(x – xi)). This formula is useful because it gives us velocity as a function of distance, rather than time. (Of course, the velocity as a function of time is simply linearly decreasing). Since the interrupt service routine that steps the motors is called for every step, we can set the velocity according to this formula as long as we know the number of steps remaining in the movement we wish to complete. Thus, in the we will set the output compare interrupt in proportion to the inverse of the square-root of the number of steps remaining in the movement, or

Tn=kn,	(2)

where T is the output compare increment (the period), n is the number of steps remaining, and k is some constant. To find k, we note that we are not actually interested in the deceleration value; we are only interested in finding k such that our initial velocity during deceleration is the same as it was before deceleration (otherwise, the velocity will jump). Thus, we want TN, the period at the first step in deceleration equal to TC, the period prior to deceleration. This condition gives us

k=TCN.	(3)

Now, in the code it is time-consuming to calculate the square-root of a number inside of an interrupt service routine (ISR). This would severely impact the performance of the other ISRs such as the input capture ISR for the sonar circuit. There are two options for this: 1) use a lookup table to find the integer square-root; or 2) calculate the square-root using an iterative algorithm. The first solution is probably the best, but it requires creating code to handle lookup tables. The second solution is more fun, so that what we do in this project.

#### Calculating the Square-Root Iteratively

Calculating the square of a number n involves simply multiplying the number by itself, or n*n. But if we wish to calculate the square of the number n + 1, and we already know the value of n, we can use the formula

n+12=n2+2n+1.	(4)

Now, 2n + 1 is simply the nth odd number (starting with n = 0). So the pseudo-code to calculate the first n squares might look like

sq_n = 0;

n_odd = 1;

For n = 0:N,

sq_n = sq_n + n_odd;

n_inc += 2;

end

Now, if we invert this algorithm, we can get the integer square root of n iteratively as follows:

sqrt_n := 0

n_odd := 1

n_count := 0

for n := 0 to N,

if n_count == n_odd

sqrt_n := sqrt_n + n_odd

n_odd := n_odd + 2

n_count := 0

end

n_count := n_count + 1

end

Note that the integer square-root is n, i.e. the floor of the square-root of n. This algorithm is almost what we want, but it starts at zero and ends at N. We want to run this algorithm backwards. Of course, we will need to know the initial value of N. So we have,

sqrt_n := sqrt_N;

n_odd := 2*N + 1;

n_count := 0;

for n := 0 to N,

if n_count == 0

sqrt_n := sqrt_n - n_odd

n_odd := n_odd - 2

n_count := n_odd

end

n_count := n_count - 1

end.

That is the basics of the control algorithm. More details can be found in the code appendix.

### Sonar Trilateration Subsystem

#### Principles of Trilateration

If a MP’s distance to a known location can be accurately determined, then the location of the MP can be narrowed down to some point on a circle of a radius equal to the distance from the known location. If the distance to two known locations is known, then the MP’s location can be narrowed down to the intersection of two circles, which is typically two points that lie on both circles. Thus, just with two known distances, the MPs location can be narrowed down to one of two possible locations. If the known locations are at the edge of the room, then one of these two possible positions can be eliminated and the MP’s location can be determined, as shown in Fig. 6.

Wall

Sonar Beacons

Robot is here

Distance determined by timing

Fig. 6. Calculating position using two sonar beacons and a timing reference. The timing difference between the reference and the sonar pulse places the robot in a circle about the sonar beacon. Using two beacons, the robot’s location is limited to two points on intersecting circles. If one of the points is behind a wall, then the robot’s location is known.

Once we calculate the distance to two beacons, we can use geometry to determine our location. To do this, it is convenient to set up a coordinate system. We will choose the origin to be located at the center of the line joining the two beacons and the x-axis parallel to this line as shown in Fig. 7.

Wall

Beacon A

(-D,0)

Robot

(x,y)

Beacon B

(D,0)

a

b

Fig. 7. Geometrical calculation of the location of the MP given the distances to the beacons.

The MP’s location (x,y) can be determined by solving the following equations:

x+D2+y2=a2	(5)

x−D2+y2=b2	(6)

The solution for x is found by simply subtracting (5) from (6), which results in:

x=a2−b24D	(7)

Once x is found, we can substitute x into (5) and (6), add and solve for y to get:

y=a2+b22−x2−D2	(8)

We can also determine the error in our location given some error in the distance measurement. The simplest way to do this is by calculating partial derivatives of (7) and solve for dx, which gives us:

dx=a⋅da−b⋅db2D	(9)

If assume that the error in a and b are identical zero-mean uncorrelated random variables, then it can be shown that the standard deviation of the error in x is a2+b22D times the standard deviation of the error in a and b. If we substitute x and y for a and b in this expression, we get x2+y2+D22D. Thus, the error in the x-direction is the least, when the MP is directly between the two beacons and increases as the MP moves away from this point.

To determine the error in the y direction, we simply take the partial derivative of (5) + (6) and substitute in for dx from (9) and solve for y. Doing this we have:

dy=12ya⋅da+b⋅db+xDa⋅da−b⋅db	(10)

Thus, the error in the y-direction is inversely proportional to the distance to the line joining the two beacons. This means that if we are near the line joining the two beacons, any small error in either distance measurement will be amplified greatly in the location coordinates! If we solve for a and b in (9) and (10) and assume that the error for a and b are independent zero-mean random variables with the same variance, we can create plots for the error in x and y as ratios for the standard deviation of the error in a and b. In simpler terms, we are looking at the sensitivity of the location error for a given measurement error as a function of the location. Fig. 8 shows the sensitivity of the x value (the abscissa) to errors in measurement as a function of location, and Fig. 9 shows the same for the y value (the ordinate).

Fig. 8. Sensitivity of the abscissa to measurement error.

Fig. 9. Sensitivity of the ordinate to measurement error.

From Fig. 8 we see that the x-axis coordinate has no “dead zones” when it comes to error sensitivity. The error sensitivity is simply increases monotonically with the distance from the mid-point of the two beacons. The y-axis coordinate however, is highly sensitive to error when it is near the extended line connecting the two beacons. This means that we must arrange the beacons so that the MP is never directly between them (this is rather difficult with more than two beacons), or disregard the measurements taken between the two beacons the MP is collinear with. Note that these marginal distributions of the error are not independent: they are correlated. But the analysis presented here is still valid, even though we did not calculate the correlation.

#### Practical Applications of the Theory

For MARVIN, the plan is to use three beacons. A third beacon can provides redundancy in case one of the beacons fails, one of the beacons is obscured by an object which is causing it to give a false reading, or the MP is collinear with two of the beacons. With only two beacons, we would have no way of knowing if one of the beacons was obscured without using additional information (such as odometry information to determine a large jump in absolute position is false). The equations for three beacons are more complicated for two reasons: 1) the coordinate system cannot be set up as nicely with three beacons and 2) the system is over-determined. In dealing with an over-determined system, we can simply reject one of the distance measurements, or we can use the least-squares method to incorporate all three readings. The problem with the least-squares method (other than the fact that it is computationally expensive), is that it treats all measurement values as equally valid. If this is not the case (i.e. one reading is erroneous due to obscuration), then least-squares will still give bogus results.

Perhaps the best method would be to calculate the location using each of the three sets of two beacons and determine if they are consistent. If they are, then simply use the average of the three. If they are not, then use odometry information and past readings to determine which (if any) of the three results match with what is expected and use that information. If none of them match, e.g. if someone picks up the MP (thus invalidating odometry information) and moves it to a location where one of the beacons is obscured resulting in a false reading, then this method will fail.

Note that all of these formulas require precise knowledge of the beacon locations. This is a disadvantage of this system as the beacon locations must be precisely measured and the values must be programmed into the software. There is an alternative however, that requires no additional hardware. If we place the beacons in unknown locations (within ear-shot of the MP), and we initially place the MP in a known location with known direction, then we can use the exact same principles outlined here to triangulate the position of the beacons. However, here the MP will serve as all three (or more) known positions for each beacon. Thus we must move the MP to three different positions using the very accurate stepper motor drive, and perform the analysis to determine the location of beacon given the distance to the three MP positions. Note that the three positions must not be collinear and it is best for the triangulation system if they are far apart. Using this method, we can take the robot to a new room, place the beacons in some good positions, place the MP at the desired room origin at the proper heading and run the alignment algorithm to determine the beacon locations.

Another difficulty is that the robot has no way of knowing its heading. We can initialize its heading, but soon the heading will be off. However, we can determine the heading by examining the sonar readings over time. This will require an orientation algorithm that moves the robot until it can get a desired reading.

#### Ultrasonic Ranging Theory

In order to use trilateration, we need to accurately know the distance to the remote beacons. The ideal signal to use is sound. As compared to radio waves, sound travels very slow. This gives us potentially a much higher resolution for the distance. The theoretical limit for the distance resolution ∆x is simply the timing resolution ∆t times the speed of sound, which, neglecting temperature variation, is ~341 m/s.  The maximum timing resolution of MARVIN’s input capture function is ~1/(8 MHz), =  125ns. Thus the theoretical distance resolution is 0.042mm. Of course, there are many other limiting factors in determining this resolution, the most significant of which is the bandwidth of the ultrasonic sensors.

The practical resolution of distance measurement is mainly due to the bandwidth of the mechanical and electrical filters used in the transmitting and receiving the sound pulse. Because many objects create sound filters are necessary to reduce interference with these other sources of sound. Also, efficient audio drivers typically have a narrow band in which they are efficient. Having a narrow-band transducer or filter necessarily means that the transient response of the filter will peak some time after the initial signal is heard. For example, Fig. 11 shows the response of a 2nd-order and a 6th-order Butterworth band-pass filter to a square-wave input (typical of the output of a 555 timer). The 2nd-order system is supposed to model the audio response of a narrow-band ultrasonic transducer given a square-wave input. The 6th-order filter is supposed to model the response at the output of the receiver’s filter. This model is shown in Fig. 10.  The filters are assumed to have a 3-dB pass-band of 2 kHz centered about 40 kHz.

555 Timer

Txc

Rx

Band-pass filter

Fig. 10. Model of the sonar transmit and receive circuits. The 555 timer outputs a square-wave at 40 kHz. The ultrasonic transmitter acts as a band-pass filter as does the ultrasonic receiver. To reduce noise, the receiver circuit also contains a filter. The overall result is that the square-wave input is filtered by three cascaded band-pass filters. If we assume that each of these filters is 2nd-order (the simplest possible linear band-pass filter), then the total system response is 6th-order.

Fig. 11. Response of a 2nd-order (green) and a 6th-order (blue) Butterworth band-pass filter to a square-wave input oscillating at the filter’s center frequency. The center frequency here is 40 kHz and the 3-dB bandwidth of the filter is 2 kHz.

You can see from fig. 6 that the 2nd-order filter response reaches 50% of the peak response after about 0.1 ms and the 6th-order filter response reaches 50% of the peak response in 0.35 ms. If we assume that a received signal could reach the threshold required to trigger anywhere between 0 and 0.35 ms, then the resulting distance resolution is now 16 cm—over half a foot! To increase the resolution (reduce the value), we must lower the receive threshold to something much less than 50%, or we must utilize some other information, such as the fact that pulses that originated further from the receiver will take longer to rise above threshold. However, we don’t actually need to figure this out as we can characterize the system and find the optimal curve fitting empirically.

#### Implementing Ultrasonic Ranging

In order to implement the ultrasonic ranging system we need a few circuits: 1) a RF transmitter on the MP, 2) an RF receiver / ultrasonic transmitter on the beacon, and 3) an ultrasonic receiver on the MP.

RF Transmitter:

The RF transmitters that I first purchased were very cheap, but they did not come with oscillators, so I need to use a crystal. The problem is that the crystal selection for RF components isn’t straightforward and different crystals needed to be used on the transmitter and receiver, even for the same frequency! I later went with a complete board that was data-in data-out, which I recommend. I needed to add an antenna, but that is trivial—it’s just a wire of the proper length. The transmitters I purchased are binary-ASK (amplitude-shift keying), which is a fancy way to say that the transmitter transmits at the given frequency for a 1, then stops transmitting for a 0. This is the simplest possible design and thus the cheapest. To select which beacons I want to signal, I use 3 different frequencies. I could (should) have used one frequency with address decoders on the receiver, but this seemed more complicated than using three transmitters. Fig. 12 shows the circuit diagram for the three RF transmitters which are placed on their own board.

Beacon:

The RF receiver is mated to the transmitter and also operates on ASK. There is one problem with ASK that causes problems. In order to determine the threshold value for which the receiver determines whether the input is 1 or 0 (there is always some thermal noise, so a threshold is needed), the receiver uses automatic gain control (AGC). Basically, it takes a moving average of the few hundreds of milliseconds of transmit time (using a low-pass filter), and uses that as the threshold. This works great if the signal is sending out lots of 0s and 1s. But if the transmitter is quiet for a period of time much longer than 100ms, then the receiver threshold drops below the noise floor, and it will output 1s about half of the time even when it is sending all zeros. The same true if you set it to transmit constantly.

To get around this problem we have to periodically send a pulse of 0s and 1s to set the threshold, otherwise the sonar will be pulsing before we actually send the RF trigger pulse, giving us a false reading. The signal that primes the receiver is generally called the preamble So the procedure is to send the preamble to set the threshold, wait a little wait (~50ms) for all of the echoes to die out, then send the trigger pulse, and wait for the ultrasonic response.

To other part of the beacon is the ultrasonic transmitter. To create this signal we need to generate a 40kHz signal. The easiest way to do this is to simply use a 555 timer with a potentiometer to adjust the frequency. One problem with the basic 555 timer oscillator circuit is that the duty cycles is always greater than 50%. As you near 50%, the power dissipation through the discharge resistor becomes large. To mitigate this problem, a transistor with a pair of diodes can  be connected to the discharge circuit to ensure that so that while charging, the current flows through turned on transistor (very low resistance), and while discharging the transistor is turned off to prevent a short-circuit to ground.

This simple method of implementing the RF on the beacon is not very good because we must transmit ultrasonic pulses during the preamble and wait for them to die down. It would be much better to use an address decoder. That way we could send preambles to all of the beacons at the same time, and the only time the beacons would transmit would be when the correct address was received.

Ultrasonic Receiver:

On the MP, an ultrasonic receiver is required to detect the pulse. This receiver must amplify a very weak signal obtained from the transducer and determine if a 40 kHz signal is present. The first attempt at accomplishing this was to use a bunch of op-amps, but I was unable to design a circuit that performed well. Once I learned that Dallas semiconductor gave out free samples for the MAX266 analog filters, I tried using them. There is also a circuit using this on the IMDL website, although I had problems with oscillations using the particular values of resistors suggested, and I didn’t like how the comparator was connected directly to the filtered output. Instead I modified the circuit by adding a voltage follower and an adjustable inverting amplifier after the MAX266 output. After this amplifier, there is a rectifying diode which connects to a low-pass filter which then connects to a comparator. The comparator also is modified slightly because it has a 100k resistor providing positive feedback from the output to the non-inverting input of the op-amp.

Depending on the mode selected on the chip MAX266 basically behaves as two second-order band-pass filters, giving us a 4th order band-pass filter. The Q value and the gain are adjustable by choosing different resistor values. For the simplest mode of the chip (the one I am using) the first stage has a gain of 22 and a Q of 22 and the second stage has a gain of 10 and a Q of 10, for an overall gain of 220, with a Q of 22.

Ultrasonic Waveguide:

The ultrasonic transducer used for the receiver is not omni-directional. In fact, it has a high attenuation of signals that are greater than about 45º on either side of it. This is a problem since we want to pick up signals from all directions that lie in a plane parallel to and just above the ground. Also it would be nice if the transducer didn’t pick up any sound that was off of this plane. To accomplish this, at least in theory, Marvin uses an ultrasonic waveguide, which consists of two reflecting surfaces, as shown in Fig. 12.

Transducer

Fig. 12. The ultrasonic waveguide. The top surface is right cone and the bottom surface is a paraboloid.

The top surface is a right cone. Ideally, it will reflect sound coming from all radial directions on the plane parallel to the ground directly downward along center of the cone. Sound waves reflecting off-center will be deflected elsewhere and lost. All reflecting paths that reflect downward are the same length, so the resulting waves remain in-phase and thus do not interfere destructively. The second reflecting surface is parabolic so that reflects parallel sound waves onto a point; this is identical to the way a satellite dish works. These paths also travel the same distance, so the sound does not destructively interfere.

#### Ultrasonic Ranging Data

Feet

ms

Fig. 13. Plot of the time delay between sending the RF pulse and receiving the ultrasonic pulse. The data was collected three times with two different RF frequencies (there was no variation between the transmitter/receiver pairs). The data follows pretty near a straight line.

Feet

ms

Fig. 14. Difference between the best-fit line of the median of the data and and the data. This represents the error between the actual data and what we would expect to receive. The root-mean-square error is about one inch. Note however we are cheating here because we are testing with the same data set that we used to get the best-fit value. This is not terrible though here because we are only training two parameters.

### Wireless Serial Interface

The other special sensor is a wireless serial interface chip that provides a transparent serial link between the MP and the PC. The board is a Wi232-DTS from radiotronix.com. They are expensive ($50 for both boards), but they provide for both the layer 1 and layer 2 (medium access control) in the OSI communications model.

### Other Sensors

The other sensors we are using on this robot are infrared (IR) ranging sensors, bump switches and a voltage divider circuit to measure battery voltage. The IR ranging sensors are the Sharp GP2D12 model and two variants. There are a total of five Sharp sensors: two GP2D120 short-range sensors (4-30cm), two GPD12 mid-range sensors (10-80cm) and one 2Y0A02-23 long-range sensor (20-150cm). I have not fully characterized the sensors yet, but I do have some information: All three IR sensors have a field of view of approximately 10º. Preliminary testing seems to confirm that the data sheets are fairly accurate when the object is directly in line with the IR. The IR sensors output an analog voltage that is roughly inversely proportional to the distance to the object in front of them. These voltages are read using the analog-to-digital conversion ports on the ATmega128.

Four of these IR sensors are mounted to fixed locations on the robot and are used for obstacle avoidance. The fifth sensor (the long-range one) is mount atop of an un-hacked servo. This allows the robot to rotate the IR while moving to scan its environment for obstacles or walls, and potentially map out the environment.

Four bump switches are placed between the robot platform and the bump skirt to indicate a collision. These switches are attached to external interrupts on the ATmega128 to asynchronously indicate the collision. Of course, the robot immediately responses to a collision by stopping dead, and indicating a  collision occurred to the PC.

Also, there is a voltage divider from the battery that connects to an analog port and reports this battery voltage back to the PC.

### Behaviors

I have not actually implemented any of the behaviors yet, but I will as soon as I am done with this report. The projected behaviors will be for the MP to orient itself in the room, and then to follow a series of navigation commands specified by the user. The robot will not need to reorient itself because it will adjust its drive parameters as it moves along. It should be able to run around the room and accurately move from place to place as long as the battery doesn’t die. Another desired behavior will be to map a room using the mounted long-range IR sensor. For media day, I would like to have a map of the room displayed on the screen and the user can click on it to instruct the MP to move to that location.

### Conclusion

Marvin is a successful robot that is able to navigate about a room using precision stepper motors and sonar trilateration. Additionally, Marvin is able to avoid obstacles in its path using IR. Marvin is also capable of scanning the room using a servo-mounted IR. Actually, Marvin’s systems are somewhat overkill for its job. The ultrasound trilateration will work well with a MP that has much less accurate drive wheels. Marvin’s accurate drive wheels would work well with a compass (to double check the heading) for navigation. Also, I didn’t get enough time to spend on the applications of Marvin, as I spend all of my time on the hardware. Fortunately, the hardware is working pretty well, and I’ve been spending the last few days on the software.

### References

The references for this project are exclusively datasheets and Internet websites.

The part numbers for the datasheets are given here:

Company		Part Number			Function

Atmel 			ATmega128 			(microcontroller)

Sparkfun.com 		SerLCD-v11			(LCD display w/ serial interface)

TI			SN754410			(Motor driver)

Dallas semiconductor	MAX266			Analog Filter

Radiotronix.com	Wi-232-DTS			Wireless RS-232 transceiver

Laipac			TLP434A/RLP434A		(RF transmitter/receiver pair)

Websites

Ultrasonic receiver:

http://www.convict.lu/Jeunes/beacon/Receiver.htm

Stepper Motor:

http://www.ams2000.com/stepping101.html

http://www.doc.ic.ac.uk/~ih/doc/stepper/

### Appendix A: Schematic Diagrams

Fig. 12. Schematic for the Ultrasonic subsystem’s RF transmitter.

Fig. 2. Schematic for the Beacon.

Fig. 3a. Part of Schematic for the Ultrasonic Receiver.

Fig. 3b. Part of Schematic for the Ultrasonic Receiver.

Fig. 4. Schematic for the Stepper Motor Drivers.

### Appendix B: Code

Algorithms.h

#ifndef _ALGORITHMS_H_

#define _ALGORITHMS_H_

// stole from at http://www.hut.fi/~ccandoli/botanica/algorithms/c/quicksort.c

/*

* Insertionsort, takes two arguments:

* int s[]: the array to be sorted

* int length: the length of the array

*

* Used for optimization purposes.

*/

template< class T, class S >

inline void insertion_sort( T data[], S length )

{

S i, j;

T elem;

for( i=1; i < length; i++ )

{

elem = data[i];

j = i;

while( j > 0 && elem < data[ j-1 ] )

{

data[j] = data[ j-1 ];

j--;

}

data[j] = elem;

}

};

template< class T, class S >

inline T median( T data[], S length )

{

T median_value;

insertion_sort< T,S >( data, length );

// if length is odd, then pick the middle value

if ( length & 0x01 )

{

median_value = data[ length >> 1 ];

}

// if it's even, take the average of the two middle values

else

{

median_value = ( data[ ( length >> 1 ) - 1 ] + data[ length >> 1 ] ) >> 1;

}

return median_value;

};

// computes f(n) = n(n+1)/2 which is the inverse of qsqrt()

inline uint16_t qsqrt_inv( uint8_t x )

{

return ( x*(x+1) ) >> 1;

};

// computes the inverse of f(n) = n(n+1)/2

// puts the remainder in the address of remainder_p

inline uint8_t qsqrt( uint16_t x, uint8_t* remainder_p )

{

uint8_t		result		= 0;

uint8_t		remainder 	= 0;

for ( ; x > 0; x-- )

{

remainder++;

if ( remainder > result )

{

result++;

remainder = 0;

}

}

if ( remainder_p != (uint8_t*)0 )

*remainder_p = remainder;

return result;

};

#endif

Analog.h

#ifndef _ANALOG_H_

#define _ANALOG_H_

void		analog_init();

uint8_t	analog( uint8_t channel );

uint8_t	analog_sum( uint8_t channel, uint8_t num_values );

uint8_t	analog_median( uint8_t channel, uint8_t num_values );

// number of ADC channels we are using

// we must use the first n channels, i.e. ADC0, ADC1, ..., ADCn

enum analog_channel_t

{

ANALOG_BATTERY_VOLTAGE = 0,

ANALOG_IR_TOP_LEFT,

ANALOG_IR_TOP_RIGHT,

ANALOG_IR_BOTTOM_LEFT,

ANALOG_IR_BOTTOM_RIGHT

};

const uint8_t	num_channels_c		= 5;

const uint8_t	ir_num_sensors_c	= 4;

#endif

Analog.c

////////////////////////////////////////////////////////////////////////////////

//////////////////////  Analog to Digital Converter ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

//

// Written by Bryan Davis, 06/15/2004

//

////////////////////////////////////////////////////////////////////////////////

//

//

//

////////////////////////////////////////////////////////////////////////////////

/////////////////////////  Pre-Processor Directives ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

#include <inttypes.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/signal.h>

#include "clock.h"

#include "circularbuffer.h"

#include "algorithms.h"

#include "analog.h"

////////////////////////////////////////////////////////////////////////////////

///////////////////////////////  Typedefs //////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

enum analog_clk_mode_t

{

ACLK_SELECT_2	= 0,

//	ACLK_SELECT_2b	= 1, // redundant

ACLK_SELECT_4 	= 2,

ACLK_SELECT_8	= 3,

ACLK_SELECT_16	= 4,

ACLK_SELECT_32	= 5,

ACLK_SELECT_64	= 6,

ACLK_SELECT_128	= 7

};

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Constants  /////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

// number of values to remember

const uint8_t	buffer_length_c		= 10;

// value of the ADMUX register using channel 0.

// this value is determined by the ADC settings

// we only need 8-bit precision, so left justify the 10-bit result so that

// the MSB of the value are in 1 register => ADLAR = 1

// Also we want to use AVCC as the reference, so REFS1:0 = 0b01

const uint8_t	admux_base_c		= _BV( REFS0 ) | _BV( ADLAR );

////////////////////////////////////////////////////////////////////////////////

/////////  Non-volatile Global Variables (modified synchronously) //////////////

////////////////////////////////////////////////////////////////////////////////

uint8_t temp_buffer[ buffer_length_c ];

////////////////////////////////////////////////////////////////////////////////

/////////  Volatile Global Variables (modified asychronously) //////////////////

////////////////////////////////////////////////////////////////////////////////

// memory allocated to store the analog data

// buffer_data stores volatile data

volatile uint8_t 	buffer_data[ num_channels_c ][ buffer_length_c ];

// circular buffer wrapper to manage the wrap-around condition

// and synchronous access to the data

CircularBuffer< volatile uint8_t, uint8_t > buffer[ num_channels_c ];

// buffer position to place newly read data

volatile uint8_t 	buffer_count_gv;

// channel we are currently processing

volatile uint8_t 	current_channel_gv;

// channel that contains the latest data

volatile uint8_t	latest_channel_gv;

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Definitions ///////////////////////////

////////////////////////////////////////////////////////////////////////////////

void analog_init()

{

// initialize the CircularBuffer data structure

for ( uint8_t channel = 0; channel < num_channels_c; channel++ )

{

buffer[ channel ].init( buffer_data[ channel ], buffer_length_c );

}

// initialize current_channel and buffer_count

current_channel_gv	= 0;

buffer_count_gv		= 0;

// initialize ADMUX to channel 0 using the settings of admux_base_c

ADMUX = admux_base_c;

// enable ADC - ADEN = 1

// start conversion - ADSC = 1

// don't use free running, let interrupt handler do that, ADFR = 0

// enable the interrupt

// use the slow clock = SYS_CLK/128 = 1/(8us) = 125kHz

ADCSRA = _BV( ADEN ) | _BV( ADSC ) | _BV( ADIE ) | ACLK_SELECT_128;

// Note that values won't be updated until after sei()

}

// returns the most recent value of the given channel

uint8_t analog( uint8_t channel )

{

return buffer[ channel ][0];

}

// returns a sum of the last N values

// N  must be a few less than buffer_length_c to ensure proper operation

uint8_t analog_sum( uint8_t channel, uint8_t num_values )

{

uint16_t sum = 0;

// hold the current index to prevent asynchronous updating of the

// buffer index

buffer[ channel ].hold_index();

// sum the values

for ( uint8_t i = 0; i < num_values; i++ )

{

sum += buffer[ channel ][ i ];

}

// release the hold

buffer[ channel ].release_index();

return sum;

}

uint8_t analog_median( uint8_t channel, uint8_t num_values )

{

// hold the current index to prevent asynchronous updating of the

// buffer index

buffer[ channel ].hold_index();

// copy the values

for ( uint8_t i = 0; i < num_values; i++ )

{

temp_buffer[i] = buffer[ channel ][i];

}

// release the hold

buffer[ channel ].release_index();

// find the median of the values - this trashes the temp buffer,

// but it doesn't matter here

return median< uint8_t, uint8_t >( temp_buffer, num_values );

}

////////////////////////////////////////////////////////////////////////////////

//////////////////// Interrupt Service Rountines ///////////////////////////////

////////////////////////////////////////////////////////////////////////////////

//

// This ISR is called when the previous analog-to-digital conversion is

// complete. It places the value of the conversion into the circular buffer

// of the channel just converted. Then it sets up for a conversion on

// the next channel. If all channels are complete, it goes back to channel 0.

// Finally, it initiates the next conversion.

//

////////////////////////////////////////////////////////////////////////////////

SIGNAL( SIG_ADC )

{

// place the conversion value into the buffer for this channel

buffer[ current_channel_gv ].push( ADCH );

// increment the channel for the next count

current_channel_gv++;

// if this is the last channel go back to the beginning

if ( current_channel_gv == num_channels_c )

{

current_channel_gv = 0;

}

// specify next channel to convert

ADMUX = admux_base_c + current_channel_gv;

// start the next conversion

ADCSRA |= _BV( ADSC );

}

////////////////////////////////////////////////////////////////////////////////

/////////////////////////// End of file ////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

Bump.h

#ifndef _BUMP_H_

#define _BUMP_H_

#include <inttypes.h>

enum bump_switch_t

{

BUMP_RIGHT = 0,

BUMP_FRONT,

BUMP_LEFT,

BUMP_REAR

};

const uint8_t bump_port_offset_c 	= 4;

const uint8_t bump_num_switches_c	= 4;

void	bump_init();

uint8_t bump_state();

bool	bump_state( bump_switch_t side );

void	bump_reset();

#endif

Bump.c

////////////////////////////////////////////////////////////////////////////////

////////////////////////////  Bump switch sensor ///////////////////////////////

////////////////////////////////////////////////////////////////////////////////

// Written by Bryan Davis, 07/28/2004

//

// This module calls an interrupt whenever the bump switches are pressed.

// bump_emergency_isr() can be configured to call any code asynchronously when

// a bump switch is activated.

//

////////////////////////////////////////////////////////////////////////////////

// Routines:

//

// bump_init()

//		Initializes the interrupt routines.

//

// bump_emergency_isr()

//		Called whenever a bump switch is pressed.

//

////////////////////////////////////////////////////////////////////////////////

/////////////////////////  Pre-Processor Directives ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

#include <stdio.h>

#include <inttypes.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/signal.h>

#include "external.h"

#include "bump.h"

#include "lcd.h"

#include "stepper_synch.h"

////////////////////////////////////////////////////////////////////////////////

///////////////////////////////  Typedefs //////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Constants  /////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

const uint8_t	port_mask_c = _BV( INT4 ) | _BV( INT5 ) | _BV( INT6 ) | _BV( INT7 );

////////////////////////////////////////////////////////////////////////////////

/////////////////////////  Global Non-Volatiles  ///////////////////////////////

////////////////////////////////////////////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

///////////////////////////  Global Volatiles  /////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

volatile uint8_t	bump_state_gv;

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Prototypes ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Definitions ///////////////////////////

////////////////////////////////////////////////////////////////////////////////

void bump_init()

{

// enable portE inputs with pull-up resistors on the interrupts

DDRE	&= ~port_mask_c;

PORTE 	|= port_mask_c;

// enable ISRs and trigger on falling edge for all interrupts

for ( uint8_t interrupt = 4; interrupt < 7; interrupt++ )

{

external_sense( interrupt, EXTERNAL_SENSE_FALLING );

external_interrupt( interrupt, true );

}

}

// reads the bump state for all switches

uint8_t bump_state( )

{

return ( bump_state_gv | ~PINE ) & port_mask_c;

}

// resets the bump state for all switches

void bump_reset()

{

bump_state_gv = 0;

}

// reads the bump state for an individual switch

bool bump_state( bump_switch_t side )

{

return ( ( bump_state_gv | ~PINE ) & _BV( side + bump_port_offset_c ) ) != 0;

}

////////////////////////////////////////////////////////////////////////////////

//////////////////// Interrupt Service Rountines ///////////////////////////////

////////////////////////////////////////////////////////////////////////////////

//

//	SIG_EXT1()

//		This ISR is called when the sonar receiver pulses the input capture

//		line. Since sonar_ping clears the TCNT1 counter register, the value in

//		ICR1 is the number of clock cycles between the ping and the pong. THis

//		routine records this value in pong_value_gv[ current_beacon_gv ], and

//		turns off the two ISRs.

//

//	SIG_OUTPUT_COMPARE1A()

//		This ISR handles sending an RF preamble to initialize the RF receivers

//		in the beacons. It also handles sending the a pulse to the desired sonar

//		beacon.

//

////////////////////////////////////////////////////////////////////////////////

// this routine is called when any bump switch is activated

inline void bump_emergency_isr()

{

// place emergency code here

sei();

stepper_halt();

lcd_line( 0, "Break Yo self!" );

}

// the right bump switch

SIGNAL( SIG_INTERRUPT4 )

{

bump_state_gv |= _BV( BUMP_RIGHT );

bump_emergency_isr();

}

// the front bump switch

SIGNAL( SIG_INTERRUPT5 )

{

bump_state_gv |= _BV( BUMP_FRONT );

bump_emergency_isr();

}

// the left bump switch

SIGNAL( SIG_INTERRUPT6 )

{

bump_state_gv |= _BV( BUMP_LEFT );

bump_emergency_isr();

}

// the rear bump switch

SIGNAL( SIG_INTERRUPT7 )

{

bump_state_gv |= _BV( BUMP_REAR );

bump_emergency_isr();

}

////////////////////////////////////////////////////////////////////////////////

/////////////////////////// End of file ////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

Circularbuffer.h

#ifndef _CIRCULAR_BUFFER_H_

#define _CIRCULAR_BUFFER_H_

// implements a circular buffer

//

// Because we use this queue for real-time systems,

// the front and the back of the queue must be volatile pointers

// to volatile data.

#include <inttypes.h>

typedef uint8_t S;

template< class T, class S >

class CircularBuffer

{

public:

CircularBuffer( T *buffer, S length )

:	m_data			( buffer ),

m_front_index	( 0 ),

m_length		( length )

{ };

// default constructor

CircularBuffer() { };

void init( T *buffer, S length )

{

m_data			= buffer;

m_front_index	= 0;

m_length		= length;

};

// returns the data 'n' time steps in the past

T operator[] ( S n ) const

{

if ( m_hold )

n += m_front_index;

else

n += m_buffered_front_index;

if ( n >= m_length )

n -= m_length;

return m_data[ n ];

};

// add a new value to the buffer.

// called by ISR and therefore not designed to be interruptted

void push( T value )

{

// decrement the front index

if ( m_front_index == 0 )

m_front_index = m_length - 1;

else

m_front_index--;

// now write the new value

m_data[ m_front_index ] = value;

};

// call this function before performing a series

// of reads from the buffer that except the data

// to be synchronized. Must call release_index()

// afterward.

void hold_index() const

{

m_buffered_front_index = m_front_index;

m_hold = true;

};

// called after all synchronized operations are performed

// after a call to hold_index()

void release_index() const

{

m_hold = false;

};

private:

// pointer to the buffer defined externally

// it is a pointer to volatile data

volatile	T* 		m_data;

// length of the buffer

S	m_length;

// index that contains the lastest value

// it is modified by the ISRs, so it may change during

// a synchronous function call

volatile	S 	m_front_index;

// a buffered index that only changes synchronously

mutable	S	m_buffered_front_index;

mutable	bool	m_hold;

};

#endif

CircularQueue.h

#ifndef _CIRCULAR_QUEUE_H_

#define _CIRCULAR_QUEUE_H_

// implements a circular queue

//

// Because we use this queue for real-time systems,

// the front and the back of the queue must be volatile pointers

// to volatile data.

#include <stdlib.h> // needed for malloc

template< class T >

class CircularQueue

{

public:

// use this when memory is allocated elsewhere (e.g. globally)

CircularQueue( T *buffer, unsigned int length )

:	m_bottom( buffer ),

m_top	( buffer + length ), 	// past end of buffer

m_front	( buffer ), 			// remove this element from the queue

m_back	( buffer ), 			// add new element here (1 past back of queue)

m_overflow( false ),

m_alloc_here( false )			// buffer allocated elsewhere

{ };

// memory allocation is handled by this class (no worries)

CircularQueue( unsigned int length )

:	m_overflow( false ),

m_alloc_here( true )

{

m_bottom 		= (T*)malloc( sizeof(T)*length );

m_top 			= m_bottom + length;

m_front			= m_bottom;

m_back			= m_bottom;

};

// destructor to clear allocated memory

~CircularQueue()

{

if ( m_alloc_here )

{

free( (void*)m_bottom );

}

};

void init( T *buffer, unsigned int length )

{

// if memory was previously allocated, free it

if ( allocated_here )

{

free( (void*)m_bottom );

}

m_bottom 		= buffer;

m_top			= buffer + length; 	// past end of buffer

m_front			= buffer; 			// remove this element from the queue

m_back			= buffer; 			// add new element here (1 past back of queue)

m_overflow		= false;

m_alloc_here	= false;			// buffer allocated elsewhere

};

void init( unsigned int length )

{

// if memory was previously allocated, free it

if ( allocated_here )

{

free( (void*)m_bottom );

}

m_bottom 		= (T*)malloc( sizeof(T)*length );

m_top 			= m_bottom + length;

m_front			= m_bottom;

m_back			= m_bottom;

m_overflow		= false;

m_alloc_here	= true;			// buffer allocated here

};

// put a new element into the back of the queue

void put( T x )

{

*m_back = x;	// place data at back of queue

m_back++;		// move back of queue up 1

//	wrap around circular queue

if ( m_back == m_top )

{

m_back = m_bottom;

}

// check for buffer overflow

if ( m_back == m_front )

{

m_overflow = true;

}

};

// remove an element from the front of the queue

T get()

{

// check if queue is empty

if ( m_front != m_back )

{

T ret_value = *m_front;	// get data from front of queue

m_front++;				// move front of queue backward

// wrap-around condition

if ( m_front == m_top )

{

m_front = m_bottom;

}

return ret_value;

}

else

{

return 0;

}

};

// returns true if the queue is empty

bool is_empty()

{

return m_front == m_back;

};

// remove all elements from the queue

void clear()

{

m_front 	= m_bottom;

m_back  	= m_bottom;

m_overflow	= false;

};

// check for an overflow condition

bool overflow()

{

return m_overflow;

};

unsigned int length()

{

unsigned int ret_value;

if ( m_back >= m_front )

{

ret_value = m_back - m_front;

}

else

{

ret_value = ( m_top - m_front ) + ( m_back - m_bottom );

}

return ret_value;

};

// reset the overflow flag

void reset_overflow()

{

m_overflow = false;

};

// print the contents of the queue

void print()

{

volatile T* temp = m_front;

while ( temp != m_back )

{

fputc( *temp, stdout );

temp++;

if ( temp == m_top ) temp = m_bottom;

}

};

private:

T* m_bottom;			// bottom of queue

T* m_top;				// one past top of queue

T* volatile m_front;	// next element to remove

T* volatile m_back;	// place new element here

volatile bool m_overflow;		// true if buffer has overflowed

bool	m_alloc_here;		// true if this class does memory management

};

#endif

Clock.h

// clock.h

#ifndef _CLOCK_H_

#define _CLOCK_H_

#include <avr/delay.h>

const uint32_t 	SYS_CLK 			= 16000000;

const uint16_t	SYS_CLK_PER_MS 		= SYS_CLK / 1000;

const uint8_t	SYS_CLK_PER_US		= SYS_CLK / 1000000;

const uint8_t	USART_CLK_DIVIDER 	= 16;

inline void wait( uint16_t milliseconds )

{

for ( uint16_t i = 0; i < milliseconds; i++ )

{

// 4 cycles per loop

_delay_loop_2( SYS_CLK_PER_MS / 4 );

}

};

inline void wait_tenths( uint16_t tenths_milliseconds )

{

for ( uint16_t i = 0; i < tenths_milliseconds; i++ )

{

// 4 cycles per loop

_delay_loop_2( SYS_CLK_PER_MS / 40 );

}

};

#endif

Comm.h

#ifndef _COMM_H_

#define _COMM_H_

////////////////////////////////////////////////////////////////////////////////

/////////////////////////  Pre-Processor Directives ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

#include <inttypes.h>

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Constants  /////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

enum comm_rx_command_t

{

COMM_RX_MOVE = 1,

COMM_RX_STOP,

COMM_RX_HALT,

COMM_RX_SERVO,

COMM_RX_LCD_LINE0,

COMM_RX_LCD_LINE1,

COMM_RX_RESET,

COMM_RX_INVALID

};

enum comm_tx_command_t

{

COMM_TX_SONAR = 1,

COMM_TX_IR,

COMM_TX_BUMP,

COMM_TX_MOVEMENT,

COMM_TX_BATTERY,

COMM_TX_RESET

};

// don't include reset

const uint8_t comm_num_tx_commands_c = 5;

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Prototypes ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

void comm_rx_command();

void comm_tx_command( uint8_t command );

void comm_rx_packet( uint8_t *command_p, uint8_t data[] );

void comm_tx_packet( uint8_t command, uint8_t data[] );

inline uint8_t comm_high_byte( uint16_t value )

{

return (uint8_t) ( value >> 8 );

};

inline uint8_t comm_low_byte( uint16_t value )

{

return (uint8_t) ( value );

};

////////////////////////////////////////////////////////////////////////////////

/////////////////////////// End of file ////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

#endif

Comm.c

////////////////////////////////////////////////////////////////////////////////

/////////////////////////  Pre-Processor Directives ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

#include <stdio.h>

#include <inttypes.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/signal.h>

#include "clock.h"

#include "timer.h"

#include "comm.h"

#include "wi232.h"

#include "sonar.h"

#include "stepper_synch.h"

#include "analog.h"

#include "bump.h"

#include "lcd.h"

#include "servo.h"

////////////////////////////////////////////////////////////////////////////////

///////////////////////////////  Typedefs //////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Constants  /////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

const uint8_t start_byte_c				= 0xFE;

const uint8_t fill_byte_c				= 0xFF;

const uint8_t comm_rx_data_length_c	= 17;

const uint8_t comm_tx_data_length_c 	= 6;

const uint8_t comm_rx_packet_length_c	= comm_rx_data_length_c + 2;

const uint8_t comm_tx_packet_length_c 	= comm_tx_data_length_c + 2;

////////////////////////////////////////////////////////////////////////////////

/////////////////////////  Global Non-Volatiles  ///////////////////////////////

////////////////////////////////////////////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

///////////////////////////  Global Volatiles  /////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Prototypes ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Definitions ///////////////////////////

////////////////////////////////////////////////////////////////////////////////

void comm_rx_command()

{

static uint8_t command;

static uint8_t data[ comm_rx_data_length_c ];

// get a packet and some data

comm_rx_packet( &command, data );

// DEBUG

sprintf( lcd_buffer(), "%d %d %d", data[0], data[1], data[2] );

lcd_line( 1, lcd_buffer() );

switch( command )

{

case COMM_RX_MOVE:

{

uint8_t  direction = data[ 0 ];

uint16_t steps = data[ 1 ]*0x100 + data[ 2 ];

stepper_move( steps, (stepper_direction_t)( direction ) );

}

break;

case COMM_RX_STOP:

{

stepper_stop();

}

break;

case COMM_RX_HALT:

{

stepper_halt();

}

break;

case COMM_RX_SERVO:

{

servo_angle( data[0] );

}

break;

case COMM_RX_LCD_LINE0:

{

lcd_line( 0, (char*)data );

}

break;

case COMM_RX_LCD_LINE1:

{

lcd_line( 1, (char*)data );

}

break;

case COMM_RX_INVALID:

default:

break;

}

}

// send out the specified packet

void comm_tx_command( uint8_t command )

{

static uint8_t i;

static uint8_t data[ comm_tx_data_length_c ];

switch ( command )

{

case COMM_TX_SONAR:

{

uint16_t pong_value;

// send out the sonar data

for ( i = 0; i < sonar_num_beacons_c; i++ )

{

pong_value = sonar_pong(i);

data[2*i] 	= comm_high_byte( pong_value );

data[2*i+1]	= comm_low_byte ( pong_value );

}

i *=2;

}

break;

case COMM_TX_IR:

{

for ( i = 0; i < ir_num_sensors_c; i++ )

{

data[i] = analog(i+1);

}

}

break;

case COMM_TX_BUMP:

{

for ( i = 0; i < bump_num_switches_c; i++ )

{

data[i] = bump_state( (bump_switch_t) i );

}

}

break;

case COMM_TX_MOVEMENT:

{

uint16_t steps_remaining = stepper_steps_remaining();

data[0] = !stepper_movement_stopped();

data[1] = comm_high_byte( steps_remaining );

data[2] = comm_low_byte ( steps_remaining );

i = 3;

}

break;

case COMM_TX_BATTERY:

{

data[0] = analog(0);

i = 1;

}

break;

case COMM_TX_RESET:

break;

}

// pad the packet with fill bytes (0xFF)

for ( ; i < comm_tx_data_length_c; i++ )

{

data[i] = fill_byte_c;

}

// send the packet

comm_tx_packet( command, data );

}

void comm_rx_packet( uint8_t *command_p, uint8_t data[] )

{

// check if an entire packet is available on the queue

if ( wi232_data_available() >= comm_rx_packet_length_c )

{

// check if the first byte is valid

if ( wi232_getchar() == start_byte_c )

{

// if so, read the packet contents

*command_p = wi232_getchar();

for ( uint8_t i = 0; i < comm_rx_data_length_c; i++ )

{

data[i] = wi232_getchar();

}

}

else

{

*command_p = COMM_RX_INVALID;

}

}

else

{

*command_p = COMM_RX_INVALID;

}

}

void comm_tx_packet( uint8_t command, uint8_t data[] )

{

// turn off interrupts to ensure that

uint8_t sreg = SREG;

cli();

wi232_putchar( start_byte_c );

wi232_putchar( command );

for ( uint8_t i = 0; i < comm_tx_data_length_c; i++ )

{

wi232_putchar( data[i] );

}

SREG = sreg;

}

////////////////////////////////////////////////////////////////////////////////

/////////////////////////// End of file ////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

External.h

#ifndef _EXTERNAL_H_

#define _EXTERNAL_H_

#include <inttypes.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/signal.h>

#include "clock.h"

enum external_sense_t

{

EXTERNAL_SENSE_LOW_LEVEL	= 0,

//  RESERVED				 	= 1,

EXTERNAL_SENSE_FALLING		= 2,

EXTERNAL_SENSE_RISING		= 3

};

const uint8_t external_sense_mask = 0x03;

inline void external_sense( uint8_t interrupt, external_sense_t sense )

{

switch ( interrupt )

{

case 0:

EICRA |= ( sense << ISC00 ) &	( external_sense_mask << ISC00 );

EICRA &= ( sense << ISC00 ) | ~( external_sense_mask << ISC00 );

break;

case 1:

EICRA |= ( sense << ISC10 ) &	( external_sense_mask << ISC10 );

EICRA &= ( sense << ISC10 ) | ~( external_sense_mask << ISC10 );

break;

case 2:

EICRA |= ( sense << ISC20 ) &	( external_sense_mask << ISC20 );

EICRA &= ( sense << ISC20 ) | ~( external_sense_mask << ISC20 );

break;

case 3:

EICRA |= ( sense << ISC30 ) &	( external_sense_mask << ISC30 );

EICRA &= ( sense << ISC30 ) | ~( external_sense_mask << ISC30 );

break;

case 4:

EICRB |= ( sense << ISC40 ) &	( external_sense_mask << ISC40 );

EICRB &= ( sense << ISC40 ) | ~( external_sense_mask << ISC40 );

break;

case 5:

EICRB |= ( sense << ISC50 ) &	( external_sense_mask << ISC50 );

EICRB &= ( sense << ISC50 ) | ~( external_sense_mask << ISC50 );

break;

case 6:

EICRB |= ( sense << ISC60 ) &	( external_sense_mask << ISC60 );

EICRB &= ( sense << ISC60 ) | ~( external_sense_mask << ISC60 );

break;

case 7:

EICRB |= ( sense << ISC70 ) &	( external_sense_mask << ISC70 );

EICRB &= ( sense << ISC70 ) | ~( external_sense_mask << ISC70 );

break;

}

}

inline void external_interrupt( uint8_t interrupt, bool enable )

{

if ( enable )

EIMSK |= _BV( interrupt );

else

EIMSK &= ~_BV( interrupt );

}

#endif

Lcd.h

// LCD.c: (actually a C++ file, but I haven't figured how to use make yet!)

//

// Call fopen_LCD() to set up the LCD as an output data stream.

//

// Uses Interupts to handle data streaming. The LCD uses USART1, which uses:

//		Tx pin = PORTD3 = pin 28

#ifndef _LCD_H_

#define _LCD_H_

#include <stdio.h>

// use BAUD_RATE baud 8N1 TX on USART1

// to transmit to the lcd

FILE* fopen_lcd();

void lcd_clear();

void lcd_blacklight_on();

void lcd_blacklight_off();

void lcd_goto_line( uint8_t line );

void lcd_line( uint8_t line, char string[] );

char* lcd_buffer();

#endif

Lcd.c

// LCD.c: (actually a C++ file, but I haven't figured how to use make yet!)

//

// Call fopen_LCD() to set up the LCD as an output data stream.

//

// Uses Interupts to handle data streaming. The LCD uses USART1, which uses:

//		Tx pin = PORTD3 = pin 28

#include <inttypes.h>

#include <stdio.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/signal.h>

#include "usart.h"

#include "lcd.h"

#include "circularqueue.h"

const int	chars_in_line_c = 16;

const long	BAUD_RATE_LCD = 9600;

const int	LCD_TX_BUFFER_SIZE = 100;

char lcd_buffer_g[ chars_in_line_c + 1 ];

// circular queues used for the USART

CircularQueue< char > lcd_tx_q( LCD_TX_BUFFER_SIZE );

FILE * lcd_out = NULL;

// counts the position of the LCD, since clear doesn't seem to work

// uint8_t lcd_cursor_pos = 0; TMP

// prototype

int lcd_putchar( char );

// use BAUD_RATE_LCD baud 8N1 TX on USART1

// to transmit to the console

FILE* fopen_lcd()

{

// set baud rate to BAUD_RATE_CONSOLE

usart1_set_baud_rate( BAUD_RATE_LCD );

// normal speed, no multi-processor

UCSR1A = 0;

// enable Tx flag

// (Tx buffer empty interupt is enabled when we actually send something)

UCSR1B = _BV( TXEN1 );

// UCSC10 = 1 & UCSC11 = 1 -> 8-bit data

UCSR1C = _BV( UCSZ10 ) | _BV( UCSZ11 );

// commands to set the display

lcd_putchar( 0x7c );

lcd_putchar( 0x04 );

lcd_putchar( 0x7c );

lcd_putchar( 0x06 );

// clear the screen

lcd_clear();

// set console_putchar as the FILE ostream character Tx-er

lcd_out = fdevopen( lcd_putchar, NULL, 0 );

return lcd_out;

}

char *lcd_buffer()

{

return lcd_buffer_g;

}

int lcd_putchar( char x )

{

// push data onto the Tx queue

lcd_tx_q.put( x );

// enable the "data register empty" interupt

// (it may already be enabled in which case no harm is done)

UCSR1B |= _BV( UDRIE1 );

// return -1 if overflow is true, otherwise return 0

// return console_tx_q.overflow() ? -1 : 0;

return 0; // for now, screw overflow - just ignore it

}

// since clear (0x01) doesn't work, write spaces until the end of the display

void lcd_clear()

{

lcd_putchar( 0xfe );

lcd_putchar( 0x01 );

}

void lcd_blacklight_on()

{

lcd_putchar( 0x7c );

lcd_putchar( 0x01 );

}

void lcd_blacklight_off()

{

lcd_putchar( 0x7c );

lcd_putchar( 0x02 );

}

void lcd_goto_line( uint8_t line )

{

lcd_putchar( 0xfe );

if ( line == 0 )

lcd_putchar( 0x80 );

else // line == 1

lcd_putchar( 0xc0 );

}

void lcd_line( uint8_t line, char string[] )

{

lcd_goto_line( line );

bool end_of_string = false;

for ( int pos = 0; pos < chars_in_line_c; pos++ )

{

if ( !end_of_string )

{

if ( string[ pos ] != 0 )

{

lcd_putchar( string[ pos ] );

}

else

{

end_of_string = true;

lcd_putchar(' '); // write a space

}

}

else // end_of_string

{

lcd_putchar(' ');

}

}

}

// the USART is ready to queue another byte to send

SIGNAL( SIG_UART1_DATA )

{

if ( !lcd_tx_q.is_empty() )

{

// if the queue is not empty, put the next character in the buffer

// this also resets the interupt flag automatically

UDR1 = lcd_tx_q.get();

}

else

{

// disable the interupt flag manually

UCSR1B &= ~_BV( UDRIE1 );

}

}

Main.c

#include <inttypes.h>

#include <avr/interrupt.h>

#include <avr/signal.h>

#include <avr/io.h>

#include "lcd.h"

#include "analog.h"

#include "clock.h"

#include "wi232.h"

#include "comm.h"

#include "stepper_synch.h"

#include "bump.h"

#include "sonar.h"

#include "servo.h"

int main()

{

fopen_wi232();

fopen_lcd();

analog_init();

sonar_init();

stepper_init();

servo_init();

stepper_on();

sei();

wait(2000);

lcd_clear();

lcd_line( 0, "Hello World!!" );

comm_tx_command( COMM_TX_RESET );

wait(2000);

while ( 1 )

{

// SONAR is first command, BATTERY is last command

for ( comm_tx_command_t command = COMM_TX_SONAR; command <= COMM_TX_BATTERY; ((uint8_t)command)++ )

{

comm_rx_command();

comm_tx_command( command );

wait(20);

}

}

}

Servo.h

#ifndef _SERVO_H_

#define _SERVO_H_

#include <inttypes.h>

void servo_init();

void servo_angle( uint8_t pulse_width );

#endif

Servo.c

#include <inttypes.h>

#include <avr/io.h>

#include "clock.h"

#include "timer.h"

#include "servo.h"

const uint8_t	ANGLE_ZERO		= 27;

const uint8_t	ANGLE_LEFT_90	= 14;

const uint8_t ANGLE_RIGHT_90	= 40;

void servo_init()

{

OCR0 = ANGLE_ZERO;

timer0_clk_asynch( true );

timer0_clk_select( CLK0_SELECT_1 );

timer0_pwm_mode( PWM8_PHASE );

timer0_output_compare_mode( OUT_COMP_SET_BELOW );

DDRB |= _BV( PORTB4 );

}

void servo_angle( uint8_t pulse_width )

{

OCR0 = pulse_width;

}

Sonar.h

#ifndef _SONAR_H_

#define _SONAR_H_

////////////////////////////////////////////////////////////////////////////////

/////////////////////////  Pre-Processor Directives ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

#include <inttypes.h>

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Constants  /////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

const uint8_t 		sonar_num_beacons_c			= 3;

const uint16_t 	sonar_invalid_pong_c		= 0;

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Prototypes ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

void 		sonar_init();

uint16_t 	sonar_pong( uint8_t beacon );

uint16_t 	sonar_ping_count();

uint16_t 	sonar_pong_count();

uint8_t	sonar_update();

bool		sonar_update( uint8_t beacon );

////////////////////////////////////////////////////////////////////////////////

/////////////////////////// End of file ////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

#endif

Sonar.c

////////////////////////////////////////////////////////////////////////////////

////////////////////////  Sonar Pulse Rx-er and RF Tx-er ///////////////////////

////////////////////////////////////////////////////////////////////////////////

// Written by Bryan Davis, 07/28/2004

//

// This module contains routines that control the triangulation sonar system.

// The routines allow the user to tell each sonar transmitter to send a RF ping

// signal to remote sonar transmitters. The transmitters will then send an

// ultrasonic pong back to the robot's sonar receiver. The ISRs determine the

// time delay between the RF/sonar ping-pong, and put the resulting time in

// a variable, which the user can request at a later time.

//

// This module uses timer 1.

//

////////////////////////////////////////////////////////////////////////////////

// Routines:

//

// sonar_init()

//		Initializes the timers and primes the remote RF receivers by transmitting

//		a sequence of 1s and 0s to calibrate the remote receivers automatic gain

//		control. It also calls the subroutine sonar_init_filter_clock().

//

// sonar_init_filter_clock()

//		Subroutine that enables a 2 MHz PWM signal that is required by the

//		sonar receiver's switched-capacitor filter on output compare pin OC2.

//

// sonar_ping( beacon )

//		Tells the sonar to ping a particular beacon with an RF signal. It also

//		enables the ISRs to receive the resulting pong.

//

// bool sonar_waiting()

//		Returns true if the sonar pong has not yet been received within the

//		timeout period. Returns false, after a pong is received or a timeout

//		occurs.

//

// uint16_t sonar_pong( beacon )

//		Returns the time in timer clock cycles for the pong to be received.

//		If the pong is not received before the timeout ends, then it returns

//		sonar_pong_invalid_c.

//

////////////////////////////////////////////////////////////////////////////////

/////////////////////////  Pre-Processor Directives ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

#include <stdio.h>

#include <inttypes.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/signal.h>

#include "clock.h"

#include "timer.h"

#include "sonar.h"

//#include "lcd.h"

////////////////////////////////////////////////////////////////////////////////

///////////////////////////////  Typedefs //////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

enum tx_state_t

{

TX_STATE_PREAMBLE,

TX_STATE_QUIET,

TX_STATE_PING,

TX_STATE_PONG

};

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Constants  /////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

const uint16_t	sonar_pulse_length_c		= 1000; // 4 ms at SYS_CLK/64

const uint16_t	pong_timeout_c				= 5000; // 20 ms

const timer123_clk_mode_t sonar_clock_c		= CLK_SELECT_64;

// transmitters to send a preamble to

const uint8_t 	preamble_tx_mask_c			= 0xFF >> ( 8 - sonar_num_beacons_c );

const uint8_t 	preamble_bits_c				= 10; 		// must be even!!

const uint16_t	preamble_bit_duration_c		= 1000; 	// 4 ms

const uint16_t	post_preamble_delay_c		= 9999; 	// 40 ms

const uint16_t	pre_preamble_delay_c 		= 1000;

////////////////////////////////////////////////////////////////////////////////

/////////////////////////  Global Non-Volatiles  ///////////////////////////////

////////////////////////////////////////////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

///////////////////////////  Global Volatiles  /////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

volatile tx_state_t	tx_state_gv;

volatile uint8_t		current_beacon_gv;

volatile uint8_t 		preamble_bit_count_gv;

volatile uint16_t 	pong_value_gv[ sonar_num_beacons_c ];

volatile uint16_t		ping_count_gv;

volatile uint16_t		pong_count_gv;

volatile uint8_t		sonar_update_gv;

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Prototypes ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

void init_filter_clock();

void rf_pulse( uint8_t beacon, bool on );

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Definitions ///////////////////////////

////////////////////////////////////////////////////////////////////////////////

void sonar_init()

{

init_filter_clock();

timer1_clk_select( sonar_clock_c );

// init the input capture hardware

timer1_input_capture_noise_canceller( true );

timer1_input_capture_edge( INPUT_CAPTURE_RISING );

// initialize the first N PORTA pins to output the RF pulses

for ( uint8_t i = 0; i < sonar_num_beacons_c; i++ )

{

DDRA |= _BV( i );

}

// initialize the ISR global volatiles

tx_state_gv				= TX_STATE_PREAMBLE;

current_beacon_gv 		= 0;

preamble_bit_count_gv	= 0;

ping_count_gv			= 0;

pong_count_gv			= 0;

sonar_update_gv			= 0;

for ( uint8_t i = 0; i < sonar_num_beacons_c; i++ )

{

pong_value_gv[ i ] = sonar_invalid_pong_c;

}

// enable the output compare interrupt

timer1a_output_compare_interrupt( true );

// set OCR1A and TCNT1 to immediately start the OC1A interrupt

OCR1A = 0;

timer1_write_counter( 0 );

}

// returns the value in the sonar array

uint16_t sonar_pong( uint8_t beacon )

{

// reset the update bit

sonar_update_gv &= ~_BV( beacon );

return pong_value_gv[ beacon ];

}

uint8_t sonar_update()

{

return sonar_update_gv;

}

bool sonar_update( uint8_t beacon )

{

return sonar_update_gv & _BV( beacon );

}

// returns the number of pings sent

uint16_t sonar_ping_count()

{

return ping_count_gv;

}

// returns the number of pongs received

uint16_t sonar_pong_count()

{

return pong_count_gv;

}

// sets PORTB7 to output a 2MHz clock signal for the MAX266 switched cap filter

inline void init_filter_clock()

{

timer2_pwm_mode( PWM8_CTC );

timer2_clk_select( CLK_SELECT_1 );

timer2_output_compare_mode( OUT_COMP_TOGGLE );

// For CTC,

// oc2_period = sys_clk_period * 2 * ( 1 + OCR2 )

// duty cycle = 50%

// period = 1/2MHz = 8 * 1/16MHz => OCR2 = 3

OCR2 = 3;

// set OC2 = PORTB7 pin for output

DDRB |= _BV( PORTB7 );

}

// move to the next beacon

inline void increment_current_beacon()

{

current_beacon_gv++;

if ( current_beacon_gv >= sonar_num_beacons_c )

{

current_beacon_gv = 0;

}

}

void rf_pulse( uint8_t beacon, bool on )

{

if ( on )

PORTA |= _BV( beacon );

else

PORTA &= ~BV( beacon );

}

////////////////////////////////////////////////////////////////////////////////

//////////////////// Interrupt Service Rountines ///////////////////////////////

////////////////////////////////////////////////////////////////////////////////

//

//	SIG_INPUT_CAPTURE1()

//		This ISR is called when the sonar receiver pulses the input capture

//		line. Since sonar_ping clears the TCNT1 counter register, the value in

//		ICR1 is the number of clock cycles between the ping and the pong. THis

//		routine records this value in pong_value_gv[ current_beacon_gv ], and

//		turns off the two ISRs.

//

//	SIG_OUTPUT_COMPARE1A()

//		This ISR handles sending an RF preamble to initialize the RF receivers

//		in the beacons. It also handles sending the a pulse to the desired sonar

//		beacon.

//

////////////////////////////////////////////////////////////////////////////////

// this interrupt is called when the input capture signal is received

SIGNAL( SIG_INPUT_CAPTURE1 )

{

// indicate a timeout has occurred

pong_value_gv[ current_beacon_gv ] =  ICR1;

sonar_update_gv |= _BV( current_beacon_gv );

// after this we restart the sequence with the next beacon

// starting with sending the preamble

tx_state_gv = TX_STATE_PREAMBLE;

TCNT1 = 0;

OCR1A = pre_preamble_delay_c;

// go to the next beacon

increment_current_beacon();

// turn off the input capture

timer1_input_capture_interrupt( false );

// turn off the pulse if we haven't already

// (in case the pong is received before the pulse is completed)

rf_pulse( current_beacon_gv, false );

pong_count_gv++;

}

// run this timer routine for the preamble

SIGNAL( SIG_OUTPUT_COMPARE1A )

{

switch ( tx_state_gv )

{

////////////////////////////////////////////////////////////////////////////

case TX_STATE_PREAMBLE:

// toggle the bit output bits

if ( !( preamble_bit_count_gv & 0x01 ) )

{

// turn on tx-ers for even counts

PORTA |= preamble_tx_mask_c;

}

else

{

// turn off tx-ers for odd counts

PORTA &= ~preamble_tx_mask_c;

}

// if this is the last bit,

// change the timer length

// and change the state to Tx the sonar pulse

if ( preamble_bit_count_gv == preamble_bits_c - 1 )

{

// after the preamble is done, wait some time

// for all sonar to die out

TCNT1 = 0;

OCR1A = post_preamble_delay_c;

// and change the state to sonar quiet

tx_state_gv = TX_STATE_QUIET;

preamble_bit_count_gv = 0;

}

else

{

// the state remains the same

TCNT1 = 0;

OCR1A = preamble_bit_duration_c;

// increment the preamble bit count

preamble_bit_count_gv++;

}

break;

////////////////////////////////////////////////////////////////////////////

case TX_STATE_QUIET:

// the next state is to stop the sonar pulse

tx_state_gv = TX_STATE_PING;

// the sonar pulse lasts for sonar_pulse_length_c

// and reset the timer for the input capture

TCNT1 = 0;

OCR1A = sonar_pulse_length_c;

// enable the input capture

timer1_clear_input_capture_flag();

timer1_input_capture_interrupt( true );

// send the good pulse on the current beacon

rf_pulse( current_beacon_gv, true );

// increment the ping count

ping_count_gv++;

break;

////////////////////////////////////////////////////////////////////////////

case TX_STATE_PING:

// the next state is to wait for the sonar pong

tx_state_gv = TX_STATE_PONG;

//call this ISR again if a timeout occurs before a pong is received

// note that pong_timeout is measured from the start of TX_STATE_QUIET

// not from this state

OCR1A = pong_timeout_c;

// turn off the pulse

rf_pulse( current_beacon_gv, false );

break;

////////////////////////////////////////////////////////////////////////////

case TX_STATE_PONG:

// Since the input capture wasn't received in time, indicate a timeout

pong_value_gv[ current_beacon_gv ] = sonar_invalid_pong_c;

sonar_update_gv |= _BV( current_beacon_gv );

// after this we restart the sequence with the next beacon

// starting with sending the preamble

tx_state_gv = TX_STATE_PREAMBLE;

TCNT1 = 0;

OCR1A = pre_preamble_delay_c;

// go to the next beacon

increment_current_beacon();

// turn off the input capture during the preamble

timer1_input_capture_interrupt( false );

break;

////////////////////////////////////////////////////////////////////////////

}

}

////////////////////////////////////////////////////////////////////////////////

/////////////////////////// End of file ////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

Stepper-synch.h

#ifndef _STEPPER_SYNCH_H_

#define _STEPPER_SYNCH_H_

#include <inttypes.h>

// enumeration for the direction of travel for the robot

// when calling stepper_move()

enum stepper_direction_t

{

STEPPER_FORWARD	= 0,

STEPPER_REVERSE	= 1,

STEPPER_LEFT	= 2,

STEPPER_RIGHT	= 3

};

// number of steps to make a right angle turn

const int 	stepper_one_revolution = 968;

const int 	stepper_right_angle = (stepper_one_revolution + 2)/4;

const int	steps_per_inch = 42;

const int	steps_per_foot = 509;

// number of steps to turn around

void	stepper_init();

void	stepper_on();

void	stepper_off();

void	stepper_move( uint16_t num_steps, stepper_direction_t direction );

void	stepper_stop();

bool	stepper_movement_stopped();

bool	stepper_movement_finished();

void 	stepper_halt();

uint16_t stepper_steps_remaining();

void	stepper_resume();

#endif

Stepper_synch.c

////////////////////////////////////////////////////////////////////////////////

//////////////////////////  Stepper Motor Driver ///////////////////////////////

////////////////////////////////////////////////////////////////////////////////

// Written by Bryan Davis, 06/13/2004 - iter1 version

//

// This module contains routines to control the stepper motors. The unipolar

// stepper motors are connected to PORTC of the ATmega128. Each motor has 4

// coils that operate independently, but all coils have a common terminal.

// The common terminal is tied high, so the controller activates a coil by

// writing a 0 to the corresponding port pin. The coils are activated in

// sequence such as:

//

// 		coil 1 11100000111000001

// 		coil 2 00111000001110000

// 		coil 3 00001110000011100

// 		coil 4 10000011100000111

//

// Stepper_poly contains routines that allow the robot to move in a polygonal

// path. This means that either both motors are moving in synchrony. They are

// either both moving forward or backward in synchrony, which sends the robot

// in a staight line (the polygon edges). Or both are moving in opposite

// directions at the same speed in which case, the robot turns in place (the

// vertices). This behavior also eliminates shaking that can occur when the

// left and right motor steps are out-of-synch.

//

////////////////////////////////////////////////////////////////////////////////

//

// stepper_init()

//		Starts the timer counting and sets the output compare modes for the

//		other routines. This must be run first. Best to run prior to sei().

//

// stepper_on()

//		Enables the motor drivers. Now PORTC pins directly control the coils

//

// stepper_off()

//		Disables the motor drivers.

//

// stepper_left_count(), stepper_right_count()

//		Returns the running count of the total number of steps. Reverse steps

//		count negative.

//

// stepper_reset_count()

//		Resets the runnning count of the number of steps to 0.

//

// stepper_left_interval( int interval ), stepper_right_interval( int interval )

//		Sets the interval in timer counts between steps for the given

//		motor. If the interval is 0, then the motor stops, but the coil(s)

// 		remain powered.

//

// stepper_speed( int left_speed, int right_speed )

//		Sets the interval using stepper_XXX_interval() so that the motor runs

//		at a percentage of the maximum speed. The percentage is linear,

//		(except	for quantization effects). The max speed is determined from

//		max_xxx_interval_c. If xxx_speed = 0, then the motor is held in

//		position (not turned off). Note that the counts will still be valid,

//		however the motor still consumes power -- actually, when speed = 0 it

//		consumes the most power).

//

// stepper_accel( int left_accel, int right_accel )

//		Sets the acceleration of the motors.

////////////////////////////////////////////////////////////////////////////////

/////////////////////////  Pre-Processor Directives ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

#include <stdio.h>

#include <inttypes.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/signal.h>

#include "clock.h"

#include "timer.h"

#include "stepper_synch.h"

#include "algorithms.h"

////////////////////////////////////////////////////////////////////////////////

///////////////////////////////  Typedefs //////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

// the modes of acceleration used by the ISR

enum accel_mode_t

{

ACCEL_MODE_CREEP,

ACCEL_MODE_GO,

ACCEL_MODE_SLOW_DOWN,

ACCEL_MODE_APPROACH,

ACCEL_MODE_STOPPED,

ACCEL_MODE_FINISHED

};

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Constants  /////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

// use the SYS_CLK / 64 for the timed counter

// for a 250kHz clock -> 4 us interval

const timer123_clk_mode_t stepper_clk_mode_c = CLK_SELECT_64;

// constants used in acceleration profile

// how often the timing ISR is called (2500 -> once every 10 ms)

const uint16_t timing_ISR_interval_c		= 1000;

// the maximum number of steps to creep

// any movement <= max_creep_steps_c will move at

// the slowest speed

const uint16_t	max_creep_steps_c			= 20;

// number of buffer steps to wait when resuming

// from ACCEL_MODE_SLOW_DOWN to ACCEL_MODE_APPROACH

// to mitigate ISR changing volatile variables

const uint8_t	approach_buffer_steps_c		= 2;

// the maximum length interval defines the length of time between steps

// while moving at the slowest possible speed.

const uint16_t	max_interval_c				= 10000;

// the minimum length interval defines the length of time between steps

// while moving at the fastest possible speed.

const uint16_t	min_interval_c				= 200;

// value of k in interval update expression

// T = T + (T^2)/(2^k)

const uint8_t	accel_exp_c					= 17;

const uint8_t	decel_exp_c					= 16;

// the quasi-sqrt of the stopping distance

// stopping distance

const uint8_t	normal_stopping_distance_c			= 210;

const uint8_t	qsqrt_normal_stopping_distance_c 	= 20;

// the length of the motor sequence - must be a power of 2

// or code must be changed.

const uint8_t seq_len_c		= 8;

const uint8_t seq_mask_c 	= seq_len_c - 1;

// left motor turns counter-clockwise for forward

const int left_seq_num_c[ seq_len_c ] =

{

3,

2,

1,

0

};

const uint8_t left_seq_c[ seq_len_c ] =

{

_BV( left_seq_num_c[0] ),

_BV( left_seq_num_c[0] ) | _BV( left_seq_num_c[1] ),

_BV( left_seq_num_c[1] ),

_BV( left_seq_num_c[1] ) | _BV( left_seq_num_c[2] ),

_BV( left_seq_num_c[2] ),

_BV( left_seq_num_c[2] ) | _BV( left_seq_num_c[3] ),

_BV( left_seq_num_c[3] ),

_BV( left_seq_num_c[3] ) | _BV( left_seq_num_c[0] )

};

const uint8_t left_mask_c = 0x0F;

// right motor turns clockwise for forward

const int right_seq_num_c[ seq_len_c ] =

{

7,

6,

5,

4

};

const uint8_t right_seq_c[ seq_len_c ] =

{

_BV( right_seq_num_c[0] ),

_BV( right_seq_num_c[0] ) | _BV( right_seq_num_c[1] ),

_BV( right_seq_num_c[1] ),

_BV( right_seq_num_c[1] ) | _BV( right_seq_num_c[2] ),

_BV( right_seq_num_c[2] ),

_BV( right_seq_num_c[2] ) | _BV( right_seq_num_c[3] ),

_BV( right_seq_num_c[3] ),

_BV( right_seq_num_c[3] ) | _BV( right_seq_num_c[0] )

};

const uint8_t right_mask_c = 0xF0;

// pin on portG used to enable the motor drivers

const int enable_pin_c = _BV( PORTG0 );

////////////////////////////////////////////////////////////////////////////////

/////////  Non-volatile Global Variables (modified synchronously) //////////////

////////////////////////////////////////////////////////////////////////////////

// flags to indicate the motor direction is reversed

bool 		left_reverse_g;

bool 		right_reverse_g;

// variable to indicate the start and min intervals for the accel. algorithm

uint16_t 	min_interval_g;

// variables to store the stopping distance and quasi-sqrt() of stopping distance

uint16_t	stopping_distance_g;

uint8_t		qsqrt_stopping_distance_g;

////////////////////////////////////////////////////////////////////////////////

/////////  Volatile Global Variables (modified asychronously) //////////////////

////////////////////////////////////////////////////////////////////////////////

// used to store the interval for the ISR to handle

volatile uint16_t		interval_gv;

// keeps track of the current mode of acceleration

volatile accel_mode_t	accel_mode_gv;

// holds the current deceleration factor, which is inversely

// proportional to the deceleration

// this is calculated so that the robot slows down smoothly

// from it's speed when getting close to the target to 0

// when it stops on the target

volatile uint16_t		decel_factor_gv;

// used to keep track of the coil sequence

volatile uint8_t		left_count_gv;

volatile uint8_t		right_count_gv;

// counter used to count steps for this motion primitive

volatile uint16_t 	step_count_gv;

// counts the steps remaining in the movement

volatile uint16_t		steps_to_stop_gv;

// used to store the quasi-sqrt of step_count_gv for acceleration

volatile uint8_t		qsqrt_step_gv;

// used to store the quasi-sqrt of step_count_gv for deceleration

volatile uint8_t		qsqrt_steps_to_stop_gv;

// holds the remainder of the quasi-sqrt()

volatile int8_t		qsqrt_remainder_gv;

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Prototypes ////////////////////////////

////////////////////////////////////////////////////////////////////////////////

uint16_t qsqrt_inv( uint8_t x );

uint8_t qsqrt( uint16_t x, uint8_t* remainder_p );

void stepper_set_stopping_distance( int qsqrt_dist );

void stepper_enable_interrupts( bool on );

////////////////////////////////////////////////////////////////////////////////

//////////////////////////////  Function Definitions ///////////////////////////

////////////////////////////////////////////////////////////////////////////////

// initializes the stepper motor registers

// does NOT enable the motor driver, set the motor speed or enable the interrupts

void stepper_init( )

{

// initialize global variables

left_count_gv 		= 0;

right_count_gv		= 0;

accel_mode_gv		= ACCEL_MODE_FINISHED;

// use PORTC0:3 for left motor

// use PORTC4:7 for right motor

// use PORTG2 to turn on or off the motor driver

PORTC = 0; // 0 is off, 1 is on

PORTG &= ~enable_pin_c; // disable motor

DDRC = 0xFF;  // enable port C as an output

DDRG |= enable_pin_c; // enable PG2 as an output

// Use the 1/8 clock prescaler

// for a frequency of 4096Hz per count

timer3_clk_select( stepper_clk_mode_c );

// use normal counting mode

timer3_pwm_mode( PWM_OFF );

}

// enables the motor drivers

void stepper_on()

{

// enable the drivers

PORTG |= enable_pin_c;

wait(250);

// initialize the counts

left_count_gv	= 0;

right_count_gv	= 0;

// make one cycle step through the sequence to properly seat the motors

// initialize the motors -- go in reverse so we end on sequence 0

for ( uint8_t cycle_count = seq_len_c - 1; cycle_count <= 0; cycle_count-- )

{

PORTC = ( left_seq_c [ cycle_count ] & left_mask_c  ) |

( right_seq_c[ cycle_count ] & right_mask_c );

wait(250);

}

}

// disables the motor drivers

void stepper_off()

{

PORTG &= ~enable_pin_c;

}

// sets the direction of the motor

void stepper_direction( stepper_direction_t direction )

{

switch ( direction )

{

case STEPPER_FORWARD:

left_reverse_g		= false;

right_reverse_g		= false;

break;

case STEPPER_REVERSE:

left_reverse_g		= true;

right_reverse_g		= true;

break;

case STEPPER_LEFT:

left_reverse_g		= false;

right_reverse_g		= true;

break;

case STEPPER_RIGHT:

left_reverse_g		= true;

right_reverse_g		= false;

break;

}

}

// causes the motor to move a given number of steps

// returns the number of steps actually moved.

void stepper_move( uint16_t num_steps, stepper_direction_t direction )

{

// in case some dumbass tries to move no steps

if ( num_steps == 0 )

{

accel_mode_gv = ACCEL_MODE_FINISHED;

return;

}

// set the direction

stepper_direction( direction );

// set the total number of steps to move

steps_to_stop_gv = num_steps;

if ( num_steps > max_creep_steps_c )

{

// tell the ISR we are accelerating

accel_mode_gv = ACCEL_MODE_GO;

//printf( "GO -- ");

// reset the current step counter

step_count_gv		= 0;

// set the stopping distance

// if the total movement is long, begin approach near the end,

// if the total movement is short, begin approach ~half-way

if ( num_steps > 2 * normal_stopping_distance_c )

{

// set the acceleration mode to 'go'

// stop the movement (done by ISR) when remaining steps

// is less than the normal stopping distance

stepper_set_stopping_distance( qsqrt_normal_stopping_distance_c );

}

else

{

// if we are going some distance less than this, we should set

// the stopping distance to be a little more than half way

stepper_set_stopping_distance( qsqrt( num_steps / 2, 0 ) );

}

}

// the number is so small that we will just creep over there at the slowest speed

// this is need to prevent some strange conditions that can happen with speeding up

// and slowing down when only a few steps are need

else // num_steps <= max_creep_steps_c

{

accel_mode_gv 	= ACCEL_MODE_CREEP;

//printf( "CREEP -- ");

}

// Start the interval at the slowest speed

interval_gv		= max_interval_c;

// enable the ISR

stepper_enable_interrupts( true );

}

// lock motors still

void stepper_halt()

{

accel_mode_gv = ACCEL_MODE_STOPPED;

}

uint16_t stepper_steps_remaining()

{

return steps_to_stop_gv;

}

bool stepper_movement_stopped()

{

return accel_mode_gv == ACCEL_MODE_FINISHED || accel_mode_gv == ACCEL_MODE_STOPPED;

}

bool stepper_movement_finished()

{

return accel_mode_gv == ACCEL_MODE_FINISHED;

}

// stop immediately

void stepper_stop()

{

// only stop if we are going

if ( accel_mode_gv == ACCEL_MODE_GO )

{

accel_mode_gv = ACCEL_MODE_SLOW_DOWN;

//printf( "SLOW_DOWN -- ");

}

}

// resume if told to stop previously

void stepper_resume()

{

// only resume if we slowed down or stopped because the movement wasn't finished

if ( accel_mode_gv == ACCEL_MODE_SLOW_DOWN || accel_mode_gv == ACCEL_MODE_STOPPED )

{

// if we are told to resume, but we almost at the target, enter ACCEL_MODE_CREEP

// approach_buffer_steps_c should be very small like 2 or 3

// to prevent a quick deceleration

if ( steps_to_stop_gv <= approach_buffer_steps_c )

{

accel_mode_gv = ACCEL_MODE_CREEP;

//printf( "CREEP -- ");

}

// if we are within the normal stopping distance,

// then we should enter approach mode ASAP. But due

// to the ISR changing the value of	steps_to_stop_gv

// while we are in this routine, it is best to tell

// it to APPROACH a few steps in the future.

else if ( steps_to_stop_gv <= stopping_distance_g + 1 )

{

stepper_set_stopping_distance( qsqrt( steps_to_stop_gv - approach_buffer_steps_c, 0 ) );

accel_mode_gv = ACCEL_MODE_GO;

//printf( "GO -- ");

}

// otherwise enter just accel_mode_go

else

{

accel_mode_gv = ACCEL_MODE_GO;

//printf( "GO -- ");

}

// resume the ISR (needed if previously in ACCEL_MODE_STOPPED)

stepper_enable_interrupts( true );

}

}

inline void stepper_set_stopping_distance( int qsqrt_dist )

{

qsqrt_stopping_distance_g = qsqrt_dist;

// inverse of the quasi-sqrt()

stopping_distance_g = qsqrt_inv( qsqrt_dist );

}

inline void stepper_enable_interrupts( bool on )

{

if ( on )

{

ETIMSK |= _BV( OCIE3A ) | _BV( OCIE3B );

}

else

{

ETIMSK &= ~_BV( OCIE3A ) | _BV( OCIE3B );

}

}

////////////////////////////////////////////////////////////////////////////////

//////////////////// Interrupt Service Rountines ///////////////////////////////

////////////////////////////////////////////////////////////////////////////////

// This ISR is called when the timer counter equals the output compare

// register, while the interrupt is enabled. It checks the reverse condition for

// both the left and right motors and moves the counter accordingly. It uses the

// LSBs of the counters to determine the next sequence of coil activation. Then

// it moves the output compare register so the interrupt tiggers again after

// interval_g counts.

////////////////////////////////////////////////////////////////////////////////

// output compare 3A is used for both motors

// it counts the steps and moves the motor to the next interval

SIGNAL( SIG_OUTPUT_COMPARE3A )

{

///////////////////////////////////////

///////// Update the Counters /////////

///////////////////////////////////////

// remaining number of steps for this movement

steps_to_stop_gv--;

step_count_gv++;

// determine the correct motor coil sequence

if ( left_reverse_g )

left_count_gv--;

else

left_count_gv++;

if ( right_reverse_g )

right_count_gv--;

else

right_count_gv++;

///////////////////////////////////////

/////////// Step the Motors ///////////

///////////////////////////////////////

PORTC = ( left_seq_c[	seq_mask_c & left_count_gv	] & left_mask_c	 ) |

( right_seq_c[	seq_mask_c & right_count_gv	] & right_mask_c );

///////////////////////////////////////

//// Update the acceleration mode /////

///////////////////////////////////////

// accel_mode_gv may be set by this ISR or may be set by an outside control

switch ( accel_mode_gv )

{

// when we are getting near the target, slow down so that we stop at exactly

// the right point

case ACCEL_MODE_GO:

if ( steps_to_stop_gv == stopping_distance_g )

{

// initialize the quasi-sqrt() of the number of steps remaining

// set the global qsqrt_steps_remaining_gv to the number of steps remaining

accel_mode_gv			= ACCEL_MODE_APPROACH;

//printf( "APPROACH -- ");

qsqrt_steps_to_stop_gv 	= qsqrt_stopping_distance_g;

qsqrt_remainder_gv 		= 0;

decel_factor_gv 		= interval_gv * qsqrt_steps_to_stop_gv;

}

break;

case ACCEL_MODE_APPROACH:

// if we are at the target, then finish this movement

if ( steps_to_stop_gv == 0 )

{

accel_mode_gv = ACCEL_MODE_FINISHED;

stepper_enable_interrupts( false );

}

// otherwise we need to calculate the interval update

// Note that we only do that in this ISR for APPROACH

// because we synch the deceleration to the number of steps left

else

{

qsqrt_remainder_gv--;

if ( qsqrt_remainder_gv < 0 )

{

qsqrt_steps_to_stop_gv--;

qsqrt_remainder_gv = qsqrt_steps_to_stop_gv;

// only need to update the interval when sqrt_step_gv changes

interval_gv = decel_factor_gv / ( qsqrt_steps_to_stop_gv + 1 );

}

}

break;

case ACCEL_MODE_CREEP:

// if we are at the target, then finish this movement

if ( steps_to_stop_gv == 0 )

{

accel_mode_gv = ACCEL_MODE_FINISHED;

stepper_enable_interrupts( false );

}

break;

// if we slowed down to a certain point, stop

// but the movement is not finished yet

case ACCEL_MODE_SLOW_DOWN:

if ( interval_gv > max_interval_c )

{

accel_mode_gv = ACCEL_MODE_STOPPED;

stepper_enable_interrupts( false );

}

break;

case ACCEL_MODE_FINISHED:

case ACCEL_MODE_STOPPED:

// turn off both ISRs

stepper_enable_interrupts( false );

break;

}

///////////////////////////////////////

/// move OCR1A to the next interval ///

///////////////////////////////////////

OCR3A += interval_gv;

}

// the output-compare-3B ISR is used to calculate the update interval for

// the outupt-compare-3A ISR. Since output-compare-1A is timing critical,

// this ISR is intertuptable by OC1A

SIGNAL( SIG_OUTPUT_COMPARE3B )

{

///////////////////////////////////////

/// Calculate the next interval ///////

///////////////////////////////////////

// depending on the acceleration mode, do different things

switch ( accel_mode_gv )

{

// accelerate to maximum speed

case ACCEL_MODE_GO:

if ( interval_gv > min_interval_c )

{

interval_gv -= (uint16_t)( ( (uint32_t)interval_gv * interval_gv ) >> accel_exp_c );

}

break;

case ACCEL_MODE_CREEP:

// do nothing -- we are already moving at the slowest speed

// => the interval stays constant

break;

// slow down to a stop

case ACCEL_MODE_SLOW_DOWN:

if ( interval_gv < max_interval_c )

{

interval_gv += (uint16_t)( ( (uint32_t)interval_gv * interval_gv ) >> decel_exp_c );

}

// otherwise, the other ISR will set the acceleration mode to ACCEL_MODE_STOPPED

break;

// approach the target and stop when we are there steps_remainder_gv steps

case ACCEL_MODE_APPROACH:

// we handle this case in the other ISR

break;

case ACCEL_MODE_STOPPED:

case ACCEL_MODE_FINISHED:

// these should never occur be entered -- just here for completeness

break;

}

//printf( "%d  ",interval_gv );

// move OCR1B to the next interval

OCR3B += timing_ISR_interval_c;

}

////////////////////////////////////////////////////////////////////////////////

/////////////////////////// End of file ////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

Timer.h

#ifndef _TIMER_H_

#define _TIMER_H_

#include <inttypes.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/signal.h>

#include "clock.h"

enum timer16_mode_t

{

PWM_OFF 			= 0,

PWM_PHASE_8_BIT 	= 1,

PWM_PHASE_9_BIT 	= 2,

PWM_PHASE_10_BIT 	= 3,

PWM_CTC_OCR			= 4,

PWM_FAST_8_BIT		= 5,

PWM_FAST_9_BIT		= 6,

PWM_FAST_10_BIT		= 7,

PWM_PHASE_FREQ_ICR	= 8,

PWM_PHASE_FREQ_OCRA	= 9,

PWM_PHASE_ICR		= 10,

PWM_PHASE_OCR		= 11,

PWM_CTC_ICR			= 12,

// RESERVED			= 13,

PWM_FAST_ICR		= 14,

PWM_FAST_OCRA		= 15

};

enum timer8_mode_t

{

PWM8_OFF			= 0x00,

PWM8_PHASE			= _BV( WGM00 ), //_BV( WGM00 ) = _BV( WGM20 )

PWM8_CTC			= _BV( WGM01 ), //_BV( WGM01 ) = _BV( WGM21 )

PWM8_FAST			= _BV( WGM00 ) | _BV( WGM01 )

};

const int timer8_mode_mask = _BV( WGM00 ) | _BV( WGM01 );

enum timer123_clk_mode_t

{

CLK_SELECT_OFF		= 0,

CLK_SELECT_1		= 1,

CLK_SELECT_8		= 2,

CLK_SELECT_64		= 3,

CLK_SELECT_256		= 4,

CLK_SELECT_1024		= 5,

CLK_SELECT_EXT_FALL	= 6,

CLK_SELECT_EXT_RISE	= 7

};

enum timer0_clk_mode_t

{

CLK0_SELECT_OFF		= 0,

CLK0_SELECT_1		= 1,

CLK0_SELECT_8		= 2,

CLK0_SELECT_32		= 3,

CLK0_SELECT_64		= 4,

CLK0_SELECT_128		= 5,

CLK0_SELECT_256		= 6,

CLK0_SELECT_1024	= 7

};

enum output_compare_mode_t

{

OUT_COMP_OFF		= 0x00,

OUT_COMP_TOGGLE		= 0x01,	// toggle pin after a match

OUT_COMP_SET_BELOW	= 0x02,	// set pin when counter is below match reg.

OUT_COMP_SET_ABOVE  = 0x03	// set pin when counter is above match reg.

};

const int output_compare_mode_mask = 0x03;

enum timer_input_capture_edge_t

{

INPUT_CAPTURE_RISING,

INPUT_CAPTURE_FALLING

};

inline uint16_t timer1_read_counter()

{

uint8_t sreg;

uint16_t val;

sreg = SREG;

cli();

val = TCNT1;

SREG = sreg;

return val;

};

inline uint16_t timer3_read_counter()

{

uint8_t sreg;

uint16_t val;

sreg = SREG;

cli();

val = TCNT3;

SREG = sreg;

return val;

};

inline void timer1_write_counter( uint16_t val )

{

uint8_t sreg;

sreg = SREG;

cli();

TCNT1 = val;

SREG = sreg;

};

inline void timer3_write_counter( uint16_t val )

{

uint8_t sreg;

sreg = SREG;

cli();

TCNT3 = val;

SREG = sreg;

};

// use to set the asynchronous timer

inline void timer0_clk_asynch( bool use_asynch )

{

if ( use_asynch )

{

ASSR |= _BV( AS0 ); // set bit

}

else

{

ASSR &= ~_BV( AS0 ); // clear bit

}

}

inline void timer0_pwm_mode( timer8_mode_t mode )

{

TCCR0 |= mode &  timer8_mode_mask;

TCCR0 &= mode | ~timer8_mode_mask;

};

inline void timer2_pwm_mode( timer8_mode_t mode )

{

TCCR2 |= mode &  timer8_mode_mask;

TCCR2 &= mode | ~timer8_mode_mask;

};

inline void timer1_pwm_mode( timer16_mode_t mode )

{

// set lowest two bits

TCCR1A |= mode & 	( _BV( WGM10 ) | _BV( WGM11 ) );

TCCR1A &= mode | ~	( _BV( WGM10 ) | _BV( WGM11 ) );

// set highest two bits

TCCR1B |= mode & 	( _BV( WGM12 ) | _BV( WGM13 ) );

TCCR1B &= mode | ~	( _BV( WGM12 ) | _BV( WGM13 ) );

};

inline void timer3_pwm_mode( timer16_mode_t mode )

{

// set lowest two bits

TCCR3A |= mode & 	( _BV( WGM30 ) | _BV( WGM31 ) );

TCCR3A &= mode | ~	( _BV( WGM30 ) | _BV( WGM31 ) );

// set highest two bits

TCCR3B |= mode & 	( _BV( WGM32 ) | _BV( WGM33 ) );

TCCR3B &= mode | ~	( _BV( WGM32 ) | _BV( WGM33 ) );

};

inline void timer0_clk_select( timer0_clk_mode_t mode )

{

TCCR0 |= mode & 	( _BV( CS00 ) | _BV( CS01 ) | _BV( CS02 ) );

TCCR0 &= mode | ~	( _BV( CS00 ) | _BV( CS01 ) | _BV( CS02 ) );

};

inline void timer1_clk_select( timer123_clk_mode_t mode )

{

TCCR1B |= mode & 	( _BV( CS10 ) | _BV( CS11 ) | _BV( CS12 ) );

TCCR1B &= mode | ~	( _BV( CS10 ) | _BV( CS11 ) | _BV( CS12 ) );

};

inline void timer2_clk_select( timer123_clk_mode_t mode )

{

TCCR2 |= mode & 	( _BV( CS20 ) | _BV( CS21 ) | _BV( CS22 ) );

TCCR2 &= mode | ~	( _BV( CS20 ) | _BV( CS21 ) | _BV( CS22 ) );

};

inline void timer3_clk_select( timer123_clk_mode_t mode )

{

TCCR3B |= mode & 	( _BV( CS30 ) | _BV( CS31 ) | _BV( CS32 ) );

TCCR3B &= mode | ~	( _BV( CS30 ) | _BV( CS31 ) | _BV( CS32 ) );

};

inline void timer0_output_compare_mode	( output_compare_mode_t mode )

{

TCCR0 |=     ( mode << COM00  ) & 	( output_compare_mode_mask << COM00 );

TCCR0 &=     ( mode << COM00  ) | ~( output_compare_mode_mask << COM00 );

};

inline void timer1a_output_compare_mode( output_compare_mode_t mode )

{

TCCR1A |=    ( mode << COM1A0 ) &	( output_compare_mode_mask << COM1A0 );

TCCR1A &=    ( mode << COM1A0 ) | ~( output_compare_mode_mask << COM1A0 );

};

inline void timer1b_output_compare_mode( output_compare_mode_t mode )

{

TCCR1A |=    ( mode << COM1B0 ) &	( output_compare_mode_mask << COM1B0 );

TCCR1A &=    ( mode << COM1B0 ) | ~( output_compare_mode_mask << COM1B0 );

};

inline void timer1c_output_compare_mode( output_compare_mode_t mode )

{

TCCR1A |=    ( mode << COM1C0 ) &	( output_compare_mode_mask << COM1C0 );

TCCR1A &=    ( mode << COM1C0 ) | ~( output_compare_mode_mask << COM1C0 );

};

inline void timer2_output_compare_mode	( output_compare_mode_t mode )

{

TCCR2 |=     ( mode << COM20  ) & 	( output_compare_mode_mask << COM20 );

TCCR2 &=     ( mode << COM20  ) | ~( output_compare_mode_mask << COM20 );

};

inline void timer3a_output_compare_mode( output_compare_mode_t mode )

{

TCCR3A |=    ( mode << COM3A0 ) &	( output_compare_mode_mask << COM3A0 );

TCCR3A &=    ( mode << COM3A0 ) | ~( output_compare_mode_mask << COM3A0 );

};

inline void timer3b_output_compare_mode( output_compare_mode_t mode )

{

TCCR3A |=    ( mode << COM3B0 ) &	( output_compare_mode_mask << COM3B0 );

TCCR3A &=    ( mode << COM3B0 ) | ~( output_compare_mode_mask << COM3B0 );

};

inline void timer3c_output_compare_mode( output_compare_mode_t mode )

{

TCCR3A |=    ( mode << COM3C0 ) &	( output_compare_mode_mask << COM3C0 );

TCCR3A &=    ( mode << COM3C0 ) | ~( output_compare_mode_mask << COM3C0 );

};

inline void timer1_input_capture_noise_canceller( bool use )

{

if ( use )

TCCR1B |= _BV( ICNC1 );

else

TCCR1B &= ~_BV( ICNC1 );

};

inline void timer3_input_capture_noise_canceller( bool use )

{

if ( use )

TCCR3B |= _BV( ICNC3 );

else

TCCR3B &= ~_BV( ICNC3 );

};

// clear on rising, set on falling

inline void timer1_input_capture_edge( timer_input_capture_edge_t edge )

{

if ( edge == INPUT_CAPTURE_FALLING )

TCCR1B |= _BV( ICES1 );

else // if ( edge == INPUT_CAPTURE_RISING )

TCCR1B &= ~_BV( ICES1 );

};

// clear on rising, set on falling

inline void timer3_input_capture_edge( timer_input_capture_edge_t edge )

{

if ( edge == INPUT_CAPTURE_FALLING )

TCCR3B |= _BV( ICES3 );

else // if ( edge == INPUT_CAPTURE_RISING )

TCCR3B &= ~_BV( ICES3 );

};

inline void timer1_clear_input_capture_flag()

{

TIFR = _BV( ICF1 );

};

inline void timer3_clear_input_capture_flag()

{

ETIFR = _BV( ICF3 );

};

inline void timer1a_clear_output_compare_flag()

{

TIFR = _BV( OCF1A );

};

inline void timer1b_clear_output_compare_flag()

{

TIFR = _BV( OCF1B );

};

inline void timer1c_clear_output_compare_flag()

{

ETIFR = _BV( OCF1C );

};

inline void timer3a_clear_output_compare_flag()

{

ETIFR = _BV( OCF3A );

};

inline void timer3b_clear_output_compare_flag()

{

ETIFR = _BV( OCF3B );

};

inline void timer3c_clear_output_compare_flag()

{

ETIFR = _BV( OCF3C );

};

inline void timer1a_output_compare_interrupt( bool enable )

{

if ( enable )

TIMSK |= _BV( OCIE1A );

else

TIMSK &= ~_BV( OCIE1A );

};

inline void timer1b_output_compare_interrupt( bool enable )

{

if ( enable )

TIMSK |= _BV( OCIE1B );

else

TIMSK &= ~_BV( OCIE1B );

};

inline void timer1c_output_compare_interrupt( bool enable )

{

if ( enable )

ETIMSK |= _BV( OCIE1C );

else

ETIMSK &= ~_BV( OCIE1C );

};

inline void timer1_input_capture_interrupt( bool enable )

{

if ( enable )

TIMSK |= _BV( TICIE1 );

else

TIMSK &= ~_BV( TICIE1 );

};

inline void timer3a_output_compare_interrupt( bool enable )

{

if ( enable )

ETIMSK |= _BV( OCIE3A );

else

ETIMSK &= ~_BV( OCIE3A );

};

inline void timer3b_output_compare_interrupt( bool enable )

{

if ( enable )

ETIMSK |= _BV( OCIE3B );

else

ETIMSK &= ~_BV( OCIE3B );

};

inline void timer3c_output_compare_interrupt( bool enable )

{

if ( enable )

ETIMSK |= _BV( OCIE3C );

else

ETIMSK &= ~_BV( OCIE3C );

};

inline void timer3_input_capture_interrupt( bool enable )

{

if ( enable )

ETIMSK |= _BV( TICIE3 );

else

ETIMSK &= ~_BV( TICIE3 );

};

#endif

Usart.h

// usart.h

//

// contains utilities useful for USART initalization

#include "clock.h"

inline void usart0_set_baud_rate( uint32_t baud_rate )

{

uint16_t baud_register = (uint16_t)( SYS_CLK / ( USART_CLK_DIVIDER * baud_rate ) - 1 );

UBRR0H = (uint8_t)( baud_register >> 8 );

UBRR0L = (uint8_t)( baud_register );

};

inline void usart1_set_baud_rate( uint32_t baud_rate )

{

uint16_t baud_register = (uint16_t)( SYS_CLK / ( USART_CLK_DIVIDER * baud_rate ) - 1 );

UBRR1H = (uint8_t)( baud_register >> 8 );

UBRR1L = (uint8_t)( baud_register );

};

Wi232.h

// wi232.h:

//

// Call fopen_ wi232 () to set up the console as an input output data stream.

// If fopen_ wi232() is the first fopen command called, then the console

// becomes stdin and stdout and stderr.

// The

//

// Uses Interupts to handle data streaming. The console uses USART0, which uses:

// 		Rx = PORTE0 = pin 2

//		Tx = PORTE1 = pin 3

#ifndef _WI232_H_

#define _WI232_H_

#include <stdio.h>

// use BAUD_RATE baud 8N1 RX & TX on USART0

// to transmit to the console

FILE* 		fopen_wi232();

int 		wi232_putchar( char x );

int			wi232_getchar();

uint16_t 	wi232_data_available();

bool 		wi232_is_empty();

#endif

Wi232.c

// wi232.c:

//

// Call fopen_wi232() to set up the wi232 as an input output data stream.

// If fopen_wi232() is the first fopen command called, then the wi232

// becomes stdin and stdout and stderr.

// The

//

// Uses Interupts to handle data streaming. The wi232 uses USART0, which uses:

// 		Rx 	= PORTE0 = pin 2

//		Tx 	= PORTE1 = pin 3

//		CTS	= PORTE2 = pin 4

#include <inttypes.h>

#include <stdio.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/signal.h>

#include "wi232.h"

#include "circularqueue.h"

#include "usart.h"

const int		_baud_rate_c		= 38400;

const int		_tx_buffer_size_c	= 256;

const int		_rx_buffer_size_c	= 256;

const uint8_t _cts_pin_c			= _BV( PINE2 );

volatile char wi232_tx_buffer[ _tx_buffer_size_c ];

volatile char wi232_rx_buffer[ _rx_buffer_size_c ];

// circular queues used for the USART

CircularQueue< volatile char > wi232_tx_q( wi232_tx_buffer, _tx_buffer_size_c );

CircularQueue< volatile char > wi232_rx_q( wi232_rx_buffer, _rx_buffer_size_c );

// prototype

int wi232_putchar( char );

int wi232_getchar();

void initialize_baud_rate();

void command( bool enable );

void write_register( uint8_t reg, uint8_t value );

bool _cts();

// use BAUD_RATE baud 8N1 RX & TX on USART0

// to transmit to the wi232

FILE* fopen_wi232()

{

// enable Rx, Tx & Rx interupt

// (Tx buffer empty interupt is enabled when we actually send something)

UCSR0B = _BV( RXEN0 )	|

_BV( TXEN0 )	|

_BV( RXCIE0 );

// UCSC00 = 1 & UCSC01 = 1 -> 8-bit data

UCSR0C = _BV( UCSZ00 ) | _BV( UCSZ01 );

initialize_baud_rate();

wi232_tx_q.clear();

wi232_rx_q.clear();

// set wi232_putchar as the stdout stream character Tx-er

// set wi232_getchar as the stdin  stream chacacter Rx-er

return fdevopen( wi232_putchar, wi232_getchar, 0 );

}

inline void initialize_baud_rate()

{

// enable command pin as output

DDRE |= _BV( PORTE3 );

command( true );

usart0_set_baud_rate( 2400 );

sei();

write_register( 78, 3 ); // set baud rate to 38400

wait(100);

cli();

usart0_set_baud_rate( 38400 );

command( false );

}

inline void command( bool enable )

{

if ( enable )

PORTE	&= ~_BV( PORTE3 );

else

PORTE	|= _BV( PORTE3 );

}

inline void write_register( uint8_t reg, uint8_t value )

{

if ( value < 0x80 )

{

// data = [ 0xFF, 2, reg, value ]

wi232_putchar( 0xFF );

wi232_putchar( 2 );

wi232_putchar( reg );

wi232_putchar( value );

}

else // value >= 0x80

{

// data = [ 0xFF, 3, reg, 0xFE, (value - 0x80) ]

wi232_putchar( 0xFF );

wi232_putchar( 3 );

wi232_putchar( reg );

wi232_putchar( 0xFE );

wi232_putchar( value - 0x80 );

}

}

int wi232_putchar( char x )

{

wi232_tx_q.put( x );

// enable the "data register empty" interupt

// (it may already be enabled in which case no harm is done)

UCSR0B |= _BV( UDRIE0 );

// return -1 if overflow is true, otherwise return 0

// return wi232_tx_q.overflow() ? -1 : 0;

return 0; // for now, screw overflow - just ignore it

}

int wi232_getchar()

{

// wait until the queue is no longer empty

while ( wi232_rx_q.is_empty() );

char x = wi232_rx_q.get();

return x;

}

uint16_t wi232_data_available()

{

return wi232_rx_q.length();

}

bool wi232_is_empty()

{

return wi232_rx_q.is_empty();

}

// returns true when CTS is low,

// i.e. it is true when it is okay to send data

inline bool _cts()

{

return !(PINE & _cts_pin_c);

}

// we received some data on USART0

// put this data into the received data queue

SIGNAL( SIG_UART0_RECV )

{

wi232_rx_q.put( UDR0 );

}

// the USART is ready to queue another byte to send

SIGNAL( SIG_UART0_DATA )

{

if ( !wi232_tx_q.is_empty() )

{

if ( !_cts() )

{

sei();

// wait until CTS is true

while ( !_cts() );

}

// if the queue is not empty, put the next character in the buffer

// this also resets the interupt flag automatically

UDR0 = wi232_tx_q.get();

}

else

{

// disable the interupt

UCSR0B &= ~_BV( UDRIE0 );

}

}