University of Florida

Intelligent Machine Design Lab

## Card Shark

(final report)

Kevin E. Kane

ID # 72860210

Dr. A. Arroyo

# Table of Contents

Abstract ……………………………………………………………………………….3

Executive Summary …………………………………………………………………..3

Introduction …………………………………………………………………………...4

Integrated System ……………………………………………………………………..4

Mobile Platform ………………………………………………………………………5

Actuation ……………………………………………………………………………...5

Sensors ………………………………………………………………………………..6

Barcode scanner

Photo-resistors

Behaviors ……………………………………………………………………………..6

Conclusion …………………………………………………………………………….7

Acknowledgements ……………………………………………………………………8

Appendix ……………………………………….……………………………………9-?

Complete Code

Line tracking circuit

# Abstract

Card Shark is an autonomous professional card-playing robot specializing in blackjack play.  Using its own deck of cards with barcodes on them the robot can place bets, read its cards and the dealer’s cards, and then makes the proper mathematical play for best long run results.  An excite toy for everyone to watch, Card Shark’s uses of entertainment could even be used directly in casinos.

Executive Summary

In an attempt to better educate the gambling community about the simple, yet exciting game of blackjack, I’ve designed Card Shark.  Card Sharks uses are completely for entertainment and spectator enjoyment for now.  In the flashy world of Gambling and card tables, a simple robot playing a traditionally human game would be an attention grabber.  Many would most likely what to even play next to the robotic gambling marvel. It does seem that future designs could also be build with more of an education/training purpose in mind.  Card Sharks could be used for teaching the game to complete novices and be a fun table companion at the same time.

Card Shark is designed to function as a completely autonomous blackjack player.  Card Shark has three main behavior functions that it performs, while in combination they create a false sense of intelligence.  It follows a highly contrast line track counter clockwise. As it performs this task, it scans the barcodes on playing cards laid out before it.  By reading and inputting these card values (or instructions as it maybe considered) Card Shark makes a blackjack playing choice.  Doubling down, splitting, hitting or standing is all part of a standard repertoire for this gambling machine.  In split and doubling down situations Card Shark’s current design drops a colored chips to better express its desires.  Complete hands are displayed on the LCD screen readable to either the dealer or observers.

With the ability to play one hand after another, and never become mentally tired, Card Shark is the first truly perfect Blackjack player.  Fun to watch and possibly even learn from, Card Shark may bring an ever more positive light to the some times intimidating game of blackjack.  If casinos desires are to keep patrons entertained and on the gaming floor having a couple Card Sharks in their casino may have the potential to bring more business and bets then before.

# Introduction

Card Shark is the first autonomous blackjack-playing robot ever (at lest I think so).  It is mostly for fun in its current form, as no serious casino would allow a robot to play its blackjack tables.  However, warehouses or other shipping companies can use the basic programming level of Card Shark’s decision-making and barcode reading to move organize and distribute products.  With a computerized system to organize and place goods in/out of storage, a company can expect exact and dependable work.

### Integrated System

At the heart of Card Shark is an ATMEL Atmega32L micro-controller mounted on a STK500 testing board.  This micro controller directly interfaces with the following devices:

- 2 “hacked” servos for mobility around the playing table

- 2 “arm” servos for placing bets

- 1 LCD for feedback to the dealer and operator

- 2 photo resistor circuits for basic line following

- 1 KaneScan barcode scanner for playing card reading

There are two movement servos that are used to drive Card Shark around the table while following the black line.  These two servos are used to help it turn left or right.  An interesting aspect of Card Shark is how wide it is.  As a result, in order to make sharp turns I wrote code that sets one wheel in reverse (although it is slow) in order to make tight turns as I desired.  This makes the Card Shark’s turns/movements appear jerky at times; however the advantages of a smaller track can then be utilized.

The “arms” are the most internal part of my design and are used to drop different colored chips from inside the chip holder areas.  These chips are pushed to the cutout holes at the bottom of the platform.  These servos are not hacked and are used to move one way and then back to their resting positions.

The LCD is used to inform the dealer and operator what Card Shark wishes to do with its decision; hit, stand, double down, or split.  It is located on the top of Card Shark.

The photo resistors are used to follow a black line.  They are incorporated in a circuit designed to give a digital (1 = white, 0 = black line) output. This keeps Card Shark in the proper place for reading the cards in order.

The Barcode scanner is used to recognize the actual card values.  Each card has a barcode on it that is scanned and input to Card Shark as it drives by.  A basic Code 39 is used for the scanner and barcodes.  The nice thing about this is that numbers are sent in ASCII form.  These means that after reading in the value the code could subtract $30 and have the correct hexadecimal value to be used. (Notice that Aces = $01 and Tens = $00)

# Mobile Platform

The platform I designed in AutoCAD and cutout with the T-tech was quite large with dimensions of 13” x 8”.  It holds the batteries and scanner on one side and the chips on the other, while still carrying the full STK500 in the middle.  That is the scanner is on the left and playing cards are on the right when facing the same direction as Card Shark.  The LCD is located on top next to the board and faces the back of Card Shark.  This seemed to be the best location for both the operator and the dealer to see what Card Shark’s playing choice is.  An area was built to hold ten AA batteries to power the robot.  On top of it is velcro to hold the scanner in place during operation.  In this design velcro is the only choice as glue means you’d be unable to replace Card Shark’s batteries and anything less would not hold the scanner in place.  Along with the wooden frame are two separate “arms” for moving chips from their holding locations to the holes cut out in the bottom layer of the platform.  A hole large enough to drop one chip bets as either the original, double down, or splitting bets.  The frame is held together using screws; in this way the separate layers of the platform can be taken apart and line following circuits potentiometers may be adjusted and the betting “arms” inside can be inspected.

# Actuation

Card Shark moves vie two servos, one for each driving wheel.  The other two servos are independently used for the betting “arms” inside the robot’s platform.  These servos are moved in three different patterns to displace three different bets; first bet, doubling down, and splitting bets.  For this reasons two of the used were bought as “hacked” servos while the remained standard servos.

As a funny side note, two of my original servos (non-hacked) were destroyed when I put an excess amount of crazy glue on the dowels placed inside the rotating shafts of the servos. They were glued stiff and wouldn’t move!

# Sensors

Sensor:	KaneScan Barcode Scanner (just happens to have my last name in it)

Part number:	02001291

Qty:		1

Discussion:	The KaneScan barcode scanner is an affordable ($85) highly accurate (4 mils) barcode scanner.  Each card in Card Shark’s deck has a barcode on the top corresponding to its value in blackjack.  Aces are coded with the number 1 and are handled by the software as either 1 or 11 like in all blackjack casino games.  Tens are given a value of 0 by the barcoding scheme but fixed in the programming code.  The barcode scanner is mounted on the left with it scanning done behind the drive wheels.  The scanner is interfaced via an RS-232 into an extra UART port built into the micro-controller.

Sensor:	Photo resistors

Part number:	none

Qty:		2

Discussion:	Card Shark uses two photo resistant cells for following a black line.  There have been many projects that have done similarly and I followed those designs tightly.  I used the same circuit design given in a report on line tracking by William the TA.  By following a black line Card Shark is sure to stay on track and read all the playing cards in proper order and then place the bets in the same general locations.

# Behaviors

First Card Shark places its starting bet to be in the hand.  Card Shark then follows the black line track around to scan the dealer’s cards and then its own cards.  It then makes the choice to hit, stand, double down, or split.  Card Shark displays this decision on its LCD screen (preceded by the card’s value read) and if needed drops the correct betting chip for doubling down or splitting. Card Shark then continues around the track prepared to read the next card if a hit or split was the play, or ready to read the next hand if doubling down or standing was the play.

These behaviors major components were worked on separately (line tracking, barcode reading, and betting) and then I tried to blend them together.  I discovered many issues to deal with when these behaviors were brought together.  One was the power needed to fully supply the STK500 board, 4 separate servos, two photo resistor circuits and one barcode scanner was a lot.  I used 10 rechargeable Energizer 2300 mAh batteries and this was barely enough.  (Notice: for the teacher/TA demo I tried different batteries with very bad results, including a very jumpy robot that wouldn’t follow a line)  Another issue with blending these behaviors is that scanning the cards is easy by hand but while driving exact alignment is needed.  Slight changes in approach angles result in unread cards.

# Conclusion

Card Shark is a fully functional blackjack-playing robot.  I know how to read/scan cards calculate its best betting solution and drop the proper chip for that decision.  It is of course only for fun and possibly for training yourself at home.  However, the basic ideas behind Card Shark are far reaching.  They could be used in moving and dropping off supplies (chips) to different location in a specified track or warehouse.  By having interchangeable barcodes, operators (dealers) can layout different patterns or objectives for an army of robots without actually changing the robots themselves.  This system would save a lot of time and effort if many such robots were in operation at a warehouse, factory, distribution center, or any other quickly changing (yet robotically feasible) manual labor locations.

There are many, many problems with this first design of Card Shark.  I will explain a couple now, but many more improvements should be made in any next generation designs.  First of all my original plans were to have Card Shark carry many doubling down bet and splitting bet chips with it as it moved.  It was to drop one at a time and still hold a supply of extra chips for following hands.  The problem was the material used in the internal “arms” was wood.  Everything was laid out in AutoCAD and finely designed to coordinate tightly with one another; however, the wood was quick to warp and bend, making my dream system impossible to utilize.  I was left sanding, carving, gluing and praying only to get it holding and dropping one chip at a time (I’m clearly not a mechanical engineer).  Second I never clearly laid a plan to make Card Shark stop and wait/drop bets at an exact location.  One black tab and a third photo-resistor would have solved this issue.  Instead, I the dealer, guessed the location of the last playing card so the bet chips would stay in the same general area.

I believed the easiest part of my project would be writing the code for playing the “basic” strategy laid out in books and online.  This wasn’t the case and what I expect to be a couple days work turned into assembly code over 400 words in length, and even then it was far from perfect.  “Arrays” and “lookup tables” coded in a higher-level language are the only ways of doing this.  I tried indirect indexing but faced more problems then I was able to tackle.  The result is a maze of code through branching and jump commands.  (I challenge any normal human being to try and follow it)

I would say that although Card Shark does not act completely to my desires or even correctly 100% of the time, I’m still proud of the work and effort I put into it.  The code is huge, all available PWMs were used, I almost ran out of ports on the STK500, and I burned, destroyed and lost more components then I’d like to admit.  It was all a learning experience.  The fact is, I learned more practical things in this short summer then any other class at UF.

# Acknowledgements

One thing I learned in this course was not to just know the right people, but to know the right questions to ask those people!  With that I’d like to thank, Steven Pickels for his extensive help with my many barcode scanner questions, William Dubel for direction with the line tracking and power supply circuits, Max Koessick for cutting out my platform and teaching me AVR assembly code.  In addition I’d like to thank a couple students in the class as well, Jeffery Cohan for the photo-resistors, Stephen Corbett for sending me PWM example code, and the builder of “please-don’t-suck-bot” for the servo wheels he offered me in my time of need.

Last I’d like to thank A. A. Arroyo and E. M. Schwartz for the opportunity they gave us to take this class.  Thanks to their efforts and teachings the world of robotics has been opened to us all.

# Appendix

Completed Code:

; *****************************

; **	Kevin E Kane         	 **

; **	7/28/2004               	 **

; **	EEL 5666, UF	 **

; ** 	Card Shark 2.d	 **

; ***************************************************************

; * Program Discription:  Is the second try at intergrating the blackjack  	*

; * playing program of Card Shark with the actuation fuctions.  These    	*

; * actuations include the servos used for driving, sensors for line      	*

; * detection and additional servos for placing bets (starting, splitting 	*

; * and doubling down.  					*

; ***************************************************************

; ***********Notes To Yourself*****************

; 1)	PortA is for LCD screen!

; 2)	PortB is for anything (pin 3 is a timer PWM output, OC0)

; 3)	PortC is for LEDs (NOTE: only pins 7, 6, 1, and 0 seem to work)

; 4)	PortD is for TX(transmiter), RX(resiever), V0, V1(eyes),

;		PWM0, PWM1 (wheels), OC2 (splitting/dd pin)

; 5)	@@@@@@ = means these lines may be removed but are used for trouble shooting

.nolist

.include "C:\Program Files\Atmel\AVR Tools\AvrAssembler\Appnotes\m32def.inc"

.list

;***** Declarations ***************************

.def	temp0		=r16	; Temporary register 0

.def	temp1		=r17	; Temporary register 1 (for UART I/O)

.def	temp2		=r18	; Temporary register 2 (for LCD screen/PortA)

.def	temp3		=r19	; Temporary register 3

.def	Delay1		=r20	; Delay variable 1

.def 	Delay2		=r21	; Delay variable 2

.def	Delay3		=r22	; Delay variable 3

.def	mpr		=r23	; "Multi-Purpose Register" (used for servos)

.def	mpr2		=r24	; "Multi-Purpose Register", number 2 (used for servos)

;***** Interrupt Vectors ******************************

.org 	$000

rjmp 	Init			; Starting Line (jump to Init)

.org	$01A

rjmp	Scan			; UART Receive Complete Interrupt Vector Address ($01A)

;******************************************************

;***** INITIALIZATIONS!!! *****************************

Init:

;       ****Port Setups****

ser		temp0

out		DDRA,temp0			; Set PORTA to all outputs

out		DDRB,temp0			; Set PORTB to all outputs

ldi		temp0,0b10111111

out		DDRC,temp0			; Set PORTC to all outputs except pin6 input

ldi		temp0,0b10110000

out		DDRD,temp0			; Set PORTD pins 4, 5 and 7 as outputs, all others are inputs

clr		temp0

out		PortA,temp0			; Set no "pullups" for PortA

out		PortB,temp0			; Set no "pullups" for PortB

out		PortC,temp0			; Set no "pullups" for PortC

ldi		temp0,0b10110000			; Set pins 4 and 5 as high initially,...

out		PortD,temp0			; ...no other "pullups" for PortD

;		****Enable Output Compare, 8-bit Timers (arms, mode 4)****

ldi		mpr,0b01110100		; (timer 0)		; mode 1 PWM, "set" on rise and...

out		TCCR0,mpr				; ..."clear" on fall, prescaler equals 256

ldi		mpr,$EF					; compare value

out		OCR0,mpr				; $EF = starting position

ldi		mpr,0b01110110	; (timer 2)		; mode 1 PWM, "set" on rise and...

out		TCCR2,mpr				; ..."clear" on fall, prescaler equals 256

ldi		mpr,$EC			; compare value = $EC (changed back at the end of init.)

out		OCR2,mpr				; NOTE:Place first bet, before LCD delays...

; ...and before Card Shark starts moving

;       ****START****

polling:				; polling for Port C, pin6 to be pushed

out		PORTB,temp0

sbis	PINC,0x06			; If (Port C, pin6 ==0)

rjmp	Start				; then jump to "start" and finish Initializations

inc		temp0				; else inc temp0 value and...

no_start:

dec		Delay1

brne	no_start

dec		Delay2

brne	no_start

rjmp	polling

Start:

;		****Stack Pointer setup****

ldi		temp0,high(Ramend)

out		SPH,temp0

ldi		temp0,low(Ramend)

out		SPL,temp0			; Stack pointer points to end of RAM ($085F)

;		****Drop first bet****

ldi		mpr,$F6				; compare value = $F6 (changed back at the end of init.)

out		OCR2,mpr				; NOTE:Place first bet, before LCD delays...

;		****Enable Output Compare, 16-bit Timer (Wheel PWMs, mode 8)****

ldi		mpr,0b11110000			; mode 8 PWM, "set" on rise and "clear" on fall

out		TCCR1A,mpr

ldi		mpr,0b00010100			; mode 8 PWM, prescaler equals 256

out		TCCR1B,mpr

ldi		mpr,$01

ldi		mpr2,$38

out		ICR1H,mpr				; Set "TOP" equal to $138

out		ICR1L,mpr2				; 20.0ms

;		****UART setup****

ldi		temp1,0b00000000

out		UBRRH,temp1

ldi		temp1,51			; Set UART for 9600 baud rate

out		UBRRL,temp1

ldi		temp1,0b10000000	; Receive complete flag

out		UCSRA,temp1

ldi		temp1,0b10010000	; Enable UART Receiver, and Receive Interrupt

out		UCSRB,temp1

ldi		temp1,0b10000110	; Enable Asynchronous UART operation,

out		UCSRC,temp1			; 8-bit data packs, and no parity

;		****Prepare LCD screen****

ldi		temp2,$00			; Set "enable" bit low/off

out		PORTA,temp2

;						(4-bit enable)

ldi		temp2,$03

call	LCDdelay

ldi		temp2,$03

call  	LCDdelay

ldi		temp2,$03

call	LCDdelay

ldi		temp2,$02

call  	LCDdelay

;						 (2-line enable)

ldi		temp2,$02

call	LCDdelay

ldi		temp2,$08

call  	LCDdelay

;						 (Display, Cursor, Blink)

ldi		temp2,$00

call	LCDdelay

ldi		temp2,$0F

call  	LCDdelay

;						 (Clear Home)

ldi		temp2,$00

call	LCDdelay

ldi		temp2,$01

call  	LCDdelay			; L.C.D. SCREEN READY!!!!

;	ldi		temp0,$99				;@@@@@@@@

;	out		PORTB,temp0			;@@@@@@@@

;		****Set Starting Variable Values****

ldi		temp0,$02

sts		final_card_number,temp0

clr		temp0

sts		card_number,temp0

sts		dealer_card,temp0

sts		players_hand_count,temp0

;		****betting arm returns to starting position****

call   	servo_delay			; delay longer for "arm" to finish its first move

ldi		mpr,$ED				; compare value = $ED

out		OCR2,mpr

ldi		temp0,$1F				;@@@@@@@@@@@@@

out		PORTB, temp0			;@@@@@@@@@@@@@

;		****Interrupt setup****

sei							; enable interrupts

;		****Lights signaling end of Init****

;	ldi		temp0,$AA			;@@@@@@@@@@

;	out		PORTC,temp0			;@@@@@@@@@@

;***********************************************

;***********************************************

;***********START OF MAIN PROGRAM!!!!***********

;*    This program follows a line while        *

;*    staying in the "mainloop" program.       *

Mainloop:	;***Start of Mainloop program*******

sbis	PIND,0x02				; is Pin number 2 of Port D (white) low?

rjmp	turn_left				; if so then jump to "turn_left"

sbis	PIND,0x03				; is Pin number 3 of Port D (white) low?

rjmp	turn_right				; if so then jump to "turn_right"

straight:

ldi		mpr,$01

ldi		mpr2,$1C

out		OCR1AH,mpr				; value $11C (middle forward)

out		OCR1AL,mpr2				; about 1.75ms

ldi		mpr,$01

ldi		mpr2,$025		; (right tire)

out		OCR1BH,mpr				; value $125 (middle forward)

out		OCR1BL,mpr2				; 1.25ms

ldi		temp0,$C3					;@@@@@@@@

out		PortB,temp0				;@@@@@@@@

ldi		Delay3,$10

call	DLY

rjmp	mainloop

turn_left:

ldi		mpr,$01

out		OCR1AH,mpr

ldi		mpr,$21					; value $119 (small reverse)

out		OCR1AL,mpr				; 1.45ms

ldi		mpr,$01

out		OCR1BH,mpr

ldi		mpr,$30					; value $130 (big forward)

out		OCR1BL,mpr				; 1.0ms

ldi		temp0,$0F					;@@@@@@@@

out 		portB,temp0					;@@@@@@@@

ldi		Delay3,$10

call	DLY

rjmp	mainloop

turn_right:

ldi		mpr,$01

out		OCR1BH,mpr

ldi		mpr,$20					; value $122 (small reverse)

out		OCR1BL,mpr				; 1.40ms

ldi		mpr,$01

out		OCR1AH,mpr

ldi		mpr,$10					; value $110 (big forward)

out		OCR1AL,mpr				; 1.0ms

ldi		temp0,$F0					;@@@@@@@@

out 		portB,temp0					;@@@@@@@@

ldi		Delay3,$10

call	DLY

rjmp	mainloop

;***********************************************

;*SUBROUTINES AND ENDS TO INTERRUPT ROUTINES!!**

;**************Subroutines**********************

;**********LCDdelay subroutine******************

LCDdelay:					; toggle PortA's pin 6 (enable pin to LCD)

out		PORTA,temp2			; load portA onto temp2

ori		temp2,0b01000000	; force pin 6 to be high/on

out		PORTA,temp2			; output new value to PortA

andi	temp2,0b10111111	; force pin 6 to be low/off

out		PORTA,temp2			; output new value to PortA

Del:						; create delay

dec 	Delay1

brne	Del

dec		Delay2

brne	Del

ret							; return from "LCDdelay" subroutine

;**********servo_delay subroutine******************

servo_delay:

ldi		Delay3,$18

DLY:

dec		Delay1

brne	DLY

dec		Delay2

brne	DLY

dec		Delay3

brne	DLY

ret

;**********dealer_card subroutine******************

dealer_card_sub1:			; store value from ZL into dealer_card (dealer's shown card)

sts		dealer_card,ZL

ldi		temp3,$0F					;@@@@@

out		PortB,temp3				;@@@@@

inc		temp0			; increament the card number your looking at ($00 --> $01)

sts		card_number,temp0		; load new value into "card_number" (value = $01)

ldi		temp2,$87			; (line up and down)

call	LCDdelay

ldi		temp2,$8C

call  	LCDdelay

ldi		temp2,$82			; (blank)

call	LCDdelay

ldi		temp2,$80

call  	LCDdelay

reti						; return from UART interrupt

;**********player_card_one subroutine******************

player_card_one1:			; store value of your first card into "players_hand_count"

sts		players_hand_count,ZL

ldi		temp3,$F0					;@@@@@

out		PortB,temp3				;@@@@@

inc		temp0			; increament the card number your looking at ($01 --> $02)

sts		card_number,temp0	; load new value into "card_number" (value = $02)

reti					; return from UART interrupt

;**********do_Split subroutine******************

do_Split1:					; rountine that actually calls to "Split"

ldi		mpr,$ED

out		OCR0,mpr			; move 1 of split bet pattern

;						(S)

ldi		temp2,$85

call	LCDdelay

ldi		temp2,$83

call  	LCDdelay

;						(p)

ldi		temp2,$87

call	LCDdelay

ldi		temp2,$80

call  	LCDdelay

;						(l)

ldi		temp2,$86

call	LCDdelay

ldi		temp2,$8C

call  	LCDdelay

;						(i)

ldi		temp2,$86

call	LCDdelay

ldi		temp2,$89

call  	LCDdelay

;						(t)

ldi		temp2,$87

call	LCDdelay

ldi		temp2,$84

call  	LCDdelay

call   	servo_delay				; delay longer for "arm" to finish its first move

ldi		mpr,$EF

out		OCR0,mpr				; move 2 of split bet pattern

rjmp	recheck

;**********do_Double subroutine******************

do_double1:					; rountine that actually calls to Double Down ("DD")

ldi		mpr,$F1

out		OCR0,mpr				; move 1 of Doubling Down bet pattern

;						(D)

ldi		temp2,$84

call	LCDdelay

ldi		temp2,$84

call  	LCDdelay

;						(D)

ldi		temp2,$84

call	LCDdelay

ldi		temp2,$84

call  	LCDdelay

call	servo_delay				; delay longer for "arm" to finish its first move

ldi		mpr,$EF

out		OCR0,mpr				;  move 2 of Doubling Down bet pattern

rjmp	done

;**********do_Hit subroutine******************

do_hit1:					; rountine that actually calls to "Hit"

;						(H)

ldi		temp2,$84

call	LCDdelay

ldi		temp2,$88

call  	LCDdelay

;						(i)

ldi		temp2,$86

call	LCDdelay

ldi		temp2,$89

call  	LCDdelay

;						(t)

ldi		temp2,$87

call	LCDdelay

ldi		temp2,$84

call  	LCDdelay

rjmp	recheck

;**********do_Stand subroutine******************

do_Stand1:					; rountine that actually calls to "Stand"

;						(S)

ldi		temp2,$85

call	LCDdelay

ldi		temp2,$83

call  	LCDdelay

;						(t)

ldi		temp2,$87

call	LCDdelay

ldi		temp2,$84

call  	LCDdelay

;						(a)

ldi		temp2,$86

call	LCDdelay

ldi		temp2,$81

call  	LCDdelay

;						(n)

ldi		temp2,$86

call	LCDdelay

ldi		temp2,$8E

call  	LCDdelay

;						(d)

ldi		temp2,$86

call	LCDdelay

ldi		temp2,$84

call  	LCDdelay

rjmp	done

;*************JUMPING ZONE #1 !!!!*************

dealer_card_sub:

rjmp	dealer_card_sub1

recheck:

rjmp	recheck1

player_card_one:

rjmp	player_card_one1

;**********************************************

;*********INTERRUPT SERVICE ROUTINE!!!*********

;******Scanner!!! (start of card anylasis)*****

Scan:					; Card has just been read

in		ZL,UDR				; reads data in and stores it in Low 8-bit Z register

subi	ZL,0x30				; subtract $30 from data read (change from ASCII to card value)

rjmp	output_number

back:

ldi		temp2,$82			; (blank)

call	LCDdelay

ldi		temp2,$80

call  	LCDdelay

cpi		ZL,$00				; compare ZL to $00, if ZL is not $00,...

brne	not_10				; ...then skip next line

ldi		ZL,$0A				; replace $00 with $0A in ZL register (10 value)

not_10:	 lds	temp0,card_number	; load temp0 with "card_number"

cpi		temp0,$00			; is this the dealer's card?

breq	dealer_card_sub		; if so, then jump to "dealer_card_sub"

cpi		ZL,$01				; compare ZL to $01, if ZL is not $01,...

brne	not_Ace				; ...then skip next two lines

ldi		temp3,$01

sts		soft,temp3			; flag for going to the soft table later

not_Ace: cpi	temp0,$01		; is this the player's first card?

breq	player_card_one		; if so, then jump to "player_card_one"

cpi		temp0,$02			; is this the player's second card?

brne	no_split_no_dd1		; if NOT, then jump to "no_split_no_dd1"

lds		temp0,players_hand_count ; is this card value the same as...

cp		temp0,ZL			; ...the first card (i.e. current hand count)

breq	question_split		; is so then jump to "question_split"

;	lds		temp0,players_hand_count		;@@@@@@?@@@????

cpi		temp0,$01			; is the first card an Ace?

breq	question_double2	; if so, then check for doubling down

cpi		ZL,$01				; is the second card an Ace?

breq	question_double2	; if so, then check for doubling down

add		temp0,ZL			; add values and see if total equals 9, 10, 11

cpi		temp0,$0B			; is it 11?

breq	do_double			; if so, then ALWAYS double down!

cpi		temp0,$0A			; is it 10?

breq	question_double		; is so then check for doubling down

cpi		temp0,$09			; is it 9?

breq	question_double 	; is so then check for doubling down

no_split_no_dd1:		; NO special bets! only soft or hard...

lds		temp3,card_number

inc		temp3				; increment "card_number"

sts		card_number,temp3

lds		temp0,players_hand_count

add		temp0,ZL			; NOTE: adding must be done here (again),...

sts		players_hand_count,temp0 	; ...incase above adding was skipped

lds		temp0,final_card_number	; load temp0 with number of cards player has (i.e. 2 -> 3 -> 4...)

out		PortB,temp3						; @@@@@@

dec		temp3			; NOTE: temp3 = "card_number" value from above

cp		temp0,temp3			; compare final card with (card_number - 1)

breq	hard_or_soft				; if equal then no more players cards to read, so jump to hard_or_soft

reti								; return from UART interrupt

hard_or_soft:			; go to hard table or soft table?

lds		temp3,soft

cpi		temp3,$01					; checking if "soft flag" is set

breq	soft_table					; if so then branch to "soft_table"

rjmp	hard_table					; else jump to "hard_table"

;***************JUMP ZONE #2 !!!!**************

do_split:

jmp		do_split1

do_double:

jmp		do_double1

do_hit:

jmp		do_hit1

do_stand:

jmp		do_stand1

question_double2:

jmp		question_double21

no_split_no_dd:

jmp		no_split_no_dd1

;*********************************************

;***********BASIC PLAY TABLES/SYSTEMS*********

;				****Split Table***

question_split:		; Checking to split or not to split,... that is the question

lds		temp0,dealer_card	; load the dealer's card into temp0 for studying

cpi		ZL,$00				; compare ZL to $00, Is it a 10, J, Q, K?

breq	no_split_no_dd		; if so then jump to "no_split_no_dd"

cpi		ZL,$01				; compare ZL to $01, Is it a Ace?

breq	do_split			; if so then jump to "do_split"

cpi		ZL,$02				; compare ZL to $02, Is it a 2?

breq	do_split						;+++++++++

cpi		ZL,$03				; compare ZL to $03, Is it a 3?

breq	do_split						;+++++++++

cpi		ZL,$04				; compare ZL to $04, Is it a 4?

breq	do_split						;+++++++

cpi		ZL,$05				; compare ZL to $05, Is it a 5?

brne	s6					; if not then check for 6's

lds		temp0,players_hand_count ; if 5, then...

add		temp0,ZL			; ...add temp0 and players_hand_count,...

rjmp	question_double		; ...then jump to question_double!

s6:	cpi		ZL,$06				; compare ZL to $06, Is it a 6?

breq	do_split						;+++++++++

cpi		ZL,$07				; compare ZL to $07, Is it a 7?

breq	do_split						;+++++++

cpi		ZL,$08				; compare ZL to $08, Is it a 8?

breq	do_split

cpi		ZL,$09				; compare ZL to $09, Is it a 9?

breq	do_split						;+++++++

reti					; NOTE: not a valied card number! (leave)

;				****Double Down Table***

; Checking to double or not to double,... that is the question

question_double:		; player has 9 or 10 total, find out if player should double down

lds		temp3,dealer_card	; loads the dealers card into temp3 for choosing

cpi		temp3,$0A			; if dealers card is a 10

breq	no_split_no_dd		; then don't dd (leave)

cpi		temp3,$0B			; if dealer's card is an Ace

breq	no_split_no_dd		; then don't dd (leave)

cpi		temp0,$0A			; compare temp0 to $10, Is total 10?

breq	do_double			; if you have 10, then dd

cpi		temp3,$09			; if...$09

breq	no_split_no_dd		; then don't dd (leave)

cpi		temp3,$08			; if...$08

breq	no_split_no_dd		; then don't dd (leave)

cpi		temp3,$08			; if...$07

breq	no_split_no_dd		; then don't dd (leave)

rjmp	do_double			; dealers card must be less then 7, player must have 9 (DD)

question_double21:		;****player has an A in first two cards, should he/she double?****

lds		temp3,dealer_card	; loads the dealer's card into temp3 for studying

rjmp	do_double				;++++++++

;				****Soft table (no DD option)***

soft_table:

inc		temp0

sts		final_card_number,temp0

lds		temp0,players_hand_count

ldi		temp0,$55

out		PortB,temp0

rjmp	do_stand				;++++++

;				****Hard table (no DD option)***

hard_table:

inc		temp0

sts		final_card_number,temp0

lds		temp0,players_hand_count

cpi		temp0,$10

breq	h16

andi	temp0,0b11110000

cpi		temp0,0b00010000

breq	d_stand				;++++++

h16:

rjmp	do_hit				;++++++

;***************JUMP ZONE #2 !!!!**************

d_stand:

rjmp		do_stand1

;**********************************************

;*************CLOSING HAND!*******************

recheck1:						; end of your choice (Hit/Split)

clr		temp0

sts		card_number,temp0

sts		dealer_card,temp0

sts		soft,temp0

sts		players_hand_count,temp0

;	ldi		temp3,$18						;@@@@@

;	out		PortB,temp3					;@@@@@

reti						; leave interrupt service routine

done:							; end of your hand (DD/Stand)

ldi		temp0,$02

sts		final_card_number,temp0

clr		temp0

sts		card_number,temp0

sts		dealer_card,temp0

sts		soft,temp0

sts		players_hand_count,temp0

;	ldi		temp0,$99						;@@@@@@@@

;	out		PORTB,temp0					;@@@@@@@@

reti						; leave interrupt service routine

output_number:

lds		temp0,card_number	; load temp0 with "card_number"

cpi		temp0,$00			; is this the dealer's card?

brne 	no_clear

ldi		temp2,$00		; (Clear Home)

call	LCDdelay

ldi		temp2,$01

call  	LCDdelay

no_clear:

cpi		ZL,0x00			; compare ZL to $00, if

breq	zero			; ZL(data - $30) is $00 then jump to zero

cpi		ZL,0x01

breq	one

cpi		ZL,0x02

breq	two

cpi		ZL,0x03

breq	three

cpi		ZL,0x04

breq	four

cpi		ZL,0x05

breq	five

cpi		ZL,0x06

breq	six

cpi		ZL,0x07

breq	seven7

cpi		ZL,0x08

breq	eight8

cpi		ZL,0x09

breq	nine9

reti		;(note not a valied card number!)	; leave interrupt service routine

seven7:

jmp		seven

eight8:

jmp		eight

nine9:

jmp		nine

zero:

ldi		temp2,$83			; (10)

call	LCDdelay

ldi		temp2,$81

call  	LCDdelay

ldi		temp2,$83

call	LCDdelay

ldi		temp2,$80

call  	LCDdelay

rjmp	back					; leave interrupt service routine

one:

ldi		temp2,$84

call	LCDdelay

ldi		temp2,$81

call  	LCDdelay

rjmp	back					; leave interrupt service routine

two:

ldi		temp2,$83

call	LCDdelay

ldi		temp2,$82

call  	LCDdelay

rjmp	back					; leave interrupt service routine

three:

ldi		temp2,$83

call	LCDdelay

ldi		temp2,$83

call  	LCDdelay

rjmp	back					; leave interrupt service routine

four:

ldi		temp2,$83

call	LCDdelay

ldi		temp2,$84

call  	LCDdelay

rjmp	back					; leave interrupt service routine

five:

ldi		temp2,$83

call	LCDdelay

ldi		temp2,$85

call  	LCDdelay

rjmp	back					; leave interrupt service routine

six:

ldi		temp2,$83

call	LCDdelay

ldi		temp2,$86

call  	LCDdelay

rjmp	back					; leave interrupt service routine

seven:

ldi		temp2,$83

call	LCDdelay

ldi		temp2,$87

call  	LCDdelay

rjmp	back					; leave interrupt service routine

eight:

ldi		temp2,$83

call	LCDdelay

ldi		temp2,$88

call  	LCDdelay

rjmp	back					; leave interrupt service routine

nine:

ldi		temp2,$83

call	LCDdelay

ldi		temp2,$89

call  	LCDdelay

rjmp	back						; leave interrupt service routine

;***********************************************************************

;**************Variables Defined!!**************************************

.org	$500

.dseg

card_number:	.byte 1		; The number of the player's card being looked at/next,

; Note: the dealer's card is "card_number" zero ( $00 )

dealer_card:	.byte 1		; value of dealer's card

players_hand_count:	.byte 1		; players total hand count

final_card_number:	.byte 1 		; holds the value that card_number is counting up to!

soft:		.byte 1		 ; pin-0 is used as a flag for when player's hand is soft

Line tracking ciruit:

(from William Dubel’s report on line tracking)

Circuit outputs logic high when a black line is detected.