Christopher P. Heagney

1 August, 2005

University of Florida

Department of Electrical and Computer Engineering

EEL 5666 - Intelligent Machine Design Laboratory

TAs: William Dubel & Steven Pickles

Instructors: Dr. A. A. Arroyo & Dr. E. M Schwartz

Final Written Report

Table of Contents:

Abstract ………………………………………………………………………………….. 3

Introduction ……………………………………………………………………………… 4

Integrated System ………………………………………………………………………... 4

Mobile Platform …………………………………………………………………………. 6

Actuation ………………………………………………………………………………… 7

Sensors …………………………………………………………………………………... 8

Behaviors …......................................................................................................................12

Conclusion ……………………………………………………………………………....13

Documentation…………………………………………………………………………...14

Appendices …...................................................................................................................14

Abstract

RoboShaQ is an autonomous, land based robot platform capable of shooting a free throw basket with the same skill as the famous pro basketball player Shaquille O'Neal.  It will move up to and detect the free throw line, measure distance to the basket, calculate desired trajectory, and fire a nerf ball into a basket.  This will be accomplished using infrared photo reflectors to detect the free throw line and line up to it.  RoboShaQ will then use infrared distance measuring electronics to determine the distance to the basket.  Taking into account the distance from the goal, RoboShaQ will then initiate a launch propelling a nerf ball through the air and into the basket.

Introduction

The idea for this project is to design and build a quick and simple land based robot.  One that is capable of simple tasks such as obstacle avoidance and line detection.  It will also be equipped with a powerful motor capable of catapulting a nerf ball through the air into a basket.  Using the high powered Atmel Atmega128 microcontroller as the 'brains' and sensitive IR photo reflectors, IR range finders, bump switches, and iMEMS sensors (Integrated Micro Electro-Mechanical Systems), the RoboShaQ will be more than well equipped to accomplish the stated goal.  Taking into account the failure of my previous project, the more sophisticated Autopilot, time is of the essence with this new design.  Having only six days to design and build a new robot was quite the challenge, but very rewarding once it was finally completed.

Integrated System

RoboShaQ was created using a Mavric-IIB Atmega128 Board (Figure 1) as the brains, and a host of sensors, including IR Range finder (Figure 2) for forward looking obstacle avoidance and distance measurement, IR Reflectors (Figure 3) for line detection, Bump Switches (Figure 4) for obstacle avoidance, ADXL320 Solid State Accelerometer (Figure 5) to measure tilt, an ADXRS401 Solid State Gyro (Figure 6) to measure rate of change of yaw, and an RC Hummer (Figure 7) as the body.

Figure 1.

Figure 2.                                                           Figure 3.

Figure 4.                                                                     Figure 5.

Figure 6.                                                                     Figure 7.

The IR range finder, accelerometer, and gyro sensors are interfaced through Port F on the Mavric-IIB board via ADC interface.  The servos are all run through the available servo channels built into the board.  The IR reflectors and bump switches are digital and interfaced through the many digital I/O pins on board.

Mobile Platform

The mobile platform is a pre-constructed plastic RC car from Toys-R-Us.  I decided to go with a pre-built RC car simply because of time issues.  After the Autopilot project was abandoned, I did not have time to order parts and wait for them to arrive.  Using an RC Hummer has the advantage of a ready to roll chassis, motors/controllers, wheels, battery back/charger, and everything I need already in the box.  Its only disadvantage is having to hack the electronics to gain control of the motors.  The final chassis has full mobility and hides all the wires and electronics inside the case, making for a nice finished presentation.

Actuation

RoboShaQ is powered by two simple RC car motors.  Motor control in this particular RC is ultra simplistic, having 3 modes, on, off, and reverse.  A lack of pwm hardware requires me to control it through software.  This was originally an issue because the RC car traveled so fast at first.  But after the catapult contraption was added, the weight was high enough to bog down the motors to a comfortable traveling speed.  Uses of simple 9 gram servos (Figure 8) to control motor direction and a powerful electric fan motor (Figure 9) to drive the catapult arm.  Servo control is easily done via the onboard servo pins.  While catapult motor control is accomplished via a 30V 10A relay.  The relay is triggered by a signal from one of the digital I/O pins, then run into a 2N2222 transistor to force the relay into position.  Software code is fairly simple also.  Servo control, as well as relay control, is shown in my software code attached below.

Figure 8.                                                  Figure 9.

Sensors

RoboShaQ comes equipped with many powerful sensors that supply a plethora of information on all external events.  These sensors include a forward looking IR range finder, IR reflectors, bump switches, dual-axis accelerometer, gyroscope, and temperature sensor.  These sensors will be broken down in detail below.

IR range finder (Figure 10)

Figure 10.

The first priority of the forward looking IR range finder is obstacle avoidance.  If an object gets too close to the front of the car, it will detect an eminent collision and take evasive action before any contact is made.  It also functions to measure the distance to the basket.  This is useful in judging how hard the catapult needs to launch the ball.  Interface with the IR range finder is accomplished via the built-in ADC port, Port F.  The ADC pin returns a value in software between 0-1024.  The closer the object is, the higher the number gets.  This allows for me to easily set the desired sensor threshold, in software, of a certain action.

IR reflectors (Figure 11)

Figure 11.

The array of IR reflectors/detectors are used in line detection.  They are mounted underneath the front bumper on the RoboShaQ.  IR reflectors vary their output as a result of the amount of IR reflected back off of an object.  Because white reflects more than black and because closer objects reflect more than farther objects, this is an easy way to accomplish line detection.  The array is interfaced through digital I/O pins.  Values returned are either 0 or 1.  Thus allowing software to easily determine if the front of the care is on a line or not.

Bump Switches (Figure 12)

Figure 12.

Bump Switches are incredibly simple.  If the button is pressed, it shorts a pullup resistor connected to the digital pin, driving it from its normal state of +5V down to ground.  Thus, allowing me to check if it has been pressed very simply by polling a digital I/O pin.  If the button has been pressed, then there has been a collision and the car must now backup and avoid the obstacle.  This is the most basic form of obstacle avoidance.

iMEMS dual-axis accelerometer (Figure 13)

Figure 13.

The Integrated Micro Electro-Mechanical Systems dual-axis accelerometer onboard RoboShaQ has no purpose for the robots current function.  It is simply a left over from the original Autopilot project I began this semester.  The sensor, originally intended to measure changes in pitch and roll of an aircraft, now still measures pitch and roll…however irrelevant that is to a ground based robot.  Although, useless, it is still functioning and returning data to the boards ADC port.

iMEMS gyroscope (Figure 14)

Figure 14.

Very similar to the accelerometer, the gyroscope on the RoboShaQ is also irrelevant.  It returns data of rate of change in the yaw axis on the car via the ADC port.  This data, although not used for the task of shooting a basket, it available and functioning without any problems.

Temperature Sensor (Figure 14)

Finally, RoboShaQ comes equipped with a temperature sensor left over from the original Autopilot project.  Originally intended to keep data on internal fuselage temperatures and control overheating issues, it now just measures room temperature around the RC car.  This sensor also interfaces through the ADC port on the Atmega128.

Behaviors

RoboShaQ is a simple, barebones robot with only basic behaviors.  Three behaviors define this robot.  First, obstacle avoidance, is accomplished using two types of sensors.  A forward looking IR range detector detects if an object is getting too close and calls for evasion before any contact takes place.  The other sensors used are bump switches places on the corners to detect contact and adjust actions accordingly.  Second, line detection, is accomplished by using IR reflectors to measure light reflected back from an object, in this case, a line on the floor.  It returns a digital on/off signal if the line on the ground reflects enough IR light back or not.  Thus, allowing for simple detection of lines or objects.  Finally, shooting the basket is accomplished by sending a digital +5V signal out to a relay that is connected to a higher voltage battery pack.  That pack then drives a powerful motor which catapults the ball into the air.  The +5V signal is driven to the desired turn on voltage via a 2N2222 transistor in line with the relay coil.

Conclusion

This project has taught me a lot about how to build a robot and how not to build a robot.  This class forces you to go back and master the basics.  With little room for error, you find yourself checking and rechecking all of the basics to find any mistakes you may have made before you hook up power to your precious robot.  That I think is the greatest benefit of this hands-on design experience.  Coming in to this class, my only experience with microcontrollers was EEL4744C.  Coming out of this class, I now know how to research parts, read pinouts, design entire board schematics, plan board layouts, send out boards to be custom made, interface a variety of components, and do much much more than I previously knew possible.  After beginning this semester with high hopes of flight, I have grown a much healthier respect for the difficulty involved with the fundamentals of flight.  I feel however, that I have conquered the complications of stability and control and may pursue my dreams of autonomous flight at a later time with better components.  When it came down to crunch time, however, I am proud that I could put together an entire robot in only the final week.  Although basic, RoboShaQ introduced me to new electronics that I would have otherwise not understood or tried to learn.  Some say that with every failure there is a success looming just around the corner.  It appears that even though my initial goal or autonomous flight remains unrealized, the lessons learned from this experience will greatly shape my future as an engineer and as a person.

You can check on my webpage http://plaza.ufl.edu/buffstud for pictures and video of the project in action.

Documentation

The following is a list of resources used:

1. Mavric-IIB Board Manual  http://www.bdmicro.com/images/mavric-iib.pdf

2. ADXL320 Data Sheet  http://www.analog.com/UploadedFiles/Data_Sheets/84033828ADXL320_0.pdf

3. ADXRS401 Data Sheet  http://www.analog.com/UploadedFiles/Data_Sheets/279107307ADXRS401_0.pdf

4. Lassen IQ Data Sheet  http://www.sparkfun.com/datasheets/GPS/Lassen%20iQ_Reference%20Manual.pdf

5. Fairchild 74LVX4245 Data Sheet  http://www.fairchildsemi.com/ds/74/74LVX4245.pdf

6. Eagle 4.14 Manual  ftp://ftp.cadsoft.de/pub/program/4.14/manual-eng.pdf

Appendices

This code is written in Bascom, a BASIC language compiler for AVR chipsets.  I have tried to make it as easy as possible to read by italicizing comments.  Enjoy  

'*******************************************************************************

'*  Christopher P. Heagney                                                                                                                  *

'*  Intelligent Machine Design Laboratory                                                                                          *

'*  Summer 2005                                                                                                                                  *

'*  University of Florida                                                                                                                       *

'*  College of Electrical and Computer Engineering                                                                           *

'*                                                                                                                                                          *

'*  Robot Name: RoboShaQ                                                                                                                *

'*  Robot Function: To detect a free throw line, line up to it, measure                                               *

'*                  to the basket, and launch a ball into the basket.                                                             *

'*******************************************************************************

'*******************************************************************************

'*  Hardware connections                                                                                                                    *

'*******************************************************************************

'Pinb.4 is connected to left IR reflector

'Pinb.5 is connected to center IR reflector

'Pinb.6 is connected to right IR reflector

'Pind.5 is connected to left front bump switch

'Pind.6 is connected to center front bump switch

'Pind.7 is connected to right front bump switch

'ADC Pin0 is connected to forward IR range finder

'ADC Pin1 is connected to x-axis accelerometer

'ADC Pin2 is connected to y-axis accelerometer

'ADC Pin3 is connected to gyroscope

'ADC Pin4 is connected to temperature sensor

'Pind.4 is connected to catapult relay

'IR range finder:  Orange(GND)   Yellow(VCC)

'Bumper switch array:  Orange(GND)   Red(VCC)

'Servo Power from board:  Grey(GND)   White(VCC)

'Servos Power:  Grey(GND)   Purple(VCC)

'IR Reflectors:  Purple(GND)   Blue(VCC)  x2

'                Black(GND)   Brown(VCC)

'                Grey(GND)   White(VCC)

'*******************************************************************************

'*  Configuations                                                                                                                                 *

'*******************************************************************************

'Config chipset and crystal frequency

$crystal = 14745600

$regfile = "m128def.dat"

'Turns on onboard LED to show power is on

Config Portb = &B00000001

Portb = 255                                                 'set portb.0 as high

'Config LCD

Config Lcdpin = Pin , Db4 = Porta.4 , Db5 = Porta.5 , Db6 = Porta.6 , Db7 = Porta.7 , E = Porta.3 , Rs = Porta.2

Config Lcd = 20 * 4                                         'configure lcd screen size

'Config ADC

Config Adc = Single , Prescaler = Auto , Reference = Avcc

Start Adc

'Config Servos

Config Servos = 4 , Servo1 = Portc.0 , Servo2 = Portc.1 , Servo3 = Portc.2 , Servo4 = Portc.3 , Reload = 20

Config Portc = Output

'Config PortD as digital input for bump switches and IR reflectors

Config Portd = &B00010000

Enable Interrupts

'*******************************************************************************

'*  Dimension Variables                                                                                                                      *

'*******************************************************************************

Dim Bootcount As Byte                                       'variable used in bootup sequence

Dim Distance As Word                                        'variable to hold IR distance value

Dim Screenrefresh As Byte                                   'used to temporarily refresh the screen

Dim Irscancomplete As Byte                                  'used in line detection

Dim Turnwhichway As Byte                                    'variable to store which direction to turn

Dim Whichsensordetectedimpact As Byte                       'which sensor detected impact?

Dim Linedetect As Byte                                      'which sensors detected a line?

Dim Motoraction As Byte                                     'what do you want the motors to do?

Dim Servo1stop As Byte                                      'Servo Configuration Values

Dim Servo1forward As Byte

Dim Servo1backward As Byte

Dim Servo2center As Byte

Dim Servo2left As Byte

Dim Servo2right As Byte

Dim Servo1 As Byte                                          'used in bootup servo configuration

Dim Servo2 As Byte

Dim Shot As Byte                                            'have we already shot the ammo?

'*******************************************************************************

'*  Declare Subroutines                                                                                                                        *

'*******************************************************************************

Declare Sub Avoidimpact(whichsensordetectedimpact As Byte)

Declare Sub Trackline(linedetect As Byte)

Declare Sub Gomotor(motoraction As Byte)

Declare Sub Shootbasket

'*******************************************************************************

'*  System bootup                                                                                                                                 *

'*******************************************************************************

Servo(1) = 55

Servo(2) = 50

Cls                                                         'clear the LCD display

Cursor Off Noblink                                          'makes the LCD cursor not blink

Lcd "Christopher Heagney"                                   'Prints my name on the first line of the LCD

Lowerline                                                   'move to the next line

Lcd "System Start in: "

'This loop will countdown 6 seconds before the system starts operating.  This

'  allows time to reprogram the controller, get the robot oriented as desired,

'  make any last second adjustments, display the programs creator, and basically

'  just look cool before everything starts.

For Bootcount = 6 To 1 Step -1                              'start a countdown loop

Locate 2 , 18                                            'place lcd curser at position 2,18

Lcd Bootcount

Locate 4 , 17

Lcd "IM  "

Waitms 250                                               'pause 250ms

Locate 4 , 17

Lcd " MD "

Waitms 250

Locate 4 , 17

Lcd "  DL"

Waitms 250

Locate 4 , 17

Lcd "I  L"

Waitms 250

Next Bootcount

'This next part is simply an initial bootup calibration.  The servos sometimes

'  move and hardcoded values are no longer accurate.  This allows the user to

'  very easily calibrate the servos before every start.  It basically starts

'  the servos at a certain value and asks if the motor is doing the desired

'  result.  If it it, they press a bump switch to let the program know it is

'  good and the program then stores that value.  This is repeated for all six

'  servo settings.  Now the program is ready to go, with all its nicely

'  calibrated values stored in memory.

Cls

Lcd "Servo 1 Diagnostics"

Locate 2 , 1

Lcd "Press Left Front"

Locate 3 , 1

Lcd "Bumper to Config"

Locate 4 , 1

Lcd "Servo 1 Forward:    "

Servo1 = 80

Do

Servo(1) = Servo1

Decr Servo1

Locate 4 , 18

Lcd Servo1

Waitms 150

Loop Until Pind.5 = 0

Servo1forward = Servo1

Waitms 150

Locate 4 , 1

Lcd "Servo 1 Stop:       "

Do

Servo(1) = Servo1

Decr Servo1

Locate 4 , 15

Lcd Servo1

Waitms 150

Loop Until Pind.5 = 0

Servo1stop = Servo1

Waitms 150

Locate 4 , 1

Lcd "Servo 1 Backward:   "

Do

Servo(1) = Servo1

Decr Servo1

Locate 4 , 19

Lcd Servo1

Waitms 150

Loop Until Pind.5 = 0

Servo1backward = Servo1

Servo(1) = Servo1stop

Waitms 300

Locate 4 , 1

Lcd "Servo 2 Left:       "

Servo2 = 70

Do

Servo(2) = Servo2

Decr Servo2

Locate 4 , 15

Lcd Servo2

Waitms 150

Loop Until Pind.5 = 0

Servo2left = Servo2

Waitms 150

Locate 4 , 1

Lcd "Servo 2 Center:     "

Do

Servo(2) = Servo2

Decr Servo2

Locate 4 , 17

Lcd Servo2

Waitms 150

Loop Until Pind.5 = 0

Servo2center = Servo2

Waitms 150

Locate 4 , 1

Lcd "Servo 2 Right:       "

Do

Servo(2) = Servo2

Decr Servo2

Locate 4 , 15

Lcd Servo2

Waitms 150

Loop Until Pind.5 = 0

Servo2right = Servo2

Waitms 150

Servo(2) = Servo2center

Wait 1

'Ok, the servos have been calibrated and their values stored in memory.  Now

'  press a button to continue to the main program.

Cls

Lcd "Press Left Bumper"

Lowerline

Lcd "To continue.."

Do

Loop Until Pind.5 = 0

Wait 1

'*******************************************************************************

'*  Main Program                                                                                                                                 *

'*******************************************************************************

Cls                                                         'clear the LCD display

Motoraction = 2                                             'set the robot in motion forward at beginning

Screenrefresh = 0                                           'counter to periodically refresh the screen

Shot = 0                                                    'default start condition is having ammo

Do

'Check to see if front IR is too close to something

Distance = Getadc(0)                                        'get IR distance measurement

If Distance > 300 Then                                      'If it's less than a threshold ammount, then panic

Whichsensordetectedimpact = 1                            'tell it impact is coming from the front

Call Avoidimpact(whichsensordetectedimpact)

End If

'Check to see if bumper switches have been pressed

If Pind.5 = 0 Then

Whichsensordetectedimpact = 3                            'tell it impact is coming from the left

Call Avoidimpact(whichsensordetectedimpact)

End If

If Pind.6 = 0 Then

Whichsensordetectedimpact = 1                            'tell it impact is coming from the front

Call Avoidimpact(whichsensordetectedimpact)

End If

If Pind.7 = 0 Then

Whichsensordetectedimpact = 2                            'tell it impact is coming from the right

Call Avoidimpact(whichsensordetectedimpact)

End If

'Check IR reflectors for line detection

If Pinb.4 = 0 And Pinb.5 = 0 And Pinb.6 = 0 Then

Linedetect = 1

Irscancomplete = 1

End If

If Irscancomplete = 1 Then

Goto Skipircheck

End If

If Pinb.4 = 0 And Pinb.5 = 0 Then

Linedetect = 2

End If

If Pinb.5 = 0 And Pinb.6 = 0 Then

Linedetect = 3

End If

If Pinb.4 = 0 And Pinb.6 = 0 Then

Linedetect = 4

End If

If Pinb.4 = 0 Then

Linedetect = 5

End If

If Pinb.5 = 0 Then

Linedetect = 6

End If

If Pinb.6 = 0 Then

Linedetect = 7

End If

'Linedetect basically stores which combination of line detectors see the line

'  or not.  Allowing for different behavioral responses.

Skipircheck:

If Linedetect <> 0 Then                                     'Did it detect a line?

Call Trackline(linedetect)                               'If so, then call trackline

End If

Call Gomotor(motoraction)

'Periodically refresh the screen

Incr Screenrefresh

If Screenrefresh = 255 Then

Screenrefresh = 0

Cls

End If

Loop

End                                                         'end the program

'*******************************************************************************

'*  Subroutines                                                                                                                                     *

'*******************************************************************************

Sub Avoidimpact(whichsensordetectedimpact As Byte)

Motoraction = 1                                          'all stop

Call Gomotor(motoraction)

Waitms 50

'If impact on the left, then tell it to turn to the right

'If impact on the right, then tell it to turn to the left

'if impact in the center, then randomly pick a direction to go

Select Case Whichsensordetectedimpact

Case 1

Locate 1 , 1

Lcd "Watchout For Impact!"

Locate 2 , 1

Lcd "Attempting Evasion.."

Turnwhichway = Rnd(1)

Case 2

Locate 1 , 1

Lcd "Impact on the Right!"

Locate 2 , 1

Lcd "Attempting Evasion.."

Turnwhichway = 0

Case 3

Locate 1 , 1

Lcd "Impact on the Left! "

Locate 2 , 1

Lcd "Attempting Evasion.."

Turnwhichway = 1

End Select

Motoraction = 3                                          'go backwards

Call Gomotor(motoraction)

Wait 2

If Turnwhichway = 0 Then

Motoraction = 4                                       'go left for a while then go straight again

Call Gomotor(motoraction)

Wait 1

Motoraction = 2

Call Gomotor(motoraction)

Else

Motoraction = 5                                       'go right for a while then go straight again

Call Gomotor(motoraction)

Wait 1

Motoraction = 2

Call Gomotor(motoraction)

End If

End Sub

Sub Trackline(linedetect As Byte)

Select Case Linedetect

'If all sensors on, then we've reached the free throw line and stop

Case 1

Locate 1 , 1

Lcd "Lined up, Ready shot"

Motoraction = 1

Call Gomotor(motoraction)

Wait 1

If Shot = 0 Then                                   'have we already shot the ammo?

Call Shootbasket                                'if not, then shoot it

Else                                               'if we did, then wait for reload

Cls

Lcd "Reload then press"

Locate 2 , 1

Lcd "Left front bumper.."

Do

Loop Until Pind.5 = 0

Wait 2

Call Shootbasket                                'once it's reloaded, shoot it!

End If

'If left and center are on, then go slowly left

Case 2

Locate 1 , 1

Lcd "Turn Left a little. "

Servo(2) = Servo2right                             'turn wheel right

Servo(1) = Servo1backward                          'then backup

Waitms 500

Servo(2) = Servo2center                            'then turn wheel straight

Servo(1) = Servo1forward                           'and go forward

'If right and center are on, then go slowly right

Case 3

Locate 1 , 1

Lcd "Turn Right a little."

Servo(2) = Servo2left                              'turn wheel right

Servo(1) = Servo1backward                          'then backup

Waitms 500

Servo(2) = Servo2center                            'then turn wheel straight

Servo(1) = Servo1forward                           'and go forward

'If left and right are on, then somethings screwed up..keep going straight

Case 4

Locate 1 , 1

Lcd "Ummm...Go Straight. "

Motoraction = 2

Call Gomotor(motoraction)

'If only left is on, then break left

Case 5

Locate 1 , 1

Lcd "Break Left.         "

Servo(2) = Servo2right                             'turn wheel right

Servo(1) = Servo1backward                          'then backup

Wait 1

Servo(2) = Servo2center                            'then turn wheel straight

Servo(1) = Servo1forward                           'and go forward

'If only center is on, then keep going straight

Case 6

Locate 1 , 1

Lcd "Keep Straight ahead."

Motoraction = 2

Call Gomotor(motoraction)

'If only right is on, then break right

Case 7

Locate 1 , 1

Lcd "Break Right.        "

Servo(2) = Servo2left                              'turn wheel right

Servo(1) = Servo1backward                          'then backup

Wait 1

Servo(2) = Servo2center                            'then turn wheel straight

Servo(1) = Servo1forward                           'and go forward

End Select

End Sub

Sub Gomotor(motoraction As Byte)

'Case 1: All stop

'Case 2: Forward

'Case 3: Backward

'Case 4: Left

'Case 5: Right

'Case 6: Left slowly

'Case 7: Right slowly

Select Case Motoraction

Case 1

Servo(2) = Servo2center

Servo(1) = Servo1stop

Case 2

Servo(2) = Servo2center

Servo(1) = Servo1forward

Case 3

Servo(2) = Servo2center

Servo(1) = Servo1backward

Case 4

Servo(2) = Servo2left

Servo(1) = Servo1forward

Case 5

Servo(2) = Servo2right

Servo(1) = Servo1forward

Case 6

Servo(2) = Servo2left

Servo(1) = Servo1forward

Waitms 100

Servo(1) = Servo1stop

Waitms 100

Servo(1) = Servo1forward

Case 7

Servo(2) = Servo2right

Servo(1) = Servo1forward

Waitms 100

Servo(1) = Servo1stop

Waitms 100

Servo(1) = Servo1forward

End Select

End Sub

Sub Shootbasket

Cls

Lcd "Fire!!              "

Portd = 255                                              'Pulse on catapult relay

Waitms 250

Portd = 0

Shot = 1                                                 'tell it that we have shot the ammo

End Sub