Jeffery the Tagnificent

Alan Schieltz

EEL5666

University of Florida

Summer 2006

Sensor Report

Introduction

Jeffery is a robot who will play tag with a tracking device.  He will attempt to find the tracking device then bump it then run away until he is bumped.  Then the process will start over.  While doing this he will attempt to avoid obstacles that are not the intended target.

Bump Sensor

The bump sensor is a button switch.  I have Vcc connected to one side of the switch and a resistor to ground as well as an input to the microprocessor board on the other side.  This will have the effect of when the switch is closed (Bumped/Pressed) the microprocessor will read Vcc and when the switch is open (Not Bumped/Not Pressed) it will read Ground.  I will have an interrupt enabled on a rising edge to detect when the sensor has been pressed.  2 bump sensors are placed on the front,  the each has their own strip of plastic to extend their reach.  This will allows me to cover the front of my robot with only 2 bumps.  The bump sensor is used to detect the tag.  The following is a circuit diagram of the bump switch.

Sonar (Devantech SRF05 Ultrasonic Ranger)

The sonar works by sending out a pulse of ultrasonic sound.  This sound then travels out in a cone shape until it hits and object and then is reflected back.  The Sonar sets a Pin high until the reflection is received or after some time out.  If you measure the time that the pin is high, then you know how far away something is, I.e.  If something is close, the pin will not be high very long.  By measuring this pulse Jeffery is able to know when things are in front of him and can then avoid them.  There is one sonar system on Jeffery on the back.  The back sensor is used to prevent collision when running away.

The following values were obtained by placing a cup the approximate distances from the sonar.  It is important to note that the cup was tall enough to cover the sensor up close.  A second test was done using a ball that was just under the sonar field of view when place up close.  The system was unable to detect the ball until it was moved 2in away from the sonar then the values were similar to the cup test.

| Distance | Value |
| --- | --- |
| 1in | 10 |
| 2in | 14 |
| 3in | 27 |
| 4in | 37 |
| 6in | 54 |
| 8in | 76 |
| 9in | 89 |
| 1fi | 104 |
| 2ft | 226 |
| Clear | 249 |

CMUcam

The CMU cam is a camera that is connected to a microprocessor.  This processor is used to control certain camera functions and it can perform basic tracking functions.  Its purpose on Jeffery is to allow him to find a blue target and relay its general position to the main processor.  This is done with a TTL serial interface.  The TW command is used to return the position of a color that was detected in the center of the viewing window.  This function was used to track the following target.  Two blue racquetballs were placed 3 in away from the camera.  The camera then was able to follow these blue racket balls about 18in away from the lens. It could also track them in a 60 degree field of view from the center of the lens.  It is important to note this was under a given get of lighting conditions and my be worse under a different set.

IR Beacon (Modified Remote Control and GP1U26X IR Receiver)

The camera is not enough to track the target.  So I added an IR system to help guide Jeffery in the right direction until the camera can take over.  The remote was modified to always have the 3 button pressed.  This way the Target is always sending out a signal.  There are 2 GP1U26X IR Receivers mounted on the front.  They read high when there is no signal and low when they see a 40khz modulated IR signal.  I connected these IR sensors to the analog to digital converter on my processor.  This would return an value of 1024 or 0 depending if it detected IR or not.  Then I would average the values with the last 9 readings taken and this would return to me a value of either 1024, if there was no IR or some value around 850 if their was IR.  Because the value changed so much I said that any average less then 950 counted as the IR dectecting the signal.  The Receivers are so good at seeing IR that if you don’t shield them they can see the remote from any direction.  To correct this I build cones out of paper and black electrical tape.  This allowed me to have directional IR detection.  The Two sensors are mounted on the left and right sides of my robot pointing forward.  These 2 sensors allow me to guide Jeffery to the target by having it go left if only the left reads, going right if only the right reads and going straight if both read.  This system works at a range of approximately 6ft.