Final Report

Name: Rafael Garcia

Robot Name: Change Collector

Course: EEL 5666 – Intelligent Machine Design Laboratory

Instructors: Dr. Antonio Arroyo

Dr. Eric M. Schwartz

TAs: Adam Barnett

Kevin Claycomb

08/07/07

Table of Contents

Abstract ……………………………………….. 2

Executive Summary ……………..…………….. 2

Introduction ……………………………….…… 3

Integrated System ………………………...……. 3

Mobile Platform ………………………….……. 4

Actuation …………………………………….… 6

Sensors ……………………………………….… 7

Behaviors …………………………………..….. 8

Conclusion ………………………………….…. 10

Appendix A (source code) …………………...… 11

Abstract

This report describes all of the systems involved in the creation of the Change Collector. Change Collector is an autonomous robot that roams across a white poster board searching for coins. When it finds one it picks it up, counts, and sorts the coin before continuing its search.

Executive Summary

The goal of this project was to construct a change collecting robot. This robot roams across the floor in search of coins and once it finds one, it picks it up, sorts it, and add its monetary value to a dollar amount of coins collected so far.

This report describes the algorithms and techniques used to accomplish the robot’s task. The robot’s mobile platform, actuation, sensors, and behaviors are explained. The entire source code (written in the C programming language) is included in Appendix A. This code was compiled using AVR Studio 4.

Introduction

The Change Collector wanders across white poster board in search of coins. It then picks them up, counts them, and continues the search. The total amount collected so far can be displayed at any time by simply pressing any one of its 16 keypad buttons. The main sensor, a CMUcam2+, both detects and distinguishes between the four main coin denominations: penny, nickel, dime, and quarter.

Integrated System

The robot has two main circuits with each having a few sub-circuits. The two main circuits have their own independent, regulated power supply. The circuits are classified as motor circuit and processor circuit.

Motor Circuit

This circuit is powered by 6 NiMH AA size batteries (7.2 V) regulated down to 4.8 volts. The regulator circuit is shown below:

Figure 1: Motor voltage regulator

Vin corresponds to the battery input and Vout is connected to three different motors across separate motor driver boards. The values for R1 and R2 are 76Ω and 220Ω respectively.

This regulation is necessary to ensure that the motors move a constant amount given a constant time and power. There are only 3 components connected to this power supply: the two front wheel motors and a motor used to drive the pulley. The two front wheel motors are controlled by a Pololu Low-Voltage Dual Serial Motor Controller (more information at www.pololu.com) powered by the independent processor circuit detailed below. The pulley motor is controlled by a separate but identical motor driver.

Processor Circuit

This circuit is powered by 5 NiMH AA size batteries (6 V) regulated down to 5V by the onboard regulator on the Maveric IIB board. The grasper-servo and arm-rotating-servo are connected directly to the 6V supply for more power and to reduce spikes seen by the processor. The CMUcam2+ is also connected directly to the 6V supply. All LEDs and the LCD screen are powered by the Maveric board. The flowchart on the next page shows the circuit hierarchy:

Figure 2: Power and Data Connection Hierarchy

Red lines represent power connections and blue lines show data signals. One important thing to note is that both servos and the CMUcam2+ are powered directly by the batteries without going through the MavericIIB board or a regulator. The camera has its own regulator and the servos can move to precise positions without the need of regulated power. The bottom portion of the chart shows the independent battery supply powering the pulley and wheel motors. These motors have a regulating circuit to ensure a constant amount moved for a given applied power and time duration.

Mobile Platform

The robot platform is a 18 x 32 cm rectangular ¼ “ birch wood board. It has rectangular holes in the front to allow the wheels to spin. The platform itself looks something like figure 3:

This platform is sturdy and has just enough area to hold all of the components needed by the robot to perform its duties. It has two caster wheels on the opposite side of the regular wheels to allow for easy turning without the need of a steering mechanism.

Figure 3: Robot Platform

Other Subsystems

CMU cam2

In addition to the camera, this system also includes a pair of 5000mcd white LEDs used to provide constant onboard illumination. A tin-foil lined box covers both the camera and the LEDs to prevent outside shadows or drastic light changes from affecting the camera. These additions have dramatically improved the camera’s performance and reliability.

LCD screen

A 16 character, 2-row (32 displayable characters total) is used to display the current status of the robot, total amount collected, and information received from the CMU camera. This display can also be used to navigate a menu that will allow certain features of the robot to be modified such as cancelling beeps, setting an amount of coins, calibrating motor speeds and timing, etc.

Beeper

A small piezo-speaker is attached next to the LED that emits beeps depending on the robot’s operation. Beeps signal changes in behaviors, bad packets received from the camera, and collision detection. This device can be switched off without the need to reprogram.

LED indicators

A set of 4 LED indicators are mounted just above the piezo-speaker and these indicate the type of coin detected as well as the current behavior. The white LED indicates a quarter was detected, the yellow one indicates a dime, the green one is a nickel, and the red one is a penny. When the robot is looking for coins, the lights flash continuously. A triggered bump switch or a high reading from the IR sensors stop the flashing until the robot spins around to avoid the obstacle.

Coin tubes

The robot has four separate coin tubes, each having the exact circumference of the type of coin they are meant to hold. They are made from transparent plastic to indicate when the tubes are almost full. At the top of the tubes are circular paper funnels used to direct the coins into the tubes and allow for a small margin of error from the grasper. The bottoms of the tubes have Velcro attached to them to allow for easy removal of the coin tubes.

Actuation

Pulley

The pulley system on the change collector controls the vertical position of the grasping mechanism. The type of string that has worked best is tooth-floss due to its wax coating and strength. A small DC motor controlled by a Pololu.com motor driver board spins the pulley wheels used to wind up the string. Rubber bands are used to transfer power from the motor to a small wheel and from this small wheel to the larger spindle holding the string. The rubber bands prevent the motor from stalling by slipping on the pulley wheels when the string tension is too high. They also keep the string still when the motor is not moving, despite the weight of the grasper.

Figure 4: Pulley System

Telescopic tubes

These colorful tubes hold the grasping mechanism and are controlled by the pulley system. They collapse and expand to allow the grasper to move down vertically to pick up coins and up to drop them in the coin tubes. Spinning along the vertical axis is prevented by a mechanical stop at the top of the tubes.

Grasper

The grasper has plastic scoops attached to the ends to pick up coins. It is powered by a small servo and is surprisingly reliable at grabbing coins off the ground, no matter what their size. It moves up and down by a pulley mechanism and the telescopic tubes mentioned in the previous section. A separate servo rotates the entire mechanism to place the grasper into a coin dropping position, based on the type of coin.

Figure 5: Grasper (without attached scoops)

Rotate servo

This standard sized servo has the entire pick-up mechanism mounted on it and its job is to precisely position the grasper into a position such that it can easily drop a coin into a specific tube. A servo is an excellent choice for this application because only 180° of movement are required and servos are very precise when calibrated properly. A multi-step movement algorithm implemented in software ensures smooth movement that will not cause the coin to be dropped accidentally.

Wheel Motors

The front wheels of the Change Collector are powered by independent 5V DC motors. A gearbox providing a 115:1 gear ratio converts the low-torque, high revolution motion of the motors to usable power. By spinning the wheels in opposite directions, the robot is able to turn in any direction.

Figure 6: Motor gearbox

Figure 7: Wheels used for Change Collector

Sensors

IR sensors

Two analog Sharp GP2Y0A21YK sensors purchased from www.pololu.com are mounted on the front of the robot to avoid obstacles while searching for coins. The stopping distance is set to about 6 inches which is not too far or too close and is on the linear portion of the graph below. The following graph taken from the datasheet shows the voltage-distance output relationship for this particular sensor.

Figure 8: Voltage-Distance graph for IR sensors

Bump switches

A pair of simple single-pole/single-throw switches with long levers are used to detect collisions. The IR sensors avoid obstacles most of the time but in rare cases these switches alert the robot of a collision. Since the wheels are mounted off-center, when the robot turns it tends to keep the wheel area of the platform still while the caster wheel area swings around and this can sometimes bump into obstacles. The moving area of the platform is precisely where the bump switches are mounted.

Figure 9: CMUcam2+

Special sensor: CMUcam2

This camera is used to detect coins on the ground. The camera notifies the robot when a coin is found and through the algorithm described earlier, it positions the robot in exactly the same position each time to pick up the coin. The camera is also used to determine the type of coin. The camera I used was purchased from www.acroname.com and has part number R245-CMUCAM2-PLUS.

The viewable area can be adjusted by varying the distance from the lens to the target image. A greater viewing area can be achieved by increasing this distance but with greater area comes lower resolution and lower image quality.

Figure 10: Original camera mounting

Initially the CMUCam2+ was mounted on the robot platform that is 4.5cm high. The camera lens extends below this height as shown in the figure to the left. The result was a very small viewable area. To remedy this, I added a 5cm stand on top of the robot platform to mount the camera at a greater distance and thus increase the viewable area. This dramatically increased the viewable area and several coins could now be seen at once. In its current form the robot can only work deal with a single coin in its view at once.

Figure 11: New camera mounting

Behaviors

A complete flowchart of the behaviors exhibited by the Change Collector is shown in figure 12.

Figure 12: Behavior Cycle

The robot initializes the pulse-width-modulation, universal-asynchronous-receiver/transmitter, motor drivers, and camera systems on start-up. It then starts moving forward, continually reading information from the camera, IR sensors, and bump switches. If the IR sensors or bump switches indicate a collision, the robot will back up and turn in a random direction to avoid the obstacle. When the camera eventually locates a coin, it initiates a centering and detecting algorithm explained in the following section.

The coin detection algorithm is detailed on the next page.

Figure 13: Coin Detection Algorithm

This flowchart shows the steps taken by the robot when detecting a coin. The source code in Appendix A (mainly, main.c) includes more specifics but this chart provides a good overview of what goes on. Through a series of tests on size and color, the Atmega128 microcontroller uses values read from the camera to determine what type of coin it sees. Once it has determined this, it goes through a series of motions to collect the coin and drop it in its corresponding tube.

Conclusion

To maintain the proper operation of the Change Collector, fresh batteries should always be used. The coin tubes should be emptied when full and the Velcro material on the underside of the coin tubes makes this easy. The robot also requires a white surface to be able to detect coins. With several hours of use, the rubber wheels will begin to stain the poster board. A poster board replacement is recommended each time batteries and changed. Except for the items mentioned, the robot should be able to autonomously perform its specified functions.

Part sources:

www.pololu.com

www.acroname.com

www.bdmicro.com

www.microcontrollershop.com

www.sparkfun.com

www.lowes.com

Appendix A (Change Collector source code)

misc.h

#ifndef __misc_H_

#define __misc_H_

#include <avr/eeprom.h>

char key; // Key pressed

char buf[16];

char printbuf[20];// Buffer used for printing to LCD

int duty;  // duty cycle used for beep frequency

int btime;  // time taken for a beep

int coin = 0; // variable stores coin type

int Bperc = 50;  // initial rotate servo value

int Bperc2 = 40;  // initial grasper servo value

int pSpeed = 0;	// initial pulley speed value

int fast = 0; // used to turn on fast (and unreliable) mode

int timeout = 0; // used when seeking takes too long

int begintime; // signals start of timer

uint16_t time30ms; // number of 30ms intervals elapsed

int size10 = 0; // number of 'size10' special cases

int mon = 0; // motor on variable

int mbdime = 0; // 'maybe dime' variable if special case 10 occurs frequently

#define NONE 0

#define QUARTER 1

#define NICKEL 2

#define PENNY 3

#define DIME 4

#define RETRY 700

#define ERASED 0 // address in EEPROM with erased value (255 if true)

#define GLUEOFF 1 // address in eeprom of glue offset value

#define PDT 2 // address in eeprom of pulley down time

#define PUT 3 // address in eeprom of pulley up time

#define PDS 4 // address in eeprom of pulley down speed

#define PUS 5 // address in eeprom of pulley up speed

#define BUT 6 // address in eeprom of back-up time value

#define NBP 7 // address in eeprom of no-beep value

#define PAMT 8 // address in eeprom of penny value

#define NAMT 9 // address in eeprom of nickel value

#define DAMT 10 // address in eeprom of dime value

#define QAMT 11 // address in eeprom of quarter value

#define TOOLONG 500 // 30ms intervals before timing out (~15sec)

// redefinitions of constants found in Motors.h for manual() and recal()

#define FORWARD 0

#define BACKWARD 1

#define PULLEY 4

#define RIGHT 3

#define LEFT 2

// prototypes needed for manual() and recal();

void init_LCD();

void print_LCD (char string[]);

void line2_LCD ();

void set_duty(int num, int percent);

void move (int mnum, int dir, int speed);

void delayms (uint16_t time) {

uint16_t counter;

int i;

for (i = 0; i < time; i++) {

counter = 0;

// loop for approximately 1ms

while (counter < 840) {

counter++;

}

counter = 0;

int dummydelay;

if (mon) {

// Make all pins inputs, disable pull-ups

DDRE = DDRE & ~(1 << 7) & ~(1 << 6) & ~(1 << 5) & ~(1 << 4);

PORTE = PORTE & ~(1 << 7) & ~(1 << 6) & ~(1 << 5) & ~(1 << 4);

// Generate a low output at pin 7

DDRE = DDRE | (1 << 7);

// Pull-up other inputs, low output at pin 7

PORTE = (PORTE | (1 << 4) | (1 << 5) | (1 << 6)) & ~(1 << 7);

for (dummydelay = 0; dummydelay < 5; dummydelay++){}

if ((PINE & (1 << 6)) == 0) {

// left bumper triggered

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

}

// Generate a low output at pin 5

DDRE = (DDRE | (1 << 5)) & ~(1 << 7);

// Pull-up other inputs, low output at pin 5

PORTE = (PORTE | (1 << 4) | (1 << 7) | (1 << 6)) & ~(1 << 5);

for (dummydelay = 0; dummydelay < 5; dummydelay++){}

if ((PINE & (1 << 4)) == 0) {

// right bumper triggered

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

}

// Make all pins inputs, disable pull-ups

DDRE = DDRE & ~(1 << 7) & ~(1 << 6) & ~(1 << 5) & ~(1 << 4);

PORTE = PORTE & ~(1 << 7) & ~(1 << 6) & ~(1 << 5) & ~(1 << 4);

}

}

}

void beep (uint16_t time, uint16_t freq) {

// check if no-beeps is set

if (eeprom_read_byte((const uint8_t*) NBP)) {

return;

}

// Set pin 7 as output

DDRA = DDRA | (1 << 7);

PORTA = PORTA & ~(1 << 7);

// Initialize duty cycle and time duration

duty = freq;

btime = time;

// Normal port operation, OC3A disconnected

TCCR3A = TCCR3A & ~(1 << 7) & ~(1 << 6);

// Wait time specified

OCR3A = TCNT3 + duty;

// Enable channel A interrupt

ETIMSK = ETIMSK | (1 << 4);

// Set timer to 1/8 prescaling

TCCR3B = TCCR3B | (1 << 1);

// block until finished beeping

while (btime > 0) {}

}

ISR (SIG_OUTPUT_COMPARE3A) {

OCR3A = TCNT3 + duty;

if ((PORTA & (1 << 7)) == 0) {

PORTA = PORTA | (1 << 7);

} else {

PORTA = PORTA & ~(1 << 7);

}

if (btime-- <= 0) {

// Stop timer

TCCR3B = TCCR3B & ~(1 << 1);

}

}

// Switch to Manual control

void manual () {

init_LCD();

sprintf(printbuf, "Manual Mode");

print_LCD(printbuf);

while (1) {

if (key == ' ') {

delayms(10);

continue;

}

sprintf(printbuf, "%c", key);

print_LCD(printbuf);

if (key == '1') {

init_LCD();

print_LCD("closing");

Bperc = Bperc - 5;

if (Bperc < 0)

Bperc = 0;

line2_LCD();

sprintf(printbuf, "P: %d", Bperc);

print_LCD(printbuf);

set_duty(2, Bperc);

} else if (key == '2') {

init_LCD();

print_LCD("opening");

Bperc = Bperc + 5;

if (Bperc > 100)

Bperc = 100;

line2_LCD();

sprintf(printbuf, "P: %d", Bperc);

print_LCD(printbuf);

set_duty(2, Bperc);

}

if (key == '5') {

init_LCD();

print_LCD("closing");

Bperc2 = Bperc2 - 5;

if (Bperc2 < 0)

Bperc2 = 0;

line2_LCD();

sprintf(printbuf, "P: %d", Bperc2);

print_LCD(printbuf);

set_duty(1, Bperc2);

} else if (key == '4') {

init_LCD();

print_LCD("opening");

Bperc2 = Bperc2 + 5;

if (Bperc2 > 100)

Bperc2 = 100;

line2_LCD();

sprintf(printbuf, "P: %d", Bperc2);

print_LCD(printbuf);

set_duty(1, Bperc2);

}

if (key == '7') {

init_LCD();

print_LCD("closing");

pSpeed = pSpeed - 40;

if (pSpeed < -40)

pSpeed = -40;

line2_LCD();

sprintf(printbuf, "P: %d", pSpeed);

print_LCD(printbuf);

if (pSpeed >= 0)

move(PULLEY, FORWARD, pSpeed);

else if (pSpeed < 0)

move(PULLEY, BACKWARD, -pSpeed);

} else if (key == '8') {

init_LCD();

print_LCD("opening");

pSpeed = pSpeed + 40;

if (pSpeed > 40)

pSpeed = 40;

line2_LCD();

sprintf(printbuf, "P: %d", pSpeed);

print_LCD(printbuf);

if (pSpeed >= 0)

move(PULLEY, FORWARD, pSpeed);

else if (pSpeed < 0)

move(PULLEY, BACKWARD, -pSpeed);

}

key = ' ';

}

}

void recal() {

key = ' ';

print_LCD("Recalibration Menu:");

line2_LCD();

char menu[20][20];

int item = 0;  // menu item number

int update = 1; // update menu items

int value = 0; // user entered value

int inspos = 0; // index position of input string

char ins[10]; // input string by user

int quit = 0; // quit recalibration

int clear; // index used for clearing ins

while(1) {

if (update) {

sprintf(menu[0], "(0)GlueOff: %d>", eeprom_read_byte((const uint8_t*) GLUEOFF));

sprintf(menu[1], "(1)Restore Defaults?");

sprintf(menu[2], "(2)Manual Control");

sprintf(menu[3], "(3)Fast Mode");

sprintf(menu[4], "(4)PulDwn T: %d>", eeprom_read_byte((const uint8_t*) PDT));

sprintf(menu[5], "(5)PulUp T: %d>", eeprom_read_byte((const uint8_t*) PUT));

sprintf(menu[6], "(6)PulDwn S: %d>", eeprom_read_byte((const uint8_t*) PDS));

sprintf(menu[7], "(7)PulUp S: %d>", eeprom_read_byte((const uint8_t*) PUS));

sprintf(menu[8], "(8)BckUpT: %d>", eeprom_read_byte((const uint8_t*) BUT));

sprintf(menu[9], "(9)No Beeps: %d>", eeprom_read_byte((const uint8_t*) NBP));

sprintf(menu[10], "(10)Set P: %d>", eeprom_read_byte((const uint8_t*) PAMT));

sprintf(menu[11], "(11)Set N: %d>", eeprom_read_byte((const uint8_t*) NAMT));

sprintf(menu[12], "(12)Set D: %d>", eeprom_read_byte((const uint8_t*) DAMT));

sprintf(menu[13], "(13)Set Q: %d>", eeprom_read_byte((const uint8_t*) QAMT));

line2_LCD();

print_LCD("                    ");

line2_LCD();

char pb[20];

sprintf(pb, "%s", menu[item]);

print_LCD(pb);

key = ' ';

update = 0;

inspos = 0;

value = 0;

for (clear = 0; clear < 10; clear++) {

ins[clear] = ' ';

}

}

switch (key) {

// scroll up

case 'A':	item--;

if (item < 0) {

item = 0;

}

update = 1;

break;

// scroll down

case 'B': 	item++;

if (item > 13) {

item = 13;

}

update = 1;

break;

// escape

case '*':	quit = 1;

break;

// enter

case '#':	value = atoi(ins);

update = 1;

switch (item) {

case 0:	eeprom_write_byte((uint8_t*) GLUEOFF, value);

break;

case 1: eeprom_write_byte((uint8_t*) GLUEOFF, 10);

eeprom_write_byte((uint8_t*) PDT, 25);							eeprom_write_byte((uint8_t*) PUT, 34);							eeprom_write_byte((uint8_t*) PDS, 35);							eeprom_write_byte((uint8_t*) PUS, 80);							eeprom_write_byte((uint8_t*) BUT, 22);							eeprom_write_byte((uint8_t*) NBP, 0);							eeprom_write_byte((uint8_t*) PAMT, 0)	;							eeprom_write_byte((uint8_t*) NAMT, 0)	;							eeprom_write_byte((uint8_t*) DAMT, 0);							eeprom_write_byte((uint8_t*) QAMT, 0);

line2_LCD();

print_LCD("                    ");

line2_LCD();

print_LCD("Defaults Restored");

break;

case 2: key = ' ';

manual();

break;

case 3: key = ' ';

fast = 1;

quit = 1;

break;

case 4: eeprom_write_byte((uint8_t*) PDT, value);

break;

case 5: eeprom_write_byte((uint8_t*) PUT, value);

break;

case 6: eeprom_write_byte((uint8_t*) PDS, value);

break;

case 7: eeprom_write_byte((uint8_t*) PUS, value);

break;

case 8: eeprom_write_byte((uint8_t*) BUT, value);

break;

case 9: eeprom_write_byte((uint8_t*) NBP, value);

break;

case 10: eeprom_write_byte((uint8_t*) PAMT, value);

break;

case 11: eeprom_write_byte((uint8_t*) NAMT, value);

break;

case 12: eeprom_write_byte((uint8_t*) DAMT, value);

break;

case 13: eeprom_write_byte((uint8_t*) QAMT, value);

break;

default:

break;

}

break;

// everything else (ie. numbers)

case '0':

case '1':

case '2':

case '3':

case '4':

case '5':

case '6':

case '7':

case '8':

case '9':	sprintf(printbuf, "%c", key);

print_LCD(printbuf);

ins[inspos++] = key;

if (inspos > 8) {

quit = 1;

}

key = ' ';

break;

case 'C':

case 'D': 	key = ' ';

break;

default:

break;

}

if (quit) {

break;

}

}

}

void start_timeout() {

time30ms = 0;

begintime = 1;

}

int check_timeout() {

if (time30ms >= TOOLONG) {

begintime = 0;

return 0;

} else {

return 1;

}

}

#endif /* _misc_H_ */

LCD.h

#ifndef __LCD_H_

#define __LCD_H_

#include "misc.h"

#define CMD 0

#define DAT 1

void send_LCD(uint8_t data, int destination) {

uint8_t nibble;

switch (destination) {

case CMD: nibble = 0x10;

nibble = nibble | (data >> 4);

PORTA = nibble;

delayms(1);

nibble = nibble & 0xEF;

PORTA = nibble;

delayms(5);

nibble = 0x10;

nibble = nibble | (data & 0x0F);

PORTA = nibble;

delayms(1);

nibble = nibble & 0xEF;

PORTA = nibble;

delayms(5);

break;

case DAT: nibble = 0x50;

nibble = nibble | (data >> 4);

PORTA = nibble;

delayms(1);

nibble = nibble & 0xEF;

PORTA = nibble;

delayms(1);

nibble = 0x50;

nibble = nibble | (data & 0x0F);

PORTA = nibble;

delayms(1);

nibble = nibble & 0xEF;

PORTA = nibble;

delayms(1);

break;

}

}

void init_LCD(void) {

// Set pins 0-6 on PORTA as output pins

DDRA = 0x7F;

PORTA = 0;

// Allow LCD to power on

delayms(15);

// Send $33 as nibbles to LCD command register, high-part first

send_LCD(0x33, CMD);

// Send $32 as nibbles to LCD command register, high-part first

send_LCD(0x32, CMD);

// Send $28 as nibbles to LCD command register, high-part first

send_LCD(0x28, CMD);

// Display on, Cursor and Cursor Blink off

send_LCD(0x0C, CMD);

// Send $01 as nibbles to LCD command register, high-part first

send_LCD(0x01, CMD);

}

void line2_LCD () {

// Send $C0 as nibbles to LCD command register, high-part first

send_LCD(0xC0, CMD);

}

void line1_LCD () {

// Send $02 as nibbles to LCD command register, high-part first

send_LCD(0x02, CMD);

}

void print_LCD (char string[]) {

// ASCII string as nibbles to LCD data register, high-part first

int i = 0;

while (string[i] != '\0') {

send_LCD(string[i], DAT);

i++;

}

}

#endif /* _LCD_H_ */

ADC.h

#ifndef __ADC_H_

#define __ADC_H_

#include "misc.h"

uint16_t read_ADC (int channel)  {

uint16_t result = 0;

// Use internal 2.56V reference and select channel

ADMUX = (1 << 7) | (1 << 6) | channel;

// Enable ADC and start conversion

ADCSRA = (1 << 7) | (1 << 6);

// Wait for conversion

while ((ADCSRA & (1 << 6)) != 0) {}

// Get result

result = ADC;

// Turn off ADC

ADCSRA = 0;

return result;

}

uint16_t avg5_ADC (int channel) {

uint16_t results[5];

uint16_t result = -1;

int i = 0;

int j = 0;

int score[5];

// empty score[] and get 5 values in results[]

for (i = 0; i < 5; i++) {

results[i] = read_ADC(channel);

score[i] = 0;

}

// compare results[] and tally up score[]

for (i = 0; i < 5; i++) {

for (j = 0; j < 5; j++) {

if (results [i] == results[j]) {

score[i] = score[i] + 1;

}

}

}

// return result with highest score or last sample if there is none

result = results[0];

for (i = 1; i < 5; i++) {

if (score[i] >= 3) {

result = results[i];

break;

}

else if (score[i]  == 2) {

result = results[i];

}

else if (i == 4 && result < 0) {

result = results[4];

}

}

return result;

}

#endif /* _ADC_H_ */

PWM.h

#ifndef __PWM_H_

#define __PWM_H_

#include "misc.h"

// duty cycles for all 3 PWMs

uint16_t up_duty[3];

uint16_t down_duty[3];

// Triple key delay from 30ms to 90ms

int delay;

// Smooth out servo movements

int newup;

int oldup;

// Call init_PWM last

void init_PWM () {

// Set PWM to 1/8 prescaling

TCCR1B = (1 << 1);

}

// Set PWM duty cycle based on 'percent' (0 < percent < 100)

void set_duty(int num, int percent) {

// offset from reglue

if (num == 1) {

percent = percent + eeprom_read_byte((const uint8_t*) GLUEOFF);

}

if (percent < 0 || percent > 100) {

return;

}

oldup = up_duty[num];

newup = 1000 + (percent * 40);

while (up_duty[num] != newup) {

cli();

if (newup < oldup) {

up_duty[num] = up_duty[num] - 1;

down_duty[num] = 40000 - up_duty[num];

} else {

up_duty[num] = up_duty[num] + 1;

down_duty[num] = 40000 - up_duty[num];

}

sei();

delayms(2);

}

cli();

up_duty[num] = 1000 + (percent * 40);

down_duty[num] = 40000 - up_duty[num];

sei();

}

void init_PWMA () {

// Initialize duty cycle

up_duty[0] = 20000; // not needed but kept anyway

down_duty[0] = 60000; // about 30ms

// Normal port operation, OCnA disconnected

TCCR1A = TCCR1A & ~(1 << 7) & ~(1 << 6);

// Wait 1.5ms

OCR1A = TCNT1 + 10;

// Enable channel A interrupt

TIMSK = TIMSK | (1 << 4);

// Set initial value for delay

delay = 0;

// initialize key 'buffer'

key = ' ';

}

void init_PWMB () {

// Initialize duty cycle

up_duty[1] = 3000;

down_duty[1] = 37000;

// Clear OC1B on compare match

TCCR1A = (TCCR1A | (1 << 5)) & ~(1 << 4);

// Wait 1.5ms

OCR1B = TCNT1 + 10;

// Set data direction of OC1B to out (PB 6)

DDRB = DDRB | (1 << 6);

// Enable channel B interrupt

TIMSK = TIMSK | (1 << 3);

}

void init_PWMC () {

// Initialize duty cycle

up_duty[2] = 3000;

down_duty[2] = 37000;

// Clear OC1B on compare match

TCCR1A = (TCCR1A | (1 << 3)) & ~(1 << 2);

// Wait 1.5ms

OCR1C = TCNT1 + 10;

// Set data direction of OC1B to out (PB 7)

DDRB = DDRB | (1 << 7);

// Enable channel C interrupt

ETIMSK = TIMSK | (1 << 0);

}

// Read key input only if key = ' '

ISR (SIG_OUTPUT_COMPARE1A) {

if (begintime) {

time30ms++;

}

int dummydelay;

//delayus(1);

for (dummydelay = 0; dummydelay < 5; dummydelay++){}

if (delay != 2) {

// Wait an amount down_duty[0]

OCR1A = TCNT1 + down_duty[0];

delay++;

return;

}

// Don't bother reading if key buffer is non-empty (ie key != ' ')

if (key != ' ') {

// Make all pins inputs, disable pull-ups

DDRC = 0;

PORTC = 0;

// Wait an amount down_duty[0]

OCR1A = TCNT1 + down_duty[0];

delay = 0;

return;

}

// Generate a low output at row 1

DDRC = (1 << 3);

// Pull-up the column inputs, low output at row 1

PORTC = ~(1 << 3);

//delayms(1);

//delayus(1);

for (dummydelay = 0; dummydelay < 5; dummydelay++){}

if ((PINC & (1 << 7)) == 0) {

key = '1';

} else if ((PINC & (1 << 6)) == 0) {

key = '2';

} else if ((PINC & (1 << 5)) == 0) {

key = '3';

} else if ((PINC & (1 << 4)) == 0) {

key = 'A';

}

// Generate a low output at row 2

DDRC = (1 << 2);

// Pull-up the column inputs, low output at row 2

PORTC = ~(1 << 2);

//delayms(1);

//delayus(1);

for (dummydelay = 0; dummydelay < 5; dummydelay++){}

if ((PINC & (1 << 7)) == 0) {

key = '4';

} else if ((PINC & (1 << 6)) == 0) {

key = '5';

} else if ((PINC & (1 << 5)) == 0) {

key = '6';

} else if ((PINC & (1 << 4)) == 0) {

key = 'B';

}

// Generate a low output at row 3

DDRC = (1 << 1);

// Pull-up the column inputs, low output at row 3

PORTC = ~(1 << 1);

//delayms(1);

//delayus(1);

for (dummydelay = 0; dummydelay < 5; dummydelay++){}

if ((PINC & (1 << 7)) == 0) {

key = '7';

} else if ((PINC & (1 << 6)) == 0) {

key = '8';

} else if ((PINC & (1 << 5)) == 0) {

key = '9';

} else if ((PINC & (1 << 4)) == 0) {

key = 'C';

}

// Generate a low output at row 4

DDRC = (1 << 0);

// Pull-up the column inputs, low output at row 4

PORTC = ~(1 << 0);

//delayms(1);

//delayus(1);

for (dummydelay = 0; dummydelay < 5; dummydelay++){}

if ((PINC & (1 << 7)) == 0) {

key = '*';

} else if ((PINC & (1 << 6)) == 0) {

key = '0';

} else if ((PINC & (1 << 5)) == 0) {

key = '#';

} else if ((PINC & (1 << 4)) == 0) {

key = 'D';

}

// Make all pins inputs, disable pull-ups

DDRC = 0;

PORTC = 0;

// Wait an amount down_duty[0]

OCR1A = TCNT1 + down_duty[0];

delay = 0;

}

ISR (SIG_OUTPUT_COMPARE1B) {

// If OC1B was set then clear, otherwise set

if ((TCCR1A & ((1 << 5) | (1 << 4))) == ((1 << 5) | (1 << 4))) {

// Clear OC1B on compare match

TCCR1A = (TCCR1A | (1 << 5)) & ~(1 << 4);

// Wait an amount up_duty[1]

OCR1B = TCNT1 + up_duty[1];

} else {

// Set OC1B on compare match

TCCR1A = TCCR1A | (1 << 5) | (1 << 4);

// Wait an amount down_duty[1]

OCR1B = TCNT1 + down_duty[1];

}

}

ISR (SIG_OUTPUT_COMPARE1C) {

// If OC1C was set then clear, otherwise set

if ((TCCR1A & ((1 << 3) | (1 << 2))) == ((1 << 3) | (1 << 2))) {

// Clear OC1C on compare match

TCCR1A = (TCCR1A | (1 << 3)) & ~(1 << 2);

// Wait an amount up_duty[1]

OCR1C = TCNT1 + up_duty[2];

} else {

// Set OC1C on compare match

TCCR1A = TCCR1A | (1 << 3) | (1 << 2);

// Wait an amount down_duty[1]

OCR1C = TCNT1 + down_duty[2];

}

}

#endif /* _PWM_H_ */

Motor.h

#ifndef __Motor_H_

#define __Motor_H_

#include "misc.h"

#define RIGHT 3

#define LEFT 2

#define FORWARD 0

#define BACKWARD 1

#define PULLEY 4

#define UP 1

#define DOWN 0

void init_mtrs(){

// Reset line for motor driver

DDRC = DDRC | (1 << 0);

PORTC = (1 << 0);

// wait for motor driver to boot

delayms (100);

// Set baud to 9600

UBRR1L = 103;

// Enable transmitter

UCSR1B = (1 << 3);

mon = 1;

}

// Use motor #2 for left, #3 for right, forward->0, back->1, 0 < speed < 127

void move (int mnum, int dir, int speed) {

if (speed > 127 || speed < 0) {

return;

}

// Send start byte

UDR1 = 0x80;

// Wait for transmission

while ((UCSR1A & (1 << 6)) == 0) {}

// clear transmit flag

UCSR1A = (1 << 6);

// Send device type

UDR1 = 0x00;

// Wait for transmission

while ((UCSR1A & (1 << 6)) == 0) {}

// clear transmit flag

UCSR1A = (1 << 6);

// Send direction and motor

UDR1 = (dir << 0) | (mnum << 1);

// Wait for transmission

while ((UCSR1A & (1 << 6)) == 0) {}

// clear transmit flag

UCSR1A = (1 << 6);

// Send speed

UDR1 = speed;

// Wait for transmission

while ((UCSR1A & (1 << 6)) == 0) {}

// clear transmit flag

UCSR1A = (1 << 6);

}

#endif /* _Motor_H_ */

CMUcam.h

#ifndef __CMUcam_H_

#define __CMUcam_H_

char recbuf[32]; // Receive buffer, must reset count = 0 before overflow

char sendbuf[32]; // Buffer used for sending commands

int count; // index of receive buffer

void init_UART0() {

// Enable receive interrupt, enable receiver, enable transmitter

UCSR0B = (1 << 7) | (1 << 4) | (1 << 3);

// Set baud rate to 115.2k

UBRR0L = 8;

}

void deinit_UART0() {

// Disable receive interrupt, receiver, and transmitter

UCSR0B = 0;

}

// sends commands to camera, guaranteed transmission, blocking

void sCam(char send[32]){

while (1) {

//beep(50, 1500);

count = 0;

int i = 0;

for(; send[i] != '\0'; i++) {

// Send character

UDR0 = send[i];

// Wait for transmission

while ((UCSR0A & (1 << 6)) == 0) {}

// clear transmit flag

UCSR0A = (1 << 6);

}

delayms(100);

if (recbuf[0] == 'A' && recbuf[1] == 'C' && recbuf[2] == 'K') {

recbuf[0] = 'E';

count = 0;

break;

}

}

}

ISR (SIG_UART0_RECV) {

if (count < 32) {

recbuf[count++] = (char) UDR0;

} else {

recbuf[31] = (char) UDR0;

}

}

int i;

int j;

int spaces;

int line = 0;

char temp[10];

int coinX = 0;

int coinY = 0;

int bx1, bx2, by1, by2; // bounding box coordinates

// Read CMUcam T-packet, stores coinX and coinY variables

// returns false if packet was bad, fills printbuf[] with results

// empties recbuf[] (by setting count = 0) for interrupt routine

int readT_CMU() {

// CMU cam readings

i = 0;

spaces = 0;

// Parse T-packet, return centroid coordinates

while (recbuf[i] != '\r' && i < 19) {

if (recbuf[i] == ' ') {

// Mark beginning index of X coordinate

if (spaces == 0) {

coinX = i + 1;

// Copy value into 'coinX' variable

} else if (spaces == 1) {

j = 0;

while (coinX != i) {

temp[j++] = recbuf[coinX++];

}

temp[j] = '\0';

coinX = atoi(temp);

// Mark beginning index of Y coordinate

coinY = i + 1;

// Copy value into 'coinY' variable

} else if (spaces == 2) {

j = 0;

while (coinY != i) {

temp[j++] = recbuf[coinY++];

}

temp[j] = '\0';

coinY = atoi(temp);

// Mark beginning index of x1 coordinate

bx1 = i + 1;

} else if (spaces == 3) {

j = 0;

while (bx1 != i) {

temp[j++] = recbuf[bx1++];

}

temp[j] = '\0';

bx1 = atoi(temp);

// Mark beginning index of y1 coordinate

by1 = i + 1;

} else if (spaces == 4) {

j = 0;

while (by1 != i) {

temp[j++] = recbuf[by1++];

}

temp[j] = '\0';

by1 = atoi(temp);

// Mark beginning index of x2 coordinate

bx2 = i + 1;

} else if (spaces == 5) {

j = 0;

while (bx2 != i) {

temp[j++] = recbuf[bx2++];

}

temp[j] = '\0';

bx2 = atoi(temp);

// Mark beginning index of y2 coordinate

by2 = i + 1;

} else if (spaces == 6) {

j = 0;

while (by2 != i) {

temp[j++] = recbuf[by2++];

}

temp[j] = '\0';

by2 = atoi(temp);

}

spaces++;

}

if (spaces == 7)

break;

printbuf[i] = recbuf[i];

i++;

}

printbuf[i] = '\0';

if (recbuf[1] != ' ') {

int c = 0;

//print_LCD(printbuf);

while (c < 32) {

recbuf[c++] = '*';

}

count = 0;

delayms(150);

//print_LCD("Bad Packet");

return 0;

}

// Print result or let me know if packet was bad

if (printbuf[0] != 'T') {

// Reset buffer index used by interrupt routine

// to prevent overflow

count = 0;

return 0;

}

count = 0;

return 1;

}

int mR = 0, mG = 0, mB = 0;

// Read CMUcam S-packet, stores mR, mG, and mB variables

// returns false if packet was bad, fills printbuf[] with results

// empties recbuf[] (by setting count = 0) for interrupt routine

int readS_CMU() {

// CMU cam readings

i = 0;

spaces = 0;

// Parse S-packet, return mean color values

while (recbuf[i] != '\r' && i < 19) {

if (recbuf[i] == ' ') {

// Mark beginning index of mR value

if (spaces == 0) {

mR = i + 1;

// Copy value into 'mR' variable

} else if (spaces == 1) {

j = 0;

while (mR != i) {

temp[j++] = recbuf[mR++];

}

temp[j] = '\0';

mR = atoi(temp);

// Mark beginning index of mG value

mG = i + 1;

// Copy value into 'mG' variable

} else if (spaces == 2) {

j = 0;

while (mG != i) {

temp[j++] = recbuf[mG++];

}

temp[j] = '\0';

mG = atoi(temp);

// Mark beginning index of mB value

mB = i + 1;

} else if (spaces == 3) {

j = 0;

while (mB != i) {

temp[j++] = recbuf[mB++];

}

temp[j] = '\0';

mB = atoi(temp);

}

spaces++;

}

if (spaces == 4)

break;

printbuf[i] = recbuf[i];

i++;

}

printbuf[i] = '\0';

if (recbuf[1] != ' ') {

int c = 0;

//print_LCD(printbuf);

while (c < 32) {

recbuf[c++] = '*';

}

count = 0;

delayms(150);

//print_LCD("Bad Packet");

return 0;

}

// Print result or let me know if packet was bad

if (printbuf[0] != 'S') {

// Reset buffer index used by interrupt routine

// to prevent overflow

count = 0;

return 0;

}

count = 0;

return 1;

}

#endif /* _CMUcam_H_ */

main.c

#include <avr/io.h>

#include <avr/interrupt.h>

#include <stdio.h>

#include <stdlib.h>

#include "misc.h"

#include "LCD.h"

#include "ADC.h"

#include "PWM.h"

#include "Motor.h"

#include "CMUcam.h"

int main (void)

{

// Check if defaults need to be restored after an erase

if (eeprom_read_byte((const uint8_t*) ERASED) != 0) {

eeprom_write_byte((uint8_t*) GLUEOFF, 10);

eeprom_write_byte((uint8_t*) PDT, 25);

eeprom_write_byte((uint8_t*) PUT, 34);

eeprom_write_byte((uint8_t*) PDS, 35);

eeprom_write_byte((uint8_t*) PUS, 80);

eeprom_write_byte((uint8_t*) BUT, 22);

eeprom_write_byte((uint8_t*) NBP, 0);

eeprom_write_byte((uint8_t*) PAMT, 0);

eeprom_write_byte((uint8_t*) NAMT, 0);

eeprom_write_byte((uint8_t*) DAMT, 0);

eeprom_write_byte((uint8_t*) QAMT, 0);

eeprom_write_byte((uint8_t*) ERASED, 0);

}

// Enable interrupts

sei();

init_PWMA();

init_PWMC();

init_PWMB();

init_PWM(); // Needed for randomly deciding turn direction

beep(200, 800);

delayms(50);

beep(200, 800);

__asm__ __volatile__ ("CP R24,R18\n\t"

"CP R24,R18\n\t");

// Turn on lights

DDRB = DDRB | (1 << 0);

PORTB = PORTB | (1 << 0);

//while(1) {}

// Enable light indicators for coins

DDRD = DDRD | (1 << 7) | (1 << 6) | (1 << 5) | (1 << 4);

PORTD = PORTD | (1 << 7) | (1 << 6) | (1 << 5) | (1 << 4);

uint16_t result = 0;

uint16_t result2 = 0;

init_LCD();

init_mtrs();

init_UART0();

set_duty(1, Bperc);

set_duty(2, Bperc2);

delayms(2000);

if (key != ' ') {

recal();

}

PORTD = PORTD & ~(1 << 7) & ~(1 << 6) & ~(1 << 5) & ~(1 << 4);

sprintf(sendbuf, "HD 1\r");

delayms(1000);

print_LCD(sendbuf);

line2_LCD();

print_LCD("Setting CMUcam...");

sCam(sendbuf);

sprintf(sendbuf, "LF\r");

delayms(1000);

line1_LCD();

print_LCD("                    ");

line1_LCD();

print_LCD(sendbuf);

sCam(sendbuf);

sprintf(sendbuf, "FD 60\r");

delayms(1000);

line1_LCD();

print_LCD("                    ");

line1_LCD();

print_LCD(sendbuf);

sCam(sendbuf);

beep(200, 400);

int readres;

int rotdrop = 30; // store drop point in this variable

int conf[5]; // must reach 2 before we're sure what the coin is

conf[QUARTER] = 0;

conf[NICKEL] = 0;

conf[PENNY] = 0;

conf[DIME] = 0;

int reres = 0;

while (1) {

// flash lights while looking for coins

if ((PORTD & ((1 << 7) | (1 << 6) | (1 << 5) | (1 << 4))) == 0) {

PORTD = (PORTD | (1 << 7) | (1 << 4)) & (~(1 << 6) & ~(1 << 5));

} else if ((PIND & ((1 << 7) | (1 << 4))) == ((1 << 7) | (1 << 4))){

PORTD = (PORTD | (1 << 6) | (1 << 5)) & (~(1 << 7) & ~(1 << 4));

} else {

PORTD = (PORTD | (1 << 7) | (1 << 4)) & (~(1 << 6) & ~(1 << 5));

}

//  CMUcam (pt1)

readres = readT_CMU();

// Clean up LCD screen

if (line == 0) {

line2_LCD();

print_LCD("                    ");

line2_LCD();

line++;

} else {

line1_LCD();

print_LCD("                    ");

line1_LCD();

line--;

}

if (readres == 0) {

beep(50, 1500);

print_LCD(printbuf);

} else {

print_LCD(printbuf);

}

// Collision Avoidance

result = avg5_ADC(0);

result2 = avg5_ADC(1);

//sprintf(buf, "%d   %d", result, result2);

//print_LCD(buf);

//print_LCD("     ");

//line2_LCD();

//continue;

//delayms(100);

// If we've found a coin track it, else move forward

// CMUcam (pt2)

if (coinX != 0 && coinY != 0) {

beep(200, 400);

init_LCD();

PORTD = PORTD & ~(1 << 7) & ~(1 << 6) & ~(1 << 5) & ~(1 << 4);

// is it the edge of the world?

if ((bx2 - bx1) >= 12) {

init_LCD();

print_LCD("Edge of the world");

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

delayms(RETRY);

delayms(RETRY);

delayms(RETRY);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

beep(50, 1500);

delayms(500);

int LSB = TCNT1 & (1 << 0);

if (LSB == 0) {

move (3, 1, 80);

move (2, 0, 80);

} else {

move (3, 0, 80);

move (2, 1, 80);

}

delayms(3000);

init_LCD();

move (3, 0, 50);

move (2, 0, 50);

mbdime = 0;

coin = NONE;

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

// reset confidence values

conf[QUARTER] = 0;

conf[NICKEL] = 0;

conf[PENNY] = 0;

conf[DIME] = 0;

continue;

}

print_LCD("Coin!");

// Stop moving

move (RIGHT, FORWARD, 0);

move (LEFT, FORWARD, 0);

delayms(1000);

// Start tracking

start_timeout();

while (coinX != 8 && check_timeout()) {

// Clean up LCD screen

line2_LCD();

print_LCD("                    ");

line2_LCD();

print_LCD(printbuf);

delayms(50);

while (!readT_CMU());

if (coinX == 0 && coinY == 0)

break;

if (coinX < 8) {

move (RIGHT, BACKWARD, 70);

move (LEFT, FORWARD, 70);

if (coinX >= 6) {

delayms(120);

} else {

delayms(240);

}

move (RIGHT, FORWARD, 0);

move (LEFT, FORWARD, 0);

} else if (coinX > 8) {

move (RIGHT, FORWARD, 70);

move (LEFT, BACKWARD, 70);

if (coinX <= 10) {

delayms(120);

} else {

delayms(240);

}

move (RIGHT, FORWARD, 0);

move (LEFT, FORWARD, 0);

}

}

while (coinY != 8 && check_timeout()) {

// Clean up LCD screen

line2_LCD();

print_LCD("                    ");

line2_LCD();

print_LCD(printbuf);

delayms(150);

while (!readT_CMU());

if (coinX == 0 && coinY == 0)

break;

if (coinY < 8) {

move (RIGHT, BACKWARD, 30);

move (LEFT, BACKWARD, 30);

delayms(50);

move (RIGHT, FORWARD, 0);

move (LEFT, FORWARD, 0);

} else if (coinY > 8) {

move (RIGHT, FORWARD, 30);

move (LEFT, FORWARD, 30);

delayms(50);

move (RIGHT, FORWARD, 0);

move (LEFT, FORWARD, 0);

}

}

// Stop moving

move (RIGHT, FORWARD, 0);

move (LEFT, FORWARD, 0);

if ((coinX == 0 && coinY == 0) || !check_timeout()) {

if (!check_timeout()) {

print_LCD("Timed out");

} else {

print_LCD("Lost coin");

}

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

mbdime = 0;

coin = NONE;

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

// reset confidence values

conf[QUARTER] = 0;

conf[NICKEL] = 0;

conf[PENNY] = 0;

conf[DIME] = 0;

// reset timeout

timeout = 0;

continue;

}

print_LCD("Centered");

line1_LCD();

print_LCD("                    ");

line1_LCD();

// print coin guess

// stop camera T-packet stream

sprintf(sendbuf, "\r");

sCam(sendbuf);

// set virtual window

sprintf(sendbuf, "VW 38 66 48 76\r");

sCam(sendbuf);

// get mean color

sprintf(sendbuf, "GM\r");

sCam(sendbuf);

while(!readS_CMU());

// stop camera S-packet stream

sprintf(sendbuf, "\r");

sCam(sendbuf);

// is it the edge of the world?

if ((bx2 - bx1) >= 12) {

init_LCD();

print_LCD("Edge of the world");

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

delayms(RETRY);

delayms(RETRY);

delayms(RETRY);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

beep(50, 1500);

delayms(500);

int LSB = TCNT1 & (1 << 0);

if (LSB == 0) {

move (3, 1, 80);

move (2, 0, 80);

} else {

move (3, 0, 80);

move (2, 1, 80);

}

delayms(3000);

init_LCD();

move (3, 0, 50);

move (2, 0, 50);

mbdime = 0;

coin = NONE;

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

// reset confidence values

conf[QUARTER] = 0;

conf[NICKEL] = 0;

conf[PENNY] = 0;

conf[DIME] = 0;

continue;

// is it in penny range

} else if (mR <= 60 && mG <= 55 && mB <= 51 && !((by2 - by1) >= 8)) {

rotdrop = 18;

print_LCD("penny");

PORTD = (PORTD | (1 << 4)) & ~(1 << 7) & ~(1 << 6) & ~(1 << 5);

conf[PENNY] = conf[PENNY] + 1;

if (conf[PENNY] < 2 && !fast) {

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

mbdime = 0;

coin = NONE;

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "LF\r");

sCam(sendbuf);

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

continue;

}

coin = PENNY;

eeprom_write_byte((uint8_t*) PAMT, eeprom_read_byte((const uint8_t*) PAMT) + 1);

} else {

// set qaurter-virtual window

sprintf(sendbuf, "VW 28 60 40 91\r");

sCam(sendbuf);

// get mean color

sprintf(sendbuf, "GM\r");

sCam(sendbuf);

while(!readS_CMU());

// stop camera S-packet stream

sprintf(sendbuf, "\r");

sCam(sendbuf);

// Special case shared by nickel/dime, retry detection

if (((bx2 - bx1) + (by2 - by1)) == 10) {

mbdime++;

if (mbdime >= 3) {

rotdrop = 8;

PORTD = (PORTD | (1 << 6)) & ~(1 << 7) & ~(1 << 5) & ~(1 << 4);

print_LCD("dime");

conf[DIME] = conf[DIME] + 1;

if (conf[DIME] < 2 && !fast) {

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

mbdime = 0;

coin = NONE;

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "LF\r");

sCam(sendbuf);

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

continue;

}

coin = DIME;

eeprom_write_byte((uint8_t*) DAMT, eeprom_read_byte((const uint8_t*) DAMT) + 1);

}

/*size10++;

print_LCD("no idea, size 10");

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

// leave the coin if there are too many 'size10' cases

if (size10 >= 4) {

int LSB = TCNT1 & (1 << 0);

if (LSB == 0) {

move (3, 1, 80);

move (2, 0, 80);

} else {

move (3, 0, 80);

move (2, 1, 80);

}

delayms(2000);

}*/

if (coin != DIME) {

move (RIGHT, BACKWARD, 30);

move (LEFT, BACKWARD, 30);

delayms(200);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

delayms(10);

coin = NONE;

continue;

}

// is it small?

} else 	if (((bx2 - bx1) + (by2 - by1)) < 10) {

rotdrop = 8;

PORTD = (PORTD | (1 << 6)) & ~(1 << 7) & ~(1 << 5) & ~(1 << 4);

print_LCD("dime");

conf[DIME] = conf[DIME] + 1;

if (conf[DIME] < 2 && !fast) {

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

mbdime = 0;

coin = NONE;

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "LF\r");

sCam(sendbuf);

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

continue;

}

coin = DIME;

eeprom_write_byte((uint8_t*) DAMT, eeprom_read_byte((const uint8_t*) DAMT) + 1);

// is it in quarter range?

} else if ((((bx2 - bx1) + (by2 - by1)) == 13) || ((by2 - by1) >= 8)) {

rotdrop = 40;

PORTD = (PORTD | (1 << 7)) & ~(1 << 6) & ~(1 << 5) & ~(1 << 4);

print_LCD("quarter");

conf[QUARTER] = conf[QUARTER] + 1;

if (conf[QUARTER] < 2 && !fast) {

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

mbdime = 0;

coin = NONE;

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "LF\r");

sCam(sendbuf);

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

continue;

}

coin = QUARTER;

eeprom_write_byte((uint8_t*) QAMT, eeprom_read_byte((const uint8_t*) QAMT) + 1);

// is it in shiny penny range?

} else if ((mR > 54) &&  ((bx2 - bx1) == 5) && ((by2 - by1) == 6)) {

rotdrop = 18;

PORTD = (PORTD | (1 << 4)) & ~(1 << 7) & ~(1 << 6) & ~(1 << 5);

print_LCD("shiny penny");

conf[PENNY] = conf[PENNY] + 1;

if (conf[PENNY] < 2 && !fast) {

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

mbdime = 0;

coin = NONE;

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "LF\r");

sCam(sendbuf);

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

continue;

}

coin = PENNY;

eeprom_write_byte((uint8_t*) PAMT, eeprom_read_byte((const uint8_t*) PAMT) + 1);

// must be a nickel

} else  if (((by2 - by1) == 6) || ((by2 - by1) == 7) ) {

rotdrop = 25;

PORTD = (PORTD | (1 << 5)) & ~(1 << 7) & ~(1 << 6) & ~(1 << 4);

print_LCD("nickel");

conf[NICKEL] = conf[NICKEL] + 1;

if (conf[NICKEL] < 2 && !fast) {

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

mbdime = 0;

coin = NONE;

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "LF\r");

sCam(sendbuf);

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

continue;

}

coin = NICKEL;

eeprom_write_byte((uint8_t*) NAMT, eeprom_read_byte((const uint8_t*) NAMT) + 1);

// no idea

} else {

print_LCD("no idea, no cases met");

move (RIGHT, BACKWARD, 50);

move (LEFT, BACKWARD, 50);

delayms(RETRY);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

mbdime = 0;

coin = NONE;

continue;

}

}

line2_LCD();

print_LCD("                    ");

line2_LCD();

sprintf(printbuf, "%d %d %d bsz:%d %d", mR, mG, mB, (bx2 - bx1), (by2 - by1));

print_LCD(printbuf);

//while (1) {}

// reset confidence values

conf[QUARTER] = 0;

conf[NICKEL] = 0;

conf[PENNY] = 0;

conf[DIME] = 0;

//size10 = 0;

mbdime = 0;

// Move back a bit to give grasper room and not damage camera

move (RIGHT, BACKWARD, 20);

move (LEFT, BACKWARD, 20);

delayms(eeprom_read_byte((const uint8_t*) BUT) * 100);

move (RIGHT, BACKWARD, 0);

move (LEFT, BACKWARD, 0);

// Automatic pick-up

// Rotate into pick-up position

int rot = 50;

int rotstop = 75;

for (;rot < rotstop; rot += 5) {

init_LCD();

sprintf(printbuf, "P: %d", rot);

print_LCD(printbuf);

set_duty(1, rot);

delayms(100);

}

init_LCD();

sprintf(printbuf, "P: %d", rot);

print_LCD(printbuf);

set_duty(1, rot);

// Drop grasper

uint16_t droptime = eeprom_read_byte((const uint8_t*) PDT) * 100;

move(PULLEY, DOWN, 35);

delayms(droptime);

move(PULLEY, DOWN, 0);

// Close grasper

int grasp = 50;

int gstop = 70;

for (;grasp < gstop; grasp += 5) {

init_LCD();

sprintf(printbuf, "P: %d", grasp);

print_LCD(printbuf);

set_duty(2, grasp);

delayms(100);

}

init_LCD();

sprintf(printbuf, "P: %d", grasp);

print_LCD(printbuf);

if (coin != QUARTER) {

set_duty(2, 73); // grasp = 70 but we want a tighter grip

} else {

set_duty(2, 70);

}

// Pull up

uint16_t pulltime = eeprom_read_byte((const uint8_t*) PUT) * 100;

move(PULLEY, UP, 80);

delayms(pulltime);

move(PULLEY, UP, 0);

// Rotate into drop position

for (;rot > rotdrop; rot -= 5) {

init_LCD();

sprintf(printbuf, "P: %d", rot);

print_LCD(printbuf);

set_duty(1, rot);

delayms(100);

}

init_LCD();

sprintf(printbuf, "P: %d", rot);

print_LCD(printbuf);

set_duty(1, rot);

// Drop coin

gstop = 50;

for (;grasp > gstop; grasp -= 5) {

init_LCD();

sprintf(printbuf, "P: %d", grasp);

print_LCD(printbuf);

set_duty(2, grasp);

delayms(100);

}

init_LCD();

sprintf(printbuf, "P: %d", grasp);

print_LCD(printbuf);

set_duty(2, grasp);

coin = NONE;

// Return arm to start position

rotstop = 50;

for (;rot < rotstop; rot += 5) {

init_LCD();

sprintf(printbuf, "P: %d", rot);

print_LCD(printbuf);

set_duty(1, rot);

delayms(100);

}

init_LCD();

sprintf(printbuf, "P: %d", rot);

print_LCD(printbuf);

set_duty(1, rot);

// return grasper to a good start position

set_duty(2, 40);

// Switch to Manual control

//manual();

coinX = 0;

coinY = 0;

// Reset virtual window

sprintf(sendbuf, "VW\r");

sCam(sendbuf);

// Look for coins again

sprintf(sendbuf, "FD 60\r");

sCam(sendbuf);

coin = NONE;

} else {

// Allow for an easy 'stop switch'

if (key != ' ') {

move (3, 1, 0);

move (2, 1, 0);

beep(50, 1500);

init_LCD();

print_LCD("Total:");

uint16_t sum;

int dol, cen;

sum = eeprom_read_byte((const uint8_t*) QAMT) * 25

+ eeprom_read_byte((const uint8_t*) DAMT) * 10

+ eeprom_read_byte((const uint8_t*) NAMT) * 5

+ eeprom_read_byte((const uint8_t*) PAMT);

line2_LCD();

if (sum > 100) {

dol = sum / 100;

cen = sum % 100;

} else {

dol = 0;

cen = sum;

}

if (cen < 10) {

sprintf(printbuf, "$%d.0%d", dol, cen);

} else {

sprintf(printbuf, "$%d.%d", dol, cen);

}

print_LCD(printbuf);

PORTD = PORTD | ((1 << 7) | (1 << 6) | (1 << 5) | (1 << 4));

delayms(2000);

PORTD = (PORTD | (1 << 6) | (1 << 5) | (1 << 4)) & ~(1 << 7);

delayms(2000);

PORTD = (PORTD | (1 << 5) | (1 << 4)) & ~(1 << 7) & ~(1 << 6);

delayms(2000);

PORTD = (PORTD | (1 << 4)) & ~(1 << 7) & ~(1 << 6) & ~(1 << 5);

delayms(2000);

PORTD = PORTD & ~(1 << 7) & ~(1 << 6) & ~(1 << 5) & ~(1 << 4);

delayms(2000);

key = ' ';

}

// search for coins while avoiding collisions

move (RIGHT, 0, 50);

move (LEFT, 0, 50);

if ((result > 900 || result2 > 900) && reres) {

move (3, 1, 0);

move (2, 1, 0);

beep(50, 1500);

delayms(500);

int LSB = TCNT1 & (1 << 0);

if (LSB == 0) {

move (3, 1, 80);

move (2, 0, 80);

} else {

move (3, 0, 80);

move (2, 1, 80);

}

delayms(3000);

init_LCD();

print_LCD("Obstacle");

}

move (3, 0, 50);

move (2, 0, 50);

if (!reres) {

reres = 1;

}

}

}

return 1;

}