Autonomous Mapping With

Laser Parallax and Digital Compass

Thomas P. Feeney

Robot: Sir Vide

Course: EEL5666 - Intelligent Machine Design Lab

Instructors: Dr. Antonio A. Arroyo and Dr. Eric M. Schwartz

Teaching Assistants: Kevin Claycomb and Adam Barnett

Table of Contents

1.	Abstract................................................................................................................	3

2.	Executive Summary.............................................................................................	4

3.	Introduction..........................................................................................................	5

4.	Integrated System.................................................................................................	6

4.1	Organization................................................................................................	6

4.2	Operation.....................................................................................................	6

4.3 	Mapping......................................................................................................	6

5.	Mobile Platform...................................................................................................	6

5.1	Underside....................................................................................................	6

5.2	Topside........................................................................................................	7

6.	Actuation..............................................................................................................	7

6.1	Motors.........................................................................................................	7

6.2	Motor Controllers........................................................................................	7

6.3	Theory of Operation....................................................................................	8

7.	Sensors.................................................................................................................	8

7.1	3-Axis Compass..........................................................................................	8

7.2	Camera........................................................................................................	8

7.3	Motor Driver Current Sensors....................................................................	8

7.4	Sensor Integration.......................................................................................	8

8.	Behaviors.............................................................................................................	8

8.1	Go Straight..................................................................................................	9

8.1	Turn.............................................................................................................	9

9.	Conclusion...........................................................................................................	9

1. Abstract

Sir Vide was intended to be an autonomous mapping robot.  Instead, he was an exercise in sensor integration that did not get completed.

The problem he would have solved would be the expansive unknown indoor environment.  He could drive around a room and map it out, providing information to rescue personnel, intelligence agencies, the disabled, and building managers.  He would be able to map the floor space of a building with laser-accuracy.

By fusing together compass data, wheel information, and camera feedback, he would have been able to build a map of where he has been and what obstacles in all of that space he faced.

2. Executive Summery

Sir Vide was intended to be an autonomous mapping robot.  Instead, he was an exercise in sensor integration that did not get completed.

The problem he would have solved would be the expansive unknown indoor environment.  He could drive around a room and map it out, providing information to rescue personnel, intelligence agencies, the disabled, and building managers.  He would be able to map the floor space of a building with laser-accuracy.

By fusing together compass data, wheel information, and camera feedback, he would have been able to build a map of where he has been and what obstacles in all of that space he faced.

3. Introduction

Sir Vide was intended to be an autonomous mapping robot.  Instead, he was an exercise in sensor integration that did not get completed.

The problem he would have solved would be the expansive unknown indoor environment.  He could drive around a room and map it out, providing information to rescue personnel, intelligence agencies, the disabled, and building managers.  He would be able to map the floor space of a building with laser-accuracy.

By fusing together compass data, wheel information, and camera feedback, he would have been able to build a map of where he has been and what obstacles in all of that space he faced.

4. Integrated System

The integrated system is the highest functional view of the robot.  As a specific physical instance in its environment, the quality of decision-making determines how autonomous a robot can be.  In this case, by fusing a compass and a camera into a powerful desktop processor, Sir Vide’s intelligence is only limited by the robustness of its programming.

4.1 Organization

The highest level object is a Master program written in Microsoft Visual C#.  This program transacts and utilizes information coming and going from multiple sensors.  It sends and receives serial data over Bluetooth in a custom packet to the Atmel microcontroller.  The Master also retrieves the images from the wireless network camera, and sends them to be processed by a custom-compiled OpenCV code library on the same computer.  The Master contains the arbiter and all state information, imparting its will to the wheels over the Bluetooth, through the microcontroller, regulation by the motor drivers, and finally to the motors.

4.2 Operation

The information received contains the current heading, pitch, roll, PWM of each motor driver, both wheel direction signals, both wheel brake signals, and the current sourced by both motor drivers.  The line information returned from OpenCV is merged with the adjusted dead reckoned position to produce a 2-dimensional map.  The most intelligence is required in interpolating the position of objects and walls in between samples, where the robot is constantly moving.

4.3 Mapping

Mapping was not implemented because I was not able to get the robot to go straight.  Had I been able to, I would have shown the point to point movement of the robot, and the associated obstacle information.  Receiving compass data at 16Hz and knowing the speed, from a known PWM and known motor current, would have resulted in a continuous path.  Then integrating the information from individual images, certain points would have associated obstacle information that could be overlaid.

5. Mobile Platform

The mobile platform was entirely built specifically for this project.  The base of the platform is an 8” x 8” x 3/8” polycarbonate octagon.  The items below it are static and hardmounted, while the items above have the freedom of placement via bolting or adhesive.  Everything that is bolted is, as often as possible, buffered by neoprene rubber washers to absorb vibration that can be transferred to the imaging system or compass.

5.1 Underside

The underside consists of the laser beam, laser beam splitter, laser beam mount, balancing ball castors, a motor mount, the 2 motors, and the 2 wheel hubs and wheels.

The motor mount is 1/8” aluminum bent into a U-shape that holds both motors back to back, within several millimeters of each other.  The wheels consist of 3 parts: a tire, a wheel, and a hub, all custom made.  The hubs are aluminum that were turned in a lathe, with the tight requirements of a set screw for tightening to the axel, and 4 holes at 90° to each other that align to the wheel.  The wheels were made from Delrin and screw on countersunk to the hub.  The tires were just large diameter rubber tubing from a set of wheels that were purchased but not used.  They were chosen for their apparent traction.  The motors will be covered in more detail in section 6.2.

The laser beam is a 635nm red laser diode with built-in driver circuit.  At 5V, the laser consumes about 40mA.  The beam splitter is a cap that fits over the laser diode with a special lens glued in.  This optic turns the single beam into a horizontal laser line.  A special mount was milled to mount both to the underside and allow for adjustment in all directions.

5.2 Topside

The only requirements of the topside mounting were to have the camera directly above the laser line, and to have the compass as high as possible.  The following components were placed topside: 7.4V LiPo battery, 18.5V LiPo battery, camera mainboard, camera and housing, Bluetooth module, 3-axis compass, Atmel board, 2 motor driver boards, 5V and 12V regulators.  The Bluetooth was wedged in up high in the camera mount for the best wireless range.  The 2 batteries and power regulators were mounted on the bottom level, where their weight would be best distributed.

6. Actuation

The only actuated parts on Sir Vide are the two wheels.  This made for a simpler mechanical complexity and simpler maintenance of software state.

6.1 Motors

The 2 motors are F 2140 Maxon Motors with a spur gearhead on one side and optical encoder on the other.  While the nominal voltage of the motors is 24.0V, they were run off an 18.5V lithium polymer battery that would vary in voltage from 21.0V to 17.0V.  This led to a nominal speed ranging from 1841rpm to 1149rpm.  The 30:1 spur gearhead would reduce this further to 60rpm to 39rpm.  The wheel encoders had a resolution of 100 counts per turn, with 2 channels for detecting direction, yielding highly accurate wheel position information.

6.2 Motor Controllers

The motor controllers were based on National LMD18200 motor drivers that I built a board around.  They had input signals of direction, brake, PWM (pulse-width modulation duty cycle), and would output signals of thermal warning and sourcing current sensing.  The outputs to the motors were an H-bridge at a maximum 3A at 55V.  In my case, the drivers didn’t even get warm at 120mA at 20V.  The direction and brake pins are currently pulled to ground, disabling reverse and brake.  The PWM inputs are directly from the Atmel output compare timers, running at 64kHz, with a varying duty cycle.  The thermal flag was not implemented, as overheating does not appear to be a problem.

6.3 Theory of Operation

The PWM of each wheel is set by the master program upon determining how much the heading needs to be changed.  The control loop does not work, resulting in an oscillatory unstable system, but the PWM is dynamically set by the C# program at 16Hz.

7. Sensors

The sensors in Sir Vide were a digital video camera, 3-axis compass, and motor driver current sensors.

7.1 3-Axis Compass

The 3-axis compass is a PNI TCM2-20.  It is programmed to output a heading, pitch, and roll at a 16Hz.  The data sheet claims an accuracy of ±0.5° with a repeatability of ±0.1° on the heading after proper calibration.  I calibrated the compass to compensate for the static magnetic field from motors.

7.2 Camera

The camera is a D-Link DCS-3220G wireless network camera.  It outputs frames of 704x480 at 3 frames per second with 600ms delay, or at 352x240 at 15 frames per second with a 450ms delay.  It was found that the image quality is sharp but poor for image processing, with the images changing format several times until OpenCV processing.

7.3 Motor Driver Current Sensors

The current sensing output is achieved by the pin outputting 377µA per amp of motor output.  By connecting a precise resistor from it to ground, and measuring the voltage with the Atmel ADC, the current can be reliably determined.  For Sir Vide, these outputs were wired, but not implemented in software due to time constraints.

7.4 Sensor Integration

The most current sensed current and heading are kept on the Atmel and transmitted to the PC as soon as available.  The PC responds to the Atmel whether it should change the PWM of the wheels.  The PC also takes into consideration the image data from the camera, to determine whether there is an obstacle ahead that would require Sir Vide to turn away from.

8. Behaviors

The behaviors were originally intended to be much more complex, encompassing full search patterns and wall following.  Enough difficulty was found fusing the camera data to the state and even keeping Sir Vide on a straight path that the behaviors were reduced to going straight, turning, and performing a simple plow search pattern.

8.1 Go Straight

Going straight was the most difficult thing.  I have not taken a controls class, so I did not know how to implement PID control.  I implemented a change in the output PWMs by 2nd order polynomial, with the acceleration and velocity constants being able to be set.  To determine if any change in the PWM needed to be made, there was an angleThreshold that was able to be changed and set during runtime.

8.2 Turn

Turning was implemented in manual control by stopping one wheel and turning the other.  A better way would have been to put one wheel in forward and the other in reverse, but I did not implement the reverse pin in software.

9. Conclusion

As a realistic summary of the work accomplished, I did accomplish a lot in tying a robot to a high-level programming environment.  But I did not accomplish nearly any of my original goals.  I did bring all of the images reliably into C# with OpenCV, which is quite difficult, but did not reach the point of being able to export salient information to the arbiter.