# EEL 5666

Intelligent Machine Design Lab

# Final Report

Project “Transformer”

An Autonomous Robot

## University of Florida

Department of Computer and Electrical Engineering

### Su Va (Andy) Fong

8/03/98

### Content

Abstract…………………………………………………………………………………..3

Executive Summary………………………………………………………………………4

Introduction……………………………………………………………………………….5

Integrated System…………………………………………………………………………6

Mobile Platform…………………………………………………………………………..7

Walking mechanism…………………………………………………………..…..7

Walking pattern……………………………………………………………………8

Tracks……………………………………………………………………………..9

Actuation…………………………………………………………………………………10

Sensors…………………………………………………………………………………...11

Motion sensor........................................................................................................11

IR sensors..............................................................................................................12

Bump sensors........................................................................................................13

Touch/Position sensors..........................................................................................14

Behaviors...........................................................................................................................15

Programming Technique...................................................................................................16

Dynamic collision avoidance................................................................................16

Walking Algorithm...............................................................................................17

Dynamic sensor position.......................................................................................18

Watchdog program................................................................................................19

Conclusion.........................................................................................................................20

Appendix – Source code...................................................................................................21

Abstract:

Before taking this class, I always wanted to build a robot. I wanted to build something with legs. When I saw the Robobug, I was so fascinated and wished to build one too. Finally, I got the chance to take this class and to build a robot. I want to build something from scratch instead of buying some already made model and assembling it. However, I found out that building a Robobug from scratch is not something I can do in one semester, especially for someone like me who knows nothing about mechanics. However, I still want to build something with legs. I read through some mechanics books and finally found the ‘kneeless’ leg mechanism. It is relativlely easy to build and to control. After more thinking and struggling, I feel that a walking robot sometimes is not very practical and it consumes a lot of power. Also, I cannot do much with just a walking robot. So, I decided to combine a tank robot and a walking robot into one. The tradeoff is that I have to make the legs shorter in order to make the transformation possible. I also need two sets of sensors for the walking mode and the tank mode because they are walking or moving in different direction.

Executive Summary:

This is the newest member in my family. Sometimes, it is a tank robot. Sometimes, it is a six-leg crawling creature. It’s characteristic is it unpredictable (random) behavior. I cannot predict when it will transform from one mode to another. It will walk around and avoid obstacles. It also likes human. It tends to look for human to play with. When it sees a human right in front of it. It will jiggle its tail. This is my newest pet.

#### Introduction

Transformer is an autonomous agent which can transform between a tank robot and a walking robot. It acts like a pet and tends to look for a human to play with. Whenever it sees a human in front of it, it will jiggle its tail. I intended to make this robot act by itself, more like a real pet that I cannot predict what it will do.

In this report, I will talk about the integrated system, mobile platform, actuation, sensors, behavior and some programming technique I used on this robot.

Integrated System

The robot is controlled by 68HC11EVBU with ME11 which has 32k memory expansion, two motor driver using output compare 2 and 3, and a 40 kHz modulated output port for IR LEDs. The ME11 also makes the 68HC11 ready for I/O port expansion. I added three input ports and output ports for reading the sensors and controlled the legs.

In order to drive the six legs, I also built a small motor driver board which contains three motor driver chips and two 74’04 chips. The motor driver is a replicate of the DC motor driver in the ME11. I made three of them and put them together in a small board. For circuit diagram, please refer to the ME11 Assembly Manual fig 8 (http://www.mil.ufl.edu/novasoft).

#### Mobile Platform

This robot will be able to transform between a walking robot and a tank robot. The body is made of aluminum angle and pivoted together. I also use aluminum channel for the legs. I choose to use aluminum because it is strong and relatively light.

##### Walking Mechanism

The leg mechanism I use is very straight forward and is relatively easy to build and control. Fig 1 shows the mechanism of a single leg.

Point A and C are pivots. Point A is attached to a motor so that point B circles around point A. As point B turns, point C can move up and down. Therefore, point D can trace out an oval orbit which enable the robot to move back and forth. Fig 2 shows one side of the robot with the legs attached.

The middle leg must be 180 degree out of phase of the other two legs. The legs on one side must be 180 degree out of phase with the legs on the other side.

Walking pattern

The walking patterns are shown in fig 3. This walking pattern is very essential and I try to maintain this walking pattern after each move when walking forward or turning left or right. To turn left, just reserve the motor direction on the left right side. To turn right, just reserve the motor direction on the right side. Some programming is need to keep the legs synchronized which will be cover in the programming technique section.

The tracks

The tracks are cut out from a toy and mounted on the side of the robot. When it transforms to a tank robot, it retracts all the legs to its highest position and let the tracks touch the ground as shown in fig 4. Fig 5 shows the top view of the robot with the position of the tracks and the legs.

#### Actuation

This robot contains eight motors. Two for the tracks and Six for the legs. Because I need relatively high torque to move the legs and the motor must be able to turn 360 degree to move the leg forward, the motors are fully hacked from some used servo. All the electronics parts are taken out from the servo. The main gear is also modified so that 360 degree turn is possible. To get full power from the motors, the power is supplied directed from the 8-pack battery pack.

The motors for the legs are driven by the motor drivers board I build using three SN754410 and two 74’04 chips. Twelve output pins are connected to the motor drivers board to control the direction of the motors and enable the motors. Zero in the direction pins will move the legs backward and one will move the legs forward. Zero in the enable pins will stop the legs and one will move the legs.

The motors for the tracks are controlled by the ME11. The signals are pulse width modulated using output compare two and three.

#### Sensors

Five types of sensors are used in this robot: Motion sensor, IR sensors, Bump sensors, Touch sensors and Leg Position sensors.

###### Motion Sensor

The motion sensor is used to detect the present of a human. It is a hack from a backyard motion activated light. The signal is tapped out from pin 7 of the op-amp on the circuit board. The signal from the op-amp is very small. The voltage sway is from -0.5 volt to +0.5 volt with a DC offset of 12v. The signal is in very low frequency. It actually looks like a DC signal in the oscilloscope. The signal will move up and down if a heat source in front of it is moving to left or right respectively. Since the signal can be negative, it is not suitable for the A/D converter of the 68HC11. I need to filter off the 12v dc offset and give it a 2.5v offset and amplify it so that the range of the signal will be limited to about 0 to 5 volt. Then, I found out that after the amplifier, the signal does not move up and down as smooth as the original signal. It suddenly jumps up for a very short time and come back down. That might not be very useful if I connect it directly to the A/D converter of the 68HC11. So, I add a comparator and a D-Flip Flop to latch the result so that I can reset the D-FF and read the result anytime I want. The final circuit is shown it fig. 6.

Because of the noise from the motors, whenever the motors move, the D-FF will be set and give a false result. I have to power up the motion sensor unit with a separate battery. When using battery to power up the motion sensor unit, it takes about five minutes to charge up the capacitors. So, I used a ten battery pack to reduce the charging time. The sensitivity will also benefit from  the ten battery pack instead of eight battery pack.

##### IR sensors

The IR sensors are hacked from the Sharp digital IR sensors. After the hacking, the sensors can give out analog signal from 1.5v to 2.5v depending on the distance of the object away from the sensors. So, that is used for collision avoidance. I used 330 ohm resistors for the IR LED and it can see while objects from 7” to 36”. The position of the IR sensors is shown in fig. 7.

##### Bump Sensors

Since the IR sensors cannot see black object, I also added four bump sensors at the four corners for collision avoidance. The bump sensors have higher priority over the IR sensors. Fig. 8 shows the position and the structure of the bump sensors. The design is borrow from “Critter” study in class.

The input port for the bump sensors are pull up to Vcc when the sensors are not bumped. The metal wire in the middle of the spring is grounded. Therefore, whenever it hit something, the input of the bump sensor will be grounded.

##### Touch/Position Sensors

The Touch sensors are momentary push buttons and the Position sensors are mini-level switches. When the switches are open, they are pulled up to Vcc; otherwise, they are pull to ground. Fig. 9 shows the position of the touch sensors and the position sensors.

The touch sensors are used to synchronize the legs’ movement. It enables the program to know when to stop the legs and when to move the legs. More about controlling the legs will be discussed in the programming technique section.

The position sensors are use for transformation from walking mode to tank mode. It enables the legs to retract to their highest position and stop.

#### Behaviors

One of the main behavior of this robot is its unpredictable behavior. I intended to make it more like a real animal, so I put a lot of random decision in the program. However, I do make some choices happen more likely over the others by comparing several bits of the TCOUNT register. Therefore, I cannot predict when it will transform and when it will look for a human to play with.

The following is a summary of the robot’s behavior :

- Dynamic Collision Avoidance

- Transform between Tank mode and Walking mode

- Tend to look for and walk to human

- When something is right in front of it, it will either turn around or transform

- When the power is low or it is tired (turn too many times) or if the legs slip or stuck, it will transform back to tank mode

- When it sees a human right in front of him, it will jiggle its tail

#### Programming Technique

In this section, I will talk about some of the programming technique that I used for the brain of transformer. I am using assembly for my program and sometimes, it takes a whole day to figure out a very small bug like missing a # sign, jump to the wrong label…etc. But I enjoy doing assembly because I have the control and I know what’s going on in terms of hardware.

##### Dynamic collision avoidance

I use the IR reading to directly calculate the speed of the track. The range of the IR reading is about 87 to 130. To control the speed of the right track, I take the reading from the front left IR sensor and subtract it by 120. If the result is positive, the object is very closed to the front left corner. So, I reverse the direction of the right track. The speed of the track is the result multiplied by 25. (The full speed of the track is 255) If the result is negative, I will move the right track forward and the speed of the track will be the result multiplied by 7. So, when the IR sees nothing, the track speed will be 231. The control of the left track is the same.

This technique is very easy to implement. It does not require a lot of if … else decision. The turn is very smooth and nature. However, there is one drop back of this technique. When an object is big and right in front of both sensors (Same reading from both the left and right IR sensors), it will slow down and finally stop. In this state, the robot is trapped and is going nowhere. Something, it will finally turn away but it takes a long time. So, I programmed to robot to either transform or turn away when the speed of both tracks is lower than certain level. That way, the robot will never be trapped again. It will “hesitate” in front of a big object and than do something to get away.

##### Walking Algorithm

The walking algorithm depends heavily on the walking pattern. The walking algorithm is very simple. By changing the pattern and the direction of the motors, I can use the same walking algorithm to move forward or backward and turn left or right. When it first transform to a walking robot, the pattern is 101010. Bit 0 of the pattern it the front left leg of the robot. Bit 1 is the middle left leg. Bit 3 is the front right leg … and so forth. A zero in the pattern means the leg is pointing backward (opposite direction of the motor movement). A one in the pattern means the leg is point forward (same direction of the motor movement). When the robot first transforms to walking mode the legs’ direction is shown in fig. 10a. For the walking algorithm, I first load in the walking pattern and store it to LEG_EN which will enable leg 1, 3 and 5. Then I read the touch sensors of the three legs, negate it and put it into LEG_EN. So leg 1, 3 and 5 will stop whenever it leaves the ground. The legs’ direction is shown in fig. 10b. Then I negate the walking pattern and store it into LEG_EN. Now, the pattern becomes 010101. So, leg 0, 2 and 4 will move. Then I read in the touch sensor and put it into LEG_EN. So, leg 0, 2 and 4 will stop when they touch the ground. Now the direction of the legs is as shown in fig 10c. That finished one moving step and I store the walking pattern back to W_PATTERN and return. If the walking algorithm is call again, it will move another step forward and the walking pattern will go back to the original 101010 as shown in fig. 10d – fig. 10e. If I want to walk backward, I just need to reverse the direction of the motors and call the walk algorithm again.

2     1     0	2     1     0 	2     1     0 	2     1     0	2     1     0

      	      	      	     	      

      	      	      	      	      

5      4     3	5      4     3	5      4     3	5      4     3	5      4     3

fig 10a	    fig 10b	   fig 10c	     fig 10d	     fig 10e

If I want to turn left, I just need to reverse the direction of the motors on the left hand side. Then, negate bit 0-2 of the walking pattern and call the walk algorithm. After the turn is done, I negate bit 0-2 of the walking pattern and store it back to W_PATTERN and I am really to make another left turn, right turn or move forward … etc. The sequence is shown in fig 11a –fig. 11d.(assuming the starting pattern is 101010)

2     1     0	2     1     0 	2     1     0 	2     1     0

      	      	      	     

      	      	      	      

5      4     3	5      4     3	5      4     3	5      4     3

fig 11a	    fig 11b	   fig 11c	     fig 11d

(101010)	(101101)	(010010)	(010101)

To make a right turn, just reverse bit 3-5 of the walking pattern and reverse the direction of the motors on the right hand side. Then, call the walk algorithm. After its done, reverse bit  3-5 of the walking pattern and store it back to W_PATTERN.

##### Dynamic Sensors Position

In my program, I have a function call “M_SET”. It will determine which sensors is the Front Left sensor, which one is the Front Right sensor, which one is the Rear sensors, etc… depending on what mode the robot is in. So, whenever it transforms, it just call that function and determine the position of the sensors. That way, I can use the same collision avoidance algorithm without changing anything.

Watchdog Program

I also use the RTI interrupt to write a small watchdog program to monitor the walking subroutine. If it stuck in the loop for more than five seconds, it will transform back to tank mode. Since the walking algorithm depends heavily on the touch sensors and the walking surface will affect the sensors’ reading dramatically, this is a safety feature for preventing any problems happen when in the walking mode.

Conclusion

The project was successful. I could make it transform as its own will. Compare to other robots, my robot is not as useful because mine is just walking around. However, that is what I intended to do. I wanted to make it act like a real pet. My future development will be adding more complex behavior, programming it to look for food and learn how to walk. In this project, I learned a lot of practical knowledge which I can never learn in other classes. Things look very easy on paper is not that easy when I actually built it. Nothing is easy when it comes to the real world. Using the right tools and finding the right parts are also critical. With the right tools, it will take just minutes to finish the job. With the wrong tools, it takes days to do the same job. Cutting aluminum with a Dremel is not a very good idea, but that’s the only tool I have. I also found that Epoxy Strip was extremely useful when I mount my bump sensor onto the body. It can mold like clay but after it’s dry, it is very strong and can stick almost anything together. It can be found in Radio Shack. I enjoy this class and doing this project. Especially working with other people in the lab overnight. Sometimes, I think I put too much pressure on myself and that could be very stressful.

Appendix – source code

* Title         : Machine Intelligence Design Lab (Summer 98)

* Filename      : T-Former.ASM

* Programmer    : (Andy) Su Va Fong

* Date          : 07/31/97

* Version       : 1.0

* Description   : This program is the brain of my robot - Transformer

**************************************************************************

* Define the address locations of the various registers and user-defined

* constants used in the program

**************************************************************************

*

BAUD    EQU     $102B  ; BAUD rate control register to set the BAUD rate

SCCR1   EQU     $102C  ; Serial Communication Control Register-1

SCCR2   EQU     $102D  ; Serial Communication Control Register-2

SCSR    EQU     $102E  ; Serial Communication Status Register

SCDR    EQU     $102F  ; Serial Communication Data Register

EOS     EQU     $04    ; User-defined End Of String (EOS) character

CR      EQU     $0D    ; Carriage Return Character

LF      EQU     $0A    ; Line Feed Character

ESC     EQU     $1B    ; Escape Charracter

BASE    EQU     $1000  ; Beginning of Registers

PORTD   EQU     $08    ; Port D

DDRD    EQU     $09    ; Data Direction Register of Port D

ADCTL   EQU     $30

OPTION  EQU     $39

ADR1    EQU     $31    ; A/D Register 1

ADR2    EQU     $32    ; A/D Register 2

ADR3    EQU     $33    ; A/D Register 3

ADR4    EQU     $34    ; A/D Register 4

TOC2    EQU     $18    ; Output Compare 2 register

TOC3    EQU     $1A    ; Output Compare 3 register

TOC4    EQU     $1C    ; Output Compare 4 register

PACTL   EQU     $26    ;

TCTL1   EQU     $20    ; Timer Control register

TMSK1   EQU     $22    ; Timer Mask1 Register

TFLG1   EQU     $23    ; Timer Flag1 Register

TFLG2   EQU     $25    ; Timer Flag2 Register

TMSK2   EQU     $24    ; Timer Mask2 Register

TCNT    EQU     $0F

BUMP1   EQU     %00000001     ; Bump Sensor 1 postion

BUMP2   EQU     %00000010     ; Bump Sensor 2 postion

BUMP3   EQU     %00000100     ; Bump Sensor 3 postion

BUMP4   EQU     %00001000     ; Bump Sensor 4 postion

* Masks

BIT0    EQU     %00000001

BIT1    EQU     %00000010

BIT2    EQU     %00000100

BIT3    EQU     %00001000

BIT4    EQU     %00010000

BIT5    EQU     %00100000

BIT6    EQU     %01000000

BIT7    EQU     %10000000

out1byt equ     $e4fF   ; out1byt subroutine address

outcrlf equ     $e508   ; outcrlf subroutine address

Period  EQU     $FFFF   ; Period of the PWM for both tracks

LEG_EN  EQU     $4000   ; Leg enable address

LEG_DIR EQU     $5000   ; Leg direction address

CDS_SEL EQU     $6000   ; CDS cell selection address

MOTION  EQU     $6000   ; Motion detection address

LED     EQU     $6000   ; LED output address

IR_ADR  EQU     $7000   ; IR LED output address

T_ADR   EQU     $5000   ; Touch sensor input adress

B_ADR   EQU     $6000   ; Bump sensor input address

POS_ADR EQU     $4000   ; Leg position sensor input address

IR_TH   EQU     120     ; IR threshold value

IR_MAX  EQU     130     ; IR range (maximum)

IR_MIN  EQU     80      ; IR range (minimum)

BACKWARD EQU    %00000000       ; Direction of legs for walking backward

FORWARD EQU     %00111111       ; Direction of legs for walking forward

RIGHT   EQU     %00000111       ; Direction of legs for turning right

LEFT    EQU     %00111000       ; Direction of legs for turning left

*

************************************************************************

* Initialize Interrupt Jump Vectors

************************************************************************

* Jump Vector for TOC2_ISR

ORG     $00DC

JMP     TOC2ISR

* Jump Vector for TOC3_ISR

ORG     $00D9

JMP     TOC3ISR

* Jump Vector for RTI_ISR

ORG     $00EB

JMP     RTI_ISR

************************************************************************

*               Define Strings for displaying messages

************************************************************************

ORG     $8000

JMP     Main

ClrScr  FCB     ESC,$5B,$32,$4A         ; ANSI sequence to clear screen

FCB     ESC,$5B,$3B,$48         ;   and move cursor to home

FCB     EOS                     ; EOS character

************************************************************************

*                       Data Section

************************************************************************

*Tank Mode Data

Track0  FCB     0       ; Left Track Speed

Track1  FCB     0       ; Right Track Speed

Dir0    FCB     0       ; Left Track Direction

Dir1    FCB     0       ; Right Track Direction

High0   FDB     $0000   ; High time of the PWM for the left track

High1   FDB     $0000   ; High time of the PWM for the right track

*Sensors Data

Touch   FCB     0       ; Reading from touch sensors

Positn  FCB     0       ; Reading from leg position sensors

FL_BUMP FCB     0       ; Reading from Front left bump sensor

FR_BUMP FCB     0       ; Reading from Front Right bump sensor

RL_BUMP FCB     0       ; Reading from Rear Left bump sensor

RR_BUMP FCB     0       ; Reading from Rear Right bump sensor

Bump    FCB     0

Motion  FCB     0       ; Reading from the motion detection

IR0     FCB     0       ; Reading from IR sensor 0

IR1     FCB     0       ; Reading from IR sensor 1

IR2     FCB     0       ; Reading from IR sensor 2

IR3     FCB     0       ; Reading from IR sensor 3

IR4     FCB     0       ; Reading from IR sensor 4

IR5     FCB     0       ; Reading from IR sensor 5

CDS0    FCB     0       ; Reading from IR CDS 0

CDS1    FCB     0       ; Reading from IR CDS 1

CDS2    FCB     0       ; Reading from IR CDS 2

CDS3    FCB     0       ; Reading from IR CDS 3

CDS4    FCB     0       ; Reading from IR CDS 4

BATTERY FCB     0       ; Reading from Battery potential meter

MODE    FCB     0       ; mode of the robot (0 - tank, 1 - walk)

FL_IR   RMB     2       ; front left IR sensor address

FR_IR   RMB     2       ; front right IR sensor address

R_IR    RMB     2       ; rear IR sensor address

CA_L    FCB     0       ; Collision avoidance (left motor control)

CA_R    FCB     0       ; Collision avoidance (right motor control)

WALK_P  FCB     %00101010       ; Walking pattern

COUNT   FDB     0       ; Count from RTI (Testing only)

W_COUNT FDB     0       ; Count for walking timeout

Timeout FCB     0       ; Timeout - set if W_count > 1250

OUTPUT3 FCB     0       ; Buffer for output port 3

HUMAN   FCB     0       ; Set when human is detected

T_C     FCB     0       ; Count how many turns are made when walking

************************************************************************

*                       MAIN PROGRAM

************************************************************************

*  Initialization

Main    LDS     #$0041  ; Initial the stack pointer

LDX     #BASE   ; Base address for system register

LDAA    #0      ; Disable legs when first start

STAA    LEG_EN

JSR     InitOC  ; Initialize Output Compare

JSR     InitRTI ; Initialize RTI

CLI                     ; Enable the interrupt system

BSET    OPTION,X BIT7   ; Turn on A/D converter

LDAA    #%00110000      ; Setting for A/D converter

STAA    ADCTL,X         ; Scan continuously and multiple channel

LDAA    #40             ; Wait until the power of A/D converter

WAIT    DECA                    ; to stablize. (Charge up the Caps)

BNE     WAIT

LDAA    #%00110000      ; Direction of PortD 5:4 for L & R tracks

STAA    DDRD,X          ; Set to output to control the tracks

JSR     Sensors         ; Get the sensors reading

JSR     Sensors         ; Usually the first reading is junky

* Always start in tank mode

LDAA    POS_ADR         ; Retract all the legs to their highest pos

ANDA    #%00111111      ; and start in tank mode

EORA    #%00111111      ; If not all of the position sensors are closed

BEQ     START           ; call the subroutine to transform to tank mode

LDAA    #1

STAA    MODE

JSR     M1_M0

* Reset the motion sensor

START   LDY     #OUTPUT3        ; Reset the D-FF in the motion detection

BCLR    0,Y BIT7        ; amplifying circuit

LDAA    OUTPUT3

STAA    MOTION

BSET    0,Y BIT7

LDAA    OUTPUT3

STAA    MOTION

REPEAT  JSR     M_SET           ; Set the position of sensors according to

; the mode

JSR     Sensors         ; Get readings from all the sensors

JSR     C_AVOID         ; Collision avoidance

* Check if Battery is ok

LDAA    BATTERY         ; Check battery

LSRA                    ; if lower than 9v, turn on a LED

CMPA    #$68            ; and transform to tank mode

BGE     B_OK

LDY     #OUTPUT3

BSET    0,Y BIT3

LDAA    OUTPUT3

STAA    LED

JSR     M1_M0

B_OK    LDAA    TCNT,X         ; Randomly check (spin around) if

ANDA    #%11111100     ; any human around

BNE     CONT

JSR     D_HUMAN        ; Check if anyone in front of me

CONT    LDAA    MODE           ; Check the mode, and call the corresponding

BEQ     A0             ; arbitrator

* Check if the legs are stuck

LDAA    Timeout        ; If stuck more than 5 secs, transform back

BEQ     A1             ; to tank mode

JSR     M1_M0

LDAA    #0             ; Reset the timeout variable

STAA    Timeout

BRA     A0             ;

A1      JSR     M0_M1          ; else transform to walking mode

JSR     MODE1          ; Call walking mode arbitrator

REPEAT_ BRA     REPEAT

A0      JSR     M1_M0          ; otherwise, transform to tank mode

JSR     MODE0          ; Call tank mode arbitrator

* Output sensors reading to terminal (Testing only)

PSHX

LDX     #IR0

JSR     out1byt

JSR     out1byt

JSR     out1byt

JSR     out1byt

JSR     out1byt

JSR     out1byt

JSR     out1byt

JSR     out1byt

JSR     out1byt

JSR     out1byt

JSR     out1byt

JSR     out1byt

jsr     outcrlf

LDX     #$FFFF

TEST    DEX

BNE     TEST

PULX

BRA     REPEAT_

************************************************************************

*                       SUBROUTINE - InitOC

************************************************************************

InitOC  PSHA

PSHX

LDX     #BASE

BCLR    TMSK2,X BIT1    ; Set the pre-scaler frequency for TCNT

BCLR    TMSK2,X BIT2    ; PR1:PR0=00 for 2MHz

* Initialize Output compare OC2 and OC3

LDAA    #%10100000      ; OM2:Ol2 = 10 for setting to low

STAA    TCTL1,X         ; OM3:OL3 = 10 for setting to low

LDAA    #%01100000      ; Enable interrupt from OC2 and OC3

STAA    TMSK1,X

PULX

PULA

RTS

************************************************************************

*           Interupt Service Routine - TOC2ISR

*       Create the PWM according to High1

************************************************************************

TOC2ISR LDX     #BASE

BRCLR   TFLG1,X BIT6 TOC2RTI   ; Check for correct interrupt

LDAA    #BIT6           ; Clear the OC2 flag

STAA    TFLG1,X

LDD     High1

BNE     MOVE1

STOP1   BCLR    TCTL1,X BIT6

BRA     TOC2RTI

MOVE1   BRSET   TCTL1,X BIT6 LASTHI1

BSET    TCTL1,X BIT6

LDD     #Period

SUBD    High1

ADDD    TOC2,X

STD     TOC2,X

BRA     TOC2RTI

LASTHI1 BCLR    TCTL1,X BIT6

LDD     High1

ADDD    TOC2,X

STD     TOC2,X

TOC2RTI RTI

************************************************************************

*           Interupt Service Routine - TOC3ISR

*       Create the PWM according to High0

************************************************************************

TOC3ISR LDX     #BASE

BRCLR   TFLG1,X BIT5 TOC3RTI   ; Check for correct interrupt

LDAA    #BIT5           ; Clear the OC3 flag

STAA    TFLG1,X

LDD     High0

BNE     MOVE0

STOP0   BCLR    TCTL1,X BIT4

BRA     TOC3RTI

MOVE0   BRSET   TCTL1,X BIT4 LASTHI0

BSET    TCTL1,X BIT4

LDD     #Period

SUBD    High0

ADDD    TOC3,X

STD     TOC3,X

BRA     TOC3RTI

LASTHI0 BCLR    TCTL1,X BIT4

LDD     High0

ADDD    TOC3,X

STD     TOC3,X

TOC3RTI RTI

************************************************************************

*           Interupt Service Routine - RTI_ISR

************************************************************************

RTI_ISR LDX     #BASE

BRCLR   TFLG2,X BIT6 RTI_RTI

LDAA    #BIT6

STAA    TFLG2,X

LDX     COUNT     ; Increase the COUNT (FOR TESTING ONLY)

INX

STX     COUNT

LDAA    MODE      ; If in walking mode

BEQ     RTI_RTI

LDX     W_COUNT   ; Start counting

INX

STX     W_COUNT

CPX     #1250     ; If in the walkin subroutine for > 5 secs

BLT     RTI_RTI

LDAA    #1      ; Timeout, tranform back to tank

STAA    Timeout

LDX     #0       ; Reset the counter

STX     W_COUNT

RTI_RTI RTI

************************************************************************

*           Tank Moving Mechanism Control - TMMC

*       Change the direction of the tracks according to Dir0 & Dir1

*       Calculate High0 & High1 according to Track0 & Track1

************************************************************************

TMMC    PSHA

PSHB

PSHX

LDX     #BASE

* Left Track

LEFT1   LDAA    Dir0

BNE     BACK0

FORW0   BCLR    PORTD,X BIT5

BRA     Set_H0

BACK0   BSET    PORTD,X BIT5

Set_H0  LDAA    Track0

LDAB    #$FF

MUL

STD     High0

* Right Track

RIGHT1  LDAA    Dir1

BNE     BACK1

FORW1   BCLR    PORTD,X BIT4

BRA     Set_H1

BACK1   BSET    PORTD,X BIT4

Set_H1  LDAA    Track1

LDAB    #$FF

MUL

STD     High1

PULX

PULB

PULA

RTS

************************************************************************

*       Sensors Data Gathering - Sensors

*       Gather all sensors information

************************************************************************

Sensors PSHA

PSHX

LDX     #BASE

LDAA    $4000

STAA    Positn

LDAA    $5000

STAA    Touch

LDAA    $6000

STAA    Bump

LDAA    #$FF                  ; Turn on IR LED

STAA    IR_ADR

LDAA    #%00010000              ; Start sampling Channel 1 - 4

STAA    ADCTL,X

CHECK   BRCLR   ADCTL,X BIT7 CHECK

LDAA    ADR1,X

STAA    IR0

LDAA    ADR2,X

STAA    IR1

LDAA    ADR3,X

STAA    IR2

LDAA    ADR4,X

STAA    IR3

* Select the first CDS

LDAA    OUTPUT3

ANDA    #%11111000

STAA    CDS_SEL

STAA    OUTPUT3

*        LDAA    #%00000000

*        STAA    CDS_SEL

LDAA    #%00010100

STAA    ADCTL,X

CHECK2  BRCLR   ADCTL,X BIT7 CHECK2

LDAA    ADR1,X

STAA    IR4

LDAA    ADR2,X

STAA    IR5

LDAA    ADR3,X

STAA    CDS0

LDAA    ADR4,X

STAA    BATTERY

*       LDAA    #0              ; Turn off the IR LED

*       STAA    IR_ADR

* Select the second CDS

LDAA    OUTPUT3

ANDA    #%11111000

EORA    #%00000001

STAA    CDS_SEL

STAA    OUTPUT3

*        LDAA    #%00000001

*        STAA    CDS_SEL

LDAA    #%00000110

STAA    ADCTL,X

LDAA    #6

WAIT3   DECA

BNE     WAIT3

LDAA    ADR1,X

STAA    CDS1

* Select the third CDS

LDAA    OUTPUT3

ANDA    #%11111000

EORA    #%00000010

STAA    CDS_SEL

STAA    OUTPUT3

*        LDAA    #%00000010

*        STAA    CDS_SEL

LDAA    #%00000110

STAA    ADCTL,X

LDAA    #6

WAIT4   DECA

BNE     WAIT4

LDAA    ADR1,X

STAA    CDS2

* Select the forth CDS

LDAA    OUTPUT3

ANDA    #%11111000

EORA    #%00000011

STAA    CDS_SEL

STAA    OUTPUT3

*        LDAA    #%00000011

*        STAA    CDS_SEL

LDAA    #%00000110

STAA     ADCTL,X

LDAA    #6

WAIT5   DECA

BNE     WAIT5

LDAA    ADR1,X

STAA    CDS3

* Select the fifth CDS

LDAA    OUTPUT3

ANDA    #%11111000

EORA    #%00000100

STAA    CDS_SEL

STAA    OUTPUT3

*        LDAA    #%00000100

*        STAA    CDS_SEL

LDAA    #%00000110

STAA     ADCTL,X

LDAA    #6

WAIT6   DECA

BNE     WAIT6

LDAA    ADR1,X

STAA    CDS4

PULX

PULA

RTS

************************************************************************

************************************************************************

*                       SUBROUTINE - InitSCI

* Description: This subroutine initializes the BAUD rate to 9600 and

*              sets up the SCI port for 1 start bit, 8 data bits and

*              1 stop bit.  It also enables the transmitter and receiver.

*              Effected registers are BAUD, SCCR1, and SCCR2.

* Input         : None.

* Output        : Initializes SCI.

* Destroys      : None.

* Calls         : None.

************************************************************************

*

InitSCI PSHA                            ; Save contents of A register

LDAA    #$30                    ; Set BAUD rate to 9600

STAA    BAUD

LDAA    #$00                    ; Set SCI Mode to 1 start bit,

STAA    SCCR1                   ;     8 data bits, and 1 stop bit.

LDAA    #%00001100              ; Enable SCI Transmitter

STAA    SCCR2                   ;     and Receiver

PULA                            ; Restore A register

RTS                             ; Return from subtoutine

*

************************************************************************

*       Set the mode of the robot (track or walking) - M_SET

*       Determine which sensors are the front sensors and the rear sensors

*       .... etc. So that I can use the same subroutine in either walking

*       or tank mode.

************************************************************************

M_SET   PSHX

PSHA

LDAA    MODE

BNE     M_WALK

* Tank Mode

M_TANK  LDX     #IR2

STX     FR_IR

LDX     #IR3

STX     FL_IR

LDX     #IR5

STX     R_IR

LDAA    #BUMP1

STAA    FL_BUMP

LDAA    #BUMP2

STAA    RL_BUMP

LDAA    #BUMP3

STAA    FR_BUMP

LDAA    #BUMP4

STAA    RR_BUMP

BRA     M_SET_E

* Walking Mode

M_WALK  LDX     #IR0

STX     FR_IR

LDX     #IR1

STX     FL_IR

LDX     #IR4

STX     R_IR

LDAA    #BUMP3

STAA    FL_BUMP

LDAA    #BUMP1

STAA    RL_BUMP

LDAA    #BUMP4

STAA    FR_BUMP

LDAA    #BUMP2

STAA    RR_BUMP

M_SET_E PULA

PULX

RTS

************************************************************************

*       Collision Avoidance - C_AVOID

************************************************************************

C_AVOID PSHA

PSHB

PSHX

C4      LDX     FL_IR

LDAA    0,X

SUBA    #120

CLV

BGT     C1

NEGA

LDAB    #7

MUL

STAB    CA_R

LDAA    #0

STAA    Dir1

BRA     C2

C1      LDAB    #25

MUL

STAB    CA_R

LDAA    #1

STAA    Dir1

C2      LDX     FR_IR

LDAA    0,X

SUBA    #120

CLV

BGT     C3

NEGA

LDAB    #7

MUL

STAB    CA_L

LDAA    #0

STAA    Dir0

BRA     CA_E

C3      LDAB    #25

MUL

STAB    CA_L

LDAA    #1

STAA    Dir0

CA_E    PULX

PULB

PULA

RTS

*

*

************************************************************************

************************************************************************

*       Arbitrator in Tank mode - MODE0

************************************************************************

MODE0   PSHA

PSHB

PSHX

* Bump sensor check

FL_C    LDAA    B_ADR

ANDA    FL_BUMP

BNE     FR_C

FL_B    LDAA    #1

STAA    Dir1

STAA    Dir0

LDAA    #$00

STAA    Track0

LDAA    #$FF

STAA    Track1

JSR     TMMC

FL_BC   LDAA    B_ADR

ANDA    FL_BUMP

BEQ     FL_BC

LDAA    #$05

FL_BD1  LDX     #$FFFF

FL_BD2  DEX

BNE     FL_BD2

DECA

BNE     FL_BD1

BRA     TRAP_

FR_C    LDAA    B_ADR

ANDA    FR_BUMP

BNE     RL_C

FR_B    LDAA    #1

STAA    Dir1

STAA    Dir0

LDAA    #$FF

STAA    Track0

LDAA    #$00

STAA    Track1

JSR     TMMC

FR_BC   LDAA    B_ADR

ANDA    FR_BUMP

BEQ     FR_BC

LDAA    #$05

FR_BD1  LDX     #$FFFF

FR_BD2  DEX

BNE     FR_BD2

DECA

BNE     FR_BD1

TRAP_   BRA     TRAP

RL_C    LDAA    B_ADR

ANDA    RL_BUMP

BNE     RR_C

RL_B    LDAA    #0

STAA    Dir1

STAA    Dir0

LDAA    #$00

STAA    Track0

LDAA    #$FF

STAA    Track1

JSR     TMMC

RL_BC   LDAA    B_ADR

ANDA    RL_BUMP

BEQ     RL_BC

LDAA    #$05

RL_BD1  LDX     #$FFFF

RL_BD2  DEX

BNE     RL_BD2

DECA

BNE     RL_BD1

BRA     TRAP

RR_C    LDAA    B_ADR

ANDA    RR_BUMP

BNE     TRAP

RR_B    LDAA    #0

STAA    Dir1

STAA    Dir0

LDAA    #$FF

STAA    Track0

LDAA    #$00

STAA    Track1

JSR     TMMC

RR_BC   LDAA    B_ADR

ANDA    RR_BUMP

BEQ     RR_BC

LDAA    #$05

RR_BD1  LDX     #$FFFF

RR_BD2  DEX

BNE     RR_BD2

DECA

BNE     RR_BD1

* if traped, turn; otherwise following the IR reading

TRAP    LDAA    CA_L

ANDA    #%11100000

BNE     MODE0_E

LDAA    CA_R

ANDA    #%11100000

BNE     MODE0_E

LDX     #BASE           ; If traps, randomly transform

LDAA    TCNT,X

ANDA    #%11110000

BNE     M0_M4

JSR     M0_M1

BRA     END

M0_M4   LDX     #BASE          ; Randomly turn left or right when traped

LDAA    TCNT,X

ANDA    #%10000000

BEQ     M9

LDAA    #0

STAA    Dir0

LDAA    #1

STAA    Dir1

BRA     M10

M9      LDAA    #0

STAA    Dir1

LDAA    #1

STAA    Dir0

M10     LDAA    #$80

STAA    Track0

STAA    Track1

JSR     TMMC

LDAA    #05

M31     LDX     #$FFFF

M30     DEX

BNE     M30

DECA

BNE     M31

BRA     END

* following the IR reading for collision avoidance

MODE0_E LDAA    CA_L

STAA    Track0

LDAA    CA_R

STAA    Track1

JSR     TMMC

END     PULX

PULB

PULA

RTS

************************************************************************

************************************************************************

*       Transform from Tank mode to Walking mode - M0_M1

************************************************************************

M0_M1   PSHA                    ; Return if already in mode 1

LDAA    MODE

BNE     M0_M1_E

LDAA    #$00            ; Stop the tracks

STAA    Track0

STAA    Track1

JSR     TMMC

LDAA    #%00101010      ; Set the leg direction

STAA    LEG_DIR

CK      LDAA    T_ADR           ; move three legs until they touch the ground

ANDA    #%00010101

BEQ     ST1

STAA    LEG_EN

BRA     CK

ST1     LDAA    T_ADR           ; move other three legs likewise

ANDA    #%00101010

BEQ     ST2

STAA    LEG_EN

BRA     ST1

ST2     LDAA    #0

STAA    LEG_EN

LDAA    #%00101010      ; Set the walking pattern

STAA    WALK_P

M0_M1_E LDAA    #1              ; Set mode to 1

STAA    MODE

PULA

RTS

************************************************************************

*       Transform from Walking mode to Tank mode - M1_M0

************************************************************************

M1_M0   PSHA                    ; Return if already in mode 0

LDAA    MODE

BEQ     M1_M0_E

LDAA    #%00010101      ; Set direction of the legs

STAA    LEG_DIR

CH      LDAA    POS_ADR         ; Move all the legs until all the position

ANDA    #%00111111      ; sensors are closed

EORA    #%00111111

BEQ     SP

STAA    LEG_EN

BRA     CH

SP      LDAA    #0

STAA    LEG_EN

M1_M0_E LDAA    #0

STAA    MODE

PULA

RTS

************************************************************************

*       Walking algorithm - WALK

************************************************************************

WALK    PSHA

PSHB

LDD     #0              ; Reset W_count

STD     W_COUNT

LDAA    Timeout         ; Check if the Timeout flag is set by the RTI

BNE     W3

LDAA    WALK_P          ; move the legs according to the walking pattern

STAA    LEG_EN

W1      LDAA    T_ADR           ; keep moving until the legs touch the round

ANDA    WALK_P

EORA    WALK_P

BEQ     W2

STAA    LEG_EN

LDAA    Timeout         ; Check for timeout

BNE     W3

BRA     W1

W2      LDAA    WALK_P          ; negate and update the walking pattern

EORA    #%00111111      ; move the other legs forward

STAA    LEG_EN

STAA    WALK_P

* Wait until all three legs left the ground

W5      LDAA    T_ADR

ANDA    WALK_P

BEQ     W5

W4      LDAA    T_ADR           ; move until the legs touch the round

ANDA    WALK_P

BEQ     W3

STAA    LEG_EN

LDAA    Timeout         ; Check for timeout

BNE     W3

BRA     W4

W3      LDAA    #0              ; Stop all the legs

STAA    LEG_EN

PULB

PULA

RTS

************************************************************************

*       TURNING LEFT - TURN_L

*       Change the direction of the legs and walking pattern and then call

*       the walk subroutine

************************************************************************

TURN_L  PSHA

LDAA    #LEFT

STAA    LEG_DIR

LDAA    WALK_P

EORA    #RIGHT

STAA    WALK_P

JSR     WALK

LDAA    WALK_P

EORA    #RIGHT

STAA    WALK_P

INC     T_C

PULA

RTS

************************************************************************

*       TURNING RIGHT - TURN_R

*       Change the direction of the legs and walking pattern and then call

*       the walk subroutine

************************************************************************

TURN_R  PSHA

LDAA    #RIGHT

STAA    LEG_DIR

LDAA    WALK_P

EORA    #LEFT

STAA    WALK_P

JSR     WALK

LDAA    WALK_P

EORA    #LEFT

STAA    WALK_P

INC     T_C

PULA

RTS

************************************************************************

*       Walking FORWARD - WALK_F

*       Change the direction of the legs and walking pattern and then call

*       the walk subroutine

************************************************************************

WALK_F  PSHA

LDAA    #FORWARD

STAA    LEG_DIR

JSR     WALK

PULA

RTS

************************************************************************

*       Walking BACKWARD - WALK_B

*       Change the direction of the legs and walking pattern and then call

*       the walk subroutine

************************************************************************

WALK_B  PSHA

LDAA    #BACKWARD

STAA    LEG_DIR

JSR     WALK

LDAA    WALK_P

EORA    #%00111111

STAA    WALK_P

PULA

RTS

************************************************************************

*       Arbitrator in WALKING mode - MODE1

************************************************************************

MODE1   PSHA

PSHB

PSHX

* Check the bump sensors

FL_C1   LDAA    B_ADR

ANDA    FL_BUMP

BNE     FR_C1

JSR     TURN_R

BRA     FL_C1

FR_C1   LDAA    B_ADR

ANDA    FR_BUMP

BNE     RL_C1

JSR     TURN_L

BRA     FR_C1

RL_C1   LDAA    B_ADR

ANDA    RL_BUMP

BNE     RR_C1

JSR     TURN_L

BRA     RL_C1

RR_C1   LDAA    B_ADR

ANDA    RR_BUMP

BNE     TRAP1

JSR     TURN_R

BRA     RR_C1

* if traped, turn; otherwise following the IR reading

TRAP1   JSR     D_MOTN          ; spin around and detect human

LDAA    CA_L

ANDA    #%11100000

BNE     MODE1_E

LDAA    CA_R

ANDA    #%11100000

BNE     MODE1_E

*        JSR     D_MOTN

LDAA    HUMAN           ; jiggle the tail if see a human

BNE     H1

JSR     S_TAIL

H1      LDX     #BASE           ; Maybe transform back to tank

LDAA    TCNT,X

ANDA    #%00010000

BNE     M1_T

JSR     M1_M0

BRA     END1

M1_T    LDX     #BASE           ; Or turn left or right

LDAA    TCNT,X

ANDA    #%10000000

BEQ     M1_9

JSR     TURN_L

JSR     TURN_L

JSR     TURN_L

JSR     TURN_L

BRA     MODE1_E

M1_9    JSR     TURN_R

JSR     TURN_R

JSR     TURN_R

JSR     TURN_R

* following the IR reading for collision avoidance

MODE1_E LDAA    Dir0

EORA    Dir1

BEQ     M1_1

LDAA    Dir0

BEQ     M1_R

M1_L    LDAA    HUMAN       ; Whenever see a human in front of it,

BNE     H2          ; jiggle the tail

JSR     S_TAIL

H2      JSR     TURN_L

BRA     END1

M1_R    LDAA    HUMAN

BNE     H3

JSR     S_TAIL

H3      JSR     TURN_R

BRA     END1

M1_1    LDAA    Dir0

BEQ     M1_F

M1_B    LDAA    HUMAN

BNE     H4

JSR     S_TAIL

H4      JSR     WALK_B

BRA     END1

M1_F    JSR     WALK_F

LDAA    T_C             ; If turned too many times, transform back

ANDA    #%11110000      ; to tank mode

BEQ     END1

LDAA    #0

STAA    T_C

JSR     M1_M0

END1    PULX

PULB

PULA

RTS

************************************************************************

*                       SUBROUTINE - InitRTI

************************************************************************

InitRTI PSHA

PSHX

LDX     #BASE

LDAA    #$00    ; interrupt every 4 ms

STAA    PACTL,X

LDAA    #%01000000

STAA    TMSK2,X

PULX

PULA

RTS

************************************************************************

*                       SUBROUTINE - D_HUMAN

*       Spin around the detect human, stop when detected. Or keep moving

*       around after a while and no human around. Return 1 in Human when

*       detected

************************************************************************

D_HUMAN PSHA

PSHX

PSHY

LDAA    MODE

BNE     T1

T0      LDAA    #0

STAA    Dir1

LDAA    #1

STAA    Dir0

LDAA    #$B0

STAA    Track0

STAA    Track1

JSR     TMMC

LDAB    #$0A

M0_M3   LDX     #$FFFF

M0_M    JSR     D_MOTN

LDAA    HUMAN

BNE     M0_M2

DEX

BNE     M0_M

DECB

BNE     M0_M3

BRA     DH_E

T1      LDAB    #10

JSR     TURN_L

JSR     D_MOTN

LDAA    HUMAN

BNE     M0_M2

DECB

BNE     T1

BRA     DH_E

M0_M2   JSR     M0_M1

DH_E    PULY

PULX

PULA

RTS

************************************************************************

*                       SUBROUTINE - D_MOTN

*       Reset the D-FF of the motion sensors. Wait for a while, if human

*       is detected, return 1 in Human.

************************************************************************

D_MOTN  PSHA

PSHY

LDAA    #0

STAA    HUMAN

LDY     #OUTPUT3

BCLR    0,Y BIT7

LDAA    OUTPUT3

STAA    MOTION

BSET    0,Y BIT7

LDAA    OUTPUT3

STAA    MOTION

LDAA    #$FF

D1      DECA

BNE     D1

LDAA    MOTION

ANDA    #%00010000

BEQ     DM_E

LDAA    #1

STAA    HUMAN

DM_E    PULY

PULA

RTS

************************************************************************

*                       SUBROUTINE - S_TAIL

*       Jiggle the tail by moving the track in the rear end back and forth

************************************************************************

S_TAIL  PSHA

PSHB

PSHX

JSR     M1_M0

LDAA    #0

STAA    Track1

LDAA    #$FF

STAA    Track0

LDAA    #0

LDAB    #6

S_W3    LDX     #$1000

S_W2    STAA    Dir0

JSR     TMMC

DEX

BNE     S_W2

EORA    #%00000001

DECB

BNE     S_W3

LDAA    #0

STAA    Track0

STAA    Track1

JSR     TMMC

JSR     M0_M1

ST_E    PULX

PULB

PULA

RTS