# ARGO: Automated Transport

## John Ferrara

Intelligent Machine Design Laboratory

EEL 5666

3 August 1998

###### Table of Contents

#### Page									Contents

3	……………………………………………………………..	Abstract

4	……………………………………………………………..	Executive Summary

5	……………………………………………………………..	Introduction

……..	Integrated Systems

6	……………………………………………………………..	Integrated Systems

……..	Mobile Platform

7	……………………………………………………………..	Mobile Platform

8          ……………………………………………………………..	Mobil Platform							……..	Actuation

9	……………………………………………………………..	Actuation

10	……………………………………………………………..	Actuation

11	……………………………………………………………..	Actuation

……..	Sensors

12	……………………………………………………………..	Sensors

...…..	Sensors

13	……………………………………………………………..	Sensors

…….	Behaviors

14	……………………………………………………………..	Behaviors

15	……………………………………………………………..	Behaviors

16	……………………………………………………………..	Behaviors

17	……………………………………………………………..	Results

18	……………………………………………………………..	Results

19	……………………………………………………………..	Results

20	……………………………………………………………..	Results

21	……………………………………………………………..	Conclusions

22	……………………………………………………………..	Conclusions

23	……………………………………………………………..	Code Listing

…

47	……………………………………………………………..	Code Listing

###### Abstract

The following is a discussion of ARGO, and an automated chauffeur.  Accomplishment of this task involved the use of infrared sensors and emitters, light/voltage converters, and bump sensors.  The robot uses a multitasking environment in which to run several processes simultaneously.  These processes are overseen by a command arbiter that determines the current behavior.  The most developed behaviors that ARGO possesses are hi differential obstacle avoidance, and lane following.  Both use “fuzzy” logic to accomplish smooth driving.

### Executive Summary

ARGO was developed to be an autonomous chauffeur of the future.  The robot is constructed from a modified RC-car: bumpers, and a sensor hood have been added.  The robot uses the Motorola 68HC11 microprocessor to multitask 14 separate processes, as well as the entire interrupt system—taken advantage of through the assembly language.  These processes constitute the behavioral patterns of the robot. Some of which are lane following, obstacle avoidance, and light finding.

These algorithms were designed using a fuzzy logic system that ensures smooth turning and smooth transitions between different speeds.  The obstacle avoidance has been refined to allow the robot to avoid obstacles at a relatively high rate of speed.

To accomplish these tasks, the robot uses five infrared emitter/detector pairs, two bump sensors, and two light/voltage converters, a high current(100+amps) motor, and a steering servo.  When used in conjunction the robot displays very good obstacle avoidance behavior, and decent lane following behavior.

ARGO uses a self-designed h-bridge motor driver that allows the car to operate at nearly a continuous range of speeds between stop and full speed. In addition, ARGO, has a software rest that enables the user all the benefits of the BUFFALO operating system plus the added benefits of special bootstrap mode.

### Introduction

The world is becoming increasingly congested.  An increasing number of people are taking to the highways to travel, resulting in a higher fatality rate due to automotive relative accidents.  What the world needs is something safer, something that can react almost immediately to changing roadway conditions.  Herein lies ARGO: the automated chauffeur.

Based upon the Motorola 68HC11 EVBU board, complimented with the ME11 board, ARGO uses an assembly written program to multitask his way around obstacles, and to stay between lane markers.  His superior digital motor driver and fuzzy logic steering enables him to act and react just like an automobile.

Herein lies the story of ARGO.

### Integrated Systems

The foundation upon which ARGO derives his behavior is a multitasking system.  Allotment of a small time slice to each of the processes running on the microprocessor yields the appearance that each process is running independent of the rest.  This infrastructure allows for the easy addition of multiple behaviors, and for the seamless interaction among all of the processes.  This interaction is facilitated by the addition of a command arbiter: an overseeing process that reviews each processes’ recommendation to change the physical state of the robot—its direction and speed—and effectively prioritizes each of the processes.  This hierarchy of competition to change ARGO’s motion dictates the external behavior.

Before ARGO may effectively interact in his environment, all of his systems must be calibrated.  The calibration begins after, and every time, the “soft” boot is initiated. Once calibrated the multitasking system is initialized with all of the process present on ARGO.

This system will continually cycle through the two motor control processes, the two servo control processes, the sensor reading, reset control (software reset), lane following, obstacle avoidance, human following (human requires an ir-emitter), light following, and horn control. Once this occurs the command arbiter invokes, according to the environment, different processes to produce varying behaviors.

A schematic of this hierarchy of processes is shown in Figure 1.

CPU

Command Arbiter

Read Sensor

Motor

Control

Servo

Control

Lane

Following

Obstacle

Avoid

Human

Follow

Light

Follow

Horn

Control

Motor

Control

Arbiter

Reset Control

##### Figure 1

### Mobile Platform

The platform upon which ARGO is based is a childhood RC-car. Since my proposal required a sturdy construction and I, lacking mechanical skills and time in which to complete all objectives in addition to the carpentry, resolved to use this car as a foundation for ARGO’s construction.  Although much of the platform would remain unchanged, room was required in which to mount the sensors, the microprocessor, the batteries, and any other switches and accessories (horn) required by my design.

After removing all transmitters used for remote control, I proceeded to construct a wooden sensor “hood.”  Upon this hood I secured the Motorola 68HC11 EVBU board, two battery packs, switches used to control reset and power, horn, and infrared transmitters and receivers.  In addition, the platform lacked any real front bumper.  Since, I would require a bumper to mount most of my sensors—IR, light, and bump—I decided to construct a sturdy bumper from PVC pipe, securing it to my car with screws and a piece of 2-by-4.  Into this pipe I was easily able to mount three IR-receivers, emitters, bump sensors, and lane following sensors.

See Figure 2 for a diagram of the overall platform design.

Original RC-car

Platform

Sensor Hood

PVC-pipe bumper

##### Figure 2

The front bumper construction provided a much needed “hub” upon which I was able to mount most of the sensors necessary for lane following and object avoidance.  After numerous testing, the bumper proved to be as strong as it was useful, able to weather a plethora of high-speed crashes.

Due to the age of the RC-platform—approximately 10 years—and overuse, the suspension had become extremely flexible. The robot would sit .25-inches above the ground, producing many problems with the ir-system—reflection from the floor—and practically eliminating the possibility of mounting light sensors on the bumper.  Thus, I locked the bumper in statum, by filling the shock’s oil-chamber with hot glue.  This worked quite well, providing me with more than 2-inches of clearance from the ground.

Once I performed these modifications to the platform, I was able to develop, nearly unfettered, most of the electrical systems needed to control all of ARGO’s behaviors.

Actuation

Bringing all of ARGO’s behaviors to fruition would require the use of two actuators: a steering servo, and a motor to control the speed.  Control of the servo proved to be extremely easy, nearly problem-free. The motor, however, became one of the greatest hurdles to overcome.

Steering Servo

To steer the robot I used a standard servomechanism.  The position of this servo depends upon the pulse width applied to the input.  A period of 20ms and a pulse width varying from 1ms to 2ms move the servo full left and full right, respectively.  Any degree of movement may be achieved by varying the pulse-width between these two boundaries.

I connected the servo directly to the microprocessor via the output compare port.  Writing code to pulse-width modulate this output compare pin—see appendix for assembly listing---I was able to accurately describe the motion of the servo in any position necessary.

Motor Control

The task of implementing obstacle avoidance and lane following on ARGO would require extremely precise control of the motor speed on the robot.  The analog speed controller used by the RC-car provided only 3 speeds (fast, faster, and 50 mph).  Thus, a complete redesign of that circuit was necessary, to enable me to have digital control of the motor was necessary.

To achieve this goal, I used a high-power MOSFET h-bridge construction.  In addition, since this circuit would require two output compares and two digital outputs to control direction, I designed additional control circuitry enabling me to use one output compare and one direction signal.  I coupled this control logic to the gates of the MOSFETs using opto-isolators.  A complete diagram of this circuit is shown in the appendix.

Accurate control of the motor proved to be one of the most difficult tasks I overcame. That I could not accurately rate the stall current of the motor—the stall current far exceeded any measuring tools at my disposal, I was forced to redesign many motor control circuits that, in retrospect, would have worked had I implemented it with the appropriate MOSFETs. After realizing that the stall current of the high-speed motor I was working with exceeded 100-Amps, I was able to design a circuit that accurately controlled the speed of the motor over a nearly continuous range between full-on and completely-off.

Lessons (Funny?)

One of the numerous lessons I learned while designing this circuit, is that the gate voltage must be equal to or greater than the source voltage.  Failure to heed this characteristic yields fiery results—burns evidence this property.  My original design had biased the gate with only 5V, while biasing the source at 7.2V.  Once I established that this would not work, I redesigned the circuit to use an opto-isolator between the control logic and the gate.  This would step-up the logic level of 5V to the battery voltage of 7.2V.

Although this seems relatively simple, I was forced to use the opto-isolator in two configurations: one for the n-type, and p-type transistors.   I pulled the gate voltage from the emitter of the BJT when biasing the N-type MOSFETs so that I could get 0.00V to turn them fully off.  Otherwise, a logic level—from the inverter—of .02 would translate, through the opto-isolator, to about .6V.  This produced a short to ground, burning up the transistor.

Conversely, to correctly bias the P-type MOSFETs, I needed to pull the gate voltage from the collector—inverting the signal to the diode of the OI-- to get a voltage equal to exactly the battery voltage to turn them completely off. Otherwise, buy the same logic, a short to ground would be produced.

Sensors

To bring ARGO to life he must have the ability to interact with his environment; that is, he needs to have some way of “feeling” his environment.  A robot must be able to read certain data—such as temperature, lighting, imminent obstacles, and color—and interpret this data, allowing the robot to make intelligent decisions based on this data.  ARGO will be no different, and herein I will endeavor to characterize all of ARGO’s current sensor developments, as well as explain any future work in this area.

ARGO uses five infrared transmitters and detectors; mounted on his front bumper and sensor hood, they allow him to detect and to avoid obstacles, and follow an ir-beacon.  In addition, I developed a sensor that will allow ARGO to detect yellow lane markers on the road’s surface.

IR-Sensors

The infrared sensor package includes the following: infrared emitters, emitting infrared light modulated at 40kHz; a SHARP infrared detector, detecting reflected infrared light.  To detect an object the light emitted from the ir-emitters is reflected by a nearby object and detected by the SHARP detectors

Light Sensors

Since ARGO is designed to be a self-driving robot, he must have the capability to detect lane markers and upon detection, follow these lanes.  To accomplish this task I received free samples of a Texas Instruments light-to-voltage converter, theTSL250.  Before modifying this device, I thoroughly tested the sensor.  The results are summarized under Experimental Results.

Realizing that this sensor would not be sufficient for my purpose: lane detection, i.e. black color detection, I sought to implement the sensor in a mechanism that would mimic the eye; that is, focus light that is approximately 6-inches from the robot.  To do this, I fitted a 35mm camera lens to a piece of 40mm PVC pipe, placing the TSL250 just beyond the focal length of the lens.  This would produce a blurred image of anything toward which the lens was directed: a drawing of the design—which I have dubbed the ARGO EYE--is as follows:

The “ARGO EYE”

PVC pipe

35mm lens

TSL250

After constructing this device, I again tested it in the same manner as before; the data tables and graphs (see experimental results) depict these results. Upon comparison of the two characteristics, I realized that the “eye” produced a response that was dependent upon where the lens was focused.  Although the voltage level changed depending upon the color focused upon, changes in the ambient lighting had a dramatic effect on the voltage level, even from one side of the car to the other.  Thus, I was forced to eliminate the effect of the ambient light through the implementation of lights strategically positioned beside each of the lane sensors.  This produces a response that could be used by my program, since the voltage level now remained relatively constant throughout the room.

Bump Sensors

ARGO uses two bump sensors mounted on the front bumper, interconnected with a thin piece of metal to allow bump sensor triggering from a “hit” between the two sensors. I input these two sensors through an OR-gate to port D input.  I did this so that a bump on either of the two sensors would alert the microprocessor that an object had passed through the ir-detection grid.

The circuit used to implement this is trivial: simply a bump sensor connected to power, ground, and port D through a pull-up resistor.

Behaviors

Obstacle Avoidance

Under normal conditions obstacle avoidance would be nearly trivial; however, at the high speeds at which ARGO can travel, obstacle avoidance becomes a daunting task.  To accomplish this task, I implemented a type of differential object avoidance in conjunction with “fuzzy” logic code.

The differential object avoidance code looks for a change in the level of any of the ir-receivers.  When a substantial change is detected the program calculates the difference between the values at the left and right receivers and uses this value to search through a table of predefined ir-differences.  When an entry in the table is greater than or equal to that passed into the subroutine, a match is found.  The location in this table is used as an index into another table of servo control pulse-widths.  The pulse-width is found, and the servo is turned correspondingly.  Since the table contains five left and five right turn directions, I have created fuzzy logic with 10-levels.

In addition to turning the servo, the obstacle avoidance code also checks to be sure that the robot has not come too close to an object.  If this does occur, however, the program will reverse the motor, putting the car into reverse, and backing away from the object.  Then with a slower initial speed, the robot is able to negotiate the object.

Lane-Following

The lane following, due to the primitive nature of the sensor being used, is also slightly rudimentary.  Since the sensor cannot see the lane until the car is directly on top of the lane marker, a full turn away from the lane is required to avoid passing over the lane.  Thus, the code follows: whenever a lane is detected, the car turns full away from it.  While this is rather simple, it works well at slow speeds.

To improve this code I would require another light/voltage converter placed in the middle of the car to sense if the lane had been passed over.  In this manner I could greatly speed up the car without risk out straying far from the lanes. However, time restriction proved too great to permit the ordering of another sensor.

Light Following

Having installed light sensors to detect the lanes, and having seen their extreme sensitivity to light, making the robot move toward a light source was a simple task.  All that this process does, when invoked by the command arbiter, is compare the two voltages levels at each of the lane sensors, and head toward the higher of the two. Although simple, it accomplishes the task.

Human Following

After I had refined my obstacle avoidance code, and increased the current output to the ir-emitters, I had left two obsolete ir-detectors.  I decided rather than take them off, I could use them to implement another behavior: following an ir-beacon held be a human.  The code compares the two ir-receiver voltages, and turns the car toward the higher sensor reading. This direction would be toward the person holding the beacon.

Horn Honking

Since I am striving to model reality with my robot in every way that I can, implementing a horn seemed like the obvious decision.  This process checks the ir-values to see if they pass beyond a certain threshold.   If they do, the program sounds the horn: two short beeps.

Smooth Motor Control

After having written the fuzzy logic for the servo mechanism, reproducing it with some slight modifications was not extremely difficult.  The program, one of those being multitasked, looks for the largest ir-value—it uses a modular subroutine to do this—and uses this to search through a table of predefined ir-levels.  After finding the matching proximity, the program then uses the entry in this table to index into a table of motor speeds.  The motor speed is retrieved and changed by the motor control process. This fuzzy logic system has 5-levels.

Experimental Layout and Results

IR-Testing

The voltage dependency on distance, as indicated by table T.A1, varies linearly.

Through software ingenuity and by boosting the output current to the ir-LEDs, I was able to use these sensors for obstacle avoidance, even at relatively high speeds.

IR Characteristic Table

| Distance(m) | Voltage | Analog Output |
| --- | --- | --- |
| 0.5 | 1.64 | 84 |
| 0.45 | 1.75 | 89 |
| 0.4 | 1.84 | 94 |
| 0.35 | 1.93 | 98 |
| 0.3 | 2.06 | 105 |
| 0.25 | 2.18 | 111 |
| 0.2 | 2.27 | 116 |
| 0.15 | 2.35 | 120 |
| 0.1 | 2.43 | 124 |
| 0.05 | 2.49 | 127 |

Lane-Sensor

My first test of the TSL250 involved the use of a single candle in a dark room.  I then varied the distance of the light source from the sensor linearly and noted the output voltage of the TSL250.

These results are summarized in the following table and graph.

| Distance(m) | Voltage |
| --- | --- |
| 1 | 0.78 |
| 0.95 | 0.83 |
| 0.9 | 1 |
| 0.85 | 1.09 |
| 0.8 | 1.15 |
| 0.75 | 1.25 |
| 0.7 | 1.31 |
| 0.65 | 1.4 |
| 0.6 | 1.5 |
| 0.55 | 1.68 |
| 0.5 | 1.76 |
| 0.45 | 2.05 |
| 0.4 | 2.2 |
| 0.35 | 2.55 |
| 0.3 | 3.25 |
| 0.25 | 3.75 |
| 0.2 | 4 |

The first tests of the ARGO “Eye” yield the following results, as summarized in the following table and plot.

| Distance(m) | Voltage |
| --- | --- |
| 1 | 0.51 |
| 0.95 | 0.55 |
| 0.9 | 0.6 |
| 0.85 | 0.6 |
| 0.8 | 0.63 |
| 0.75 | 0.7 |
| 0.7 | 0.71 |
| 0.65 | 0.75 |
| 0.6 | 0.8 |
| 0.55 | 0.85 |
| 0.5 | 0.91 |
| 0.45 | 1.1 |
| 0.4 | 1.25 |
| 0.35 | 1.5 |
| 0.3 | 1.7 |
| 0.25 | 2.1 |
| 0.2 | 2.45 |
| 0.15 | 2.72 |
| 0.1 | 3.75 |
| 0.05 | 4 |

In this initial data sampling, the lens was tilted forward so that it focused approximately .5 meters in front of the “eye.”  Thus, the response of the eye way negligible until the flame had reached that point.  After this realization, I conducted another test to corroborate this claim.  This time I focused the “eye” on the flame at each distance, and noted the output voltage.

| Distance(m) |  | Voltage |  |
| --- | --- | --- | --- |
| 1 | 1.55 |  |  |
| 0.95 | 2.5 |  |  |
| 0.9 | 3.85 |  |  |
| 0.85 | 4 |  |  |

The “eye” could be made to focus on a light source approximately one meter in front of the car.

Now, that I had tested the light response of the lane-sensor, I needed to determine if the sensor would be able to detect changes in the color, evidencing a lane marker. The following test was made to determine if the “eye” could distinguish color under ambient light.

| Color |  | Distance(m) |  | Voltage |  |
| --- | --- | --- | --- | --- | --- |
| Blue |  | 0.2 |  | 1.53 |  |
| Blue |  | 0.4 |  | 1.6 |  |
| Blue |  | 0.5 |  | 1.7 |  |
| Blue |  | 0.8 |  | 1.65 |  |
| Color | Distance(m) |  | Voltage |  |  |
| White | 0.2 |  | 3.1 |  |  |
| White | 0.4 |  | 2.8 |  |  |
| White | 0.5 |  | 2.75 |  |  |
| White | 0.8 |  | 2.72 |  |  |

Conclusions

Through the construction of ARGO I have learned many things, some of which I would like to forget—the burns from the transistors.  Someone once said, ”Patience is a virtue.” After having completed this project—missing a little shy of my initial goals, I have to agree.  Through the many hours spent debugging the motor driver, to the many hours looking for that missing pound sign in my assembly code, I have learned its significance.

In addition, I have learned the fundamental difference between theory and reality: two worlds juxtaposed, yet not.  It takes a lot of work to bring the two together, but once you have done that, everything seems a lot more clear.

Philosophy leads to science, and so too does this paper.  When I began the semester I intended to make a robot that would follow a yellow lanes and avoid obstacles at the same time.  I have a robot that follows black lanes, but cannot, due to the lack of another light/voltage sensor, make it avoid obstacles at the same time.  I intended to have the motor control circuit finished by the end of the third week.  I finished that circuit at the end of the eighth week, without an analog current meter.

These limitations only served to make me try harder to attain my goals.  These goals I have met, and future goals have become even loftier.  I intend to further my work on this robot, implementing a camera for its vision and a compass assist in destination finding.  Building on a larger scale will enable me to use metal detection—my original, although impractical, lane following design—to follow lanes.  This will eliminate the highly problematic ambient lighting differences, differences that only worsen when the robot is brought outside, where it will eventually be able to function.  I have numerous ideas about that which I wish to do with this robot.  All that I need right now, however, is a break.

Appendix

Code Listing

*Author: John Ferrara

*Title:  Demo1.asm

ADCTL	EQU	$1030

ADR1	EQU	$1031

ADR2    EQU     $1032

ADR3    EQU     $1033

ADR4    EQU     $1034

TCNT    EQU     $100E  ; TCNT High byte

TFLG2   EQU     $1025  ; Contains RTIF flag

TMSK2   EQU     $1024    ; RTII enable flag

PACTL   EQU     $1026  ; RTI Timer control

BAUD    EQU     $102B  ; BAUD rate control register to set the BAUD rate

SCCR1   EQU     $102C  ; Serial Communication Control Register-1

SCCR2   EQU     $102D  ; Serial Communication Control Register-2

SCSR    EQU     $102E  ; Serial Communication Status Register

SCDR    EQU     $102F  ; Serial Communication Data Register

TOC2    EQU    $1018

TOC3    EQU     $101A

PORTA   EQU     $1000

PORTD   EQU     $1008

DDRD    EQU     $1000

TCTL1      EQU     $1020

TMSK1	EQU	$1022

TFLG1	EQU	$1023

OPTION	EQU	$1039

BASE	EQU	$1000

BIT76   EQU     %11000000

BIT54	EQU	%00110000

BIT7    EQU     %10000000

BIT6    EQU     %01000000

BIT5	EQU	%00100000

BIT4	EQU	%00010000

BIT3    EQU     %00001000

BIT1    EQU    %00000010

BIT2    EQU     %00000100

INV5	EQU	%11011111

EOS     EQU     $04    ; User-defined End Of String (EOS) character

CR      EQU     $0D    ; Carriage Return Character

LF      EQU     $0A    ; Line Feed Character

ESC     EQU     $1B    ; Escape Charracter

*

************************************************************************

* Initialize Interrupt Jump Vectors

************************************************************************

*

*RTI-INTERRUPT: USED FOR MULTITASKING

ORG      $00EB

JMP      RTI_ISR

*OC2-INTERRUPT: USED FOR SERVO CONTROL

ORG     $00DC

JMP     OC2_ISR

*OC3-INTERRUPT: USED FOR MOTOR SPEED CONTROL

ORG     $00D9

JMP     OC3_ISR

*IRQ-INTERRUPT: USED FOR THE "SOFT" RESET OF THE EVBU

ORG     $00EE

JMP     IRQ_ISR

*

************************************************************************

* Define Strings and Reserve Variable memory space for system use

*   such as CPT, DSPT, CurPID, etc.

********************************************************************

ORG $8000

*MULTITASKING VARIABLES***********************************************************

CPT	RMB 2

RMB 2

RMB 2

RMB 2

RMB 2

RMB 2

RMB 2

RMB 2

RMB 2

RMB 2

DSPT

FDB $80FF

FDB $81FF

FDB $82FF

FDB $83FF

FDB $84FF

FDB $85FF

FDB $86FF

FDB $87FF

FDB $88FF

FDB $89FF

MaxProc FCB $09

CurrPID	RMB 1

temp    FCB $00

TEMP2	RMB 2

STACK	RMB 2

*MOTOR-CONTROL VARIABLES***********************************************************

OC3Period       FDB     10000   ;20ms period for the servo control pulses

OC3HIGH         RMB     2

OC3LOW          RMB     2

OC3_CHANGE	RMB	2

SPEED_BOOST     FDB     6000

DIRECTION	RMB	1

DIR_PREV        RMB     1

*SERVO AND OBSTACLE AVOIDANCE VARIABLES***********************************************

OC2Period	FDB	40000	;20ms period for the servo control pulses

OC2HIGH		RMB	2

OC2LOW		RMB	2

OC2_CHANGE	RMB	2

IR_TABLE

RF_IR		RMB	1

LF_IR		RMB	1

MF_IR           RMB     1

TRF_IR		RMB	1

TLF_IR		RMB	1

CURR_MAX	RMB	1

MAX_IR		RMB	1

RF_INIT_IR	RMB	1

LF_INIT_IR      RMB     1

MF_INIT_IR      RMB     1

TRF_INIT_IR	RMB	1

TLF_INIT_IR	RMB	1

RF_OLD          RMB     1

LF_OLD          RMB     1

*FUZZY LOGIC TABLES***********************************************

T_INDEX         RMB     1

TURN_FOUND      RMB     2

DIFF_IR         FCB     4

FCB     7

FCB     13

FCB     20

TURN_SMOOTH     FDB     200

FDB     300

FDB     500

FDB     600

DIFF_SPEED	FCB	85

FCB	95

FCB	105

FCB	115

SMOOTH_SPEED

FDB	4500

FDB	3500

FDB	2700

FDB	2000

*cOMMAND ARBITRATOR VARIABLES***********************************************

EXEC_HORN	RMB	1

EXEC_AVOID 	RMB	1

EXEC_LANE	RMB	1

*LANE FOLLOWING SENSORS***********************************************

RIGHT_LIGHT     RMB     1

LEFT_LIGHT      RMB     1

LEFT_INIT_L     RMB     1

RIGHT_INIT_L    RMB     1

RIGHT           RMB     1

LEFT            RMB     1

RL_OLD		RMB	1

LL_OLD		RMB	1

*RESET VARIABLES:"SOFT" REBOOT***********************************************

STARTUP         RMB     1

****************************************************************************

****************************************************************************

ORG     $9000

SOFT_RESET

LDS     #$41  ;Initialize Stack Pointer

LDAA	#0

STAA	STARTUP

JSR     INIT_MULTITASK

TYS

JSR     InitRTI

JSR     INIT_DIRECTION

JSR     INIT_OC

CLI

BRA	WAIT_START

STOP_RESET

LDS	#$41

WAIT_START

LDAA	STARTUP

CMPA	#1

BNE	WAIT_START

Main    LDS     #$0041  ;Initialize Stack Pointer

SEI

JSR     INIT_ANALOG

JSR	INIT_PORTS

JSR     INIT_IR

JSR	INIT_SPEED

JSR     INIT_DIRECTION

JSR     INIT_LIGHTS

JSR     INIT_MULTITASK

TYS     ;initialize stack pointer

LDX     #CONTROL_RESTART

JSR     Spawn

LDX     #READ_SENSORS

JSR     Spawn

LDX     #ARBITRATE_MOTOR

JSR     Spawn

LDX     #CONTROL_MOTOR

JSR     Spawn

LDX     #CONTROL_SERVO

JSR     Spawn

LDX     #AVOID_OBSTACLES

JSR     Spawn

LDX	#HEINOUS_NOISE

JSR	Spawn

LDX     #FOLLOW_LANE

JSR     Spawn

CLI

Command_Arbit

LDAA    #0

STAA    EXEC_AVOID

STAA    EXEC_HORN

LDAA    #1

STAA    EXEC_LANE

LDD     #2000

STD     OC3_CHANGE

JSR     Delay

R_1

LDAA    RIGHT_LIGHT

ANDA    #$80

BNE     R_1

LDAA    LEFT_LIGHT

ANDA    #$80

BNE     R_1

LDAA    #1

STAA    EXEC_AVOID

STAA    EXEC_HORN

LDAA    #0

STAA    EXEC_LANE

LDD     #2200

STD     OC3_CHANGE

JSR     Delay

R_2

LDAA    RIGHT_LIGHT

ANDA    #$80

BNE     R_2

LDAA    LEFT_LIGHT

ANDA    #$80

BNE     R_2

LDAA    #0

STAA    EXEC_AVOID

STAA    EXEC_HORN

LDAA    #1

STAA    EXEC_LANE

E_DEMO

LDAA    RIGHT_LIGHT

ANDA    #$80

BNE     E_DEMO

LDAA    LEFT_LIGHT

ANDA    #$80

BNE     E_DEMO

SWI

********************************************************************

********************************************************************

*MULTITASKING PROCESSES***************************************

********************************************************************

********************************************************************

*SUBROUTINE: CONTROL_RESTART

*PURPOSE: CONTROLS THE SOFTWARE RESTART OF THE PROCESSOR

*********************************************************************

CONTROL_RESTART

LDAA	STARTUP

CMPA	#1

BEQ	END_RESTART

JSR     INIT_MULTITASK

JMP	STOP_RESET

END_RESTART

BRA	CONTROL_RESTART

********************************************************************

*SUBROUTINE: READ_SENSORS

*PURPOSE: READS THE ANALOG PORTS CONTAINING IR_SENSORS

*********************************************************************

READ_SENSORS

LDAA    #$30	;FIRST, READ THE FIRST FOUR ANALOG PORTS(ALL IR)

STAA	ADCTL

WAIT_SENS

LDAA	ADCTL	;WAIT FOR 4-CONVERSIONS TO COMPLETE

ANDA	#BIT7

BEQ	WAIT_SENS

LDAA    #$30    ;CLEAR THE CONVERSIONS FLAG

STAA	ADCTL

LDAA    RF_IR

STAA    RF_OLD

LDAA    ADR1    ;GET THE SENSOR READINGS AND PUT INTO GLOBAL

STAA	RF_IR	;VARIABLES

LDAA    LF_IR

STAA    LF_OLD

LDAA    ADR2

STAA	LF_IR

LDAA    ADR3

STAA    MF_IR

LDAA	ADR4

STAA	TRF_IR

LDAA	#$34

STAA	ADCTL

*NOW, GET THE NEXT FOUR ANALOG PORT READINGS

WAIT_SENS2

LDAA	ADCTL	;WAIT FOR 4-CONVERSIONS TO COMPLETE

ANDA	#BIT7

BEQ	WAIT_SENS2

LDAA    #$34    ;CLEAR THE CONVERSIONS FLAG

STAA	ADCTL

LDAA    ADR1    ;GET THE SENSOR READINGS AND PUT INTO GLOBAL

STAA	TLF_IR	;VARIABLES

LDAA    ADR2

LDAB	RIGHT_LIGHT

STAB	RL_OLD

STAA	RIGHT_LIGHT

LDAA    ADR3

LDAB	LEFT_LIGHT

STAB	LL_OLD

STAA    LEFT_LIGHT

JMP	READ_SENSORS

********************************************************************

*SUBROUTINE: FOLLOW_LANE

*PURPOSE: FOLLOWS THE YELLOW LANE

*********************************************************************

FOLLOW_LANE

LDAA    EXEC_LANE

CMPA    #1

BNE     FOLLOW_LANE

LDAA    RIGHT_LIGHT

SUBA    LEFT_LIGHT

CMPA    #7

BGT     LEFT_LANE

LDAA    LEFT_LIGHT

SUBA    RIGHT_LIGHT

CMPA    #7

BGT     RIGHT_LANE

BRA     BETWEEN_LANES

RIGHT_LANE

LDD     #3600

STD     OC2_CHANGE

JMP     FOLLOW_LANE

LEFT_LANE

LDD     #2400

STD     OC2_CHANGE

JMP     FOLLOW_LANE

BETWEEN_LANES

LDD     #3000

STD     OC2_CHANGE

JMP     FOLLOW_LANE

********************************************************************

*SUBROUTINE: FOLLOW_HUMAN

*PURPOSE: FOLLOWS A HUMAN WITH AN IR EMITTER

*********************************************************************

FOLLOW_HUMAN

LDAA    TRF_IR

CMPA    TLF_IR

BGT     HUMAN_RIGHT

HUMAN_LEFT

LDD     #3400

STD     OC2_CHANGE

BRA     FOLLOW_HUMAN

HUMAN_RIGHT

LDD     #2600

STD     OC2_CHANGE

BRA     FOLLOW_HUMAN

********************************************************************

*SUBROUTINE: FIND_LIGHT

*PURPOSE: MOVES INTO THE LIGHT, CARROL-ANNE

*********************************************************************

FIND_LIGHT

LDAA    RIGHT_LIGHT

LSRA

LDAB    LEFT_LIGHT

LSRB

SBA

BGT     LIGHT_RIGHT

LIGHT_LEFT

LDD     #3400

STD     OC2_CHANGE

BRA     END_LIGHT

LIGHT_RIGHT

LDD     #2600

STD     OC2_CHANGE

END_LIGHT

JMP     FIND_LIGHT

********************************************************************

*SUBROUTINE: AVOID_OBSTACLES

*PURPOSE: AVOIDS ONCOMING CARS

*********************************************************************

AVOID_OBSTACLES

LDAA    EXEC_AVOID

CMPA    #1

BNE     AVOID_OBSTACLES

LDAA    PORTD

ANDA    #BIT3

BNE     GO_REVERSE_LI

LDAA	RF_IR

CMPA    #125

BGT     GO_REVERSE

LDAA    LF_IR

CMPA    #125

BGT     GO_REVERSE

LDAA    MF_IR

CMPA    #125

BGT     GO_REVERSE

LDAA    #0

STAA    DIRECTION

LDAA    RF_IR

CMPA    RF_OLD

BGT     NEED_TURN

LDAA    LF_IR

CMPA    LF_OLD

BGT     NEED_TURN

LDAA    RF_IR

CMPA    RF_INIT_IR + 5

BGT     NEED_TURN

LDAA    LF_IR

CMPA    LF_INIT_IR + 5

BGT     NEED_TURN

BRA     STRAIGHT

NEED_TURN

LDD     #2500

STD     OC3_CHANGE

LDAA    RF_IR

CMPA    LF_IR

BGT     TURN_LEFT

LDAA    LF_IR

CMPA    RF_IR + 3

BGT     TURN_RIGHT

JMP     AVOID_OBSTACLES

GO_REVERSE

LDAA    #1

STAA    DIRECTION

LDAA    RF_IR

CMPA    LF_IR

BGT     REV_LEFT

REV_RIGHT

LDD     #3300

STD     OC3_CHANGE

BRA     END_REVERSE

REV_LEFT

LDD     #2700

STD     OC2_CHANGE

END_REVERSE

JSR     HONK_DELAY

JMP     AVOID_OBSTACLES

STRAIGHT

LDD     #3500

STD     OC3_CHANGE

LDD	#3000

STD	OC2_CHANGE

JMP     END_AVOID

TURN_LEFT

LDAA    RF_IR

SUBA    LF_IR

JSR     SEARCH_TABLE_IR

LDD     #3000

SUBD    TURN_FOUND

STD     OC2_CHANGE

BRA     END_AVOID

TURN_RIGHT

LDAA    LF_IR

SUBA    RF_IR

JSR     SEARCH_TABLE_IR

LDD     #3000

ADDD    TURN_FOUND

STD     OC2_CHANGE

BRA     END_AVOID

END_AVOID

JMP	AVOID_OBSTACLES

********************************************************************

*SUBROUTINE: HEINOUS_NOISE

*PURPOSE: HONKS THE CAR HORN

*********************************************************************

HEINOUS_NOISE

LDAA    EXEC_HORN

CMPA    #1

BNE     HEINOUS_NOISE

LDAA    RF_IR

CMPA    #100

BGT     HONK

LDAA    LF_IR

CMPA    #100

BGT     HONK

BRA     HEINOUS_NOISE

HONK

LDAA    #$F1

STAA    $7000

JSR     HONK_DELAY

LDAA    #$F0

STAA    $7000

JSR     HONK_DELAY

LDAA    #$F1

STAA    $7000

JSR     HONK_DELAY

LDAA    #$F0

STAA    $7000

JMP     HEINOUS_NOISE

********************************************************************

*SUBROUTINE: CONTROL_MOTOR

*PURPOSE: aDJUSTS THE SPEED OF THE MOTOR

*********************************************************************

CONTROL_MOTOR

LDD	OC3_CHANGE

STD	OC3HIGH

LDD	OC3Period

SUBD	OC3HIGH

STD	OC3LOW

LDAA	DIRECTION

CMPA	#0

BEQ	FORWARD

REVERSE

LDAA	PORTD

ORAA	#BIT2

STAA	PORTD

BRA     END_MOTORCONTROL

FORWARD

LDAA	PORTD

ANDA	#$FB

STAA	PORTD

END_MOTORCONTROL

JMP	CONTROL_MOTOR

********************************************************************

*SUBROUTINE: ARBITRATE_MOTOR

*PURPOSE: CONTROLS THE TRANSITION SPEED OF THE MOTOR

*********************************************************************

ARBITRATE_MOTOR

LDAA	EXEC_LANE

CMPA	#1

BNE	CHANGE_SPEED

LDD	#2000

STD	OC3_CHANGE

BRA	ARBITRATE_MOTOR

CHANGE_SPEED

LDAA	DIRECTION

CMPA	DIR_PREV

BEQ	SAME_DIR

LDD	SPEED_BOOST

STD	OC3_CHANGE

JSR	HONK_DELAY

SAME_DIR

LDAA    DIRECTION

STAA	DIR_PREV

LDAA	#3

LDY	#MAX_IR

LDX	#IR_TABLE

JSR	FIND_MAX

LDAA	MAX_IR

LDX	#DIFF_SPEED

LDY	#SMOOTH_SPEED

JSR     SEARCH_TABLE_MOT

CMPA	#1

BNE	SLOWEST_SPEED

STX	OC3_CHANGE

BRA	END_ARBMOT

SLOWEST_SPEED

LDX	#1900

STX	OC3_CHANGE

END_ARBMOT

JMP	ARBITRATE_MOTOR

********************************************************************

*SUBROUTINE: CONTROL_SERVO

*PURPOSE:ADJUSTS THE POSITION OF THE SERVO

*********************************************************************

CONTROL_SERVO

LDD	OC2_CHANGE

STD	OC2HIGH

LDD	OC2Period

SUBD	OC2HIGH

STD	OC2LOW

JMP	CONTROL_SERVO

******************************INITIALIZATION CODES*******************************8

********************************************************************

*SUBROUTINE: INIT_MULTITASK

*PURPOSE: INITIALIZES THE CPT AND STACK POINTERS

*********************************************************************

INIT_MULTITASK

LDAB    #$00

LDX     #CPT

LDAA    #$0A

Zero    LDY     #$0     ;this code zeros-out the CPT(table)

STY	0,X

INX

INX

INCB

CBA

BNE	Zero

LDAA	#$00	;initialize the starting PID

STAA	CurrPID

LDX	#DSPT

LDY	0,X

LDX     #CPT	;setup the initial stack pointer for the first

STY     0,X	;...process

INY

LDAA    #BIT6	;clear the RTI-flag so the next process is not

STAA    TFLG2	;immediately interruted

RTS

********************************************************************

*SUBROUTINE: SET_ANALOG

*PURPOSE: CONFIGURES THE A/D SYSTEM FOR LATER USE

*********************************************************************

INIT_ANALOG

LDAA	#$80	;POWER-UP A/D-SYSTEM

STAA	OPTION

LDAA	#40

WAIT_AN

DECA            ;WAIT FOR CHARGE PUMP TO STABILIZE

BNE     WAIT_AN

RTS

********************************************************************

*SUBROUTINE: INIT_IR

*PURPOSE: SETS-UP INITIAL IR CONDITIONS (SELF-CALIBRATION)

*********************************************************************

INIT_IR

LDAA    #$30

STAA	ADCTL

WAIT_IR	LDAA	ADCTL	;WAIT FOR 4-CONVERSIONS TO COMPLETE

ANDA	#BIT7

BEQ	WAIT_IR

LDAA    #$30    ;CLEAR THE CONVERSIONS FLAG

STAA	ADCTL

LDAA    ADR1    ;GET THE SENSOR READINGS AND PUT INTO GLOBAL

STAA	RF_INIT_IR	;VARIABLES

LDAA    ADR2

STAA	LF_INIT_IR

LDAA    ADR3

STAA    MF_INIT_IR

LDAA	ADR4

STAA	TRF_INIT_IR

LDAA    #$34

STAA	ADCTL

WAIT_IR2

LDAA	ADCTL	;WAIT FOR 4-CONVERSIONS TO COMPLETE

ANDA	#BIT7

BEQ	WAIT_IR2

LDAA    #$34    ;CLEAR THE CONVERSIONS FLAG

STAA	ADCTL

LDAA    ADR1    ;GET THE SENSOR READINGS AND PUT INTO GLOBAL

STAA	TLF_INIT_IR	;VARIABLES

RTS

INIT_LIGHTS

LDAA    #$34

STAA	ADCTL

WAIT_LTS

LDAA	ADCTL	;WAIT FOR 4-CONVERSIONS TO COMPLETE

ANDA	#BIT7

BEQ     WAIT_LTS

LDAA    #$34    ;CLEAR THE CONVERSIONS FLAG

STAA	ADCTL

LDAA    ADR2    ;GET THE SENSOR READINGS AND PUT INTO GLOBAL

STAA    RIGHT_INIT_L     ;VARIABLES

LDAA    ADR3

STAA    LEFT_INIT_L

RTS

********************************************************************

*SUBROUTINE: INIT_PORTS

*PURPOSE: INITIALIZES ALL PORTS TO BE EITHER INPUTS OR OUTPUTS

*********************************************************************

INIT_PORTS

LDAA    #$F7

STAA    DDRD

LDAA    #$F0

STAA    $7000

LDAA    #0

STAA    PORTA

RTS

********************************************************************

*SUBROUTINE: INIT_SPEED

*PURPOSE: STARTS THE CAR AT AN INITAL SLOW SPEED

*********************************************************************

INIT_SPEED

LDD     #3200

STD     OC3HIGH

STD     OC3_CHANGE

LDD     OC3Period

SUBD    OC3HIGH

STD     OC3LOW

RTS

********************************************************************

*SUBROUTINE: INIT_DIRECTION

*PURPOSE: STARTS THE CAR OUT IN FORWARD GOING STRAIGHT

*********************************************************************

INIT_DIRECTION

LDAA	PORTD

ANDA	#$FB

STAA	PORTD

LDAA	#0

STAA	DIRECTION

LDAA    #1

STAA    DIR_PREV

LDD     #3000   ;START WITH SERVOS STRAIGHT

STD	OC2HIGH

STD     OC2_CHANGE

LDD	OC2Period

SUBD	OC2HIGH

STD	OC2LOW

RTS

********************************************************************

* Subroutine:  InitRTI

* Function:    This routine enables RTIs and sets the RTI rate to

*              32.77ms.

* Input:       None

* Output:      Initializes RTI

********************************************************************

InitRTI LDAA    #$88    ;set the interrupt rate to 32.77ms

STAA    PACTL

LDAA	#$40

STAA	TMSK2

RTS		;return to the main

********************************************************************

*SUBROUTINE: INIT_OC

*PURPOSE: INITIALIZES ALL OUTPUT COMPARES

*********************************************************************

INIT_OC

LDAA    #%10100000   ;CLEAR OC2 AND 0C3 LINES TO ZERO

STAA    TCTL1

LDAA    #%01100000   ;ENABLE OC2 AND OC3 INTERRUPT

STAA    TMSK1

RTS

*********************************INTERRUPT SERVICE ROUTINES**************************8

********************************************************************

*SUBROUTINE: OC2_ISR

*PURPOSE: CONTROLS THE OUTPUT COMPARE 2

*********************************************************************

OC2_ISR

LDAA    TFLG1   ;CHECK FOR LEGAL INTERRUPT

ANDA    #BIT6

BEQ     END_OC2

LDAA    #BIT6   ;CLEAR THE INTERRUPT

STAA    TFLG1

LDAA    TCTL1   ;CHECK IF LAST PULSE WAS HIGH OR LOW

ANDA    #BIT6

BEQ     LASTHIGH_OC2

LDAA    TCTL1    ;SET NEXT PULSE TO BE LOW

EORA    #$40

STAA    TCTL1

LDD     TOC2 ;SET OC2 HIGH TIME

ADDD    OC2HIGH

STD     TOC2

BRA     END_OC2

LASTHIGH_OC2

LDAA    TCTL1    ;SET THE NEXT PULSE TO BE HIGH

EORA    #$40

STAA    TCTL1

LDD     TOC2

ADDD    OC2LOW

STD     TOC2

END_OC2

RTI            ;RETURN FROM INTERRUPT

********************************************************************

*SUBROUTINE: OC3_ISR

*PURPOSE:CONTROLS OUTPUT COMPARE 3

*********************************************************************

OC3_ISR

LDAA    TFLG1   ;CHECK FOR LEGAL INTERRUPT

ANDA    #BIT5

BEQ     END_OC3

LDAA    #BIT5   ;CLEAR THE INTERRUPT

STAA    TFLG1

LDAA    TCTL1   ;CHECK IF LAST PULSE WAS HIGH OR LOW

ANDA    #BIT4

BEQ     LASTHIGH_OC3

LDAA    TCTL1    ;SET NEXT PULSE TO BE LOW

EORA    #$10

STAA    TCTL1

LDD     TOC3 ;SET OC2 HIGH TIME

ADDD    OC3HIGH

STD     TOC3

BRA     END_OC3

LASTHIGH_OC3

LDAA    TCTL1    ;SET THE NEXT PULSE TO BE HIGH

EORA    #$10

STAA    TCTL1

LDD     TOC3

ADDD    OC3LOW

STD     TOC3

END_OC3

RTI            ;RETURN FROM INTERRUPT

********************************************************************

*SUBROUTINE: IRQ_ISR

*PURPOSE:USED FOR THE REBOOT PROCESS

*********************************************************************

IRQ_ISR

LDAA	STARTUP

CMPA	#0

BEQ	GO_SYSTEMS

OFF_SYSTEMS

LDAA	#0

STAA	STARTUP

BRA     END_IRQ

GO_SYSTEMS

LDAA	#1

STAA	STARTUP

END_IRQ

RTI

*

********************************************************************

* Interrupt Service Routine (ISR): RTI_ISR

* Function:  This ISR services the Real-Time Interrupts.

* This ISR should do the followings:

*  - Clear RTI flag.

*  - Update current SP in CPT[Current PID]

*  - Find next PID

*  - Update CurPID

*  - Load New SP from CPT[Next PID]

********************************************************************

RTI_ISR

LDX     #BASE

LDAA    TFLG2

ANDA    #BIT6

BEQ     END_ISR	;check for valid interrupt

LDX	#CPT	;store the current stack pointer to the appropriate

LDAA    CurrPID	;entry in the CPT, as dictated by CurrPID

ASLA		;to get the offset into the table must mult.

STAA    temp	;CurrPID by 2

LDAB    temp

ABX

TSY

DEY

STY     0,X       ;save the current stack pointer of the current process

LDAA    CurrPID

INCA              ;...to the appropriate slot in the CPT

*                         ;check to see if the process is the last one

FndNxt  CMPA    #$0A      ;is so the next process to be run is process[0]

BEQ	REFRESH

LDX     #CPT	;determine the next nonzero entry in the CPT

STAA    temp	;...this is the stack pointer we need to use

ASL     temp	;for the next process

LDAB    temp

ABX

INCA

LDY     0,X       ;I must skip zeroed entried here, since my KILL

BEQ	FndNxt	  ;does not account for them

BRA	SET

REFRESH LDX     #CPT

LDY     0,X        ;restart the process poll

LDAA    #$0

STAA    CurrPID

INY

TYS

BRA     RT_ISR

SET	INY

TYS

DECA                ;transfer the correct processes SP[Proc.] to the SP

STAA    CurrPID

RT_ISR  LDAA    #BIT6	;clear the RTI-flag so the next process is not

STAA    TFLG2	;immediately interruted

END_ISR RTI

********************************************************************

********************************************************************

*SUBROUTINES***************************************

********************************************************************

********************************************************************

*SUBROUTINE: SEARCH_TABLE

*PURPOSE: FINDS THE APPROPRIATE SERVO DIRECTION FOR HIGH-SPEED

*OBSTACLE AVOIDANCE

*INPUT: THE TABLE TO SEARCH IN THE X-REGISTER

*	THE TABLE WITH THE CORRESPONDING ENTRY SEARCHED FOR

*	IN ACCUMULATOR A IS THE VALUE TO MATCH IN THE TABLES

*********************************************************************

SEARCH_TABLE_IR

LDAB    #0

STAB    T_INDEX

LDX     #DIFF_IR

ST_LOOP

LDAB    0,X

CBA

BLT     FOUND_ENTRY

INX

LDAB     T_INDEX

INCB

STAB    T_INDEX

CMPB    #4

BLT     ST_LOOP

LDD     #700

STD     TURN_FOUND

BRA     END_SEARCH

FOUND_ENTRY

LDAB    T_INDEX

ASLB

LDX     #TURN_SMOOTH

ABX

LDD     0,X

END_SEARCH

STD     TURN_FOUND

RTS

SEARCH_TABLE_MOT

*

********************************************************************

* Subroutine: Spawn

* Function:   generates a new process

* Input:      X: starting address of the process

* Output:     A: PID of the process just created, or

*                $FF if no slots are available

* Destroys:   Contents of A register

* Side effects: Creates the initial stack for the process.  This

*               stack must have the process PID in A, and %01000000

*               in CCR.

********************************************************************

Spawn

PSHY		;put these on the stack so they are not destroyed

PSHB

STX	TEMP2

LDAA	#$00	;intialize the CPT position=0

LDY	#CPT

FndPlc  LDX     0,Y     ;find the next empty spot in the CPT

INY             ;POINT TO NEXT ENTRY IN THE TABLE

INY

INCA

CMPA    #11     ;if A = 9 then the table has been completely

BEQ     FULL	;scanned and there are no zero entries

CPX     #$0

BNE     FndPlc

DECA

STAA	temp	;store the displacement into the table in temp

ASL     temp    ;multiply temp by 2=>index into table(CPT)

LDAB    temp

LDX     #DSPT   ;get default stack table

ABX

LDY     0,X  ;get appropriate default stack pointer

LDX     #CPT    ;put into the appropriate CPT slot

ABX

STY     0,X

STAA    temp

LDY     0,X        ;initialize the stack for the new process

LDX	TEMP2

DEY

STX     0,Y

DEY

DEY

LDX     #$0

STX     0,Y

DEY

DEY

LDX     #$0

STX     0,Y

DEY

LDAA    temp

STAA    0,Y

DEY

LDAB    #$0

STAB    0,Y

DEY

LDAB    #$40

STAB    0,Y

DEY        		;pulled out DEY here

ASL     temp

LDAB    temp

LDX     #CPT

ABX

STY     0,X

PULB		;restore variables used for this subroutine

PULY

BRA     RT_RTS

FULL 	PULB

PULY

LDAA    #$FF

RT_RTS  RTS             ; return from subroutine

********************************************************************

* Subroutine: Kill

* Function:   removes a currently active process

* Input:      A: process ID to kill

* Output:     A: process ID just killed

* Destroys:   None

********************************************************************

Kill    PSHB

STAA    temp

ASL     temp

LDAB    temp

PSHX		;SAVE THESE REGISTERS SOP THEY ARE NOT CHANGED

PSHY

LDY	#$0	;zero-out the corresponding entry of the table

LDX	#CPT

ABX

STY     0,X

PULY            ;restor the registers used during the subroutine

PULX

PULB

RTS		;return from subroutine

*

********************************************************************

*

* Subroutine:  Delay

* Input:       None

* Output:      Provides a delay by simple looping

* Destroys:    None

* Note:        If you're looking to save memory, this function

*              may be rewritten or subsumed by Process since

*              Process is the only routine to call it.

********************************************************************

*

Delay   PSHA             ;

PSHB             ;

PSHX             ; Save registers

PSHY             ;

LDX    #10    ; Load outer loop counter

Outer   LDY    #10000    ; Load inner loop counter

Inner

DEY              ; Decrement inner counter

BNE    Inner     ; Branch if >0 to inner loop

DEX              ; Decrement outer counter

BNE    Outer     ; Branch if >0 to outer loop

PULY             ; Restore registers

PULX             ;

PULB             ;

PULA             ;

RTS              ; Return from subroutine

HONK_DELAY

LDX     #20000

H_LOOP

DEX

CPX     #0

BNE     H_LOOP

RTS

********************************************************************

*

* Subroutine:  FIND_MAX

* Input:       ARRAY INDEX:X-REGISTER; ARRAY SIZE: A ACCUMULATOR;Y:WHERE TO STORE THE MAX

* Output:      FINDS MAXIMUM OF A GIVEN ARRAY; PASSED IN THE X-REGISTER

* Destroys:    None

********************************************************************

*

FIND_MAX

LDAB	0,X

STAB	CURR_MAX

DECA

MAX_LOOP

LDAB	0,X

CMPB	CURR_MAX

BLT	SAME_MAX

STAB	CURR_MAX

SAME_MAX

INX

DECA

CMPA	#0

BNE	MAX_LOOP

LDAB	CURR_MAX

STAB	0,Y

RTS