RoboSpeedy

EEL 5666 Final Technical Report

Author:

Steve Kerslake

Submitted in Partial Fulfillment of the Requirements

for EEL 5666 Robotics Course

Date

03 August 1999

# Abstract

This paper concerns the development and testing of an autonomous robot.  This robot is better known as RoboSpeedy.  It was designed for high-speed navigation of large indoor spaces.  The goal was a robot that could navigate a course in the same manner as a remotely controlled racecar. The central platform of RoboSpeedy is a Tamaya remote-controlled racecar chassis.  This platform was expanded to allow the mounting of various sensors and electronics.  RoboSpeedy has an Intel 8051GB micro-controller and 32K of SRAM.  The micro-controller receives input from sonar, a flux-gate compass, and hacked Sharp infrared sensors.  It uses these inputs to control the steering servo, the sonar turret servo, the motor control relays, and the motor itself.

# Executive Summary

RoboSpeedy was designed for high-speed navigation of large indoor spaces.  The goal was a robot that could navigate a course in the same manner as a remotely controlled racecar. The central platform of RoboSpeedy is a Tamaya remote-controlled racecar chassis.  The bodywork was designed on AutoCAD and milled out in the IMDL using the T-Tech.  I mounted the wooden body panels to the plastic chassis using #10 all thread.  The wooden platform was completely immobile with respect to the plastic chassis.  The entire wooden bodywork was reinforced with steel angle brackets.  I wanted to make sure that RoboSpeedy was as crashproof as possible.  This platform provided mounting space and protection for the various sensors and electronics that RoboSpeedy used.

RoboSpeedy has an Intel 8051GB micro-controller and 32K of SRAM.  These are installed on a PCB manufactured by Tecny Electronics.  This board also allows the mounting of an LM7805 voltage regulator and a MAX232.  The micro-controller receives input from sonar, a flux-gate compass, and hacked Sharp infrared sensors.  The Polaroid sonar was purchased from Mekatronics.  This sonar has a range of fifteen feet and a simple continuous DC output.  The IR sensors were also purchased from Mekatronics.  These sensors are operated in the normal IMDL analog fashion.  The compass is a Precision Navigation flux-gate magnetometer with a digital output.

The micro-controller uses these inputs to control the steering servo, the sonar turret servo, the motor control relays, and the motor itself.  All of the motors and relays are optically isolated from the micro-controller.  The opto-isolators will prevent the processor from being reset in the event of low battery power.   The servos and relays operate from an eight-pack of AA batteries; the motor runs from a 7.2-volt DC battery pack with the same ground as the servos.  Finally the micro-controller has its own circuit and battery pack.

# Introduction

For those of you who were absent during the abstract and the executive summary this paper concerns the development and testing of an autonomous robot.  This robot is better known as RoboSpeedy.  It was designed for high-speed navigation of large indoor spaces.  The goal was a robot that could navigate a course in the same manner as a remotely controlled racecar.  This task is difficult only because the sensors that are available for our robots have a short range and a low resolution.  I propose that a “steerable” sonar and a flux-gate compass could be used to give an autonomous agent the ability to operate at high speeds.

The Intel 8051GB provides RoboSpeedy’s “reflexes.”  I initially decided to use the Intel processor because it has a 22MHz crystal.  I didn’t realize until after I received it that it divided the crystal speed by twelve internally.  This, however, did not prove to be a handicap.  This is because the 8051 instructions typically only require one or two machine cycles to execute.  In the end, the 8051GB turned out to be an awesome robotics micro-controller.  This is due to the large number of hardware controlled features that allow the programmer to set up the operation and then forget about it.  These features include the Programmable Counter Array, the A/D converter, and the self-resetting interrupt system.  The problems that I had with the 8051GB were that I had no software pre-written for me and I had no information resources available to me.  This meant that I had to spend valuable time researching and learning how to program the 8051GB.

# Integrated System

Note:  RoboSpeedy is physically complete.  Unfortunately, I have not had time to implement the programming in the way that I wanted to.  The time factor has been further aggravated by the fact that I have had to do it all in assembler.  I have written code to test all of the robot’s systems.  They are all powered up and work together.  The only software that is incomplete is the final version of the navigation software.  I have written a simple version of this software that uses the IR sensors and the sonar.  This version will not pan the sonar and it does not accept input from the compass (although the compass is functional and the sonar does pan).

RoboSpeedy is designed to look for the longest available path and follow that path until a more optimal path becomes available.  It does this by panning the sonar until it locates the best path.  It then compares heading of the best path to the actual heading of the robot.  If these paths do not match it will change direction until they do.  If, for example, sonar detects an optimal path at left 90 and its current heading is 270.  The robot will determine that left 90 is at 180 by subtracting 90 from 270.  It will then turn to the left until it reaches 180.  This heading matching also makes sure that the robot travels in a straight line.  If it veers from the desired course the compass will detect this deviation and correct for it.

The distance available on the best path governs the robot’s speed. Obviously, a longer path will dictate a higher speed than a short one.  Placing the motor in reverse implements braking.  This is touchy because this can cause the robot to swap ends (rapidly rotate 180 degrees).  Due to the acceleration properties of the motor, optimal speed control will only be gained through extensive testing.

Find

the best

heading

Go

in that

direction

Accelerate

to full

speed

Is the path clear?

Is the path

half clear?

Is the path

a quarter

clear?

Is the path

blocked?

Accelerate

to half

speed

Accelerate

to quarter

speed

STOP!!

Turn the sonar

to left 90

Turn the sonar

to 0

Turn the sonar

to left 30

Turn the sonar

to left 60

Get data from A2D

Get data from A2D

Get data from A2D

Turn the sonar

to right 30

Turn the sonar

to right 60

Turn the sonar

to right 90

Get data from A2D

Get data from A2D

Get data from A2D

Find the optimum

of the

seven bearings

Is L90 > L60?

Discard L90.

Compare

L90 to L60

Are they equal?

Discard L60

Repeat with

result and next

heading

Find the best heading

# Mobile Platform

The mobile platform is based on a Tamaya remote-controlled racecar frame. The chassis has a full suspension that will help to dampen vibration.  The original springs have been replaced with much stiffer ones to allow for the heavy battery load.  The chassis holds the original 7.2-volt battery pack and two extra AA eight packs.  The drive motor is original but the controller has been replaced with a digital controller that I assembled from parts in the lab.

The body of the platform is constructed of plywood and it is mounted to the chassis with #10 all-thread and it is reinforced with steel angle brackets.  The body houses and protects all of the electronics.  The electronics have been mounted completely within the body to prevent any damage in the event of a crash.  This design includes a raised platform that places the sonar 11 inches from the ground.  This was done to prevent ground interference and it may be possible to lower the platform in the future.

The entire system consists of two completely separate electric circuits.  The motor control and servo circuits, and the sensor and micro-controller circuits.  These circuits were separated to prevent potential noise and power problems in the micro-controller circuit.  The sensor and micro-controller circuit consisted of the 40-KHz generator for the IR LED’s, the IR sensors, the compass, the sonar, and the micro-controller. The diagram for this circuit is shown in Figure 1.

# Actuation

The entire system consists of two completely separate electric circuits.  The motor control and servo circuits, and the sensor and micro-controller circuits.  These circuits were separated to prevent potential noise and power problems in the micro-controller circuit.  The sensor and micro-controller circuit consisted of the 40-KHz generator for the IR LED’s, the IR sensors, the compass, the sonar, and the micro-controller. The diagram for this circuit is shown in Figure 1.

Figure 1.  Micro-controller and sensors circuit.

The motor-controller and servo circuit consisted of the power mosfets for the motor and the motor relays and the power supply for the servos.  The diagram for this circuit is shown in Figure 2.

Figure 2.  Motor controller circuit.

The micro-controller controlled the power motor and the servos with pulse width modulation.  The signals were transmitted from the micro-controller to the mosfets, relays, and servos via opto-isolators.  The non-inverting buffer was used because the micro-controller can not drive the opto-isolator LED’s.  This circuit is detailed in Figure 3.

Figure 3. Opto-Isolator Circuit.

Separate test code was written for steering servo, the sonar servo, and the motor-controller.  This test code is included in the appendix.

# Sensors

Compass Configuration

Speedy's compass is mounted on the chassis, and it is positioned as far as possible from the motor.  This is necessary because the Vector compass is very sensitive to magnetic fields.  It is a compass after all.  The V2G is a gimbaled compass.  I chose a gimbaled compass because my robot may not always be level.  The gimbals are designed to allow up to a 20-degree tilt.  The digital compass communicates with the microcontroller with a serial interface.  It can act as either the master or the slave in a serial peripheral role.  I am using the compass as the slave because the 8051 will not act as a slave.  Speedy will use the relative headings generated by the compass to help it steer and to avoid obstacles.  Readings from the compass will provide feedback to Speedy's steering and allow it to maintain a straight course.  Referencing the direction the sonar is pointing to the current bearing of the robot will help the sonar guide the robot around obstacles.

The Interface

The compass outputs a sixteen bit binary number to represent the heading in degrees.  The numbers are from 0 the 359 decimal, which corresponds to 0 to 167 hex.  The entire interface utilizes 8 of the microcontroller I/O pins.  I am currently using 10 pins but since I don't intend to calibrate the compass I will not need /Cal and CI.  The pins are used as follows:

P4.0 = /Slave Select			P3.3 = /Master - Slave

P4.1 = Serial Clock			P3.2 = /Reset

P4.2 = Serial Data Out

P4.3 = /Poll – Continuous

P4.4 = /Calibrate

P4.5 =	Calibrate Indicator

P4.6 = End of Conversion

P4.7 = Power

In order to get the compass to operate a very meticulous order of operations must be employed. The following is an overview of the steps required to get an output:

- /M-S must be pulled high.

- /BCD-Bin must be high or not connected.

- The /Res pin should be tied low for high resolution or high for low resolution.

- To request a reading:

- Pulse /P-C low for at least 10msec.  There is no maximum time for holding /P-C low, but it must be high before /SS is taken low.  Also /SS must be high before taking /P-C low.

- The Vector calculates heading while EOC is low.  The Vector will drop EOC low after /P-C is dropped low to indicate that it is calculating.  When it is through calculating it will set EOC high.

- After EOC goes high you have to wait 10msecs to take /SS low.  /SS must be held low to clock data out.

- SCLK must then be clocked by the microcontroller.  It must be high when /SS is taken low.  /SS must be low for at least 5msecs before taking SCLK low for the first time.

- Take SCLK low.

- Take SCLK high and read the first data bit from SDO.

- Repeat this cycle seven more times, reading a bit each time SCLK goes high.

- After the eighth SCLK you must wait another 5msecs.

- Then SCLK is clocked eight more times.

- When SCLK goes high for the sixteenth time it should be kept that way until /SS is returned high.

The timing for this operation is very important.  If it is not followed exactly the compass will lock up until it is reset.

The total amount of time required for a compass reading is calculated as follows:

10msecs for /P-C.

80-100msecs for EOC.

10msecs after EOC to set /SS low.

5msecs after /SS goes low before SCLK goes low.

5msecs after 8th bit.

Maximum clock rate of 1Mhz for 16 clock cycles

(I ran the clock at 40Khz)

110msecs

I will probably use EOC as an interrupt.  In other words I will poll the compass and then wait for the interrupt before servicing the rest of the compass operation.  This will allow me to poll the compass and then go back to work until the calculations are completed.

I have included my test code as an addendum to this report.

Testing

I tested the compass under various conditions.  Initially I tested it by itself to see how repeatable its calculations were.  I did this by taking several readings in a row while the compass was sitting still.  These results are listed in Table 2.  I then tested repeatability by rotating the compass 90 degrees between measurements.  I did this in four different directions.  These results are in table 1.

| Run | Heading 1 | Heading 2 | Heading 3 | Heading 4 |
| --- | --- | --- | --- | --- |
| 1 | 279 | 190 | 21 | 118 |
| 2 | 281 | 190 | 23 | 113 |
| 3 | 278 | 189 | 20 | 118 |
| 4 | 283 | 191 | 20 | 113 |
| 5 | 278 | 188 | 20 | 115 |
| Mean | 279.8 | 189.6 | 20.80 | 115.4 |

Table 1

| Run | Heading 1 | Heading 2 | Heading 3 | Heading 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 16 | 113 | 276 | 184 |  |  |  |
| 2 | 16 | 113 | 275 | 184 |  |  |  |
| 3 | 16 | 113 | 276 | 184 |  |  |  |
| 4 | 16 | 112 | 275 | 184 |  |  |  |
| 5 | 16 | 112 | 277 | 184 |  |  |  |
| 6 | 16 | 113 | 275 | 184 |  |  |  |
| 7 | 16 | 112 | 276 | 184 |  |  |  |
| 8 | 16 | 113 | 276 | 184 |  |  |  |
| 9 | 16 | 113 | 275 | 184 |  |  |  |
| 10 | 16 | 113 | 275 | 184 |  |  |  |
| Mean | 16 | 112.70 | 275.60 | 184.00 |  | 275 | 184 |

Table 2

I repeated these tests after placing the compass in the chassis.  There was no significant change.  Finally, I repeated the first tests with the motor running at various speeds.  Table 3 and 4 contain the results of these tests.

| Run | Heading 1 | Heading 2 | Heading 3 | Heading 4 |
| --- | --- | --- | --- | --- |
| Original w/o Motor running | 280 | 190 | 21 | 118 |
| 1 | 285 | 193 | 24 | 121 |
| 2 | 286 | 193 | 27 | 116 |
| 3 | 283 | 192 | 25 | 121 |
| 4 | 288 | 194 | 23 | 116 |
| 5 | 282 | 191 | 25 | 118 |
| Mean | 284.8 | 192.6 | 24.80 | 118.4 |

Table 3

| Run | Heading 1 | Heading 2 | Heading 3 | Heading 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Original w/o Motor running | 16 | 113 | 276 | 184 |  |  |  |
| 1 | 19 | 113 | 276 | 184 |  |  |  |
| 2 | 20 | 113 | 275 | 184 |  |  |  |
| 3 | 19 | 113 | 276 | 184 |  |  |  |
| 4 | 19 | 112 | 275 | 184 |  |  |  |
| 5 | 19 | 112 | 277 | 184 |  |  |  |
| 6 | 20 | 113 | 275 | 184 |  |  |  |
| 7 | 18 | 112 | 276 | 184 |  |  |  |
| 8 | 19 | 113 | 276 | 184 |  |  |  |
| 9 | 19 | 113 | 275 | 184 |  |  |  |
| 10 | 20 | 113 | 275 | 184 |  |  |  |
| Mean | 19.2 | 112.7 | 275.70 | 184.00 |  | 275 | 184 |

Table 4

### Sonar

The sonar was purchased from Mekatronics and has a range of 15 feet.  I regulated the voltage to the sonar with a 7809 voltage regulator.  This prevents the range from changing as the batteries voltage drops.  I tested the sonar in the lab and ranged it using a voltmeter and the 9” tiles.  The data I obtained is included in Table 5.

| Range (inches) | Voltage 1st Run | Voltage 2nd Run | Difference |
| --- | --- | --- | --- |
| 9 | .3 | .25 | .05 |
| 18 | .514 | .509 | .005 |
| 27 | .761 | .766 | .005 |
| 36 | 1.01 | 1.00 | .01 |
| 45 | 1.27 | 1.26 | .01 |
| 54 | 1.52 | 1.52 | 0.0 |
| 63 | 1.77 | 1.76 | .01 |
| 72 | 2.02 | 2.02 | 0.0 |
| 81 | 2.28 | 2.28 | 0.0 |
| 90 | 2.54 | 2.53 | .01 |
| 99 | 2.80 | 2.78 | .02 |
| 108 | 3.04 | 3.04 | 0.0 |
| 117 | 3.31 | 3.28 | .03 |
| 126 | 3.56 | 3.55 | .01 |
| 135 | 3.84 | 3.80 | .04 |
| 144 | 4.08 | 4.06 | .02 |
| 153 | 4.32 | 4.32 | 0.0 |
| 162 | 4.59 | 4.51 | .08 |
| 171 | 4.84 | 4.80 | .04 |
| 180 | 4.96 | 4.96 | 0.0 |

Table 5 Sonar test results

The IR sensors were Sharp sonar sensors with the analog hack.  They perform the same as everyone else’s IR sensors.

# Behaviors

The simplified navigation program that I implemented has the following basic behaviors:

- Front left IR/Front Right IR – If sensor level > 100 then turn wheels to left and back up.

- Rear IR – if sensor level > 100 straighten wheels and go forward

- If left IR  10 > right IR and left IR > 100 then go right

- If right IR 10 > left IR and left IR > 100 then go left

- Sonar > 15 ft – Go fast

- Sonar > 9 ft and not going fast -> go medium; if going fast, go slow

- Sonar > 6 ft and not going fast -> go slow; if going fast, reverse

- Sonar < 1 ft -> reverse

This is really basic.  The platform that I have constructed is capable of performing much more complex behaviors.  I want to implement much more complex behaviors, but I don’t have the skills yet.  Hopefully, when I learn how to write for the Intel in C, I will be able to implement these behaviors.

# Appendix

This is the simplified navigation code for RoboSpeedy

SON_SPD                                                                                                       PAGE 1

8000                   1            org     8000h

8000 028200            2            jmp     8200h

3

4            $MOD51GB

5

6    ;Pulse width signals

7    PWM_MOT         EQU     R0

8    PWM_STR         EQU     R1

9    DATA_BYTE       EQU     R4

10    Milkyway        EQU     R1

11    ;Motor Direction

00F9                12    STP_GO          EQU     P5.1

00F8                13    FOR_REV         EQU     P5.0

14

15    ;Speeds

00B0                16    FWD_LO          EQU     0B0H    ;C0

00A0                17    FWD_MED         EQU     0A0H    ;B0

00A0                18    FWD_HI          EQU     0A0H    ;B0

0080                19    REV_LO          EQU     080H    ;90

20

21    ;Steering Directions

003F                22    Straight        EQU     03fH

0030                23    Full_Left       EQU     030H

0037                24    Half_Left       EQU     037H

003A                25    Qtr_Left        EQU     03AH

004D                26    Full_Right      EQU     04DH

0045                27    Half_Right      EQU     045H

0042                28    Qtr_Right       EQU     042H

29

30

31

32    ;Sonar ranges

00F5                33    Sonar_15        EQU     245

00D0                34    Sonar_12        EQU     208

009B                35    Sonar_9         EQU     155

0067                36    Sonar_6         EQU     103

0033                37    Sonar_3         EQU     51

001A                38    Sonar_1         EQU     26

39

8200                  40            org     8200h

8200                  41    main:

42            ;enable A/D operation

8200 759710           43            mov     ACON,#010h

44

45            ;STP_GO is cleared to stop motor

46            ;FOR_REV is set to go in reverse

47

48            ;initalize the motor and steering PWM waveforms

8203 7590FF           49            mov     P1,#0FFH        ;must set all bits on port1 to 0

8206 78B0             50            mov     PWM_MOT,#FWD_LO   ;set motor for 1/8 speed

8208 88FC             51            mov     CCAP2h,PWM_MOT

820A 793C             52            mov     PWM_STR,#03Ch   ;set steering servo to center

820C 89FB             53            mov     CCAP1h,PWM_STR

820E C2F8             54            clr     FOR_REV           ;set relays for FORWARD

8210 C2F9             55            clr     STP_GO          ;block PWM signal to motor

8212 1282F3           56            call    PCA_INIT

57

8215 D2F9             58            setb    STP_GO

SON_SPD                                                                                                       PAGE 2

8217 1282FF           59    Scan: call    lngDelay

60

821A C3               61    L_Rear: clr     C             ; Clear carry bit

821B E594             62            mov     A,AD1         ; get the left rear IR

821D 9464             63            subb    A,#100        ; subtract threshold from value

821F 402E             64            jc      L_Front       ; jump if not too close

65

8221                  66    rev2for:

8221 30F929           67            jnb     STP_GO,stp_rev2  ;already stopped

8224 30F826           68            jnb     FOR_REV,stp_rev2 ;or going forward

8227 C2F9             69            clr     STP_GO           ;stop motor

8229 C2F8             70            clr     FOR_REV          ;place in forward

822B 793F             71            mov     PWM_STR,#Straight ;center wheels

822D 89FB             72            mov     CCAP1H,PWM_STR    ;

822F 1282FF           73            call    lngDelay

8232 78B0             74            mov     PWM_MOT,#FWD_LO  ;set motor for 1/8 speed

8234 88FC             75            mov     CCAP2h,PWM_MOT

8236 D2F9             76            setb    STP_GO           ;start motor

8238 128314           77            call    PRINT

823B 466F7277         78            db      'Forward from IR\n',0,

823F 61726420

8243 66726F6D

8247 2049525C

824B 6E00

824D                  79    stp_rev2:

824D 80C8             80            jmp     Scan

81

82

824F                  83    L_Front:

824F C3               84            clr     C             ; Clear carry bit

8250 E5A4             85            mov     A,AD2         ; get the left front IR

8252 9464             86            subb    A,#100        ; subtract threshold from value

8254 4003             87            jc      R_Front       ; jump if not too close

88

8256 028260           89            jmp     for2rev

90

8259                  91    R_Front:

8259 C3               92            clr     C             ; Clear carry bit

825A E5B4             93            mov     A,AD3         ; get the left front IR

825C 9464             94            subb    A,#100        ; subtract threshold from value

825E 402E             95            jc      Sonar         ; jump if not too close

96

8260                  97    for2Rev:

8260 30F929           98            jnb     STP_GO,stp_rev0         ;already stopped

8263 20F826           99            jb      FOR_REV,stp_rev0        ;or backing

8266 C2F9            100            clr     STP_GO                  ;stop motor

8268 D2F8            101            setb    FOR_REV                 ;reverse it

826A 7931            102            mov     PWM_STR,#031h           ;turn wheels to right

826C 89FB            103            mov     CCAP1H,PWM_STR          ;

826E 1282FF          104            call    lngDelay                ;Give it 1/2 a second

8271 7880            105            mov     PWM_MOT,#REV_LO         ;set motor for 1/8 speed

8273 88FC            106            mov     CCAP2h,PWM_MOT

8275 D2F9            107            setb    STP_GO                  ;start motor

8277 128314          108            call    PRINT

827A 4261636B        109            db      'Backing from IR\n',0,

827E 696E6720

8282 66726F6D

8286 2049525C

SON_SPD                                                                                                       PAGE 3

828A 6E00

828C                 110    stp_rev0:

828C 8089            111            jmp     Scan

112

828E C3              113    Sonar:  clr     C

828F E584            114            mov     A,AD0           ; Get the sonar value

8291 94F5            115            subb    A,#Sonar_15     ; subtract threshold from value

8293 4016            116            jc      Ahd_hlf         ; jump if distance is < 15 ft

117

8295 78A0            118            mov     PWM_MOT,#FWD_HI  ;set motor for High speed

8297 88FC            119            mov     CCAP2h,PWM_MOT

8299 128314          120            call    PRINT

829C 41686561        121            db      'Ahead Full\n',0,

82A0 64204675

82A4 6C6C5C6E

82A8 00

122

82A9 4117            123            jmp     Scan

124

82AB                 125    Ahd_hlf:

82AB C3              126            clr     C

82AC E584            127            mov     A,AD0           ; Get the sonar value

82AE 949B            128            subb    A,#Sonar_9      ; subtract threshold from value

82B0 401B            129            jc      Ahd_slow        ; jump if distance is < 9 ft

82B2 B8A0AB          130            cjne    PWM_MOT,#FWD_HI,for2Rev

131                                    ; If motor is in high gear

132                                    ; turn it off

133

82B5 78A0            134            mov     PWM_MOT,#FWD_MED  ;set motor for Medium speed

82B7 88FC            135            mov     CCAP2h,PWM_MOT

82B9 128314          136            call    PRINT

82BC 41686561        137            db      'Ahead Medium\n',0,

82C0 64204D65

82C4 6469756D

82C8 5C6E00

138

82CB 4117            139            jmp     Scan

140

82CD                 141    Ahd_slow:

82CD C3              142            clr     C

82CE E584            143            mov     A,AD0           ; Get the sonar value

82D0 9433            144            subb    A,#Sonar_3      ; subtract threshold from value

82D2 408C            145            jc      for2rev         ; jump if distance is < 3 ft

82D4 B8A089          146            cjne    PWM_MOT,#FWD_HI,for2rev

147                                    ; If motor is in high gear

148                                    ; turn it off

149

82D7 78B0            150            mov     PWM_MOT,#FWD_LO  ;set motor for Low speed

82D9 88FC            151            mov     CCAP2h,PWM_MOT

82DB 128314          152            call    PRINT

82DE 41686561        153            db      'Ahead Slow\n',0,

82E2 6420536C

82E6 6F775C6E

82EA 00

154

82EB 4117            155            jmp     Scan

156

82ED                 157    All_Stop:

SON_SPD                                                                                                       PAGE 4

82ED C2F9            158            clr     STP_GO

82EF 4117            159            jmp     Scan

160

161

162

163

164

82F1 80FE            165    wait:   jmp     wait

166

167

168

169            ;initializes the programmable counter array for the motor

170            ;and the steering servo

82F3                 171    PCA_INIT:

82F3 75D906          172            mov     CMOD,#006h      ;set PCA frequency to external

82F6 75DB42          173            mov     CCAPM1,#42h     ;put PCA module 1 into PWM mode

82F9 75DC42          174            mov     CCAPM2,#42h     ;put PCA module 0 into PWM mode

82FC D2DE            175            setb    CR              ;start the PCA clock

82FE 22              176            ret

177

82FF                 178    lngDelay:

=1   179            $INCLUDE(lngDelay.inc)

82FF 7D0A       =1   180            mov   R5,#00Ah

8301 7E35       =1   181    delay2: mov     R6,#035h

8303 7FFF       =1   182    delay3: mov     R7,#0FFh

8305 DFFE       =1   183    delay4: djnz    R7, delay4              ;wait >500msec

8307 DEFA       =1   184            djnz    R6, delay3

8309 DDF6       =1   185            djnz    R5, delay2

830B 22         =1   186            ret

187

830C                 188    cout:

=1   189            $INCLUDE(cout.inc)

830C 3099FD     =1   190            JNB     TI,$            ;Wait until transmission completed.

830F C299       =1   191            CLR     TI              ;Clear interrupt flag.

8311 F599       =1   192            MOV     SBUF,A          ;Write out character.

8313 22         =1   193            RET

194

8314                 195    PRINT:

=1   196            $INCLUDE(PRINT.inc)

8314 C0E0       =1   197            PUSH    ACC      ; push the A and DPTR registers so their contents

8316 C083       =1   198            PUSH    DPH      ; don't get lost in case they are being used

8318 C082       =1   199            PUSH    DPL      ; before calling this function.

=1   200

=1   201    ; move the stack pointer down low in order to pop the first string's byte

=1   202    ; address

831A E581       =1   203            MOV     A,SP

831C 24FD       =1   204            ADD     A,#0FDH

831E F581       =1   205            MOV     SP,A

8320 D083       =1   206            POP     DPH

8322 D082       =1   207            POP     DPL

=1   208

=1   209    ; send (PRINT) the string back to the PC computer

8324            =1   210    READ_TX:

8324 E4         =1   211            CLR     A

8325 93         =1   212            MOVC    A,@A+DPTR

8326 A3         =1   213            INC     DPTR

=1   214

8327 B45C0C     =1   215            CJNE    A,#'\',SND

SON_SPD                                                                                                       PAGE 5

=1   216

832A E4         =1   217            CLR     A

832B 93         =1   218            MOVC    A,@A+DPTR

832C A3         =1   219            INC     DPTR

=1   220

832D B46E06     =1   221            CJNE    A,#'n',SND

=1   222

8330 740D       =1   223            MOV     A,#0DH

8332 710C       =1   224            ACALL   cout

8334 740A       =1   225            MOV     A,#0AH

=1   226

8336 710C       =1   227    SND:    ACALL   cout

8338 B400E9     =1   228            CJNE    A,#0,READ_TX

=1   229

=1   230    ; place the new return addres in stack

833B C082       =1   231            PUSH    DPL

833D C083       =1   232            PUSH    DPH

=1   233

=1   234    ; move the stack pointer up high in order to pop the A and DPTR registers

833F E581       =1   235            MOV     A,SP

8341 2403       =1   236            ADD     A,#3

8343 F581       =1   237            MOV     SP,A

8345 D082       =1   238            POP     DPL

8347 D083       =1   239            POP     DPH

8349 D0E0       =1   240            POP     ACC

834B 22         =1   241            RET

242

834C                 243    SEND_BYTE:

=1   244            $INCLUDE(SEND_BYT.inc)

834C 3099FD     =1   245            JNB    TI, SEND_BYTE           ; WAIT FOR THE TI FLAG TO SET

834F C299       =1   246            CLR             TI                                ; CLEAR THE TI FLAG ONCE SENT

8351 F599       =1   247                    MOV     SBUF,A                  ; SEND THE CONTENTS OF REGISTER A

8353 22         =1   248            RET                             ; RETURN

=1   249    ;*****************************************************************************

=1   250

=1   251

=1   252

253

254

255

256            end

VERSION 1.2h ASSEMBLY COMPLETE, 0 ERRORS FOUND

SON_SPD                                                                                                       PAGE 6

ACC. . . . . . . . . . . . . . .  D ADDR  00E0H  PREDEFINED

ACON . . . . . . . . . . . . . .  D ADDR  0097H  PREDEFINED

AD0. . . . . . . . . . . . . . .  D ADDR  0084H  PREDEFINED

AD1. . . . . . . . . . . . . . .  D ADDR  0094H  PREDEFINED

AD2. . . . . . . . . . . . . . .  D ADDR  00A4H  PREDEFINED

AD3. . . . . . . . . . . . . . .  D ADDR  00B4H  PREDEFINED

AHD_HLF. . . . . . . . . . . . .  C ADDR  82ABH

AHD_SLOW . . . . . . . . . . . .  C ADDR  82CDH

ALL_STOP . . . . . . . . . . . .  C ADDR  82EDH  NOT USED

CCAP1H . . . . . . . . . . . . .  D ADDR  00FBH  PREDEFINED

CCAP2H . . . . . . . . . . . . .  D ADDR  00FCH  PREDEFINED

CCAPM1 . . . . . . . . . . . . .  D ADDR  00DBH  PREDEFINED

CCAPM2 . . . . . . . . . . . . .  D ADDR  00DCH  PREDEFINED

CMOD . . . . . . . . . . . . . .  D ADDR  00D9H  PREDEFINED

COUT . . . . . . . . . . . . . .  C ADDR  830CH

CR . . . . . . . . . . . . . . .  B ADDR  00DEH  PREDEFINED

DATA_BYTE. . . . . . . . . . . .    REG4         NOT USED

DELAY2 . . . . . . . . . . . . .  C ADDR  8301H

DELAY3 . . . . . . . . . . . . .  C ADDR  8303H

DELAY4 . . . . . . . . . . . . .  C ADDR  8305H

DPH. . . . . . . . . . . . . . .  D ADDR  0083H  PREDEFINED

DPL. . . . . . . . . . . . . . .  D ADDR  0082H  PREDEFINED

FOR2REV. . . . . . . . . . . . .  C ADDR  8260H

FOR_REV. . . . . . . . . . . . .    NUMB  00F8H

FULL_LEFT. . . . . . . . . . . .    NUMB  0030H  NOT USED

FULL_RIGHT . . . . . . . . . . .    NUMB  004DH  NOT USED

FWD_HI . . . . . . . . . . . . .    NUMB  00A0H

FWD_LO . . . . . . . . . . . . .    NUMB  00B0H

FWD_MED. . . . . . . . . . . . .    NUMB  00A0H

HALF_LEFT. . . . . . . . . . . .    NUMB  0037H  NOT USED

HALF_RIGHT . . . . . . . . . . .    NUMB  0045H  NOT USED

LNGDELAY . . . . . . . . . . . .  C ADDR  82FFH

L_FRONT. . . . . . . . . . . . .  C ADDR  824FH

L_REAR . . . . . . . . . . . . .  C ADDR  821AH  NOT USED

MAIN . . . . . . . . . . . . . .  C ADDR  8200H  NOT USED

MILKYWAY . . . . . . . . . . . .    REG1         NOT USED

P1 . . . . . . . . . . . . . . .  D ADDR  0090H  PREDEFINED

P5 . . . . . . . . . . . . . . .  D ADDR  00F8H  PREDEFINED

PCA_INIT . . . . . . . . . . . .  C ADDR  82F3H

PRINT. . . . . . . . . . . . . .  C ADDR  8314H

PWM_MOT. . . . . . . . . . . . .    REG0

PWM_STR. . . . . . . . . . . . .    REG1

QTR_LEFT . . . . . . . . . . . .    NUMB  003AH  NOT USED

QTR_RIGHT. . . . . . . . . . . .    NUMB  0042H  NOT USED

READ_TX. . . . . . . . . . . . .  C ADDR  8324H

REV2FOR. . . . . . . . . . . . .  C ADDR  8221H  NOT USED

REV_LO . . . . . . . . . . . . .    NUMB  0080H

R_FRONT. . . . . . . . . . . . .  C ADDR  8259H

SBUF . . . . . . . . . . . . . .  D ADDR  0099H  PREDEFINED

SCAN . . . . . . . . . . . . . .  C ADDR  8217H

SEND_BYTE. . . . . . . . . . . .  C ADDR  834CH

SND. . . . . . . . . . . . . . .  C ADDR  8336H

SONAR. . . . . . . . . . . . . .  C ADDR  828EH

SONAR_1. . . . . . . . . . . . .    NUMB  001AH  NOT USED

SONAR_12 . . . . . . . . . . . .    NUMB  00D0H  NOT USED

SONAR_15 . . . . . . . . . . . .    NUMB  00F5H

SONAR_3. . . . . . . . . . . . .    NUMB  0033H

SONAR_6. . . . . . . . . . . . .    NUMB  0067H  NOT USED

SON_SPD                                                                                                       PAGE 7

SONAR_9. . . . . . . . . . . . .    NUMB  009BH

SP . . . . . . . . . . . . . . .  D ADDR  0081H  PREDEFINED

STP_GO . . . . . . . . . . . . .    NUMB  00F9H

STP_REV0 . . . . . . . . . . . .  C ADDR  828CH

STP_REV2 . . . . . . . . . . . .  C ADDR  824DH

STRAIGHT . . . . . . . . . . . .    NUMB  003FH

TI . . . . . . . . . . . . . . .  B ADDR  0099H  PREDEFINED

WAIT . . . . . . . . . . . . . .  C ADDR  82F1H

This code is for the precision navigation compass.  It utilizes the 8051 interrupt system.

CMPS_INT                                                                                                      PAGE 1

1            $MOD51GB

2

00C0                 3            EOC     equ     P4.0

00C1                 4            SS      equ     P4.1

00C2                 5            MS      equ     P4.2

00C3                 6            RESET   equ     P4.3

00C4                 7            SCLK    equ     P4.4

00C5                 8            SDO     equ     P4.5

00C6                 9            POLL    equ     P4.6

00C7                10            POW     equ     P4.7

11

12            CompCount  equ  R7

13            DataByte   equ  R4

14

8000                  15            org     8000h

8000 028200           16            jmp     8200h

17

802B                  18            org     802bh

802B 0282AD           19            jmp     T2_ISR

20

8100                  21            org     8100h

8100 0000             22            db      0,0

8200                  23            org     8200h

8200 128285           24            call    Comp_Init       ;Initialize the compass

8203 128389           25            call    Print

8206 54686520         26            db      'The compass is initialized\n',0,

820A 636F6D70

820E 61737320

8212 69732069

8216 6E697469

821A 616C697A

821E 65645C6E

8222 00

8223 1282A2           27            call    T2_Init         ;Initialize T2

8226 128389           28            call    Print

8229 54696D65         29            db      'Timer 2 is initialized\n',0,

822D 72203220

8231 69732069

8235 6E697469

8239 616C697A

823D 65645C6E

8241 00

30

8242 D2C2             31            setb    MS

8244 128389           32    ComRd:  call    Print

8247 5C6E2020         33            db      '\n     Press the g key to get a compass heading\n',0,

824B 20202050

824F 72657373

8253 20746865

8257 2067206B

825B 65792074

825F 6F206765

8263 74206120

8267 636F6D70

826B 61737320

826F 68656164

8273 696E675C

8277 6E00

CMPS_INT                                                                                                      PAGE 2

8279 128379           34    start:  call    cin

827C B467FA           35            cjne    A,#'g',start

36

827F D2AD             37            setb    ET2             ;enable Timer 2 interrupt

8281 D2AF             38            setb    EA              ;enable interrupts

39

8283 80FE             40    wait:   jmp     wait

41

8285                  42    Comp_Init:

43            ;set all the bits on the compass

8285 D2C7             44            setb    POW             ;turn power off

8287 D2C6             45            setb    POLL            ;set the poll bit

8289 D2C1             46            setb    SS              ;set the slave select bit

828B D2C3             47            setb    RESET           ;set the NOT Reset line

828D D2C4             48            setb    SCLK            ;set the shift clock

828F D2C5             49            setb    SDO             ;setting SDO will set that pin on the

50                                    ;8051 as an input

8291 D2C0             51            setb    EOC             ;same for EOC

52

8293 C2C7             53            clr     POW             ;turn power on

54

55            ;Initialize the counter for the compass process controller

8295 7F00             56            mov     CompCount,#0

8297 C2C3             57            clr     RESET           ;reset compass

8299 128363           58            call    Delay10         ;wait 10msecs

829C D2C3             59            setb    RESET           ;clear reset

829E 12836C           60            call    lngDelay        ;wait .5secs

61

62

82A1 22               63            ret

64

82A2                  65    T2_Init:

66            ;load the timer to run the compass

82A2 C2CA             67            clr     TR2             ;turn Timer 2 off

82A4 75CDB7           68            mov     TH2,#0B7h       ;load the T2 high byte

82A7 75CCCB           69            mov     TL2,#0CBh       ;load the T2 low byte

82AA D2CA             70            setb    TR2             ;turn Timer 2 on

82AC 22               71            ret

72

82AD                  73    T2_ISR:

=1    74            $INCLUDE(T2_ISR.INC)

82AD C2CF       =1    75            clr     TF2             ;Clear the T2 interrupt flag

82AF C2CA       =1    76            clr     TR2             ;Stop the timer

82B1 75CDB7     =1    77            mov     TH2,#0B7h       ;Load the timer with the value

82B4 75CCCB     =1    78            mov     TL2,#0CBh       ;needed for a 10msec interval

82B7 D2CA       =1    79            setb    TR2             ;turn timer back on

82B9 0F         =1    80            inc     CompCount       ;Increment the process counter

=1    81

82BA BF0103     =1    82            cjne    CompCount,#1,temp0

82BD 0282FB     =1    83            jmp     CLR_POLL

82C0 BF0203     =1    84    temp0:  cjne    CompCount,#2,temp1

82C3 028300     =1    85            jmp     SET_POLL

82C6 BF0303     =1    86    temp1:  cjne    CompCount,#3,temp2

82C9 028305     =1    87            jmp     WAIT_EOC

82CC BF0403     =1    88    temp2:  cjne    CompCount,#4,temp3

82CF 02830D     =1    89            jmp     CLR_SS

82D2 BF0503     =1    90    temp3:  cjne    CompCount,#5,temp4

82D5 028312     =1    91            jmp     READ_SDO

CMPS_INT                                                                                                      PAGE 3

82D8 BF0603     =1    92    temp4:  cjne    CompCount,#6,temp5

82DB 028312     =1    93            jmp     READ_SDO

82DE 128389     =1    94    temp5:  call    Print

82E1 54686973   =1    95            db      'This didnt work!! \n\n',0,

82E5 20646964

82E9 6E742077

82ED 6F726B21

82F1 21205C6E

82F5 5C6E00

82F8 02835D     =1    96            jmp     exit_T2

=1    97

82FB            =1    98    CLR_POLL:

=1    99

82FB C2C6       =1   100            clr     POLL              ;clr POLL

82FD 02833E     =1   101            jmp     end_T2

=1   102

8300            =1   103    SET_POLL:

8300 D2C6       =1   104            setb    POLL              ;set POLL

8302 02833E     =1   105            jmp     end_T2

=1   106

8305            =1   107    WAIT_EOC:

8305 20C036     =1   108            jb      EOC,end_T2        ;wait for the EOC flag to be

8308 7F02       =1   109            mov     CompCount,#2      ;set

830A 02833E     =1   110            jmp     end_T2

=1   111

830D C2C1       =1   112    CLR_SS: clr     SS                ; set slave select for data recovery

830F 02833E     =1   113            jmp     end_T2

=1   114

=1   115

8312 7E08       =1   116    READ_SDO:  mov     R6,#08

=1   117

8314            =1   118    ReadSDO:

8314 C2C4       =1   119            clr     SCLK

8316 12835E     =1   120            call    clkDelay

8319 D2C4       =1   121            setb    SCLK

831B 7420       =1   122            mov     A, #020h        ;set A.5

831D 55C0       =1   123            anl     A,P4            ;compare A.5 with P4.5(SDO)

831F 6006       =1   124            jz      zero            ;jump if P4.5(EOC) is not set

=1   125

8321 EC         =1   126            mov     A, DataByte     ;load R4 into A

8322 D3         =1   127            setb    C               ;set the carry bit

8323 33         =1   128            rlc     A               ;roll A left with carry

8324 FC         =1   129            mov     DataByte, A     ;put contents of A back into R4

8325 612B       =1   130            ajmp    rdDone          ; done with read

=1   131

8327 EC         =1   132    zero:   mov     A, DataByte     ;load R4 into A

8328 C3         =1   133            clr     C               ;clear the carry bit

8329 33         =1   134            rlc     A               ;roll A left with carry

832A FC         =1   135            mov     DataByte, A     ;put contents of A back into R4

=1   136

832B 12835E     =1   137    rdDone: call   clkdelay         ;delay for low clock

832E DEE4       =1   138            djnz   R6, ReadSDO      ;this loop will execute 8 times

=1   139

8330 BF050B     =1   140    done8:  cjne   CompCount,#5,end_T2

=1   141                                    ;SDO_READ will execute when

=1   142                                    ;CompCount = 5 & 6

=1   143                                    ;done8 will only run when CompCount=5

8333 EC         =1   144            mov    A,DataByte       ;mov Data to A

CMPS_INT                                                                                                      PAGE 4

8334 758381     =1   145            mov    DPH,#081h        ;first 8 bits will be saved

8337 758200     =1   146            mov    DPL,#000h        ;at 8100h

833A F0         =1   147            movx   @DPTR,A          ;

833B 02833E     =1   148            jmp    end_T2           ;end the routine

=1   149

=1   150

833E BF061C     =1   151    end_T2: cjne    CompCount,#6,exit_T2

8341 D2C1       =1   152            setb    SS              ;Slave select off

8343 EC         =1   153            mov     A,DataByte

8344 758381     =1   154            mov     DPH,#081h

8347 758201     =1   155            mov     DPL,#001h

834A F0         =1   156            movx    @DPTR,A

834B 7F00       =1   157            mov     CompCount,#00

834D 758200     =1   158            mov     DPL,#00h

8350 E0         =1   159            movx    A,@DPTR

8351 F583       =1   160            mov     DPH,A

8353 8C82       =1   161            mov     DPL,DATABYTE

8355 1283C1     =1   162            call    pint16u

8358 128389     =1   163            call    print

835B 5C00       =1   164            db      '\',0,

=1   165

=1   166

=1   167

835D            =1   168    exit_T2:

835D 32         =1   169            reti

170

835E 7873            171    clkdelay: mov   R0,#073h

8360 D8FE            172    clk:    djnz    R0,clk

8362 22              173            ret

174

8363 7825            175    delay10: mov    R0,#025h

8365 79FF            176    delay:  mov     R1,#0FFh

8367 D9FE            177    delay1: djnz    R1, delay1              ;wait >10 msec

8369 D8FA            178            djnz    R0, delay

836B 22              179            ret

180

181

836C 780A            182    lngDelay: mov   R0,#00Ah

836E 7935            183    delay2: mov     R1,#035h

8370 7AFF            184    delay3: mov     R2,#0FFh

8372 DAFE            185    delay4: djnz    R2, delay4              ;wait >500msec

8374 D9FA            186            djnz    R1, delay3

8376 D8F6            187            djnz    R0, delay2

8378 22              188            ret

189

8379                 190    cin:

=1   191            $INCLUDE(cin.inc)

8379 3098FD     =1   192            JNB     RI,$             ; WAIT FOR THE RI FLAG TO SET

837C C298       =1   193            CLR     RI               ; CLEAR THE RECEIVE FLAG ONCE RECIEVED

837E E599       =1   194            MOV     A,SBUF           ; PLACE RECEIVED BYTE IN REGISTER A

8380 22         =1   195            RET

196

8381                 197    cout:

=1   198            $INCLUDE(cout.inc)

8381 3099FD     =1   199            JNB     TI,$            ;Wait until transmission completed.

8384 C299       =1   200            CLR     TI              ;Clear interrupt flag.

8386 F599       =1   201            MOV     SBUF,A          ;Write out character.

8388 22         =1   202            RET

CMPS_INT                                                                                                      PAGE 5

203

8389                 204    PRINT:

=1   205            $INCLUDE(Print.inc)

8389 C0E0       =1   206            PUSH    ACC      ; push the A and DPTR registers so their contents

838B C083       =1   207            PUSH    DPH      ; don't get lost in case they are being used

838D C082       =1   208            PUSH    DPL      ; before calling this function.

=1   209

=1   210    ; move the stack pointer down low in order to pop the first string's byte

=1   211    ; address

838F E581       =1   212            MOV     A,SP

8391 24FD       =1   213            ADD     A,#0FDH

8393 F581       =1   214            MOV     SP,A

8395 D083       =1   215            POP     DPH

8397 D082       =1   216            POP     DPL

=1   217

=1   218    ; send (PRINT) the string back to the PC computer

8399            =1   219    READ_TX:

8399 E4         =1   220            CLR     A

839A 93         =1   221            MOVC    A,@A+DPTR

839B A3         =1   222            INC     DPTR

=1   223

839C B45C0C     =1   224            CJNE    A,#'\',SND

=1   225

839F E4         =1   226            CLR     A

83A0 93         =1   227            MOVC    A,@A+DPTR

83A1 A3         =1   228            INC     DPTR

=1   229

83A2 B46E06     =1   230            CJNE    A,#'n',SND

=1   231

83A5 740D       =1   232            MOV     A,#0DH

83A7 7181       =1   233            ACALL   cout

83A9 740A       =1   234            MOV     A,#0AH

=1   235

83AB 7181       =1   236    SND:    ACALL   cout

83AD B400E9     =1   237            CJNE    A,#0,READ_TX

=1   238

=1   239    ; place the new return addres in stack

83B0 C082       =1   240            PUSH    DPL

83B2 C083       =1   241            PUSH    DPH

=1   242

=1   243    ; move the stack pointer up high in order to pop the A and DPTR registers

83B4 E581       =1   244            MOV     A,SP

83B6 2403       =1   245            ADD     A,#3

83B8 F581       =1   246            MOV     SP,A

83BA D082       =1   247            POP     DPL

83BC D083       =1   248            POP     DPH

83BE D0E0       =1   249            POP     ACC

83C0 22         =1   250            RET

251

83C1                 252    pint16u:

=1   253            $INCLUDE(pint16u.inc)

=1   254            ;print 16 bit unsigned integer in DPTR, using base 10.

=1   255            ;warning, destroys r2, r3, r4, r5, psw.5

83C1 C0E0       =1   256            push    acc

83C3 E8         =1   257            mov     a, r0

83C4 C0E0       =1   258            push    acc

83C6 C2D5       =1   259            clr     psw.5

83C8 AA82       =1   260            mov     r2, dpl

CMPS_INT                                                                                                      PAGE 6

83CA AB83       =1   261            mov     r3, dph

=1   262

83CC 7C10       =1   263    pint16a:mov     r4, #16 ;ten-thousands digit

83CE 7D27       =1   264            mov     r5, #39

83D0 911F       =1   265            acall   pint16x

83D2 6007       =1   266            jz      pint16b

83D4 2430       =1   267            add     a, #'0'

83D6 128381     =1   268            lcall   cout

83D9 D2D5       =1   269            setb    psw.5

=1   270

83DB 7CE8       =1   271    pint16b:mov     r4, #232        ;thousands digit

83DD 7D03       =1   272            mov     r5, #3

83DF 911F       =1   273            acall   pint16x

83E1 7003       =1   274            jnz     pint16c

83E3 30D507     =1   275            jnb     psw.5, pint16d

83E6 2430       =1   276    pint16c:add     a, #'0'

83E8 128381     =1   277            lcall   cout

83EB D2D5       =1   278            setb    psw.5

=1   279

83ED 7C64       =1   280    pint16d:mov     r4, #100        ;hundreds digit

83EF 7D00       =1   281            mov     r5, #0

83F1 911F       =1   282            acall   pint16x

83F3 7003       =1   283            jnz     pint16e

83F5 30D507     =1   284            jnb     psw.5, pint16f

83F8 2430       =1   285    pint16e:add     a, #'0'

83FA 128381     =1   286            lcall   cout

83FD D2D5       =1   287            setb    psw.5

=1   288

83FF EA         =1   289    pint16f:mov     a, r2           ;tens digit

8400 ABF0       =1   290            mov     r3, b

8402 75F00A     =1   291            mov     b, #10

8405 84         =1   292            div     ab

8406 7003       =1   293            jnz     pint16g

8408 30D505     =1   294            jnb     psw.5, pint16h

840B 2430       =1   295    pint16g:add     a, #'0'

840D 128381     =1   296            lcall   cout

=1   297

8410 E5F0       =1   298    pint16h:mov     a, b            ;and finally the ones digit

8412 8BF0       =1   299            mov     b, r3

8414 2430       =1   300            add     a, #'0'

8416 128381     =1   301            lcall   cout

=1   302

8419 D0E0       =1   303            pop     acc

841B F8         =1   304            mov     r0, a

841C D0E0       =1   305            pop     acc

841E 22         =1   306            ret

=1   307

=1   308    ;ok, it's a cpu hog and a nasty way to divide, but this code

=1   309    ;requires only 21 bytes!  Divides r2-r3 by r4-r5 and leaves

=1   310    ;quotient in r2-r3 and returns remainder in acc.  If Intel

=1   311    ;had made a proper divide, then this would be much easier.

=1   312

841F 7800       =1   313    pint16x:mov     r0, #0

8421 08         =1   314    pint16y:inc     r0

8422 C3         =1   315            clr     c

8423 EA         =1   316            mov     a, r2

8424 9C         =1   317            subb    a, r4

8425 FA         =1   318            mov     r2, a

CMPS_INT                                                                                                      PAGE 7

8426 EB         =1   319            mov     a, r3

8427 9D         =1   320            subb    a, r5

8428 FB         =1   321            mov     r3, a

8429 50F6       =1   322            jnc     pint16y

842B 18         =1   323            dec     r0

842C EA         =1   324            mov     a, r2

842D 2C         =1   325            add     a, r4

842E FA         =1   326            mov     r2, a

842F EB         =1   327            mov     a, r3

8430 3D         =1   328            addc    a, r5

8431 FB         =1   329            mov     r3, a

8432 E8         =1   330            mov     a, r0

8433 22         =1   331            ret

332

333            END

VERSION 1.2h ASSEMBLY COMPLETE, 0 ERRORS FOUND

CMPS_INT                                                                                                      PAGE 8

ACC. . . . . . . . . . . . . . .  D ADDR  00E0H  PREDEFINED

B. . . . . . . . . . . . . . . .  D ADDR  00F0H  PREDEFINED

CIN. . . . . . . . . . . . . . .  C ADDR  8379H

CLK. . . . . . . . . . . . . . .  C ADDR  8360H

CLKDELAY . . . . . . . . . . . .  C ADDR  835EH

CLR_POLL . . . . . . . . . . . .  C ADDR  82FBH

CLR_SS . . . . . . . . . . . . .  C ADDR  830DH

COMPCOUNT. . . . . . . . . . . .    REG7

COMP_INIT. . . . . . . . . . . .  C ADDR  8285H

COMRD. . . . . . . . . . . . . .  C ADDR  8244H  NOT USED

COUT . . . . . . . . . . . . . .  C ADDR  8381H

DATABYTE . . . . . . . . . . . .    REG4

DELAY. . . . . . . . . . . . . .  C ADDR  8365H

DELAY1 . . . . . . . . . . . . .  C ADDR  8367H

DELAY10. . . . . . . . . . . . .  C ADDR  8363H

DELAY2 . . . . . . . . . . . . .  C ADDR  836EH

DELAY3 . . . . . . . . . . . . .  C ADDR  8370H

DELAY4 . . . . . . . . . . . . .  C ADDR  8372H

DONE8. . . . . . . . . . . . . .  C ADDR  8330H  NOT USED

DPH. . . . . . . . . . . . . . .  D ADDR  0083H  PREDEFINED

DPL. . . . . . . . . . . . . . .  D ADDR  0082H  PREDEFINED

EA . . . . . . . . . . . . . . .  B ADDR  00AFH  PREDEFINED

END_T2 . . . . . . . . . . . . .  C ADDR  833EH

EOC. . . . . . . . . . . . . . .    NUMB  00C0H

ET2. . . . . . . . . . . . . . .  B ADDR  00ADH  PREDEFINED

EXIT_T2. . . . . . . . . . . . .  C ADDR  835DH

LNGDELAY . . . . . . . . . . . .  C ADDR  836CH

MS . . . . . . . . . . . . . . .    NUMB  00C2H

P4 . . . . . . . . . . . . . . .  D ADDR  00C0H  PREDEFINED

PINT16A. . . . . . . . . . . . .  C ADDR  83CCH  NOT USED

PINT16B. . . . . . . . . . . . .  C ADDR  83DBH

PINT16C. . . . . . . . . . . . .  C ADDR  83E6H

PINT16D. . . . . . . . . . . . .  C ADDR  83EDH

PINT16E. . . . . . . . . . . . .  C ADDR  83F8H

PINT16F. . . . . . . . . . . . .  C ADDR  83FFH

PINT16G. . . . . . . . . . . . .  C ADDR  840BH

PINT16H. . . . . . . . . . . . .  C ADDR  8410H

PINT16U. . . . . . . . . . . . .  C ADDR  83C1H

PINT16X. . . . . . . . . . . . .  C ADDR  841FH

PINT16Y. . . . . . . . . . . . .  C ADDR  8421H

POLL . . . . . . . . . . . . . .    NUMB  00C6H

POW. . . . . . . . . . . . . . .    NUMB  00C7H

PRINT. . . . . . . . . . . . . .  C ADDR  8389H

PSW. . . . . . . . . . . . . . .  D ADDR  00D0H  PREDEFINED

RDDONE . . . . . . . . . . . . .  C ADDR  832BH

READSDO. . . . . . . . . . . . .  C ADDR  8314H

READ_SDO . . . . . . . . . . . .  C ADDR  8312H

READ_TX. . . . . . . . . . . . .  C ADDR  8399H

RESET. . . . . . . . . . . . . .    NUMB  00C3H

RI . . . . . . . . . . . . . . .  B ADDR  0098H  PREDEFINED

SBUF . . . . . . . . . . . . . .  D ADDR  0099H  PREDEFINED

SCLK . . . . . . . . . . . . . .    NUMB  00C4H

SDO. . . . . . . . . . . . . . .    NUMB  00C5H

SET_POLL . . . . . . . . . . . .  C ADDR  8300H

SND. . . . . . . . . . . . . . .  C ADDR  83ABH

SP . . . . . . . . . . . . . . .  D ADDR  0081H  PREDEFINED

SS . . . . . . . . . . . . . . .    NUMB  00C1H

START. . . . . . . . . . . . . .  C ADDR  8279H

CMPS_INT                                                                                                      PAGE 9

T2_INIT. . . . . . . . . . . . .  C ADDR  82A2H

T2_ISR . . . . . . . . . . . . .  C ADDR  82ADH

TEMP0. . . . . . . . . . . . . .  C ADDR  82C0H

TEMP1. . . . . . . . . . . . . .  C ADDR  82C6H

TEMP2. . . . . . . . . . . . . .  C ADDR  82CCH

TEMP3. . . . . . . . . . . . . .  C ADDR  82D2H

TEMP4. . . . . . . . . . . . . .  C ADDR  82D8H

TEMP5. . . . . . . . . . . . . .  C ADDR  82DEH

TF2. . . . . . . . . . . . . . .  B ADDR  00CFH  PREDEFINED

TH2. . . . . . . . . . . . . . .  D ADDR  00CDH  PREDEFINED

TI . . . . . . . . . . . . . . .  B ADDR  0099H  PREDEFINED

TL2. . . . . . . . . . . . . . .  D ADDR  00CCH  PREDEFINED

TR2. . . . . . . . . . . . . . .  B ADDR  00CAH  PREDEFINED

WAIT . . . . . . . . . . . . . .  C ADDR  8283H

WAIT_EOC . . . . . . . . . . . .  C ADDR  8305H

ZERO . . . . . . . . . . . . . .  C ADDR  8327H

This is code for the compass that does not utilize the interrupt system.

CMPS_TST                                                                                                      PAGE 1

1

2

3            $MOD51GB

4

00C0                 5            EOC     equ     P4.0

00C1                 6            SS      equ     P4.1

00C2                 7            MS      equ     P4.2

00C3                 8            RESET   equ     P4.3

00C4                 9            SCLK    equ     P4.4

00C5                10            SDO     equ     P4.5

00C6                11            POLL    equ     P4.6

00C7                12            POW     equ     P4.7

13

14

8000                  15            org     8000h

16

8000 D2C7             17            setb    POW

8002 D2C6             18            setb    POLL

8004 D2C1             19            setb    SS

8006 D2C3             20            setb    RESET

8008 D2C4             21            setb    SCLK

800A D2C5             22            setb    SDO

800C D2C0             23            setb    EOC

24

800E C2C7             25            clr     POW             ;turn power on

26

8010 1280A7           27            call   delay10         ;wait for it to stabalize

28

8013 C2C3             29            clr     RESET           ;reset compass

30

8015 1280A7           31            call   delay10        ;wait 10msec

32

8018 D2C3             33            setb    RESET           ;clear reset

34

801A 1280B9           35            call   lngDelay       ;wait .5secs

36

801D D2C2             37            setb    MS

38

801F 1280D6           39    ComRd:  call    Print

8022 5C6E2020         40            db      '\n     Press the g key to get a compass heading\n',0,

8026 20202050

802A 72657373

802E 20746865

8032 2067206B

8036 65792074

803A 6F206765

803E 74206120

8042 636F6D70

8046 61737320

804A 68656164

804E 696E675C

8052 6E00

41

8054 1280C6           42    start:  call    cin

8057 B467FA           43            cjne    A,#'g',start

44

805A C2C6             45            clr     POLL              ;clr POLL

46

CMPS_TST                                                                                                      PAGE 2

805C 1280A7           47            call   delay10

48

805F D2C6             49            setb    POLL              ;set POLL

50

8061 30C0FD           51    waitEOC: jnb    EOC, waitEOC    ;wait for EOC to reset

52

8064 1280A7           53            call   delay10         ;wait >10msecs

54

8067 C2C1             55            clr     SS              ; set slave select for data recovery

56

8069 1280B0           57            call   delay5         ;wait >5msecs

58

806C 7D02             59            mov     R5,#02

806E 7E08             60    SDOrd:  mov     R6,#08

61

8070 C2C4             62    ReadSDO: clr    SCLK

8072 1280A2           63            call   clkdelay

8075 D2C4             64            setb    SCLK

8077 7420             65            mov     A, #020h          ;set A.5

8079 55C0             66            anl     A,P4            ;compare A.3 with P4.3

807B 6006             67            jz      zero            ;jump if P4.3 is not set

68

807D EC               69            mov     A, R4           ;load R4 into A

807E D3               70            setb    C               ;set the carry bit

807F 33               71            rlc     A               ;roll A left with carry

8080 FC               72            mov     R4, A           ;put contents of A back into R4

8081 0187             73            ajmp    rdDone          ; done with read

74

8083 EC               75    zero:   mov     A, R4           ;load R4 into A

8084 C3               76            clr     C               ;clear the carry bit

8085 33               77            rlc     A               ;roll A left with carry

8086 FC               78            mov     R4, A           ;put contents of A back into R4

79

8087 1280A2           80    rdDone: call   clkdelay                ;delay for low clock

808A DEE4             81            djnz    R6, ReadSDO

82

808C DD0C             83    done8:  djnz    R5, hlfdelay    ; Read second half of data

84

808E D2C1             85            setb    SS              ; Slave select off

86

8090 8F83             87            mov     DPH, R7

8092 8C82             88            mov     DPL, R4

8094 128117           89            call    pint16u

90

8097 02801F           91            ljmp    ComRd

92

93

809A 1280B0           94    hlfdelay: call delay5         ;Wait >5msecs

809D EC               95            mov     A, R4           ;transfer MSB to R7

809E FF               96            mov     R7, A           ;      "    "

809F 02806E           97            ljmp    SDOrd          ;Start next 8 bits

98

80A2 7873             99    clkdelay: mov   R0,#073h

80A4 D8FE            100    clk:    djnz    R0,clk

80A6 22              101            ret

102

80A7 7825            103    delay10: mov    R0,#025h

80A9 79FF            104    delay:  mov     R1,#0FFh

CMPS_TST                                                                                                      PAGE 3

80AB D9FE            105    delay1: djnz    R1, delay1              ;wait >10 msec

80AD D8FA            106            djnz    R0, delay

80AF 22              107            ret

108

80B0 7812            109    delay5: mov    R0,#012h

80B2 79FF            110    del:  mov     R1,#0FFh

80B4 D9FE            111    del1: djnz    R1, del1              ;wait >5 msec

80B6 D8FA            112            djnz    R0, del

80B8 22              113            ret

114

115

116

80B9 780A            117    lngDelay: mov   R0,#00Ah

80BB 7935            118    delay2: mov     R1,#035h

80BD 7AFF            119    delay3: mov     R2,#0FFh

80BF DAFE            120    delay4: djnz    R2, delay4              ;wait >500msec

80C1 D9FA            121            djnz    R1, delay3

80C3 D8F6            122            djnz    R0, delay2

80C5 22              123            ret

124

125

80C6 3098FD          126    cin:    JNB     RI,$             ; WAIT FOR THE RI FLAG TO SET

80C9 C298            127            CLR     RI               ; CLEAR THE RECEIVE FLAG ONCE RECIEVED

80CB E599            128            MOV     A,SBUF           ; PLACE RECEIVED BYTE IN REGISTER A

80CD 22              129            RET

130

131

132

80CE 3099FD          133    cout:   JNB     TI,$            ;Wait until transmission completed.

80D1 C299            134            CLR     TI              ;Clear interrupt flag.

80D3 F599            135            MOV     SBUF,A          ;Write out character.

80D5 22              136            RET

137

80D6 C0E0            138    PRINT:  PUSH    ACC      ; push the A and DPTR registers so their contents

80D8 C083            139            PUSH    DPH      ; don't get lost in case they are being used

80DA C082            140            PUSH    DPL      ; before calling this function.

141

142    ; move the stack pointer down low in order to pop the first string's byte

143    ; address

80DC E581            144            MOV     A,SP

80DE 24FD            145            ADD     A,#0FDH

80E0 F581            146            MOV     SP,A

80E2 D083            147            POP     DPH

80E4 D082            148            POP     DPL

149

150    ; send (PRINT) the string back to the PC computer

80E6                 151    READ_TX:

80E6 E4              152            CLR     A

80E7 93              153            MOVC    A,@A+DPTR

80E8 A3              154            INC     DPTR

155

80E9 B45C0C          156            CJNE    A,#'\',SND

157

80EC E4              158            CLR     A

80ED 93              159            MOVC    A,@A+DPTR

80EE A3              160            INC     DPTR

161

80EF B46E06          162            CJNE    A,#'n',SND

CMPS_TST                                                                                                      PAGE 4

163

80F2 740D            164            MOV     A,#0DH

80F4 11CE            165            ACALL   cout

80F6 740A            166            MOV     A,#0AH

167

80F8 11CE            168    SND:    ACALL   cout

80FA B400E9          169            CJNE    A,#0,READ_TX

170

171    ; place the new return addres in stack

80FD C082            172            PUSH    DPL

80FF C083            173            PUSH    DPH

174

175    ; move the stack pointer up high in order to pop the A and DPTR registers

8101 E581            176            MOV     A,SP

8103 2403            177            ADD     A,#3

8105 F581            178            MOV     SP,A

8107 D082            179            POP     DPL

8109 D083            180            POP     DPH

810B D0E0            181            POP     ACC

810D 22              182            RET

183

810E 540F            184    UTIL_BINTOASC:  anl     a,#00fh                 ; Keep Only Low Bits

8110 2490            185                    add     a,#090h                 ; Add 144

8112 D4              186                    da      a                       ; Decimal Adjust

8113 3440            187                    addc    a,#040h                 ; Add 64

8115 D4              188                    da      a                       ; Decimal Adjust

8116 22              189                    ret                             ; Return To Caller

190

191            ;print 16 bit unsigned integer in DPTR, using base 10.

8117                 192    pint16u:        ;warning, destroys r2, r3, r4, r5, psw.5

8117 C0E0            193            push    acc

8119 E8              194            mov     a, r0

811A C0E0            195            push    acc

811C C2D5            196            clr     psw.5

811E AA82            197            mov     r2, dpl

8120 AB83            198            mov     r3, dph

199

8122 7C10            200    pint16a:mov     r4, #16 ;ten-thousands digit

8124 7D27            201            mov     r5, #39

8126 3175            202            acall   pint16x

8128 6007            203            jz      pint16b

812A 2430            204            add     a, #'0'

812C 1280CE          205            lcall   cout

812F D2D5            206            setb    psw.5

207

8131 7CE8            208    pint16b:mov     r4, #232        ;thousands digit

8133 7D03            209            mov     r5, #3

8135 3175            210            acall   pint16x

8137 7003            211            jnz     pint16c

8139 30D507          212            jnb     psw.5, pint16d

813C 2430            213    pint16c:add     a, #'0'

813E 1280CE          214            lcall   cout

8141 D2D5            215            setb    psw.5

216

8143 7C64            217    pint16d:mov     r4, #100        ;hundreds digit

8145 7D00            218            mov     r5, #0

8147 3175            219            acall   pint16x

8149 7003            220            jnz     pint16e

CMPS_TST                                                                                                      PAGE 5

814B 30D507          221            jnb     psw.5, pint16f

814E 2430            222    pint16e:add     a, #'0'

8150 1280CE          223            lcall   cout

8153 D2D5            224            setb    psw.5

225

8155 EA              226    pint16f:mov     a, r2           ;tens digit

8156 ABF0            227            mov     r3, b

8158 75F00A          228            mov     b, #10

815B 84              229            div     ab

815C 7003            230            jnz     pint16g

815E 30D505          231            jnb     psw.5, pint16h

8161 2430            232    pint16g:add     a, #'0'

8163 1280CE          233            lcall   cout

234

8166 E5F0            235    pint16h:mov     a, b            ;and finally the ones digit

8168 8BF0            236            mov     b, r3

816A 2430            237            add     a, #'0'

816C 1280CE          238            lcall   cout

239

816F D0E0            240            pop     acc

8171 F8              241            mov     r0, a

8172 D0E0            242            pop     acc

8174 22              243            ret

244

245    ;ok, it's a cpu hog and a nasty way to divide, but this code

246    ;requires only 21 bytes!  Divides r2-r3 by r4-r5 and leaves

247    ;quotient in r2-r3 and returns remainder in acc.  If Intel

248    ;had made a proper divide, then this would be much easier.

249

8175 7800            250    pint16x:mov     r0, #0

8177 08              251    pint16y:inc     r0

8178 C3              252            clr     c

8179 EA              253            mov     a, r2

817A 9C              254            subb    a, r4

817B FA              255            mov     r2, a

817C EB              256            mov     a, r3

817D 9D              257            subb    a, r5

817E FB              258            mov     r3, a

817F 50F6            259            jnc     pint16y

8181 18              260            dec     r0

8182 EA              261            mov     a, r2

8183 2C              262            add     a, r4

8184 FA              263            mov     r2, a

8185 EB              264            mov     a, r3

8186 3D              265            addc    a, r5

8187 FB              266            mov     r3, a

8188 E8              267            mov     a, r0

8189 22              268            ret

269

270            END

VERSION 1.2h ASSEMBLY COMPLETE, 0 ERRORS FOUND

CMPS_TST                                                                                                      PAGE 6

ACC. . . . . . . . . . . . . . .  D ADDR  00E0H  PREDEFINED

B. . . . . . . . . . . . . . . .  D ADDR  00F0H  PREDEFINED

CIN. . . . . . . . . . . . . . .  C ADDR  80C6H

CLK. . . . . . . . . . . . . . .  C ADDR  80A4H

CLKDELAY . . . . . . . . . . . .  C ADDR  80A2H

COMRD. . . . . . . . . . . . . .  C ADDR  801FH

COUT . . . . . . . . . . . . . .  C ADDR  80CEH

DEL. . . . . . . . . . . . . . .  C ADDR  80B2H

DEL1 . . . . . . . . . . . . . .  C ADDR  80B4H

DELAY. . . . . . . . . . . . . .  C ADDR  80A9H

DELAY1 . . . . . . . . . . . . .  C ADDR  80ABH

DELAY10. . . . . . . . . . . . .  C ADDR  80A7H

DELAY2 . . . . . . . . . . . . .  C ADDR  80BBH

DELAY3 . . . . . . . . . . . . .  C ADDR  80BDH

DELAY4 . . . . . . . . . . . . .  C ADDR  80BFH

DELAY5 . . . . . . . . . . . . .  C ADDR  80B0H

DONE8. . . . . . . . . . . . . .  C ADDR  808CH  NOT USED

DPH. . . . . . . . . . . . . . .  D ADDR  0083H  PREDEFINED

DPL. . . . . . . . . . . . . . .  D ADDR  0082H  PREDEFINED

EOC. . . . . . . . . . . . . . .    NUMB  00C0H

HLFDELAY . . . . . . . . . . . .  C ADDR  809AH

LNGDELAY . . . . . . . . . . . .  C ADDR  80B9H

MS . . . . . . . . . . . . . . .    NUMB  00C2H

P4 . . . . . . . . . . . . . . .  D ADDR  00C0H  PREDEFINED

PINT16A. . . . . . . . . . . . .  C ADDR  8122H  NOT USED

PINT16B. . . . . . . . . . . . .  C ADDR  8131H

PINT16C. . . . . . . . . . . . .  C ADDR  813CH

PINT16D. . . . . . . . . . . . .  C ADDR  8143H

PINT16E. . . . . . . . . . . . .  C ADDR  814EH

PINT16F. . . . . . . . . . . . .  C ADDR  8155H

PINT16G. . . . . . . . . . . . .  C ADDR  8161H

PINT16H. . . . . . . . . . . . .  C ADDR  8166H

PINT16U. . . . . . . . . . . . .  C ADDR  8117H

PINT16X. . . . . . . . . . . . .  C ADDR  8175H

PINT16Y. . . . . . . . . . . . .  C ADDR  8177H

POLL . . . . . . . . . . . . . .    NUMB  00C6H

POW. . . . . . . . . . . . . . .    NUMB  00C7H

PRINT. . . . . . . . . . . . . .  C ADDR  80D6H

PSW. . . . . . . . . . . . . . .  D ADDR  00D0H  PREDEFINED

RDDONE . . . . . . . . . . . . .  C ADDR  8087H

READSDO. . . . . . . . . . . . .  C ADDR  8070H

READ_TX. . . . . . . . . . . . .  C ADDR  80E6H

RESET. . . . . . . . . . . . . .    NUMB  00C3H

RI . . . . . . . . . . . . . . .  B ADDR  0098H  PREDEFINED

SBUF . . . . . . . . . . . . . .  D ADDR  0099H  PREDEFINED

SCLK . . . . . . . . . . . . . .    NUMB  00C4H

SDO. . . . . . . . . . . . . . .    NUMB  00C5H

SDORD. . . . . . . . . . . . . .  C ADDR  806EH

SND. . . . . . . . . . . . . . .  C ADDR  80F8H

SP . . . . . . . . . . . . . . .  D ADDR  0081H  PREDEFINED

SS . . . . . . . . . . . . . . .    NUMB  00C1H

START. . . . . . . . . . . . . .  C ADDR  8054H

TI . . . . . . . . . . . . . . .  B ADDR  0099H  PREDEFINED

UTIL_BINTOASC. . . . . . . . . .  C ADDR  810EH  NOT USED

WAITEOC. . . . . . . . . . . . .  C ADDR  8061H

ZERO . . . . . . . . . . . . . .  C ADDR  8083H

This program will test the A/D inputs.  In this case that includes all three IR’s and the sonar.

AD0123                                                                                                        PAGE 1

1

2    ; --- WARNING:  DO NOT APPLY MORE THAN 5 VOLTS TO ANY OF THE A/D INPUTS

3

4            $MOD51GB

5

8000                   6            ORG     8000H

7

8000 128033            8    START:  CALL   CIN

9    ;        MOV     P1,#000h

8003 B473FA           10            CJNE    A,#'s',START

11

12

13    ;                       AIF            A/D Interrupt Flag

14    ;                       |ACE           A/D Conversion Enable

15    ;                       ||ACS1         A/D Channel Select 1

16    ;                       |||ACS0        A/D Channel Select 0

17    ;                       ||||AIM        A/D Input Mode

18    ;                       |||||ATM       A/D Triger Mode

19    ;                       ||||||

20    ;                       ||||||

8006 759710           21            MOV     ACON,#00010000B   ; ENABLE A/D OPERATION

22

8009                  23    WAIT:

24

8009 7401             25            MOV     A,#1

800B 113B             26            ACALL   COUT          ; THROUGH THE SERIAL PORT

800D E584             27            MOV     A,AD0         ; ALSO SEND THE READING

800F 113B             28            ACALL   COUT          ; THROUGH THE SERIAL PORT

29

8011 7402             30            MOV     A,#2

8013 113B             31            ACALL   COUT          ; THROUGH THE SERIAL PORT

8015 E594             32            MOV     A,AD1         ; ALSO SEND THE READING

8017 113B             33            ACALL   COUT          ; THROUGH THE SERIAL PORT

34

8019 7403             35            MOV     A,#3

801B 113B             36            ACALL   COUT          ; THROUGH THE SERIAL PORT

801D E5A4             37            MOV     A,AD2         ; ALSO SEND THE READING

801F 113B             38            ACALL   COUT          ; THROUGH THE SERIAL PORT

39

8021 7404             40            MOV     A,#4

8023 113B             41            ACALL   COUT          ; THROUGH THE SERIAL PORT

8025 E5B4             42            MOV     A,AD3         ; ALSO SEND THE READING

8027 113B             43            ACALL   COUT          ; THROUGH THE SERIAL PORT

44

8029 12802E           45            CALL    DELAY

802C 80DB             46            SJMP    WAIT

47

802E DAFE             48    DELAY:  DJNZ    R2,$

8030 DBFC             49            DJNZ    R3,DELAY

8032 22               50            RET

51

8033                  52    CIN:

8033 3098FD           53            JNB     RI,$            ; WAIT FOR THE TI FLAG TO SET

8036 C298             54            CLR     RI               ; CLEAR THE TI FLAG ONCE SENT

8038 E599             55            MOV     A,SBUF           ; SEND THE CONTENTS OF REGISTER A

803A 22               56            RET                      ; RETURN

803B                  57    COUT:

803B 3099FD           58            JNB     TI,$            ; WAIT FOR THE TI FLAG TO SET

AD0123                                                                                                        PAGE 2

803E C299             59            CLR     TI               ; CLEAR THE TI FLAG ONCE SENT

8040 F599             60            MOV     SBUF,A           ; SEND THE CONTENTS OF REGISTER A

8042 22               61            RET                      ; RETURN

62

63            END

VERSION 1.2h ASSEMBLY COMPLETE, 0 ERRORS FOUND

AD0123                                                                                                        PAGE 3

ACON . . . . . . . . . . . . . .  D ADDR  0097H  PREDEFINED

AD0. . . . . . . . . . . . . . .  D ADDR  0084H  PREDEFINED

AD1. . . . . . . . . . . . . . .  D ADDR  0094H  PREDEFINED

AD2. . . . . . . . . . . . . . .  D ADDR  00A4H  PREDEFINED

AD3. . . . . . . . . . . . . . .  D ADDR  00B4H  PREDEFINED

CIN. . . . . . . . . . . . . . .  C ADDR  8033H

COUT . . . . . . . . . . . . . .  C ADDR  803BH

DELAY. . . . . . . . . . . . . .  C ADDR  802EH

RI . . . . . . . . . . . . . . .  B ADDR  0098H  PREDEFINED

SBUF . . . . . . . . . . . . . .  D ADDR  0099H  PREDEFINED

START. . . . . . . . . . . . . .  C ADDR  8000H

TI . . . . . . . . . . . . . . .  B ADDR  0099H  PREDEFINED

WAIT . . . . . . . . . . . . . .  C ADDR  8009H

This program tests the motor controller.  It will allow you to run the motor up and down, and it will allow you to shift back and forth into reverse and forward.

PWM_MOT                                                                                                       PAGE 1

1    ; NOTE:  THIS IS A NON-BOOTUP PROGRAM, IT DOESN'T HAVE LINES TO CLEAR THE WDT

2    ; SINCE THAT'S BEEN TAKEN CARE OF BY THE SMALL-C MONITOR.

3

4            $MOD51GB

5

6    PWMCNT  EQU     R2                  ; use R2 as the PMW duty cycle counter

7

8000                   8            ORG     8000H

9

8000 7590FF           10            MOV     P1,#00FFH           ; IN ORDER FOR THE PWM SIGNAL TO TAKE EFFECT,

11                                        ; ALL OUTPUT BITS ON PORT 1 MUST BE SET HIGH

12

8003 7A7F             13            MOV     PWMCNT,#07FH         ; START THE PW VALUE WITH 01

8005 8AFC             14            MOV     CCAP2H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

8007 128039           15            CALL    PCA_INIT            ; INITIALIZE THE PWM

16

800A                  17    MAIN_LOOP:

18

19

800A 128044           20            CALL    CIN                 ; WAIT FOR A KEY TO BE PRESSED

800D B43105           21            CJNE    A,#'1',K2           ; IF KEY='1'

8010 0A               22            INC     PWMCNT              ; INCREMENT THE PULSE WIDTH TIMER VALUE

8011 8AFC             23            MOV     CCAP2H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

8013 80F5             24            SJMP    MAIN_LOOP

25

8015 B43205           26    K2:     CJNE    A,#'2',K3           ; IF KEY='2'

8018 1A               27            DEC     PWMCNT              ; DECREMENT THE PULSE WITH TIMER VALUE

8019 8AFC             28            MOV     CCAP2H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

801B 80ED             29            SJMP    MAIN_LOOP

30

801D B43304           31    K3:     CJNE    A,#'3',K4           ; IF KEY='3'

8020 C2F9             32            CLR     P5.1                ; TURN OFF MOTOR

8022 80E6             33            SJMP    MAIN_LOOP

34

35

8024 B43404           36    K4:     CJNE    A,#'4',KQ           ; IF KEY='4'

8027 D2F9             37            SETB    P5.1                ; TURN ON MOTOR

8029 80DF             38            SJMP    MAIN_LOOP

39

40

802B B47104           41    KQ:     CJNE    A,#'q',KW               ; IF KEY='q'

802E C2F8             42            CLR     P5.0                    ; GO FORWARD

8030 80D8             43            SJMP    MAIN_LOOP

44

8032 B477D5           45    KW:     CJNE    A,#'w',MAIN_LOOP        ; IF KEY='w'

8035 D2F8             46            SETB    P5.0                    ; GO BACKWARD

8037 80D1             47            SJMP    MAIN_LOOP

48

8039                  49    PCA_INIT:

8039 75D906           50            MOV     CMOD,#006H          ; PCA FREQUENCY = 1/Timer 0 overflow

803C 75DC42           51            MOV     CCAPM2,#42H         ; PUT PCA MODULE 2 INTO PWM MODE

803F 8AFC             52            MOV     CCAP2H, PWMCNT      ; INITIALIZE DUTY CYCLE

8041 D2DE             53            SETB    CR                  ; START THE PCA CLOCK

8043 22               54            RET

55

8044                  56    CIN:

8044 3098FD           57            JNB     RI,$             ; WAIT FOR A BYTE

8047 C298             58            CLR     RI               ; CLEAR THE RECEIVE FLAG ONCE RECIEVED

PWM_MOT                                                                                                       PAGE 2

8049 E599             59            MOV     A,SBUF           ; GET THE RECEIVED BYTE INTO THE A REGISTER

804B 22               60            RET

61

62

63            END

64

VERSION 1.2h ASSEMBLY COMPLETE, 0 ERRORS FOUND

PWM_MOT                                                                                                       PAGE 3

CCAP2H . . . . . . . . . . . . .  D ADDR  00FCH  PREDEFINED

CCAPM2 . . . . . . . . . . . . .  D ADDR  00DCH  PREDEFINED

CIN. . . . . . . . . . . . . . .  C ADDR  8044H

CMOD . . . . . . . . . . . . . .  D ADDR  00D9H  PREDEFINED

CR . . . . . . . . . . . . . . .  B ADDR  00DEH  PREDEFINED

K2 . . . . . . . . . . . . . . .  C ADDR  8015H

K3 . . . . . . . . . . . . . . .  C ADDR  801DH

K4 . . . . . . . . . . . . . . .  C ADDR  8024H

KQ . . . . . . . . . . . . . . .  C ADDR  802BH

KW . . . . . . . . . . . . . . .  C ADDR  8032H

MAIN_LOOP. . . . . . . . . . . .  C ADDR  800AH

P1 . . . . . . . . . . . . . . .  D ADDR  0090H  PREDEFINED

P5 . . . . . . . . . . . . . . .  D ADDR  00F8H  PREDEFINED

PCA_INIT . . . . . . . . . . . .  C ADDR  8039H

PWMCNT . . . . . . . . . . . . .    REG2

RI . . . . . . . . . . . . . . .  B ADDR  0098H  PREDEFINED

SBUF . . . . . . . . . . . . . .  D ADDR  0099H  PREDEFINED

This program tests the steering servo.  It will allow you center the wheels, turn them full left or right, and to step them left or right.

PWM_STR                                                                                                       PAGE 1

1

2    ; NOTE:  THIS IS A NON-BOOTUP PROGRAM, IT DOESN'T HAVE LINES TO CLEAR THE WDT

3    ; SINCE THAT'S BEEN TAKEN CARE OF BY THE SMALL-C MONITOR.

4

5            $MOD51GB

6

7    PWMCNT  EQU     R2                  ; use R2 as the PMW duty cycle counter

8

8000                   9            ORG     8000H

10

8000 7590FF           11            MOV     P1,#00FFH           ; IN ORDER FOR THE PWM SIGNAL TO TAKE EFFECT,

12                                        ; ALL OUTPUT BITS ON PORT 1 MUST BE SET HIGH

13

8003 7A3B             14            MOV     PWMCNT,#03BH         ; START THE PW VALUE WITH 01

8005 8AFB             15            MOV     CCAP1H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

8007 128038           16            CALL    PCA_INIT            ; INITIALIZE THE PWM

17

800A                  18    MAIN_LOOP:

19

800A 128043           20            CALL    CIN                 ; WAIT FOR A KEY TO BE PRESSED

800D B43106           21            CJNE    A,#'1',K2           ; IF KEY='1'

8010 7A31             22            mov     PWMCNT,#031h        ; Turn wheels full left

8012 8AFB             23            MOV     CCAP1H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

8014 80F4             24            SJMP    MAIN_LOOP

25

8016 B43206           26    K2:     CJNE    A,#'2',K3           ; IF KEY='2'

8019 7A3B             27            mov     PWMCNT,#03bh        ; center wheels

801B 8AFB             28            MOV     CCAP1H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

801D 80EB             29            SJMP    MAIN_LOOP

30

801F B43306           31    K3:     CJNE    A,#'3',KQ           ; IF KEY='3'

8022 7A4B             32            mov     PWMCNT,#04bh        ; Turn the wheels full right

8024 8AFB             33            MOV     CCAP1H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

8026 80E2             34            SJMP    MAIN_LOOP

35

8028 B47105           36    KQ:     CJNE    A,#'q',KW           ; IF KEY='q'

802B 0A               37            inc     PWMCNT              ; tick wheels to left

802C 8AFB             38            MOV     CCAP1H, PWMCNT      ; LOAD THE PWM R

802E 80DA             39            SJMP    MAIN_LOOP

40

8030 B477D7           41    KW:     CJNE    A,#'w',MAIN_LOOP

8033 1A               42            dec     PWMCNT

8034 8AFB             43            MOV     CCAP1H,PWMCNT

8036 80D2             44            SJMP    MAIN_LOOP

45

8038                  46    PCA_INIT:

8038 75D906           47            MOV     CMOD,#006h

803B 75DB42           48            MOV     CCAPM1,#42h

803E 8AFB             49            MOV     CCAP1H,PWMCNT

8040 D2DE             50            SETB    CR

8042 22               51            RET

52

8043                  53    CIN:

8043 3098FD           54            JNB     RI,$             ; WAIT FOR A BYTE

8046 C298             55            CLR     RI               ; CLEAR THE RECEIVE FLAG ONCE RECIEVED

8048 E599             56            MOV     A,SBUF           ; GET THE RECEIVED BYTE INTO THE A REGISTER

804A 22               57            RET

58

PWM_STR                                                                                                       PAGE 2

59

60            END

61

VERSION 1.2h ASSEMBLY COMPLETE, 0 ERRORS FOUND

PWM_STR                                                                                                       PAGE 3

CCAP1H . . . . . . . . . . . . .  D ADDR  00FBH  PREDEFINED

CCAPM1 . . . . . . . . . . . . .  D ADDR  00DBH  PREDEFINED

CIN. . . . . . . . . . . . . . .  C ADDR  8043H

CMOD . . . . . . . . . . . . . .  D ADDR  00D9H  PREDEFINED

CR . . . . . . . . . . . . . . .  B ADDR  00DEH  PREDEFINED

K2 . . . . . . . . . . . . . . .  C ADDR  8016H

K3 . . . . . . . . . . . . . . .  C ADDR  801FH

KQ . . . . . . . . . . . . . . .  C ADDR  8028H

KW . . . . . . . . . . . . . . .  C ADDR  8030H

MAIN_LOOP. . . . . . . . . . . .  C ADDR  800AH

P1 . . . . . . . . . . . . . . .  D ADDR  0090H  PREDEFINED

PCA_INIT . . . . . . . . . . . .  C ADDR  8038H

PWMCNT . . . . . . . . . . . . .    REG2

RI . . . . . . . . . . . . . . .  B ADDR  0098H  PREDEFINED

SBUF . . . . . . . . . . . . . .  D ADDR  0099H  PREDEFINED

This program tests the sonar servo.  It will allow you to place the sonar in any one of seven different positions from any other position.

PWM_SON                                                                                                       PAGE 1

1

2    ; NOTE:  THIS IS A NON-BOOTUP PROGRAM, IT DOESN'T HAVE LINES TO CLEAR THE WDT

3    ; SINCE THAT'S BEEN TAKEN CARE OF BY THE SMALL-C MONITOR.

4

5            $MOD51GB

6

7    PWMCNT  EQU     R2                  ; use R2 as the PMW duty cycle counter

8

8000                   9            ORG     8000H

10

8000 7590FF           11            MOV     P1,#00FFH           ; IN ORDER FOR THE PWM SIGNAL TO TAKE EFFECT,

12                                        ; ALL OUTPUT BITS ON PORT 1 MUST BE SET HIGH

13

8003 7A3B             14            MOV     PWMCNT,#03BH         ; START THE PW VALUE WITH 01

8005 8AFA             15            MOV     CCAP0H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

8007 12804C           16            CALL    PCA_INIT            ; INITIALIZE THE PWM

17

800A                  18    MAIN_LOOP:

19

800A 128057           20            CALL    CIN                 ; WAIT FOR A KEY TO BE PRESSED

800D B43106           21            CJNE    A,#'1',K2           ; IF KEY='1'

8010 7A62             22            MOV     PWMCNT,#062h        ; Place the sonar at 90 left

8012 8AFA             23            MOV     CCAP0H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

8014 80F4             24            SJMP    MAIN_LOOP

25

8016 B43206           26    K2:     CJNE    A,#'2',K3           ; IF KEY='2'

8019 7A56             27            MOV     PWMCNT,#056h        ; Place the sonar at 60 left

801B 8AFA             28            MOV     CCAP0H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

801D 80EB             29            SJMP    MAIN_LOOP

30

801F B43306           31    K3:     CJNE    A,#'3',K4           ; IF KEY='3'

8022 7A48             32            MOV     PWMCNT,#048h        ; Place the sonar at 30 left

8024 8AFA             33            MOV     CCAP0H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

8026 80E2             34            SJMP    MAIN_LOOP

35

8028 B43406           36    K4:     CJNE    A,#'4',K5           ; IF KEY='4'

802B 7A3B             37            MOV     PWMCNT,#03bh        ; Place the sonar at 0

802D 8AFA             38            MOV     CCAP0H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

802F 80D9             39            SJMP    MAIN_LOOP

40

8031 B43506           41    K5:     CJNE    A,#'5',K6           ; IF KEY='5'

8034 7A2D             42            MOV     PWMCNT,#02dh        ; Place the sonar at 30 right

8036 8AFA             43            MOV     CCAP0H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

8038 80D0             44            SJMP    MAIN_LOOP

45

803A B43606           46    K6:     CJNE    A,#'6',K7           ; IF KEY='6'

803D 7A21             47            MOV     PWMCNT,#021h        ; Place the sonar at 60 right

803F 8AFA             48            MOV     CCAP0H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

8041 80C7             49            SJMP    MAIN_LOOP

50

8043 B437C4           51    K7:     CJNE    A,#'7',MAIN_LOOP    ; IF KEY='7'

8046 7A15             52            MOV     PWMCNT,#015h        ; Place the sonar at 90 right

8048 8AFA             53            MOV     CCAP0H, PWMCNT      ; LOAD THE PWM REGISTER WITH THE PW VALUE

804A 80BE             54            SJMP    MAIN_LOOP

55

804C                  56    PCA_INIT:

804C 75D906           57            MOV     CMOD,#006H          ; PCA FREQUENCY = 1/Timer 0 overflow

804F 75DA42           58            MOV     CCAPM0,#42H         ; PUT PCA MODULE 0 INTO PWM MODE

PWM_SON                                                                                                       PAGE 2

8052 8AFA             59            MOV     CCAP0H, PWMCNT      ; INITIALIZE DUTY CYCLE

8054 D2DE             60            SETB    CR                  ; START THE PCA CLOCK

8056 22               61            RET

62

8057                  63    CIN:

8057 3098FD           64            JNB     RI,$             ; WAIT FOR A BYTE

805A C298             65            CLR     RI               ; CLEAR THE RECEIVE FLAG ONCE RECIEVED

805C E599             66            MOV     A,SBUF           ; GET THE RECEIVED BYTE INTO THE A REGISTER

805E 22               67            RET

68

69

70            END

71

VERSION 1.2h ASSEMBLY COMPLETE, 0 ERRORS FOUND

PWM_SON                                                                                                       PAGE 3

CCAP0H . . . . . . . . . . . . .  D ADDR  00FAH  PREDEFINED

CCAPM0 . . . . . . . . . . . . .  D ADDR  00DAH  PREDEFINED

CIN. . . . . . . . . . . . . . .  C ADDR  8057H

CMOD . . . . . . . . . . . . . .  D ADDR  00D9H  PREDEFINED

CR . . . . . . . . . . . . . . .  B ADDR  00DEH  PREDEFINED

K2 . . . . . . . . . . . . . . .  C ADDR  8016H

K3 . . . . . . . . . . . . . . .  C ADDR  801FH

K4 . . . . . . . . . . . . . . .  C ADDR  8028H

K5 . . . . . . . . . . . . . . .  C ADDR  8031H

K6 . . . . . . . . . . . . . . .  C ADDR  803AH

K7 . . . . . . . . . . . . . . .  C ADDR  8043H

MAIN_LOOP. . . . . . . . . . . .  C ADDR  800AH

P1 . . . . . . . . . . . . . . .  D ADDR  0090H  PREDEFINED

PCA_INIT . . . . . . . . . . . .  C ADDR  804CH

PWMCNT . . . . . . . . . . . . .    REG2

RI . . . . . . . . . . . . . . .  B ADDR  0098H  PREDEFINED

SBUF . . . . . . . . . . . . . .  D ADDR  0099H  PREDEFINED

This program was written to test the IR sensor collision avoidance.  It is the direct ancestor of the sonar navigation program.

LO_SPD2                                                                                                       PAGE 1

8000                   1            org     8000h

8000 028200            2            jmp     8200h

3

4            $MOD51GB

5

6    PWM_MOT EQU     R0

7    PWM_STR EQU     R1

00F9                 8    STP_GO  EQU     P5.1

00F8                 9    FOR_REV EQU     P5.0

10

11

8200                  12            org     8200h

8200                  13    main:

14            ;enable A/D operation

8200 759710           15            mov     ACON,#010h

16

17            ;STP_GO is cleared to stop motor

18            ;FOR_REV is set to go in reverse

19

20            ;initalize the motor and steering PWM waveforms

8203 7590FF           21            mov     P1,#0FFH        ;must set all bits on port1 to 0

8206 78C0             22            mov     PWM_MOT,#0c0h   ;set motor for 1/8 speed

8208 88FC             23            mov     CCAP2h,PWM_MOT

820A 793B             24            mov     PWM_STR,#03Bh   ;set steering servo to center

820C 89FB             25            mov     CCAP1h,PWM_STR

820E C2F8             26            clr     FOR_REV           ;set relays for FORWARD

8210 C2F9             27            clr     STP_GO          ;block PWM signal to motor

8212 128280           28            call    PCA_INIT

29

8215 D2F9             30            setb    STP_GO

8217 12828C           31    IRScan:   call    lngDelay

32

821A C3               33    L_Rear: clr     C             ; Clear carry bit

821B E594             34            mov     A,AD1         ; get the left rear IR

821D 9464             35            subb    A,#100        ; subtract threshold from value

821F 4026             36            jc      L_Front       ; jump if value is less than threshold

37

8221 30F921           38            jnb     STP_GO,stp_rev2  ;already stopped

8224 30F81E           39            jnb     FOR_REV,stp_rev2 ;or going forward

8227 C2F9             40            clr     STP_GO           ;stop motor

8229 C2F8             41            clr     FOR_REV          ;place in forward

822B 793B             42            mov     PWM_STR,#03bh    ;center wheels

822D 89FB             43            mov     CCAP1H,PWM_STR   ;

822F 12828C           44            call    lngDelay

8232 78C0             45            mov     PWM_MOT,#0C0h           ;set motor for 1/8 speed

8234 88FC             46            mov     CCAP2h,PWM_MOT

8236 D2F9             47            setb    STP_GO           ;start motor

8238 1282A1           48            call    PRINT

823B 466F7277         49            db      'Forward\n',0,

823F 6172645C

8243 6E00

8245                  50    stp_rev2:

8245 80D0             51            jmp     IRScan

52

53

8247                  54    L_Front:

8247 C3               55            clr     C             ; Clear carry bit

8248 E5A4             56            mov     A,AD2         ; get the left front IR

LO_SPD2                                                                                                       PAGE 2

824A 946E             57            subb    A,#110        ; subtract threshold from value

824C 4003             58            jc      R_Front       ; jump if value is less than threshold

59

824E 028258           60            jmp     forward

61

8251                  62    R_Front:

8251 C3               63            clr     C             ; Clear carry bit

8252 E5B4             64            mov     A,AD3         ; get the left front IR

8254 946E             65            subb    A,#110        ; subtract threshold from value

8256 40BF             66            jc      IRScan        ; jump if value is less than threshold

67

8258                  68    forward:

8258 30F921           69            jnb     STP_GO,stp_rev0         ;already stopped

825B 20F81E           70            jb      FOR_REV,stp_rev0        ;or backing

825E C2F9             71            clr     STP_GO                  ;stop motor

8260 D2F8             72            setb    FOR_REV                 ;reverse it

8262 7931             73            mov     PWM_STR,#031h           ;turn wheels to right

8264 89FB             74            mov     CCAP1H,PWM_STR          ;

8266 12828C           75            call    lngDelay                ;Give it 1/2 a second

8269 7899             76            mov     PWM_MOT,#099h           ;set motor for 1/8 speed

826B 88FC             77            mov     CCAP2h,PWM_MOT

826D D2F9             78            setb    STP_GO                  ;start motor

826F 1282A1           79            call    PRINT

8272 4261636B         80            db      'Backing\n',0,

8276 696E675C

827A 6E00

827C                  81    stp_rev0:

827C 8099             82            jmp     IRScan

83

827E 80FE             84    wait:   jmp     wait

85

86

87

88            ;initializes the programmable counter array for the motor

89            ;and the steering servo

8280                  90    PCA_INIT:

8280 75D906           91            mov     CMOD,#006h      ;set PCA frequency to external

8283 75DB42           92            mov     CCAPM1,#42h     ;put PCA module 1 into PWM mode

8286 75DC42           93            mov     CCAPM2,#42h     ;put PCA module 0 into PWM mode

8289 D2DE             94            setb    CR              ;start the PCA clock

828B 22               95            ret

96

828C                  97    lngDelay:

=1    98            $INCLUDE(lngDelay.inc)

828C 7D0A       =1    99            mov   R5,#00Ah

828E 7E35       =1   100    delay2: mov     R6,#035h

8290 7FFF       =1   101    delay3: mov     R7,#0FFh

8292 DFFE       =1   102    delay4: djnz    R7, delay4              ;wait >500msec

8294 DEFA       =1   103            djnz    R6, delay3

8296 DDF6       =1   104            djnz    R5, delay2

8298 22         =1   105            ret

106

8299                 107    cout:

=1   108            $INCLUDE(cout.inc)

8299 3099FD     =1   109            JNB     TI,$            ;Wait until transmission completed.

829C C299       =1   110            CLR     TI              ;Clear interrupt flag.

829E F599       =1   111            MOV     SBUF,A          ;Write out character.

82A0 22         =1   112            RET

LO_SPD2                                                                                                       PAGE 3

113

82A1                 114    PRINT:

=1   115            $INCLUDE(PRINT.inc)

82A1 C0E0       =1   116            PUSH    ACC      ; push the A and DPTR registers so their contents

82A3 C083       =1   117            PUSH    DPH      ; don't get lost in case they are being used

82A5 C082       =1   118            PUSH    DPL      ; before calling this function.

=1   119

=1   120    ; move the stack pointer down low in order to pop the first string's byte

=1   121    ; address

82A7 E581       =1   122            MOV     A,SP

82A9 24FD       =1   123            ADD     A,#0FDH

82AB F581       =1   124            MOV     SP,A

82AD D083       =1   125            POP     DPH

82AF D082       =1   126            POP     DPL

=1   127

=1   128    ; send (PRINT) the string back to the PC computer

82B1            =1   129    READ_TX:

82B1 E4         =1   130            CLR     A

82B2 93         =1   131            MOVC    A,@A+DPTR

82B3 A3         =1   132            INC     DPTR

=1   133

82B4 B45C0C     =1   134            CJNE    A,#'\',SND

=1   135

82B7 E4         =1   136            CLR     A

82B8 93         =1   137            MOVC    A,@A+DPTR

82B9 A3         =1   138            INC     DPTR

=1   139

82BA B46E06     =1   140            CJNE    A,#'n',SND

=1   141

82BD 740D       =1   142            MOV     A,#0DH

82BF 5199       =1   143            ACALL   cout

82C1 740A       =1   144            MOV     A,#0AH

=1   145

82C3 5199       =1   146    SND:    ACALL   cout

82C5 B400E9     =1   147            CJNE    A,#0,READ_TX

=1   148

=1   149    ; place the new return addres in stack

82C8 C082       =1   150            PUSH    DPL

82CA C083       =1   151            PUSH    DPH

=1   152

=1   153    ; move the stack pointer up high in order to pop the A and DPTR registers

82CC E581       =1   154            MOV     A,SP

82CE 2403       =1   155            ADD     A,#3

82D0 F581       =1   156            MOV     SP,A

82D2 D082       =1   157            POP     DPL

82D4 D083       =1   158            POP     DPH

82D6 D0E0       =1   159            POP     ACC

82D8 22         =1   160            RET

161

82D9                 162    SEND_BYTE:

=1   163            $INCLUDE(SEND_BYT.inc)

82D9 3099FD     =1   164            JNB    TI, SEND_BYTE           ; WAIT FOR THE TI FLAG TO SET

82DC C299       =1   165            CLR             TI                                ; CLEAR THE TI FLAG ONCE SENT

82DE F599       =1   166                    MOV     SBUF,A                  ; SEND THE CONTENTS OF REGISTER A

82E0 22         =1   167            RET                             ; RETURN

=1   168    ;*****************************************************************************

=1   169

=1   170

LO_SPD2                                                                                                       PAGE 4

=1   171

172

173

174

175            end

VERSION 1.2h ASSEMBLY COMPLETE, 0 ERRORS FOUND

LO_SPD2                                                                                                       PAGE 5

ACC. . . . . . . . . . . . . . .  D ADDR  00E0H  PREDEFINED

ACON . . . . . . . . . . . . . .  D ADDR  0097H  PREDEFINED

AD1. . . . . . . . . . . . . . .  D ADDR  0094H  PREDEFINED

AD2. . . . . . . . . . . . . . .  D ADDR  00A4H  PREDEFINED

AD3. . . . . . . . . . . . . . .  D ADDR  00B4H  PREDEFINED

CCAP1H . . . . . . . . . . . . .  D ADDR  00FBH  PREDEFINED

CCAP2H . . . . . . . . . . . . .  D ADDR  00FCH  PREDEFINED

CCAPM1 . . . . . . . . . . . . .  D ADDR  00DBH  PREDEFINED

CCAPM2 . . . . . . . . . . . . .  D ADDR  00DCH  PREDEFINED

CMOD . . . . . . . . . . . . . .  D ADDR  00D9H  PREDEFINED

COUT . . . . . . . . . . . . . .  C ADDR  8299H

CR . . . . . . . . . . . . . . .  B ADDR  00DEH  PREDEFINED

DELAY2 . . . . . . . . . . . . .  C ADDR  828EH

DELAY3 . . . . . . . . . . . . .  C ADDR  8290H

DELAY4 . . . . . . . . . . . . .  C ADDR  8292H

DPH. . . . . . . . . . . . . . .  D ADDR  0083H  PREDEFINED

DPL. . . . . . . . . . . . . . .  D ADDR  0082H  PREDEFINED

FORWARD. . . . . . . . . . . . .  C ADDR  8258H

FOR_REV. . . . . . . . . . . . .    NUMB  00F8H

IRSCAN . . . . . . . . . . . . .  C ADDR  8217H

LNGDELAY . . . . . . . . . . . .  C ADDR  828CH

L_FRONT. . . . . . . . . . . . .  C ADDR  8247H

L_REAR . . . . . . . . . . . . .  C ADDR  821AH  NOT USED

MAIN . . . . . . . . . . . . . .  C ADDR  8200H  NOT USED

P1 . . . . . . . . . . . . . . .  D ADDR  0090H  PREDEFINED

P5 . . . . . . . . . . . . . . .  D ADDR  00F8H  PREDEFINED

PCA_INIT . . . . . . . . . . . .  C ADDR  8280H

PRINT. . . . . . . . . . . . . .  C ADDR  82A1H

PWM_MOT. . . . . . . . . . . . .    REG0

PWM_STR. . . . . . . . . . . . .    REG1

READ_TX. . . . . . . . . . . . .  C ADDR  82B1H

R_FRONT. . . . . . . . . . . . .  C ADDR  8251H

SBUF . . . . . . . . . . . . . .  D ADDR  0099H  PREDEFINED

SEND_BYTE. . . . . . . . . . . .  C ADDR  82D9H

SND. . . . . . . . . . . . . . .  C ADDR  82C3H

SP . . . . . . . . . . . . . . .  D ADDR  0081H  PREDEFINED

STP_GO . . . . . . . . . . . . .    NUMB  00F9H

STP_REV0 . . . . . . . . . . . .  C ADDR  827CH

STP_REV2 . . . . . . . . . . . .  C ADDR  8245H

TI . . . . . . . . . . . . . . .  B ADDR  0099H  PREDEFINED

WAIT . . . . . . . . . . . . . .  C ADDR  827EH

Last, but not least, is this attempt at getting the compass integrated.  My data is getting corrupted somewhere.  I didn’t have time to digest my errors

ROBOSPDY                                                                                                      PAGE 1

8000                   1            org     8000h

8000 028200            2            jmp     8200h

3

4            $MOD51GB

5

6    ;Pulse width signals

7    PWM_MOT         EQU     R0

8    PWM_STR         EQU     R1

9

10    ;Compass data register

11    CompCount       EQU     R2

12    DataByte        EQU     R4

13

14    ;Compass result direction flag

00FF                15    Left_Flag       EQU     P5.7

00FE                16    Right_Flag      EQU     P5.6

17

18    ;Motor Direction

00F9                19    STP_GO          EQU     P5.1

00F8                20    FOR_REV         EQU     P5.0

21

22    ;Speeds

00C0                23    FWD_LO          EQU     0C0H

00B0                24    FWD_MED         EQU     0B0H

00B0                25    FWD_HI          EQU     0B0H

0090                26    REV_LO          EQU     090H

27

28    ;Steering Directions

003F                29    Straight        EQU     03fH

0030                30    Full_Left       EQU     030H

0037                31    Half_Left       EQU     037H

003A                32    Qtr_Left        EQU     03AH

004D                33    Full_Right      EQU     04DH

0045                34    Half_Right      EQU     045H

0042                35    Qtr_Right       EQU     042H

36

37    ;Sonar ranges

00F5                38    Sonar_15        EQU     245

00D0                39    Sonar_12        EQU     208

009B                40    Sonar_9         EQU     155

0067                41    Sonar_6         EQU     103

0033                42    Sonar_3         EQU     51

001A                43    Sonar_1         EQU     26

44

45    ;Compass Pins

00C0                46    EOC     equ     P4.0

00C1                47    SS      equ     P4.1

00C2                48    MS      equ     P4.2

00C3                49    RESET   equ     P4.3

00C4                50    SCLK    equ     P4.4

00C5                51    SDO     equ     P4.5

00C6                52    POLL    equ     P4.6

00C7                53    POW     equ     P4.7

54

55    ;Compass data

0081                56    Base      equ     081h

0000                57    Actual_HI equ     000h

0001                58    Actual_LO equ     001h

ROBOSPDY                                                                                                      PAGE 2

59

0002                60    Desired_HI  equ     002h

0003                61    Desired_LO  equ     003h

62

802B                  63            org     802bh

802B 028495           64            jmp     T2_ISR

65

8100                  66            org     8100h

8100 00000000         67            db      0,0,0,0

68

69

8200                  70            org     8200h

8200                  71    main:

72            ;enable A/D operation

8200 759710           73            mov     ACON,#010h

74

75            ;STP_GO is cleared to stop motor

76            ;FOR_REV is set to go in reverse

77

78            ;initalize the motor and steering PWM waveforms

8203 7590FF           79            mov     P1,#0FFH        ;must set all bits on port1 to 0

8206 78C0             80            mov     PWM_MOT,#FWD_LO   ;set motor for 1/8 speed

8208 88FC             81            mov     CCAP2h,PWM_MOT

820A 793C             82            mov     PWM_STR,#03Ch   ;set steering servo to center

820C 89FB             83            mov     CCAP1h,PWM_STR

820E C2F8             84            clr     FOR_REV           ;set relays for FORWARD

8210 C2F9             85            clr     STP_GO          ;block PWM signal to motor

8212 12855A           86            call    PCA_INIT

87

88            ;Set up the compass Inits

8215 12846B           89            call    Comp_Init       ;Initialize the compass

8218 128589           90            call    Print

821B 54686520         91            db      'The compass is initialized\n',0,

821F 636F6D70

8223 61737320

8227 69732069

822B 6E697469

822F 616C697A

8233 65645C6E

8237 00

8238 12848A           92            call    T2_Init         ;Initialize T2

823B 128589           93            call    Print

823E 54696D65         94            db      'Timer 2 is initialized\n',0,

8242 72203220

8246 69732069

824A 6E697469

824E 616C697A

8252 65645C6E

8256 00

95

96    ;****************************************************************************

8257 858183           97            mov     DPH,Base

825A 850282           98            mov     DPL,Desired_HI

825D 7400             99            mov     A,#0

825F F0              100            movx    @DPTR,A

8260 A3              101            inc     DPTR

8261 745A            102            mov     A,#05Ah

8263 F0              103            movx    @DPTR,A

ROBOSPDY                                                                                                      PAGE 3

104    ;****************************************************************************

105

106    ;        setb    ET2             ;enable Timer 2 interrupt

107    ;        setb    EA              ;enable interrupts

108

8264 D2F9            109            setb    STP_GO

110

8266 128574          111    Scan: call    lngDelay

112

113

8269 128589          114    L_Rear: call    Print

826C 4C5200          115            db      'LR',0,

826F C3              116            clr     C             ; Clear carry bit

8270 E594            117            mov     A,AD1         ; get the left rear IR

8272 9464            118            subb    A,#100        ; subtract threshold from value

8274 402E            119            jc      L_Front       ; jump if not too close

120

8276                 121    rev2for:

8276 30F929          122            jnb     STP_GO,stp_rev2  ;already stopped

8279 30F826          123            jnb     FOR_REV,stp_rev2 ;or going forward

827C C2F9            124            clr     STP_GO           ;stop motor

827E C2F8            125            clr     FOR_REV          ;place in forward

8280 793F            126            mov     PWM_STR,#Straight ;center wheels

8282 89FB            127            mov     CCAP1H,PWM_STR    ;

8284 128574          128            call    lngDelay

8287 78C0            129            mov     PWM_MOT,#FWD_LO  ;set motor for 1/8 speed

8289 88FC            130            mov     CCAP2h,PWM_MOT

828B D2F9            131            setb    STP_GO           ;start motor

828D 128589          132            call    PRINT

8290 466F7277        133            db      'Forward from IR\n',0,

8294 61726420

8298 66726F6D

829C 2049525C

82A0 6E00

82A2                 134    stp_rev2:

82A2 80C2            135            jmp     Scan

136

137

82A4                 138    L_Front:

82A4 128589          139            call    Print

82A7 4C4600          140            db      'LF',0,

82AA C3              141            clr     C             ; Clear carry bit

82AB E5A4            142            mov     A,AD2         ; get the left front IR

82AD 9464            143            subb    A,#100        ; subtract threshold from value

82AF 4003            144            jc      R_Front       ; jump if not too close

145

82B1 0282C1          146            jmp     for2rev

147

82B4                 148    R_Front:

82B4 128589          149            call    Print

82B7 524600          150            db      'RF',0,

82BA C3              151            clr     C             ; Clear carry bit

82BB E5B4            152            mov     A,AD3         ; get the left front IR

82BD 9464            153            subb    A,#100        ; subtract threshold from value

82BF 402E            154            jc      Compass_Calc  ; jump if not too close

155

156

82C1                 157    for2Rev:

ROBOSPDY                                                                                                      PAGE 4

82C1 30F929          158            jnb     STP_GO,stp_rev0         ;already stopped

82C4 20F826          159            jb      FOR_REV,stp_rev0        ;or backing

82C7 C2F9            160            clr     STP_GO                  ;stop motor

82C9 D2F8            161            setb    FOR_REV                 ;reverse it

82CB 7931            162            mov     PWM_STR,#031h           ;turn wheels to right

82CD 89FB            163            mov     CCAP1H,PWM_STR          ;

82CF 128574          164            call    lngDelay                ;Give it 1/2 a second

82D2 7890            165            mov     PWM_MOT,#REV_LO         ;set motor for 1/8 speed

82D4 88FC            166            mov     CCAP2h,PWM_MOT

82D6 D2F9            167            setb    STP_GO                  ;start motor

82D8 128589          168            call    PRINT

82DB 4261636B        169            db      'Backing from IR\n',0,

82DF 696E6720

82E3 66726F6D

82E7 2049525C

82EB 6E00

82ED                 170    stp_rev0:

82ED 4166            171            jmp     Scan

172

82EF                 173    Compass_Calc:

82EF 128392          174            call    Compass         ;get differential heading

82F2 E582            175            mov     A,DPL           ;from DPL  ( < B4 )

82F4 7002            176            jnz     TurnGo          ;if heading matches goto Scan(sonarsoon???)

82F6 4166            177            jmp     Scan

178

82F8 C3              179    TurnGo: clr     C               ;clear carry

82F9 7F0A            180            mov     R7,#10          ;load R7 with 10

82FB 9F              181            subb    A,R7            ;is difference >10?

82FC 4022            182            jc      undr_10

82FE E582            183            mov     A,DPL

8300 C3              184            clr     C

8301 7F19            185            mov     R7,#25

8303 9F              186            subb    A,R7            ;is difference >25?

8304 400D            187            jc      undr_25

8306 20FE05          188            jb      Right_Flag,Right_Full

8309                 189    Left_Full:

8309 7930            190            mov     PWM_STR,#Full_Left

830B 02832D          191            jmp     comp_str

830E                 192    Right_Full:

830E 794D            193            mov     PWM_STR,#Full_Right

8310 02832D          194            jmp     comp_str

195

8313                 196    undr_25:

8313 20FE05          197            jb      Right_Flag,Right_Half

8316                 198    Left_Half:

8316 7937            199            mov     PWM_STR,#Half_Left

8318 02832D          200            jmp     comp_str

201

831B                 202    Right_Half:

831B 7945            203            mov     PWM_STR,#Half_Right

831D 02832D          204            jmp     comp_str

205

8320                 206    undr_10:

8320 20FE05          207            jb      Right_Flag,Right_Qtr

8323                 208    Left_Qtr:

8323 7937            209            mov     PWM_STR,#Half_Left

8325 02832D          210            jmp     comp_str

8328                 211    Right_Qtr:

ROBOSPDY                                                                                                      PAGE 5

8328 7937            212            mov     PWM_STR,#Half_Left

832A 02832D          213            jmp     comp_str

214

832D                 215    comp_str:

832D 89FB            216            mov     CCAP1H,PWM_STR

832F 4166            217            jmp     Scan

218

219

220

221

222

8331 C3              223    Sonar:  clr     C

8332 E584            224            mov     A,AD0           ; Get the sonar value

8334 94F5            225            subb    A,#Sonar_15     ; subtract threshold from value

8336 4016            226            jc      Ahd_hlf         ; jump if distance is < 15 ft

227

8338 78B0            228            mov     PWM_MOT,#FWD_HI  ;set motor for High speed

833A 88FC            229            mov     CCAP2h,PWM_MOT

833C 128589          230            call    PRINT

833F 41686561        231            db      'Ahead Full\n',0,

8343 64204675

8347 6C6C5C6E

834B 00

232

834C 4166            233            jmp     Scan

234

834E                 235    Ahd_hlf:

834E C3              236            clr     C

834F E584            237            mov     A,AD0           ; Get the sonar value

8351 949B            238            subb    A,#Sonar_9      ; subtract threshold from value

8353 401B            239            jc      Ahd_slow        ; jump if distance is < 9 ft

8355 B8B036          240            cjne    PWM_MOT,#FWD_HI,All_Stop

241                                    ; If motor is in high gear

242                                    ; turn it off

243

8358 78B0            244            mov     PWM_MOT,#FWD_MED  ;set motor for Medium speed

835A 88FC            245            mov     CCAP2h,PWM_MOT

835C 128589          246            call    PRINT

835F 41686561        247            db      'Ahead Medium\n',0,

8363 64204D65

8367 6469756D

836B 5C6E00

248

836E 4166            249            jmp     Scan

250

8370                 251    Ahd_slow:

8370 C3              252            clr     C

8371 E584            253            mov     A,AD0           ; Get the sonar value

8373 9433            254            subb    A,#Sonar_3      ; subtract threshold from value

255    ;***************************************************************************

256    ;        jc      for2rev         ; jump if distance is < 3 ft

257    ;***************************************************************************

8375 B8B016          258            cjne    PWM_MOT,#FWD_HI,All_Stop

259                                    ; If motor is in high gear

260                                    ; turn it off

261

8378 78C0            262            mov     PWM_MOT,#FWD_LO  ;set motor for Low speed

837A 88FC            263            mov     CCAP2h,PWM_MOT

ROBOSPDY                                                                                                      PAGE 6

837C 128589          264            call    PRINT

837F 41686561        265            db      'Ahead Slow\n',0,

8383 6420536C

8387 6F775C6E

838B 00

838C 4166            266            jmp     Scan

267

838E                 268    All_Stop:

838E C2F9            269            clr     STP_GO

8390 4166            270            jmp     Scan

271

8392                 272    Compass:

8392 C004            273            push    04

8394 128410          274            call    load_actual     ;get actual from memory

8397 12841D          275            call    load_desired    ;get desired from memory

839A 128428          276            call    get_actual      ;put actual into DPTR

839D 128444          277            call    get_desired     ;put desired into B/Acc

83A0 128457          278            call    util_subbad     ;Actual-Desired

83A3 4021            279            jc      Neg             ;if actual is less than desired jump to Neg

83A5 75F000          280            mov     B,#0            ;load B/Acc with 180

83A8 74B4            281            mov     A,#0B4h

83AA 128457          282            call    util_subbad     ;subtract 180 from DPTR

83AD 4042            283            jc      Left_Pos        ;if Actual-Desired is <180 jump Left_Pos

83AF                 284    Right_Pos:

83AF D2FE            285            setb    Right_Flag      ;set flag for right turn

83B1 128444          286            call    get_desired     ;put desired into B/Acc

83B4 758301          287            mov     DPH,#1          ;load DPTR with 360

83B7 758267          288            mov     DPL,#067h

83BA 128448          289            call    util_adcbad     ;Add 360 to desired->(DPTR)

83BD 8EF0            290            mov     B,R6            ;Load B/Acc with actual

83BF EF              291            mov     A,R7            ;

83C0 128457          292            call    util_subbad     ;get final difference

293                                    ;put it in DPTR

83C3 02840D          294            jmp     end_sub

295

83C6 8B83            296    Neg:    mov     DPH,R3          ;put desired in DPTR

83C8 8C82            297            mov     DPL,R4

83CA 8EF0            298            mov     B,R6            ;put actual in B/Acc

83CC EF              299            mov     A,R7

83CD 128457          300            call    util_subbad     ;DPTR = DPTR - B/Acc

83D0 75F000          301            mov     B,#0

83D3 74B4            302            mov     A,#0B4h

83D5 128457          303            call    util_subbad     ;if Desired-Actual < 180 jump

83D8 4027            304            jc      Right_Neg

83DA                 305    Left_Neg:

83DA D2FF            306            setb    Left_Flag

83DC 8EF0            307            mov     B,R6            ;put actual in B/Acc

83DE EF              308            mov     A,R7

83DF 758301          309            mov     DPH,#1          ;add 360 to actual

83E2 758267          310            mov     DPL,#067h

83E5 128448          311            call    util_adcbad     ;Actual+360->DPTR

83E8 8BF0            312            mov     B,R3            ;Put desired into

83EA EC              313            mov     A,R4            ;B/Acc

83EB 128457          314            call    util_subbad     ;get final difference

315                                    ;place it in DPTR

83EE 02840D          316            jmp     end_sub

317

83F1                 318    Left_Pos:

ROBOSPDY                                                                                                      PAGE 7

83F1 D2FF            319            setb    Left_Flag       ;set bit for left turn

83F3 8E83            320            mov     DPH,R6          ;put actual in DPTR

83F5 8F82            321            mov     DPL,R7

83F7 8BF0            322            mov     B,R3            ;put desired in B/Acc

83F9 8CF0            323            mov     B,R4

83FB 128457          324            call    util_subbad     ;get final difference

325                                    ;place it in DPTR

83FE 02840D          326            jmp     end_sub

327

8401                 328    Right_Neg:

8401 D2FE            329            setb    Right_Flag      ;set bit for right turn

8403 8B83            330            mov     DPH,R3          ;put desired in DPTR

8405 8C82            331            mov     DPL,R4

8407 8EF0            332            mov     B,R6            ;put actual in B/Acc

8409 EF              333            mov     A,R7

840A 128457          334            call    util_subbad     ;get final difference

335                                    ;place it in DPTR

840D                 336    end_sub:

840D D004            337            pop     04

840F 22              338            ret

339

8410                 340    load_actual:

8410 758381          341            mov     DPH,#Base

8413 758200          342            mov     DPL,#Actual_HI

8416 E0              343            movx    A,@DPTR         ;get high byte of compass data

8417 FE              344            mov     R6,A            ;store it in R6

8418 A3              345            inc     DPTR            ;get low byte of data

8419 E0              346            movx    A,@DPTR         ;

841A FF              347            mov     R7,A            ;store low byte in R7

841B A3              348            inc     DPTR            ;

841C 22              349            ret

350

841D                 351    load_desired:

841D 758381          352            mov     DPH,#Base

8420 758202          353            mov     DPL,#Desired_HI

8423 E0              354            movx    A,@DPTR         ;get desired heading

8424 FB              355            mov     R3,A            ;high byte

8425 A3              356            inc     DPTR            ;get desired heading

8426 FC              357            mov     R4,A            ;low byte

8427 22              358            ret

359

8428                 360    get_actual:

8428 8E83            361            mov     DPH,R6          ;move actual into DPTR

842A 8F82            362            mov     DPL,R7

842C C002            363            push    02

842E C003            364            push    03

8430 C004            365            push    04

8432 C005            366            push    05

8434 C0D0            367            push    PSW

8436 1285C9          368            call    pint16u

8439 D0D0            369            pop     PSW

843B D005            370            pop     05

843D D004            371            pop     04

843F D003            372            pop     03

8441 D002            373            pop     02

8443 22              374            ret

375

8444                 376    get_desired:

ROBOSPDY                                                                                                      PAGE 8

8444 8BF0            377            mov     B,R3            ;move desired into B/Acc

8446 EC              378            mov     A,R4

8447 22              379            ret

380

8448                 381    util_adcbad:

8448 C0E0            382            push    acc                     ; Make Sure Acc Gets Saved

844A 2582            383            add     a,dpl                   ; Add 'A' To DPL

844C F582            384            mov     dpl,a                   ; Move Result To DPL

844E E583            385            mov     a,dph                   ; Get DPH

8450 35F0            386            addc    a,b                     ; Add 'B' To DPH + CY

8452 F583            387            mov     dph,a                   ; Move Bck To DPH

8454 D0E0            388            pop     acc                     ; Recover Original 'A'

8456 22              389            ret                             ; Return To Caller

390

8457                 391    util_subbad:

8457 C0E0            392            push    acc                     ; Make Sure Acc Gets Saved

8459 C3              393            clr     c                       ; Clear For SUBB

845A C582            394            xch     a,dpl                   ; Swap

845C 9582            395            subb    a,dpl                   ; Subtract

845E F582            396            mov     dpl,a                   ; Move Back To DPL

8460 E583            397            mov     a,dph                   ; Get DPH

8462 95F0            398            subb    a,b                     ; Subtract + CY

8464 F583            399            mov     dph,a                   ; Move Back To DPH

8466 D0E0            400            pop     acc                     ; Recover Original 'A'

8468 22              401            ret                             ; Return To Caller

402

403

404

8469 80FE            405    wait:   jmp     wait

406

407

846B                 408    Comp_Init:

409            ;set all the bits on the compass

846B D2C7            410            setb    POW             ;turn power off

846D D2C6            411            setb    POLL            ;set the poll bit

846F D2C1            412            setb    SS              ;set the slave select bit

8471 D2C3            413            setb    RESET           ;set the NOT Reset line

8473 D2C4            414            setb    SCLK            ;set the shift clock

8475 D2C5            415            setb    SDO             ;setting SDO will set that pin on the

416                                    ;8051 as an input

8477 D2C0            417            setb    EOC             ;same for EOC

418

8479 C2C7            419            clr     POW             ;turn power on

420

421            ;Initialize the counter for the compass process controller

847B 7A00            422            mov     CompCount,#0

847D C2C3            423            clr     RESET           ;reset compass

847F 12856B          424            call    Delay10         ;wait 10msecs

8482 D2C3            425            setb    RESET           ;clear reset

8484 128574          426            call    lngDelay        ;wait .5secs

8487 D2C2            427            setb    MS              ;set compass as slave

8489 22              428            ret

429

848A                 430    T2_Init:

431            ;load the timer to run the compass

848A C2CA            432            clr     TR2             ;turn Timer 2 off

848C 75CDB7          433            mov     TH2,#0B7h       ;load the T2 high byte

848F 75CCCB          434            mov     TL2,#0CBh       ;load the T2 low byte

ROBOSPDY                                                                                                      PAGE 9

8492 D2CA            435            setb    TR2             ;turn Timer 2 on

8494 22              436            ret

437

8495                 438    T2_ISR:

=1   439            $INCLUDE(T2_ISR.INC)

8495 C0E0       =1   440            push    Acc

8497 C0D0       =1   441            push    PSW

8499 C007       =1   442            push    07

849B C006       =1   443            push    06

849D C004       =1   444            push    04

849F C003       =1   445            push    03

84A1 C082       =1   446            push    082h

84A3 C083       =1   447            push    083h

=1   448

84A5 C2CF       =1   449            clr     TF2             ;Clear the T2 interrupt flag

84A7 C2CA       =1   450            clr     TR2             ;Stop the timer

84A9 75CDB7     =1   451            mov     TH2,#0B7h       ;Load the timer with the value

84AC 75CCCB     =1   452            mov     TL2,#0CBh       ;needed for a 10msec interval

84AF D2CA       =1   453            setb    TR2             ;turn timer back on

84B1 0A         =1   454            inc     CompCount       ;Increment the process counter

=1   455

84B2 BA0103     =1   456            cjne    CompCount,#1,temp0

84B5 0284F3     =1   457            jmp     CLR_POLL

84B8 BA0203     =1   458    temp0:  cjne    CompCount,#2,temp1

84BB 0284F8     =1   459            jmp     SET_POLL

84BE BA0303     =1   460    temp1:  cjne    CompCount,#3,temp2

84C1 0284FD     =1   461            jmp     WAIT_EOC

84C4 BA0403     =1   462    temp2:  cjne    CompCount,#4,temp3

84C7 028505     =1   463            jmp     CLR_SS

84CA BA0503     =1   464    temp3:  cjne    CompCount,#5,temp4

84CD 02850A     =1   465            jmp     READ_SDO

84D0 BA0603     =1   466    temp4:  cjne    CompCount,#6,temp5

84D3 02850A     =1   467            jmp     READ_SDO

84D6 128589     =1   468    temp5:  call    Print

84D9 54686973   =1   469            db      'This didnt work!! \n\n',0,

84DD 20646964

84E1 6E742077

84E5 6F726B21

84E9 21205C6E

84ED 5C6E00

84F0 028549     =1   470            jmp     exit_T2

=1   471

84F3            =1   472    CLR_POLL:

=1   473

84F3 C2C6       =1   474            clr     POLL              ;clr POLL

84F5 028536     =1   475            jmp     end_T2

=1   476

84F8            =1   477    SET_POLL:

84F8 D2C6       =1   478            setb    POLL              ;set POLL

84FA 028536     =1   479            jmp     end_T2

=1   480

84FD            =1   481    WAIT_EOC:

84FD 20C036     =1   482            jb      EOC,end_T2        ;wait for the EOC flag to be

8500 7A02       =1   483            mov     CompCount,#2      ;set

8502 028536     =1   484            jmp     end_T2

=1   485

8505 C2C1       =1   486    CLR_SS: clr     SS                ; set slave select for data recovery

8507 028536     =1   487            jmp     end_T2

ROBOSPDY                                                                                                      PAGE 10

=1   488

=1   489

850A 7E08       =1   490    READ_SDO:  mov     R6,#08

=1   491

850C            =1   492    ReadSDO:

850C C2C4       =1   493            clr     SCLK

850E 128566     =1   494            call    clkDelay

8511 D2C4       =1   495            setb    SCLK

8513 7420       =1   496            mov     A, #020h        ;set A.5

8515 55C0       =1   497            anl     A,P4            ;compare A.5 with P4.5(SDO)

8517 6006       =1   498            jz      zero            ;jump if P4.5(EOC) is not set

=1   499

8519 EC         =1   500            mov     A, DataByte     ;load R4 into A

851A D3         =1   501            setb    C               ;set the carry bit

851B 33         =1   502            rlc     A               ;roll A left with carry

851C FC         =1   503            mov     DataByte, A     ;put contents of A back into R4

851D A123       =1   504            ajmp    rdDone          ; done with read

=1   505

851F EC         =1   506    zero:   mov     A, DataByte     ;load R4 into A

8520 C3         =1   507            clr     C               ;clear the carry bit

8521 33         =1   508            rlc     A               ;roll A left with carry

8522 FC         =1   509            mov     DataByte, A     ;put contents of A back into R4

=1   510

8523 128566     =1   511    rdDone: call   clkdelay         ;delay for low clock

8526 DEE4       =1   512            djnz   R6, ReadSDO      ;this loop will execute 8 times

=1   513

8528 BA050B     =1   514    done8:  cjne   CompCount,#5,end_T2

=1   515                                    ;SDO_READ will execute when

=1   516                                    ;CompCount = 5 & 6

=1   517                                    ;done8 will only run when CompCount=5

852B EC         =1   518            mov    A,DataByte       ;mov Data to A

852C 758381     =1   519            mov    DPH,#081h        ;first 8 bits will be saved

852F 758200     =1   520            mov    DPL,#000h        ;at 8100h

8532 F0         =1   521            movx   @DPTR,A          ;

8533 028536     =1   522            jmp    end_T2           ;end the routine

=1   523

=1   524

8536 BA0610     =1   525    end_T2: cjne    CompCount,#6,exit_T2

8539 D2C1       =1   526            setb    SS              ;Slave select off

853B EC         =1   527            mov     A,DataByte

853C 758381     =1   528            mov     DPH,#081h

853F 758201     =1   529            mov     DPL,#001h

8542 F0         =1   530            movx    @DPTR,A

8543 7A00       =1   531            mov     CompCount,#00

8545 758200     =1   532            mov     DPL,#00h

8548 E0         =1   533            movx    A,@DPTR

=1   534    ;        mov     DPH,A

=1   535    ;        mov     DPL,DATABYTE

=1   536    ;        call    pint16u

=1   537    ;        call    print

=1   538    ;        db      '\',0,

=1   539

=1   540

=1   541

8549            =1   542    exit_T2:

=1   543    ;        call    Print

=1   544    ;        db      'I',0,

8549 D083       =1   545            pop     083h

ROBOSPDY                                                                                                      PAGE 11

854B D082       =1   546            pop     082h

854D D003       =1   547            pop     03

854F D004       =1   548            pop     04

8551 D006       =1   549            pop     06

8553 D007       =1   550            pop     07

8555 D0D0       =1   551            pop     PSW

8557 D0E0       =1   552            pop     Acc

8559 32         =1   553            reti

554

555            ;initializes the programmable counter array for the motor

556            ;and the steering servo

855A                 557    PCA_INIT:

855A 75D906          558            mov     CMOD,#006h      ;set PCA frequency to external

855D 75DB42          559            mov     CCAPM1,#42h     ;put PCA module 1 into PWM mode

8560 75DC42          560            mov     CCAPM2,#42h     ;put PCA module 0 into PWM mode

8563 D2DE            561            setb    CR              ;start the PCA clock

8565 22              562            ret

563

8566 7F73            564    clkdelay: mov     R7,#073h

8568 DFFE            565    clk:      djnz    R7,clk

856A 22              566              ret

567

856B 7E25            568    delay10:  mov     R6,#025h

856D 7FFF            569    delay:    mov     R7,#0FFh

856F DFFE            570    delay1:   djnz    R7, delay1    ;wait >10 msec

8571 DEFA            571              djnz    R6, delay

8573 22              572              ret

573

8574                 574    lngDelay:

=1   575            $INCLUDE(lngDelay.inc)

8574 7D0A       =1   576            mov   R5,#00Ah

8576 7E35       =1   577    delay2: mov     R6,#035h

8578 7FFF       =1   578    delay3: mov     R7,#0FFh

857A DFFE       =1   579    delay4: djnz    R7, delay4              ;wait >500msec

857C DEFA       =1   580            djnz    R6, delay3

857E DDF6       =1   581            djnz    R5, delay2

8580 22         =1   582            ret

583

8581                 584    cout:

=1   585            $INCLUDE(cout.inc)

8581 3099FD     =1   586            JNB     TI,$            ;Wait until transmission completed.

8584 C299       =1   587            CLR     TI              ;Clear interrupt flag.

8586 F599       =1   588            MOV     SBUF,A          ;Write out character.

8588 22         =1   589            RET

590

8589                 591    PRINT:

=1   592            $INCLUDE(PRINT.inc)

8589 C0E0       =1   593            PUSH    ACC      ; push the A and DPTR registers so their contents

858B C083       =1   594            PUSH    DPH      ; don't get lost in case they are being used

858D C082       =1   595            PUSH    DPL      ; before calling this function.

=1   596

=1   597    ; move the stack pointer down low in order to pop the first string's byte

=1   598    ; address

858F E581       =1   599            MOV     A,SP

8591 24FD       =1   600            ADD     A,#0FDH

8593 F581       =1   601            MOV     SP,A

8595 D083       =1   602            POP     DPH

8597 D082       =1   603            POP     DPL

ROBOSPDY                                                                                                      PAGE 12

=1   604

=1   605    ; send (PRINT) the string back to the PC computer

8599            =1   606    READ_TX:

8599 E4         =1   607            CLR     A

859A 93         =1   608            MOVC    A,@A+DPTR

859B A3         =1   609            INC     DPTR

=1   610

859C B45C0C     =1   611            CJNE    A,#'\',SND

=1   612

859F E4         =1   613            CLR     A

85A0 93         =1   614            MOVC    A,@A+DPTR

85A1 A3         =1   615            INC     DPTR

=1   616

85A2 B46E06     =1   617            CJNE    A,#'n',SND

=1   618

85A5 740D       =1   619            MOV     A,#0DH

85A7 B181       =1   620            ACALL   cout

85A9 740A       =1   621            MOV     A,#0AH

=1   622

85AB B181       =1   623    SND:    ACALL   cout

85AD B400E9     =1   624            CJNE    A,#0,READ_TX

=1   625

=1   626    ; place the new return addres in stack

85B0 C082       =1   627            PUSH    DPL

85B2 C083       =1   628            PUSH    DPH

=1   629

=1   630    ; move the stack pointer up high in order to pop the A and DPTR registers

85B4 E581       =1   631            MOV     A,SP

85B6 2403       =1   632            ADD     A,#3

85B8 F581       =1   633            MOV     SP,A

85BA D082       =1   634            POP     DPL

85BC D083       =1   635            POP     DPH

85BE D0E0       =1   636            POP     ACC

85C0 22         =1   637            RET

638

85C1                 639    SEND_BYTE:

=1   640            $INCLUDE(SEND_BYT.inc)

85C1 3099FD     =1   641            JNB    TI, SEND_BYTE           ; WAIT FOR THE TI FLAG TO SET

85C4 C299       =1   642            CLR             TI                                ; CLEAR THE TI FLAG ONCE SENT

85C6 F599       =1   643                    MOV     SBUF,A                  ; SEND THE CONTENTS OF REGISTER A

85C8 22         =1   644            RET                             ; RETURN

=1   645    ;*****************************************************************************

=1   646

=1   647

=1   648

649

85C9                 650    pint16u:

=1   651            $INCLUDE(PINT16U.INC)

=1   652            ;print 16 bit unsigned integer in DPTR, using base 10.

=1   653            ;warning, destroys r2, r3, r4, r5, psw.5

85C9 C0E0       =1   654            push    acc

85CB E8         =1   655            mov     a, r0

85CC C0E0       =1   656            push    acc

85CE C2D5       =1   657            clr     psw.5

85D0 AA82       =1   658            mov     r2, dpl

85D2 AB83       =1   659            mov     r3, dph

=1   660

85D4 7C10       =1   661    pint16a:mov     r4, #16 ;ten-thousands digit

ROBOSPDY                                                                                                      PAGE 13

85D6 7D27       =1   662            mov     r5, #39

85D8 D127       =1   663            acall   pint16x

85DA 6007       =1   664            jz      pint16b

85DC 2430       =1   665            add     a, #'0'

85DE 128581     =1   666            lcall   cout

85E1 D2D5       =1   667            setb    psw.5

=1   668

85E3 7CE8       =1   669    pint16b:mov     r4, #232        ;thousands digit

85E5 7D03       =1   670            mov     r5, #3

85E7 D127       =1   671            acall   pint16x

85E9 7003       =1   672            jnz     pint16c

85EB 30D507     =1   673            jnb     psw.5, pint16d

85EE 2430       =1   674    pint16c:add     a, #'0'

85F0 128581     =1   675            lcall   cout

85F3 D2D5       =1   676            setb    psw.5

=1   677

85F5 7C64       =1   678    pint16d:mov     r4, #100        ;hundreds digit

85F7 7D00       =1   679            mov     r5, #0

85F9 D127       =1   680            acall   pint16x

85FB 7003       =1   681            jnz     pint16e

85FD 30D507     =1   682            jnb     psw.5, pint16f

8600 2430       =1   683    pint16e:add     a, #'0'

8602 128581     =1   684            lcall   cout

8605 D2D5       =1   685            setb    psw.5

=1   686

8607 EA         =1   687    pint16f:mov     a, r2           ;tens digit

8608 ABF0       =1   688            mov     r3, b

860A 75F00A     =1   689            mov     b, #10

860D 84         =1   690            div     ab

860E 7003       =1   691            jnz     pint16g

8610 30D505     =1   692            jnb     psw.5, pint16h

8613 2430       =1   693    pint16g:add     a, #'0'

8615 128581     =1   694            lcall   cout

=1   695

8618 E5F0       =1   696    pint16h:mov     a, b            ;and finally the ones digit

861A 8BF0       =1   697            mov     b, r3

861C 2430       =1   698            add     a, #'0'

861E 128581     =1   699            lcall   cout

=1   700

8621 D0E0       =1   701            pop     acc

8623 F8         =1   702            mov     r0, a

8624 D0E0       =1   703            pop     acc

8626 22         =1   704            ret

=1   705

=1   706    ;ok, it's a cpu hog and a nasty way to divide, but this code

=1   707    ;requires only 21 bytes!  Divides r2-r3 by r4-r5 and leaves

=1   708    ;quotient in r2-r3 and returns remainder in acc.  If Intel

=1   709    ;had made a proper divide, then this would be much easier.

=1   710

8627 7800       =1   711    pint16x:mov     r0, #0

8629 08         =1   712    pint16y:inc     r0

862A C3         =1   713            clr     c

862B EA         =1   714            mov     a, r2

862C 9C         =1   715            subb    a, r4

862D FA         =1   716            mov     r2, a

862E EB         =1   717            mov     a, r3

862F 9D         =1   718            subb    a, r5

8630 FB         =1   719            mov     r3, a

ROBOSPDY                                                                                                      PAGE 14

8631 50F6       =1   720            jnc     pint16y

8633 18         =1   721            dec     r0

8634 EA         =1   722            mov     a, r2

8635 2C         =1   723            add     a, r4

8636 FA         =1   724            mov     r2, a

8637 EB         =1   725            mov     a, r3

8638 3D         =1   726            addc    a, r5

8639 FB         =1   727            mov     r3, a

863A E8         =1   728            mov     a, r0

863B 22         =1   729            ret

730

731

732

733            end

VERSION 1.2h ASSEMBLY COMPLETE, 0 ERRORS FOUND

ROBOSPDY                                                                                                      PAGE 15

ACC. . . . . . . . . . . . . . .  D ADDR  00E0H  PREDEFINED

ACON . . . . . . . . . . . . . .  D ADDR  0097H  PREDEFINED

ACTUAL_HI. . . . . . . . . . . .    NUMB  0000H

ACTUAL_LO. . . . . . . . . . . .    NUMB  0001H  NOT USED

AD0. . . . . . . . . . . . . . .  D ADDR  0084H  PREDEFINED

AD1. . . . . . . . . . . . . . .  D ADDR  0094H  PREDEFINED

AD2. . . . . . . . . . . . . . .  D ADDR  00A4H  PREDEFINED

AD3. . . . . . . . . . . . . . .  D ADDR  00B4H  PREDEFINED

AHD_HLF. . . . . . . . . . . . .  C ADDR  834EH

AHD_SLOW . . . . . . . . . . . .  C ADDR  8370H

ALL_STOP . . . . . . . . . . . .  C ADDR  838EH

B. . . . . . . . . . . . . . . .  D ADDR  00F0H  PREDEFINED

BASE . . . . . . . . . . . . . .    NUMB  0081H

CCAP1H . . . . . . . . . . . . .  D ADDR  00FBH  PREDEFINED

CCAP2H . . . . . . . . . . . . .  D ADDR  00FCH  PREDEFINED

CCAPM1 . . . . . . . . . . . . .  D ADDR  00DBH  PREDEFINED

CCAPM2 . . . . . . . . . . . . .  D ADDR  00DCH  PREDEFINED

CLK. . . . . . . . . . . . . . .  C ADDR  8568H

CLKDELAY . . . . . . . . . . . .  C ADDR  8566H

CLR_POLL . . . . . . . . . . . .  C ADDR  84F3H

CLR_SS . . . . . . . . . . . . .  C ADDR  8505H

CMOD . . . . . . . . . . . . . .  D ADDR  00D9H  PREDEFINED

COMPASS. . . . . . . . . . . . .  C ADDR  8392H

COMPASS_CALC . . . . . . . . . .  C ADDR  82EFH

COMPCOUNT. . . . . . . . . . . .    REG2

COMP_INIT. . . . . . . . . . . .  C ADDR  846BH

COMP_STR . . . . . . . . . . . .  C ADDR  832DH

COUT . . . . . . . . . . . . . .  C ADDR  8581H

CR . . . . . . . . . . . . . . .  B ADDR  00DEH  PREDEFINED

DATABYTE . . . . . . . . . . . .    REG4

DELAY. . . . . . . . . . . . . .  C ADDR  856DH

DELAY1 . . . . . . . . . . . . .  C ADDR  856FH

DELAY10. . . . . . . . . . . . .  C ADDR  856BH

DELAY2 . . . . . . . . . . . . .  C ADDR  8576H

DELAY3 . . . . . . . . . . . . .  C ADDR  8578H

DELAY4 . . . . . . . . . . . . .  C ADDR  857AH

DESIRED_HI . . . . . . . . . . .    NUMB  0002H

DESIRED_LO . . . . . . . . . . .    NUMB  0003H  NOT USED

DONE8. . . . . . . . . . . . . .  C ADDR  8528H  NOT USED

DPH. . . . . . . . . . . . . . .  D ADDR  0083H  PREDEFINED

DPL. . . . . . . . . . . . . . .  D ADDR  0082H  PREDEFINED

END_SUB. . . . . . . . . . . . .  C ADDR  840DH

END_T2 . . . . . . . . . . . . .  C ADDR  8536H

EOC. . . . . . . . . . . . . . .    NUMB  00C0H

EXIT_T2. . . . . . . . . . . . .  C ADDR  8549H

FOR2REV. . . . . . . . . . . . .  C ADDR  82C1H

FOR_REV. . . . . . . . . . . . .    NUMB  00F8H

FULL_LEFT. . . . . . . . . . . .    NUMB  0030H

FULL_RIGHT . . . . . . . . . . .    NUMB  004DH

FWD_HI . . . . . . . . . . . . .    NUMB  00B0H

FWD_LO . . . . . . . . . . . . .    NUMB  00C0H

FWD_MED. . . . . . . . . . . . .    NUMB  00B0H

GET_ACTUAL . . . . . . . . . . .  C ADDR  8428H

GET_DESIRED. . . . . . . . . . .  C ADDR  8444H

HALF_LEFT. . . . . . . . . . . .    NUMB  0037H

HALF_RIGHT . . . . . . . . . . .    NUMB  0045H

LEFT_FLAG. . . . . . . . . . . .    NUMB  00FFH

LEFT_FULL. . . . . . . . . . . .  C ADDR  8309H  NOT USED

ROBOSPDY                                                                                                      PAGE 16

LEFT_HALF. . . . . . . . . . . .  C ADDR  8316H  NOT USED

LEFT_NEG . . . . . . . . . . . .  C ADDR  83DAH  NOT USED

LEFT_POS . . . . . . . . . . . .  C ADDR  83F1H

LEFT_QTR . . . . . . . . . . . .  C ADDR  8323H  NOT USED

LNGDELAY . . . . . . . . . . . .  C ADDR  8574H

LOAD_ACTUAL. . . . . . . . . . .  C ADDR  8410H

LOAD_DESIRED . . . . . . . . . .  C ADDR  841DH

L_FRONT. . . . . . . . . . . . .  C ADDR  82A4H

L_REAR . . . . . . . . . . . . .  C ADDR  8269H  NOT USED

MAIN . . . . . . . . . . . . . .  C ADDR  8200H  NOT USED

MS . . . . . . . . . . . . . . .    NUMB  00C2H

NEG. . . . . . . . . . . . . . .  C ADDR  83C6H

P1 . . . . . . . . . . . . . . .  D ADDR  0090H  PREDEFINED

P4 . . . . . . . . . . . . . . .  D ADDR  00C0H  PREDEFINED

P5 . . . . . . . . . . . . . . .  D ADDR  00F8H  PREDEFINED

PCA_INIT . . . . . . . . . . . .  C ADDR  855AH

PINT16A. . . . . . . . . . . . .  C ADDR  85D4H  NOT USED

PINT16B. . . . . . . . . . . . .  C ADDR  85E3H

PINT16C. . . . . . . . . . . . .  C ADDR  85EEH

PINT16D. . . . . . . . . . . . .  C ADDR  85F5H

PINT16E. . . . . . . . . . . . .  C ADDR  8600H

PINT16F. . . . . . . . . . . . .  C ADDR  8607H

PINT16G. . . . . . . . . . . . .  C ADDR  8613H

PINT16H. . . . . . . . . . . . .  C ADDR  8618H

PINT16U. . . . . . . . . . . . .  C ADDR  85C9H

PINT16X. . . . . . . . . . . . .  C ADDR  8627H

PINT16Y. . . . . . . . . . . . .  C ADDR  8629H

POLL . . . . . . . . . . . . . .    NUMB  00C6H

POW. . . . . . . . . . . . . . .    NUMB  00C7H

PRINT. . . . . . . . . . . . . .  C ADDR  8589H

PSW. . . . . . . . . . . . . . .  D ADDR  00D0H  PREDEFINED

PWM_MOT. . . . . . . . . . . . .    REG0

PWM_STR. . . . . . . . . . . . .    REG1

QTR_LEFT . . . . . . . . . . . .    NUMB  003AH  NOT USED

QTR_RIGHT. . . . . . . . . . . .    NUMB  0042H  NOT USED

RDDONE . . . . . . . . . . . . .  C ADDR  8523H

READSDO. . . . . . . . . . . . .  C ADDR  850CH

READ_SDO . . . . . . . . . . . .  C ADDR  850AH

READ_TX. . . . . . . . . . . . .  C ADDR  8599H

RESET. . . . . . . . . . . . . .    NUMB  00C3H

REV2FOR. . . . . . . . . . . . .  C ADDR  8276H  NOT USED

REV_LO . . . . . . . . . . . . .    NUMB  0090H

RIGHT_FLAG . . . . . . . . . . .    NUMB  00FEH

RIGHT_FULL . . . . . . . . . . .  C ADDR  830EH

RIGHT_HALF . . . . . . . . . . .  C ADDR  831BH

RIGHT_NEG. . . . . . . . . . . .  C ADDR  8401H

RIGHT_POS. . . . . . . . . . . .  C ADDR  83AFH  NOT USED

RIGHT_QTR. . . . . . . . . . . .  C ADDR  8328H

R_FRONT. . . . . . . . . . . . .  C ADDR  82B4H

SBUF . . . . . . . . . . . . . .  D ADDR  0099H  PREDEFINED

SCAN . . . . . . . . . . . . . .  C ADDR  8266H

SCLK . . . . . . . . . . . . . .    NUMB  00C4H

SDO. . . . . . . . . . . . . . .    NUMB  00C5H

SEND_BYTE. . . . . . . . . . . .  C ADDR  85C1H

SET_POLL . . . . . . . . . . . .  C ADDR  84F8H

SND. . . . . . . . . . . . . . .  C ADDR  85ABH

SONAR. . . . . . . . . . . . . .  C ADDR  8331H  NOT USED

SONAR_1. . . . . . . . . . . . .    NUMB  001AH  NOT USED

ROBOSPDY                                                                                                      PAGE 17

SONAR_12 . . . . . . . . . . . .    NUMB  00D0H  NOT USED

SONAR_15 . . . . . . . . . . . .    NUMB  00F5H

SONAR_3. . . . . . . . . . . . .    NUMB  0033H

SONAR_6. . . . . . . . . . . . .    NUMB  0067H  NOT USED

SONAR_9. . . . . . . . . . . . .    NUMB  009BH

SP . . . . . . . . . . . . . . .  D ADDR  0081H  PREDEFINED

SS . . . . . . . . . . . . . . .    NUMB  00C1H

STP_GO . . . . . . . . . . . . .    NUMB  00F9H

STP_REV0 . . . . . . . . . . . .  C ADDR  82EDH

STP_REV2 . . . . . . . . . . . .  C ADDR  82A2H

STRAIGHT . . . . . . . . . . . .    NUMB  003FH

T2_INIT. . . . . . . . . . . . .  C ADDR  848AH

T2_ISR . . . . . . . . . . . . .  C ADDR  8495H

TEMP0. . . . . . . . . . . . . .  C ADDR  84B8H

TEMP1. . . . . . . . . . . . . .  C ADDR  84BEH

TEMP2. . . . . . . . . . . . . .  C ADDR  84C4H

TEMP3. . . . . . . . . . . . . .  C ADDR  84CAH

TEMP4. . . . . . . . . . . . . .  C ADDR  84D0H

TEMP5. . . . . . . . . . . . . .  C ADDR  84D6H

TF2. . . . . . . . . . . . . . .  B ADDR  00CFH  PREDEFINED

TH2. . . . . . . . . . . . . . .  D ADDR  00CDH  PREDEFINED

TI . . . . . . . . . . . . . . .  B ADDR  0099H  PREDEFINED

TL2. . . . . . . . . . . . . . .  D ADDR  00CCH  PREDEFINED

TR2. . . . . . . . . . . . . . .  B ADDR  00CAH  PREDEFINED

TURNGO . . . . . . . . . . . . .  C ADDR  82F8H

UNDR_10. . . . . . . . . . . . .  C ADDR  8320H

UNDR_25. . . . . . . . . . . . .  C ADDR  8313H

UTIL_ADCBAD. . . . . . . . . . .  C ADDR  8448H

UTIL_SUBBAD. . . . . . . . . . .  C ADDR  8457H

WAIT . . . . . . . . . . . . . .  C ADDR  8469H

WAIT_EOC . . . . . . . . . . . .  C ADDR  84FDH

ZERO . . . . . . . . . . . . . .  C ADDR  851FH