University de Aveiro

Electronics and Telecommunications Department

MOBILE ROBOTICS

“Mobile robot for surveillance purposes”

Students:	António Branco  &  Pedro Kulzer

Course:  	Mobile Robotics

Professors:	Keith Doty  &  Scott Jantz

3rd  July,  1995

# ABSTRACT

The robot discussed in this paper is intended to be used as a surveillance robot and programmed to avoid obstacles and to follow movement using optic flow. The adaptive obstacle avoidance behaviour, allows the robot to adapt to new environments, as well as enabling it to explore into large or small rooms. It’s not intended that the robot covers the entire room when passing through it, it’s only needed that it can wander about the rooms freely, adapting itself to different environments.

# EXECUTIVE SUMMARY

The robot described in this report, intends to serve as a surveillance robot. Its main behaviours are following movement and avoiding obstacles. The robot uses IR sensors to avoid obstacles, using an adaptive algorithm for adjusting each sensor’s threshold. There are three contact switches, at the front of the robot, which are used for collision detection. This happens due to blind spots or to behavioural responses relative to the adaptive behavioural algorithm approach. It’s intended that the robot, wanders around the rooms and adapts to new environments, doing so it can monitor any movement and sound the alarm. The movement detection is done using optic flow, by comparing two consecutive images from the robot’s, spinning motor’s retina. It behaves well when changing from one environment to a different one, for example when going from a high reflective environment to a lesser one or the opposite, or when going through doors. Nevertheless the robot can get “claustrophobic”, when put in a very confined environment, but because this is a particular environment, not likely to happen in a surveillance patrol, we find this emergent functionality understandable.

It was essential to add noise to the decision of the direction to turn to, when the robot had a obstacle in front of it, because without it the robot sometimes got locked in closed paths.

We accomplished smooth changes in the motors speed, using dynamics (attractors) for desired speed for both motors, as well as adding up the contributions of the several sensors instead of using a winner-takes-all method. For the threshold adaptation as well, we used a similar dynamics.

# INTRODUCTION

For this surveillance robot, the main purpose is only to wander around the corridors, in order to detect intruders. It is not desired to have the robot following walls too closely or having all the space covered, since the main purpose is just to “look around” for motion. The robot keeps itself faraway from walls, having the capability of adapting it’s behaviours, in order to be able to enter and explore narrow spaces, and to adapt easily to different reflective environments.

Earlier work done with Dr. Gregor Schöner [1], in the field of target acquisition and obstacle avoidance through optic flow of the retina image, inspired us to use some of the basic dynamic equations techniques for the motor speed control and adaptive threshold process. The adaptive threshold was also strongly inspired through another work in neural networks [2], where we studied the mechanisms and dynamics of neuron learning and habituation.

In surveillance robots, we expect to get an overall behaviour which permits an efficient coverage and exploration of the entire space where it should wander about.

# INTEGRATED SYSTEM

Fig. 1 - Block diagram of the complete system. The batteries feeds the power supply electronics which in turn will give a stabilised voltage to all the microcontroller, actuator and sensor circuitry. Only the motors are directly fed by the batteries.

The IR emitters emit light which is reflected by the surrounding obstacle surfaces. The amount of reflection is received by the IR receivers which communicate it to the microcontroller. With this information, the program can decide what behaviour to acquire in order to avoid those obstacles.

In case of collisions against the obstacles due to blind spots or specific behavioural responses, the bumpers transmit this information to the microcontroller which will take the appropriate actions.

The microcontroller actuates the motor drivers which in turn will drive the appropriate power to the motors.

# MOBILE PLATFORM

Fig. 2 - Upper view of the wooden mobile platform of the robot. On the left, we can clearly see the two wheels and the caster wheel which support the platform. The shape is round for perfect adaptation to the overall wheels motion and to avoid getting stuck on a sharp edge. The wheels are inside the outer circle of the structure, to avoid getting stuck by the outstanding wheels. The batteries are put on the rear to exert weight on the caster wheel, leaving at the same time a free front part for sensor applications. The microcontroller board and all the additional electronics are supported by four screws in the centre of the platform.

The heart of the whole system is the MC68HC11 microcontroller platform. This controller has an external 32Kbyte SRAM mapped into the upper half memory area. The robot is powered by 8 AA NiCd batteries.

# ACTUATION

There is only one type of actuation, namely the motor speeds. There are two bi-directional DC motor drivers which feed the desired power into the motors independently according to the commands given  by the microcontroller. Each motor has a speed range of -100.0% to 100.0%, where these values correspond to the maximum backward and forward speeds yielded by the motors. The power dosage mechanism is achieved by a PWM modulation technique usually used for DC motor drives.

Grf. 1 - General form of the dynamics used for controlling each motor’s speed. We set the attractor to have the desired final speed for the motor and the dynamics forces slowly the real speed to achieve that value. This guarantees some controlled smoothness in motion changes.

The corresponding equation that yields this dynamic behaviour is the following:

dVdt=V=Vdesired−V     Eq. 1

Grf. 2 - This is the resulting temporal response for the motor’s speed, due to the dynamics shown previously. The speed approaches the desired attractor value describing an exponential curve.

# SENSORS

The Table 1 shows the sensors available and their properties. We have installed three IR sensors in the front of the robot, at left, centre and right, to provide proximity detection so that the robot can avoid obstacles. We have also installed three contact switches (bumpers) on the front which enable the robot to successfully recover from collisions due to blind spots or specific behavioural responses.

| Sensor Type | Function | Location | Number |
| --- | --- | --- | --- |
| Infrared (IR) | Proximity detection of obstacles | Front | 3 |
| Contact Switches | Collision Detection against obstacles | Front | 3 |
| Photo Transistor | Optic Flow | Front | 1 (with a motor spinning mirror) |

Tab. 1 - Sensor suite

Grf. 3 - This is the dynamics used for the adaptive threshold calculation, being of the same form as the one used for motor speeds. The desired value for the final threshold is always the nose’s received value.

This dynamics can be programmed with a simple weighted average, which can be shown as follows. We want the following dynamics:

TH=dTHdt=Nose−TH     Eq. 2

To achieve this, we must make approximations to get an incremental programmable solution to the differential equation:

dTH»THnew−THold     Eq. 3

THnew−THold»Nose−THolddt     Eq. 4

THnew»1−dt⋅THold+dt⋅Nose     Eq. 5

Admitting that dt is a finite time increment (say 0.01), we’ll have the following programmable approximation:

TH=0.99TH+0.01Nose     Eq. 6

This is a weighted average of the previous threshold and the present sensor value, whose final value leads obviously to the sensor. The largest the value of the chosen time step, the fastest the threshold will be attracted towards its final value. The value of 0.015 showed to be a good experimental choice, which balances somehow the trade-off between fast adaptation (for fast get-through among crowding obstacles) and slow adaptation (for not forgetting so fast the present obstacle density).

The same steps can be taken to reach a similar result for the speed dynamics.

# BEHAVIOURS

## Obstacle Avoidance Behaviour

We implemented a low level obstacle avoidance mechanism, which consists simply in inspecting regularly, if the distance in certain direction is above a threshold. The action taken depends directly on the direction on which the obstacle proximity was detected.

## Adaptive Threshold

Although this is not a strictly a behaviour, it is a useful mechanism that activity influences the obstacle avoidance behaviour. When the robot finds itself free of obstacles, or if it has enough space to wander freely, the threshold will decrease to a minimum value. When this happens, the robot will stay far away from walls, moving itself through the middle of the room. Remember that this is a surveillance robot, with visual sensing mechanism, so it doesn’t need to cover all the space in the room, it just needs to have overall view, in order to detect movement.

When a robot faces a narrowing space it will increase the threshold, in order to see less obstacles and try to go through. For example, when passing through doors, after period of indecision, the robot will pass through the middle.

If the robot finds itself too enclosed by obstacles, it will further try to go through them, by increasing the threshold. If the robot is really trapped by obstacles, it will continue to increase the threshold trying to find a way through the obstacles, eventually leading to a collision.

There is an independent threshold for each sensor direction, which equally independent behavioural adaptation.

Fig. 3 - Illustration of the better surveillance performance of the approach in A. Here, we use the adaptive threshold where the robot sees far walls and corrects its path accordingly sooner. The path in B is the normally taken one for the “myope” robot. The key to the desired behavior is to let the robot always in the most sensitive state possible in each moment, desensitising just when needed (this is accomplished by the adaptive threshold).

## Collision Bumper Behaviour

If the robot collides against an obstacle, due to blind spots or the attempt to pass through too closely standing objects, it will sense the contact switches and it will behave in conformity. As soon as the collision takes place, the respective threshold  is immediately lowered, giving rise to a increase of sensibility on the side it bumped.

# EXPERIMENTAL LAYOUT AND RESULTS

If the robot is wandering through an highly reflective environment, and suddenly the environment changes to a less reflective one, the robot will approximate more to the obstacles. This sometimes leads robot to collide to dark objects, when in the highly reflective environment he was highly adapted to the enclosing objects. The bumpers correct this kind of temporary “blindness”. This is what happens to human sight, when for example, going into a dark room after being exposed to bright light.

When going through doors, the robot “sniffs” around for a while, adapting its thresholds to the new situation, passing through it successfully.

When we tried leaving the robot in a small spaced environment, we observed many times that the robot got stuck on cyclic path, which was highly undesired for a surveillance robot. To solve this we introduced random noise in the choice of the direction to take when the centre nose is activated. This in turn, caused a initial problem of indecision, because the robot couldn’t decide between different successive choices. We solved this simply by forcing it to take only one decision while the centre nose was being activated.

The robot doesn’t behave well in a maze or in a permanent too enclosing environment, since it starts feeling trapped and starts trying to get out bumping into the walls, we call this the claustrophobia emergent functionality.

# CONCLUSIONS

The adaptive bumper-aided obstacle avoidance behaviour seemed to operate in a desired fashion. The robot does get claustrophobia in confined spaces such as labyrinths, which is explained by the behavioural tendency of the robot getting out or through these situations and as it can’t get out easily he gets confused. This is not a fundamental problem, because the surveillance robot is not intended to work in such environment.

The main goal we intended was to have the robot adapting to new environments easily, which worked well, with the help from the bumpers for extreme cases.

For future work there’s still missing the optic flow part, which is needed for the following behaviour. This will be accomplished with a mirror spinning at a linear velocity, reflecting the world’s image to a photo transistor.

# DOCUMENTATION

[1] A.Branco, R.Costa, P.Kulzer; “Robot obstacle avoidance using a behavioural approach”, Non-linear dynamical systems course, University of Aveiro 1995.

[2] P.Kulzer, A.Branco; “Character recognition using neural networks”, Final project, University of Aveiro 1994.

# APPENDICES

OBSTACLE.C

float Noses[3];   /* Left, Center, Right */

float Bumpers[3];

float Thresholds[3];

float DV_Right, DV_Left;

float V_Right,V_Left;

float Rand;

int   TurningCenter;   /* Flag to hold random value when dynamics turning on center */

float Random()

{

return(((float)(analog(7) & 0b00000001)*2.0)-1.0);   /* Returns either 1.0 or -1.0 */

}

void BumpersSense()

{

int I;

while(1)

{

Bumpers[0]=(float)analog(4);

Bumpers[1]=(float)analog(5);

Bumpers[2]=(float)analog(3);

for (I=0;I<=2;I++)

if (Bumpers[I]==0.0)

Thresholds[I]=Noses[I]-10.0;

};

}

void NosesSense()

{

int I;

while(1)

for (I=0;I<=2;I++)

{

Noses[I]=(float)analog(I);

Thresholds[I]=(0.985*Thresholds[I])+(0.015*Noses[I]);

};

}

void AvoidObstacles()

{

float DV_Left_L,DV_Left_R,DV_Left_C;

float DV_Right_L,DV_Right_R,DV_Right_C;

float Weight_L,Weight_C,Weight_R;

while(1)

{

DV_Left_L=0.0;

DV_Left_C=0.0;

DV_Left_R=0.0;

DV_Right_L=0.0;

DV_Right_C=0.0;

DV_Right_R=0.0;

Weight_L=0.0;

Weight_C=0.0;

Weight_R=0.0;

if (Noses[0]>(Thresholds[0]+5.0))

{

DV_Left_L=0.0-V_Left;

DV_Right_L=-40.0-V_Right;

Weight_L=Noses[0]-Thresholds[0];

};

if (Noses[1]>(Thresholds[1]+5.0))

{

if (TurningCenter==0)

{

Rand=Random();

TurningCenter=1;

};

DV_Left_C=((30.0*Rand)-V_Left);

DV_Right_C=((-30.0*Rand)-V_Right);

Weight_C=Noses[1]-Thresholds[1];

}

else

TurningCenter=0;

if (Noses[2]>(Thresholds[2]+5.0))

{

DV_Left_R=-40.0-V_Left;

DV_Right_R=0.0-V_Right;

Weight_R=Noses[2]-Thresholds[2];

};

if ((Weight_L+Weight_C+Weight_R)>0.0)

{

DV_Left=((DV_Left_L*Weight_L)+(DV_Left_C*Weight_C)+(DV_Left_R*Weight_R))/(Weight_L+Weight_C+Weight_R);

DV_Right=((DV_Right_L*Weight_L)+(DV_Right_C*Weight_C)+(DV_Right_R*Weight_R))/(Weight_L+Weight_C+Weight_R);

}

else

{

DV_Left=100.0-V_Left;

DV_Right=100.0-V_Right;

};

V_Left=V_Left+(0.2*DV_Left);

V_Right=V_Right+(0.2*DV_Right);

motor(0,V_Left);

motor(1,V_Right);

};

}

void Initialize()

{

V_Left=0.0;

V_Right=0.0;

DV_Left=0.0;

DV_Right=0.0;

Thresholds[0]=100.0;

Thresholds[1]=100.0;

Thresholds[2]=100.0;

TurningCenter=0;

}

void main()

{

Initialize();

poke(0x4000,255);

start_process(NosesSense());

start_process(BumpersSense());

start_process(AvoidObstacles());

}

# AUTHORS

Pedro Kulzer              António Branco