GAROB

University of Aveiro

Department of Electronics and Telecommunications

Tiago Oliveira

Alfredo França

Dr. Keith L. Doty

Scott Jantz

July 1995

# 2.	Table of Contents

2. Table of Contents

3. Abstract

4. Executive Summary

5. Introduction

6. Integrated System

6.1. The Base

6.2. Memory Organization

6.3. The Behaviors

6.4. Programming the robot

6.4.1. Wall Avoidance Behavior

6.4.2. Garbage Finding Behavior

6.4.3. Garbage Dropping Behavior

6.4.4. Recharging Behavior

7. Mobile Platform

8. Actuation

8.1. Summary

8.2. Peripheral Device Selection

8.3. Motors

8.4. IR LED’s

9. Sensors

9.1. Summary

9.2. Proximity detection

9.3. Garbage Detection

9.4. Recharging

10. Behaviors

10.1. Summary

10.2. Collision Avoidance Behavior

10.3. Garbage Collecting Behavior

10.4. Garbage Dropping Behavior

10.5. Charger Finding

11. Experimental Layout and Results

12. Conclusion

13. Documentation

14. Appendices

14.1. Program Code

14.2. Schematic Diagram

# 3.	Abstract

This report describes a system for collecting garbage. The system is based on a mobile robot, GAROB, battery powered. Its function is to find garbage and then push it to the wall. GAROB only carries one piece of garbage at once, so when there is garbage in gripper all other garbage in floor is considered an obstacle. GAROB also avoids obstacles. When the batteries are almost discharged, GAROB will recharge itself.

# 4.	Executive Summary

Our robot(GAROB), avoids obstacles, walls, catches garbage, and avoids garbage when it already have garbage in the gripper, and when the batteries are low it looks for the charger and recharges the batteries..

# 5.	Introduction

This report describes a robot built for the discipline Mobile Robotics of the Master Course in Electronics and Telecommunications at the University of Aveiro. The robot we built, GAROB (GAROB stands for GArbage ROBot), is intended to find garbage and then pushes it to the wall. The robot moves randomly and have to avoid obstacles that appear in his way. Other feature of GAROB is the capacity to automatic recharging when the batteries are low. When batteries of GAROB are low (this is only simulated with a switch due to the lack of time), it starts looking for a beam from the charger. In the charger we have five MLED81 IR LED’s, an oscillator for driving the LED’s, and a power supply. When the beam from the charger is found, GAROB will follow it. When GAROB bumps into the contacts at front of the charger, it knows that has to stop completely for charging. After this it will “awake” and will do the same thing  we talk before. For covering the greatest part of the room, the charger must be located at one corner of the room. For guiding the robot, the charger has a tower permanently emitting an IR beam.

The robot is based on the Motorola MC68HC11A1 microcontroller and was built using an M68HC11EVBU Universal Board from Motorola. All the circuitry developed was buit in wrapping.

GAROB doesn’t have any map, so it simply moves around without any destiny. When it finds a piece of garbage it tries to catch it. If it had successfully grabbed the piece of garbage, GAROB pushes it forward until it hits a wall. Then GAROB goes back a little and continues moving around trying to get some more garbage.

# 6.	Integrated System

## 6.1.	The Base

For building the robot we used the Motorola M68HC11EVBU Universal Board. This board is separated in two physically separated parts:

- a 68HC11 microcontroller clocked at 8 Mhz frequency;

- part for mounting the circuitry developed by us.

## 6.2.	Memory Organization

The robot operates in expanded multiplexed mode, that is, it makes use of an external RAM for storing the program and variables.

All I/O devices, with exception to the motors, are memory mapped, so they are accessed using a memory location. The motors are not memory mapped because they have to be running “all” the time. The memory organization is as shown in figure 6.1. Of the total 64 kbytes of memory that microcontroller addresses, the upper 32 kbytes are reserved to the RAM. The I/O devices are located between the address 0x4000 and 0x7FFF.

| RAM | FFFF 8000 |
| --- | --- |
| I/O DEVICES | 7FFF 4000 |
|  | 3FFF 0000 |

Figure 6.1	Memory map

## 6.3.	The Behaviors

A diagram of the whole system can be viewed in figure 6.2.

Figure 6.2	Overall system

## 6.4.	Programming the robot

The robot was programmed in C language. We used Interactive C (IC), a real time C command interpreter that allows multitasking. The program is downloaded from an IBM-PC compatible computer to the robot’s RAM using IC.

For simplifying the reading of the program code, we used the following convention:

| Definitions | ® | all capital characters (e.g. MOTOR_RIGHT, IR_DELAY) |
| --- | --- | --- |
| Global variables | ® | begin with a capital character (e.g. SpeedRight, ThresFrontFar) |
| Local variables | ® | begin with a non-capital character (e.g. nextMove, timeRotation) |

### 6.4.1.	Wall Avoidance Behavior

This behavior gives GAROB the capability of avoiding obstacles. Its flow chart is represented in figure 6.1.

Figure 6.1	Flow chart of the Wall Avoidance behavior

The action makes distance >= near may be represented by another flow chart like the one represented in figure 6.2. In that chart the decision boxes only take effect if distance < near as can be seen in the chart represented in figure 6.1.

Figure 6.2	Flow chart of makes distance >= near

In both flow charts, the action rotates random is achieved by randomly choosing a direction and then rotating to that direction during a random time. There’s no random number generator available in IC. So the answer to the questions How do we get a random direction? and How do we get a random interval of time? is in the system clock. For getting a pseudo-random number we read the system clock in milliseconds. If we want a direction, we keep only the last bit. If this bit is 0 (zero) we turn to the right or rotate clockwise, if it is 1 we turn to the left or rotate anticlockwise. If we want a time interval, we keep only the last 12 bits, for having an interval of time that may vary between 0 and 212 = 4096 milliseconds (note, however that the 12 bits are not mandatory, we may have used less or more bits).

### 6.4.2.	Garbage Finding Behavior

This behavior is responsible for finding garbage. The action to be taken after garbage is found depends on the actual state of the gripper. If it is empty, the robot must collect the garbage, otherwise GAROB must deviate from garbage. The flow chart for this behavior is represented in figure 6.3.

Figure 6.3.	Flow chart of the Garbage Finding behavior

### 6.4.3.	Garbage Dropping Behavior

After grabbing the garbage, it must have to be dropped. This behavior leaves the garbage near a wall. Its flow chart is represented in figure 6.4.

Figure 6.4.	Flow chart of the Garbage Dropping behavior

### 6.4.4.	Recharging Behavior

This is the behavior that permits automatic recharging when the batteries are low. Its flow chart is represented in figure 6.5.

Figure 6.5.	Flow chart of the Recharging behavior

# 7.	Mobile Platform

The robot platform is a wood made, circular shaped platform. For it to move, we supplied it with three wheels. The two front wheels are drive wheels, and the back wheel is a caster. The platform is shown in figure 7.1. Its diameter is 23 cm and the distance between the two drive wheels is of 22 cm.

Figure 7.1	GAROB's down view

At the front (bottom side) of the robot there’s a gripper that will permit GAROB to catch garbage. The gripper has a rectangular form and its dimensions, in centimetres, can be seen in figure 7.2. The pieces of garbage must not exceed any of these dimensions, otherwise the robot normal behavior will be affected.

Figure 7.2	Gripper dimensions

The EVBU board is fixed on top of the platform. At the back right corner of the robot there are two switches. The inner most switch is to switch  GAROB between the ON state and STANDBY state. In the ON state, GAROB is on. The STANDBY state is intended to keep the loaded program in memory while the robot is “without life”. The outer most switch switches between the Special-Bootstrap and the Expanded Multiplexed modes. After changing from one mode to another, the robot must be reseted.

The motors are glued to the platform. Initially the wheels were glued, but one wheel insisted in fall apart so we screw it up. The caster is screwed to platform. The gripper is of plastic material and is also screwed to the platform.

Up in the platform there’s a tower for supporting the sensors to follow the beam from the charger. At the front periphery there are two contacts that permit GAROB to autonomously recharging.

# 8.	Actuation

## 8.1.	Summary

GAROB has two DC motors, each one coupled to each drive wheel. In this manner we can move it around in any direction by simply controlling the speed of both motors.

| Actuator Type | Function | Location | Quantity |
| --- | --- | --- | --- |
| DC motor | Move the robot | Bottom at both sides | 2 |
| Infrared (IR) | Wall/obstacle avoidance | At top periphery, front and both sides | 3 |
| Infrared (IR) | Find/avoid garbage | At bottom periphery, in both sides | 2 |

Table 8.1	Description of actuators

## 8.2.	Peripheral Device Selection

The peripheral device selection is made with the 74138 chip. There are eight outputs for selecting an external device, numbered from CS0 to CS7. The association between the outputs and the memory addresses is shown in table 8.2. The odd CS outputs may only address devices for reading. The even CS outputs may only address devices for writing.

|  |  |  |  | R/W |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A15 | A14 | A13 | A12 | W | R |  | Address |
| 0 | 1 | 0 | 0 | CS0 | CS1 |  | 0x4ddd |
| 0 | 1 | 0 | 1 | CS2 | CS3 |  | 0x5ddd |
| 0 | 1 | 1 | 0 | CS4 | CS5 |  | 0x6ddd |
| 0 | 1 | 1 | 1 | CS6 | CS7 |  | 0x7ddd |

Table 8.2	Addresses of output devices (d means don’t care)

For accessing one of the CS outputs, we make a poke(add, data) or peek(add) to the correspondent address if it is a read-only device or a write-only device, respectively.

## 8.3.	Motors

For controlling the motors we used the chip L293 which allows the control of two motors. The circuit used to drive the motors is shown in figure 8.1. The algorithm for controlling the motors was already made, so we simply use it as it was.

Figure 8.1	Motor drive

## 8.4.	IR LED’s

The IR LED's used are the MOTOROLA MLED81.

The IR LED’s are selected by the signal CS0, so for accessing them we write a byte into the memory position 0x4000. This byte indicates what are the LED’s to activate by putting a logical 1 in the corresponding bit position. The correspondence between the bit positions and the physical device are shown in figure 8.2..

| Function | Garbage detection |  | Charger finding |  |  | Proximity detection |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location | bottom right | bottom left |  |  |  | top left | top center | top right |
| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Figure 8.2	IR LED’s bit positions

The IR sensors used are sensible only to 40 kHz, so we have to modulate at 40 kHz the IR light from the IR LED’s. To achieve this modulation, the output control (OC) pin of the latch 74374 is driven by a 40 kHz signal. This 40 kHz signal is obtained by dividing the microcontroller E-clock (2 Mhz) by 50 using the 74390 chip. The clock for writing a byte to the latch is one of the chip select (CS) signals. In our case, the CS0 is used, that is, the latch is at address 0x4ddd as can be seen from the figure 8.2. The circuit used is depicted in figure 8.3.

Figure 8.3	IR module

# 9.	Sensors

## 9.1.	Summary

The sensors used are SHARP GP1U528X sensors with a little modification to allow receiving an analogue signal based on the infrared intensity signal received.

The output signal from each sensor depends on the IR light that reaches it. This light is the light emitted by the corresponding LED after being reflected by some surface. So, each IR module will serve to measure the distance between it and an obstacle. The IR sensors available are sensible only to 40 kHz infrared light, so the IR LED's for this effect must be driven by a 40 kHz source.

After constructing an IR module, we took measurements of the signal at the output pin of the IR sensor and related them to the distance of the reflective surface. The reflective surface used is a matte grey box and the system used is represented in figure 9.1. The obtained results are shown in table 9.1, where the column value represents the value returned by the sensor after A/D conversion. However notice that these values depend on the reflective surface, namely on its color (darker colors reflect lesser than lighter colors).

Figure 9.1	Configuration for measuring value vs. distance

| distance (cm) | value |
| --- | --- |
| 0 | 130 |
| 6 | 130 |
| 8 | 125 |
| 9 | 120 |
| 11 | 115 |
| 13 | 110 |
| 15 | 105 |
| 17 | 100 |
| 20 | 95 |
| 25 | 90 |

Table 9.1	Sensor read value vs. obstacle distance

Procedure for measuring a distance:

- activate the corresponding IR LED;

- wait for the transients to die;

- read the value at the IR sensors;

- deactivate the corresponding IR LED.

All the sensors used are described in table 9.2.

| Sensor Type | Function | Location | Quantity |
| --- | --- | --- | --- |
| Infrared (IR) | Wall/obstacle avoidance | At top periphery, front and both sides | 3 |
| Infrared (IR) | Find/avoid garbage | At bottom periphery, in both sides | 2 |
| Infrared (IR) | Find charger | Top of tower | 3 |
| Infrared (IR) | Detect garbage in gripper | Gripper | 1 |
|  | Batteries state |  |  |
| Contact | Charger connected | Front |  |

Table 9.2	Description of sensors

The IR sensors for Wall/obstacle avoidance are the pair of the IR LED’s for Wall/obstacle avoidance. The IR sensors for Find/avoid garbage are the pair of the IR LED’s for Find/avoid garbage.

## 9.2.	Proximity detection

All sensors used for proximity detection are sensible only to infrared light modulated at 40 kHz. Each IR sensor is associated with one IR LED. Each set of IR LED plus IR sensor will be called an IR module.

For detecting obstacles, like walls, we supplied GAROB with three IR modules disposed as shown in figure 9.2 a). The majority of the movement is made in the forward direction, so it explains why the robot only have sensors at the front side.

## 9.3.	Garbage Detection

For finding pieces of garbage, GAROB has two IR modules, one on each side of the gripper as can be seen in figure 9.2 b).

Figure 9.2	Sensor position: a) top view; b) down view

## 9.4.	Recharging

For detecting the beam emitted by the charger, GAROB has a tower with three sensors in its top, at the same height than the emitter LED’s of the charger. GAROB is guided by the sensor that has the greater intensity of all the three sensors.

The docking is sensed by the presence of a voltage at the robot contacts for charging.

The state of the batteries have to be monitored. The best way to do this is by using an analogue input, but we have no more analogue inputs. So we are going to use a switch to full the state of the batteries.

# 10.	Behaviors

## 10.1.	Summary

To achieve the proposed goals GAROB was designed for, it must have at least four behaviors:

·	avoiding obstacles (named wall_avoidance);

·	recharging batteries (named find_charger);

·	dropping garbage (named drop_garbage);

·	collecting garbage (named find_garbage).

In order to select one of this behaviors, it will be needed a procedure that decides which one is the active behavior. This procedure is called behavior_arbitrate. The active behavior attribution follows a priority scheme. The behaviors were listed in order of priority from the highest priority behavior (wall_avoidance) to the lowest priority behavior (collecting_garbage).

When collecting garbage, and if after pointing GAROB to the piece of garbage, it can't be put on the gripper, then GAROB moves back a little and the next behavior will be decided by the behavior arbitration procedure. The backward move is necessary because the garbage could have stayed at the bottom of the robot and limitates its movement.

## 10.2.	Collision Avoidance Behavior

With this behavior the robot is given the feature of obstacle avoidance. To achieve this, the robot needs to know the distance between itself and the obstacles that appear in the direction of movement. This distance will be measured with an IR beam, using an IR LED and an IR sensor.

The whole visibility field of the robot is separated in three fields: far-zone, near-zone and danger-zone. This separation is shown in figure 10.1.

Figure 10.1	Visibility fields

When GAROB enters the far-zone he slows down to allow more accuracy. When it reaches the near-zone, it moves away from the obstacles. If GAROB enters the danger-zone, it goes back a little and simply turns in a random direction during a random interval of time. This random move is also made when an obstacle in the near-zone is detected with the front sensor.

The slowing down in the far zone enables more accuracy in approaching obstacles and a better turning away from obstacles.

Initially for deviating from obstacles, GAROB made a turn, but this strategy isn’t really good, because GAROB insisted in touching the walls with its the lateral sides. We changed the turning to a rotation and the behavior improved significantly.

At the first sight, the danger-zone seems unnecessary, but this is not true. Because the Wall Avoidance sensors don’t cover the whole area in front of the robot, there are some obstacles like chair legs and some corners that aren’t seen by GAROB, so it bumps into these obstacles. After bumping, the wheels continue rotating and, in most cases, the robot position is slightly changed. If after this position change, a sensor sees the obstacle, the obstacle is in the danger-zone, so GAROB has to move back to continue.

## 10.3.	Garbage Collecting Behavior

With this behavior, GAROB will try to find and grab small pieces of garbage, that is, put them inside the gripper. To achieve this goal there are two sets of IR modules (sensor plus LED), that detect if the garbage is at the left side or at the right side of the gripper. Based on information from this modules, GAROB will rotate in order to align the gripper with the piece of garbage. Then the robot moves forward to put the garbage inside the gripper.

The gripper has an IR barrier that enables GAROB to know if there is something at the gripper. With this barrier there is almost no limitation on the shape of the pieces of garbage to be detected, except those already described.

## 10.4.	Garbage Dropping Behavior

With this behavior GAROB will go toward a wall to drop the garbage that’s on the gripper. Before reaching the wall, GAROB reduces the speed in order to hit the wall. When GAROB reaches the wall, it stops and then goes back and turn random. In this way the garbage that was on the gripper has stayed by the wall.

## 10.5.	Charger Finding

This behavior will measure the batteries charge. When the batteries are too low, the robot must have to go toward charger to charge itself. With this behavior GAROB accomplishes this goal.

This behavior is already made and is working correctly. We do not yet try to join this behavior with the other behaviors.

# 11.	Experimental Layout and Results

All behaviors are working properly when used independently, but when we try to join them we experienced some problems. We solve all the problems except one: the robot insists in go over the garbage when the gripper has garbage in it.

We do not try yet joining the find charge behavior with all the others.

# 12.	Conclusion

As a conclusion GAROB works fine. We ennumerate here the problems we had to face

Software

- With many behaviors, arbitration become more complex and difficult. Sometimes the reality is very different than we thought it, and robot have behaviors that are very strange.

Hardware

- When we try to apply an IR barrier in the gripper for detecting garbage, we experienced some problems because we do not have another analogue input, so we tried to use  the Sharp sensors unmodified to have a digital signal. For using the sensors this way, we have to turn on and off the 40 kHz IR light at a frequency lower than approximately 100 Hz. Using this modulation, the sensor outputs a logical 1 when there is garbage at the gripper, but if there is no garbage at the gripper the sensor outputs a square wave with the frequency of the IR light. So the logical level is undefined. We tried another possibilities like a contact switch, but this solution isn’t so good because garbage could be very light and not close the switch contacts. We made some experiments with IR barriers, but the luck wasn’t with us. All this IR barriers didn’t work very well and we spent a lot of time trying to put them to work. One last possibility is to free an analogic port, but to do this we have to use only two IR sensors (left and right) for wall avoidance. As a temporary solution we put a switch connected to a digital port for simulating the presence of garbage, so we can implement and try the software for this behaviors.

- Because we are running out of time, we will simulate the battery charge checking with a switch. This switch is connected to a digital port, and when on means that batteries are low, so the robot can pursuit the beam to charger. When the switch is off GAROB can do everything else: collect garbage, avoid walls....

# 13.	Documentation

Joseph L. Jones, Anita M. Flynn, “Mobile Robots”, A K Peters

Fred Martin, “The 6.270 Robot Builder’s Guide”, The MIT LEGO Robot Design Competition

“M68HC11EVBU Universal Board User’s Manual”, Motorola

“M68HC11 E Series Technical Data”, Motorola

“M68HC11 Referebce Manual”, Motorola

# 14.	Appendices

## 14.1.	Program Code

/*

*

* July 1995

*

*/

/* DEFINITIONS */

int RIGHT = 0,

LEFT = 1;

int FALSE = 0,

TRUE = 1;

/* next move values */

int FREEZE     = 0,     /* stop */

FWD_SLW_SP = 1,     /* move forward at slow speed */

FWD_HLF_SP = 2,     /* move forward at half speed */

FWD_FUL_SP = 3,     /* move forward at full speed */

BAK_SLW_SP = 4,     /* move backward at slow speed */

TURN_LEFT  = 5,     /* turn left */

TURN_RIGHT = 6,     /* turn right */

ROT_CLKW   = 7,     /* rotate clockwise */

ROT_ACLKW  = 8;     /* rotate anticlockwise */

int MOTOR_RIGHT = 0,    /* motor positions */

MOTOR_LEFT = 1;

long IR_DELAY = 50L;    /* delay time for IR commutation */

float S0 =  10.0,       /* motor speeds */

S1 =  20.0,

S2 =  40.0,

S3 =  60.0,

S4 =  80.0,

S5 = 100.0;

int PORT_A = 0x1000;    /* port A address */

/* GLOBAL VARIABLES */

float SpeedRight = 0.0,   /* actual motor speeds */

SpeedLeft = 0.0,

SpeedRightOld = 0.0,/* old motor speeds */

SpeedLeftOld = 0.0;

int ExistGarbage = FALSE; /* exists garbage in greeper? */

int NextMoveAvoid   = FREEZE,    /* next move for obstacle avoidance */

NextMoveColl    = FWD_FUL_SP,/* next move for collecting garbage */

NextMoveDrop    = FREEZE,    /* next move for dropping garbage */

NextMoveCharger = FREEZE;

/* eye parameters definition */

int EYE_ADDRESS = 0x4000; /* eyes memory address */

int EyeMask = 0x07,       /* normal eyes data position mask */

EyeRightBit = 0,      /* normal right eye data position */

EyeCenterBit = 1,     /* normal center eye data position */

EyeLeftBit = 2;       /* normal left eye data position */

int EyeDownMask = 0x30,   /* down eyes data position mask */

EyeDownRightBit = 4,  /* down right eye data position */

EyeDownLeftBit = 5;   /* down left eye data position */

int EyeTowerRightBit = 3,

EyeTowerCenterBit = 6,

EyeTowerLeftBit = 7;

/* distance IR thresholds levels attribution for visibility fields */

int ThresFrontFar    = 95,   /*  95 *//* threshold levels for IR sensors */

ThresFrontNear   = 115,  /* 115 */

ThresFrontDanger = 126,  /* 126 */

ThresSideFar    = 89,    /*  95 */

ThresSideNear   = 110,   /* 110 */

ThresSideDanger = 125;   /* 125 */

/* threshold levels for collecting garbage */

int ThresGarbageColLo =  90,

ThresGarbageColHi = 100;

/* eye readings */

int eyeRight = 0,

eyeCenter = 0,

eyeLeft = 0;

int eyeDownRight = 0,

eyeDownLeft = 0;

int eyeTowerRight = 0,

eyeTowerCenter = 0,

eyeTowerLeft = 0;

/*

* void wait(int m_second)

* July 1995

*

* Stops the process during m_second miliseconds.

*/

void wait(int m_second) {

long stopTime;

stopTime = mseconds() + (long)m_second;

while( stopTime > mseconds() ) defer();

}

/*

* void freeze()

* July 1995

*

* Stop the robot

*/

void freeze() {

SpeedRight = 0.0;

SpeedLeft = 0.0;

}

/*

* void forward_fullSpeed()

* July 1995

*

* Make the robot moves forward at full speed.

*/

void forward_fullSpeed() {

SpeedRight = S5;

SpeedLeft = S5;

}

/*

* void forward_halfSpeed()

* July 1995

*

* Make the robot moves forward at half speed.

*/

void forward_halfSpeed() {

SpeedRight = S3;

SpeedLeft = S3;

}

/*

* void forward_slowSpeed()

* July 1995

*

* Make the robot moves forward at slow speed.

*/

void forward_slowSpeed() {

SpeedRight = S1;

SpeedLeft = S1;

}

/*

* void backward_slowSpeed()

* July 1995

*

* Make the robot moves backward at slow speed.

*/

void backward_slowSpeed() {

SpeedRight = -S1;

SpeedLeft = -S1;

}

/*

* void turn_left()

* July 1995

*

* Make the robot turns left.

*/

void turn_left() {

SpeedLeft = S0;

SpeedRight = S3;

}

/*

* void turn_right()

* July 1995

*

* Make the robot turns right.

*/

void turn_right() {

SpeedRight = S0;

SpeedLeft = S3;

}

/*

* void rotate_clockwise()

* July 1995

*

* Make the robot rotates clockwise.

*/

void rotate_clockwise() {

SpeedRight = -S1;

SpeedLeft = S1;

}

/*

* void rotate_anticlockwise()

* July 1995

*

* Make the robot rotates anticlockwise.

*/

void rotate_anticlockwise() {

SpeedRight = S1;

SpeedLeft = -S1;

}

/*

* void sensor_read()

* July 1995

*

* Takes readings from the IR sensors.

*/

void sensor_read() {

while (1) {

poke(EYE_ADDRESS, EyeMask | EyeDownMask);

msleep(IR_DELAY);

eyeRight = analog(EyeRightBit);

eyeCenter = analog(EyeCenterBit);

eyeLeft = analog(EyeLeftBit);

eyeDownRight = analog(EyeDownRightBit);

eyeDownLeft = analog(EyeDownLeftBit);

poke(EYE_ADDRESS, 0);

msleep(IR_DELAY);

wait(50);

}

}

/*

* void motor_write()

* July 1995

*

* Make the motors turn at the defined speed.

*/

void motor_write() {

while(1) {

if ( (SpeedRightOld*SpeedRight < 0.0) ||

(SpeedLeftOld*SpeedLeft < 0.0) ) {     /* inverting motor rotation */

motor(MOTOR_RIGHT, 0.0);          /* stop motors */

motor(MOTOR_LEFT, 0.0);

}

motor(MOTOR_RIGHT, SpeedRight);

motor(MOTOR_LEFT, SpeedLeft);

SpeedRightOld = SpeedRight;         /* saves actual speed values */

SpeedLeftOld = SpeedLeft;

wait(100);

}

}

/*

* void behavior_arbitrate()

* July 1995

*

* Arbitrates the active behavior based on the actual conditions.

*/

void behavior_arbitrate() {

int nextMove;

while (1) {

/*if ( ExistGarbage == TRUE )

if ( NextMoveDrop == FWD_FUL_SP )

nextMove = NextMoveColl;

else

nextMove = NextMoveDrop;

else

if ( NextMoveAvoid == FWD_FUL_SP )

nextMove = NextMoveColl;

else

nextMove = NextMoveAvoid;*/

nextMove = NextMoveCharger;

if      ( nextMove == FREEZE     ) freeze();

else if ( nextMove == FWD_SLW_SP ) forward_slowSpeed();

else if ( nextMove == FWD_HLF_SP ) forward_halfSpeed();

else if ( nextMove == FWD_FUL_SP ) forward_fullSpeed();

else if ( nextMove == BAK_SLW_SP ) backward_slowSpeed();

else if ( nextMove == TURN_LEFT  ) turn_left();

else if ( nextMove == TURN_RIGHT ) turn_right();

else if ( nextMove == ROT_CLKW   ) rotate_clockwise();

else if ( nextMove == ROT_ACLKW  ) rotate_anticlockwise();

wait(100);

}

}

/*

* void wall_avoidance()

* July 1995

*

* Behavior for avoiding walls.

*/

void wall_avoidance() {

int timeRotation,

dirRotation;

while (1) {

if( (eyeRight > ThresSideDanger) ||

(eyeLeft > ThresSideDanger)  ||

(eyeCenter > ThresFrontDanger) ) {      /* inside danger-zone */

NextMoveAvoid = BAK_SLW_SP;

wait(300);

timeRotation = (int)mseconds() & 0x0FFF;

dirRotation = (int)mseconds() & 0x0001;

if ( dirRotation == LEFT ) NextMoveAvoid = ROT_ACLKW;

else NextMoveAvoid = ROT_CLKW;

wait(timeRotation);

}

else

if ( eyeCenter > ThresFrontNear ) {       /* inside near-zone */

timeRotation = (int)mseconds() & 0x03FF;

dirRotation = (int)mseconds() & 0x0001;

if ( dirRotation == LEFT ) NextMoveAvoid = ROT_ACLKW;

else NextMoveAvoid = ROT_CLKW;

wait(timeRotation);

}

else if ( (eyeLeft > ThresSideNear) && (eyeLeft > eyeRight) )

NextMoveAvoid = ROT_CLKW;/*turn_right();*/

else if ( eyeRight > ThresSideNear )

NextMoveAvoid = ROT_ACLKW;/*turn_left();*/

else

if ( (eyeRight > ThresSideFar) ||

(eyeLeft > ThresSideFar)  ||

(eyeCenter > ThresFrontFar) )

NextMoveAvoid = FWD_SLW_SP;             /* inside far-zone */

else NextMoveAvoid = FWD_FUL_SP;          /* outside visibility field */

wait(100);

}

}

/*

* void collect_garbage(int side)

* July 1995

*

* This procedure rotates the robot to the desired direction

* until it sees no more garbage at that side.

*/

void collect_garbage(int side)

{

int eyeTmp;

if ( side == RIGHT ) {

NextMoveColl = ROT_CLKW;

eyeTmp = eyeDownRight;

}

else {

NextMoveColl = ROT_ACLKW;

eyeTmp = eyeDownLeft;

}

/* wait for the garbage to be in front of the robot */

while ( eyeTmp > ThresGarbageColLo ) {

if ( side == RIGHT ) eyeTmp = eyeDownRight;

else eyeTmp = eyeDownLeft;

}

NextMoveColl = FWD_FUL_SP;

}

/*

* void find_garbage()

* July 1995

*

* Behavior for finding garbage.

*/

void find_garbage() {

while(1) {

if ( /*(eyeRight < ThresSideFar) &&*/

(eyeDownRight > ThresGarbageColLo) &&

(eyeDownRight < ThresGarbageColHi))     /* garbage at the right side */

if ( ExistGarbage == FALSE ) collect_garbage(RIGHT);

else collect_garbage(LEFT);

else if ( /*(eyeLeft < ThresSideFar) &&*/

(eyeDownLeft > ThresGarbageColLo) &&

(eyeDownLeft < ThresGarbageColHi)) /* garbage at the left side */

if ( ExistGarbage == FALSE ) collect_garbage(LEFT);

else collect_garbage(RIGHT);

wait(100);

}

}

/*

* void drop_garbage()

* July 1995

*

* Behavior for dropping garbage.

*/

void drop_garbage()

{

while(1) {

if( (eyeRight  > ThresSideDanger ) ||

(eyeLeft   > ThresSideDanger ) ||

(eyeCenter > ThresFrontDanger) ) {

NextMoveDrop = FREEZE; wait(2000);

NextMoveDrop = BAK_SLW_SP; wait(3000);

NextMoveDrop = TURN_LEFT; wait(2000);

}

else if ( (eyeRight > ThresSideFar) ||

(eyeLeft > ThresSideFar)  ||

(eyeCenter > ThresFrontFar) )

NextMoveDrop = FWD_SLW_SP;

else

NextMoveDrop = FWD_FUL_SP;

wait(100);

}

}

void find_charger() {

while (1) {

eyeTowerRight = analog(EyeTowerRightBit);

eyeTowerCenter = analog(EyeTowerCenterBit);

eyeTowerLeft = analog(EyeTowerLeftBit);

if ( (peek(PORT_A) & 0x02) != 0 ) {

NextMoveCharger = FREEZE;       /* charger attached */

sleep(3.0);

}

else {

if ( (eyeTowerLeft > 127)   ||

(eyeTowerCenter > 127) ||

(eyeTowerRight > 127) )

NextMoveCharger = FWD_SLW_SP;   /* approaching charger */

else

if ( (eyeTowerLeft > eyeTowerCenter) &&

(eyeTowerCenter > eyeTowerRight) )

NextMoveCharger = TURN_LEFT;

else if ( (eyeTowerRight > eyeTowerCenter) &&

(eyeTowerCenter > eyeTowerLeft) )

NextMoveCharger = TURN_RIGHT;

else

NextMoveCharger = FWD_FUL_SP;

}

wait(100);

}

}

/* ------------------------------------------------------------------ */

void main()

{

int sensor_read_pid,

motor_write_pid,

wall_avoidance_pid,

find_garbage_pid,

drop_garbage_pid,

find_charger_pid,

behavior_arbitrate_pid;

ExistGarbage = TRUE;

NextMoveAvoid = FWD_FUL_SP;

NextMoveDrop = FWD_FUL_SP;

NextMoveColl = FWD_FUL_SP;

NextMoveCharger = FWD_FUL_SP;

sensor_read_pid = start_process(sensor_read());

motor_write_pid = start_process(motor_write());

/*  wall_avoidance_pid = start_process(wall_avoidance());

find_garbage_pid = start_process(find_garbage());

drop_garbage_pid = start_process(drop_garbage());*/

find_charger_pid = start_process(find_charger());

behavior_arbitrate_pid = start_process(behavior_arbitrate());

}

## 14.2.	Schematic Diagram