University of Aveiro

Electronics and Telecommunications’ Department

Cyclop

Autonomous Mobile Robots

Professor Keith L. Doty

Scott Jantz

July 95

Rosa Maria Charneca Pasadas - 6319 ET

Rui Jorge Ferreira da Costa - 5691 ET

Table of Contents

Abstract	3

Introduction	4

Integrated System	4

Mobile Platform	4

Actuation	6

Sensors	7

Behaviors	7

Experimental Layout and Results	10

Conclusion	14

Documentation	15

Appendices	16

Abstract

This paper is a brief description of Cyclop (presented firstly as Fly): an autonomous mobile robot that was designed to do simple obstacle avoidance, bump detection and floor absence detection (in order to prevent Cyclop from falling into holes or abysses). It integrates six IR sensors: three in the front for obstacle avoidance (used in the first presentation), two in front of Cyclop’s two wheels for abyss detection and one pointing to a rotating mirror that is supposed to perform a 360º obstacle detection around the robot (this is Cyclop’s most salient characteristic), hence its name. It also integrates three bumpers in the front to perform bump detection.

Cyclop has two actuators (motor drivers) that respond accordingly to the stimulus input at his sensors.

The obstacle avoidance algorithm used in the first presentation was performed by a non-linear dynamic system differential equation for heading direction dynamics and a linear function for speed dynamics.

By the time this report was written we were not able yet to let Cyclop use its rotating IR sensor rather than the three frontal IR sensors to perform obstacle avoidance. We will do our best to accomplish that until the final presentation.

Executive Summary

In this project the obstacle avoidance behavior was implemented in the first stage with three IR sensors in Cyclop’s front. Later on we tried to do it with one IR led and one IR sensor that were above a rotating mirror, and finally we used just one IR sensor above the mirror and three pointing ahead IR leds in Cyclop’s front.

The three pointing ahead leds illuminate the obstacles and the mirror reflects the beams (reflected from the obstacles) to the IR sensor that’s above it.

To drive the motor that rotates the mirror, we used a PWM signal that was programmed in the MC68HC11A0FN microprocessor in an adaptive way. For some unknown reason the PWM signal changed in an unusual way during program’s execution and we decided to make a circuit that generates the PWM signal.

This circuit compares data from a counter and data that was defined by the user with four switches, giving a PWM signal as a result of this comparison.

This evolution will be explained in more detail in the “Experimental Layout and Results” item.

Introduction

This project is integrated in the M. Sc. in Electronics Engineering subject “Autonomous Mobile Robots” that is still taking place in the University of Aveiro (June/July, 95). It consists of designing an autonomous robot, Cyclop, that can perform obstacle avoidance using a rotating IR sensor. We tested a particular algorithm for obstacle avoidance with three frontal IR sensors that worked fairly well. This last sensor gave us (and still is giving) many practical problems explained in detail in the “Experimental Layout and Results” item.

Integrated System

The following figure is Cyclop’s organisational description.

Obstacle Avoidance

Speed Dynamics

Random Turn

Vertigo Behavior

Bumper Behavior

Three frontal IR sensors

Arbitration Network

Cyclop sensor

Abyss Sensor

Bumper sensors

figure 5

Mobile Platform

At first we thought about using a 45º mirror and make the adjustment of the angle between the beam and the floor adjusting the angle between the IR led and the rotational axis. As one can see in the next figure, this is a basic error, for if in one instant the beam is pointing down, it will point up after the mirror rotates 180º.

a

a

45º

45º

i

a

II

I

Motor

Motor

figure 6

To achieve a solution we need axial symmetry. The angle of the beam will have to be determined by the angle between the mirror and the floor:

· The IR emitter must be placed on the rotational axis

· b must be chosen to design g

g

b

a

a

a

IR emitter/receiver

figure 7

2a+b=90º and a+g=90º Þ g=45+b/2

The mobile platform was designed as shown in the figure below.

figure 8

Actuation

Arbitration process’ end

Arbitration process’ beginning

Use bump behavior’s output

Use vertigo behavior’s output

Use obstacle avoidance behavior’s output

Did I hit something?

Is there a hole?

figure 9

Sensors

The following table summarises Fly's sensors.

| Physical Sensor | Function | Location | Quantity |
| --- | --- | --- | --- |
| IR | avoid obstacles | front, front left, front right | 3 |
| IR | avoid obstacles | up, pointed at the rotating mirror | 1 |
| IR | avoid holes | under the robot’s board, before the wheels | 2 |
| Bumper | Bump detection | front | 3 |

The bumpers are simple switches that give the robot a digital input. They were designed the same way our colleagues’ A. Branco and P. Kulzer.

Behaviors

For the first presentation we just implemented two behaviors: obstacle avoidance (using the three frontal IR sensors) with controlled speed and turn back (when it reached a local minimum that he could not leave if it had only the obstacle avoidance behavior).

The obstacle avoidance behavior is based on a non-linear dynamic system differential equation, i.e., something like:

dq/dt = f(q),

where f(.) is a non-linear function [5].

Let us now introduce the concept of repellor: a repellor is a zero of the previous equation with negative values of dq/dt  at its left and positive at the right, at some neighbourhood of that point. Like this, if we are close to the zero at its right, dq/dt >0, so q increases, getting away from the zero. The same happens at the left of the zero: we get more and more away from the zero, hence the name.

Attractor

Repellor

Zooming the root’s neighbourhood...

Zooming the root’s neighbourhood...

d/dt

d/dt

figure 10

We understand easily the concept of attractor under the same ideas.

We can not use simple linear functions like the ones above to implement obstacle avoidance for a repellor has a limited influence around him, and the repellor represented has unlimited influence.

In our task we used the following function as the basic repellor:

Repellor

d/dt

figure 11

As the obstacle gets closer, the height and width of  the two pulses become higher. The integration of the three frontal sensors is accomplished by an addition of the resultant repellor functions.

The speed dynamics is supposed to be directly related to the duty-cycle of the motor drivers, and is accomplished by a simple linear function:

V = Vmax -m*(Sensor-S0), where m=(Vmax-Vmin)/(Sat-S0)

S0

Ssat

Vmin

Vmax

Speed

Sensor Readings

figure 12

When the robot falls in local minima its speed decreases to a certain value. When the robot’s arbitration network detects that, it switches to the turn back behavior.

Mathematical representation of the world might be seen by some as traditional AI [4]. It must be seen as well under another point of view: mathematical language is a synthetic one. One can say much using few words, thus reducing substantially the code.

Cyclop also supports bump and vertigo behaviour.

After a bump it turns back. Afterwards it turns to the left or to the right if it felt a right or a left bump, respectively.

After a dangerous height detection it turns back for a while and performs a random rotation to avoid it.

Experimental Layout and Results

As we already said in previous items, by the time this report was written we had not succeeded yet in accomplishing a reasonable obstacle avoidance behavior using only the rotating IR sensor. In this item we will describe the problems we experienced and the causes we think that caused them.

These are the waveforms our hardware put at the ADC’s input:

a) Colimed illuminating leds, uncolimed sensor

Obstacles

(the authors)

Analog reading:156

b) Uncolimed illuminating leds, colimed sensor

Analog reading: 116-119

Synchronizing led’s pulses

figure 13

In the beginning we used both a led and an IR sensor above the mirror, as close as possible to the motor’s rotational axis. The result was a pulse width as large as in  situation a). Afterwards we put three colimed leds in the front of the robot, instead of close to the sensor. The situation did not improve very much. Finally we uncolimed the three leds and colimed the sensor, improving the results, as we can see in situation b).

{...}

/*Duty cycle processing*/

error = IrMotorPeriod-DESIRED_IR_MOTOR_PERIOD;

Integral += error*Ki;/*Determining integral additive...*/

/*if(Integral>0.8) Integral=0.8;

if(Integral<-0.8) Integral=-0.8;/*Limiting integral additive...*/

dutyCycle = Kp*error + Integral;/*PI controller*/

if (dutyCycle> 1.0) dutyCycle=1.0;

if (dutyCycle< 0.0) dutyCycle=0.0;

{... }

For the first test our PWM was generated by the CPU and updated according to a PI algorithm:

The period read from the board was unstable. So, we programmed a constant PWM.

We simply used the OC1M4 and OC1D4 bits to program port A’s pin 4, just like the following process’ code extract::

...

bit_set(OC1M, 0b00010000);

while(1){

bit_set(OC1D, 0b00010000);

wait(0.07);

bit_clear(OC1D, 0b00010000);

wait(0.03);}

...

The resulting PWM was barely correct, but in some periods there was no signal:

figure 14

Afterwards we tried to use the HC11’s TOC4 register. The PWM was OK, but the system time was not functioning properly. It would measure time delays of 1 second while we measured 5 seconds in our watches.

Finally we used a binary counter, a binary comparator and a 555 to generate our PWM, as can be seen in the electrical scheme. We used a very simple routine to detect the signal that our synchronising led gives when the mirror faces the back of the robot, as one can see in the “vision()” function, in the appendix.

The mirror’s rotation period read from the board was usually lower than the correct one, read from the oscilloscope and it varied a lot, while the oscilloscope waveform had a stable period. There were times, however, when the period read from the board was quite stable. Usually this occurred when the vision routine was the only one running. In this situation the ADC values read from the sensor were also reasonably stable.

In summary, when the period read from the board was too short compared to the one read from the oscilloscope or when it was unstable, the ADC values were inaccurate.

Conclusion

Without a stable period it is impossible to relate the samples we are taking to their positions, because we are basing ourselves on the last period to calculate the next sample period. Even if you do a completely asynchronous routine, you still have to decide when are you going to sample again, so the last rotating period must be close to the next one.

We are sure that the smaller period is not due to close obstacles’ readings because we took measurements with the motor standing still and they were quite lower than the synchronising led’s signal.

We are also sure that the ADC is working properly because there were some experiments in which the period read from the board was reasonably stable (errors of about 16%) and close to the oscilloscope’s period and we did have coherent ADC readings.

We suspect that the following factors somehow give problems to the system timer:

· Using processes: we compared just the sample acquisition routine as a process and as a function inside an infinite loop in the main function. The second situation’s results were more reasonable.

·Using variables inside the functions: Somehow the results were more logic using all variables as global, at least in some cases.

We have a fair obstacle avoidance and speed dynamic algorithm for the three frontal obstacle avoidance IR sensors and operational bump and vertigo behaviors, although there are calibration problems in peculiar situations of the vertigo sensors.

Our robot’s main characteristic is the rotating IR sensor. Its hardware is OK and the vision routine works only if it is the only running routine (otherwise the rotating period becomes extremely incoherent and unstable).

This problems might be due to Interactive C problems or some subtle error in our code, though it is strange that we don’t have those problems when the routine is running alone.

We still have one or two ideas to try. We would also like to test the routine in an assembly program to compare the results during this weekend.

As the main characteristic does not work, we are forced to look at this project as a failure. It was a project where we learnt a great deal, but it did not work. So, due to the pragmatic character of this course, this project was a failure. So, perhaps we shouldn’t call this robot Cyclop but Waterloo...

Documentation

[1] Tracy L. Anderson and Max Donath, "Animal Behavior as a Paradigm for Developing Robot Autonomy", edited by Pattie Maes, MIT/Elsevier; pp 145-168.

[2] Randall D. Beer, Hillel J. Chiel, and Leon S. Sterling, "A Biological Perspective on Autonomous Agent Design", edited by Pattie Maes, MIT/Elsevier; pp169-186.

[3] Valentino Braintenberg, "Vehicles: Experiments in Synthectic Psychology".

[4] R. A. Brooks, “Elephants don’t play chess” - extracted from “Designing Autonomous Agents”, edited by Pattie Maes, MIT/Elsevier, pp3-15.

[5] John J, D’Azzo, Constantine H. Houpis, “Linear Control System Analysis and Design Conventional and Modern”, McGraw-Hill International Editions, 3rd edition, pp 21, 505-541.

[6] Keith L. Doty and Akram Bou-Ghannam, "Controlling Situated Agent Behaviors with Consistent World Modeling and Reasoning".

[7] Keith L. Doty and Reid R. Harrison, "Sweep Stractegies for a Sensory-Driven, Behavior-Based Vacuum Cleanning Agent", AAAI 1993 Fall Symposium Series Instantiating Real-world Agents.

[8] Keith L. Doty, Stefano Caselli, Reid R. Harrison, Francesco Zanichelli, "Landmark Map Construction and Navigation in Enclosed Environments".

[9] Keith L. Doty, Stefano Caselli, Reid R. Harrison, Francesco Zanichelli, "Mobile Robot Navigation in Enclosed Large-scale Space".

[10] Keith L. Doty, "Position Paper on Lessons Learned from Implemented Software Architectures for Physical Agents".

[11] Keith L. Doty and Steven Louis Seed, "Autonomous Agent Map Construction in Unknown Enclosed Environments", MLC-COLT'94 Robot Learning Workshop, Rutgers, New Brunswick, N.J.

[12] Joseph L. Jones, Anita M. Flynn, "Mobile Robots Inspiration to Implementation", A.K.Peters, Ltd.

[13] K.S. Fu, R.C. Gonzalez, C.S.G. Lee, "Robotics Control, Sensing, Vision, and Intelligence", McGraw-Hill International Editions, pp. 267-293.

[14] Richard D. Klafter, Thomas A. Chmielewski, Michael Negin, "Robotic Enginnering An Integrated Approach", Prentice Hall, pp. 314-508, pp. 665-691.

[15] Fred Martin, "The 6.270 Robot Builder's Guide for the 1992 MIT LEGO Robot design Competition".

[16] Gregor Schoner and Michael Dose, “A dynamical systems approach to task-level system integration used to plan and control autonomous vehicle motion”.

# Appendices

Source code for Obstacle avoidance, vertigo and bump behaviors:

/*CONSTANTS*/

/*IR sensor readings when there are no obstacles*/

int Sensor0=105;

/*IR Sensor saturation readings*/

int SensorSat=160;

/*IR Sensor security reading*/                  /*######*/

int SensorSec=150;

/*IR left, center and right sensors' positions*/

float L_SENSOR_DEGREE = -45.0;

float C_SENSOR_DEGREE = 0.0;

float R_SENSOR_DEGREE = 45.0;

/*Repellor's width factor*/

float WIDTH_FACTOR = 4.0;

/*Obstacle's weight factor*/

float OBSTACLE_WEIGHT = 1.5;

/*Motor Compensation (one of our motor is slower than the other)*/

float MOTOR_COMPENS = 0.81;

/*Speed dynamics parameter (slope)*/

float m = 90.0/( (float) (SensorSat-Sensor0));

/*GLOBAL VARIABLES*/

/*IR Sensor readings initializations*/

int lSensor=Sensor0,

cSensor=Sensor0,

rSensor=Sensor0,

lDownSensor=SensorSec,                      /*######*/

rDownSensor=SensorSec;

/*Bumper readings initializations*/

int lBumpSensor,                 /*$$$$*/

cBumpSensor,

rBumpSensor;

/*Motor power obstacle avoidance behaviour's output*/

float lMotorObavoid, rMotorObavoid;

/*Vertigo behaviour's output*/          /*#######*/

int vertigoBack,

vertigoRight,

vertigoLeft;

/*Bump behaviour's output*/        /*$$$$$$$*/

int bumpBack,

bumpRight,

bumpLeft;

/* behavior's output flags */      /*&&&&&&*/

int bumpFlag=0,

vertigoFlag=0,

avoidFlag=0;

/*Speed dynamics' output*/

float Speed_dSpeed;

/*Robot's speed*/

float speed;

/*AUXILIARY FUNCTIONS*/

float abs(float x)

{

if (x<0.0) return(-x);

else return x;

}

float repellor(float x, float w)

/*

"x" is the obstacle's position

"w" is the repellor's width (allways positive)

*/

{

if ( x<-w || x==0.0 || x>w)

return 0.0;

else if (-w<=x && x<0.0)

return (-1.0);

else if (x>0.0 && x<=w)

return 1.0;

}

void wait(int milisec)

{

long timer_a;

timer_a = mseconds()+ (long) milisec;

while(timer_a > mseconds())

{

defer();

}

}

int random()

{

return (int) mseconds() & 0x3FF;

}

void turn_back(float speed)

{

motor(0, -speed);

motor(1, -speed);

wait(random()+500);

}

void turn_left(float speed)

{

motor(0, -speed);

motor(1, speed);

wait(random()+500);

}

void turn_right(float speed)

{

motor(0, speed);

motor(1, -speed);

wait(random()+500);

}

void random_turn(float speed)

{

if (random() & 1)

turn_left(speed);

else

turn_right(speed);

}

void sound(float frequency, float length)

{

pokeword(0x26,(int)(1E6 / frequency));

bit_set(0x1020,0b10000000);   /*PA7*/

sleep(length);

bit_clear(0x1020,0b10000000);

pokeword(0x26,0);

bit_clear(0x1000,8);          /*turn power to spkr off*/

}

/*PROCESSES/BEHAVIOURS*/

/*Sensing process*/

void sensing()

{

while(1)

{

lSensor=analog(1);

cSensor=analog(2);

rSensor=analog(0);

lDownSensor=analog(3);                      /*######*/

rDownSensor=analog(5);

lBumpSensor=digital(2);             /*$$$$$$*/

cBumpSensor=digital(1);

rBumpSensor=digital(0);

}

}

/*Speed dynamics process*/

void dSpeed()

{

int maximum;

while(1)

{

maximum = lSensor;

if (cSensor>maximum) maximum=cSensor;

if (rSensor>maximum) maximum=rSensor;

Speed_dSpeed = 100.0-m*( (float) (maximum-Sensor0));

}

}

/*Obstacle avoidance behaviour's process*/

void obavoid()

{

float headDot;

float left, center, right;

while(1)

{

left= (float) (lSensor-Sensor0);

center= (float) (cSensor-Sensor0);

right= (float) (rSensor-Sensor0);

headDot = left*repellor(-L_SENSOR_DEGREE,

left*WIDTH_FACTOR)+

center*repellor(-C_SENSOR_DEGREE,

center*WIDTH_FACTOR)+

right*repellor(-R_SENSOR_DEGREE,

right*WIDTH_FACTOR);

headDot = headDot/( (float) (SensorSat-Sensor0));

lMotorObavoid = speed*(1.0+OBSTACLE_WEIGHT*headDot);

rMotorObavoid = speed*(1.0-OBSTACLE_WEIGHT*headDot);

avoidFlag=1;

}

}

/*highs avoidance behaviour's process*/          /*####*/

void vertigo()

{

while(1)

{

if (lDownSensor < SensorSec && rDownSensor < SensorSec)

{

vertigoFlag=1;

vertigoBack=1;

vertigoRight=(random() & 1);

vertigoLeft=(random() & 1);

}

else if (lDownSensor < SensorSec)

{

vertigoFlag=1;

vertigoBack=1;

vertigoRight=0;

vertigoLeft=1;

}

else if (rDownSensor < SensorSec)

{

vertigoFlag=1;

vertigoBack=1;

vertigoLeft=0;

vertigoRight=1;

}

else vertigoFlag=0;

}

}

/*bump behaviour's process*/            /*$$$$$$$$*/

void bump()

{

while(1)

{

if (!lBumpSensor && !rBumpSensor)

{

bumpFlag=1;

bumpBack=1;

bumpRight=(random() & 1);

bumpLeft=(random() & 1);

}

else if (!lBumpSensor)

{

bumpFlag=1;

bumpBack=1;

bumpRight=0;

bumpLeft=1;

}

else if (!rBumpSensor)

{

bumpFlag=1;

bumpBack=1;

bumpLeft=0;

bumpRight=1;

}

else if (!cBumpSensor)

{

bumpFlag=1;

bumpBack=1;

bumpRight=(random() & 1);

bumpLeft=(random() & 1);

}

else bumpFlag=0;

}

}

/*Arbitration/integration process*/

void arbitration()

{

while(1)

{

if (vertigoFlag)               /*######*/

{

if (vertigoBack==1)

{

turn_back(30.0);

random_turn(30.0);

}

if (vertigoLeft==1)

{

turn_left(30.0);

random_turn(30.0);

}

if (vertigoRight==1)

{

turn_right(30.0);

random_turn(30.0);

}

/*      sound(1000.,.2);*/

}

if (bumpFlag)

{

if (bumpBack)  turn_back(25.0);

if (bumpLeft)  turn_left(25.0);

if (bumpRight) turn_right(25.0);

/*      sound(1500.,.2);*/

}

if (avoidFlag)

{

speed=Speed_dSpeed;

if (speed<26.0 && abs(lMotorObavoid-rMotorObavoid)<1.0)

random_turn(40.0);

else

{

motor(1,lMotorObavoid);

motor(0,MOTOR_COMPENS*rMotorObavoid);

}

}

}

}

/*MAIN FUNCTION*/

void main()

{

poke(0x4000,0x37);

start_process(sensing());

start_process(vertigo());

start_process(bump());

start_process(dSpeed());

start_process(obavoid());

start_process(arbitration());

}

Source code for the last version of the rotating sensor processes

/*CONSTANTS*/

int SYNCHRO_IR_HI=150;

int SYNCHRO_IR_LOW=135;

/*synchronizing led's thresholds*/

/*Rotating IR sensor readings when there are no obstacles*/

int rSensor0=119;

/*Rotating IR Sensor saturation readings*/

int rSensorSat=160;

int MAX_CELLS=16;/*Number of visual cells*/

float INV_MAX_CELLS = 1./( (float) MAX_CELLS);

int MAX_INDEX=4*MAX_CELLS;

/*Thus we define a matrix of a greater size than we really

need (prevent from getting out of its range*/

/*Repellor's width factor*/

float WIDTH_FACTOR = 0.4;

/*Obstacle's weight factor*/

float OBSTACLE_WEIGHT = 1.5/( (float) (rSensorSat-rSensor0));

/*Motor Compensation (one of our motor is slower than the other)*/

float MOTOR_COMPENS = 0.81;

/*Speed dynamics parameter (slope)*/

float rm = 95.0/( (float) (rSensorSat-rSensor0));

/*GLOBAL VARIABLES*/

int FirstVision=1;

/*Auxiliary flag to 1st peak synchronization*/

long IrMotorPeriod= (long) 2000;

int retina[64];/* "MAX_INDEX", to prevent array out

of range error*/

long time[64];/*instants of the retina samples*/

int index;

float lMotorObavoidRot, rMotorObavoidRot;

/*Motor power obstacle avoidance behaviors' output from rotating sensor*/

float speed=100.;

/*AUXILIARY FUNCTIONS*/

float repellor(float x, float w)

/*

"x" is the obstacle's position

"w" is the repellor's width (allways positive)

*/

{

if ( (x < -w) || (x == 0.0) || (x > w) )

return 0.0;

else if ((-w <= x) && (x < 0.0))

return (1.0);

else if ((x > 0.0) && (x <= w))

return (-1.0);

}/*float repellor(float x, float w)*/

/*PROCESSES*/

long tf, t;

long deltaT;

/*Rotating IR sensor's sample period*/

int indy, ReadingRot;

void vision()

{

/*

The following processing is based on the synchronizing LED's signal.

It gives a narrow pulse when the mirror passes by the back of the

robot.

*/

while (FirstVision)

{

if ( analog(4) >= SYNCHRO_IR_HI ) FirstVision=0;

}

while(analog(4) > SYNCHRO_IR_LOW);

/*Waits for negative transition*/

deltaT = (long) (( (float) IrMotorPeriod)*INV_MAX_CELLS);

/*Determining the rotating IR sensor's sample period*/

reset_system_time();

/*Negative transition instant determination*/

t= (long) 0;

indy=0;

while(SYNCHRO_IR_HI > (ReadingRot=analog(4)) )

/*Processing until positive transition */

{

tf=mseconds();

if (tf>=t && indy<MAX_INDEX)

{

retina[indy] = ReadingRot;

t += deltaT;

time[indy] = tf;

indy++;

}

}/*while(SYNCHRO_IR_HI > analog(4))*/

indy--;

index = indy;

IrMotorPeriod = tf;

/*Updating IR Motor Period*/

}/*void vision()*/

float headDot, Readings;

int indb, IMIN, IMAX;

float factor, additive;

/*Obstacle avoidance behaviour's process (Rotating IR sensor)*/

void rotObavoid()

{

headDot = 0.0;

IMIN = (3*index) >> 3;/*3/8index*/

IMAX = (5*index) >> 3;/*5/8index*/

additive = -0.5*( (float) index);

factor = ( (float) index)/( (float) IrMotorPeriod);

for (indb=IMIN; indb<=IMAX; indb++)

{

Readings = ( (float) retina[indb])-( (float) rSensor0);

if (Readings<0.0) Readings = 0.0;

headDot += Readings*repellor( -(additive+factor*

( (float) time[indb])),

Readings*WIDTH_FACTOR);

}

headDot = headDot;

lMotorObavoidRot = speed*(1.0+OBSTACLE_WEIGHT*headDot);

rMotorObavoidRot = speed*(1.0-OBSTACLE_WEIGHT*headDot);

}/*void rotObavoid()*/

void main()

{

poke(0x4000, 72);

/*Let's turn on the lights...*/

while(1)

{

vision();

/*rotObavoid();

motor(1, lMotorObavoidRot);

motor(0, rMotorObavoidRot);*/

}

}