/*	Program file	:	control.c					*/

/* 	Revision		: 	2.0					*/

/*	Programers	:	Joaquim Ferreira and Numo Gomes		*/

/*	Date		:	1995, July 24th

/*	Global declarations and initialisations				*/

int     pid1,pid2,pid3,pid4,

Left, Right,Center, FloorRight, FloorLeft,Floor0,Floor1,

FloorMissing=0,

ObjectRight,ObjectLeft,

ObjectFront,NoObject=1,

SUM_M=500,SUM_T=500,TS=95,T0=95,T1=105,T2=95,T3=115,T4=105,

s_right,s_left,s_front,

digital_in,

bump,

bumper_front,

bumper_right,

bumper_left,

bumper_back,

hungry,overload,

docking_area,

random_turn=0,

motor_right=1,

motor_left=0;

float full_speed=85.0,

half_speed=50.0,time,

wtime=0.2,spin_time=0.2;

/*	Self calibration module		*/

void get_thresholds()

{

if ( !(FloorMissing) && (NoObject) ) /* I’m running cool */

if ( (SUM_M > SUM_T+15) || (SUM_M < SUM_T-15) )  /* There was a modification on the enviorenement */

{

poke(0x4000,0xff);

sleep(wtime);

T0=analog(0); /* These will become the new thresholds for	*/

T1=analog(1); /* obstacles and floor detection	    */

T2=analog(2);

T3=analog(3);

T4=analog(4);

poke(0x4000,0x00);

SUM_T=T0+T1+T2+T3+T4; /* This will become the new perception of environmental brightness */

}

}

void sensor_module()

{

while(1)

{

/* Avoidance readings */

poke(0x4000,0xff); /* Turn on the LED’s */

sleep(wtime); /* Wait for the transients to die */

Floor0=analog(0);

Right=analog(1); /* Take the readings */

Center=analog(2);

Left=analog(3);

Floor1=analog(4);

poke(0x4000,0x00);

/* Beacon Searching readings */

s_front=analog(5);

s_right=analog(6);

s_left =analog(7);

/* Bump readings */

digital_in=peek(0x5000); /* Take the readings (negative logic) */

digital_in = ~digital_in; /* Invert the readings and go to positive logic  */

bumper_front = digital_in & 0x01;

bumper_right = digital_in & 0x02; /* Mask the individual bump switches */

bumper_left  = digital_in & 0x04;

bumper_back  = digital_in & 0x08;

bump=bumper_front || bumper_right || bumper_left;

/* Hungry and overload readings */

overload = digital_in & 0x10;

hungry   = digital_in & 0x20;

docking_area=((s_front >= 124) || (s_left >= 124) || (s_right >= 124)) && (hungry || overload);

/* If the beacon is blinding me and I’m hungry or overload then I reach the docking area */

SUM_M=Floor0+Floor1+Right+Left+Center; /* How much I see from every where?	   */

ObjectRight=(Right > T1); /* Is there an obstacle by my...*/

ObjectLeft=(Left > T3);

ObjectFront=(Center > T2);

FloorRight=(Floor0 > T0); /* Is there floor by my... */

FloorLeft=(Floor1 > T4);

FloorMissing=( !(FloorRight) || !(FloorLeft) );

NoObject=( !(ObjectRight) && !(ObjectLeft) && !(ObjectFront) );

}

}

void avoid_objects()

{

float f_speed,h_speed;

while(1)

{

if (hungry || overload) /* If hungry or overload, then move at half speed to save battery energy */

{

f_speed=full_speed/2.0;

h_speed=half_speed/2.0;

}

else

{

f_speed=full_speed;

h_speed=half_speed;

}

if (bumper_front)

{

motor(motor_right,-full_speed);

motor(motor_left,-full_speed);

sleep(spin_time);

motor(motor_left,full_speed);

sleep(spin_time);

}

else if (bumper_left)

{

motor(motor_left,-half_speed);

motor(motor_right,-half_speed);

sleep(spin_time);

motor(motor_right,full_speed);

sleep(spin_time);

}

else if (bumper_right)

{

motor(motor_left,-half_speed);

motor(motor_right,-half_speed);

sleep(spin_time);

motor(motor_left,full_speed);

sleep(spin_time);

}

else if (bumper_back)

{

motor(motor_right,full_speed);

motor(motor_left,full_speed);

sleep(spin_time);

motor(motor_left,-full_speed);

sleep(spin_time);

}

else if (docking_area) {while(1);} /* If inside docking area, do not avoid obstacles */

else if ((!FloorRight) && (FloorLeft) )

{

motor(motor_right,-full_speed);

motor(motor_left,0.0);

}

else if ( (FloorRight) && !(FloorLeft) )

{

motor(motor_left,-full_speed);

motor(motor_right,0.0);

}

else if ( !(FloorRight) && !(FloorLeft) )

{

motor(motor_right,-full_speed);

motor(motor_left,-half_speed);

}

else if( (NoObject) && !(FloorMissing) )

{

motor(motor_left,f_speed);

motor(motor_right,f_speed);

}

else if ( (ObjectFront) && !(FloorMissing) )

{

if (((int)seconds() % 30) >= 15) /* Change rotation direction every 15 seconds */

{

motor(motor_right,f_speed);

motor(motor_left,-f_speed);

}

else

{

motor(motor_right,-f_speed);

motor(motor_left,f_speed);

}

}

else if ((ObjectRight) && !(ObjectLeft || ObjectFront) && !(FloorMissing))

{

motor(motor_left,f_speed);

motor(motor_right,0.0);

}

else if ((ObjectLeft) && (!(ObjectRight || ObjectFront)) && !(FloorMissing))

{

motor(motor_left,0.0);

motor(motor_right,f_speed);

}

}

}

void search_station(void)

{

while(1)

{

while((!hungry) && (!overload)); /* If I’m not hungry  then I will do something else (next process) */

if (((s_front > TS) || (s_left > TS) || (s_right > TS)) && (!docking_area)) 	/* I found the beacon but I’m not in the docking area. Then... */

{

if ((s_front >= s_right) && (s_front >= s_left))

{

motor(motor_left,half_speed);

motor(motor_right,half_speed);

}

else if ((s_right > s_left) && (s_right > s_front))

{

motor(motor_left,half_speed);

motor(motor_right,0.0);

}

else if ((s_left > s_right) && (s_left > s_front))

{

motor(motor_left,0.0);

motor(motor_right,half_speed);

}

}

}

}

/* This is the fan control module! */

void fan_control(void)

{

while(1)

{

if (overload || hungry) poke(0x1000,0);

else poke(0x1000,0xff);

}

}

void main()

{

pid1=start_process(sensor_module());

pid4=start_process(fan_control());

pid2=start_process(avoid_objects());

pid3=start_process(search_station());

while(1)

{

if (docking_area && bump) 	 /* I was looking for the maintenance station */

/* because I was hungry or overloaded an I */

/* have just found it. Then...      	    	*/

{

motor(motor_right,0.0);	/* Stop the motors... */

motor(motor_left,0.0);

kill_process(pid1);		/* ...and have some peaceful rest!! */

kill_process(pid2);

kill_process(pid3);

poke(0x1000,0);

kill_process(pid4);

}

}

}